Instruction Sheet for the Candidate

| Qualification | Basic Machining |
| --- | --- |
| Competency Standard | Perform Metal/Bench Work |
| Purpose of Assessment | Formative assesment- |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this competency standard you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Select an appropriate Material and Make the following according to given dimensions - Develop Name Plate manually - Prepare Dovetail Joint - Cut Threads on Work Piece with tap and die |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to “Select appropriate tools and material and work as per given instructions “ demonstrating the following criteria: |
| Minimum Evidence Required | Name Plate: - Select marking tools - Hold the sheet in vice. - Cut sheet as per drawing - Perform surface finishing with file - Level the surface with tri-square - Mark the plate as per name requirements - Punch the marked area - Perform finishing with sand paper Dovetail Joint - Select marking tools - Cut sheet as per drawing - Perform surface finishing with file - Level the surface of both work pieces with tri-square - Mark both work pieces according to drawing - Create outer notch on work piece using flat file and hacksaw - Create inner notch using hacksaw and chisel - Compare both pieces by inserting outer notch into inner notch - Perform finishing with sand paper Cut Threads on Work Piece with tap and die - Identify different kind of taps & die according to requirement - Identify the work piece clamping method. - Apply tap and die alignment. - Apply lubricants while threading. - Avoid unwanted engraving and slips. - Identify proper threading procedure |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Basic Machining |
| Competency Standard | Perform Metal/Bench Work |
| Purpose of Assessment | Formative assesment- |
| Assessment Task | - Develop Name Plate manually - Prepare Dovetail Joint - Cut Threads on Work Piece with tap and die |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| Name Plate: |  |  |
| - Select marking tools |  |  |
| - Hold the sheet in vice. |  |  |
| - Cut sheet as per drawing |  |  |
| - Perform surface finishing with file |  |  |
| - Level the surface with tri-square |  |  |
| - Mark the plate as per name requirements |  |  |
| - Punch the marked area |  |  |
| - Perform finishing with sand paper |  |  |
| Dovetail Joint |  |  |
| - Select marking tools |  |  |
| - Cut sheet as per drawing |  |  |
| - Perform surface finishing with file |  |  |
| - Level the surface of both work pieces with tri-square |  |  |
| - Mark both work pieces according to drawing |  |  |
| - Create outer notch on work piece using flat file and hacksaw |  |  |
| - Create inner notch using hacksaw and chisel |  |  |
| - Compare both pieces by inserting outer notch into inner notch |  |  |
| - Perform finishing with sand paper |  |  |
| Cut Threads on Work Piece with tap and die |  |  |
| - Identify different kind of taps & die according to requirement |  |  |
| - Identify the work piece clamping method. |  |  |
| - Apply tap and die alignment. |  |  |
| - Apply lubricants while threading. |  |  |
| - Avoid unwanted engraving and slips. |  |  |
| - Identify proper threading procedure |  |  |
| - Create inner notch using hacksaw and chisel |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Basic Machining |
| --- | --- |
| Competency Standard | Perform Metal/Bench Work |
| Purpose of Assessment | Formative assesment- |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Develop Name Plate manually - Prepare Dovetail Joint - Cut Threads on Work Piece with tap and die |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select marking tools |  |  |  |  |  |
|  | Hold the sheet in vice. |  |  |  |  |  |
|  | Cut sheet as per drawing |  |  |  |  |  |
|  | Perform surface finishing with file |  |  |  |  |  |
|  | Level the surface with tri-square |  |  |  |  |  |
|  | Mark the plate as per name requirements |  |  |  |  |  |
|  | Punch the marked area |  |  |  |  |  |
|  | Perform finishing with sand paper |  |  |  |  |  |
|  | Dovetail Joint |  |  |  |  |  |
|  | Select marking tools |  |  |  |  |  |
|  | Cut sheet as per drawing |  |  |  |  |  |
|  | Perform surface finishing with file |  |  |  |  |  |
|  | Level the surface of both work pieces with tri-square |  |  |  |  |  |
|  | Mark both work pieces according to drawing |  |  |  |  |  |
|  | Create outer notch on work piece using flat file and hacksaw |  |  |  |  |  |
|  | Create inner notch using hacksaw and chisel |  |  |  |  |  |
|  | Compare both pieces by inserting outer notch into inner notch |  |  |  |  |  |
|  | Perform finishing with sand paper |  |  |  |  |  |
|  | Cut Threads on Work Piece with tap and die |  |  |  |  |  |
|  | Identify different kind of taps & die according to requirement |  |  |  |  |  |
|  | Identify the work piece clamping method. |  |  |  |  |  |
|  | Apply tap and die alignment. |  |  |  |  |  |
|  | Apply lubricants while threading. |  |  |  |  |  |
|  | Avoid unwanted engraving and slips. |  |  |  |  |  |
|  | Identify proper threading procedure |  |  |  |  |  |
|  | Create inner notch using hacksaw and chisel |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Basic Machining |
| --- | --- |
| Competency Standard | Perform Metal/Bench Work |
| Purpose of Assessment | Formative assesment- |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define basic measurement |  |  |
|  | Describe clamping/holding methods. |  |  |
|  | Understanding of chiseling process |  |  |
|  | Knowledge of radius gauge |  |  |
|  | Describe basic measuring /Marking /cutting tools |  |  |
|  | Describe use of round file |  |  |
|  | Understanding of types of fits. |  |  |
|  | Describe types of chisels |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________