Instruction Sheet for the Candidate

| Qualification | Foundry & Casting |
| --- | --- |
| Competency Standard | Preform Pattern Making |
| Purpose of Assessment | Formative Assessment – Pattren making |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Make a pattern for a pipe casting - Practice of making a pattern, on CNC Router - Make a single piece pattern of any English word from a wood plate - Make two-piece patterns (step pulley) along with core prints shaped pattern - Application of CNC Router for complex. |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Develop the Patterns given in Annexure-A as per given dimension and specification “ demonstrating the following criteria: |
| Minimum Evidence Required | - Prepare layout by adding allowances (draft, machining, and shrinkage, shaking). Arrange square/ block deodar wood piece. - Face and turn outer end by holding wooden piece chuck. - Prepare another core print diameter, mark parting line. Cut the pattern and fit dowel pin holes and assemble the pattern symmetrically. - Prepare the drawing in CAD system keeping in view allowances for the job. Arrange a wood block and clamp it on the table of CNC router. Send the drawing in CAM system. - Select the cutting tool with respect to the radius and fillets as per material and operation. Enter the raw material detail. Check the tool off setting, See the simulation before starting the work. - Remove the burr from the sharp corner. At completion check dimensions. - Polish the pattern with varnish for storing it. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Foundry & Casting |
| Competency Standards | Preform Pattern Making |
| Purpose of Assessment | Formative Assessment – Pattren making |
| Assessment Task | - Make a pattern for a pipe casting - Practice of making a pattern, on CNC Router - Make a single piece pattern of any English word from a wood plate - Make two-piece patterns (step pulley) along with core prints shaped pattern - Application of CNC Router for complex. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare layout by adding allowances (draft, machining, and shrinkage, shaking). |  |  |
| - Facing and turning by holding wooden piece. |  |  |
| - Prepare core print diameter and mark parting line. |  |  |
| - Cut the pattern and fit dowel pin holes and assemble the pattern symmetrically. |  |  |
| - Prepare the drawing in CAD system. Send the drawing in CAM system. |  |  |
| - Clamp it on the table of CNC router. |  |  |
| - Select the cutting tool as per material and operation. Enter the raw material detail. Check the tool off setting. |  |  |
| - Simulation before starting the work. |  |  |
| - Remove the burr from the sharp corner and check dimensions. |  |  |
| - Polish the pattern with varnish. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Foundry & Casting |
| --- | --- |
| Competency Standards | Preform Pattern Making |
| Purpose of Assessment | Formative Assessment – Pattren making |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Make a pattern for a pipe casting 2. Practice of making a pattern, on CNC Router 3. Make a single piece pattern of any English word from a wood plate 4. Make two-piece patterns (step pulley) along with core prints shaped pattern. 5. Application of CNC Router for complex as given in Annexure-A |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepare layout by adding allowances (draft, machining, and shrinkage, shaking). Arrange square/ block deodar wood piece. |  |  |  |  |  |
|  | Face and turn outer end by holding wooden piece chuck. |  |  |  |  |  |
|  | Prepare another core print diameter, mark parting line. Cut the pattern and fit dowel pin holes and Assemble the pattern symmetrically. |  |  |  |  |  |
|  | Prepare the drawing in CAD system keeping in view allowances for the job. Arrange a wood block and clamp it on the table of CNC router. Send the drawing in CAM system. |  |  |  |  |  |
|  | Select the cutting tool with respect to the radius and fillets as per material and operation. Enter the raw material detail. Check the tool off setting, See the simulation before starting the work. |  |  |  |  |  |
|  | Remove the burr from the sharp corner. At completion check dimensions. |  |  |  |  |  |
|  | Polish the pattern with varnish for storing it. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Foundry & Casting |
| --- | --- |
| Competency Standards | Preform Pattern Making |
| Purpose of Assessment | Formative Assessment – Pattren making |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Question Define allowance and allowances used for pattern making? |  |  |
|  | Candidate’s response |  |  |
|  | Question Describe pattern making machines? |  |  |
|  | Candidate’s response |  |  |
|  | Question Tools required for pattern making |  |  |
|  | Candidate’s response |  |  |
|  | Question Importance of pattern making. |  |  |
|  | Candidate’s response |  |  |
|  | Question What are the pattern making techniques? |  |  |
|  | Candidate’s response |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________