Instruction Sheet for the Candidate

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| --- | --- |
| Competency Standard | Perform Cleaning and Finishing Of Casting |
| Purpose of Assessment | Formative Assessment – Perform Cleaning and Finishing Of Casting |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Cleaning and finishing of casting. - Cleaning and finishing of casting by sand blasting and hand grinding. - Cleaning and finishing by tumbling barrel machine. |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to “Perform Cleaning and finishing of casting by sand blasting, hand grinding tumbling barrel machine“ demonstrating the following criteria: |
| Minimum Evidence Required | 1- Cleaning and finishing of casting. - Brake mold and pick aluminum casting. - Remove sand and cut sprue runner and riser with hand hacksaw. - Hold casting in vice and remove projected part by cape chisel and hammer and apply wire brush. - Remove all fins and casting is ready. 2- Cleaning and finishing by sand blasting and hand grinding. - Place cast iron component in sand blasting machine. - Hold sand blasting gun and direct stream towards casting. - Apply on the surface and change direction. - Bring the casting out after all surface is cleaned. - Clean excessive parts by hand grinder after holding it in vice. - Component is finished now. 3- Cleaning and finishing by tumbling barrel machine. - Charge the cast component by opening hole of drum. - Close drum after charging. - Run machine at 20 rpm and carry out this for 45 min. - Stop machine and open charging hole. - Drop cast component on ground before other cleaning operations. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| Competency Standards | Perform Cleaning and Finishing Of Casting |
| Purpose of Assessment | Formative Assessment – Perform Cleaning and Finishing Of Casting |
| Assessment Task | - Cleaning and finishing of casting. - Cleaning and finishing of casting by sand blasting and hand grinding. - Cleaning and finishing by tumbling barrel machine. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| 1- Cleaning and finishing of casting. |  |  |
| - Brake mold and pick aluminum casting. |  |  |
| - Remove sand and cut sprue runner and riser with hand hacksaw. |  |  |
| - Hold casting in vice and remove projected part by cape chisel and hammer and apply wire brush. |  |  |
| - Remove all fins and casting is ready. |  |  |
| 2- Cleaning and finishing by sand blasting and hand grinding. |  |  |
| - Place cast iron component in sand blasting machine. |  |  |
| - Hold sand blasting gun and direct stream towards casting. |  |  |
| - Apply on the surface and change direction. |  |  |
| - Bring the casting out after all surface is cleaned. |  |  |
| - Clean excessive parts by hand grinder after holding it in vice. |  |  |
| - Component is finished now. |  |  |
| 3- Cleaning and finishing by tumbling barrel machine. |  |  |
| - Charge the cast component by opening hole of drum. |  |  |
| - Close drum after charging. |  |  |
| - Run machine at 20 rpm and carry out this for 45 min. |  |  |
| - Stop machine and open charging hole. |  |  |
| - Drop cast component on ground before other cleaning operations. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| --- | --- |
| Competency Standards | Perform Cleaning and Finishing Of Casting |
| Purpose of Assessment | Formative Assessment – Perform Cleaning and Finishing Of Casting |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Cleaning and finishing of casting. - Cleaning and finishing of casting by sand blasting and hand grinding. - Cleaning and finishing by tumbling barrel machine. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| 1- Cleaning and finishing of casting. |  |  |  |  |  |  |
| 1. | Brake mold and pick aluminum casting. |  |  |  |  |  |
| 2. | Remove sand and cut sprue runner and riser with hand hacksaw. |  |  |  |  |  |
| 3. | Hold casting in vice and remove projected part by cape chisel and hammer and apply wire brush. |  |  |  |  |  |
| 4. | Remove all fins and casting is ready. |  |  |  |  |  |
| 2- Cleaning and finishing by sand blasting and hand grinding. |  |  |  |  |  |  |
| 5. | Place cast iron component in sand blasting machine. |  |  |  |  |  |
| 6. | Hold sand blasting gun and direct stream towards casting. |  |  |  |  |  |
| 7. | Apply on the surface and change direction. |  |  |  |  |  |
| 8. | Bring the casting out after all surface is cleaned. |  |  |  |  |  |
| 9. | Clean excessive parts by hand grinder after holding it in vice. |  |  |  |  |  |
| 10. | Component is finished now. |  |  |  |  |  |
| 3- Cleaning and finishing by tumbling barrel machine. |  |  |  |  |  |  |
| 11. | Charge the cast component by opening hole of drum. |  |  |  |  |  |
| 12. | Close drum after charging. |  |  |  |  |  |
| 13. | Run machine at 20 rpm and carry out this for 45 min. |  |  |  |  |  |
| 14. | Stop machine and open charging hole. |  |  |  |  |  |
| 15. | Drop cast component on ground before other cleaning operations. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| --- | --- |
| Competency Standards | Perform Cleaning and Finishing Of Casting |
| Purpose of Assessment | Formative Assessment – Perform Cleaning and Finishing Of Casting |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Question Describe tools used for cleaning of casting? |  |  |
|  | Candidate’s response |  |  |
|  | Question Define sand blasting? |  |  |
|  | Candidate’s response |  |  |
|  | Question Describe use of hand grinder? |  |  |
|  | Candidate’s response |  |  |
|  | Question What are Charge of cupola? |  |  |
|  | Candidate’s response |  |  |
|  | Question Describe operation of tumbling barrel machine? |  |  |
|  | Candidate’s response |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________