Instruction Sheet for the Candidate

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |  |
| --- | --- | --- |
| Competency Standard | Perform Sand Testing |  |
| Purpose of Assessment | Formative Assessment – Perform Sand Testing |  |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |  |
| Guidance for Candidate | To meet this standard you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Clay contents test of molding sand - Permeability number test for molding sand - Green compressive strength test of molding sand |  |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to “Perform Clay contents test, Permeability number test and Green compressive strength test of molding sand“ demonstrating the following criteria: |  |
| Minimum Evidence Required | 1- Clay contents test of molding sand . - Take sample of 50-gram dry sand and put sample in AFS clay jar. - Add 475 cc distilled water and 20cc 3% solution of NAOH and stir it. - Wash sand on stirrer and fill jar up to 6 inch and allow it to settle down. - Siphon off upper 5-inch water and again add distilled water upto 6 inch. - Siphon off upper 5 inch and repeat practice until water is clear to depth of 5 inch. - Remove sand grains from jar, dry and weigh. Difference between dried weight and original weight multiplied by 2 give clay % 2- Permeability number test for molding sand - Prepare permeability meter and install ram tube with molding sand specimen. - Place ram tube on tapered nose piece and fill outer cylinder with water. - Install a bell and allowed to sink by its own weight by multi position cock. - Record permeability number when air escapes through nozzle by sand specimen. - Calculate permeability number by formula. 3- Green compressive strength test of molding sand. - Pour 50 mm sample in ram tube and give three blows of ram rod. - Remove specimen from ram tube with pusher rod. - Place specimen in compressive strength tester. - Apply load by turning hand wheel. - Record reading when specimen fail. - Calculate compressive strength by formula. |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| Competency Standards | Perform Sand Testing |
| Purpose of Assessment | Formative Assessment – Perform Sand Testing |
| Assessment Task | - Clay contents test of molding sand - Permeability number test for molding sand - Green compressive strength test of molding sand |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| 1- Clay contents test of molding sand |  |  |
| 1. Take sample of 50-gram dry sand and put sample in AFS clay jar. |  |  |
| 2. Add 475 cc distilled water and 20cc 3% solution of NAOH and stir it. |  |  |
| 3. Wash sand on stirrer and fill jar up to 6 inch and allow it to settle down. |  |  |
| 4. Siphon off upper 5-inch water and again add distilled water upto 6 inch. |  |  |
| 5. Siphon off upper 5 inch and repeat practice until water is clear to depth of 5 inch. |  |  |
| 6. Remove sand grains from jar, dry and weigh. Difference between dried weight and original weight multiplied by 2 give clay % |  |  |
| 2- Permeability number test for molding sand |  |  |
| 7. Prepare permeability meter and install ram tube with molding sand specimen. |  |  |
| 8. Place ram tube on tapered nose piece and fill outer cylinder with water. |  |  |
| 9. Install a bell and allowed to sink by its own weight by multi position cock. |  |  |
| 10. Record permeability number when air escapes through nozzle by sand specimen. |  |  |
| 11. Calculate permeability number by formula. |  |  |
| 3- Green compressive strength test of molding sand. |  |  |
| 12. Pour 50 mm sample in ram tube and give three blows of ram rod. |  |  |
| 13. Remove specimen from ram tube with pusher rod. |  |  |
| 14. Place specimen in compressive strength tester. |  |  |
| 15. Apply load by turning hand wheel. |  |  |
| 16. Record reading when specimen fail. |  |  |
| 17. Calculate compressive strength by formula. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| --- | --- |
| Competency Standards | Perform Sand Testing |
| Purpose of Assessment | Formative Assessment – Perform Sand Testing |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Clay contents test of molding sand - Permeability number test for molding sand - Green compressive strength test of molding sand |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| 1- Clay contents test of molding sand |  |  |  |  |  |  |
|  | 1. Take sample of 50-gram dry sand and put sample in AFS clay jar. |  |  |  |  |  |
|  | 2. Add 475 cc distilled water and 20cc 3% solution of NAOH and stir it. |  |  |  |  |  |
|  | 3. Wash sand on stirrer and fill jar up to 6 inch and allow it to settle down. |  |  |  |  |  |
|  | 4. Siphon off upper 5-inch water and again add distilled water up to 6 inch. |  |  |  |  |  |
|  | 5. Siphon off upper 5 inch and repeat practice until water is clear to depth of 5 inch. |  |  |  |  |  |
|  | 6. Remove sand grains from jar, dry and weigh. Difference between dried weight and original weight multiplied by 2 give clay % |  |  |  |  |  |
| 2- Permeability number test for molding sand |  |  |  |  |  |  |
|  | 7. Prepare permeability meter and install ram tube with molding sand specimen. |  |  |  |  |  |
|  | 8. Place ram tube on tapered nose piece and fill outer cylinder with water. |  |  |  |  |  |
|  | 9. Install a bell and allowed to sink by its own weight by multi position cock. |  |  |  |  |  |
|  | 10. Record permeability number when air escapes through nozzle by sand specimen. |  |  |  |  |  |
|  | 11. Calculate permeability number by formula. |  |  |  |  |  |
| 3- Green compressive strength test of molding sand. |  |  |  |  |  |  |
|  | 12. Pour 50 mm sample in ram tube and give three blows of ram rod. |  |  |  |  |  |
|  | 13. Remove specimen from ram tube with pusher rod. |  |  |  |  |  |
|  | 14. Place specimen in compressive strength tester. |  |  |  |  |  |
|  | 15. Apply load by turning hand wheel. |  |  |  |  |  |
|  | 16. Record reading when specimen fail. |  |  |  |  |  |
|  | 17. Calculate compressive strength by formula. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| --- | --- |
| Competency Standards | Perform Sand Testing |
| Purpose of Assessment | Formative Assessment – Perform Sand Testing |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Question Define AFS in sand? |  |  |
|  | Candidate’s response |  |  |
|  | Question Describe testing of foundry sand? |  |  |
|  | Candidate’s response |  |  |
|  | Question Define Properties of green sand? |  |  |
|  | Candidate’s response |  |  |
|  | Question What Type of strength measure in sand test? |  |  |
|  | Candidate’s response |  |  |
|  | Question Define Permeability? |  |  |
|  | Candidate’s response |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________