Instruction Sheet for the Candidate

| Qualification | Level-5 in Mechanical Technology (Metrology). |
| --- | --- |
| Competency standards | 9.9.2. Take measurements with combination set |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Perform leveling with square head as spirit level. - Measure depth with square head as depth gauge - Measure height with square head as height gauge - Perform leveling /measure angle of surfaces with protractor Head - Perform centering with center head - Find diameter of round bar |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Take the measurements of an object with the help of Combination Set”. |
| Minimum Evidence Required | Leveling, Depth and Height Measurement with square head 1. Place the spirit level on horizontal work piece. 2. Check the deviation of bubbles 3. Level the surface by inserting shims. 4. Place square head on vertical work piece. 5. Note the deviation and correct the level. 6. Check the other geometrical shapes. 7. In order to measure the depth of work piece, apply force at the head to retain contact with the component. 8. Set square head and record the depth. 9. Measure height of work piece with and without the help of height gauge attachment. Angle Measurement with Protractor Head 10. Place protractor head of combination set on work piece. 11. Loose screws of revolving circular disk and adjust the bubble of level to minimize deviation. 12. Study the angle between slopes of a component. Centering and diameter measurement by Centre Head 13. Place the center head on round stock. 14. Trace the line with scriber. 15. Revolve the center head by 90o and draw another line. 16. Find center of round stock by bisecting point of lines. 17. In order to measure diameter, Note the reference point and measure point by avoiding parallax 18. Find the difference between reference point and measure point. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Level-5 in Mechanical Technology (Metrology) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | 1. Perform leveling with square head as spirit level. 2. Measure depth with square head as depth gauge 3. Measure height with square head as height gauge 4. Perform leveling /measure angle of surfaces with protractor Head 5. Perform centering with center head 6. Find diameter of round bar |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Place the spirit level on horizontal work piece. |  |  |
| - Check the deviation of bubbles |  |  |
| 3. Level the surface by inserting shims. |  |  |
| 4. Place square head on vertical work piece. |  |  |
| 5. Note the deviation and correct the level. |  |  |
| 6. Check the other geometrical shapes. |  |  |
| - In order to measure the depth of work piece, apply force at the head to retain contact with the component. |  |  |
| - Set square head and record the depth. |  |  |
| - Measure height of work piece with and without the help of height gauge attachment. |  |  |
| - Place protractor head of combination set on work piece. |  |  |
| - Loose screws of revolving circular disk and adjust the bubble of level to minimize deviation. |  |  |
| - Study the angle between slopes of a component. |  |  |
| - Place the center head on round stock. |  |  |
| - Trace the line with scriber. |  |  |
| - Revolve the center head by 90o and draw another line. |  |  |
| - Find center of round stock by bisecting point of lines. |  |  |
| - In order to measure diameter, Note the reference point and measure point by avoiding parallax |  |  |
| - Find the difference between reference point and measure point. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Level-5 in Mechanical Technology (Calibration). |
| --- | --- |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Perform leveling with square head as spirit level. 2. Measure depth with square head as depth gauge 3. Measure height with square head as height gauge 4. Perform leveling /measure angle of surfaces with protractor Head 5. Perform centering with center head 6. Find diameter of round bar |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Place the spirit level on horizontal work piece. |  |  |  |  |  |
|  | Check the deviation of bubbles |  |  |  |  |  |
|  | Level the surface by inserting shims. |  |  |  |  |  |
|  | Place square head on vertical work piece. |  |  |  |  |  |
|  | Note the deviation and correct the level. |  |  |  |  |  |
|  | Check the other geometrical shapes. |  |  |  |  |  |
|  | In order to measure the depth of work piece, apply force at the head to retain contact with the component. |  |  |  |  |  |
|  | Set square head and record the depth. |  |  |  |  |  |
|  | Measure height of work piece with and without the help of height gauge attachment. |  |  |  |  |  |
|  | Place protractor head of combination set on work piece. |  |  |  |  |  |
|  | Loose screws of revolving circular disk and adjust the bubble of level to minimize deviation. |  |  |  |  |  |
|  | Study the angle between slopes of a component. |  |  |  |  |  |
|  | Place the center head on round stock. |  |  |  |  |  |
|  | Trace the line with scriber. |  |  |  |  |  |
|  | Revolve the center head by 90o and draw another line. |  |  |  |  |  |
|  | Find center of round stock by bisecting point of lines. |  |  |  |  |  |
|  | In order to measure diameter, Note the reference point and measure point by avoiding parallax |  |  |  |  |  |
|  | Find the difference between reference point and measure point. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Level-5 in Mechanical Technology (Metrology). |
| --- | --- |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What are the parts of the combination set? |  |  |
|  | What is the purpose of Scriber in Square head? |  |  |
|  | What is the purpose of spirit level in square head? |  |  |
|  | What is the purpose of locking nut in the combination set? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________