Instruction Sheet for the Candidate

| Qualification | Level-5 in Mechanical Technology (Calibration). |
| --- | --- |
| Competency standards | 9.7.2. Calibration of Mechanical Measuring Instruments/Tools |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): 1: Calibrate Vernier Caliper. 2: Calibrate Micrometer Screw Gauge. 3: Calibrate Dial Gauge. 4: Calibrate Gauges. 5: Calibrate Sensors. 6: Calibrate Tachometer. |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Calibrate the Vernier Caliper, Screw Gauge, Dial Gauge, Gauges, Sensors and Tachometer”. |
| Minimum Evidence Required | Vernier Caliper and Micrometer Screw Gauge - Close the jaws of both Vernier Caliper and Micrometer Screw Gauge. - Check the zero error. - If there is any zero error, open up the jaws and precisely match the both jaws and then assemble the device. - Check the zero error again. If the zeros of both Vernier scale and main scales (Circular scale for micrometer) are coinciding with each other, then the Vernier caliper and Micrometer Screw Gauge are calibrated. Dial Gauge - In order to check the calibration of Dial Gauge, place the gauge on a flat surface and check its correctness. - Check the zero error. - If there is any zero error then check the push needle, either its loose or tight? If spring is loose, then open the spring and replace it. - If pin is working properly then, there are two possibilities either there is a fault in the surface base plate or in the gauge needle. - Check the flatness of surface base plate. - If everything is working properly with the base plate, then disassemble the gauge and start fixing its needle by checking its position. - After fixing the gauge needle, close the gauge and check its zero error again. if it gives an exact value then the gauge is calibrated. Gauges - Verify the type of gauge first. - Check the zero error - If there is any zero error, then carefully disassemble the gauge by pouring the fluid in a clean bowl. - After dismantling each component of the gauge now check the needle’s base either its loose or tight, if it is loose then fix the problem and re-assemble the gauge and check its zero error again. if needle is on exactly upon zero then the gauge is calibrated Sensors - Make it sure that which type of sensor needs to be calibrated first. - Trace the fault. - After knowing the fault, fix it. Tachometer - Check the speed of any revolving part by using the tachometer. - If rpm is not directly proportional to the regulatory switch i.e., by increasing or decreasing rpm it has some fluctuations, then it needs to be fixed. - Carefully, dismantle the device and find the fault, either it is mechanical issue or some electronic issue. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Level-5 in Mechanical Technology (Calibration) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | 1: Calibrate Vernier Caliper 2: Calibrate Micrometer Screw Gauge 3: Calibrate Dial Gauge 4: Calibrate Gauges 5: Calibrate Sensors 6: Calibrate Tachometer |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Close the jaws of both Vernier Caliper and Micrometer Screw Gauge. |  |  |
| - Check the zero error. |  |  |
| - If there is any zero error, open up the jaws and precisely match the both jaws and then assemble the device. |  |  |
| - Check the zero error again. If the zeros of both Vernier scale and main scales (Circular scale for micrometer) are coinciding with each other, then the Vernier caliper and Micrometer Screw Gauge are calibrated. |  |  |
| - In order to check the calibration of Dial Gauge, place the gauge on a flat surface and check its correctness. |  |  |
| - Check the zero error. |  |  |
| - If there is any zero error then check the push needle, either its loose or tight? If spring is loose, then open the spring and replace it. |  |  |
| - If pin is working properly then, there are two possibilities either there is a fault in the surface base plate or in the gauge needle. |  |  |
| - Check the flatness of surface base plate. |  |  |
| - If everything is working properly with the base plate, then disassemble the gauge and start fixing its needle by checking its position. |  |  |
| - After fixing the gauge needle, close the gauge and check its zero error again. if it gives an exact value then the gauge is calibrated. |  |  |
| - Verify the type of gauge first. |  |  |
| - Check the zero error |  |  |
| - If there is any zero error, then carefully disassemble the gauge by pouring the fluid in a clean bowl. |  |  |
| - After dismantling each component of the gauge now check the needle’s base either its loose or tight, if it is loose then fix the problem and re-assemble the gauge and check its zero error again. if needle is on exactly upon zero then the gauge is calibrated |  |  |
| - Make it sure that which type of sensor needs to be calibrated first. |  |  |
| - Trace the fault. |  |  |
| - After knowing the fault, fix it. |  |  |
| - Check the speed of any revolving part by using the tachometer. |  |  |
| - If rpm is not directly proportional to the regulatory switch i.e., by increasing or decreasing rpm it has some fluctuations, then it needs to be fixed. |  |  |
| - Carefully, dismantle the device and find the fault, either it is mechanical issue or some electronic issue. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Level-5 in Mechanical Technology (Calibration). |
| --- | --- |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Perform the calibration of Vernier Caliper. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Close the jaws of both Vernier Caliper and Micrometer Screw Gauge. |  |  |  |  |  |
|  | Check the zero error. |  |  |  |  |  |
|  | If there is any zero error, open up the jaws and precisely match the both jaws and then assemble the device. |  |  |  |  |  |
|  | Check the zero error again. If the zeros of both Vernier scale and main scales (Circular scale for micrometer) are coinciding with each other, then the Vernier caliper and Micrometer Screw Gauge are calibrated. |  |  |  |  |  |
|  | In order to check the calibration of Dial Gauge, place the gauge on a flat surface and check its correctness. |  |  |  |  |  |
|  | Check the zero error. |  |  |  |  |  |
|  | If there is any zero error, then check the push needle, either its loose or tight? If spring is loose, then open the spring and replace it. |  |  |  |  |  |
|  | If pin is working properly then, there are two possibilities either there is a fault in the surface base plate or in the gauge needle. |  |  |  |  |  |
|  | Check the flatness of surface base plate. |  |  |  |  |  |
|  | If everything is working properly with the base plate, then disassemble the gauge and start fixing its needle by checking its position. |  |  |  |  |  |
|  | After fixing the gauge needle, close the gauge and check its zero error again. if it gives an exact value then the gauge is calibrated. |  |  |  |  |  |
|  | Verify the type of gauge first. |  |  |  |  |  |
|  | Check the zero error |  |  |  |  |  |
|  | If there is any zero error, then carefully disassemble the gauge by pouring the fluid in a clean bowl. |  |  |  |  |  |
|  | After dismantling each component of the gauge now check the needle’s base either its loose or tight, if it is loose then fix the problem and re-assemble the gauge and check its zero error again. if needle is on exactly upon zero then the gauge is calibrated |  |  |  |  |  |
|  | Make it sure that which type of sensor needs to be calibrated first. |  |  |  |  |  |
|  | Trace the fault. |  |  |  |  |  |
|  | After knowing the fault, fix it. |  |  |  |  |  |
|  | Check the speed of any revolving part by using the tachometer. |  |  |  |  |  |
|  | If rpm is not directly proportional to the regulatory switch i.e., by increasing or decreasing rpm it has some fluctuations, then it needs to be fixed. |  |  |  |  |  |
|  | Carefully, dismantle the device and find the fault, either it is mechanical issue or some electronic issue. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Level-5 in Mechanical Technology (Calibration). |
| --- | --- |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Name the main components of Vernier Caliper and Micrometer Screw Gauge? |  |  |
|  | Define Least Count? |  |  |
|  | What is the Zero Error? |  |  |
|  | Name the types of zero error? |  |  |
|  | Formula of Least Count = ? |  |  |
|  | How many types of fluids commonly use in the gauge? |  |  |
|  | What is the purpose of fluid inside the gauge? |  |  |
|  | Can we use dial gauge as a reference to set micrometer and caliper? |  |  |
|  | Write the relation between linear and rotational speed? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________