Instruction Sheet for the Candidate

| Qualification | Hydraulics and Hydraulic Machinery(Level-5) |
| --- | --- |
| Competency Standard | Operate Hydraulic Press |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Operate hydraulic press |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Operate hydraulic press “demonstrating the following criteria: |
| Minimum Evidence Required | - Prepare the work piece as per size requirement. - Inspect the press for oil level and other necessary elements. - Turn on the hydraulic press and select the load according to the thickness of the material. - Place the work piece between the die and bench. - Exert a force on the work piece with the help of punch. This would prepare the work piece to fit in the die. - Turn off the hydraulic press and note all reading and calculate power required to drive the hydraulic press. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Hydraulics and Hydraulic Machinery |
| Competency Standard | Operate Hydraulic Press |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Operate hydraulic press |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare the work piece as per size requirement. |  |  |
| - Inspect the press for oil level and other necessary elements. |  |  |
| - Turn on the hydraulic press and select the load according to the thickness of the material |  |  |
| - Place the work piece between the die and bench. |  |  |
| - Exert a force on the work piece with the help of punch. This would prepare the work piece to fit in the die. |  |  |
| - Turn on/off the hydraulic press and note all reading and calculate power required to drive the hydraulic press. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Hydraulics and Hydraulic Machinery |
| --- | --- |
| Competency Standard | Operate Hydraulic Press |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Operate Hydraulic Press |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepared the work piece as per size requirement. |  |  |  |  |  |
|  | Inspect the press for oil level and other necessary elements. |  |  |  |  |  |
|  | Turned on the hydraulic press and select the load according to the thickness of the material |  |  |  |  |  |
|  | Placed the work piece between the die and bench. |  |  |  |  |  |
|  | Exerted a force on the work piece with the help of punch. This would prepare the work piece to fit in the die. |  |  |  |  |  |
|  | Turned on/off the hydraulic press and note all reading and calculate power required to drive the hydraulic press. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Hydraulics and Hydraulic Machinery |
| --- | --- |
| Competency Standard | Operate Hydraulic Press |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define a hydraulic press. |  |  |
|  | Describe working principle of hydraulic press. |  |  |
|  | Describe Pascal’s Law. |  |  |
|  | Define Mechanical Advantage of a machine. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________