Instruction Sheet for the Candidate

| Qualification | Hydraulics and Hydraulic Machinery(Level-5) |
| --- | --- |
| Competency Standard | Analyze the performance of Turbines |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Calculate the performance of impulse turbine/ Pelton Wheel - Calculate the performance of Reaction Turbine/ Francis Turbine |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Check Performance of Turbines “demonstrating the following criteria: |
| Minimum Evidence Required | - Keep the nozzle-opening to the required position. - Allow the water into the turbine and the turbine will start rotating. - Fix the weigh hanger to the rope of the brake drum with no load on weight hanger. - Vary the gate vale, keep the head constant using the pressure gauge to the required head in case of experiment on constant head or keep the speed constant using the tachometer to the required speed in case of experiment on constant speed. - Note down the following readings of Pressure gauge reading G, Vacuum gauge reading V, Speed of the turbine N, Manometer readings h1, h2, Load on weight hanger T1, Spring balance reading indicating the frictional loss between the brake drum and rope T2 - Calculate the efficiency of impulse turbine/ Pelton Wheel. - Calculate the efficiency of Reaction Turbine/ Francis Turbine. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Hydraulics and Hydraulic Machinery |
| Competency Standard | Analyze the performance of Turbines |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | Calculate the performance of impulse turbine/ Pelton Wheel Calculate the performance of Reaction Turbine/ Francis Turbine |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Keep the nozzle-opening to the required position. |  |  |
| - Allow the water into the turbine and the turbine will start rotating. |  |  |
| - Fix the weigh hanger to the rope of the brake drum with no load on weight hanger. |  |  |
| - Vary the gate vale, keep the head constant using the pressure gauge to the required head in case of experiment on constant head or keep the speed constant using the tachometer to the required speed in case of experiment on constant speed. |  |  |
| - Note down the following readings of Pressure gauge reading G, Vacuum gauge reading V, Speed of the turbine N, Manometer readings h1, h2, Load on weight hanger T1, Spring balance reading indicating the frictional loss between the brake drum and rope T2 |  |  |
| - Calculate the efficiency of impulse turbine/ Pelton Wheel. |  |  |
| - Calculate the efficiency of Reaction Turbine/ Francis Turbine. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Hydraulics and Hydraulic Machinery |
| --- | --- |
| Competency Standard | Analyze the performance of Turbines |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Calculate the performance of impulse turbine/ Pelton Wheel - Calculate the performance of Reaction Turbine/ Francis Turbine |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Kept the nozzle-opening to the required position. |  |  |  |  |  |
|  | Allowed the water into the turbine and the turbine will start rotating. |  |  |  |  |  |
|  | Fixed the weigh hanger to the rope of the brake drum with no load on weight hanger. |  |  |  |  |  |
|  | Differed the gate vale, keep the head constant using the pressure gauge to the required head in case of experiment on constant head or keep the speed constant using the tachometer to the required speed in case of experiment on constant speed. |  |  |  |  |  |
|  | Noted down the following readings of Pressure gauge reading G, Vacuum gauge reading V, Speed of the turbine N, Manometer readings h1, h2, Load on weight hanger T1, Spring balance reading indicating the frictional loss between the brake drum and rope T2 |  |  |  |  |  |
|  | Calculated the efficiency of impulse turbine/ Pelton Wheel. |  |  |  |  |  |
|  | Calculated the efficiency of Reaction Turbine/ Francis Turbine. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Hydraulics and Hydraulic Machinery |
| --- | --- |
| Competency Standard | Analyze the performance of Turbines |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define working principle of Impulse Turbine |  |  |
|  | Define Pelton wheel construction. |  |  |
|  | Describe working principle of Reaction Turbine. |  |  |
|  | Describe spiral casing purpose. |  |  |
|  | Describe runner and bucket |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________