Instruction Sheet for the Candidate

| Qualification | Hydraulics and Hydraulic Machinery(Level-5) |
| --- | --- |
| Competency Standard | Analyze the performance of Pumps |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Calculate the performance of Centrifugal Pump. - Calculate the performance of Reciprocating Pump. |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Check Performance of Pumps “demonstrating the following criteria: |
| Minimum Evidence Required | - Prime the pump and Start the motor and set the vacuum gauge reading to the required head. - Note down the following readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x. - Take at least 5 sets of readings by varying the head through delivery valve and note down the readings. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Hydraulics and Hydraulic Machinery |
| Competency Standard | Analyze the performance of Pumps |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Calculate the performance of Centrifugal Pump. - Calculate the performance of Reciprocating Pump. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prime the pump and Start the motor and set the vacuum gauge reading to the required head. |  |  |
| - Note down the following readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x. |  |  |
| - Take at least 5 sets of readings by varying the head through delivery valve and note down the readings. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Hydraulics and Hydraulic Machinery |
| --- | --- |
| Competency Standard | Analyze the performance of Pumps |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Calculate the performance of Centrifugal Pump. - Calculate the performance of Reciprocating Pump. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prime the pump and Started the motor and set the vacuum gauge reading to the required head. |  |  |  |  |  |
|  | - Noted the following readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x. |  |  |  |  |  |
|  | Able to take at least 5 sets of readings by varying the head through delivery valve and note down the readings. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Hydraulics and Hydraulic Machinery |
| --- | --- |
| Competency Standard | Analyze the performance of Pumps |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe centrifugal pump working principle |  |  |
|  | Describe pump casing and Describe impeller |  |  |
|  | Describe working principle of Reaction Turbine. |  |  |
|  | Describe reciprocating pump. |  |  |
|  | Differentiate between centrifugal and reciprocating pump. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________