Instruction Sheet for the Candidate

| Qualification | (Mechanical Technology) Level 2 |
| --- | --- |
| Competency Standards | - Verify Ohm’s Law & Kirchhoff’s Law by `implementing Series & Parallel circuits - Overhaul the Lead Acid Batteries & implement their series & Parallel circuits - Measure Electrical power, Energy & Power Factor - Implement Electromagnet to observe various effects & verify Faraday’s law - Make & Solder the Joints - Carryout basic Electrical Installation - Install Single Phase Electrical Wiring - Identify Basic Electronics Components - Design a Rectifier using Diode - Design different types of Filters - Design the Voltage Multipliers (Doublers, Tripler & Quadrupler) - Carry out the implementation of Zener& Light Emitting Diode |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): Perform Electromagnetic effect as per requirement Control lamp with two way switch as per requirement. Design Voltage Regulator and measure voltage, current power Implement seven segment display |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to Perform Electromagnetic effect as per requirement Control lamp with two way switch as per requirement. Design Voltage Regulator and measure voltage, current power Implement seven segment display |
| Minimum Evidence Required | Write down all core steps to assess the above written task (yourself) - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil,bring iron nail near the iron pieces and record the observation - Take copper rod (5cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horseshoe magnet - Reduce the rheostat resistance & record the effect on copper rod. - Increase the rheostat resistance & record the effect on copper rod. - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet, mark points on paper where the compass needle stop & Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4 - Connect Galvano meter with coil,Move permanent magnet inside the coil, vary its speed and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvano meter. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvano meter. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvano meter. - Take step down transformer and connect its secondary with a Galvano meter and primary winding with a DC battery through a rheostat. - Continuously variate the rheostat and observe the reading on the Galvano meter - Measure, Connect cables, Perform & Insulate Joints - Operate multi-meter for voltage and current - Perform continuity test - Perform polarity test - Perform earthing test - Perform insulation test - Record test results - Interpret electrical drawing/ document - Identify, Select cable gauge & colors - Connect cables & Insulate Joints - Draw wiring diagram, Current path, layout of single pole switch, socket & two-way switch circuit. - Mark on working board according to layout diagram. - Install accessories, Lay wires in duct/pipe &Make connections according to wiring diagram. - Check the circuit before & after connect the main supply - Identify the Diodes, Resistors, Capacitors, Inductors & IC’s - Recognize coding, rating of resistor, capacitors, inductors - Design series & Parallel circuit of Resistor - Use formulas to calculate the resistance , capacitance , inductance in Series & parallel circuit - Design Parallel and series circuit of Capacitor - Apply the appropriate ICs Packages in circuit - Identify the required components for Rectifier circuits. - Construct circuit Diagram of half wave, Centered Tap & Full Wave Rectifier - Observe the input and Output wave form on oscilloscope - Calculate the ripple factor,output voltage using proper formulas - Draw the Choke Input/L Section Filter & capacitor Input/π Section Filter - Select the proper component for L & π type filter - Place the components of L & π type filter on bread board. - Give the pulsating DC of rectifier to the input of L & π type filter and Check the output signal on oscilloscope - Record the difference between input and output - Check the input and output voltage of the transformer. - Check required output voltage at each section of a rectifier. - Identify the faulty section and its faulty component/components. - Replace the faulty components - Check the output of repaired rectifier - Draw the diagram of voltage Doubler, Tripler & Quadruple circuit - Select required components for voltage Doubler Tripler & Quadruple circuit according to the circuit diagram - Place and interconnect the components of voltage Doubler Tripler & Quadruple on bread board as per diagram. - Give proper AC voltage at the input of voltage Doubler Tripler & Quadruple circuit form the output of stepdown transformer. - Observe input and output wave form of voltage Doubler Tripler & Quadruple circuit and record the difference between them. - Draw the circuit diagram of Zener voltage Regulator circuit. - Select, place & interconnect the required components on the breadboard according to the circuit diagram - Give DC voltage form the output of filter section of a rectifier. - Change the load or input voltage and measure the output voltage of regulator. - Record the difference between input and output - Draw the Seven Segment Display Circuit - Select, place & interconnect the required components for seven segment display - Apply the input to Seven Segment Display and Verify the required output. - Generate the output report. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Level 2 |
| Competency Standards | - Verify Ohm’s Law & Kirchhoff’s Law by `implementing Series & Parallel circuits - Overhaul the Lead Acid Batteries & implement their series & Parallel circuits - Measure Electrical power, Energy & Power Factor - Implement Electromagnet to observe various effects & verify Faraday’s law - Make & Solder the Joints - Carryout basic Electrical Installation - Install Single Phase Electrical Wiring - Identify Basic Electronics Components - Design a Rectifier using Diode - Design different types of Filters - Design the Voltage Multipliers (Doublers, Tripler & Quadrupler) - Carry out the implementation of Zener& Light Emitting Diode |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Perform Electromagnetic effect as per requirement Control lamp with two way switch as per requirement. Design Voltage Regulator and measure voltage, current power Implement seven segment display |

I can……………….

| Performance Criteria |  |  | Yes | No |
| --- | --- | --- | --- | --- |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | “Level-2” |
| --- | --- |
| Competency Standards | - Verify Ohm’s Law & Kirchhoff’s Law by `implementing Series & Parallel circuits - Overhaul the Lead Acid Batteries & implement their series & Parallel circuits - Measure Electrical power, Energy & Power Factor - Implement Electromagnet to observe various effects & verify Faraday’s law - Make & Solder the Joints - Carryout basic Electrical Installation - Install Single Phase Electrical Wiring - Identify Basic Electronics Components - Design a Rectifier using Diode - Design different types of Filters - Design the Voltage Multipliers (Doublers, Tripler & Quadrupler) Carry out the implementation of Zener& Light Emitting Diode |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Re-Write above Design Task again |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil,bring iron nail near the iron pieces and record the observation - Take copper rod (5cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horseshoe magnet - Reduce the rheostat resistance & record the effect on copper rod. - Increase the rheostat resistance & record the effect on copper rod. - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet, mark points on paper where the compass needle stop & Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4 - Connect Galvano meter with coil,Move permanent magnet inside the coil, vary its speed and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvano meter. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvano meter. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvano meter. - Take step down transformer and connect its secondary with a Galvano meter and primary winding with a DC battery through a rheostat. - Continuously variate the rheostat and observe the reading on the Galvano meter - Measure, Connect cables, Perform & Insulate Joints - Operate multi-meter for voltage and current - Perform continuity test - Perform polarity test - Perform earthing test - Perform insulation test - Record test results - Interpret electrical drawing/ document - Identify, Select cable gauge & colors - Connect cables & Insulate Joints - Draw wiring diagram, Current path, layout of single pole switch, socket & two-way switch circuit. - Mark on working board according to layout diagram. - Install accessories, Lay wires in duct/pipe &Make connections according to wiring diagram. - Check the circuit before & after connect the main supply - Identify the Diodes, Resistors, Capacitors, Inductors & IC’s - Recognize coding, rating of resistor, capacitors, inductors - Design series & Parallel circuit of Resistor - Use formulas to calculate the resistance , capacitance , inductance in Series & parallel circuit - Design Parallel and series circuit of Capacitor - Apply the appropriate ICs Packages in circuit - Identify the required components for Rectifier circuits. - Construct circuit Diagram of half wave, Centered Tap & Full Wave Rectifier - Observe the input and Output wave form on oscilloscope - Calculate the ripple factor,output voltage using proper formulas - Draw the Choke Input/L Section Filter & capacitor Input/π Section Filter - Select the proper component for L & π type filter - Place the components of L & π type filter on bread board. - Give the pulsating DC of rectifier to the input of L & π type filter and Check the output signal on oscilloscope - Record the difference between input and output - Check the input and output voltage of the transformer. - Check required output voltage at each section of a rectifier. - Identify the faulty section and its faulty component/components. - Replace the faulty components - Check the output of repaired rectifier - Draw the diagram of voltage Doubler, Tripler & Quadruple circuit - Select required components for voltage Doubler Tripler & Quadruple circuit according to the circuit diagram - Place and interconnect the components of voltage Doubler Tripler & Quadruple on bread board as per diagram. - Give proper AC voltage at the input of voltage Doubler Tripler & Quadruple circuit form the output of stepdown transformer. - Observe input and output wave form of voltage Doubler Tripler & Quadruple circuit and record the difference between them. - Draw the circuit diagram of Zener voltage Regulator circuit. - Select, place & interconnect the required components on the breadboard according to the circuit diagram - Give DC voltage form the output of filter section of a rectifier. - Change the load or input voltage and measure the output voltage of regulator. - Record the difference between input and output - Draw the Seven Segment Display Circuit - Select, place & interconnect the required components for seven segment display - Apply the input to Seven Segment Display and Verify the required output. Generate the output report. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | “Level-2” |
| --- | --- |
| Competency Standards | - Verify Ohm’s Law & Kirchhoff’s Law by `implementing Series & Parallel circuits - Overhaul the Lead Acid Batteries & implement their series & Parallel circuits - Measure Electrical power, Energy & Power Factor - Implement Electromagnet to observe various effects & verify Faraday’s law - Make & Solder the Joints - Carryout basic Electrical Installation - Install Single Phase Electrical Wiring - Identify Basic Electronics Components - Design a Rectifier using Diode - Design different types of Filters - Design the Voltage Multipliers (Doublers, Tripler & Quadrupler) - Carry out the implementation of Zener& Light Emitting Diode |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________