Instruction Sheet for the Candidate

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| --- | --- |
| Competency Standard | Perform Casting |
| Purpose of Assessment | Summative Assessment – Perform Casting |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Practice use of optical pyrometer - Practice of complete operation of cupola furnace - Practice of casting of non-ferrous metal |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to “Perform Optical Pyrometer, Operation of Cupola Furnace and Casting of non-Ferrous“ demonstrating the following criteria: |
| Minimum Evidence Required | 1- Practice use of optical pyrometer - Focus optical pyrometer on molten cast iron. - Fit red filter between eye and filament and absorption screen between objective lens and filament. - Install rheostat and measure value of current. - Note the reading of pyrometer. 2- Practice of complete operation of cupola furnace - Repair refractory lining of tap hole and slag hole of furnace. - Prepare sand be sand close bottom door. - Pour soft wood inside cupola and light it up. - Add coke from coke bed charge. - Pour iron charge plus limestone. - Fill cupola by Adding coke layer then metal layer turn by turn. - Start air blast when droplets of metal start tickling down through coke opening and from tap hole. - Stop the air and again start air blast and receive molten metal. - Open tap hole after some time to receive molten metal. - Repeat tapping after regular interval. - Stop to charge further melting is finished. - Pour metal in moulds and wait for solidification. - Break up mould and clean casting. 3-Practice of casting of non-ferrous metal - Place crucible with 10kg aluminium in crucible pit furnace. - Open gas and light spark and run blower. - Pick out crucible after proper melting. - Skim off slag and pour molten metal in the mould. - Breakup mould and clean casting after cooling of casting. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| Competency Standards | Perform Casting |
| Purpose of Assessment | Summative Assessment – Perform Casting |
| Assessment Task | - Practice use of optical pyrometer - Practice of complete operation of cupola furnace - Practice of casting of non-ferrous metal |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| 1- Practice use of optical pyrometer |  |  |
| - Focus optical pyrometer on molten cast iron. |  |  |
| - Fit red filter between eye and filament and absorption screen between objective lens and filament. |  |  |
| - Install rheostat and measure value of current. |  |  |
| - Note the reading of pyrometer. |  |  |
| 2- Practice of complete operation of cupola furnace |  |  |
| - Repair refractory lining of tap hole and slag hole of furnace. |  |  |
| - Prepare sand be sand close bottom door. |  |  |
| - Pour soft wood inside cupola and light it up. |  |  |
| - Add coke from coke bed charge. |  |  |
| - Pour iron charge plus limestone. |  |  |
| - Fill cupola by Adding coke layer then metal layer turn by turn. |  |  |
| - Start air blast when droplets of metal start tickling down through coke opening and from tap hole. |  |  |
| - Stop the air and again start air blast and receive molten metal. |  |  |
| - Open tap hole after some time to receive molten metal. |  |  |
| - Repeat tapping after regular interval. |  |  |
| - Stop to charge further melting is finished. |  |  |
| - Pour metal in moulds and wait for solidification. |  |  |
| - Break up mould and clean casting. |  |  |
| 3-Practice of casting of non-ferrous metal |  |  |
| - Place crucible with 10kg aluminium in crucible pit furnace. |  |  |
| - Open gas and light spark and run blower. |  |  |
| - Pick out crucible after proper melting. |  |  |
| - Skim off slag and pour molten metal in the mould. |  |  |
| - Breakup mould and clean casting after cooling of casting. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| --- | --- |
| Competency Standards | Perform Casting |
| Purpose of Assessment | Summative Assessment – Perform Casting |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Practice use of optical pyrometer - Practice of complete operation of cupola furnace - Practice of casting of non-ferrous metal |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| 1- Practice use of optical pyrometer |  |  |  |  |  |  |
| 1. | Focus optical pyrometer on molten cast iron. |  |  |  |  |  |
| 2. | Fit red filter between eye and filament and absorption screen between objective lens and filament. |  |  |  |  |  |
| 3. | Install rheostat and measure value of current. |  |  |  |  |  |
| 4. | Note the reading of pyrometer. |  |  |  |  |  |
| 2- Practice of complete operation of cupola furnace |  |  |  |  |  |  |
| 5. | Repair refractory lining of tap hole and slag hole of furnace. |  |  |  |  |  |
| 6. | Prepare sand be sand close bottom door. |  |  |  |  |  |
| 7. | Pour soft wood inside cupola and light it up. |  |  |  |  |  |
| 8. | Add coke from coke bed charge. |  |  |  |  |  |
| 9. | Pour iron charge plus limestone |  |  |  |  |  |
| 10. | Fill cupola by Adding coke layer then metal layer turn by turn. |  |  |  |  |  |
| 11. | Start air blast when droplets of metal start tickling down through coke opening and from tap hole. |  |  |  |  |  |
| 12. | Stop the air and again start air blast and receive molten metal. |  |  |  |  |  |
| 13. | Open tap hole after some time to receive molten metal. |  |  |  |  |  |
| 14. | Repeat tapping after regular interval. |  |  |  |  |  |
| 15. | Stop to charge further melting is finished. |  |  |  |  |  |
| 16. | Pour metal in moulds and wait for solidification. |  |  |  |  |  |
| 17. | Break up mould and clean casting. |  |  |  |  |  |
| 3-Practice of casting of non-ferrous metal |  |  |  |  |  |  |
| 18. | Place crucible with 10kg aluminium in crucible pit furnace. |  |  |  |  |  |
| 19. | Open gas and light spark and run blower. |  |  |  |  |  |
| 20. | Pick out crucible after proper melting. |  |  |  |  |  |
| 21. | Skim off slag and pour molten metal in the mould. |  |  |  |  |  |
| 22. | Breakup mould and clean casting after cooling of casting. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Level-5 Mechanical Technology (Foundry & Casting) |
| --- | --- |
| Competency Standards | Perform Casting |
| Purpose of Assessment | Summative Assessment – Perform Casting |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Question Describe Optical pyrometer and its use? |  |  |
|  | Candidate’s response |  |  |
|  | Question What are Advantages of pyrometer? |  |  |
|  | Candidate’s response |  |  |
|  | Question Describe Cupola furnace and how to prepare cupola for melting? |  |  |
|  | Candidate’s response |  |  |
|  | Question What are Charge of cupola? |  |  |
|  | Candidate’s response |  |  |
|  | Question Describe operation of Crucible furnace? |  |  |
|  | Candidate’s response |  |  |
|  | Question Elaborate properties of Aluminum and its alloys? |  |  |
|  | Candidate’s response |  |  |
| 7. | Question Describe chemical reaction in melting of aluminum melting? |  |  |
|  | Candidate’s response |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________