Instruction Sheet for the Candidate

| Qualification | Level-5 in Mechanical Technology (Metrology) |
| --- | --- |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Verify the measurements of the given object (Annexure-A) by using Graduated, Micrometer and Vernier tools. - Make the comparison of the readings taken by above mentioned tools as well. - Knowledge Assessment |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Verify the measurements of the given object (Annexure-A) by using Graduated, Micrometer and Vernier tools and make the comparison of their results as well. “ demonstrating the following criteria: |
| Minimum Evidence Required | Graduated tools - Clean the tools and surface of work piece. - Place the rulers including steel, Hook and Folding ruler properly on the designated work pieces. - Take the required dimensions of work piece and note them separately on the data table. Micrometer - To measure the external dimensions by using Outside Micrometer, open the gauge slightly larger than the part to be measured 5. Set anvil squarely against reference surface of part 6. Using ratchet, slowly until it clicks once. 7. Record readings in the data table. 8. To measure the internal dimensions by using Inside Micrometer, open the gauge slightly smaller than the part to be measured 9. Set anvil squarely against reference surface of part with ratchet, slowly until it clicks once. 10. Record readings in the data table. 11. To measure the depth of the holes or slot by using Depth Micrometer, access the depth of groove by steel rule 12. Insert the suitable insertion rod. 13. Place the depth micrometer gauge on the deeper groove and take reading. 14. Record readings in the data table. Vernier tools 15. Place the work piece on surface plate. 16. Open the lock screws. 17. Slide the jaws up to the width or size of jaws. 18. Open slightly larger than the part to be measured. 19. Set anvil squarely against reference surface of part 20. Maintain the proper pressure on the jaws with screw. 21. Lock the movable jaw with knurled screw 22. Measure and record the dimensions. 23. Hold the work piece on angle plate. 24. Open the lock screws. 25. Set the height gauge on stud to check it with fine adjusting screw. 26. Lock the measuring head. 27. Record the reading. 28. Hold the work piece on sample plate. 29. Open the lock screws. 30. Insert the gauge inside the work piece with fine adjusting screw. 31. Lock the measuring head. 32. Record the reading 33. In the end verify the measurements taken by measuring tools against the given object and also compare the results taken by the measuring tools with each other as well. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Level-5 in Mechanical Technology (Metrology) |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Verify the measurements of the given object (Annexure-A) by using Graduated, Micrometer and Vernier tools and make the comparison of their results as well.. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Clean the tools and surface of work piece. |  |  |
| - Place the rulers including steel, Hook and Folding ruler properly on the designated work pieces. |  |  |
| - Take the required dimensions of work piece and note them separately on the data table. |  |  |
| - To measure the external dimensions by using Outside Micrometer, open the gauge slightly larger than the part to be measured. |  |  |
| - Set anvil squarely against reference surface of part. |  |  |
| - Using ratchet, slowly until it clicks once. |  |  |
| - Record readings in the data table. |  |  |
| - To measure the internal dimensions by using Inside Micrometer, open the gauge slightly smaller than the part to be measured. |  |  |
| - Set anvil squarely against reference surface of part with ratchet, slowly until it clicks once. |  |  |
| - Record readings in the data table. |  |  |
| - To measure the depth of the holes or slot by using Depth Micrometer, access the depth of groove by steel rule. |  |  |
| - Insert the suitable insertion rod. |  |  |
| - Place the depth micrometer gauge on the deeper groove and take reading. |  |  |
| - Record readings in the data table. |  |  |
| - Place the work piece on surface plate. |  |  |
| - Open the lock screws. |  |  |
| - Slide the jaws up to the width or size of jaws. |  |  |
| - Open slightly larger than the part to be measured. |  |  |
| - Set anvil squarely against reference surface of part |  |  |
| - Maintain the proper pressure on the jaws with screw. |  |  |
| - Lock the movable jaw with knurled screw |  |  |
| - Measure and record the dimensions. |  |  |
| - Hold the work piece on angle plate. |  |  |
| - Open the lock screws. |  |  |
| - Set the height gauge on stud to check it with fine adjusting screw. |  |  |
| - Lock the measuring head. |  |  |
| - Record the reading. |  |  |
| - Hold the work piece on sample plate. |  |  |
| - Open the lock screws. |  |  |
| - Insert the gauge inside the work piece with fine adjusting screw. |  |  |
| - Lock the measuring head. |  |  |
| - Record the reading |  |  |
| - In the end verify the measurements taken by measuring tools against the given object and also compare the results taken by the measuring tools with each other as well. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Level-5 in Mechanical Technology (Metrology) |
| --- | --- |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Verify the measurements of the given object (Annexure-A) by using Graduated, Micrometer and Vernier tools and make the comparison of their results as well.. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Clean the tools and surface of work piece. |  |  |  |  |  |
|  | Place the rulers including steel, Hook and Folding ruler properly on the designated work pieces. |  |  |  |  |  |
|  | Take the required dimensions of work piece and note them separately on the data table. |  |  |  |  |  |
|  | To measure the external dimensions by using Outside Micrometer, open the gauge slightly larger than the part to be measured. |  |  |  |  |  |
|  | Set anvil squarely against reference surface of part. |  |  |  |  |  |
|  | Using ratchet, slowly until it clicks once. |  |  |  |  |  |
|  | Record readings in the data table. |  |  |  |  |  |
|  | To measure the internal dimensions by using Inside Micrometer, open the gauge slightly smaller than the part to be measured. |  |  |  |  |  |
|  | Set anvil squarely against reference surface of part with ratchet, slowly until it clicks once. |  |  |  |  |  |
|  | Record readings in the data table. |  |  |  |  |  |
|  | To measure the depth of the holes or slot by using Depth Micrometer, access the depth of groove by steel rule. |  |  |  |  |  |
|  | Insert the suitable insertion rod. |  |  |  |  |  |
|  | Place the depth micrometer gauge on the deeper groove and take reading. |  |  |  |  |  |
|  | Record readings in the data table. |  |  |  |  |  |
|  | Place the work piece on surface plate. |  |  |  |  |  |
|  | Open the lock screws. |  |  |  |  |  |
|  | Slide the jaws up to the width or size of jaws. |  |  |  |  |  |
|  | Open slightly larger than the part to be measured. |  |  |  |  |  |
|  | Set anvil squarely against reference surface of part |  |  |  |  |  |
|  | Maintain the proper pressure on the jaws with screw. |  |  |  |  |  |
|  | Lock the movable jaw with knurled screw |  |  |  |  |  |
|  | Measure and record the dimensions. |  |  |  |  |  |
|  | Hold the work piece on angle plate. |  |  |  |  |  |
|  | Open the lock screws. |  |  |  |  |  |
|  | Set the height gauge on stud to check it with fine adjusting screw. |  |  |  |  |  |
|  | Lock the measuring head. |  |  |  |  |  |
|  | Record the reading. |  |  |  |  |  |
|  | Hold the work piece on sample plate. |  |  |  |  |  |
|  | Open the lock screws. |  |  |  |  |  |
|  | Insert the gauge inside the work piece with fine adjusting screw. |  |  |  |  |  |
|  | Lock the measuring head. |  |  |  |  |  |
|  | Record the reading |  |  |  |  |  |
|  | In the end verify the measurements taken by measuring tools with each other as well. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Level-5 in Mechanical Technology (Metrology) |
| --- | --- |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Name the two systems of measurement? |  |  |
|  | 1 inch = ?m |  |  |
|  | Why micrometer is better than Vernier Caliper? |  |  |
|  | How many types of errors can be encountered during the use of micrometers? |  |  |
|  | What is the formula to calculate the Least count of Micrometer? |  |  |
|  | Name the different types of Vernier tools? |  |  |
|  | What is the Principle of Vernier Calipers? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________