Instruction Sheet for the Candidate

| Qualification | Hydraulics and Hydraulic Machinery (Level-5) |
| --- | --- |
| Competency Standard | - Verify Bernoulli’s Equation. - Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking. |
| Purpose of Assessment | Summative Assessment (Level-5) |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Measurement of Velocity Head, Pressure Head and Datum Head. - Check the performance of friction pipe apparatus. - Examine loss of head due to sudden enlargement, Contraction and entrance in a pipe. - Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch. - Construct hydraulic circuit using Accumulator stored Energy by DAC - Measure pressure at various connections in hydraulic circuits. |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Verify Bernoulli’s Equation and Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking. “demonstrating the following criteria: |
| Minimum Evidence Required | Verify Bernoulli’s Equation. - Power on the hydraulic bench and Open inlet valve to allow the water to flow out of the tank. - Adjust the valves to obtain continuous flow at inlet and outlet valve. - Ensure continuous pressure head in the piezometer tube and note the reading. - Note the length and diameter of the pipe present between hydraulic bench and pressure tank. - Calculate the velocity head and pressure head by changing input and output supply and note the readings. - Calculate coefficient of discharge. - Determine the total discharge in the circuit with the help of left and right limbs of U-tube manometer. Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking. - Operate hydraulic circuit. - Drop the pressure using pressure gauge before the piston starts moving. - Construct Hydraulic Circuit Diagram. - Construct electric circuit diagram. - Press the push button to start fluid supply and will enable the cylinder for forward stroke supply. - Press the second push button to complete backward stroke of piston, this will enable the supply of side coil and the backward stroke of piston would be completed and as it enters the surroundings of limit switches the supply would be disconnected. - Change 5/2 DCV and complete the opposite circuit. - When the circuit is burnt/completed the turn off the power supply. - install pressure gauges on the points where pressure is to be noted. - Ensure fluid flow through the whole circuit. - Complete the circuit and note the readings of all gauges. Enter all readings in observation table. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Hydraulics and Hydraulic Machinery |
| Competency Standard | - Verify Bernoulli’s Equation. - Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking. |
| Purpose of Assessment | Summative Assessment (Level-5) |
| Assessment Task | - Measurement of Velocity Head, Pressure Head and Datum Head. - Check the performance of friction pipe apparatus. - Examine loss of head due to sudden enlargement, Contraction and entrance in a pipe. - Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch. - Construct hydraulic circuit using Accumulator stored Energy by DAC - Measure pressure at various connections in hydraulic circuits. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Power on the hydraulic bench and Open inlet valve to allow the water to flow out of the tank. |  |  |
| - Adjust the valves to obtain continuous flow at inlet and outlet valve. |  |  |
| - Ensure continuous pressure head in the piezometer tube and note the reading. |  |  |
| - Note the length and diameter of the pipe present between hydraulic bench and pressure tank. |  |  |
| - Calculate the velocity head and pressure head by changing input and output supply and note the readings. |  |  |
| - Calculate coefficient of discharge. |  |  |
| - Determine the total discharge in the circuit with the help of left and right limbs of U-tube manometer |  |  |
| - Operate hydraulic circuit. |  |  |
| - Drop the pressure using pressure gauge before the piston starts moving. |  |  |
| - Construct Hydraulic Circuit Diagram. |  |  |
| - Construct electric circuit diagram. |  |  |
| - Press the push button to start fluid supply and will enable the cylinder for forward stroke supply. |  |  |
| - Press the second push button to complete backward stroke of piston, this will enable the supply of side coil and the backward stroke of piston would be completed and as it enters the surroundings of limit switches the supply would be disconnected. |  |  |
| - Change 5/2 DCV and complete the opposite circuit. |  |  |
| - When the circuit is burnt/completed the turn off the power supply. |  |  |
| - install pressure gauges on the points where pressure is to be noted. |  |  |
| - Ensure fluid flow through the whole circuit. |  |  |
| - Complete the circuit and note the readings of all gauges. |  |  |
| - Enter all readings in observation table. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Hydraulics and Hydraulic Machinery |
| --- | --- |
| Competency Standard | - Verify Bernoulli’s Equation. - Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking. |
| Purpose of Assessment | Summative Assessment (Level-5) |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Measurement of Velocity Head, Pressure Head and Datum Head. - Check the performance of friction pipe apparatus. - Examine loss of head due to sudden enlargement, Contraction and entrance in a pipe. - Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch. - Construct hydraulic circuit using Accumulator stored Energy by DAC - Measure pressure at various connections in hydraulic circuits. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Powered on the hydraulic bench and Open inlet valve to allow the water to flow out of the tank. |  |  |  |  |  |
|  | Adjusted the valves to obtain continuous flow at inlet and outlet valve. |  |  |  |  |  |
|  | Ensured continuous pressure head in the piezometer tube and note the reading. |  |  |  |  |  |
|  | Noted the length and diameter of the pipe present between hydraulic bench and pressure tank. |  |  |  |  |  |
|  | Calculated the velocity head and pressure head by changing input and output supply and note the readings. |  |  |  |  |  |
|  | Calculated coefficient of discharge. |  |  |  |  |  |
|  | Determined the total discharge in the circuit with the help of left and right limbs of U-tube manometer. |  |  |  |  |  |
|  | Operated hydraulic circuit. |  |  |  |  |  |
|  | Dropped the pressure using pressure gauge before the piston starts moving. |  |  |  |  |  |
|  | Constructed Hydraulic Circuit Diagram. |  |  |  |  |  |
|  | Constructed electric circuit diagram. |  |  |  |  |  |
|  | Pressed the push button to start fluid supply and will enable the cylinder for forward stroke supply. |  |  |  |  |  |
|  | Pressed the second push button to complete backward stroke of piston, this will enable the supply of side coil and the backward stroke of piston would be completed and as it enters the surroundings of limit switches the supply would be disconnected. |  |  |  |  |  |
|  | Changed 5/2 DCV and complete the opposite circuit. |  |  |  |  |  |
|  | When the circuit is burnt/completed the turned off the power supply. |  |  |  |  |  |
|  | installed pressure gauges on the points where pressure is to be noted. |  |  |  |  |  |
|  | Ensured fluid flow through the whole circuit. |  |  |  |  |  |
|  | Completed the circuit and note the readings of all gauges. |  |  |  |  |  |
|  | Entered all readings in observation table. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Hydraulics and Hydraulic Machinery |
| --- | --- |
| Competency Standard | - Verify Bernoulli’s Equation. - Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking. |
| Purpose of Assessment | Summative Assessment (Level-5) |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define velocity head |  |  |
|  | Define pressure head |  |  |
|  | Describe co-efficient of loss |  |  |
|  | Describe effects of friction on fluid flow |  |  |
|  | Describe loss of head |  |  |
|  | Describe the function of hydraulic circuit. |  |  |
|  | Describe 5/2 double acting side pilot operated DCV. |  |  |
|  | Describe proximity switch or limit switch |  |  |
|  | Describe double acting hydraulic cylinder. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________