Instruction Sheet for the Candidate

| Qualification | Advance Welding(Mechanical Technology)Level 5 part I |
| --- | --- |
| Competency Standards | - Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions - Flux Cored Arc Welding (FCAW) - Perform Submerged Arc Welding - Perform Forged Welding - Perform Friction Welding - Non-Destructive Welding Tests - Destructive Welding Tests |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): Make overhead (4F) and overhead (4G) Welds on Carbon Steel Plate by using GTAW. |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to Make overhead (4F) and overhead (4G) Welds on Carbon Steel Plate by using GTAW. |
| Minimum Evidence Required | Write down all core steps to assess the above written task (yourself) - Identify welding requirements from the job, welding procedure specifications and/or technical drawings - Prepare GTAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Overhead (4F) and Overhead (4G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Advance Welding(Mechanical Technology)Level 5 part I |
| Competency Standards | - Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions - Flux Cored Arc Welding (FCAW) - Perform Submerged Arc Welding - Perform Forged Welding - Perform Friction Welding - Non-Destructive Welding Tests - Destructive Welding Tests |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Make overhead (4F) and overhead (4G) Welds on Carbon Steel Plate by using GTAW. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify welding requirements from the job, welding procedure specifications and/or technical drawings |  |  |
| - Prepare GTAW welding machine in accordance with welding procedure specifications/ manufacturer instructions |  |  |
| - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions |  |  |
| - Connect welding machine to an independent power supply |  |  |
| - Set polarity indicated in the welding procedure specifications |  |  |
| - Maintain gap between electrode and base metal as per standard practices |  |  |
| - Carry out welding in Overhead (4F) and Overhead (4G) positions following standard procedures |  |  |
| - Deposit root pass as per welding procedure specifications/job requirements |  |  |
| - Deposit filling passes as per welding procedure specifications/job requirements |  |  |
| - Deposit capping pass as per welding procedure specifications/job requirements |  |  |
| - Check root, filling and capping passes for any visual discontinuities as per acceptance standards |  |  |
| - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects |  |  |
| - Carry out finishing work of welds following standard procedures |  |  |
| - Inspect weld visually and mark any visual defects, as required |  |  |
| - Clean work area in accordance with workplace safety practices |  |  |
| - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Advance Welding(Mechanical Technology)Level 5 part I |
| --- | --- |
| Competency Standards | - Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions - Flux Cored Arc Welding (FCAW) - Perform Submerged Arc Welding - Perform Forged Welding - Perform Friction Welding - Non-Destructive Welding Tests - Destructive Welding Tests |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Make overhead (4F) and overhead (4G) Welds on Carbon Steel Plate by using GTAW. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identify welding requirements from the job, welding procedure specifications and/or technical drawings |  |  |  |  |  |
|  | Prepare GTAW welding machine in accordance with welding procedure specifications/ manufacturer instructions |  |  |  |  |  |
|  | Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions |  |  |  |  |  |
|  | Connect welding machine to an independent power supply |  |  |  |  |  |
|  | Set polarity indicated in the welding procedure specifications |  |  |  |  |  |
|  | Maintain gap between electrode and base metal as per standard practices |  |  |  |  |  |
|  | Carry out welding in Overhead (4F) and Overhead (4G) positions following standard procedures |  |  |  |  |  |
|  | Deposit root pass as per welding procedure specifications/job requirements |  |  |  |  |  |
|  | Deposit filling passes as per welding procedure specifications/job requirements |  |  |  |  |  |
|  | Deposit capping pass as per welding procedure specifications/job requirements |  |  |  |  |  |
|  | Check root, filling and capping passes for any visual discontinuities as per acceptance standards |  |  |  |  |  |
|  | Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects |  |  |  |  |  |
|  | Carry out finishing work of welds following standard procedures |  |  |  |  |  |
|  | Inspect weld visually and mark any visual defects, as required |  |  |  |  |  |
|  | Clean work area in accordance with workplace safety practices |  |  |  |  |  |
|  | Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Advance Welding(Mechanical Technology)Level 5 part I |
| --- | --- |
| Competency Standards | - Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions - Flux Cored Arc Welding (FCAW) - Perform Submerged Arc Welding - Perform Forged Welding - Perform Friction Welding - Non-Destructive Welding Tests - Destructive Welding Tests |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain various welding positions |  |  |
|  | State the purpose of using shielding gas in TIG welding |  |  |
|  | Describe the principle of FCAW welding. |  |  |
|  | Describe the features of equipment used in SAW. |  |  |
|  | What are common defects of welding joints? |  |  |
|  | Describe the principle of forged weld. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________