Instruction Sheet for the Candidate

| Qualification | Design of Mechanical Members(Mechanical Technology)Level 5 |
| --- | --- |
| Competency Standards | - Establish and maintain the Occupational safety and health safety - Calculate Stresses in Machine Parts - Calculator diameter of cylinder for hoop and longitudinal stresses - Calculate thickness and diameter of spherical shell for circumferential stresses - Design welded joints for transverse and parallel fillet under static and fatigue loading - Calculate stresses due to initial tightening and external load on screws - Design dimension of square and rectangular keys - Design hollow and solid shaft subjected to twisting moment only - Design flange coupling for specific torque - Design the dimensions of helical spring - Design bearings, speed, lubrication, pressure, viscosity and coefficient of friction - Design and draw the CAM profile |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): Task 1: - Design a pressure vessel so that its circumferential stress and longitudinal stresses must not exceed 25Mpa and 38 MPa. The Pressure vessel is 1.5m in diameter and generates steam at pressure of 2 N/mm2. (Annexure A). - Develop Cam Profile: Outstroke 60 degree, Dwell 45 degree and Return stroke 90 degree remaining 165 degrees are dwell. Follower Stroke 40 mm and base circle radius 50 mm. (Annexure D) - Use various loadings on a helical spring and measure its deflections Task 2: - Assemble a shaft, key and flange coupling as shown in (Annexure B) also find the size of the bolt for following design situation: A 30 Nm torque transmission flange coupling is used. Four bolts at a radius 35 mm would be used to couple the flanges, find the size of the bolt if allowable shear stress must not exceed 28 MPa. - Use (Annexure C) to identify more suitable bearing in given loading conditions. Name all the bearing shown in (Annexure C) - Perform Transverse Fillet Weld and test the weld under Fatigue Loading (Annexure E) |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Task 1: - Write the data clearly - Mention the Hoop stress and Longitudinal Stress using appropriate Formulae - Find the wall thickness using appropriate formulae - Collect data - Draw Velocity Profile - Develop CAM Profile - Identify suitable helical Spring - Attach it to the hanger - Apply various loads and write down deflections Task 2: - Find the outer diameter of the shaft - Find the inner diameter of the flange coupling - Position the key in keyway - Assemble the shaft, key and flange coupling using appropriate tools - Identify the data and clearly write it down. - Use standard formulae of torque to calculate force - Find the total load on bolts - Find the diameter of the bolt - See the Annexure and identify Bearing types - Confirm the loadings on the bearings - Identify suitable bearing in this situation - Select Electrode, weld machine as per material requirement - Perform transverse fillet Weld as per Annexure E - Observe parallel fillet weld under fatigue loadings |

Annexure A

Pressure Vessel Design

Annexure B

Shaft, Flange Coupling and Key

Annexure C

Bearing Types

Annexure D

Knife edge Cam and Follower (degrees and millimeter)

ANNEXURE E

Transverse Welding Joint

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Design of Mechanical Members(Mechanical Technology)Level 5 |
| Competency Standards | - Establish and maintain the Occupational safety and health safety - Calculate Stresses in Machine Parts - Calculator diameter of cylinder for hoop and longitudinal stresses - Calculate thickness and diameter of spherical shell for circumferential stresses - Design welded joints for transverse and parallel fillet under static and fatigue loading - Calculate stresses due to initial tightening and external load on screws - Design dimension of square and rectangular keys - Design hollow and solid shaft subjected to twisting moment only - Design flange coupling for specific torque - Design the dimensions of helical spring - Design bearings, speed, lubrication, pressure, viscosity and coefficient of friction - Design and draw the CAM profile |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): Task 1: - Design a pressure vessel so that its circumferential stress and longitudinal stresses must not exceed 25Mpa and 38 MPa. The Pressure vessel is 1.5m in diameter and generates steam at pressure of 2 N/mm2. (Annexure A). - Develop Cam Profile: Outstroke 60 degree, Dwell 45 degree and Return stroke 90 degree remaining 165 degrees are dwell. Follower Stroke 40 mm and base circle radius 50 mm. (Annexure D) - Use various loadings on a helical spring and measure its deflections Task 2: - Assemble a shaft, key and flange coupling as shown in (Annexure B) also find the size of the bolt for following design situation: A 30 Nm torque transmission flange coupling is used. Four bolts at a radius 35 mm would be used to couple the flanges, find the size of the bolt if allowable shear stress must not exceed 28 MPa. - Use (Annexure C) to identify more suitable bearing in given loading conditions. Name all the bearing shown in (Annexure C) - Perform Transverse Fillet Weld and test the weld under Fatigue Loading (Annexure E) |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Write the data clearly |  |  |
| - Mention the Hoop stress and Longitudinal Stress |  |  |
| - Find the wall thickness using appropriate formulae |  |  |
| - Collect data |  |  |
| - Draw Velocity Profile |  |  |
| - Develop CAM Profile |  |  |
| - Identify suitable helical Spring |  |  |
| - Attach it to the hanger |  |  |
| - Apply various loads and write down deflections |  |  |
| - Find the outer diameter of the shaft |  |  |
| - Find the inner diameter of the flange coupling |  |  |
| - Position the key in keyway |  |  |
| - Assemble the shaft, key and flange coupling using appropriate tools |  |  |
| - Identify the data and clearly write it down. |  |  |
| - Use standard formulae of torque to calculate force |  |  |
| - Find the total load on bolts |  |  |
| - Find the diameter of the bolt |  |  |
| - See the Annexure and identify Bearing types |  |  |
| - Confirm the loadings on the bearings |  |  |
| - Identify suitable bearing in this situation |  |  |
| - Select Electrode, weld machine as per material requirement |  |  |
| - Attach it to the hanger |  |  |
| - Apply various loads and write down deflections |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Design of Mechanical Members(Mechanical Technology)Level 5 |
| --- | --- |
| Competency Standards | - Establish and maintain the Occupational safety and health safety - Calculate Stresses in Machine Parts - Calculator diameter of cylinder for hoop and longitudinal stresses - Calculate thickness and diameter of spherical shell for circumferential stresses - Design welded joints for transverse and parallel fillet under static and fatigue loading - Calculate stresses due to initial tightening and external load on screws - Design dimension of square and rectangular keys - Design hollow and solid shaft subjected to twisting moment only - Design flange coupling for specific torque - Design the dimensions of helical spring - Design bearings, speed, lubrication, pressure, viscosity and coefficient of friction - Design and draw the CAM profile |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): Task 1: - Design a pressure vessel so that its circumferential stress and longitudinal stresses must not exceed 25Mpa and 38 MPa. The Pressure vessel is 1.5m in diameter and generates steam at pressure of 2 N/mm2. (Annexure A). - Develop Cam Profile: Outstroke 60 degree, Dwell 45 degree and Return stroke 90 degree remaining 165 degrees are dwell. Follower Stroke 40 mm and base circle radius 50 mm. (Annexure D) - Use various loadings on a helical spring and measure its deflections Task 2: - Assemble a shaft, key and flange coupling as shown in (Annexure B) also find the size of the bolt for following design situation: A 30 Nm torque transmission flange coupling is used. Four bolts at a radius 35 mm would be used to couple the flanges, find the size of the bolt if allowable shear stress must not exceed 28 MPa. - Use (Annexure C) to identify more suitable bearing in given loading conditions. Name all the bearing shown in (Annexure C) - Perform Transverse Fillet Weld and test the weld under Fatigue Loading (Annexure E) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Write the data clearly |  |  |  |  |  |
|  | Mention the Hoop stress and Longitudinal Stress |  |  |  |  |  |
|  | Find the wall thickness using appropriate formulae |  |  |  |  |  |
|  | Collect data |  |  |  |  |  |
|  | Draw Velocity Profile |  |  |  |  |  |
|  | Develop CAM Profile |  |  |  |  |  |
|  | Identify suitable helical Spring |  |  |  |  |  |
|  | Attach it to the hanger |  |  |  |  |  |
|  | Apply various loads and write down deflections |  |  |  |  |  |
|  | Find the outer diameter of the shaft |  |  |  |  |  |
|  | Find the inner diameter of the flange coupling |  |  |  |  |  |
|  | Position the key in keyway |  |  |  |  |  |
|  | Assemble the shaft, key and flange coupling using appropriate tools |  |  |  |  |  |
|  | Identify the data and clearly write it down. |  |  |  |  |  |
|  | Use standard formulae of torque to calculate force |  |  |  |  |  |
|  | Find the total load on bolts |  |  |  |  |  |
|  | Find the diameter of the bolt |  |  |  |  |  |
|  | See the Annexure and identify Bearing types |  |  |  |  |  |
|  | Confirm the loadings on the bearings |  |  |  |  |  |
|  | Identify suitable bearing in this situation |  |  |  |  |  |
|  | Select Electrode, weld machine as per material requirement |  |  |  |  |  |
|  | Perform transverse fillet Weld as per Annexure E |  |  |  |  |  |
|  | Observe parallel fillet weld under fatigue loadings |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Design of Mechanical Members(Mechanical Technology)Level 5 |
| --- | --- |
| Competency Standards | - Establish and maintain the Occupational safety and health safety - Calculate Stresses in Machine Parts - Calculator diameter of cylinder for hoop and longitudinal stresses - Calculate thickness and diameter of spherical shell for circumferential stresses - Design welded joints for transverse and parallel fillet under static and fatigue loading - Calculate stresses due to initial tightening and external load on screws - Design dimension of square and rectangular keys - Design hollow and solid shaft subjected to twisting moment only - Design flange coupling for specific torque - Design the dimensions of helical spring - Design bearings, speed, lubrication, pressure, viscosity and coefficient of friction - Design and draw the CAM profile |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Mention Lame’s Equation |  |  |
|  | Differentiate between tensile, compressive and shear stress |  |  |
|  | Explain Transverse stress |  |  |
|  | Mention various types of welding |  |  |
|  | Mention types of fittings for a shaft and bearing |  |  |
|  | Mention bearing types and their applications |  |  |
|  | Explain CAM movement concentric and eccentric |  |  |
|  | Why do we develop velocity profile for a CAM movement |  |  |
|  | Mention types of couplings |  |  |
|  | Differentiate between static and dynamics load |  |  |
|  | Differentiate between tension spring and compression spring |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________