Instruction Sheet for the Candidate

| Qualification | Advance machining (Mechanical Technology) Level 5 |
| --- | --- |
| Competency Standards | - Perform Milling Operations - Generate Gears on Hobbing Machine - Perform broaching operation - Perform CNC Lathe Operations - Perform CNC Milling Operations - Perform CNC EDM Wire-Cut Operations - Perform advance mechanical machining processes - Perform Laser Beam Machining |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): Make a spur gear according to the requirement |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to Make a spur gear according to the requirement |
| Minimum Evidence Required | - Interpret drawing and arrange the material according to job requirement - Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank. - Generate the program for turning and drilling of the Blank on CNC Lathe machine to make the Outer diameter and bore according to the size. - Feed the program, run simulation if there is any error debug it, and run the machine to make the job according to requirement. - Clamped the blank on CNC milling machine after turning and boring operation. - Generate the program for CNC milling machine to make the required teeth on blank. - Feed the program to the CNC milling machine, run simulation if there is any error debug it, and perform milling operation to make the gear. - Perform the broaching operation to make internal key ways. - Generate Program For CNC EDM wire to make the keyways on the bore. - Feed the program to the CNC EDM, run simulation if there is any error debug it, and perform cutting operation to make internal slot on Gear. - Generate Program For CNC Laser/Water jet to make holes on Gear according to Custom requirements. - Feed the program to the CNC Laser/Water jet run simulation if there is any error debug it, and perform cutting operation to make it as per requirements. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Advance machining (Mechanical Technology)Level 5 |
| Competency Standards | - Perform Milling Operations - Generate Gears on Hobbing Machine - Perform broaching operation - Perform CNC Lathe Operations - Perform CNC Milling Operations - Perform CNC EDM Wire-Cut Operations - Perform advance mechanical machining processes - Perform Laser Beam Machining |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Re-Write above Design Task again |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Interpret drawing and arrange the material according to job requirement |  |  |
| - Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank. |  |  |
| - Generate the program for turning and drilling of the Blank on CNC Lathe machine to make the Outer diameter and bore according to the size. |  |  |
| - Feed the program, run simulation if there is any error debug it, and run the machine to make the job according to requirement. |  |  |
| - Clamped the blank on CNC milling machine after turning and boring operation. |  |  |
| - Generate the program for CNC milling machine to make the required teeth on blank. |  |  |
| - Feed the program to the CNC milling machine, run simulation if there is any error debug it, and perform milling operation to make the gear. |  |  |
| - Perform the broaching operation to make internal key ways. |  |  |
| - Generate Program For CNC EDM wire to make the keyways on the bore. |  |  |
| - Feed the program to the CNC EDM, run simulation if there is any error debug it, and perform cutting operation to make internal slot on Gear. |  |  |
| - Generate Program For CNC Laser/Water jet to make holes on Gear according to Custom requirements. |  |  |
| - Feed the program to the CNC Laser/Water jet run simulation if there is any error debug it, and perform cutting operation to make it as per requirements. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Advance machining (Mechanical Technology)Level 5 |
| --- | --- |
| Competency Standards | - Perform Milling Operations - Generate Gears on Hobbing Machine - Perform broaching operation - Perform CNC Lathe Operations - Perform CNC Milling Operations - Perform CNC EDM Wire-Cut Operations - Perform advance mechanical machining processes - Perform Laser Beam Machining |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Re-Write above Design Task again |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Interpret drawing and arrange the material according to job requirement |  |  |  |  |  |
|  | Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank. |  |  |  |  |  |
|  | Generate the program for turning and drilling of the Blank on CNC Lathe machine to make the Outer diameter and bore according to the size. |  |  |  |  |  |
|  | Feed the program, run simulation if there is any error debug it, and run the machine to make the job according to requirement. |  |  |  |  |  |
|  | Clamped the blank on CNC milling machine after turning and boring operation. |  |  |  |  |  |
|  | Generate the program for CNC milling machine to make the required teeth on blank. |  |  |  |  |  |
|  | Feed the program to the CNC milling machine, run simulation if there is any error debug it, and perform milling operation to make the gear. |  |  |  |  |  |
|  | Perform the broaching operation to make internal key ways. |  |  |  |  |  |
|  | Generate Program For CNC EDM wire to make the keyways on the bore. |  |  |  |  |  |
|  | Feed the program to the CNC EDM, run simulation if there is any error debug it, and perform cutting operation to make internal slot on Gear. |  |  |  |  |  |
|  | Generate Program For CNC Laser/Water jet to make holes on Gear according to Custom requirements. |  |  |  |  |  |
|  | Feed the program to the CNC Laser/Water jet run simulation if there is any error debug it, and perform cutting operation to make it as per requirements. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Advance machining (Mechanical Technology)Level 5 |
| --- | --- |
| Competency Standards | - Perform Milling Operations - Generate Gears on Hobbing Machine - Perform broaching operation - Perform CNC Lathe Operations - Perform CNC Milling Operations - Perform CNC EDM Wire-Cut Operations - Perform advance mechanical machining processes - Perform Laser Beam Machining |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe Milling Operations |  |  |
|  | Describe Types of CNC machines . |  |  |
|  | Describe Types of Indexing |  |  |
|  | Describe Hobbing Machine |  |  |
|  | Describe Broaching Process. |  |  |
|  | Describe CNC Lathe machine operations |  |  |
|  | Describe G codes and M codes |  |  |
|  | Describe Mechanism of working of CNC Milling Machine |  |  |
|  | Enlist Different CAM Software and format. |  |  |
|  | Describe working Mechanism of CNC EDM wire cut machine |  |  |
|  | Describe USM process for machining |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________