Level-3 Equivalent to G-II

Curriculum of National Certificate Level - 3 in Mechanical Technology

Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

1.	Introduction	5

2.	Purpose of the Qualification:	5

3.	Overall objectives of training program	5

4.	Entry level of trainees	6

5.	Minimum qualification for teachers	6

6.	Duration of the course:	6

7.	Description and structure of the course	7

8.	Overview of the Curriculum.	11

9.	Scheme of Studies	11

10.	Detail of Modules	12

(0715-M&MT-2) National Qualification Certificate of Level 3 in Mechanical Technology	12

0715.6	Basic Welding	12

0715.6.1.	Interpret Drawing and Execute to Fabricate Parts	12

0715.6.2.	Evaluate the Use of Hand and Power Tools	14

0715.6.3.	Perform Pre-Welding Operations	17

0715.6.4.	Perform Oxy Acetylene Welding	20

0715.6.5.	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)	22

0715.6.6.	Perform Gas Welding on Mild Steel Plates (1F, 2F, 3F)	25

0715.6.7.	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), Hori (2F, 2G), Ver (3F, 3G) and O. head (4F, 4G) Positions	27

0715.6.8.	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions	33

0715.6.9.	Perform Soldering and Brazing Operations	39

0715.7	Foundry & Casting	41

0715.7.1.	Preform Pattern Making	41

0715.7.2.	Perform Molding	46

0715.7.3.	Perform Core Making	50

0715.7.4.	Perform Foundry Work	52

0715.7.5.	Perform casting	63

0715.7.6.	Perform Cleaning and Finishing of Casting	66

0715.7.7.	Perform Sand Testing	68

0715.8	Metrology	71

0715.8.1.	Take measurements with graduated tools	71

0715.8.2.	Take measurements with combination set	73

0715.8.3.	Perform Levelling	76

0715.8.4.	Take measurements through various gauges	78

0715.8.5.	Perform measurements through Micrometer	80

0715.8.6.	Measure dimensions with Vernier tools	82

0715.8.7.	Measure angles with angle measuring Instruments.	84

0715.8.8.	Perform different measurements	86

0715.8.9.	Measure Threads and Gears	90

0715.8.10.	Compare measurement with Comparators	91

0715.9	Plastic machine processes	92

0715.9.1.	Operate Injection Molding Machine	92

0715.9.2.	Operate Blow Molding Machine	96

0715.9.3.	Operate Plastic Film Extrusion machines (Blown/Cast)	101

0715.9.4.	Operate Pipe & Profile Extrusion Machines	105

0715.9.5.	Operate Rotational Molding Machines	108

0715.9.6.	Operate Foam Processing Machines	111

0715.9.7.	Operate Compression/Transfer Molding Machines	114

0715.9.8.	Operate Multi-Color Printing Machines (Rotogravure)	117

0715.9.9.	Operate Multi-Color Printing Machines (Flexographic)	120

0715.10	Communication Skills	123

0715.10.1.	Perform Basic Communication	123

0715.10.2.	Communicate at Workplace	125

0715.10.3.	Perform Advance Communication	128

0715.10.4.	Perform Managerial Communication	130

11.	List of Machinery, Tools & Equipment.	133

2.	Members of the Curriculum Development Committee.	138

# Introduction

Mechanical engineering is the discipline that applies engineering, physics, engineering mathematics, and materials science principles to design, analyze, manufacture, and maintain mechanical systems. It is one of the oldest and broadest of the engineering disciplines.

Mechanical Technology Courses, classes and training Programs. Mechanical technology courses are available through undergraduate and advanced degree programs. Curricula typically include courses in mechanical principles, processes and materials, welding, safety and quality control. Mechanical engineering technicians help mechanical engineers manufacture industrial machinery and other equipment. Most employers prefer to hire candidates with associate's degrees or other postsecondary training in mechanical engineering technology.

The importance of mechanical engineering may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being conscious of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechanical Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation

# Purpose of the Qualification:

The purpose of this qualifications is to set high professional standards for Mechanical sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for Mechanical industry

- Enable the existing workforce to capacitate themselves in new technologies and methods

# Overall objectives of training program

The Mechanical qualification of level 3 consists of both the theoretical and practical details and having the following Occupations

0715.6	Basic Welding

0715.7	Foundry & Casting

0715.8	Metrology

0715.9	Plastic Processing Machine operator

0715.10	Communication Skills

# Entry level of trainees

The entry for National Certificate level 3, in Mechanical Technology are

- A person having National Vocational Certificate level 2, in Mechanical Technology.

# Minimum qualification for teachers

- B. Tech in Mechanical Technology 4 Years

- B.Sc. / B.E. in Mechanical Engineering 4 Years

- D. A. E. in Mechanical Technology with 3 Years teaching experience

- Must be able to communicate effectively

Medium of instruction

English, and Urdu.

# Duration of the course:

The entire Modules for Level-3 are composed of 39 Modules that will be covered in 629 Learning hours. Duration of the course is proposed to be Six months. The total weightage for technical competencies is 629 hours. A total of 171 hours has been reserved for allied subjects that are Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical Competencies are given as under:

Level - 2 = 6 months.

Level - 3 = 6 months.

Level - 4 = 1 Year.

Level - 5 = 1 Year.

The distribution of contact hours and credit hours in level-3 is given below:

Level – 3 = 6 months (16 weeks)

Total.	629 hours. & 62.9 Credits

Theory.	125 hours (19.8 %)

Practice.	504 hours (80.2 %)

# Description and structure of the course

Following is the structure of the course:

(0715-M&MT-2) National Qualification Certificate level-3 in “Mechanical Technology”

0715.6. Basic Welding

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Interpret Drawing and Execute to Fabricate Parts | 3 | Technical | 4 | 12 | 16 | 1.6 |
| 2 | Evaluate the Use of Hand and Power Tools | 3 | Technical | 2 | 12 | 14 | 1.4 |
| 3 | Perform Pre-Welding Operations | 3 | Technical | 3 | 12 | 15 | 1.5 |
| 4 | Perform Oxy Acetylene Welding | 3 | Technical | 2 | 9 | 11 | 1.1 |
| 5 | Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting) | 3 | Technical | 4 | 12 | 16 | 1.6 |
| 6 | Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F) | 3 | Technical | 2 | 9 | 11 | 1.1 |
| 7 | Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 3 | Technical | 6 | 27 | 33 | 3.3 |
| 8 | Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 3 | Technical | 4 | 18 | 22 | 2.2 |
| 9 | Perform Soldering and Brazing Operations | 3 | Technical | 2 | 6 | 8 | 0.8 |
|  | Total |  |  | 29 | 117 | 146 | 14.6 |

0715.7. Foundry & Casting

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
|  | Preform Pattern Making | 3 | Technical | 3 | 15 | 18 | 1.8 |
|  | Perform Molding | 3 | Technical | 4 | 12 | 16 | 1.6 |
|  | Preform Core Making | 3 | Technical | 3 | 9 | 12 | 1.2 |
|  | Perform Foundry Work | 3 | Technical | 9 | 27 | 36 | 3.6 |
|  | Perform casting | 3 | Technical | 4 | 21 | 25 | 2.5 |
|  | Perform Cleaning and Finishing Of Casting | 3 | Technical | 3 | 9 | 12 | 1.2 |
|  | Perform Sand Testing | 3 | Technical | 5 | 15 | 20 | 2 |
|  | Total |  |  | 31 | 108 | 139 | 13.9 |

0715.8. Metrology

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
|  | Take measurements with graduated tools | 3 | Technical | 2 | 12 | 14 | 1.4 |
|  | Take measurements with combination set | 3 | Technical | 4 | 21 | 25 | 2.5 |
|  | Perform Levelling | 3 | Technical | 2 | 12 | 14 | 1.4 |
|  | Take measurements through various gauges | 3 | Technical | 2 | 12 | 14 | 1.4 |
|  | Perform measurements through Micrometer | 3 | Technical | 3 | 15 | 18 | 1.8 |
|  | Measure dimensions with Vernier tools | 3 | Technical | 3 | 9 | 12 | 1.2 |
|  | Measure angles with angle measuring Instruments | 3 | Technical | 3 | 15 | 18 | 1.8 |
|  | Perform different measurements | 3 | Technical | 5 | 27 | 32 | 3.2 |
|  | Measure Threads and Gears | 3 | Technical | 2 | 9 | 11 | 1.1 |
|  | Compare measurement with Comparators | 3 | Technical | 2 | 9 | 11 | 1.1 |
|  | Total |  |  | 28 | 141 | 169 | 16.9 |

0715.9. Plastic Processing Machine operator

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
|  | Operate Injection Moulding Machine | 3 | Technical | 4 | 18 | 22 | 2.2 |
|  | Operate Blow Molding Machine | 3 | Technical | 4 | 18 | 22 | 2.2 |
|  | Operate Plastic Film Extrusion machines (Blown/Cast) | 3 | Technical | 4 | 15 | 19 | 1.9 |
|  | Operate Pipe & Profile Extrusion Machines | 3 | Technical | 4 | 12 | 16 | 1.6 |
|  | Operate Rotational Molding Machines | 3 | Technical | 3 | 12 | 15 | 1.5 |
|  | Operate Foam Processing Machines | 3 | Technical | 2 | 6 | 8 | 0.8 |
|  | Operate Compression/Transfer Molding Machines | 3 | Technical | 3 | 9 | 12 | 1.2 |
|  | Operate Multi-Color Printing Machines (Rotogravure) | 3 | Technical | 3 | 12 | 15 | 1.5 |
|  | Operate Multi-Color Printing Machine (Flexographic) | 3 | Technical | 2 | 12 | 14 | 1.4 |
|  | Total |  |  | 29 | 114 | 143 | 14.3 |

0715.10. Communication Skills

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Perform Basic Communication | 3 | Generic | 1.5 | 4.5 | 6 | 0.6 |
| 2 | Communicate at Workplace | 3 | Generic | 2.0 | 6.0 | 8 | 0.8 |
| 3 | Perform Advance Communication | 3 | Generic | 1.5 | 4.5 | 6 | 0.6 |
| 4 | Perform Managerial Communication | 3 | Generic | 3.0 | 9.0 | 12 | 1.2 |
|  | Total |  |  | 8 | 24 | 32 | 3.2 |

# Overview of the Curriculum.

National Certificate Level-3 in Mechanical Technology

| S# | Occupations | Number of Modules | Theory | Practice | Duration |
| --- | --- | --- | --- | --- | --- |
|  | Basic Welding | 9 | 29 | 117 | 146 |
|  | Foundry & Casting | 7 | 31 | 108 | 139 |
|  | Metrology | 10 | 28 | 141 | 169 |
|  | Plastic Processing Machine operator | 9 | 29 | 114 | 143 |
|  | Communication Skills | 4 | 8 | 24 | 32 |
|  | Total | 39 | 125 | 504 | 629 |

- Scheme of Studies

(0715-M&MT-2) National Qualification Certificate of level-3 in “Mechanical Technology”

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit | Periods per weak |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 0715.6 | Basic Welding | Technical | 29 | 117 | 146 | 14.6 | 2 | 7 | 9 |
| 0715.7 | Foundry & Casting | Technical | 31 | 108 | 139 | 13.9 | 2 | 7 | 9 |
| 0715.8 | Metrology | Technical | 28 | 141 | 169 | 16.9 | 2 | 9 | 11 |
| 0715.9 | Plastic Processing Machine operator | Technical | 29 | 114 | 143 | 14.3 | 2 | 7 | 9 |
| 0715.10 | Communication Skills | Soft skills | 8 | 24 | 32 | 3.2 | 1 | 2 | 3 |
| Total |  |  | 125 | 504 | 629 | 62.9 | 9 | 32 | 41 |

Periods per weak

7 periods in a day and 6 days in a week = 7 x 6 = 42 periods in a week (Maximum)

- Detail of Modules

# (0715-M&MT-2) National Qualification Certificate of Level 3 in Mechanical Technology

- Basic Welding

### Interpret Drawing and Execute to Fabricate Parts

Objective: This module covers the knowledge and skills required to Interpret Manufacturing Drawings, Interpret Welding Symbols, Recognize Material Specifications and Bill of Material (BOM) and Manufacture Parts.

Duration: 16 Hours							Theory: 04 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Interpret Manufacturing Drawings | Trainee will be able to - Recognize basics of lines used in engineering drawings - Describe uses of lines in engineering drawings - Recognize and explain orthographic and isometric views of a drawing - Identify manufacturing requirements according to drawings - Prepare job layout according to manufacturing requirements | - Elements of engineering drawing - Drawing symbols - Dimensioning techniques | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Measuring devices for drawing | Class Room And Lab |
| - Interpret Welding Symbols | Trainee will be able to - Identify basic and supplementary welding symbols used in manufacturing drawings - Identify and differentiate between types of welds and joints - Identify welding requirements according to welding symbols given in the manufacturing drawings | - Various Welding symbols - Types of welds and joints - Welding procedure | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Standard charts for welding symbols Standard charts representing various welding joints | Class Room And Lab |
| - Recognize Material Specifications and Bill of Material (BOM) | Trainee will be able to - Identify material specifications according to manufacturing drawing - Identify bill of material (BOM) according to manufacturing drawing | - Specification of materials - BOM | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Existing format/Template of BOM reflecting specs of materials | Class Room And Lab |
| - Manufacture Parts | Trainee will be able to - Interpret dimensional tolerances according to manufacturing drawing - Assemble and tack weld parts according to manufacturing drawing | - Dimensional Tolerances - Tack welding as per manufacturing drawing | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Welding equipment as per requirements | Class Room And Lab |

### Evaluate the Use of Hand and Power Tools

Objective: This module covers the knowledge and skills required to put on hand tools and put on power tools

Duration:  14 Hours							Theory: 2 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU-1. Put on hand tools | Trainee will be able to: - Select hand tools appropriate to the task requirements. - Use hand tools to produce desired outcomes to job specifications which may include finish, tension, size or shape. - Adhere to all safety requirements before, during and after use. - Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use. - Maintain tools, including hand sharpening according to standard operational procedures, principles and techniques. - Store hand tools safely in appropriate location according to standard operational procedures and manufacturers’ recommendations. | - Applications of different hand tools in a general engineering context - Common faults and/or defects in hand tools - Procedures for marking unsafe or faulty tools for repair - Routine maintenance requirements for a range of hand tools - Storage location and procedures for a range of hand tools - Hazards and control measures associated with using hand tools - Benefits and limits of cutting and shaping metal with auxiliary equipment - Environmental benefits of maintaining auxiliary equipment - Clamping /securing methods | Theory-01Hrs Practice-06Hrs Total- 07Hrs | Various hand tools relevant to welding process | Class Room and Lab |
| LU-2. Put on power tools | Trainee will be able to: - Select power tools appropriate to the task requirements. - Use power tools for a determined sequence of operations-which may include clamping, alignment and adjustment to produce desired outcomes-to job specifications which may include finish, size or shape. - Adhere to all safety requirements before, during and after use. - Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use. - Maintain tools, including hand sharpening according to standard operational procedures, principles and techniques. - Store power tools safely in appropriate location according to standard operational procedures and manufacturers’ recommendations. | - Applications of different power tools in a general engineering context - Common faults and/or defects in power tools - Procedures for marking unsafe or faulty tools for repair - Routine maintenance requirements for a range of power tools - Storage location and procedures for a range of power tools - Hazards and control measures associated with using power tools - Benefits and limits of cutting and shaping metal with auxiliary equipment - Environmental benefits of maintaining auxiliary equipment - Clamping /securing methods - Adjustment/alignments to a range of power tools - Tool sharpening techniques for a range of power tools | Theory- 01Hrs Practice-06Hrs Total- 07Hrs | Various power tools relevant to welding process | Class Room and Lab |

### Perform Pre-Welding Operations

Objective: This module covers the knowledge and skills required to Set Welding Equipment, Prepare materials for welding, Cut and Prepare Edge/s of Base Materials and Prepare Welding Consumables

Duration: 15 Hours								Theory: 03 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1 Set Welding Equipment | Trainee will be able to: - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch Make carburizing flame by increasing acetylene gas quantity - Make neutral flame by adjusting both gases at same quantity - Make oxidizing flame by increasing oxygen gas quantity - Adjust pressure of both gas cylinders with the help of regulator - Select the correct size of the nozzle - Set both gas flame of welding torch as per standard of objects on map | - Use welding torch - Identify Gas pressure regulators - Temperature and its units - Making of different flames - Use of different flames | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Acetylene gas cylinder Oxygen gas cylinder Welding torch | Class Room and Lab |
| LU2. Prepare materials for welding | Trainee will be able to: - Select and obtain required materials as per job requirements - Select appropriate marking tools as per job requirements - Mark the area to be cut as per drawing/job requirements | - Material selection as per requirements - Use of marking tools | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Material for making a job as per drawing specs Marking tools required for welding | Class Room and Lab |
| LU3. Cut and Prepare Edge/s of Base Materials | Trainee will be able to: - Select appropriate cutting equipment as per job requirements - Set-up cutting equipment as per manufacturer’s instructions/job requirements - Cut the base material as per job specifications and dimensions provided in the drawing - Prepare edges of the base materials as per drawing/WPS - Check dimensions of the prepared edges as per drawing/WPS - Select proper tools and chemicals for cleaning - Clean the edges of the base materials as per job requirements | - Selection of cutting equipment - Use of cutting equipment - Identification of cleaning chemicals and tools - Use of measuring tools to verify the drawing specs with the actual part. | Theory-01Hrs Practice-03Hrs Total-04Hrs | Cutting equipment Measuring tools Cleaning chemicals | Class Room and Lab |
| LU.4. Prepare Welding Consumables | Trainee will be able to: - Select relevant welding consumables as per job requirements/WPS - Prepare consumables in accordance with required specifications | - Selection of consumables as applied to a particular job requirement as per laid down specs | Theory-0.5Hrs Practice-03Hrs Total-3.5Hrs | Consumables used in welding of job as per drawing specs | Class Room and Lab |

### Perform Oxy Acetylene Welding

Objective: This module covers the knowledge and skills required to Practice of making Tee Joint, Practice of making Lap Joint and Practice of Making Butt Joint

Duration: 11 Hours			                                           Theory: 02 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice of making Tee Joint | Trainee will be able to: - Choose Work piece as per drawing - Straiten it with the help of hammer and anvil - Grind/file the work pieces on grinding machine/file to prepare the edges flat and parallel to each other - Place the Bottom piece on work table and place the Top plate at 900 to each other as per drawing - Set the flame of welding torch as per standard - Complete the bead as per standard | - Use of T-Joint - Grinding/File work - Use of proper flame for oxy-acetylene welding - Use of welding torch to complete the bead | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Oxygen cylinder Acetylene cylinder Gas welding Torch Work piece as per drawing specs Grinding Machine Files Work table | Class Room and La b |
| - Practice of making Lap Joint | Trainee will be able to: - Choose Work piece as per drawing - Straiten it with the help of hammer and anvil - Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other - Place the Bottom piece on work table and place the Top plate along the marked line - Set the flame of welding torch as per standard - Complete the bead as per standard | - Use of Lap-Joint - Grinding/File work - Use of proper flame for oxy-acetylene welding - Use of welding torch to complete the bead | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Oxygen cylinder Acetylene cylinder Gas welding Torch Work piece as per drawing specs Grinding Machine Files Work table | Class Room and Lab |
| - Practice of Making Butt Joint | Trainee will be able to: - Select Work piece as per drawing - Straiten it with the help of hammer and anvil - Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other - Place the work pieces parallel to each other - Set the flame of welding torch as per standard - Complete the bead as per standard | - Use of Butt-Joint - Grinding/File work - Use of proper flame for oxy-acetylene welding - Use of welding torch to complete the bead | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Oxygen cylinder Acetylene cylinder Gas welding Torch Work piece as per drawing specs Grinding Machine Files Work table | Class Room and Lab |

### Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)

Objective: This module covers the knowledge and skills required to cut metal by oxy-acetylene Welding Manually, Cut metal by oxy-acetylene Welding Automatically, Cut metal by Grinding Manually and Cut metal by Grinding Automatically

Duration: 16 Hours								Theory: 04 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Cut metal by oxy-acetylene Welding Manually | Trainee will be able to: - Choose Work piece as per drawing - Straiten it with the help of hammer and anvil if required - Set the flame of welding torch in oxidizing flame as per standard - Start cutting for one side of work piece - Maintain standard distance between welding torch nozzle and work piece - Complete the cut as per standard | - Metal work - Various flames - Preparation of oxidizing flame - Types of torch nozzle - Manual cutting of metal with oxidizing flame | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Work piece as per drawing specs Hammer & Anvil Oxygen Cylinder Acetylene cylinder Gas welding torch Torch Nozzles | Class Room and Lab |
| - Cut metal by oxy-acetylene Welding Automatically | Trainee will be able to: - Identify Work piece as per drawing - Straiten it with the help of hammer and anvil if required - Set the flame of welding torch in oxidizing flame as per standard - Set cutting torch in Auto Feed machine - Set Feed Rate of machine - Start cutting for one side of work piece - Maintain standard distance between welding torch nozzle and work piece - Complete the cut as per standard | - Metal work - Various flames - Preparation of oxidizing flame - Types of torch nozzle - Manual cutting of metal with oxidizing flame - Feed Rate for Automatic machine | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Work piece as per drawing specs Hammer & Anvil Oxygen Cylinder Acetylene cylinder Gas welding torch Torch Nozzles Automatic machine for operation of torch nozzle | Class Room and Survey Field |
| - Cut metal by Grinding Manually | Trainee will be able to: - Select Work piece as per drawing - Straiten it with the help of hammer and anvil if required - Perform marking on work piece where cutting is required - Set disk grinder on grinding machine - Set r.p.m of disk grinding machine as per standard - Cut on marked line with disk grinder by hand | - Metal work - Marking of work piece - Grit selection for grinder to cut metal piece - Function of grinding machine | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Metal piece for cutting Hammer & Anvil Marking tools Grinding Machine Disk for cutting on Grinder | Class Room and Lab |
| - Cut metal by Grinding Automatically | Trainee will be able to: - Choose Work piece as per drawing - Straiten it with the help of hammer and anvil if required - Perform marking on work piece where cutting is required - Set disk grinder on grinding machine - Set r.p.m of disk grinding machine as per standard - Set the guide blade of disk grinding machine - as per requirement - Set feed of disk grinding machine - Cut on marked line with disk grinding machine | - Metal work - Marking of work piece - Grit selection for grinder to cut metal piece - Function of grinding machine - Feed rate - Operation of Automatic Grinding Machine | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Metal piece for cutting Hammer & Anvil Marking tools Grinding Machine Disk for cutting on Grinder Automatic Machine to run the grinder for cutting | Class Room and Lab |

### Perform Gas Welding on Mild Steel Plates (1F, 2F, 3F)

Objective: This module covers the knowledge and skills required to perform Gas Welding on Mild Steel Plates (1F), Gas Welding on Mild Steel Plates (2F) and Gas Welding on Mild Steel Plates (3F)

Duration: 11 Hours								Theory: 02 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Gas Welding on Mild Steel Plates (1F) | The trainee will be able - Open gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch - Ignite acetylene gas with help of spark lighter - Open oxygen gas knob of welding torch - Set work piece as per standard - Perform fore hand welding method - Perform post welding operations | - Use of regulator to adjust gas pressure - Forming neutral flame for gas welding - Fore hand welding method - Post welding operations | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Oxygen gas Acetylene gas Cylinder key for regulator Work piece as per drawing specs | Class Room and Lab |
| LU2. Gas Welding on Mild Steel Plates (2F) | The trainee will be able - Open gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch - Ignite acetylene gas with help of spark lighter - Open oxygen gas knob of welding torch - Set work piece as per standard - Perform welding in right hand welding method - Perform post welding operations | - Use of regulator to adjust gas pressure - Forming neutral flame for gas welding - Right hand welding method - Post welding operations | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Oxygen gas Acetylene gas Cylinder key for regulator Work piece as per drawing specs | Class Room and Lab |
| LU3. Gas Welding On Mild Steel Plates (3F) | The trainee will be able - Open gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch - Ignite acetylene gas with help of spark lighter - Open oxygen gas knob of welding torch - Set work piece as per standard - Perform welding in downward welding method - Perform post welding operations | - Use of regulator to adjust gas pressure - Forming neutral flame for gas welding - Downward welding method - Post welding operations | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Oxygen gas Acetylene gas Cylinder key for regulator Work piece as per drawing specs | Class Room and Lab |

### Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), Hori (2F, 2G), Ver (3F, 3G) and O. head (4F, 4G) Positions

Objective: This module covers the knowledge and skills required to Prepare Welding Machine and Accessories for GMAW, Make Fillet Welds on Carbon Steel Plate, Make Groove Welds on Carbon Steel Plate, Make Fillet Welds on Carbon Steel Plate, Make Groove Welds on Carbon Steel Plate and Perform Post Welding Operations

Duration: 33 Hours						                Theory: 06 Hours				         Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Welding Machine and Accessories for GMAW | The trainee will be able - Identify welding requirements from the job, welding procedure specifications and/or technical drawings - Prepare GMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer instructions - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications | - Various types of welding processes - advantages of GMAW - The principle of MIG welding - Various welding positions - Personal Protective Equipment required for MIG welding and their use - The correct method to wear PPE - Specifications/ classification of electrode/s required for the job - safe working practices to be followed while carrying out MIG welding - Identify hazards associated with MIG welding and take remedial measures - Describe Welding codes and standards - Polarity setting according to standard specifications | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Wire feed unit Gun Assembly Separate power source Earth connection High pressure inert gas Work pieces to be welded | Class Room and Lab |
| - Make Fillet Welds on Carbon Steel Plate (1F, 2F) | The trainee will be able - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the job for fillet weld as per drawing specs - Carry out welding in Flat (1F) and Horizontal (2F) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Fillet lap joint - Tee-fillet joint | Theory-01Hrs Practice-06Hrs Total- 07Hrs | Wire feed unit Gun Assembly Separate power source Earth connection High pressure inert gas Work pieces to be welded | Class Room and Lab |
| - Make Groove Welds on Carbon Steel Plate (1G, 2G) | The trainee will be able - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the job for grove weld as per drawing specs - Carry out welding in Flat (1G) and Horizontal (2G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Corner joint | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Wire feed unit Gun Assembly Separate power source Earth connection High pressure inert gas Work pieces to be welded | Class Room and Lab |
| - Make Fillet Welds on Carbon Steel Plate (3F, 4F) | The trainee will be able - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the job for fillet weld as per drawing specs - Carry out welding in Vertical (3F) and Overhead (4F) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Fillet lap joint - Tee-fillet joint | Theory-01Hrs Practice-06Hrs Total- 07Hrs | Wire feed unit Gun Assembly Separate power source Earth connection High pressure inert gas Work pieces to be welded | Class Room and Lab |
| - Make Groove Welds on Carbon Steel Plate (3G, 4G) | The trainee will be able - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the job for Grove weld as per drawing specs - Carry out welding in Vertical (3G) and Overhead (4G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Corner joint | Theory-01Hrs Practice-06Hrs Total- 07Hrs | Wire feed unit Gun Assembly Separate power source Earth connection High pressure inert gas Work pieces to be welded | Class Room and Lab |
| - Perform Post Welding Operations | The trainee will be able - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines | - Visual welding defects - Personal Protective Equipment required for MIG welding and their use - The correct method to wear PPE | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Chemicals to find welding techniques Cleaning equipment Repair tools | Class Room and Lab |

### Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions

Objective: This module covers the knowledge and skills required to perform Shielded Metal Arc Welding (SMAW) operations in Flat (1F, 1G) and Horizontal (2F, 2G) positions at workplace. The standard covers specific knowledge of performing Shielded Metal Arc Welding (SMAW) by selecting and setting up welding equipment, installing consumables, adjusting welding parameters and making fillet and groove welds in Flat (1F, 1G) and Horizontal (2F, 2G) positions of plate. The standard also covers post welding operations comprising cleaning, measuring, inspecting and repairing welds at workplace

Duration:  22 Hours							       Theory: 04 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Welding Machine and Accessories for SMAW | The trainee will be able to: - Identify welding requirements from the job, welding procedure specifications and/or technical drawings - Prepare SMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice | Class Room, and Lab |
| - . Make Fillet Welds on Carbon Steel Plate (1F, 1G) | The trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld Maintain gap between electrode and base metal as per standard practices - Prepare the work piece/job for fillet weld - Carry out welding in Flat (1F) and Flat (1G) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice | Class Room, and Lab |
| - Make Groove Welds on Carbon Steel Plate (2F, 2G) | The trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the work piece/job for groove weld - Carry out welding in Horizontal (2F) and Horizontal (2G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice | Class Room, and Lab |
| - . Make Fillet Welds on Carbon Steel Plate (3F, 3G) | The trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirement to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the work piece/job for fillet weld - Carry out welding in Vertical (3F) and Vertical (3G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice | Class Room, and Lab |
| - . Make Groove Welds on Carbon Steel Plate (4F, 4G) | The trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the work piece/job for groove weld - Carry out welding in Overhead (4F) and Overhead (4G) positions following standard procedures Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice | Class Room, and Lab |
| - Perform Post Welding Operations | The trainee will be able to: - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines | - Visual welding defects - Welding codes and standards - Specifications/ classification of electrode/s required for the job | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice | Class Room, and Lab |

### Perform Soldering and Brazing Operations

Objective: This module covers the knowledge and skills required to perform Soldering Operation and Brazing Operation

Duration:  08 Hours							       Theory: 02 Hours				Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Soldering Operation | The trainee will able to: - Perform marking as per drawing - Cut the metal sheet according to drawing using shearing machine - Straighten the material with help of hammer - File work if required - Perform soldering operation as per standard | - Soldering Techniques - Sheet metal gauge & cutting - Disadvantages of soldering | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Sheet metal Soldering gun | Class Room and Lab |
| LU2. Brazing Operation | The trainee will able to: - Perform marking as per drawing - Cut the metal sheet according to drawing using shearing machine - Straighten the material with help of hammer - File work if required - Open gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Select the correct size of the nozzle - Set flame to carburizing flame as per standard - Use copper filler rod as filler metal - Perform brazing as per standard | - Brazing Techniques - Preparation of carburizing flame, neutral flame and oxidizing flame | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Oxygen gas Acetylene gas Nozzles Copper wire | Class Room and Lab |

- Foundry & Casting

### Preform Pattern Making

Objective: This module covers the knowledge and skills required to make a pattern for a pipe casting, Practice of making a pattern, on CNC Router, Make a single piece pattern of any English word from a wood plate 1 x 4 x 7cm, Make two-piece patterns (step pulley) along with core prints, shaped pattern and Application of CNC Router for complex

Duration: 18 Hours							Theory: 03 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Make a pattern for a pipe casting | The trainee will be able to - Prepare layout by adding allowances (draft, machining, shrinkage, shaking) - Arrange a 60mm square deodar wood piece with 160 mm length. - Face outer end by holding wooden piece chuck. - Turn outer diameter 50mm with shrinkage allowance. - Turn outer diameter 25mm and into length 25mm for core print. - Repeat procedure by changing direction of pattern. - Prepare another core print diameter. - Remove pattern from lathe machine and mark parting line. - Cut the pattern and fit dowel pin holes. - Assemble the pattern symmetrically. | - types of Pattern, - allowance and allowances used for pattern making - pattern making techniques | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Vernier caliper Deodar wood Hacksaw Tri square Steel tape Wood work lath Chisels Wood work files | Class Room And Lab |
| - Practice of making a pattern, on CNC Router | The trainee will be able to - Prepare the drawing in CAD system keeping in view allowances for the job. - Arrange a wood block size 75 mm* 310 mm*310 mm and clamp it on the table of CNC router. - Send the drawing in CAM system. - Select the cutting tool as per material and operation. - Enter the raw material detail. - Check the tool off setting - See the simulation before starting the work. - Locate the pattern. - Press the push bottom to start the operation. - Pattern will be prepared as per drawing automatically. - Draw out pattern from fixture - Close the machine and check the dimensions of pattern as per drawing 70 300 300 | - CNC machine operations. - Pattern drafting. | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Vernier caliper CNC router machine | Class Room And Lab |
| - Make a single piece pattern of any English word from a wood plate 1 x 4 x 7cm | The trainee will be able to - Prepare the drawing of word E in CAD system along with draft and machining allowance. - Send the drawing in CAM system. - Enter the raw material size in machine. - Select the engraving cutting tool - Check the simulation. - Get a rectangular plate of deoder wood and prepare it in 1 6 9cm size and hold the job in fixture at proper location on reference point on the table of CNC. - Press the start button to start the operation. - Check the tool off- setting. - Machine started engraving the word E on the wood plate - Remove the job from fixture after completion - Check the size of job along with draft and machining allowance. - Switch off the machine. - Check the dimension of pattern as per requirement. | - CNC machine operations. - Pattern drafting. - CAD/CAM techniques | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Pattern material Measuring instruments CNC router machine | Class Room And Lab |
| - Make two-piece patterns (step pulley) along with core prints | The trainee will be able to - Arrange deodar wood block 3 square and length 8. - Hold the job in machine chuck and lathe center. - Mount the turning tool on machine. - Turn the diameter as per drawing - After preparing the dia 70mm and 50 mm, turn the size of core prints as shown in fig. - Mark the parting line on pattern with the help of try square and pencil. - Hold the pattern in vice and cut it in longitudinally in two halves. - Mark the dowel holes and insert the dowel pins in one side of pattern - Again, hold in chuck and turn the accurate dia meter like 65mm, 45mm, and 25mm core print dia. - The pattern is ready as per drawing. - Polish the pattern with varnish and store it | - Types of woods used for pattern making - Advantages and Disadvantage of metallic pattern. - Pattern making machines. - Wood work lath. - Types of chisels and their uses. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Varnish Wood block Abrasive paper. Paint Wood router machine tool Pattern material Measuring instruments Turning tool Hammer Spanner Plane drill Wood saw | Class Room And Lab |
| - shaped pattern | The trainee will be able to - Prepare drawing in auto CAD considering the pattern allowances. - Send the drawing in CAM. Feed the size of raw material along with nature of material - Select the cutting with respect to the radius and fillets - Observe the simulation for fool proofing - Switch on the CNC router - Check the tool off setting - Locate and clamp the work piece at proper location - Consider the tool compensation - Press the button to start the machine. | - CNC router machine operations. | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | CNC router machine Cutting tools | Class Room And Lab |

### Perform Molding

Objective: This module covers the knowledge and skills required to Basic moulding with two-piece pattern, Practice of bedding in pit molding method, Practice of molding irregular pattern with CO2 process and Practice of moulding with three-piece pattern by CO2 process.

Duration: 16 Hours							Theory: 04 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Basic moulding with two-piece pattern | Trainee will be able to: - Prepare green sand by using sieve sand with riddle - Place pattern on molding plate form (without dowel) - Place drag part of the mould and ram moulding sand by jolt and squeezing head of rammer. - Roll over the drag part and strike off. Extra green sand. - Place other half of the pattern (with dowel) and match it. Place runner at appropriate place. - Create a parting line by pouring river sand on drag part. - Place cope part and sprue pin on runner. Add riddled moulding sand and ram it with jolt and squeezing head. - Strike off extra sand and make pouring basin. - Make vents for gas escaping with vent wire at appropriate place. - Remove cope part and make pouring gate with gate cutter. - Make cavity by drawing out pattern halves politely. | - Basic Molding - Types of sand | Theory-01Hrs Practice-03Hrs Total- 4Hrs | Shovel Riddle Lifter Trovel Gate cutter Molding box Sprue pin Runner Sprue pin Vent wire rammer Shovel Riddle Lifter Trovel Gate cutter Molding box Sprue pin rammer | Class Room and Lab |
| - Practice of bedding in pit molding method. | Trainee will be able to: - Dig a pit on floor. - Prepare coke bed by ramming moulding sand on it with jolt and squeezing end of rammer - Place vent pipes and then pattern and press it in sand. - Place the cope on pattern and place sprue pins on runner for gating system. - Fill coup with molding sand and ram it. - Pick up cope and draw pattern. - Prepare gate with gate cutter and repair mold cavity. - Place coup over the bed within locating pegs. | - Molding and its types. - Properties of green sand | Theory-01Hrs Practice-03Hrs Total- 4Hrs | Molding box Sprue pin Runner Sprue pin Vent wire rammer | Class Room and Lab |
| - Practice of molding irregular pattern with CO2 process | Trainee will be able to: - Riddle silica sand. - Mix sodium silicate in silica sand. - Put the drag part of the mould on molding board. - Place pattern on molding board in taper cutting. - Pour silicate molding sand in and ram it with jolt and squeezing end of rammer alternatively - Turn over the drag in and remove extra sand. - Place coup over drag and sprue pin and runner for gating system. - Fill molding sand and ram it. - Do the venting and pick up coup from drag. - Pick up coup part, draw out pattern and repair mold cavity. - Blow out dropped sand and pass CO2. - Assemble coup and drag and place it in casting area. | - Repairing mound and its precautions - CO2 sands | Theory-01Hrs Practice-03Hrs Total- 4Hrs | rammer Shovel Riddle Lifter Trovel Gate cutter Molding box Sprue pin rammer Gate cutter Molding box Sprue pin rammer Sodium silicate CO2 cylinder Silica sand Molding tools Three piece pattern | Class Room and Lab |
| - Practice of moulding with three-piece pattern by CO2 process. | Trainee will be able to: - Riddle silica sand. - Mix sodium silicate in silica sand. - Add adequate moisture in it. - Put the drag on molding board. - Place pattern on molding board in taper cutting. - Pour and sieve sodium silicate molding sand in and ram it with jolt and squeezing head of rammer. - Roll over the drag and expose pattern. - Ram sand around pattern and in corners. - Place cope over the check and through guide pins. - Place third part over middle part. - Fill molding sand and level off sand. - Perform venting and cut poring basin. - Pick up coup and check and draw pattern from check. - Pass CO2 and assemble mold, check and drag. | - Cleaning process. - Molding techniques | Theory-01Hrs Practice-03Hrs Total- 4Hrs | Gate cutter Molding box Sprue pin rammer Sodium silicate CO2 cylinder Silica sand Molding tools Three-piece pattern | Class Room and Lab |

### Perform Core Making

Objective: This module covers the knowledge and skills required to Practice of making round core, Practice of making half core and Practice of baking and assembling half core.

Duration: 12 Hours								Theory: 3 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice of making round core | Trainee will be able to: - Mix riddle sand with molasses as per required and heat to attain molasses sand properties - Fill split box with core sand. - Ram core and do venting. - Remove clamp and split core box. - Put core in oven for baking. - Draw out core from oven and core is ready. | - Core - Types of core | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Molding tools Split box core sand Molding tools Split box Left- and right-hand core box. Two halves of core | Class Room and Lab |
| - Practice of making half core. | Trainee will be able to: - Mix riddle sand with molasses as per required. - Get single piece core box and fill it core sand. - Ram sand in core cavity and strike off excessive sand. - Place core on metallic core plate and put plate in oven. - Switch off oven and draw out baked core. | - Properties of molasses sand - Types of core venting - Steps for core making | Theory-01Hrs Practice-03Hrs Total- 04Hrs | core sand Molding tools Split box | Class Room and Lab |
| - Practice of baking and assembling half core. | Trainee will be able to: - Connect batch type core baking oven with electricity. - Set core on core plate and place it oven. - Set heating temperature. - Draw out core from oven and apply paste on one part of parting line and place it in fixture. - Place another half on it and give half an hour for drying. - Laminate small parting line for proper finish. | - Methods of supporting core - Core making accessories | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Left- and right-hand core box. Two halves of core | Class Room and Lab |

### Perform Foundry Work

Objective: This module covers the knowledge and skills required to Carry out Sand casting With Single Piece Pattern Make English Letters (Alphabet), Make Paper Weight, Perform Casting for Hexagonal Pattern, perform casting With Split Pattern, Cast Anvil, Cast Journal Bearing Body, Cast Pulley and Practice casting through shell molding

Duration: 36 Hours			                                           Theory: 09 Hours				Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Carryout Sand casting With Single Piece Pattern | Trainee will be able to: - Prepare green sand - Place the drag on Moulding Board - Place the pattern in the drag - Pour sand in drag and perform ramming operation - Remove excessive sand from drag with strike of bar. - Flip the drag and sprinkle parting sand - Fit the cope on drag and place runner riser and sprue pin accordingly - Fill cope with green sand and perform ramming operation - Remove excessive sand from cope with strike of bar. - Perform venting as per standard and produce the poring basin and draw out sprue pin. - Split cope and drag and extract pattern, runner and produce gating system. - Pre heat the mold after matching coup and drag to minimize moisture content. - ignite crucible furnace and start blower - Charge metal to be melt. After melting dross out slag by adding slag coagulants. - Pour molten metal with help of crucible and let it cool in the mold. | - Moulding tools - Sand used according to casting. - Types of mould - Casting processes. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Moulding board Moulding box Shovel Riddle Rammer Trowels Slicks Strike off bar Swab Vent wire Sprue pin Sprue cutter Draw spike Gate cutter Wooden pattern | Class Room and Lab |
| - Make English Letters (Alphabet) | Trainee will be able to: - Prepare green sand - Place the drag on Moulding Board - Place the alphabet ‘A’ pattern in the drag - Pour sand and perform ramming operation - Remove excessive sand from drag with strike of bar. - Flip the drag and sprinkle parting sand - Fix the cope on drag and place runner and riser accordingly - Fill cope with green sand and perform ramming operation - Remove excessive sand from cope - Perform venting as per standard and Produce the gates. - Split cope and drag and extract pattern - Remove moisture by preheating the mould. - Melt metal in the crucible to be pour. | - cope and drag - reaming | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Moulding box Shovel Riddle Rammer Trowels Slicks Strike off bar Swab Vent wire Sprue pin Sprue cutter Draw spike Gate cutter Wooden pattern ‘A’ shaped pattern | Class Room and Lab |
| - Make Paper Weight | Trainee will be able to: - . Prepare green sand - . Place the drag on Moulding Board - Place the pattern of paper weight in drag part of the mould. - Pour sand and perform ramming operation - Remove excessive sand from drag - Flip the drag and sprinkle parting sand - Fix the cope on drag and place runner and riser accordingly - Fill cope with green sand and perform ramming operation - Remove excessive sand from cope with the help of strike of bar. - Perform venting as per standard and Produce the gating system. - Split cope and drag and extract pattern - Remove moisture content of coup and drag by preheating. - Charge molten metal to be melt in crucible. - dross out slag by adding slag coagulant with the help of skimmer. - Pour molten metal with help of crucible and let it cool. | - reaming - tempering - riddling - venting - gating | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Moulding box Shovel Riddle Rammer Trowels Slicks Strike off bar Swab Vent wire Sprue pin Sprue cutter Draw spike Gate cutter Wooden pattern ‘A’ shaped pattern Paper weight rectangular pattern | Class Room and Lab |
| - Perform Casting for Hexagonal Pattern | Trainee will be able to: - Prepare green sand - Place the drag on Moulding Board - Place the hexagonal pattern in drag - Pour sand and perform ramming operation - Remove excessive sand from drag with the help of strike of bar. - Flip the drag and sprinkle parting sand to create parting line between mould halves. - Match cope on drag and place runner and riser accordingly - Fill cope with green sand and perform ramming operation - Remove excessive sand from cope with the help of strike of bar. - Perform venting as per standard and Produce the gate under runner - Split cope and drag - Carefully extract pattern and repair Mould cavity if required. - Pour molten metal with help of crucible and let it cool. | - Moulding tools - Types of sand used according to casting. - Types of mould - Casting processes. - Slag coagulants and their types. - cope and drag | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Moulding box Shovel Riddle Rammer Trowels Slicks Strike off bar Hexagonal pattern | Class Room and Lab |
| - Perform casting With Split Pattern | Trainee will be able to: - Prepare green sand - Place the drag on Moulding Board - Place the pattern without dowel pin in the drag - Pour sand and perform ramming operation - Remove excessive sand from drag - Flip the drag and place the pattern with dowel pins on the pattern without dowel pins, and clamp them firmly - Fix the cope on drag and place runner and riser accordingly - Fill cope with green sand and perform ramming operation - Remove excessive sand from cope - Perform venting as per standard and Produce the gate under runner - Split cope and drag - Carefully extract pattern and repair cavity. - Pour molten metal with help of crucible and let it cool. | - Types of mould - Casting processes. - Slag coagulants and their types. - cope and drag - reaming | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Riddle Rammer Trowels Slicks Strike off bar Swab Vent wire Sprue pin Sprue cutter Draw spike Gate cutter Split piece pattern (Bush) | Class Room and Lab |
| - Cast Anvil | Trainee will be able to: - Prepare molasses sand. - Place the drag on Moulding Board - Place the Anvil shaped pattern in the drag as per drawing - Pour sand and perform ramming operation - Remove excessive sand from drag - Flip the drag and sprinkle parting sand - Fix the cope on drag and place runner and riser accordingly - Fill cope with green sand and perform ramming operation - Remove excessive sand from cope - . Perform venting as per standard and Produce the gate under runner - Split cope and drag - Carefully extract pattern and repair Mould cavities. - Preheat the mould. - Melt molten metal in crucible. - Dross out slag with the help of skimmer by adding slag coagulants. - Pour molten metal from crucible into mould. | - Moulding tools - Types of sand used according to casting. - Types of mould - Casting processes. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Shovel Riddle Rammer Trowels Slicks Strike off bar Swab Vent wire Sprue pin Sprue cutter Draw spike Gate cutter Wooden pattern | Class Room and Lab |
| - Cast Journal Bearing Body | Trainee will be able to: - Prepare green sand - Place the drag on Moulding Board - Place the split pattern of journal bearing in the drag as per drawing - Pour sand and perform ramming operation - Remove excessive sand from drag with the help of strike of bar. - Flip the drag and sprinkle parting sand - Fix the cope on drag and place runner, sprue pin and riser accordingly - Fill cope with green sand and perform ramming operation - Remove excessive sand from cope - Perform venting as per standard and Produce pouring basin. - Split cope and drag and produce gating system. - Carefully extract pattern and repair Mould cavities. - Preheat the mould to remove moisture content before pouring. - Melt metal to be pour in crucible - Dross out slag by adding slag coagulants with the help of skimmer. - Pour molten metal into the mould from crucible and let it cool. | - reaming - tempering - riddling - venting - gating | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Moulding board Moulding box Shovel Riddle Rammer Trowels Slicks Strike off bar Swab Vent wire Sprue pin Sprue cutter Draw spike Gate cutter Journal Bearing split pattern | Class Room and Lab |
| - Cast Pulley | Trainee will be able to: - Prepare molasses sand. - Place the drag on Moulding Board - Place the pulley shaped pattern in the drag as per drawing - Pour sand and perform ramming operation - Remove excessive sand from drag. - Flip the drag and sprinkle parting sand - Fix the cope on drag and place runner and riser accordingly - Fill cope with molasses sand and perform ramming operation - Remove excessive sand from cope - Perform venting as per standard and Produce the gate under runner - Split cope and drag - Carefully extract pattern and repair Mould cavities if required. - Give curing time to mould. - charge metal to be melt in crucible and melt it. - skim off slag from surface of the molten metal and Pour it into the mold from crucible. | - Casting processes. - Slag coagulants and their types. - cope and drag | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Moulding board Moulding box Shovel Riddle Rammer Trowels Slicks Strike off bar Swab Vent wire Sprue pin Sprue cutter Draw spike Gate cutter Wooden pattern | Class Room and Lab |
| - Practice casting through shell molding | - Trainee will be able to: - Prepare sand mixed thermosetting binder. - Pour this sand into a mould - Firmly clamp heated match plate pattern on the molding box. - Invert or roll over the mold box containing sand mixed with thermosetting material. - Reposition the moulding box so that all loose sand fell down and form a hard shell after curing. - Heat sand shell for a few minutes in oven for the purpose of complete curing. - Stripped shell mould from the pattern. - Assemble or match two halves of the shell mould and support them by metal shots and pouring sand into a box. - Pour molten metal into the cavity | - Advantages and disadvantages of shell moulding. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Moulding board Moulding box pattern | Class Room and Lab |

### Perform casting

Objective: This module covers the knowledge and skills required to Practice use of optical pyrometer, Practice of complete operation of cupola furnace and Practice of casting of non-ferrous metal.

Duration: 25 Hours								Theory: 04 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice use of optical pyrometer | Trainee will be able to: - Focus optical pyrometer on molten cast iron. - Fit red filter between eye and filament and absorption screen between objective lens and filament. - Install rheostat and measure value of current. - Note the reading of pyrometer | - Optical pyrometer and its use - Advantages of pyrometer | Theory-01Hrs Practice-06Hrs Total- 07Hrs | Optical pyrometer Battery Molten cast iron Ammeter | Class Room and Lab |
| - Practice of complete operation of cupola furnace | Trainee will be able to: - Repair refractory lining of tap hole and slag hole of furnace. - Prepare sand be sand close bottom door. - Pour soft wood inside cupola and light it up. - Add coke from coke bed charge. - Pour iron charge plus limestone. - Fill cupola by Adding coke layer then metal layer turn by turn. - Start air blast when droplets of metal start tickling down through coke opening and from tap hole. - Stop the air and again start air blast and receive molten metal. - Open tap hole after some time to receive molten metal. - Repeat tapping after regular interval. - Stop to charge further melting is finished. - Pour metal in moulds and wait for solidification. - Break up mould and clean casting. | - Cupola furnace and how to prepare cupola for melting. - Charging of cupola - Preparation of cupola lining - Tapping process of cupola furnace. | Theory-2Hrs Practice-09Hrs Total- 11Hrs | Cupola furnace with all accessories Ladles Tongue Bot stick Bolt clay Pig iron Mold Coke Calcium carbonate | Class Room and Lab |
| - Practice of casting of non-ferrous metal | Trainee will be able to: - Place crucible with 10kg aluminum in crucible pit furnace. - Open gas and light spark and run blower. - Pick out crucible after proper melting. - Skim off slag and pour molten metal in the mould. - Breakup mould and clean casting after cooling of casting.RLs | - operation of Crucible furnace - properties of Aluminum and its alloys - Drossing - Chemical reaction in melting of aluminum melting. | Theory-01Hrs Practice-06Hrs Total- 07Hrs | Crucible pit furnace Crucible Pick up tongs Mold Natural gas Goggles | Class Room and Lab |

### Perform Cleaning and Finishing of Casting

Objective: This module covers the knowledge and skills required to Cleaning and finishing of casting, Cleaning and finishing of casting by sand blasting and hand grinding, Cleaning and finishing by tumbling barrel machine.

Duration: 12 Hours								Theory: 03 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Cleaning and finishing of casting | Trainee will be able to: - Brake mold and pick aluminum casting. - Remove sand and cut sprue runner and riser with hand hacksaw. - Hold casting in vice and remove projected part by cape chisel and hammer and apply wire brush. - Remove all fins and casting is ready. educed Levels & Recheck by formula | - Tools used for cleaning of casting. - Sand blasting and its procedure. - Use of hand grinder. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Wire brush Chisel Hammer Vice file Hand grinder | Class Room and Lab |
| - Cleaning and finishing of casting by sand blasting and hand grinding. | Trainee will be able to: - Place cast iron component in sand blasting machine. - Hold sand blasting gun and direct stream towards casting. - Apply on the surface and change direction. - Bring the casting out after all surface is cleaned. - Clean excessive parts by hand grinder after holding it in vice. - Component is finished now. | - properties of sand blasting - operation of tumbling barrel machine - Hydro-blasting | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Sand blasting machine Blasting media Grinder | Class Room and Lab |
| - Cleaning and finishing by tumbling barrel machine. | Trainee will be able to: - Charge the cast component by opening hole of drum. - Close drum after charging. - Run machine at 20 rpm and carry out this for 45 min. - Stop machine and open charging hole. - Drop cast component on ground before other cleaning operations | - pickling process - Fettling and barrel machine operation | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Tumbling barrel machine Star shaped cast iron Wrench | Class Room and Lab |

### Perform Sand Testing

Objective: This module covers the knowledge and skills required to Test	Moisture contents of molding sand, Clay contents test of molding sand, Permeability number test for molding sand, Green compressive strength test of molding sand and Testing fineness number of various sand samples

Duration: 20 Hours						                Theory: 05 Hours				         Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Moisture contents of molding sand | Trainee will be able to: - Prepare sand and take some sample and weight it. - Put the pan in electric drying oven and dry at 105 degrees. - Reweight sample and note difference. - Calculate percentage of moisture. | - testing of foundry sand - Properties of green sand. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Digital balance Moisture drying oven Sample pan | Class Room and Lab |
| - Clay contents test of molding sand | Trainee will be able to: - Take sample of 50-gram dry sand and put sample in AFS clay jar. - Add 475 cc distilled water and 20cc 3% solution of NAOH and stir it. - Wash sand on stirrer and fill jar up to 6 inch and allow it to settle down. - Siphon off upper 5-inch water and again add distilled water upto 6 inch. - Siphon off upper 5 inch and repeat practice until water is clear to depth of 5 inch. - Remove sand grains from jar, dry and weigh. Difference between dried weight and original weight multiplied by 2 give clay % | - quality of moulding sand - AFS in sand | Theory-01Hrs Practice-03Hrs Total- 04Hrs | AFS graduated clay jar Thermometer Sodium hydroxide Stirrer Oven | Class Room and Lab |
| - Permeability number test for molding sand | Trainee will be able to: - Prepare permeability meter and install ram tube with molding sand specimen. - Place ram tube on tapered nose piece and fill outer cylinder with water. - Install a bell and allowed to sink by its own weight by multi position cock. - Record permeability number when air escapes through nozzle by sand specimen. - Calculate permeability number by formula. | - Permeability of sand - AFS standard permeability meter - Parts of permeability meter. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Permeability meter Standard specimen Calculator Molding sand | Class Room and Lab |
| - Green compressive strength test of molding sand | Trainee will be able to: - Pour 50 mm sample in ram tube and give three blows of ram rod. - Remove specimen from ram tube with pusher rod. - Place specimen in compressive strength tester. - Apply load by turning hand wheel. - Record reading when specimen fail. - Calculate compressive strength by formula. | - compressive strength tester - Types of strength test - prepare specimen for compression test | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Molding sand Sand rammer Compressive strength tester | Class Room and Lab |
| - Testing fineness number of various sand samples | Trainee will be able to: - Prepare sample of 50-gram dried molding sand. - Arrange 11 sieves in specific order. - Pour dried sand on top sieve and shake it for 15 min. - Collect portion from each sieve and weight and calculate percentage. - Calculate multiplier factor and add all multiplications and divide by sum of total percentages. The number obtained is fineness number. | - Sieve analysis of green sand. - coal dust in moulding sand | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Green sand Sieves of different sizes Balance | Class Room and Lab |

- Metrology

### Take measurements with graduated tools

Objective: This module covers the knowledge and skills required to take measurements Steel rule, take measurements Hook rule, Take measurements folding rule, Take measurements with Trammels.

Duration: 14 Hours							Theory: 02 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurements Steel rule | The trainee will be able to - Place ruler on different work pieces. - Take dimension and record multiple readings on each work piece. - Take average of readings. - Record the results. | - Decimal and fraction of an inch - Measurement techniques - Calculation of area, volume of geometric shapes. - Arc and circle | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Steel rule Work piece Paper Calculator Pencil | Class Room And Lab |
| - Take measurements Hook rule | The trainee will be able to - Place hook rule on given different work piece. - Take dimension and record multiple readings on each work piece. - Take average of readings. - Record the results | - Decimal and fraction of an inch - Measurement techniques - Calculation of area, volume of geometric shapes. - Arc and circle | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Work piece Paper Calculator Pencil Hook ruler | Class Room And Lab |
| - Take measurements Folding rule | The trainee will be able to - Take suitable work piece for measurement. - Measure the dimensions of work pieces. - Compute surface area, volume, of given work pieces. - Record the results in data table | - Decimal and fraction of an inch - Measurement techniques - Calculation of area, volume of geometric shapes. - Arc and circle | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Folding ruler Work piece Paper Calculator Pencil | Class Room And Lab |
| - Take measurements with Trammels | The trainee will be able to - Open the trammel according to required dimension. - Draw circle or arc by fixing one point in the dot and rotate. | - Decimal and fraction of an inch - Measurement techniques - Calculation of area, volume of geometric shapes. - Arc and circle | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Trammels | Class Room And Lab |

### Take measurements with combination set

Objective: This module covers the knowledge and skills required to Take Measurement with Square head, Perform leveling with square head as spirit level, Measure depth with square head as depth gauge, Measure height with square head as height gauge, Perform leveling /measure angle of surfaces with protractor Head, Perform centering with center head and Find diameter of round bar.

Duration: 25 Hours							Theory: 04 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take Measurement with Square head | Trainee will be able to: - Place the combination set on given drawing sheet. - Draw layout of a component with the help of combination square. | - Measurement techniques - Knowledge of dimensioning of drawings and layouts | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Combination set | Class Room and Lab |
| - Perform leveling with square head as spirit level | Trainee will be able to: - Place the spirit level on horizontal work piece. - Check the deviation of bubbles - Level the surface by inserting shims. - Place square head on vertical work piece. - Note the deviation and correct the level. - Check the other geometrical shapes. | - Measurement techniques - Knowledge of dimensioning of drawings and layouts | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil Spirit level Square head | Class Room and Lab |
| - Measure depth with square head as depth gauge | Trainee will be able to: - Place the square head on work piece. - Apply force at the head to retain contact with the component. - Set square head and record the depth. | - Measurement techniques - Knowledge of dimensioning of drawings and layouts | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Square head | Class Room and Lab |
| - Measure height with square head as height gauge | Trainee will be able to: - Place the square head on work piece. - Measure height of work piece with and without the help of height gauge attachment. | - Measurement techniques - Knowledge of dimensioning of drawings and layouts | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Square head | Class Room and Lab |
| - Perform leveling /measure angle of surfaces with protractor Head | Trainee will be able to: - Place protractor head of combination set on work piece. - Loose screws of revolving circular disk and adjust the bubble of level to minimize deviation. - Study the angle between slopes of a component | - Measurement techniques - Knowledge of dimensioning of drawings and layouts | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Protractor head | Class Room and Lab |
| - Perform centering with center head | Trainee will be able to: - Place the center head on round stock. - Trace the line with scriber. - Revolve the center head by 90o and draw another line. - Find center of round stock by bisecting point of lines. | - Measurement techniques - Knowledge of dimensioning of drawings and layouts | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Round stock | Class Room and Lab |
| - Find diameter of round bar | Trainee will be able to: - Place the center head on round stock - Note the reference point and measure point by avoiding parallax - Find the difference between reference point and measure point. | - Measurement techniques - Knowledge of dimensioning of drawings and layouts | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Round stock | Class Room and Lab |

### Perform Levelling

Objective: This module covers the knowledge and skills required to Exercise of cast iron, granite and glass surface plate, Check level of surfaces with straight edge and spirit level, Check level by Engineer’s level and measurements with Engineer’s parallel

Duration: 14 Hours								Theory: 2 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Exercise of cast iron, granite and glass surface plate | Trainee will be able to: - Place the work piece with angle plate on surface plate. - Check squareness and parallelism of components. - Check the squareness of angle plate with dial indicator. | - Measurement techniques - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Surface plates | Class Room and Lab |
| - Check level of surfaces with straight edge and spirit level | Trainee will be able to: - Place straight edge diagonally on the surface plate - Place spirit level on the straight edge. - Check the level with bubble. - Adjust the level by feeler gauge. . | - Measurement techniques - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Straight edge and spirit level | Class Room and Lab |
| - Check level by Engineer’s level | Trainee will be able to: - Clean the level and surface - Place the level on work piece. - Record the difference by changing the position of level. - Calculate mean value. - Maintain the level of plane by providing packing as per requirement | - Measurement techniques - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Engineer`s level | Class Room and Lab |
| - measurements with Engineer’s parallel | Trainee will be able to: - Insert the engineer’s parallel in work piece. - Measure the taper or diameter of cylindrical surface. | - Measurement techniques - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Engineer`s parallel | Class Room and Lab |

### Take measurements through various gauges

Objective: This module covers the knowledge and skills required to Take measurement with fixed gauge and plug gauge, Take measurement with adjustable gauge, Take measurement with small hole gauge, Take measurement with telescope gauge.

Duration: 14 Hours			                                           Theory: 02 Hours				Practice:12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with fixed gauge and plug gauge. | Trainee will be able to: - Check/Compare the dimension of provided work piece against reference standard with the help of fixed gauge. - The use of plug gauge to determine either dimension is of acceptable size or over size. | - Knowledge of dimensioning and measurement techniques - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Fix gauge Plug gauge | Class Room and Lab |
| - Take measurement with adjustable gauge | Trainee will be able to: - Operation of Insert both “go” and “not go” gauges in the work piece. - Check the sizes and record the results. | - Knowledge of dimensioning and measurement techniques - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Adjustable gauge | Class Room and Lab |
| - Take measurement with small hole gauge | Trainee will be able to: - The hole gauge operation - Measurements and Record the result. | - Knowledge of dimensioning and measurement techniques - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Hole gauge | Class Room and Lab |
| - Take measurement with telescope gauge | Trainee will be able to: - the telescope gauges - Adjustment of the gauge and determination of the size of work piece | - Knowledge of dimensioning and measurement techniques - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Telescope gauge | Class Room and Lab |

### Perform measurements through Micrometer

Objective: This module covers the knowledge and skills required to Take measurement with outside micro-meter, Take measurement with inside micrometer, Take measurement with depth micrometer, Measure threads with micrometer and Take measurement with Vernier micrometer.

Duration: 18 Hours								Theory: 03 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with outside micro-meter | Trainee will be able to: - Clean the gauge and surface of work piece - Open slightly larger than the part to be measured - Set anvil squarely against reference surface of part - Using ratchet, slowly until it clicks once - Record reading and make average. | - Measurement techniques - Tools usage - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil micrometer | Class Room and Lab |
| - Take measurement with inside micrometer | Trainee will be able to: - Clean the gauge and surface of work piece. - Open gauge slightly smaller than the part to be measured - Set anvil squarely against reference surface of part with ratchet, slowly until it click once. - Record reading and make average. | - Measurement techniques - Tools usage - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil micrometer | Class Room and Lab |
| - Take measurement with depth micrometer | Trainee will be able to: - Clean the gauge and surface of work piece. - Access the depth of groove by steel rule - Insert the suitable insertion rod - Place the depth micrometer gauge on the deeper groove and take reading. | - Measurement techniques - Tools usage - Geometric shapes | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil micrometer | Class Room and Lab |
| - Measure threads with micrometer | Trainee will be able to: - Clean the gauge and the threads of work piece. - Use the suitable set of thread shape tips according to thread size which is to be measured. - Record the reading. | - Measurement techniques - Tools usage - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil micrometer |  |
| - Take measurement with Vernier micrometer | Trainee will be able to: - Place specimen on the surface plate - Take reading accurately. - Calculate mean reading. | - Measurement techniques - Tools usage - Geometric shapes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil micrometer |  |

### Measure dimensions with Vernier tools

Objective: This module covers the knowledge and skills required to Take measurement with Vernier caliper, Take measurement with height gauge and Take measurement with Vernier depth gauge.

Duration: 12 Hours								Theory: 03 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with Vernier caliper | Trainee will be able to: - Clean the gauge and surface of work piece. - Place the work piece on surface plate. - Open the lock screws. - Slide the jaws up to the width or size of jaws. - Open slightly larger than the part to be measured. - Set anvil squarely against reference surface of part - Maintain the proper pressure on the jaws with screw. - Lock the movable jaw with knurled screw - Measure and record the dimensions. | - Vernier gauges - Measurement techniques - Dimensioning | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil Vernier caliper | Class Room and Lab |
| - Take measurement with height gauge | Trainee will be able to: - Place cast iron component in sand blasting machine. - Hold sand blasting gun and direct stream towards casting. - Apply on the surface and change direction. - Bring the casting out after all surface is cleaned. - Clean excessive parts by hand grinder after holding it in vice. - Component is finished now. | - Vernier gauges - Measurement techniques - Dimensioning | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil Vernier caliper | Class Room and Lab |
| - Take measurement with Vernier depth gauge | Trainee will be able to: - Clean the gauge and surface of work piece - Hold the work piece on angle plate. - Open the lock screws. - Set the height gauge on stud to check it with fine adjusting screw. - Lock the measuring head. - Record the reading. | - Vernier gauges - Measurement techniques - Dimensioning | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil Vernier caliper | Class Room and Lab |

### Measure angles with angle measuring Instruments.

Objective: This module covers the knowledge and skills required to Take measurement with Bevel protractor, Measure angles with Vernier Bevel protractor, Measure angles with Dial protractor, Measure angles with Steel protractor and Measure angles with Sine bar.

Duration: 18 Hours						                Theory: 03 Hours				         Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with Bevel protractor. | Trainee will be able to: - Clean the instrument and work piece - Set the work piece and protractor as per requirement - Make adjustment of the protractor for accurate measurement. - Record and tabulate the results. | - Measurement techniques - Knowledge of Angles | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Bevel protractor | Class Room and Lab |
| - Measure angles with Vernier Bevel protractor | Trainee will be able to: - set the protractor on work piece properly - Read the Vernier scale - Measure and record the angle for clock wise and counter clock wise directions | - Measurement techniques - Knowledge of Angles | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil Vernier Bevel protractor | Class Room and Lab |
| - Measure angles with Dial protractor | Trainee will be able to: - Set the protractor on work piece properly - Read the dial scale - Measure the angle accurately | - Measurement techniques - Knowledge of Angles | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Dial protractor | Class Room and Lab |
| - Measure angles with Steel protractor | Trainee will be able to: - Set the protractor on work piece - Measure the angle | - Measurement techniques - Knowledge of Angles | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Steel protractor | Class Room and Lab |
| - Measure angles with Sine bar | Trainee will be able to: - Select proper size Sine Bar - Set the Sine Bar with gauge block to measure taper - Calculate tapper angle accurately. | - Measurement techniques - Knowledge of Angles | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Sine bar | Class Room and Lab |

### Perform different measurements

Objective: This module covers the knowledge and skills required to take measurement with dial caliper, Take measurement with dial thickness gauge, Take measurement with dial Indicator, Exercise on gauge blocks, Exercise on tool makers’ microscope, Practice on Profile Projector, Practice Of Digital Instruments, Measure with coordinate measuring machine and Calculate tolerance and allowances.

Duration: 32 Hours						                Theory: 05 Hours				         Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with dial caliper | Trainee will be able to: - Set the pointer on zero properly - Set the work piece correctly - Read the dial scale accurately - Record the results. | - Measurement techniques - Knowledge of Angles - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Dial caliper | Class Room and Lab |
| - Take measurement with dial thickness gauge | Trainee will be able to: - Remove zero error - Set the work piece properly - Measure the size accurately - Record the results | - Measurement techniques - Knowledge of Angles - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil Dial thickness gauge | Class Room and Lab |
| - Take measurement with dial Indicator | Trainee will be able to: - Set the Indicator on magnetic stand - Set the pointer on zero - Set the pointer on work piece - Read the dial scale | - Measurement techniques - Knowledge of Angles - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Dial indicator | Class Room and Lab |
| - Exercise on gauge blocks | Trainee will be able to: - Adjust the gauge block as per desired size - Hold the specimen in position - Measure the angle accurately sine bar and gauge blocks | - Measurement techniques - Knowledge of Angles - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Gauge blocks | Class Room and Lab |
| - Exercise on tool makers microscope | Trainee will be able to: - Prepare the microscope for examining the job - Hold the job on anvil - Observe the size and profile of the job by setting eye piece - Record the observations | - Measurement techniques - Knowledge of Angles - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Tool marks microscope | Class Room and Lab |
| - Practice on Profile Projector | Trainee will be able to: - Make complete setting of work piece on Profile Projector - Set the axis - Observe the size | - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Profile projector | Class Room and Lab |
| - Practice Of Digital Instruments | Trainee will be able to: - Select digital instrument and specimen for measurement. - Take measurement of specimen with digital instrument | - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Digital instruments | Class Room and Lab |
| - . Measure with coordinate measuring machine | Trainee will be able to: - Hold the job properly. - Set and locate zero point. - Take the dimensions for different coordinates properly. - Take point to point/linear measurements. - Take measurements profile in 2D and 3D. | - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Coordinate measurement machine | Class Room and Lab |
| - Calculate tolerance and allowances | Trainee will be able to: - Measure the job for upper and lower limits. - Calculate the maximum and minimum size. - Calculate tolerance and allowance - Represent tolerance in results. | - Digital measuring instruments - Knowledge of dimensioning and layouts. - ISO standards of fits and tolerance - ISO systems of fits and limits - Knowledge of components e.g. shafts, holes | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Measuring tools | Class Room and Lab |

### Measure Threads and Gears

Objective: This module covers the knowledge and skills required to Measure threads with thread gauges, Measure gear dimensions with gear tooth caliper and Measure gear dimensions with gear testing machine.

Duration: 11 Hours						                Theory: 02 Hours				         Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure threads with thread gauges | Trainee will be able to: - Place suitable sized wire in thread of work piece - Take the dimensions. - Note pitch of thread and calculate depth of thread by using formula. | - Threads and their types - Measurement techniques - Gears, terminologies related to gears and its types | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Thread gauge | Class Room and Lab |
| - Measure gear dimensions with gear tooth caliper | Trainee will be able to: - Adjust the jaws of caliper into the tooth of gear work piece. - Measure the tooth thickness, cordials addendum and cordials thickness. - Compare the measured values with theoretical results. | - Threads and their types - Measurement techniques - Gears, terminologies related to gears and its types | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil Gear tooth caliper | Class Room and Lab |
| - Measure gear dimensions with gear testing machine | Trainee will be able to: - Explain working principle of gear inspection machines and draw schematic. - Measurements of spur, bevel, helical with gear testing machine | - Threads and their types - Measurement techniques - Gears, terminologies related to gears and its types | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Gear testing machine | Class Room and Lab |

### Compare measurement with Comparators

Objective: This module covers the knowledge and skills required to Exercise on Mechanical Comparator, Exercise on Electronics Comparator and Exercise on Electrical Comparator.

Duration: 11 Hours						                Theory: 02 Hours				         Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Exercise on Mechanical Comparator | Trainee will be able to: - Clean the instrument and work piece - Set the work piece and protractor as per requirement - Make adjustment of the protractor for accurate measurement. - Record and tabulate the results. | - Measurement techniques - Knowledge about Instrumentation | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Mechanical Comparator | Class Room and Lab |
| - Exercise on Electronics Comparator | Trainee will be able to: - set the protractor on work piece properly - Read the Vernier scale - Measure and record the angle for clock wise and counter clock wise directions | - Measurement techniques - Knowledge about Instrumentation | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Paper Work piece Calculator Pencil Electronics Comparator | Class Room and Lab |
| - Exercise on Electrical Comparator | Trainee will be able to: - Set the protractor on work piece properly - Read the dial scale - Measure the angle accurately | - Measurement techniques - Knowledge about Instrumentation | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Paper Work piece Calculator Pencil Dial protractor | Class Room and Lab |

- Plastic machine processes

- Operate Injection Molding Machine

Objective: This module covers the skills and knowledge required to Select the mould and the machine required for making the product, Inspect the injection moulding machine and carry out necessary adjustments, prior to operation, Set injection-moulding machine for operation, Carry out safety checks prior to operation, Perform sample inspection and Finish and deliver product.

Duration: 22 Hours							Theory: 04 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Select the mould and the machine required for making the product | Trainee will be able to - Identify size of the mould and the machine to fit the mould. - Measure parameters like the specifications, finish of the product - Calculate required, cycle times and estimated total time for making the order etc. - Identify production plan | - The process of production with injection moulding - Basic properties of polymers - Product quality expected | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Injection moulding machine, Mould and ancillary equipment Measuring tape Personal Protective Equipment. Vernier calliper of appropriate range | Classroom And Lab |
| - Inspect the injection moulding machine and carry out necessary adjustments, prior to operation. | Trainee will be able to - Control the following system: - Electrical - hydraulic - pneumatic - and water circulation systems - Identify necessary corrections and carry out lubrication to make the injection moulding machine is operational. - Check if control safety doors and other safety devices are functioning properly. - Inspect abnormalities of the machine if any, and take the corrective action taken recorded in the logbook. | - Safety procedures to be followed. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Personal protective equipment Machine setting instruments | Classroom And Lab |
| - Set injection-moulding machine for operation | Trainee will be able to - Identify job order read and material required for production noted, material, tools and equipment - Prepare/arrange the storage of finished product - Change mould according to the requirement of the job following safety procedures - Set processing parameters according to the data sheet and to suit the quality of the product. | - Interpretation of production instructions - Different moulds used in production. - Chemical raw material usage, safe storage and care with reference to Material Safety Data Sheets (MSDS) | Theory-01Hrs Practice-03Hrs Total- 04Hrs | General mechanic’s tools, Gauge glass, rubber cones Set of gaskets Gland packing | Lab |
| - Carry out safety checks prior to operation | Trainee will be able to - Select and feed all raw materials - Prepare safety checklist for all safety points as per the checklist, and take corrective action where necessary, before starting. - Use personal protective equipment, to suit production process - Switch on machine and set adjustments as necessary, by listening to any unusual noises and feeling for any undue vibrations, and by checking each crucial point of the machine. - Keep informed relevant authorities for any abnormalities. | - Safety procedures to be followed. - Pre-conditioning and drying | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Mould lifting equipment Machine setting instruments Computer with an installed / designed mould heater pa ckage for recording production | Classroom And Lab |
| - Perform sample inspection. | Trainee will be able to - Sample each production according to company policy and product specification. - Check sample physically for quality inspection. - Take necessary steps to fine-tune the process based on sample checked. - Maintain quality assurance approval prior to production | - Faults in products and methods of identification and rectification | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Copper rod Cutter to separate the items Toolbox or tool car containing all necessary tools Hot runner nozzle controller (optional) Temperature measuring instrument. | Lab |
| - Finish and deliver product. | Trainee will be able to - Produce required number of products as per the job card switch off and machine. - Check Quality as per sampling plan, packed and store as planned for easy retrieval preventing any damage or contamination. - Make Necessary entries in production documents. - Store runners and sprues and any rejected items separately for reprocessing. - Transfer the last shot attached to the mould for cleaning and storage. | - Product quality expected | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Production Documents | Lab |

- Operate Blow Molding Machine

Objective: This module covers the skills and knowledge required to Select the mould and the machine required for making the product, Carry out pre inspections and prepare / set the blow-moulding machine for operation, Select and feed raw material, Operate the machine and perform sample production and inspections, Carry out mass (commercial) production as necessary, deliver product and finish the job.

Duration: 22 Hours							Theory: 4 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Select the mould and the machine required for making the product. | Trainee will be able to: - Identify size of the mould and the machine to fit the mould. - Measure parameters like the specifications, finish of the product - Calculate required, cycle times and estimated total time for making the order etc. - Identify production plan. | - Basic knowledge and function of Melt Flow - The process of production with blow moulding | Theory-0.5Hrs Practice-03Hrs Total- 05Hrs | Pocket measuring tape (3m) | Lab & Classroom |
| - Carry out pre inspections and prepare / set the blow-moulding machine for operation. | Trainee will be able to: - Carry out to make the blow moulding machine operational. - Check the individual components movement before running the machine with material in normal practice. - Make the machine safe for operation, by ensuring safety doors and other safety devices, are functioning properly. - Take the abnormalities of the machine if any, and the corrective recorded in the logbook. - Prepare and arrange necessary tools and gauges and accessories such as mould temperature controller, air ejector etc. - Check the parison profile with parison controller | - Index (MFI) and density of raw materials - Melt pressure and temperature - Product quality expected - Safety procedures to be followed with blow moulding machine - Planning of production based on the capacity and output of the machine - The process of production with blow moulding - Chemical raw material usage, safe storage and care. - Product quality expected - Safety procedures to be followed with blow moulding machine - Planning of production based on the capacity and output of the machine | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Pre-heaters Chiller and cooling towers | Lab & Classroom |
| - Select and feed raw material | Trainee will be able to: - Select and Check required raw material for making the product, in correct quantity as appropriate. - Make Arrangements for, preheating where necessary, and facilities for mixing of any fillers and additives and supply of raw material ensured for continuous operation. | - Feeding of raw materials on to blow moulding machine preconditioning if necessary - Basic Properties of Polymers - Safe working with chemicals and machinery - Selection and feeding of raw materials at appropriate times in correct quantities | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Water pumps Air compressors Material mixers A computer with an installed / designed Personal Protective Equipment. | Lab & Classroom |
| - Operate the machine and perform sample production and inspections. | Trainee will be able to: - Check all safety points before starting the machine, and operator safety ensured by wearing of the appropriate Personal Protective Equipment. - Do Necessary adjustments to the machine and sample production commenced, according to company policy and product specifications. - Check the required product specifications verified by either physically the samples or sending for quality checks. - Carry out Any corrections needed to fine tune the process, and exact requirements of the product obtained. | - Quality testing method and identification of faults - Basic First aid with respect to handling of accidents. - Basic firefighting methods & equipment - Operational capability of a blow moulding machine - The assembly and disassembly of moulds and other parts and components. - Identification of faults in products and methods of rectification - Shut down machine | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Personal protective equipment Package for recording production Tool box or Tool car containing all necessary tool Parison controller | Lab |
| - Carry out mass (commercial) production as necessary, deliver product and finish the Job. | Trainee will be able to: - Commence the mass production, check the performance of machine and feeding of the raw material continued, in process inspection done, and adjustments made so as to ensure required quality of the product. - Ensure required number of products as per the production plan (job card) are produced and machine is stopped after job completion - Check quality as per sampling plan, packed and stored as planned for easy retrieval. - Make necessary entries in production documents. - Separate the rejects and off-cuts properly segregated and stored for re-processing. - Transfer the last shot attached to the mould after proper lubrication and for cleaning and storage to avoid corrosion | - The assembly and disassembly of moulds and other parts and components. - Feeding of raw materials on to blow moulding machine preconditioning if necessary - Identification of faults in products and methods of rectification - Ensuring output rate based on production plan. - Shut down machine | Theory-0.5Hrs Practice-06Hrs Total- 6.5Hrs | Production Documents | Lab |

- Operate Plastic Film Extrusion machines (Blown/Cast)

Objective: This module covers the skills and knowledge required to carry out pre inspections and prepare the plastic film extrusion machine for operation, set extrusion machine for operation, operate the machine and commence production, perform sample inspections of product, finish and transfer product for storage.

Duration: 19 Hours								Theory: 4 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Carry out pre inspections and prepare the plastic film extrusion machine for operation. | Trainee will be able to: - Identify any corrections or adjustments to the extrusion machine after inspecting the machine. - Check abnormalities of the machine if any and take the corrective action followed by recording in the logbook along with remarks for the day. | - The extrusion processes - Product quality expected | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Pocket measuring tape Thickness gauge and feeler gauge | Classroom & Lab |
| - Set extrusion machine for operation | Trainee will be able to: - Verify the job requirements and priority from production plan. - Identify material tools and equipment required for production, from - Process job order and arrange a location for storage of finished product. - Make the die change where necessary according to the requirement of the job - Perform job and make adjustments on the machine. - Clean the web rollers and contact arms etc. for safety of machine - Refer the process parameters set to the product data sheet | - Extrusion Dies - Optimization of output - Functions of Melt flow index (MFI) and density of Raw Materials. - Material handling - Mixing, melting, conveying and pressure generation of raw material in the extruder | Theory-01Hrs Practice-03Hrs Total- 04Hrs | A computer with an installed / designed package for recording production | Classroom & Lab |
| - Operate the machine and commence production | Trainee will be able to: - Obtain the quantities of raw material with the quality as specified for the product and sufficient for the work shift. - Check the selected material as appropriate. - Check all safety points before starting the machine using a checklist. - Calculate total time required to complete the production estimated. - Operate machine and make adjustments as necessary by checking each crucial point/function of the machine and by making finer adjustments. - Inform the relevant authority of any defects. - Commence production and take sample according to company policy and product specification. | - Screw characteristics - Zonal distribution and functions of the screw. - Identification of faults in products and methods of rectification - Extruder output calculations - Safety procedures to be followed with extrusion machine - Safety in handling chemicals with reference to material safety data sheet (msds). | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Suitable material handling equipment Surface treatment test pen. Marker pen and adhesive tape, Personal Protective Equipment | Classroom & Lab |
| - Perform sample inspections of product | Trainee will be able to: - Forward samples physically for quality and/or for such inspections. - Any corrections necessary to fine tune the production identified. - Make adjustments on machine and/or check raw material to get the exact requirement of the product. - Obtain the quality assurance approval to prior to production - Performance of machine checked prior to commencing mass production. - Check raw material fed, to continue mass production. - Carry out in- process inspection and necessary adjustments made to produce a product to the required quality standards. - Revise the process parameters recorded for future reference. | - Basic Quality testing methods - Basic polymer rheological and thermal properties | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Appropriate tools Surface treating equipment (optional) Die lifting equipment. Knife | Classroom & Lab |
| - Finish and transfer product for storage | Trainee will be able to: - Identify the required quantity of the product according to the job card. - Check quality as per sampling plan. - Check the output and machine stopped / job changed, when required quantity is produced. - Keep product packed and stacked, similar items together at the appropriate place for easy retrieval prevent any damage or contamination - Make entries in production documents as necessary. - Store all remaining material accounted and returned to where applicable | - Basic first aid with respect to handling of accidents. - Basic firefighting methods & equipment - Production quality based on the capacity and output of the machine - Effects of environmental conditions on the extrusion process - Preventive maintenance of machine or equipment | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Production Documents | Lab |

- Operate Pipe & Profile Extrusion Machines

Objective: This module covers the skills and knowledge required to set extrusion machine for operation, operate the machine and commence production, perform sample inspections of product and finish and deliver product.

Duration: 16 Hours							Theory: 04 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set extrusion machine for operation | Trainee will be able to: - Carryout pre inspections and prepare the pipe or profile extrusion machine for operation - Verify job requirements and priority from production plan. - Identify material tools & equipment required for production, from job order and a location for storage of finished product arranged. - Change dies where necessary according to the requirement of the job as per job card and adjustments made on the machine. - Pre-check the machine performed to verify whether the cooling though cutter, printing and the haul off are working correctly - Set processing parameters by referring to the product data sheet | - Operational capability of pipe and profile extrusion machine - The process of production - Knowledge of raw material - Product quality expected and basic Quality testing methods | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Extrusion Machine, dies Measuring tape Computer with installed / package for production | Classroom & Lab |
| - Operate the machine and commence production | Trainee will be able to: - Select and feed all raw material - Check all safety points before starting the machine using a checklist. - Total time required to complete the production set. - Switch on the machine and make adjustments as necessary by checking each crucial point /function of the machine and by making finer adjustments. Inform relevant authority of any defects. - Commence production and sample according to company policy and product specifications. | - Safety procedures to be followed - Chemical raw material usage, safe storage and care, including Material Safety Data Sheet (MSDS) - Basic properties of polymers - Knowledge of functions of Melt flow index (MFI) and density of Raw Materials. - Material handling - Conveying, melting, mixing and pressure generation - Maintenance of product quality through adjustment of machine and raw material input - Use of Personal Protective Equipment (PPE) | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Personal Protective Equipment Vernier Calliper | Lab |
| - Perform sample inspections of product | Trainee will be able to: - Check samples physically for quality and/or forwarded for such inspections. - Carry out any corrections necessary to fine tune the process identified. - Check adjustments made on machine and / or raw material to get the exact requirement of the product. - Carryout mass production as necessary | - Identification of faults in products and methods of rectification - Purging and shutdown methods - Pipe/profile calibration technique - Sharpening of circular saw - Basic knowledge on quality checking parameters | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Trimming Knife Platform scale for product weighing Thickness gauge and feeler gauge General hand tools | Lab |
| - Finish and deliver product | Trainee will be able to: - Identify required quantity of the product according to the job card/production plan. - Make Product inspected and any further adjustments as necessary. - Keep Product packed and stacked, similar items together at the appropriate place for easy retrieval. - Make Entries in production documents as necessary. - Check Output and machine stopped / job changed, when required quantity is produced. - Store Rejected items and off-cuts separately for re-processing. | - Safe working with chemicals and machinery - Material handling - The assembly and disassembly of parts and components of machine. - Controlling of melt temperature and pressures according to the process parameters - Operation and maintenance of pipe & profile printing machinery - Waste handling | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Material handling equipment Die lifting equipment Circumference tape Personal Protective Equipment Vernier Calliper | Lab |

- Operate Rotational Molding Machines

Objective: This module covers the skills and knowledge required to set rotational moulding machine for operation, operate the machine and commence production and finish the product and prepare mould for next cycle.

Duration: 15 Hours								Theory: 03 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set Rotational Molding machine for operation | Trainee will be able to: - Carryout pre inspections and prepare/ Rotational Molding Machine for operation - Verify Job requirements and priority from production plan. - Identify Material required for production, tools & equipment required, from job order and a location for storage of finished product arranged. - Change molds where necessary according to the requirement of the job as per job card and instructions from superior officer. - Carry out final check on mold mounting - Apply mold release agent and bake. - Set processing parameters by referring to the product data sheet and adjustment made on the machine as necessary. | - The process of production with rotational molding machines - Expected product quality - Safety procedures to be followed with rotational molding machine | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Rotational molding machine tools & equipment Computer with an installed / designed package Measuring tape | Classroom & Lab |
| - Operate the machine and commence production | Trainee will be able to: - Select and feed all raw material - Check all safety points before starting the machine using a checklist. - Set total cycle time required for finishing the product according to product requirement. - Activate and adjust machine by checking each crucial point /function of the machine and by making finer adjustments. - Commence production and check product according to specifications and quality requirements. | - Chemical raw material usage, safe storage and care including Material Safety Data Sheet (MSDS) - Functions of Melt flow index (MFI) and density of Raw Materials. - Material handling - Basic First aid with respect to handling of accidents. - Basic firefighting equipment and methods. | Theory-01Hrs Practice-06Hrs Total- 07Hrs | Thickness measuring equipment for recording production Trimming Knife Mold lifting equipment Platform scale for product weighing Personal Protective Equipment. | Classroom & Lab |
| - Finish the product and Prepare mold for next cycle | Trainee will be able to: - Obtain finished product by trimming flash, punching holes and application of stickers - Attach basic accessories where required. - Check required quantity against job card transfer product to storage area safely. - Make production documents as necessary. - Clean mold and release mold agent if necessary. - Store rejected items separately for re-processing | - Consolidation of production documents, recording and sorting the documents - Waste handling | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Material handling equipment. General hand tools | Lab |

- Operate Foam Processing Machines

Objective: This module covers the skills and knowledge required to set foam processing machine for operation, operate the machine and commence production and check and transfer product for further processing.

Duration: 08 Hours								Theory: 02 Hours				Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set foam processing machine for operation | Trainee will be able to: - Carryout pre inspections on the foam processing machine and mold. - Verify job requirements and priority from production plan. - Identify material, tools & equipment required for production from job order - Arrange a location for storage of finished product arranged. - Change mold where necessary according to the requirement of the job as per job card and adjustments made on the machine. - Set process parameters by referring to the product data sheet and to suit product, production speed quality and standard wastage. | - The process of production with rotational molding machines | Theory-0.5Hrs Practice1.5Hrs Total- 2Hrs | Foam processing machine Computer with an installed / designed package Measuring tape | Classroom & Lab |
| - Operate the machine and commence production | Trainee will be able to: - Select and feed all raw material - Check all safety points before starting the machine using a checklist. - Set Total time required for finishing the product - Switch on Machine and adjustments made as necessary - Check each crucial point /function of the machine and by making finer adjustments. - Commence production and intermediate product of “foam - bun” checked as for specification. - Adjust machine and / or check raw material to get the exact requirement of the product by analyzing the quality requirement of final product. | - Expected product quality - Safety procedures to be followed with rotational molding machine - Chemical raw material usage, safe storage and care including Material Safety Data Sheet (MSDS) - Functions of Melt flow index (MFI) and density of Raw Materials. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Thickness measuring equipment for recording production Trimming Knife Mold lifting equipment Platform scale for product weighing Personal Protective Equipment. | Classroom & Lab |
| - Check and transfer product for further processing | Trainee will be able to: - Identify required quantity of the product according to the job card. - Inspect Product and any further adjustments made as necessary. - Make Entries in production documents as necessary. - Check output and stop machine /job, when required quantity is produced, following correct shut down procedure. - Transfer product to storage area for curing. | - Material handling - Consolidation of production documents, recording and sorting the documents | Theory-0.5Hrs Practice1.5Hrs Total- 2Hrs | Material handling equipment. General hand tools Relevant test equipment. | Lab |

- Operate Compression/Transfer Molding Machines

Objective: This module covers the skills and knowledge required to set compression/transfer moulding machine for operation, operate the machine and commence production and finish and transfer product.

Duration: 12 Hours								Theory: 03 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set compression/ transfer molding machine for operation | Trainee will be able to: - Carryout pre inspections on the compression/transfer molding machine and molds. - Verify requirements and priority verified from production plan. - Material required for production, tools & equipment required, identify from job order and a location for storage of finished product arranged. - Change mold where necessary according to the requirement of the job as per job card and adjustments made on the machine. - Set processing parameters by referring to the product data sheet and to suit product, production speed, quality and standard wastage. | - The process of production with compression molding machines | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Compression moulding machine & equipment Computer with an installed / designed package Measuring tape | Classroom & Lab |
| - Operate the machine and commence production | Trainee will be able to: - Select and feed all raw materials - Check all safety points before starting the machine using a check-list. - Set total time required for finishing the product - Switch on machine and make necessary adjustments by checking each crucial point /function of the machine and by making finer adjustments. - Inform relevant authority of any defects. - Commence production and sample produced according to company policy. | - Expected product quality - Safety procedures to be followed with transfer molding machine - Chemical raw material usage, safe storage and care including Material Safety Data Sheet (MSDS) - Functions of Melt flow index (MFI) and density of Raw Materials. - Basic First aid with respect to handling of accidents. - Basic firefighting equipment and methods. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Thickness measuring equipment for recording production Trimming Knife Mold lifting equipment Platform scale for product weighing Personal Protective Equipment. | Classroom & Lab |
| - Check and transfer product for further processing | Trainee will be able to: - Perform sample product inspections - Carryout mass production as necessary required quantity of the product identified according to the job card. - Inspect product and any further adjustments made as necessary. - Pack and stack Products keeping similar items together at the appropriate place for easy retrieval. - Make entries in production documents as necessary. | - Material handling - Consolidation of production documents and their handling | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Material handling equipment. General hand tools Relevant test equipment. Production record sheets | Lab |

- Operate Multi-Color Printing Machines (Rotogravure)

Objective: This module covers the skills and knowledge required to Set printing machine for operation, operate the machine and commence production and Finish and transfer product for storage.

Duration: 15 Hours								Theory: 03 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set printing machine for operation | Trainee will be able to: - Carryout pre-inspections on the printing machine. - Verify job requirements and priority from production plan. - Clean all web rollers as per given instruction. - Identify material, tools & equipment required for production, from job order and a location for storage of finished product arranged. - Make adjustments on the machine and printing cylinders changed where necessary according to requirement of the job. - Set processing parameters by referring to the product data sheet. - Plan production according to total time allocated for the job as per job card taking in to consideration the setting up time. | - The process of production with printing machines | Theory-01Hrs Practice03Hrs Total- 04Hrs | Rotogravure Computer with an installed / designed package Measuring tape | Classroom & Lab |
| - Operate the machine and commence production | Trainee will be able to: - Inspect product and make any further adjustments necessary. - Pack and stack Products, keeping similar items together at the appropriate place for easy retrieval and for quality check and to prevent any damage or contamination. - Check output and machine stopped / job changed, when required quantity is produced, following correct shut down procedure where necessary. - Make entries in production documents as necessary. - Account all remaining material and returned to store where applicable | - Expected product quality - Safety procedures to be followed with printing machine - Chemical raw material usage, safe storage and care including Material Safety Data Sheet (MSDS) - Basic First aid with respect to handling of accidents. - Basic firefighting equipment and methods. | Theory-01Hrs Practice-03Hrs Total-04Hrs | Thickness measuring equipment for recording production Trimming Knife Platform scale for product weighing Personal Protective Equipment. | Classroom & Lab |
| - Check and transfer product for further processing | Trainee will be able to: - Perform sample product inspections - Carryout mass production as necessary required quantity of the product identified according to the job card. - Inspect product and any further adjustments made as necessary. - Pack and stack Products keeping similar items together at the appropriate place for easy retrieval. - Make entries in production documents as necessary. | - Material handling - Consolidation of production documents, recording and sorting the documents | Theory-01Hrs Practice06Hrs Total- 07Hrs | Material handling equipment. General hand tools Production record sheets | Lab |

- Operate Multi-Color Printing Machines (Flexographic)

Objective: This module covers the skills and knowledge required to Set printing machine for operation, operate the machine and commence production and Finish and transfer product for storage.

Duration: 14 Hours								Theory: 02 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set printing machine for operation | Trainee will be able to: - Carryout pre-inspections on the printing machine. - Verify job requirements and priority from production plan. - Clean all web rollers as per given instruction. - Identify material, tools & equipment required for production, from job order and a location for storage of finished product arranged. - Make adjustments on the machine and printing cylinders changed where necessary according to requirement of the job. - Set Processing parameters by referring to the product data sheet. - Plan Production according to total time allocated for the job as per job card taking in to consideration the setting up time. | - Testing with viscosity cups - Mayer rod testing - Proper use of auto viscosity controllers - Selecting, sharpening, mounting, positioning and use of doctor blades | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Pocket measuring tape (3m) • Viscosity cup Printing machine tools & equipment A computer with an installed / designed package | Class room & Lab |
| - Select design and feed material required for printing | Trainee will be able to: - Obtain Quantities of material with the quality as specified for the product and sufficient for the work shift - Check selected material including inks and solvents as appropriate. - Arrange continuous supply of material to complete job/for the work shift. - Mount printing plates with the correct design as per job ticket on the machine. - Install appropriate tooling (Anilox Rollers/ Cutters / Perforation units) required for the given design. | - Printing plates - Print drying requirements | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Personal Protective Equipment. for recording production Platform scale Mixing rod Mayer rod |  |
| - Operate the machine and commence production | Trainee will be able to: - Ensure that all safety points checked before starting the machine using a checklist. - Switch on Machine and adjustments made as necessary by checking each crucial point /function of the machine and by making finer adjustments. Inform relevant authority of any defects. - Perform test run prior to commencing production. - Check compliance of the color/ registration/ positioning/ perforation and alignment, with the sample provided for guidance and production commenced by making finer adjustments. | - Storage of inks - Formulation of colors - Register control - Knowledge of color sequence - Quality testing methods | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Thickness gauge Knife Material handling equipment Dyne test marker pen scotch tape to treat | Classroom & Lab |
| - Check and transfer product for further processing | Trainee will be able to: - Inspect product inspected and make any further adjustments as necessary. - Pack and Stack Products, keeping similar items together at the appropriate place for easy retrieval for quality checks and to prevent any damage or contamination. - Check Output and machine stopped / job changed, when required quantity is produced, following correct shut down procedure where necessary. - Make Entries in production documents as necessary. - Account all remaining material and returned to stores where applicable. | - Effects of environmental conditions - Knowledge of print defects, how to recognize them, the probable causes and remedies. - Firefighting equipment and methods. - First aid treatment especially in the event of injury from harmful chemical. | Theory-0.5Hrs Practice-03Hrs Total- 3.5Hrs | Marker pen surface treatment level or ink General tools Compass Printing cylinder mounting tables & tools Magnifying glass | Lab |

- Communication Skills

- Perform Basic Communication

Objective: This module covers the knowledge and skills required to Make series circuit and measure voltage and verify KVL, Make parallel circuit, measure current and verify KCL, Verify resistance of a resistor, Find unknown value of ohms law.

Duration: 06 Hours							Theory: 1.5 Hours					Practice: 4.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Work in a Team to achieve intended outcomes | Trainee will be able to - Treat team members with respect and maintain positive relationships to achieve common organizational goals - Get work related information from team members/supervisors and identify interrelated work activities to avoid confusion - Adopt communication skills, appropriate to work activities and organizational procedures - Identify problems and resolve them through discussion and mutual agreement | - Learn and understand Types of communication - Workplace dress code - the role of team members and functionality of the teams | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Follow Supervisor’s instructions as per organizational SOPs | Trainee will be able to - Receive the instructions from Supervisor - Carry out the instructions of the supervisor - Report to the supervisor as per organizational SOPs | - Technology Skills - Interview Skills | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Develop Generic communication work skills at workplace | Trainee will be able to - Develop foundation skills for communication at workplace: - Reading Skills - Writing skills - Verbal communication skills - Learning skills - Problem solving skills - Self-Management Skills - Technology Skills - Interview Skills - Communicate foundation skills required to perform work activity | - Reading Skills - Writing skills - Verbal communication skills - Problem solving skills - Self-Management Skills | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |

- Communicate at Workplace

Objective: This module covers the knowledge and skills required to Construct series combination of batteries, Construct series combination of batteries, Prepare Electrolyte for battery, Check specific gravity of Electrolyte, Determine internal resistance of cell, Charge the battery with appropriate voltage.

Duration: 08 Hours							Theory: 02 Hours					Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Communicate within the organization | Trainee will be able to - Communicate within a department for successful interaction - Communication with other departments. - Use various media to communicate effectively. - Communicate verbally and non-verbally using professionalism | - Importance of intra and inter organizational communication - Defining Modes of communication | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Communicate outside the organization | Trainee will be able to - Deal with vendors - Deal with clients/customers - Interact with other organizations - Use various media to communicate effectively Work with people of different cultures / backgrounds | - effective communication in workgroup - Basics of business communication | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Communicate effectively in workgroup | Trainee will be able to - Assess the issues to provide relevant suggestion to group members - Resolve the issues/ problems /conflicts within the group - Arrange group working sessions to increase the level of participation in the group processes - Communicate messages to group members clearly to ensure interpretation is valid - Communicate style /manner to reflect professional standards/ awareness of appropriate cultural practices - Act upon constructive feedback | - the importance of teamwork | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Communicate in writing | Trainee will be able to - Identify relevant procedures for written information - Use strategies to ensure correct communication in writing. i.e. - correct composition - clarity - comprehensiveness - accuracy - appropriateness - Draft assigned written information for approval, ensuring it is written within designated timeframes - Ensure written information meets required standards of style, format and detail - Seek assistance / feedback to aid communication skills development | - communicating through writing | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  |  |

- Perform Advance Communication

Objective: This module covers the knowledge and skills required to Measure power using Volt-Ampere meter method, Measure power using watt meter, Measure consumed energy with Energy meter, Measure power factor with voltmeter, ampere meter and watt meter, Measure power factor with power factor meter, Improve power factor & measure it with power factor meter.

Duration: 06 Hours							Theory: 1.5 Hours					Practice: 4.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Demonstrate professional skills | Trainee will be able to - Use different modes of communication to communicate - Speaking - reading - Writing - Listening - Presentation - visual representation etc - Illustrate business terms used at workplace - Upgrade professional skills by attending trainings, webinars, conferences etc. - Perform Continuous professional development required at workplace - Develop interview skills | - Explaining the training skills - Identification of the professional skills - Describing the advanced language skills - Understanding of the assessment and trainees feedback methods - Direct and indirect communication methods - Explaining the need of the training type at the work place | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Plan and Organize work | Trainee will be able to - Identify task requirements. - Plan steps to complete tasks. - Organize work. - Review planning and organizing process |  | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Provide trainings at workplace | Trainee will be able to - Assess the need for training - Prepare trainees for the learning experience - Present training session - Support trainees in managing their own learning - Facilitate group learning - Provide opportunity for practice - Provide feedback on progress on trainees - Review delivery experience |  | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |

- Perform Managerial Communication

Objective: This module covers the knowledge and skills required to Construct Electromagnet, implement circuit to determine the effect on current carrying conductor in magnetic field, Determine the effect on conductor by varying the current with the help of rheostat, Plot magnetic lines of forces of bar magnet, Verify Faradays law by moving magnet inside the coil, Verify Faradays law by moving coil near the magnet field, Verify emf through induction.

Duration: 12 Hours							Theory: 03 Hours					Practice: 09Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Provide Motivational Support to others | Trainee will be able to - Recognize and reward people for their achievements. - Acknowledge people for their contributions. - Encourage people to feel good about their accomplishments. - Find creative ways to make people’s work rewarding. - Signals own commitment to a process by being personally present and involved at key events. - Give presentations that energize groups. | - Explaining the motivational skills - Identification of the problem-solving techniques - Understanding of the advanced language skills - Communicating trends for career development - Describe communication channels - Describing the career development plan | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Communicate with team leaders | Trainee will be able to - Provide opportunities for people to learn to work together as a team. - Enlist the active participation of everyone. - Promote cooperation with other work units. - Ensure that all team members are treated fairly. - Recognize and encourage the behaviors that contribute to teamwork |  | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Manage interview to provide information | Trainee will be able to - Plan an interview - Conduct an interview - Provide follow up action as a result of the interview |  | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Apply problem solving techniques at workplace | Trainee will be able to - Identify the issues (Diagnostic Information Gathering) - Analyze everyone’s interests (Analytical Thinking) - List the possible solutions/options - Evaluate the options - Select the option(s) - Document the agreement - Agree on contingences, monitoring and evaluation (Forward Thinking) - Openness to different and new ways of doing things (Flexibility) - Communicate ethically - Manage conflict |  | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Communicate for business in career development | Trainee will be able to - Adopt channels of communication - Adopt 7 c’s of communication |  | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |
| - Communicate trends in career development | Trainee will be able to - Plan for career development as per current trends - Develop a model for career development - Communicate trends with subordinates |  | Theory-0.5 Hrs Practice-1.5 Hrs Total- 02 Hrs |  | Classroom |

- List of Machinery, Tools & Equipment.

National Qualification Certificate level-3 in “Mechanical Technology”

(FOR A CLASS OF 25 STUDENTS)

    - Basic Welding

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Layout tools. | 05 Nos | 1 | Welding electrode Size 12 No and10 No | 5 Packet each |
| 2 | Measuring devices. | 05 Nos | 2 | Metal Filler rod | 5 Kg |
| 3 | Hand held calculator. | 05 Nos | 3 | Welding flux | 5 Packet |
| 4 | Work bench | 05 Nos | 4 | Solder wire | 0.5 kg |
| 5 | Bench vice | 05 Nos | 5 | Copper Filler rod | 1 kg |
| 6 | Hammer | 05 Nos | 6 | Cotton waste | 10 kg |
| 7 | Tri-square | 05 Nos | 7 | Kerosene Oil | 10 liters |
| 8 | Hand hacksaw | 05 Nos |  |  |  |
| 9 | Scriber | 05 Nos |  |  |  |
| 10 | Vernier caliper | 05 Nos |  |  |  |
| 11 | Flat File | 05 Nos |  |  |  |
| 12 | Number/alphabet punch | 25 sets each |  |  |  |
| 13 | Hand drill machine | 05 Nos |  |  |  |
| 14 | Disk grinder | 05 Nos |  |  |  |
| 15 | Pin grinder | 05 Nos |  |  |  |
| 16 | Oxygen cylinder | 05 Nos |  |  |  |
| 17 | Acetylene gas cylinder | 05 Nos |  |  |  |
| 18 | Pressure regulators | 05 Nos |  |  |  |
| 19 | Cylinder key | 05 Nos |  |  |  |
| 20 | Welding torch | 05 Nos |  |  |  |
| 21 | Rubber house pipe | 05 Nos |  |  |  |
| 22 | Back fire arrester | 05 Nos |  |  |  |
| 23 | Spark lighter | 05 Nos |  |  |  |
| 24 | Steel wire brush | 05 Nos |  |  |  |
| 25 | Pedestal grinder | 05 Nos |  |  |  |
| 26 | Solder gun | 05 Nos |  |  |  |
| 27 | Marking tools | 05 Nos |  |  |  |

    - Foundry & Casting

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Vernier caliper | 25 Nos | 1 | Deodar wood | 05 cubic. ft |
| 2 | Hacksaw | 25 Nos | 2 | Varnish | 05 liters |
| 3 | Tri square | 25 Nos | 3 | Wood block | 25 Nos |
| 4 | Steel tape | 25 Nos | 4 | Abrasive paper. | 50 Nos |
| 5 | Wood lathe | 05 Nos | 5 | Paint | 10 liters |
| 6 | Chisels | 25 Nos | 6 | Pattern material | 25 Nos |
| 7 | Wood work files | 25 Nos | 7 | Sodium silicate | 05 kg |
| 8 | CNC router machine | 01 No | 8 | CO2 cylinder | 05 Nos |
| 9 | Wood router machine tool | 05 Nos | 9 | Silica sand | 05 cubic ft |
| 10 | Measuring instruments | 25 Nos | 10 | Three-piece pattern | 25 Nos |
| 11 | Turning tool | 05 Nos | 11 | core sand | 05 cubic ft |
| 12 | Hammer | 25 Nos | 12 | Two halves of core | 25 Nos |
| 13 | Spanner | 05 Nos | 13 | Wooden pattern | 25 Nos |
| 14 | Plane drill | 05 Nos | 14 | ‘A’ shaped pattern | 25 Nos |
| 15 | Wood saw | 01 no | 15 | Paper weight rectangular pattern | 25 Nos |
| 16 | Shovel | 25 Nos | 16 | Hexagonal pattern | 25 Nos |
| 17 | Riddle | 25 Nos | 17 | Split piece pattern (Bush) | 25 Nos |
| 18 | Lifter | 25 Nos | 18 | Skimmer. | 25 Nos |
| 19 | Trowel | 25 Nos | 19 | Journal Bearing split pattern | 25 Nos |
| 20 | Gate cutter | 25 Nos | 20 | Coke | 10 kg |
| 21 | Molding box | 25 Nos | 21 | Calcium carbonate | 10 kg |
| 22 | Sprue pin | 25 Nos | 22 | Pig iron | 50 kg |
| 23 | Runner | 25 Nos | 23 | Star shaped cast iron | 50 kg |
| 24 | Vent wire | 25 Nos | 24 | Sodium hydroxide | 05 kg |
| 25 | Sand rammer | 25 Nos | 25 | Molding sand | 15 cubic ft |
| 26 | Left- and right-hand core box. | 25 Nos | 26 | Green sand | 10 cubic ft |
| 27 | Split box | 25 Nos |  |  |  |
| 28 | Slicks | 25 Nos |  |  |  |
| 29 | Strike off bar | 25 Nos |  |  |  |
| 30 | Swab | 25 Nos |  |  |  |
| 31 | Sprue cutter | 25 Nos |  |  |  |
| 32 | Draw spike | 25 Nos |  |  |  |
| 33 | Optical pyrometer | 05 Nos |  |  |  |
| 34 | Cupola furnace with all accessories | 01 No |  |  |  |
| 35 | Ladles | 05 Nos |  |  |  |
| 36 | Tongue | 05 Nos |  |  |  |
| 37 | Crucible pit furnace | 01 No |  |  |  |
| 38 | Crucible | 05 Nos |  |  |  |
| 39 | Pick up tongs | 05 Nos |  |  |  |
| 40 | Wire brush | 25 Nos |  |  |  |
| 41 | Hand grinder | 05 Nos |  |  |  |
| 42 | Sand blasting | 01 No |  |  |  |
| 43 | Tumbling barrel machine | 01 No |  |  |  |
| 44 | Compressive strength tester | 01 No |  |  |  |
| 45 | Digital balance | 05 Nos |  |  |  |
| 46 | Moisture drying oven | 05 Nos |  |  |  |
| 47 | Sample pan | 25 Nos |  |  |  |
| 48 | AFS graduated clay jar | 25 Nos |  |  |  |
| 49 | Thermometer | 25 Nos |  |  |  |
| 50 | Stirrer | 25 Nos |  |  |  |
| 51 | Permeability meter | 25 Nos |  |  |  |
| 52 | Standard specimen | 25 Nos |  |  |  |
| 53 | Sieves of different sizes | 25 Nos |  |  |  |
| 54 | Balance | 05 Nos |  |  |  |

    - Metrology

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Steel rule | 25 Nos | 1 | Cotton Waste | 10kg |
| 2 | Work piece | 25 Nos | 2 | Kerosine oil | 05 liters |
| 3 | Surface plate | 25 Nos |  |  |  |
| 4 | Steps and collars | 25 Nos |  |  |  |
| 5 | Hook rule | 25 Nos |  |  |  |
| 6 | Folding rule | 25 Nos |  |  |  |
| 7 | Trammel | 25 Nos |  |  |  |
| 8 | Combination set | 25 Nos |  |  |  |
| 9 | Scriber | 25 Nos |  |  |  |
| 10 | Tri square | 25 Nos |  |  |  |
| 11 | Divider | 25 Nos |  |  |  |
| 12 | Round stock | 25 Nos |  |  |  |
| 13 | Straight edge and spirit level | 25 Nos |  |  |  |
| 14 | Engineer’s level | 25 Nos |  |  |  |
| 15 | Engineer’s parallel | 25 Nos |  |  |  |
| 16 | Fix gauge | 25 Nos |  |  |  |
| 17 | Surface gauge | 25 Nos |  |  |  |
| 18 | Dial indicator | 25 Nos |  |  |  |
| 19 | Adjustable gauge | 25 Nos |  |  |  |
| 20 | Hole gauge | 25 Nos |  |  |  |
| 21 | Telescope | 25 Nos |  |  |  |
| 22 | Micrometer | 25 Nos |  |  |  |
| 23 | Vernier caliper | 25 Nos |  |  |  |
| 24 | Bevel protractor | 25 Nos |  |  |  |
| 25 | Vernier bevel protractor | 25 Nos |  |  |  |
| 26 | Dial protractor | 25 Nos |  |  |  |
| 27 | Steel protractor | 25 Nos |  |  |  |
| 28 | Sine bar | 25 Nos |  |  |  |
| 29 | Dial thickness gauge | 25 Nos |  |  |  |
| 30 | Gauge blocks | 25 Nos |  |  |  |
| 31 | Tool makers microscope | 25 Nos |  |  |  |
| 32 | Digital instruments | 25 Nos |  |  |  |
| 33 | Coordinate measuring machines | 01 No |  |  |  |
| 34 | ISO tables of fits and tolerance | 01 No |  |  |  |
| 35 | Measurement tools | 25 Nos |  |  |  |
| 36 | Thread gauge | 25 Nos |  |  |  |
| 37 | Gear tooth caliper | 25 Nos |  |  |  |
| 38 | Gear testing machine | 01 No |  |  |  |
| 39 | Mechanical comparator | 01 No |  |  |  |
| 40 | Electronic comparator | 01 No |  |  |  |
| 41 | Electrical comparator | 01 No |  |  |  |

    - Plastic machine processes

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Measuring tape | 25 Nos | 1 | Copper rod | 05 kg |
| 2 | Personal Protective Equipment. | 25 Nos | 2 | Marker pen and adhesive tape, | 25 Nos |
| 3 | Vernier caliper of appropriate range | 25 Nos | 3 | Circumference tape | 25 Nos |
| 4 | Injection molding machine, | 01 No | 4 | Production record sheets | 25 Nos |
| 5 | Mold and ancillary equipment | 02 Nos | 5 | Pen or pencil for noting records | 25 Nos |
| 6 | Mold lifting equipment | 01 No | 6 | Dyne test marker pen scotch tape to treat | 25 Nos |
| 7 | Machine setting instruments | 05 Nos | 7 | Marker pen surface treatment level or ink | 25 Nos |
| 8 | Parison controller | 01 No |  |  |  |
| 9 | Cutter to separate the items | 05 Nos |  |  |  |
| 10 | Toolbox or tool car containing all necessary tools | 05 Nos |  |  |  |
| 11 | Computer with an installed / designed | 05 Nos |  |  |  |
| 12 | Mold heater package for recording production | 05 Nos |  |  |  |
| 13 | Hot runner nozzle controller (optional) | 05 Nos |  |  |  |
| 14 | Temperature measuring instrument. | 25 Nos |  |  |  |
| 15 | Pocket measuring tape (3m) | 25 Nos |  |  |  |
| 16 | Pre-heaters | 05 Nos |  |  |  |
| 17 | Chiller and cooling towers | 01 No |  |  |  |
| 18 | Water pumps | 05 Nos |  |  |  |
| 19 | Air compressors | 01 No |  |  |  |
| 20 | Material mixers | 01 No |  |  |  |
| 21 | Thickness gauge and feeler gauge | 25 Nos |  |  |  |
| 22 | Precision weighing scale (lab scale) for sample weighing | 25 Nos |  |  |  |
| 23 | Platform scale for product weighing | 01 No |  |  |  |
| 24 | Surface treatment test pen. | 05 Nos |  |  |  |
| 25 | Die lifting equipment. | 01 No |  |  |  |
| 26 | Trimming Knife recording production | 25 Nos |  |  |  |
| 27 | Extrusion Machine, dies | 01 No |  |  |  |
| 28 | Rotational molding machine tools & equipment | 01 No |  |  |  |
| 29 | Material handling equipment. | 01 No |  |  |  |
| 30 | Foam processing machine | 01 No |  |  |  |
| 31 | A computer for recording production | 01 No |  |  |  |
| 32 | Compression molding machine, | 01 No |  |  |  |
| 33 | Compression molding machine tools & equipment | 01 set |  |  |  |
| 34 | Printing machine tools & equipment | 01 Set |  |  |  |
| 35 | Compass | 25 Nos |  |  |  |
| 36 | Printing cylinder mounting tables & tools. | 01 No |  |  |  |
| 37 | Magnifying glass | 25 Nos |  |  |  |

- Members of the Curriculum Development Committee.

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Fazal Rehman | Associate Professor | G.C.T Peshawar KP-TEVTA |
| 2 | Engr. Jamal Akbar | Associate Professor | GPI, Peshawar KP-TEVTA |
| 3 | Engr. Amina Irfan | Lecturer | MS Industrial Engineering Lahore |
| 4 | Engr. Saba Sadiq | Lecturer | University of Lahore Campus Islamabad |
| 5 | Engr. Amir Amin | HOD | Polytechnic College Lahore |
| 6 | Engr. Adil Qadeer | Lecturer | University of Lahore |
| 7 | Engr. Umar Farooq | Instructor | Swedish College Gujrat |
| 8 | Engr. Farooq Iftikhar | Metallurgy engineer | PCSIR Lahore |
| 9 | Engr. Muhammad Arsalan Khan | Lecturer | University of Lahore |
| 10 | Engr. Inayat-ur-Rehman | DACUM, Facilitator | Peshawar |
| 11 | Engr. Muhammad Yasir Khan | Asstt. Director | NAVTTC Islamabad |