Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 1&2 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Measure dimensions with Vernier tools |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 1Hourtime frame (for practical demonstration & assessment): CU1. Take measurement with Vernier caliper. CU2. Take measurement with height gauge. CU3. Take measurement with Vernier depth gauge. |
| Time: 1 Hour. | During a practical assessment, under observation by an assessor, you are required to: |
| Minimum Evidence Required | - Take measurement with Vernier caliper. - Clean the gauge and surface of work piece.. - Place the work piece on surface plate.. - Open the lock screws.. - Slide the jaws up to the width or size of jaws.. - Open slightly larger than the part to be measured.. - Set anvil squarely against reference surface of part. - Maintain the proper pressure on the jaws with screw.. - Lock the movable jaw with knurled screw. - Measure and record the dimensions.. - Take measurement with height gauge. - Clean the gauge and surface of work piece. - Hold the work piece on angle plate.. - Open the lock screws.. - Set the height gauge on stud to check it with fine adjusting screw.. - Lock the measuring head.. - Record the reading.. - Take measurement with Vernier depth gauge. - Clean the gauge and surface of work piece. - Hold the work piece on sample plate.. - Open the lock screws.. - Insert the gauge inside the work piece with fine adjusting screw. - Lock the measuring head. - Record the reading. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 1&2 in Mechatronics Technology |
| Competency Standard | Measure dimensions with Vernier tools |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Take measurement with Vernier caliper - Take measurement with height gauge - Take measurement with Vernier depth gauge |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Clean the gauge and surface of work piece. |  |  |
| - Place the work piece on surface plate. |  |  |
| - Open the lock screws. |  |  |
| - Slide the jaws up to the width or size of jaws. |  |  |
| - Open slightly larger than the part to be measured. |  |  |
| - Set anvil squarely against reference surface of part |  |  |
| - Maintain the proper pressure on the jaws with screw. |  |  |
| - Lock the movable jaw with knurled screw |  |  |
| - Measure and record the dimensions. |  |  |
| - Clean the gauge and surface of work piece |  |  |
| - Hold the work piece on angle plate. |  |  |
| - Open the lock screws. |  |  |
| - Set the height gauge on stud to check it with fine adjusting screw. |  |  |
| - Lock the measuring head. |  |  |
| - Record the reading. |  |  |
| - Clean the gauge and surface of work piece |  |  |
| - Hold the work piece on sample plate. |  |  |
| - Open the lock screws. |  |  |
| - Insert the gauge inside the work piece with fine adjusting screw. |  |  |
| - Lock the measuring head. |  |  |
| - Record the reading |  |  |
| - Clean the gauge and surface of work piece. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 1&2 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Measure dimensions with Vernier tools |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Take measurement with Vernier caliper - Take measurement with height gauge - Take measurement with Vernier depth gauge |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Cleaned the gauge and surface of work piece. |  |  |  |  |  |
|  | Placed the work piece on surface plate. |  |  |  |  |  |
|  | Opened the lock screws. |  |  |  |  |  |
|  | Slides the jaws up to the width or size of jaws. |  |  |  |  |  |
|  | Opened slightly larger than the part to be measured. |  |  |  |  |  |
|  | Set anvil squarely against reference surface of part |  |  |  |  |  |
|  | Maintained the proper pressure on the jaws with screw. |  |  |  |  |  |
|  | Locked the movable jaw with knurled screw |  |  |  |  |  |
|  | Measured and record the dimensions. |  |  |  |  |  |
|  | Hold the work piece on angle plate. |  |  |  |  |  |
|  | Set the height gauge on stud to check it with fine adjusting screw. |  |  |  |  |  |
|  | Locked the measuring head. |  |  |  |  |  |
|  | Insert the gauge inside the work piece with fine adjusting screw. |  |  |  |  |  |
|  | Recorded the reading |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 1&2 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Measure dimensions with Vernier tools |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain measurements by Vernier gauge? |  |  |
|  | Explain Dimensions and Types of dimensions? |  |  |
|  | Explain Measurement techniques? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________