Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 1&2 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Perform measurements through Micrometer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 1.5 Hourtime frame (for practical demonstration & assessment): CU1. Take measurement with outside micro-meter. CU2. Take measurement with inside micrometer. CU3. Take measurement with depth micrometer. CU4. Measure threads with micrometer. CU5. Take measurement with Vernier micrometer. |
| Time: 1.5 Hour. | During a practical assessment, under observation by an assessor, you are required to: |
| Minimum Evidence Required | - Take measurement with outside micro-meter. - Clean the gauge and surface of work piece. - Open slightly larger than the part to be measured. - Set anvil squarely against reference surface of part. - Using ratchet, slowly until it click once. - Record reading and make average. - Take measurement with inside micrometer. - Clean the gauge and surface of work piece.. - Open gauge slightly smaller than the part to be measured. - Set anvil squarely against reference surface of part with ratchet, slowly until it click once.. - Record reading and make average.. - Take measurement with depth micrometer. - Clean the gauge and surface of work piece.. - Access the depth of groove by steel rule. - Insert the suitable insertion rod. - Place the depth micrometer gauge on the deeper groove and take reading.. - Measure threads with micrometer. - Clean the gauge and the threads of work piece.. - Use the suitable set of thread shape tips according to thread size which is to be measured.. - Record the reading.. - Take measurement with Vernier micrometer. - Clean the surface.. - Place specimen on the surface plate. - Take reading accurately.. - Calculate mean reading.. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 1&2 in Mechatronics Technology |
| Competency Standard | Perform measurements through Micrometer |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Take measurement with outside micro-meter - Take measurement with inside micrometer - Take measurement with depth micrometer - Measure threads with micrometer - Take measurement with Vernier micrometer |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Clean the gauge and surface of work piece |  |  |
| - Open slightly larger than the part to be measured |  |  |
| - Set anvil squarely against reference surface of part |  |  |
| - Using ratchet, slowly until it click once |  |  |
| - Record reading and make average |  |  |
| - Clean the gauge and surface of work piece. |  |  |
| - Open gauge slightly smaller than the part to be measured |  |  |
| - Set anvil squarely against reference surface of part with ratchet, slowly until it click once. |  |  |
| - Record reading and make average. |  |  |
| - Clean the gauge and surface of work piece. |  |  |
| - Access the depth of groove by steel rule |  |  |
| - Insert the suitable insertion rod |  |  |
| - Place the depth micrometer gauge on the deeper groove and take reading. |  |  |
| - Clean the gauge and the threads of work piece. |  |  |
| - Use the suitable set of thread shape tips according to thread size which is to be measured. |  |  |
| - Record the reading. |  |  |
| - Clean the surface. |  |  |
| - Place specimen on the surface plate |  |  |
| - Take reading accurately. |  |  |
| - Calculate mean reading. |  |  |
| - Clean the gauge and surface of work piece |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 1&2 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Perform measurements through Micrometer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Take measurement with outside micro-meter - Take measurement with inside micrometer - Take measurement with depth micrometer - Measure threads with micrometer - Take measurement with Vernier micrometer |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Cleaned the gauge and surface of work piece |  |  |  |  |  |
|  | Open slightly larger than the part to be measured |  |  |  |  |  |
|  | Adjusted anvil squarely against reference surface of part |  |  |  |  |  |
|  | Using ratchet, slowly tight the object until it click once |  |  |  |  |  |
|  | Recordedthe observed reading and calculate average |  |  |  |  |  |
|  | Openedthe gauge slightly smaller than the part to be measured |  |  |  |  |  |
|  | Accessed the depth of groove by steel rule |  |  |  |  |  |
|  | Inserted the suitable insertion rod |  |  |  |  |  |
|  | Place the depth micrometer gauge on the deeper groove and take reading. |  |  |  |  |  |
|  | Used suitable thread measuring set according to thread size. |  |  |  |  |  |
|  | Recorded the thread reading on paper. |  |  |  |  |  |
|  | Taken reading accurately. |  |  |  |  |  |
|  | Calculate mean reading. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 1&2 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Perform measurements through Micrometer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain measurements through Micrometer? |  |  |
|  | Explain measurement error in micrometer? |  |  |
|  | Explain least count of given gauge? |  |  |
|  | Describe the types of measurements and its units? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________