National Vocational Certificate Level - 02 in Mechatronics Technology

(Curriculum)

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1. 	Introduction	8

1.1.	Purpose of the training program:	9

1.2.	Overall objectives of training program	9

1.3.	Competencies to be gained after completion of course	10

1.4.	Entry level of trainees	20

1.5.	Minimum qualification for teachers	20

1.6.	Medium of instruction	20

1.7.	Duration of the course:	21

1.8.	Description and structure of the course	22

2.	Overview of the Curriculum for Mechatronics Technology	30

3.	Detail of Modules.	32

3.1. 	Maintain Occupational Health and Safety:	32

3.1.1.	Maintain Health and Safety	32

3.1.2.	Perform safety practices at workplace	37

3.1.3.	Develop professionalism.	41

3.2.	Basic Machining	46

3.2.1.	Perform Metal / Bench Work	46

3.2.2.	Perform general machining	51

3.2.3.	Perform cutting on Metal Circular/Power Heck Saw	54

3.2.4.	Perform tool sharpening	56

3.2.5.	Perform Basic Lathe Machine Operations	59

3.2.6.	Perform Drilling Machine Operations	67

3.2.7.	Perform Shaper, Planar and Slotter Machining Operations	70

3.2.8.	Perform Milling Operations	75

3.3.	Electrical Essentials & Networks	87

3.3.1.	Operate Measuring Instruments.	87

3.3.2.	Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.	90

3.3.3.	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence	93

3.3.4.	Construct Electromagnet to See Various Effects & Verify Faradays Law	96

3.3.5.	Verify Series and Parallel Combination of Capacitors & Determine Break down Voltage of Capacitor	100

3.3.6.	Maintain Tools & Equipment.	102

3.3.7.	Make Cable/Wire Joints	104

3.3.8.	Prepare and Install Distribution Boards	107

3.3.9.	Carryout Basic Electrical Installation	110

3.3.10.	Install Single Phase Electrical Wiring	113

3.3.11.	Perform Testing of Electrical Wiring	118

3.3.12.	Operate Single Phase Transformer	123

3.3.13.	Perform Tests on Single Phase Transformer	126

3.4.	Metrology	129

3.4.1.	Take measurements with graduated tools	129

3.4.2.	Take measurements with combination set	132

3.4.3.	Perform Leveling.	135

3.4.4.	Take measurements with gauges	137

3.4.5.	Perform measurements through Micrometer	139

3.4.6.	Measure dimensions with Vernier tools	142

3.4.7.	Measure angles with angle measuring Instruments.	144

3.4.8.	Measure Threads and Gears	147

3.5.	Basic Engineering Drawing	150

3.5.1.	Perform Basic Manual Drawing.	150

3.5.2.	Construct different Engineering Curves	159

3.5.3.	Construct Multi-View Drawings.	171

3.5.4.	Draw basic drawing using CAD.	180

3.5.5.	Develop 2D using CAD software.	182

3.5.6.	Develop 3D drawing using CAD software.	184

3.6.	Welding	187

3.6.1.	Interpret Part and Assembly Drawing	187

3.6.2.	Evaluate the Use of Hand and Power Tools	190

3.6.3.	Perform Pre-Welding Operations	193

3.6.4.	Perform Oxy Acetylene Welding	196

3.6.5.	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)	198

3.6.6.	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)	202

3.6.7.	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	205

3.6.8.	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	212

3.6.9.	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	219

3.6.10.	Flux Cored Arc Welding (FCAW)	226

3.6.11.	Perform Soldering and Brazing Operations	230

3.7.	Microprocessor and Microcontroller	232

3.7.1.	Microprocessor applications	232

3.7.2.	Identify Microcontroller Types and its Architecture	234

3.7.3.	Interface Microcontroller with system	236

3.7.4.	Carryout various Microcontroller Applications	239

3.7.5.	Apply Microcontroller to build Control System	242

3.8.	Analog Electronics	244

3.8.1.	Identify Basic Electronics Components	244

3.8.2.	Design Rectifier using Diode	247

3.8.3.	Carry out Diode Application	249

3.8.4.	Implement Bipolar Junction Transistor (BJT) in Different Applications	250

3.8.5.	Implement Field Effect Transistor (FET) in Different Applications	255

3.8.6.	Implement Thyristor Family in Various Application	258

3.8.7.	Application of Operational Amplifier	260

3.9.	Computer Programming	263

3.9.1.	Installation of C++, IDE for Arduino and Python	263

3.9.2.	Carryout Basic C++ Programming	266

3.9.3.	Carryout Basic Programming using python	269

3.9.4.	Perform basic programming using microcontrollers	273

3.10.	Computer Integrated Machines	276

3.10.1.	Perform CNC Machine Operations	276

3.10.2.	Perform CNC Lathe Operations	279

3.10.3.	Perform CNC Milling Operations	286

3.11.	Machines and Mechanism	293

3.11.1.	Demonstrate Simple Mechanisms	293

3.11.2.	Perform Kinematic Analysis of Simple Mechanisms	296

3.11.3.	Demonstrate Mechanical Power Transmission Elements.	298

3.11.4.	Perform Kinematic Analysis of Mechanical Power Transmission Elements.	300

3.12.	AC & DC machines	303

3.12.1.	Verify Basic Laws of Electrical Machines.	303

3.12.2.	Analyze an Alternator	309

3.12.3.	Analyze Single Phase Motors	313

3.12.4.	Analyze Special Purpose Motors	316

3.12.5.	Analyze Three Phase Motors.	319

3.12.6.	Analyze Dc Generators	330

3.12.7.	Perform Tests on DC Generators	335

3.12.8.	Analyze Dc Motors.	340

3.12.9.	Perform Tests On DC Motors.	344

3.13.	Digital Electronics	353

3.13.1.	Verify Truth Tables of Digital Gates	353

3.13.2.	Construct & Verify Combinational Logic Circuit	357

3.13.3.	Construct and Verify Function of Flip Flops	361

3.13.4.	Construct Multi vibrator with 555 IC	363

3.13.5.	Construct Shift Registers and Counters with the Help of Flip Flops	365

3.14.	Sensor & Actuators	367

3.14.1.	Test Sensors and Transducers.	367

3.14.2.	Calibrate Sensors and Transducers.	376

3.14.3.	Interface Sensors and Transducers.	394

3.14.4.	Test Actuators	399

3.14.5.	Interface and control Actuator with sensors.	404

3.15.	Programmable logic controllers (PLCs)	409

3.15.1.	Install PLC software and Simulator	409

3.15.2.	Interface PLC with system (1)	416

3.15.3.	Program PLC using Ladder Logic	418

3.15.4.	Commissioning and Testing PLC	422

3.15.5.	Commissioning and Testing PLC	427

3.15.6.	Carry out Industrial Automation on PLC	430

3.15.7.	Interface PLC with system (2)	434

3.16.	Designing of Mechanical Members	438

3.16.1.	Calculate Stresses in Machine Parts	438

3.16.2.	Calculate diameter of cylinder for hoop and longitudinal stresses	441

3.16.3.	Calculate thickness and diameter of spherical shell for circumferential stresses	443

3.16.4.	Design welded joints for transverse and parallel fillet under static and fatigue loading	447

3.16.5.	Calculate stresses due to initial tightening and external load on screws	454

3.16.6.	Design dimension of square and rectangular keys	458

3.16.7.	Design shafts subjected to twisting moment	462

3.16.8.	Design flange coupling for specific torque	469

3.17.	Industrial Process Control & Instrumentation	472

3.17.1.	Design & Implement a process control	472

3.17.2.	Install Transducer & Transmitters used in Industrial Applications	475

3.17.3.	Install the process Regulators	478

3.17.4.	Install the Variable Frequency Drive (VFD)	481

3.17.5.	Install Pneumatic & Hydraulic systems	483

3.18.	Robotics	486

3.18.1.	Identify Various Parts of Robots	486

3.18.2.	Identify Various Parts of Robot Arm	490

3.18.3.	Develop Robot program	493

3.18.4.	Develop robotic arm structure	496

3.18.5.	Test the Robot	499

3.19.	Mechatronic Projects	502

3.19.1.	Access Control System using RFID Reader RC522	502

3.19.2.	Finger Print Sensor Based Biometric Attendance System	504

3.19.3.	Humidity and Temperature Monitoring System	506

3.19.4.	Smart Stick for Blinds	508

3.19.5.	Perform Line Following Robot	510

3.19.6.	Perform Smart Phone Control Robot Using Bluetooth	512

3.19.7.	Perform IoT Controlled based home automation system	515

3.19.8.	Perform Smart Phone Control Door Lock System	518

3.19.9.	Perform Security System Using Motion Detection	521

3.19.10.	Perform Water-level Detection in Tank	523

3.19.11.	Perform Mini CNC 2D Plotter	525

3.19.12.	Perform Voice Controlled Robot	527

3.20.	Hydraulics & Pneumatics	529

3.20.1.	Operate Hydraulic Bench and Its Functions	529

3.20.2.	Calibrate Bourdon Tube and Diaphragm Pressure Gauge	532

3.20.3.	Operate Hydraulic Press	534

3.20.4.	Verify Bernoulli’s Equation	537

3.20.5.	Analyze the performance of Pumps	540

3.20.6.	Construct circuit for Double Acting Hydraulic Cylinder	542

3.20.7.	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder	545

3.20.8.	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking	547

3.20.9.	Set Speed and Direction of Hydraulic Motor	552

3.20.10.	Operate Pressure Measuring Instruments	554

3.20.11.	Operate Flow Regulating Devices	560

4.	List of Personal Protective Equipment	565

5. Complete List of Tools, Equipment, Machines and Consumables	566

# 1. 	Introduction

Mechatronics is an interdisciplinary field that combines elements of engineering, machining, information technology, robotics, and electronics. It synchronize these diverse fields for the purpose of designing, fabrication and field testing of the production lines and manufacturing processes in order to optimize their performance and efficiencies. In this field several mechanical systems such as hydraulic, pneumatic, camera, electric motor and engines are interface, configure and combine in order to perform the tasks with increase efficiency and performance. Since several different mechanical, electrical and optical systems are inference, the continual operational maintenance, configuration and upgradation is required which need trained main power in the field of mechatronics in order to run the industrial processes and machines with any system failure or breakdown.

Typically, a mechatronics technologist need to perform several task such as standalone operation / testing of the mechanical and electronics systems in order to diagnose the faulty parts. These task may attribute to the checking, installation and configuration of electronic circuits, motor drives, hydraulics/pneumatic moving system. The industry is lacking the expert technologist who has sound knowledge and command over all mechanical and electronics systems especially when these are interface and working in the close loop environment.

Mechatronics technologists can work in several fields, but they are commonly found in the manufacturing and automation sector. Several industrial process and production lines can be optimize by implementing fine mechanical precision in environmental conditions that may be dangerous or inefficient  for human hands and can be perform optimally by adding mechatronic solutions.Mechatronics has had a measurable impact both on the speed with which new products can be assembled.

The professionals that are skilled in this area of mechatronics are in high demand on production lines that range from projects as large as automotive vehicle manufacturing to the smallest microelectronic products. For instance, blood pressure monitoring device has pneumatic pump, a sensor to sense the blood pressure and electronics / microprocessor based circuit to measure the blood pressure. Similarly, several home appliances such as washing machines, ovens and refrigerators are now equipped with sophisticated mechatronics systems in order to optimize their products in terms of performance and electrical power consumption. Therefore, to repair and maintain such home appliances, the traditional technician are fail to work because of their training explicitly in the field of mechanical, electrical or electronics. Being conscious of these facts, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechatronics Technology under National Vocational Qualifications Framework (NVQF). Followings are the competency standards which are   developed by the Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Mechatronics industries from public and private sector, Engineering Universities and Technical Education of the country.

  - Purpose of the training program:

The purpose of thetraining is to set the high professional standards for the Mechatronics technological sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of the skills attained through formal or informal pathways

- Improve the quality and effectiveness of the training and assessment for Mechatronics Technological sector

- Enabling / helping / facilitating the existing workforce to indulge themselves in new technologies and methods

## Competencies to be gained after completion of course

Detail of the competency standards included in qualification are given below:

Health and Safety

0714E&A1.	Maintain Health and Safety

0714E&A2.	Perform safety practices at work place

0714E&A3.	Develop Professionalism

Basic Machining

0714E&A4.	Perform Metal / Bench Work

0714E&A5	Perform general machining

0714E&A6	Perform cutting on Metal Circular/Power Heck Saw

0714E&A7	Perform tool sharpening

0714E&A8	Perform Basic Lathe Machine Operations

0714E&A9	Perform Drilling Machine Operations

0714E&A10	Perform Shaper, Planar and Slotter Machining Operations

0714E&A11	Perform Milling Operations

Electrical Essentials & Networks

0714E&A12	Operate Measuring Instruments

0714E&A13	Verify Ohm’s & Kirchhoff’s Laws by Implementing Series/Parallel Circuits

0714E&A14	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

0714E&A15	Construct Electromagnet to See Various Effects & Verify Faradays Law.

0714E&A16 Verify Law of Combination of Capacitor & Determine Break Down Voltage of Capacitor

0714E&A17	Maintain Tools & Equipment

0714E&A18	Make Cable/Wire Joints

0714E&A19	Prepare and Install Distribution Boards

0714E&A20	Carryout Basic Electrical Installations

0714E&A21	Install Single Phase Electrical Wiring

0714E&A22	Perform Testing of Electrical Wiring

0714E&A23	Operate Single Phase Transformer

0714E&A24	Perform Tests on Single Phase Transformer

Digital Skills

0714E&A25	Install computer operating systems and hardware

0714E&A26	Operate digital media technology

0714E&A27	Perform computer operations

0714E&A28	Use computer applications

0714E&A29	Create user documentation

0714E&A30	Create technical documentation

0714E&A31	Create basic databases

0714E&A32	Use social media tools for collaboration and engagement

0714E&A33	E-Commerce- SEO (Search Engine Optimization)

0714E&A34	E-Commerce- SCM (Supply Chain Management)

0714E&A35	E-Commerce- Social Media Marketing

0714E&A36	Use digital devices

0714E&A37	Operate word-processing applications

0714E&A38	Operate spreadsheet applications

0714E&A39	Operate presentation packages

0714E&A40	Perform writing and editing tasks

Metrology

0714E&A41	Take measurements with graduated tools

0714E&A42	Take measurements with combination set

0714E&A43	Perform Levelling

0714E&A44	Take measurements with gauges

0714E&A45	Perform measurements through Micrometer

0714E&A46	Measure dimensions with Vernier tools

0714E&A47	Measure angles with angle measuring Instruments

0714E&A48	Measure Threads and Gears

Basic Engineering Drawings

0714E&A49	Perform Basic Manual Drawing

0714E&A50	Construct different Engineering Curves

0714E&A51	Construct multi-view drawings

0714E&A52	Develop 2D CAD drawings

0714E&A53	Develop 3D CAD drawing

Welding

0714E&A54	Interpret Part and Assembly Drawing

0714E&A55	Evaluate the Use of Hand and Power Tools

0714E&A56	Perform Pre-Welding Operations

0714E&A57	Perform Oxy Acetylene Welding

0714E&A58	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)

0714E&A59	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)

0714E&A60	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A61	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A62	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A63	Flux Cored Arc Welding (FCAW)

0714E&A64	Perform Soldering and Brazing Operations

Microprocessor and Microcontroller

0714E&A65.Design Microprocessor applications

0714E&A66	Identify Microcontroller Types and its Architecture

0714E&A67	Interface Microcontroller with system

0714E&A68	Carryout various Microcontroller Applications

0714E&A69	Apply Microcontroller to build Control System

Analogue Electronics

0714E&A70	Identify Basic Electronics Components

0714E&A71	Design a Rectifier Using Diode

0714E&A72	Carry Out Diode Application

0714E&A73	Implement Bipolar Junction Transistor (BJT) in Different Applications

0714E&A74	Implement Field Effect Transistors (FET) In Different Applications

0714E&A75	Implement (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in various application

0714E&A76	Design Operation Amplifier

Soft Skills

0714E&A77.	Develop workplace policy and procedures for sustainability

0714E&A78.	Manage meetings

0714E&A79.	Manage recruitment selection and induction processes

0714E&A80.	Manage personal work priorities and professional development

0714E&A81.	Manage workforce planning

0714E&A82.	Undertake project work

0714E&A83.	Identify and communicate trends in career development

0714E&A84.	Apply specialist interpersonal and counseling interview skills

0714E&A85.	Work safely in an office environment

0714E&A86.	Develop workplace documents

0714E&A87.	Prepare and implement negotiation

0714E&A88.	Maintain professionalism in the workplace

0714E&A89.	Organize schedules

Computer Programming

0714E&A90.	Installation of C++, IDE for Arduino and Python

0714E&A91.	Carryout Basic C++ Programming

0714E&A92.	Carryout Basic Programming using python

0714E&A93.	Perform basic programming using microcontrollers

Computer Integrated Manufacturing (CIM)

0714E&A94.	Perform CNC Machine Operations

0714E&A95.	Perform CNC Lathe Operations

0714E&A96.	Perform CNC Milling Operations

Machines and Mechanism

0714E&A97.	Demonstrate Simple Mechanisms

0714E&A98.	Perform Kinematic Analysis of Simple Mechanisms

0714E&A99.	Demonstrate Mechanical Power Transmission Elements

0714E&A100.	Perform Kinematic Analysis of Mechanical Power Transmission Elements

AC & DC Machines

0714E&A101.	Verify Basic Laws of Electrical Machines

0714E&A102.	Analyse an Alternator

0714E&A103.	Analyse Single Phase Motors

0714E&A104.	Analyse Special Purpose Motors

0714E&A105.	Analyse Three Phase Motors.

0714E&A106.	Analyze Dc Generators

0714E&A107.	Perform Tests on DC Generators

0714E&A108.	Analyze Dc Motors.

0714E&A109.	Perform Tests On DC Motors.

Digital Electronics

0714E&A110.	Verify Truth Tables of Digital Gates

0714E&A111.	Construct & Verify Combinational Logic Circuit

0714E&A112.	Construct and Verify Function of Flip Flops

0714E&A113.	Construct Multivibrator with 555 IC

0714E&A114.	Construct Shift Registers and Counters with the Help of Flip Flops

Sensors and Actuators

0714E&A115.	Test Sensors and Transducers

0714E&A116	Calibrate Sensors and Transducers

0714E&A117	Interface the Sensor & Transducers

0714E&A118	Test Actuators

0714E&A119	Interface and control Actuator with sensors

Programmable Logic Controllers (PLCS)

0714E&A120	Install PLC software and Simulator

0714E&A121	Interface PLC with system

0714E&A122	Program PLC using Ladder Logic

0714E&A123	Design and Test the PLC

0714E&A124	Carry out Industrial Automation on PLC

Designing of Mechanical Members

0714E&A125	Calculate Stresses in Machine Parts

0714E&A126	Calculate diameter of cylinder for hoop and longitudinal stresses

0714E&A127	Calculate thickness and diameter of spherical shell for circumferential stresses

0714E&A128	Design welded joints for transverse and parallel fillet under static and fatigue loading

0714E&A129	Calculate stresses due to initial tightening and external load on screws

0714E&A130	Design dimension of square and rectangular keys

0714E&A131	Design shafts subjected to twisting moment only

0714E&A132	Design flange coupling for specific torque

Industrial Process Control & Instrumentation

0714E&A133	Design & Implement a process control

0714E&A134	Install Transducer & Transmitters used in Industrial Applications

0714E&A135	Install the process Regulators

0714E&A136	Install the Variable Frequency Drive (VFD)

0714E&A137	Install Pneumatic & Hydraulic systems

Robotics

0714E&A138	Identify Various Parts of Robots

0714E&A139	Identify Various Parts of Robot Arm

0714E&A140	Develop Robot program

0714E&A141	Develop robotic arm structure

0714E&A142	Test the Robot

Mechatronic Projects

0714E&A143	Access Control System using RFID Reader RC522

0714E&A144	Finger print Sensor Based Biometric Attendance System

0714E&A145	Humidity and Temperature Monitoring System

0714E&A146	Smart Stick for Blinds

0714E&A147	Perform Line Following Robot

0714E&A148	Perform Smart Phone Control Robot Using Bluetooth

0714E&A149	Perform IoT Controlled based home automation system

0714E&A150	Perform Smart Phone Control Door Lock System

0714E&A151	Perform Security System Using Motion Detection

0714E&A152	Perform Water-level Detection in Tank

0714E&A153	Perform Mini CNC 2D Plotter

0714E&A154	Perform Voice Controlled Robot

Hydraulic and Pneumatics

0714E&A155	Operate Hydraulic Bench and Its Functions

0714E&A156	Calibrate Bourdon Tube and Diaphragm Pressure Gauge

0714E&A157	Operate Hydraulic Press

0714E&A158	Verify Bernoulli’s Equation

0714E&A159	Analyse the performance of Pumps

0714E&A160	Construct circuit for Double Acting Hydraulic Cylinder

0714E&A161	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder

0714E&A162	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking

0714E&A163	Set Speed and Direction of Hydraulic Motor

0714E&A164	Operate Pressure Measuring Instruments

0714E&A165	Operate Flow Regulating Devices

Entrepreneurial Skill

0714E&A166	Develop Entrepreneurial Skills

0714E&A167	Apply project information management and communications techniques

0714E&A168	Apply project human resources management approaches

0714E&A169	Direct human resources management of a project program

0714E&A170	Develop a project management plan

0714E&A171	Maintain business resources

0714E&A172	Develop a sales plan

0714E&A173	Plan and implement business-to-business marketing

0714E&A174	Address customer needs

0714E&A175	Manage personal finances

0714E&A176	Solve problems which jeopardize safety and security

0714E&A177	Coordinate a work team

0714E&A178	Lead small teams

0714E&A179	Plan and organize work

0714E&A180	Develop teams and individuals

0714E&A181	Apply problem solving techniques in the workplace using critical thinking

0714E&A182	Manage human resource services

  - Entry level of trainees

The entry for D.A. E National Certificate level 5, in Mechatronics Technology are

- A person having National Vocational Certificate level 4, in Mechatronics Technology.

- A person having Matric certificate with Science subjects

## Minimum qualification for teachers

- B.Sc. / B.E. in Mechatronics Engineering.

- 4 Years B. Tech in Mechatronics Technology with 3 Years teaching experience.

- D. A. E. in Mechatronics Technology with 7 Years teaching experience

- Must be able to communicate effectively

## Medium of instruction

English, and Urdu.

## Duration of the course:

The proposed curriculum is composed of 182 modules that will be covered in 4,927Learning hours. Duration of the course is proposed to be Three years.

Level - 2 = 6 months.

Level - 3 = 6 months

Level - 4 = 1 Year

Level - 5 = 1 Year

The distribution of contact hours is given below:

Total.		4927 hours.

Theory.	1347hours (27.34 %)

Practical.	3580 hours (72.66 %)

## Description and structure of the course

Following is the structure of the course:

| LEVEL 2 MECHATRONICS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Competency Standard |  | Category | Contact hr. |  |  | Cr. Hr. |
|  |  |  | Th. | Pr. | Total |  |
| Maintain Occupational Health and Safety | 2 | Generic | 1 | 27 | 28 | 2.8 |
| Adopt Safety Regulations, Labor Protection Laws, Environmental Protection Laws at Workplace | 2 | Generic | 1 | 27 | 28 | 2.8 |
| Perform Metal/Bench Work | 2 | Technical | 7 | 21 | 28 | 2.8 |
| Perform general machining | 2 | Technical | 4 | 12 | 16 | 1.6 |
| Perform cutting on Metal Circular/Power Heck Saw | 2 | Technical | 2 | 12 | 14 | 1.4 |
| Perform tool sharpening | 2 | Technical | 2 | 12 | 14 | 1.4 |
| Perform Basic Lathe Machine Operations | 3 | Technical | 4 | 36 | 40 | 4 |
| Perform Drilling Machine Operations | 2 | Technical | 3 | 18 | 21 | 2.1 |
| Perform Shaper, Planar and Slotter Machining Operations | 2 | Technical | 7 | 21 | 28 | 2.8 |
| Perform Milling Operations | 2 | Technical | 12 | 36 | 48 | 4.8 |
| Take measurements with graduated tools | 2 | Technical | 4 | 12 | 16 | 1.6 |
| Take measurements with combination set | 2 | Technical | 3 | 18 | 21 | 2.1 |
| Perform ling | 2 | Technical | 4 | 12 | 16 | 1.6 |
| Take measurements with gauges | 2 | Technical | 4 | 12 | 16 | 1.6 |
| Perform measurements through Micrometer | 2 | Technical | 5 | 10 | 15 | 1.5 |
| Measure dimensions with Vernier tools | 2 | Technical | 6 | 12 | 18 | 1.8 |
| Measure angles with angle measuring Instruments | 2 | Technical | 5 | 10 | 15 | 1.5 |
| Measure Threads and Gears | 2 | Technical | 6 | 12 | 18 | 1.8 |
| Operate Measuring Instruments | 3 | Technical | 6 | 24 | 30 | 3 |
| Verify Ohm’s & Kirchhoff’s Laws by Implementing Series/Parallel Circuits | 3 | Technical | 7 | 21 | 28 | 2.8 |
| Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence | 3 | Technical | 7 | 21 | 28 | 2.8 |
| Construct Electromagnet to See Various Effects & Verify Faradays Law | 3 | Technical | 7 | 21 | 28 | 2.8 |
| Verify Law of Combination of Capacitor & Determine Break Down Voltage of Capacitor | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Maintain Tools & Equipment | 3 | Technical | 5 | 10 | 15 | 1.5 |
| Make Cable/Wire Joints | 3 | Technical | 5 | 10 | 15 | 1.5 |
| Prepare and Install Distribution Boards | 3 | Technical | 3 | 18 | 21 | 2.1 |
| Carryout Basic Electrical Installations | 3 | Technical | 6 | 15 | 21 | 2.1 |
| Install Single Phase Electrical Wiring | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Perform Testing of Electrical Wiring | 3 | Technical | 3 | 18 | 21 | 2.1 |
| Operate Single Phase Transformer | 3 | Technical | 3 | 18 | 21 | 2.1 |
| Perform Tests on Single Phase Transformer | 3 | Technical | 5 | 10 | 15 | 1.5 |
| Total |  |  | 149 | 542 | 691 | 69.1 |
| Percentage |  |  | 21.563 | 78.437 |  |  |

# Overview of the Curriculum for Mechatronics Technology

| Module Title and Aim | Learning Units | Theory Days/hours | Workplace Days/hours | Timeframe of modules |
| --- | --- | --- | --- | --- |

- Detail of Modules.

## 3.1. Maintain Occupational Health and Safety:

### @Maintain Health and Safety

Objectives:This module covers the skills and knowledge required to ensure safety of machine, knowledge of safety alarm in robotic working area, personal protective equipment (PPE), Maintain First-aid Box, Maintain Fire protection and Machines safety.

Duration:  28 Hours						Theory: 1 Hours						Practical:27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Ensure safeguarding devices Off condition | - Check the alarm or indicator or devices which represent the Human Presence in the Robotic working area. - Check and test the working of mechanical limits and Robotic working area. - Check and test the fixed barrier devices, which prevent contact with moving parts of Robot. - Check and test the interlock barrier and guards during off load. - Analyze and interpret the data. - Report the results. | - Explain how to check the safeguard devices. - Describe critical check points of safeguard devices. - Knowledge and understanding of possible risk to maintain cleanliness and safety. Practical Activity: Make a workshop layout and mark safety devices in this layout. | Total: 4 Hrs Theory:1 Hrs Practice:3 Hrs | - Safety alarm - Barrier - Electrician tool kit - Flat screw driver set - General tools kit - Insulation tester - Safe guard device - Safety goggles - Safety harness belt - Safety helmet - Safety mask - Safety Shoes - Insulation tape - Teflon tape - Different tags and locks - maintenance manuals - Logbook - Handbooks | Class Room Training Workshop. Lab/ Field Visit. |
| - Ensure Safeguarding devices in on condition | - Locate and distinguish the various types’ areas, i.e. Supervisor area, operator area, user area and restricted area for area labeling. - Check and observe the absence of human presence in the restricted area with the naked eye and verify with the Human presence sensor output. - Check and test the starting parameter for starting Robot, i.e. input power, frequency, lubrication, etc. - Check and test working of the Emergency braking mechanism, while emergency power stop is not recommended. - Analyse and interpret the data. - Report the results. | - Define Guarding methods. - Explain how to check the safeguard devices. - Describe critical check points of safeguard devices. - Knowledge and understanding of possible risk to maintain cleanliness and safety. Practical Activity: Make a safety audit action plan and check sheet. | Total: 3 Hrs Theory:0 Hrs Practice:3 Hrs | - Safety alarm - Barrier - Electrician tool kit - Flat screw driver set - General tools kit - Insulation tester - Safe guard device - Safety goggles - Safety harness belt - Safety helmet - Safety mask - Safety Shoes - Cotton gloves - Cotton waste - Different tags and locks - Equipment maintenance manuals - Logbook - Handbooks - Design books/ sheets | Class Room Training Workshop. Lab/ Field Visit. |
| - Ensure safety devices during ON load condition | - Check and test the ON load sensing devices, interlocks, ON/OFF load torque and feedback mechanism. - Compare the on load and offload data. - Analyze and interpret data. - Report the results. | - Explain how to check the safeguard devices. - Define Guarding methods before the load condition. - Describe critical check points of safeguard devices. - Knowledge and understanding of possible risk to maintain cleanliness and safety. Practical Activity: Make a list of check points, which are to be verifying before a loading on devices. | Total: 3 Hrs Theory:0 Hrs Practice:3 Hrs | - Load sensing device. - interlocks - Electrician tool kit - General tools kit - Insulation tester - Safe guard device - Safety goggles - Safety harness belt - Safety helmet - Safety mask - Safety Shoes | Class Room Training Workshop. Lab/ Field Visit. |
| - Ensure Personal Protective Equipment (PPE). | - Arrange personal protective equipment as per requirements - Wear correct personal protective equipment - Store PPE at appropriate place after use. | - Explain why we need (PPEs) Personal protective equipment. - Describe the application of PPEs. - Knowledge and understanding of possible risk to maintain cleanliness and safety. Practical Activity: - Identify PPEs. - Identify PPEs according to the work environment/requirement or condition. | Total: 3 Hrs Theory:0 Hrs Practice:3 Hrs | - Personal Protective Equipment - Cotton gloves - Safety goggles - Safety harness belt - Safety helmet - Safety mask - Safety Shoes | Class Room Training Workshop. Lab/ Field Visit. |
| - Maintain First-aid Box | - Ensure availability of first aid box - Check first aid box for requisite emergency - Check expiry of medicines - Perform first aid treatment against electric shocks - Perform first aid treatment/bandages against minor injuries. | - Describe First-Aid-Box. - Define periodic checkups of First Aid Box. - Knowledge about the necessary steps to be taken in case of accident or incident. Practical Activity: Make a list of First Aid Box items. | Total: 3 Hrs Theory:0 Hrs Practice:3 Hrs | - First Aid Box | Class Room Training Workshop. Lab/ Field Visit. |
| - Maintain Fire Extinguisher | - Check expiry of fire extinguisher. - Operate fire extinguisher. - Replace fire extinguisher. - Ensure that the fire brigade is at stand by(for major emergency). | - Describe types of fire. - Define the PASS technique for using fire extinguisher. - Define application of different types of fire extinguishers. - Define periodic checkups of Fire extinguisher. - Knowledge about the necessary steps to be taken in case of accident or incident. Practical Activity: Demonstrate the use of fire extinguisher. | Total: 6 Hrs Theory:0 Hrs Practice:6 Hrs | - Fire Extinguisher - Fire Bucket - Fire Blanket | Class Room Training Workshop. Lab/ Field Visit. |
| - Ensure Safeguard of Machines | - Maintain radiator shield - Maintain alternator fan shield - Maintain heat resister material on silencer - Cover main circuit breaker - Lock canopy doors | - Define Guarding methods. - Describe types of safe guards. - Explain how to check the safeguard devices. - Describe critical check points of safeguard devices. - Knowledge and understanding of possible risk to maintain cleanliness and safety. Practical Activity: Demonstrate the safeguarding of any available machine. | Total: 6 Hrs Theory:0 Hrs Practice:6 Hrs | - Fix Safe guards - Moveable safe guards. - safety Tape | Class Room Training Workshop. Lab/ Field Visit. |

### @Adopt Safety Regulations, Labor Laws at workplace

Objectives:This competency standard covers the skills and knowledge required to ensure safety of machine, knowledge of safety alarm in robotic working area, personal protective equipment (PPE), Maintain First-aid Box, Maintain Fire protection and Machines safety.

Duration:  28 Hours					Theory:  1Hours						Practical: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adopt company policies and procedures | - Ensure company’s safety policy. - Adopt company safety procedure. - Advocate worker with company safety policy. - Implement Safety sign board as per standard. | - Describe what the safety policy matters are. - Explain different safety signs and precautions in the work place. Practical Activity: - Prescribe safety sign according to work and work place requirements. - Perform advocacy at the work safety requirements. | Total: 04Hrs Theory: 01Hrs Practice:03 Hrs | - PPEs - Safety shoes - Helmet - Safety gloves - Safety cloth - First Aid kits - Fire extinguisher - Fire blanket - Safety Tape | Class Room Training Workshop. Lab/ Field Visit. |
| - Attain health & safety training | - Take required health and safety training. - Implement work hazardous material information system (WHMIS). - Adopt first aid cardio respiratory, resuscitation and CPR. | - Explain different safety signs and precautions in the work place. - Knowledge and understanding of possible risk to identify hazardous. - Describe CPR techniques. Practical Activity: - Perform CPR. - Identify hazardous material at work place. | Total: 06 Hrs Theory: 0 Hrs Practice:06 Hrs | - Safety shoes - Helmet - Safety gloves - Safety cloth - First Aid kits - Fire extinguisher - Fire blanket - PPEs - Safety Tape | Class Room Training Workshop. Lab/ Field Visit. |
| - Prepare for emergencies | - Check Take emergency response training. - Ensure practice of emergency exercises. - Check the emergency alarms. - Ensure regular practice of gathering the workers in assembly area during the emergency. | - Explain safety exit requirements. - Describe safety steps in account of emergencies. - Describe types of safety alarms and their inspection. Practical Activity: - Make a workshop layout and mark safety devices in this layout. - Make a safety audit action plan and check sheet. | Total: 06Hrs Theory: 0Hrs Practical:06Hrs | - Safety shoes - Helmet - Safety gloves - Safety cloth - First Aid kits - Fire extinguisher - Fire blanket | Class Room Training Workshop. Lab/ Field Visit. |
| - Respond to emergencies | - Arrange Follow emergency plan - Communicate instructions to co workers - Assess risk and determine course of action - Operate emergency equipment and supplies - Ensure that the ambulance is at stand by (for emergency) | - Explain the safety precautions in the workshop. - Describe safety precautionary steps in case of emergencies. - Knowledge to operate emergency response equipment i.e. Fire extinguisher etc. Practical Activity: - Make anemergency plan. - Demonstrate to operate fire extinguisher. | Total: 03Hrs Theory: 0Hrs Practical:03Hrs | - Safety shoes - Helmet - Safety gloves - Safety cloth - First Aid kits - Fire extinguisher - Fire blanket | Class Room Training Workshop. Lab/ Field Visit. |
| - CU5. Maintain First-aid Box | - Ensure availability of first aid box - Check first aid box for requisite emergency - Check expiry of medicines - Perform first aid treatment against electric shocks - Perform first aid treatment/bandages against minor injuries. | - Explain why we need First Aid Box at work place. - Describe the items contain in First Aid Box. - Knowledge about preliminary first aid treatment. Practical Activity: - Demonstrate preliminary first aid treatment. - Make a first aid box items list. | Total: 03Hrs Theory: 0Hrs Practical:03Hrs | - First Aid Box | Class Room Training Workshop. Lab/ Field Visit. |
| - Maintain Fire Extinguisher | - Check expiry of fire extinguisher. - Operate fire extinguisher. - Replace fire extinguisher. - Ensure that the fire brigade is at stand by(for major emergency). | - Knowledge to operate emergency response equipment i.e. Fire extinguisher etc. Practical Activity: - Demonstrate to operate fire extinguisher. | Total: 03Hrs Theory: 0Hrs Practical:03Hrs | - Fire extinguisher | Class Room Training Workshop. Lab/ Field Visit. |
| - Ensure Safeguard of Machines | - Maintain radiator shield - Maintain alternator fan shield - Maintain heat resister material on silencer - Cover main circuit breaker - Lock canopy doors | - Identify the safety precautions in the workshop. Practical Activity: - Make a workshop layout and mark safety devices in this layout. - Make a safety audit action plan and check sheet. | Total: 03 Hrs Theory: 0Hrs Practical:03Hrs | - Fix Safe guards - Moveable Safe guards | Class Room Training Workshop. Lab/ Field Visit. |

    - @Perform Metal / Bench Work

Objective: This competency standard covers the skills and knowledge required to Develop Name Plate manually, Carry out Sawing, Prepare Inside Calliper, Prepare Bottle Opener, Prepare Dovetail Joint, Prepare Tri Square (small size), ,Cut Threads on Work Piece, Prepare Funnel, Prepare Drawer Handle, Cut Pipe Threads and Prepare spanner (small size).

Duration:  28 Hours					Theory: 7 Hours					Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop NamePlate manually | Trainee will be able to: - Select the marking tools - Hold the sheet in vice. - Cut the sheet as per given drawing - Perform surface finishing with file - Level the surface with tri-square - Mark the plate as per name requirements - Punch the marked area - Perform finishing with sand paper | - Introduction to basic measurement - Introduction to measuring tools - Introduction to marking tools. - Introduction to cutting tools - Clamping / holding methods. | Theory:1 Hrs Practice:3 Hrs Total: 4 Hrs |  | Class Room and workshop |
| - Prepare DovetailJoint | Trainee will be able to: - Select marking tools - Cut sheet as per drawing - Perform surface finishing with file - Level the surface of both work pieces with tri-square - Mark both work pieces according to drawing - Create outer notch on work piece using flat file and hacksaw - Create inner notch using hacksaw and chisel - Compare both pieces by inserting outer notch into inner notch - Perform the finishing with the sand paper | • Types of chisels • Chiseling process • Understanding of types of fits. | Theory:1 Hrs Practice:3 Hrs Total: 4 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/ alphabet punch Round file Metal working chisel Punching tools Tap set Tap handle Pipe vice | Class Room and workshop |
| - Prepare BottleOpener | Trainee will be able to: - Select marking tools - Cut sheet as per drawing - Perform surface finishing with file - Level the surface with tri-square - Mark radius as per drawing - Develop radius as per drawing - Make the notch with round file - Perform finishing with sand paper | - Knowledge of radius gauge - Describe use of round file - Knowledge of different kind of taps & die according to requirement | Theory- 1 Hrs Practical-3 Hrs Total- 4 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/ alphabet punch Round file Metal working chisel Punching toolsTap set Tap handle Pipe vice | Class Room and workshop |
| - Cut Threads onWork Piece withtap and die | Trainee will be able to - Identify different kind of taps & die according to requirement - Identify the work piece clamping method. - Apply the tap and die alignment. - Apply the lubricants while threading. - Avoid the unwanted engraving and slips. - Identify the proper threading procedure | - Knowledge of calculation for drill size for internal threading | Theory:1 Hrs Practice:3 Hrs Total: 4 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/ alphabet punch Round file Metal working chisel Punching toolsTap set Tap handle Pipe vice | Class Room and workshop |
| - Cut Pipe Threads | Trainee will be able to - Select marking tools - Cut pipe as per drawing - Select die as per pipe size - Set die into die holder - Select relevant vice for pipe clamping - Perform pipe threading using appropriate method - Perform finishing with sand paper | - Knowledge about threading by die and taps - Knowledge of standard bolts | Theory:1 Hrs Practice:3 Hrs Total: 4 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/ alphabet punch Round file Metal working chisel Punching toolsTap set Tap handle Pipe vice | Class Room |
| - Prepare spanner (small size) | Trainee will be able to - Select marking tools - Cut sheet as per drawing - Perform surface finishing with file - Level the surface with tri-square - Mark radius as per drawing - Develop radius as per drawing - Make the notch with round file - Perform finishing with sand paper | - Understanding proper use of hand shear - knowledge of flat lock seam end in metal sheet working | Theory:1 Hrs Practice:3 Hrs Total: 4 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/ alphabet punch Round file Metal working chisel Punching toolsTap set Tap handle Pipe vice |  |
| - Prepare Funnel | Trainee will be able to - Select marking tools - Cut sheet as per drawing - Perform surface finishing with file - Mark the sheet according to drawing - Cut the sheet with hand shear - Create radius of funnel using appropriate tools - Perform flat lock seam bend using bench vice - Perform finishing with sand paper. | knowledge of flat lock seam end in metal sheet working | Theory:1 Hrs Practice:3 Hrs Total: 4 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/ alphabet punch Round file Metal working chisel Punching toolsTap set Tap handle Pipe vice |  |

### @Perform general machining

Objective: This module covers the knowledge and  skills required to determine jobrequirements and sequence of operations, Select and mount tools, perform machiningoperations and Measure components, Adjust and maintain machine

Duration:  16	Hours					Theory: 4Hours						Practice: 12Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Carry out Sawing | Trainee will be able to: - Mark the job according to given drawing - Select appropriate blade according to job requirement - Set blade in frame of hacksaw as per procedure - Ensure the work piece is clamped firmly and properly - Adopt methods and techniques for sawing that is appropriate to jobrequirement - Follow marked line during sawing to ensure accuracy. | - Basic measurement - Basic measuring /Marking /cutting tools - Clamping / holding methods - Introduction machine shop tool their working and safety procedure during work. - Marking tools. - Safety procedure. - Different type of measuring tools. - Proper Usage of Measuring Tools - Type of measuring tools. - Machine types according to their work and rotation. - Maintenance and its types | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Jigs and fixtures - Vice - Measuring tools - Marking tools - Work holding devices - Cutting tools - Machine tools - Gauges - Files - Set of spanners - Set of Allen keys | Class Room and workshop |
| - Select and Mount Tools | Trainee will be able to: - Select a proper tool for the job - Check the sharpness. - Mount the tool in the machine - Check the position of tools | - Basic measurement - Basic measuring /Marking /cutting tools - Clamping / holding methods - Safety procedure. - Machine types according to their work and rotation. - Mounting Types - Maintenance and its types | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Jigs and fixtures - Vice/chuck - Measuring tools - Work holding devices - Set of spanners - Hammer - Set of Allen keys | Class Room and workshop |
| - Prepare forMachining operations | Trainee will be able to: - Mark the work piece - Set the parameters for the job - Check the requirements for the maximum tool life. - Clamped the work piece correctly. - Prevent the workpiece from the damage - Take product safety requirements. - Take personal safety requirements. - Perform the Machining | - Marking tools. - safety procedure. - Type of measuring tools. - Machine types according to their work and rotation. - Maintenance and its types | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Jigs and fixtures - Vice - Measuring tools - Marking tools - Work holding devices - Files - Set of spanners - Set of Allen keys | Class Room and workshop |
| - Measurecomponents, Adjust and maintain machine | Trainee will be able to - Use appropriate instruments or gauges to check components are compliantwith measurement requirements and specifications. - Carry out routine maintenance and adjustments as required including slideand collar adjustment, cleaning and lubrication | - Marking tools. - safety procedure. - Different type of measuring tools. - Proper Usage of Measuring Tools - Maintenance and its types | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Jigs and fixtures - Vice - Measuring tools - Marking tools - Work holding devices - Cutting tools - Machine tools - Gauges - Files - Set of spanners - Set of Allen keys | Class Room and workshop |

### @Perform cutting on Metal Circular/Power Heck Saw

Objective: This competency standard covers the skills and knowledge required to Carry outSawing and Carry out Sawing at different angles

Duration:  14	Hours					Theory: 2Hours					Practice: 12Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Carry out Sawing | Trainee will be able to: - Mark the job according to given drawing - Select appropriate blade according to job requirement - Set blade in frame of hacksaw as per procedure - Ensure the work piece is clamped firmly and properly - Adopt methods and techniques for sawing that is appropriate to jobrequirement - Follow marked line during sawing to ensure accuracy. | - Basic measurement - Types of hacksaw frames - Basic measuring / Marking / cutting tools - Clamping / holding methods - Methods and techniques of sawing. - Basic measurement - Types of hacksaw frames - Basic measuring / Marking /cutting tools - Clamping / holding methods. - Methods and techniques of sawing | Theory-1 Hrs Practical-6 Hrs Total- 7 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/ alphabet punch Round file Metal working chisel Punching tools Tap set Tap handle Pipe vice | Class Room and workshop |
| - Carry out Sawingat different angles | Trainee will be able to: - Mark the job according to given drawing - Select appropriate blade according to job requirement - Set blade in frame of metal circular saw as per procedure. - Ensure the blade tightness and rotating side. - Ensure the work piece is clamped firmly and properly - Adopt the methods and techniques for sawing that is appropriate to jobrequirement - Follow marked line during sawing to ensure accuracy | - Basic measurement - Types of hacksaw frames - Basic measuring / Marking / cutting tools - Clamping / holding methods - Methods and techniques of sawing. - Basic measurement - Types of hacksaw frames - Basic measuring / Marking /cutting tools - Clamping / holding methods. - Methods and techniques of sawing | Theory-1 Hrs Practical-6 Hrs Total- 7 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/ alphabet punch Round file Metal working chisel Punching tools Tap set Tap handle Pipe vice | Class Room and workshop |

### @Perform tool sharpening

Objective: This competency standard covers the skills and knowledge required to perform offhand grinding and Sharp single point cutting tool on grinding

Duration:  14	Hours			Theory: 2Hours				Practice: 12Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform offhand grinding | Trainee will be able to: - Select the proper size and shape of grinding wheel. - Observe personal and workplace safety. - Hold the work piece firmly against the rotating wheel by placing it on the tool rest. - Avoid over heating of the job Adopt technique and methods which are safe. - Produce component according to work operations. | - Types of different grinding machines. - Type, size and shape of wheels and abrasive. - Technique of holding work piece against rotating wheel. - Importance of using coolant. - Methods and techniques for off-hand grinding. - Selecting correct standing position during grinding. - Specific safety precautions and guidelines. - Types of different grinding machines. - Type, size and shape of wheels and abrasive. - Technique of holding work piece against rotating wheel. - Importance of using coolant. - Methods and techniques for off-hand grinding. - Selecting correct standing position during grinding. - Specific safety precautions and guidelines | Theory-1 Hrs Practical-6 Hrs Total- 7 Hrs | - D-type bevel protector - Grinding Machine - Personal Protective Equipment - Coolant - Wheel Dresser stand - T6. Dresser | Class Room and workshop |
| - Sharp single point cutting toolon grinding | Trainee will be able to: - Select the proper size and shape of grinding wheel. - Observe the personal and workplace safety. - Hold the work piece firmly against the rotating wheel by placing it on the tool rest. - Use the coolant at intervals to avoid over heating of the job. - Adopt technique and methods which are safe. - Sharp the tool according to work requirements. | - Types of different grinding machines. - Type, size and shape of wheels and abrasive. - Technique of holding work piece against rotating wheel. - Importance of using coolant. - Methods and techniques for off-hand grinding. - Selecting correct standing position during grinding. - Specific safety precautions and guidelines. - Types of different grinding machines. - Type, size and shape of wheels and abrasive. - Technique of holding work piece against rotating wheel. - Importance of using coolant. - Methods and techniques for off-hand grinding. - Selecting correct standing position during grinding. - Specific safety precautions and guidelines | Theory-1 Hrs Practical-6 Hrs Total- 7 Hrs | - D-type bevel protector - Grinding Machine - Personal Protective Equipment - Coolant - Wheel Dresser stand - T6. Dresser | Class Room and workshop |

### @Perform Basic Lathe Machine Operations

Objective: This competency standard covers the skills and knowledge required to Perform cantering operations, Perform facing Operations, Perform turning operations, Perform drilling or boring operations, Perform step turning operations, Perform knurling Operations, Taper turning by tail stock off-set method, Taper turning by plain taper turning attachment, Taper turning by telescopic taper turning attachment and Perform Internal and External threading Operations.

Duration:  40	Hours					Theory: 4Hours						Practice: 36Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Centeringoperations | Trainee will be able to: - Select the facing tools according to the job requirement. - Mount and set the required work-holding devices, work piece and cutting tools. - Follow the correct specifications for the part or component to beproduced. - Select the safe procedures and tools to accomplish the work. - Adjust the operating parameters (e.g. speed and feed) of machinetool for centering the job. - Ensure all safety mechanisms are in followed. | - Safety precautions involved in the work. - Methods and techniques of the mounting and setting of a work-piece. - Methods and techniques for the adjustment of operating parameters of the machines andtools. - Procedure for the adjustment of speed and feed. - Calculation of the speed and feed. - Use of the holding and cutting tools - Reading, understanding and interpretation of the work’s specifications, drawings andsketches. | Theory-0 Hrs Practical-3 Hrs Total- 3 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| - Perform facingOperations | Trainee will be able to: - Select the facing tools according to job requirement. - Mount and set the required work ? ?-holding devices, work pieceand cutting tools. - Follow the correct specifications for the jop / part or component tobe produced. - Select safe procedures and tools to accomplish the work. - Adjust the operating parameters (e.g. speed and feed) of machinetool to achieve the work specification. - Ensure all safety mechanisms are followed | - Facing Method and technique - Method and technique of the setting up and adjusting the machine. - Techniques to check the quality of component produced. - Procedure of the shutting down of machine and equipment after closure of activities. - Safety precautions and procedures need to be observed during work. - Types of the drilling or boring tools and their function. - Procedure of mounting and setting up of work-holding devices, work pieces and cutting tools. | Theory-0 Hrs Practical-3 Hrs Total- 3 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| - Perform turningoperations | - Obtain and follow the work specifications, drawings or sketches toaccomplish the work. - Set up and adjust the machine as per work specifications and Procedures. - Ensure the components produced have the required quality andspecified dimensional accuracy. - Shut down the machine and equipment on what you want to say? | - Turning Method and techniques. - Method and technique of the setting up and adjusting the machine. - Techniques to check the quality of component produced. - Procedure of the shutting down of machine and equipment after closure of activities. - Safety precautions and procedures need to be observed during work. - Types of the drilling or boring tools and their function. - Procedure of mounting and setting up of work-holding devices, work pieces and cuttingtools. - Method and technique of the adjusting RPM of a lathe machine. | ;Theory-0 Hrs Practical-6 Hrs Total- 6 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| - . Perform drillingor boring operations | - Select the drilling or boring tools according to the drawings. - Mount and set the required work?? (use proper word)-holdingdevices, work piece and cutting tools - Adjust the RPM of machine according to the cutting speed. - Perform the boring operation according to the drawing. - Check quality of the component produced at different intervals. - Observe the personal and workplace safety | - Safety precautions involved in the work. - Methods and techniques of the mounting and setting of a work-piece. - Methods and techniques for the adjustment of operating parameters of the machines andtools. - Procedure for the adjustment of speed and feed. - Calculation of the speed and feed. - Use of the holding and cutting tools - Reading, understanding and interpretation of the work’s specifications, drawings andsketches. - Method and technique of the setting up and adjusting the machine. - Types of the drilling or boring tools and their function. - Safe boring procedures. - Techniques of checking quality of components. | Theory-0 Hrs Practical-6 Hrs Total- 6 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| - Perform step turning operations | - Mount and set the required work-holding devices, work piece andcutting tools. - Select and adjust the appropriate speeds and feeds of turningmachine. - Produce a component which matches the work specifications usingappropriate methods and techniques. - Check the quality of the component produced at various intervals. - Follow the safety precautions to ensure safe work and to avoid anyinjury | - Methods and techniques of adjusting speeds and feeds of turning machine. - Procedure for the adjustment of speed and feed. - Calculation of the speed and feed. - Use of the holding and cutting tools - Reading, understanding and interpretation of the work’s specifications, drawings andsketches. - Method and technique of the setting up and adjusting the machine. - Techniques to check the quality of component produced. - Procedure of the shutting down of machine and equipment after closure of activities. | Theory-0 Hrs Practical-3 Hrs Total- 3 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| - Perform knurlingOperations | - Select the knurling tool according to drawing. - Set the tool and work piece in the machine according to theprocedure. - Adopt the methods and techniques in order to produce properknurling on the work piece. - Select and adjust an appropriate speeds and feeds of the lathemachine. - Use the coolants during knurling to achieve a smooth impression onthe work piece. - Observe the personal and workplace safety | - Types of knurling tools. - Types of knurling. - Procedure of setting tools and work piece in the machine. - Methods of knurling. - Procedure of adjusting speeds and feeds of a lathe machine. Importance of using acoolants during a knurling. | Theory-0 Hrs Practical-3 Hrs Total- 3 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| - Taper turning bytail stock off-setmethod | - Loosen the tailstock clamp out. - Offset tailstock required amount. - Centre the cutting tool. - Setup the cutting tool for a parallel turning. - No clear what is need to be said?? - Check the taper for an accuracy using the taper ring gauge. - Finish and turn the taper according to the required size in order to fit | - Kinds of tapers. - Types of taper turning methods. - Calculation of tapers. - Methods and techniques of adjusting speeds and feeds of turning machine. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| - Taper turning byplain taper turningattachment | - Remove the binding screw that cross the slide to cross the feedscrew and nut. - Tighten the lock screw and set the cutting tool in the center. - Set the work piece in the lathe machine and mark the length oftaper. - Use the binding screw in order to connect the sliding block and side of taper’s attachment. - Select the depth of a feed cut by the compound rest and feedhandle. - Take a light cut and recheck the taper fit. - Finish the turn and fit the taper to a gauge | - Knowledge of a lathe operations - Use of a dial indicator - Types of a threading tool. - Types of a threading. - Procedure of setting tools and work piece in the machine. - Methods of threading. - Procedure of adjusting speeds and feeds of a lathe machine. Importance of using thecoolants during the knurling. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| - Taperturning by telescopic taper Turningattachment | - Clean and oil the guide bar. - Loose lock screws and offset end of guide bar, - Set the bar to required taper in degrees. - Tighten the lock screw and set cutting tool on center. - Set the workpiece in lathe and mark the length of a taper andtighten the connecting screw on a sliding block. - Move the carriage until the center of attachment is opposite to thelength of taper. - Lock the anchor and bracket to the lathe bed. - Take a cut and select the depth of a cut. - Readjust the taper attachment, - Take a light cut and recheck thetaper fit. - Finish the turn and fit the taper to a gauge. | - Method and technique of the setting up and adjusting the machine. - Techniques to check the quality of component produced. - Procedure of the shutting down of machine and equipment after closure of activities. - Safety precautions and procedures need to be observed the during work. - Procedure of mounting and setting up of work-holding devices, work pieces and cutting tools. - Method and technique of the adjusting RPM of a lathe machine. - Coolants during the knurling. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |
| LU10. Perform Internaland ExternalthreadingOperations | - Mount and set the required work-holding devices, work piece and cutting tools - Select and adjust the appropriate speeds and feeds of the turning machine. - Produce a component which matches the work specifications usingan appropriate methods and techniques. - Check the quality of a component produced at the various tintervals. - Use the Proper cutting tool with a required dimensions. - Follow the safety precautions in order to ensure safe workingenvironment to avoid accidents and injuries | - Procedure of the shutting down of machine and equipment after closure of activities. - Method and technique of the adjusting RPM of a lathe machine. - Safe boring procedures. - Techniques of checking quality of components. - Methods and techniques of adjusting speeds and feeds of turning machine. - Procedure of setting tools and work piece in the machine. - Methods of knurling. - Procedure of adjusting speeds and feeds of a lathe machine. Importance of using acoolants during a knurling. - Knowledge of a lathe operations - Use of a dial indicator - Types of a threading tool. - Types of a threading. - Procedure of setting tools and work piece in the machine. - Methods of threading. - Procedure of adjusting speeds and feeds of a lathe machine. Importance of using thecoolants during the knurling. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Lathe Machine - Cutting Tools - Measuring Tools - Personal Protective Equipment - Files - Vernier Caliper - Checking gauges - Knurling Tools - Threading Tools | Class Room and workshop |

### @Perform Drilling Machine Operations

Overview: This competency standard covers the skills and knowledge required to produce holes using drilling machine, Perform counter boring and counter sinking and perform machine reaming

Duration:  21	Hours					Theory: 3Hours						Practice: 18Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Produce holesusing drillingmachine | Trainee will be able to: - Observe the personal and work place safety. - Set up the drilling machine for producing holes according to a jobrequirement. - Manipulate the machine tool controls safely and correctly in line withthe operational procedures. - Produce components to the required quality and within the specifieddimensional accuracy. - Carry out quality sampling checks at suitable intervals. - Shut down the equipment to a safe condition on conclusion of themachining activities not clear what is going to be said | - Safety precautions. - Procedure of the setting up of a drilling machine. - Safe procedure for an operating drilling machines. - Types of the drilling machines. - Selecting and adjusting speed and feed of drilling machine. - Importance of coolants in drilling operations. - Methods and techniques of quality checks. - Different types of drilling tools and their implications. - Importance of selecting right drilling tool for the job specifications. - Methods and techniques for positioning the work-piece in the drill to ensure properalignment and stability during drilling. - Using speeds and feeds chart for different types of materials and their hardness. - Specific safety precautions during boring and sinking operations. - Safety precautions. - Selecting reamer according to hole size. - Types of reamers (straight teeth or helical teeth). - Method of setting reamer in the drill chuck. - Importance of using lubricants during reaming. - Importance of alignment of the reamer during operations | Theory-1 Hrs Practical-6 Hrs Total- 7 Hrs | - Drilling Machines - Drill chuck with Key - Machine Vice - Marking Tools - Measuring Tools - Drill Sleeve and Socket - Personal Protective Equipment - Counter drill - Cutting oil - Tri square - Measuring Tool | Class Room and workshop |
| - Perform counter boring and countersinking | Trainee will be able to: - Select the relevant tools according to the information given inengineering drawings and job specifications. - Ensure the tooling is correct in terms of size, shape, type, and gradein order to perform the job / work. - Position the work-piece in the drill in such a way that it is aligned,secured and stable during drilling. - Adjust the speeds and feeds of the drill according to the size, type,and hardness of the work-piece material, in order to performs theoptimum cutting without damage to the work-piece. | - Selecting and adjusting speed and feed of drilling machine. - Importance of coolants in drilling operations. - Importance of selecting right drilling tool for the job specifications. - Methods and techniques for positioning the work-piece in the drill to ensure properalignment and stability during drilling. - Using speeds and feeds chart for different types of materials and their hardness. - Specific safety precautions during boring and sinking operations. | Theory-1 Hrs Practical-6 Hrs Total- 7 Hrs | - Drilling Machines - Drill chuck with Key - Machine Vice - Marking Tools - Measuring Tools - Drill Sleeve and Socket - Personal Protective Equipment - Counter drill - Cutting oil - Tri square - Measuring Tool | Class Room and workshop |
| - Perform machinereaming | - Observe the personal and workplace safety. - Clamp the work-piece in the vice properly. - Select the reamer according to the hole’s size and drawingrequirements - Set the reamer in the drill chuck according to procedure. - Use the lubricants during reaming for the smooth cutting. - Ensure the proper alignment of the reamer during the operations | - Safety precautions. - Selecting reamer according to hole size. - Types of reamers (straight teeth or helical teeth). - Method of setting reamer in the drill chuck. - Importance of using lubricants during reaming. - Importance of alignment of the reamer during operations | Theory-1 Hrs Practical-6 Hrs Total-7 Hrs | - Drilling Machines - Drill chuck with Key - Machine Vice - Marking Tools - Measuring Tools - Drill Sleeve and Socket - Personal Protective Equipment - Counter drill - Cutting oil - Tri square - Measuring Tool | Class Room and workshop |

### @Perform Shaper, Planar and Slotter Machining Operations

Objective: This competency standard covers the skills and knowledge required to produce asquared shape work piece, Produce V shaped work piece, machining a Rack Gear, T-slotMachining, Machining Irregular Surfaces, Machining External Keyways and Machining InternalKeyways

Duration:  28	Hours					Theory: 7Hours						Practice: 21Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Produce a squared shape work piece | Trainee will be able to: - Identify the safety hazards related the shaping operations in order to avoid injury / accident. - Select a point cutting tool and set the machine as per requirements. - Mount a cutting tool and the work piece in the machine. - Check the quality of the component at suitable intervals. - Shut down the machine at safe position after completion of work. | - List the safety hazards related to the shaper machine operations. - Use of Dial indicator - Method of the mounting of cutting tool - Checking of a right angle with the tri square. - Explain the square facing procedure. - Safety guidelines and procedures. - Safety checks for operating a shaper machine. - Interpreting the information given in the engineering drawings and job specifications | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Shaper, Planar or Slotter - Machine Vice - Tri square - Vernier Calliper - Dial indicator with magnet stand - Point cutting tools - Personal Protective Equipment - Tri square/bevel protector | Class Room and workshop |
| - Produce V-shaped workpiece | Trainee will be able to: - Identify the safety hazards related to the shaping operations in order to to avoid any injury / accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work Correct according to the “CU1” | - List the safety hazards related to the shaper machine operations. - Use of Dial indicator - Method of the mounting of cutting tool - Checking of a right angle with the tri square. - Explain the square facing procedure. - Safety guidelines and procedures. - Safety checks for operating a shaper machine. - Interpreting the information given in the engineering drawings and job specifications | Theory1 Hrs Practical-3 Hrs Total- 4 Hrs | - Shaper, Planar or Slotter - Machine Vice - Tri square - Vernier Calliper - Dial indicator with magnet stand - Point cutting tools - Personal Protective Equipment - Tri square/bevel protector | Class Room and workshop |
| - Machining a RackGear | Trainee will be able to: - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Set the job/Tool Movement According to specified speed - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work - Correct according to the “CU1” | - List the safety hazards related to the shaper machine operations. - Use of Dial indicator - Method of the mounting of cutting tool - Checking of a right angle with the tri square. - Explain the square facing procedure. - Safety guidelines and procedures. - Safety checks for operating a shaper machine. - Interpreting the information given in the engineering drawings and job specifications | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Shaper, Planar or Slotter - Machine Vice - Tri square - Vernier Calliper - Dial indicator with magnet stand - Point cutting tools - Personal Protective Equipment - Tri square/bevel protector | Class Room and workshop |
| - T-slot Machining | Trainee will be able to - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job - requirements. Mount cutting tool and work piece in the machine. - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work | - List the safety hazards related to the shaper machine operations. - Use of Dial indicator - Method of the mounting of cutting tool - Checking of a right angle with the tri square. - Explain the square facing procedure. - Safety guidelines and procedures. - Safety checks for operating a shaper machine. - Interpreting the information given in the engineering drawings and job specifications | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Shaper, Planar or Slotter - Machine Vice - Tri square - Vernier Calliper - Dial indicator with magnet stand - Point cutting tools - Personal Protective Equipment - Tri square/bevel protector | Class Room and workshop |
| - MachiningIrregularSurfaces | Trainee will be able to - Identify safety hazards related with shaping operations and takeappropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Use Different feed and speed of cutting and different points according to given drawing - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work - Correct according to the “CU1” | - List the safety hazards related to the shaper machine operations. - Use of Dial indicator - Method of the mounting of cutting tool - Checking of a right angle with the tri square. - Explain the square facing procedure. - Safety guidelines and procedures. - Safety checks for operating a shaper machine. - Interpreting the information given in the engineering drawings and job specifications | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Shaper, Planar or Slotter - Machine Vice - Tri square - Vernier Calliper - Dial indicator with magnet stand - Point cutting tools - Personal Protective Equipment - Tri square/bevel protector | Class Room and workshop |
| - Machining External Keyways | Trainee will be able to - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work Correct according to the “CU1” | - List the safety hazards related to the shaper machine operations. - Use of Dial indicator - Method of the mounting of cutting tool - Checking of a right angle with the tri square. - Explain the square facing procedure. - Safety guidelines and procedures. - Safety checks for operating a shaper machine. - Interpreting the information given in the drawings and job specifications | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Shaper, Planar or Slotter - Machine Vice - Tri square - Vernier Calliper - Dial indicator with magnet stand - Point cutting tools - Personal Protective Equipment - Tri quare/bevel protector | Class Room and workshop |
| - Machining internal Keyways | Trainee will be able to - Identify safety hazards related with shaping operations and takeappropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to jobrequirements. - Mount cutting tool and work piece in the machine. - Check quality of the component at suitable intervals | - List the safety hazards related to the shaper machine operations. - Use of Dial indicator - Method of the mounting of cutting tool - Checking of a right angle with the tri square. - Explain the square facing procedure. - Safety guidelines and procedures. - Safety checks for operating a shaper machine. - Interpreting the information given in the drawings and job specifications | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Shaper, Planar or Slotter - Machine Vice - Tri square - Vernier Calliper - Dial indicator with magnet stand - Point cutting tools - Personal Protective Equipment - Tri square/bevel protector | Class Room and workshop |

### @Perform Milling Operations

Objective: This competency standard covers the skills and knowledge required to Prepare Blankfor Generating the Gear, Select Tools and Equipment for Gear Cutting, Produce a squareshaped work piece, Generate spur gear (Direct Indexing) On Milling Machine, Generate spurgear (Differential Indexing) On Milling Machine, Generate Helical Gear On Milling Machine,Perform slotting or grooving on work piece, Perform drilling or boring using milling machine,Milling a T slot, Bevel gear cutting on milling machine, Practice of spur rack cutting and Practiceof helical rack cutting

Duration: 48	Hours					Theory: 12 Hours						Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Blank for Generating theGear | Trainee will be able to: - Interpret the drawing and arrange the material according to the job - requirement - Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank - Check and verify the dimensions of the blank for generating the gear as per drawing | - List the safety hazards related with the milling machines and its operations. - Use of dial indicator - Methods of mounting the cutters - Checking of right angle with the tri- square. - Explain the square milling procedure. - Safety guidelines and procedures. - Safety checks for milling machine under operation. - Interpretation of the information given in the engineering drawings and job specifications. - Knowledge of spur gear design - Identifying the safety hazards associated with the milling machine operations. | Theory-1 Hrs Practical-3 Hrs Total- 4Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Select Tools and Equipment for Gear Cutting | Trainee will be able to: - Select the material, type, shape and size of cutter(s) according to the job requirements - Arrange the measuring instruments and holding devices to attain accuracy of the work as according to the prescribed method | - Safety checks for milling machine under operation. - Interpretation of the information given in the engineering drawings and job specifications. - Knowledge of spur gear design - Identifying the safety hazards associated with the milling machine operations. - Explain the procedure of slotting and grooving. - Quality checks procedures and techniques. | Theory-1 Hrs Practical-3 Hrs Total- 4Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Produce a square shaped work piece | Trainee must be able to: - Identify the safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. Correct according to the “CU1” - Dial the machine vice according to job requirement. - Select the cutters and set the machine as per requirements. - Mount a cutters and the work piece in the machine. - Produce a part matching the process plan and the part print specifications. Not clear? - Check the quality of component at the recommended intervals. - Shut down the machine at safe position after completion of the work /job. | - Checking of right angle with the tri- square. - Explain the square milling procedure. - Safety guidelines and procedures. - Quality checks procedures and techniques. | Theory-1 Hrs Practical-3 Hrs Total- 4Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Generate spur gear (Direct Indexing) On Milling Machine. | Trainee must be able to - Select the gear cutter and indexing plate on a milling machine - Mount the indexing plate on a indexing head. - Centre the indexing head and its tail stock. - Fix the indexing head and tail stock on the milling table. - ENGAGE (why capitalized ??) worm shaft from worm wheel. - Adjust the speed, feed and direction of the cutter. - Mount the Gear blank on the mandrel. - Hold the one side of the mandrel on chuck of indexing head and other side in the tail stock - Start the machine and perform the cutting operation at zero point and - in vertical direction. - Repeat the above step in horizontal direction Apply the depth for a rough cut and configure the machine automatically ?? in the longitudinal direction - Move the table back at zero point. - Apply the full depth for final cut and configure machine automatically?? in the forward direction. - Repeat the process simultaneously?? Until the tooth is obtained. | - Knowledge of spur gear design - Identifying the safety hazards associated with the milling machine operations. - Method and technique for adjusting the RPM of milling machine. | Theory-1 Hrs Practical-3 Hrs Total- 4Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Generate spur gear (Differential Indexing) On Milling Machine | Trainee must be able to - Select the gear cutter and an indexing plate on the milling machine - Mount the indexing plate on the indexing head. - Centre the indexing head and its tail stock. - Fix the indexing head and tail stock on the milling table. - Revolve the index plate forward and / or backward?? Connecting word is missing?? part of turn while index crank turn to proper spacing - Choose the number close to required division which can be indexed by simple indexing - Adjust the speed feed and direction of the cutter. - Mount the Gear blank on the mandrel. - Hold one side of the mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. (correct according toprevious CU1) - Apply depth for rough cut and engage machine automatically in longitudinal direction correct according to previous CU1) - Move the table back at the zero point. - Apply the full depth for the final cut and engage the machine automatically?? In forward direction. - Repeat the process simultaneously?? until tooth is obtained | - Knowledge of spur gear design - Identifying the safety hazards associated with the milling machine operations. - Method and technique for adjusting the RPM of milling machine. | Theory-1 Hrs Practical-3 Hrs Total- 4Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Generate Helical Gear On Milling Machine | Trainee will be able to: - Select a indexing plate to machine a helical gear on a manual Machine. Correct according to the “CU1” and CU2 - Mount gear set to engage lead screw and indexing head spindle - Centre indexing head and its tail stock. - Fix indexing head and tail stock on milling table. - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained. | - Checking of right angle with the tri- square. - Explain the square milling procedure. - Safety guidelines and procedures. - Safety checks for milling machine under operation. - Knowledge of spur gear design - Identifying the safety hazards associated with the milling machine operations. - Explain the procedure of slotting and grooving. - Quality checks procedures and techniques. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Perform slotting or grooving on work piece | Trainee will be able to: - Identify safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. Correct according to the “CU1” and CU2 - Set the work piece in machine vice according to procedure. - Select the appropriate cutter as per specifications. - Adjust the milling cutter for slotting and grooving. - Determine the touching point of the work piece. - Produce slotting or grooving on the work piece to the required quality. - Check quality of the component at suitable intervals. - Shut down the machine at safe position after finishing the work. P0714E&A Observe personal and workplace safety at all time | - List the safety hazards related with the milling machines and its operations. - Explain the procedure of slotting and grooving. - Quality checks procedures and techniques. - Types of drilling or boring tools and their function. - Procedure of mounting and setting up of work-holding devices, work pieces and cuttingtools. | Theory-1 Hrs Practical-3 Hrs Total- 4Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Perform drilling or boring using milling machine | Trainee will be able to: - Identify safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. Correct according to the “CU1” and CU2 - Select drill or boring tools according to drawings. - Mount and set the required work-holding devices, work piece andcutting tools. - The RPM of machine according to the standard chart. Perform the boring operation according to the drawing. - Check quality of the component produced at different intervals. - Shut down the machine at safe position after finishing the work. - Observe personal and workplace safety at all time. | - Types of drilling or boring tools and their function. - Procedure of mounting and setting up of work-holding devices, work pieces and cuttingtools. - Method and technique for adjusting the RPM of milling machine. - Safe Boring and milling procedures. - Techniques for checking of the quality of the components. |  | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Milling a T slot | Trainee will be able to: - Layout the position of a T slot. - Square the vertical milling machine with machine table. - Mount the work in milling machine. - Machine the center slot to the proper depth of T slot by the end mill. - Remove the end mill and mount proper T slot cutter. - Machine the lower part of the slot. | - Explain the procedure of slotting and grooving. - Quality checks procedures and techniques. - Procedure of mounting and setting up of work-holding devices, work pieces and cuttingtools. - Method and technique for adjusting the RPM of milling machine. - Techniques for checking of the quality of the components. - Milling machine operations, | Theory-1 Hrs Practical-1 Hrs Total- 2 Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Bevel gear cutting on milling machine | - Cut the materials to the required size - Cut the work piece with a lathe that is shaped into a gear blank. - Cut the gear with a Conflux generator. - Remove the Burrs on the teeth with a deburring machine. | - List the safety hazards related with the milling machines and its operations. - Use of dial indicator - Safety checks for milling machine under operation. - Interpretation of the information given in the engineering drawings and job specifications. - Identifying the safety hazards associated with the milling machine operations. - Quality checks procedures and techniques. - Procedure of mounting and setting up of work-holding devices, work pieces and cuttingtools. - Method and technique for adjusting the RPM of milling machine. - Milling machine operations, - Bevel gear calculation | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Practice of Spur rack cutting. | - Hold the work piece in milling machine. - Hold the milling cutter in the rack milling attachment. - Hold the cutter at 90 degree for the spur cutting. - Move the table for each tooth by rack indexing attachment. - Continue the operation until the required length is obtained. | - Knowledge of spur gear design - Identifying the safety hazards associated with the milling machine operations. - Quality checks procedures and techniques. - Method and technique for adjusting the RPM of milling machine. - Safe Boring and milling procedures. - Techniques for checking of the quality of the components. - Milling machine operations, - Spur rack calculation | Theory-0.5 Hrs Practical-2 Hrs Total- 2.5 Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |
| - Practice of helical rack cutting | Trainee will be able to: - Hold the work piece in milling machine. - Hold the milling cutter in the rack milling attachment. - Hold the cutter at some (which angle??) angle for helical cutting. - Move the table for each tooth by rack indexing attachment. Continue the operation until the required length is obtained | - Knowledge of spur gear design - Identifying the safety hazards associated with the milling machine operations. - Quality checks procedures and techniques. - Method and technique for adjusting the RPM of milling machine. - Safe Boring and milling procedures. - Techniques for checking of the quality of the components. - Milling machine operations, - Spur rack calculation | Theory-0.5 Hrs Practical-2 Hrs Total- 2.5 Hrs | - Milling machine - Machine Vice - Tri square - Vernier Caliper - Dial indicator with magnet stand - Milling cutters - Personal Protective Equipment - Milling machine and its accessories - Slotting cutter - Vernier caliper - Depth gauge - End mil cutter - Boring tools - Internal Micrometer | Class Room and workshop |

- Electrical Essentials & Networks

### @Operate Measuring Instruments.

Objective: This competency standard covers the skills and knowledge required to Operate Ampere meter, Volt meter & Ohm meter.

Duration:  30 Hours			Theory: 6 Hours			Practical: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Operate Ampere meter. | Trainee would be able to - Identify Ampere meter. - Make a parallel circuit with three different resistors on breadboard. - Adjust proper range of Ampere meter as per load. - Connect Ampere meter in series to each resistor to measure the current. - Turn on the supply and note the reading of current against each resistor. | - Define current and its units. - Define resistance and its units. - Describe series circuit. - Describe parallel circuit. - Enlist the name of various meters. - Describe purpose of Ampere meter. - Explain how Ampere meter is connected in the circuit. - How to use breadboard for circuit development? Practical Activity: Measure current across each resistor (connected in series) in an active circuit with the help of Ampere meter. | Total: 5 Hrs. Theory:1 Hrs. Practice:4 Hrs. | - Multi meters. - Resistors of different values. - Small pieces of copper wire. - Breadboard. | Class Room and Workshop |  |
| - Operate Volt meter. | Trainee wouldbe able to: - Identify volt meter. - Make a series circuit with three different resistors on bread board. - Adjust proper range of Volt meter as per load. - Connect volt meter in circuit. - Take the reading. | - Define voltage and its units. - Define resistance and its units. - Describe series circuit. - Describe purpose of Volt meter - Explain how voltmeter is connected in the circuit. Practical Activity: Measure voltage across each resistor (connected in series) in an active circuit with the help of Volt meter. | Total: 5 Hrs. Theory:1 Hrs. Practice:4 Hrs. | - Multi meters. - Resistors of different values. - Small pieces of copper wire. - Breadboard. | Class Room and Workshop |  |
| - Operate Ohm meter. | Trainee would be able to: - Identify Ohm meter. - Adjust proper range of Ohm meter as per resistance value. - Connect Ohm meter with resistor. - Take the reading. | - Define resistance and its units. - Define Ohm’s Law. - Define variable resistor. - Describe purpose of Ohm meter. Practical Activity: Measure resistance across different resistors in an active circuit with the help of Ohm meter. | Total: 5 Hrs. Theory:1 Hrs. Practice:4 Hrs. | - Multi meters. - Resistors of different values. - Small pieces of copper wire. - Breadboard. - Variable Power supply | Class Room and Workshop |
| - Measure AC voltage with oscilloscope. | Trainee would be able to - Set function generator to generate ac voltage signal of suitable value of voltage and frequency. - Select a proper volt/division and give AC voltage to oscilloscope to visualize the signal. - Read number of divisions between the +ve and -ve peaks of the signal along the y-axis of screen. - Apply the formula to calculate the actual amplitude of AC voltage. - Apply the formula to calculate RMS value of AC voltage. - Apply the formula to calculate the average value of AC voltage. | - Explain calibration procedure of oscilloscope - Explain different controls of an oscilloscope. - Define trigger, channel, frequency, and time period. - Explain RMS value of AC signal. - Explain use of probes of oscilloscope Practical Activity: Make AC voltage measurement with the help of oscilloscope. | Total: 5 Hrs. Theory:1 Hrs. Practice:4 Hrs. | - Cathode-ray oscilloscope. - Multi-meter. - Oscillator/signal generator. - Resistor. - Breadboard. - DC power supply. - Function generator. | Class Room and Workshop |
| - Measure current with oscilloscope. | Trainee would be able to - Select the proper load to measure the current. - Connect the low value of resistance in series with the load. - Connect the oscilloscope across low value of resistor and measure the voltage. - Apply ohms law by using value of low value of resistor and voltage across it and find out current. - Apply formula to calculate RMS and Average value of current. | - Explain calibration procedure of oscilloscope - Explain procedure of current measurement - Explain Ohm’s Law. - Describe the procedure of calculating RMS and Average value Practical Activity: Measure RMS (Root Mean Square) and Peak value of AC signal with oscilloscope. | Total: 5 Hrs. Theory:1 Hrs. Practice:4 Hrs. | - Cathode-ray oscilloscope. - Multi-meter. - Oscillator/signal generator. - Resistor. - Breadboard. - DC power supply. - Function generator. | Class Room and Workshop |
| - Measure frequency of AC signal with oscilloscope. | Trainee would be able to - Select a proper time/division and give AC voltage to oscilloscope. - Calculate number of divisions between the one AC cycles of the AC signal. - Apply the formula to calculate the actual frequency of AC voltage. - Apply formula to calculate the period of signal. | - Explain calibration procedure of oscilloscope - Explain AC signal in detail. - Explain procedure of frequency measurement - Explain Ohm’s Law. - Describe the procedure of calculating frequency and period of a signal Practical Activity: Observe amplitude, frequency, period and phase shift of AC waveforms on oscilloscope. | Total: 5 Hrs. Theory:1 Hrs. Practice:4 Hrs. | - Cathode-ray oscilloscope. - Multi-meter. - Oscillator/signal generator. - Resistor. - Breadboard. - DC power supply. - Function generator. | Class Room and Workshop |

### @Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.

Objective:This competency standard covers the skills and knowledge required to Make series circuit and measure voltage and verify KVL, make parallel circuit and measure current and verify KCL, verify resistance of a resistor, Find unknown value of ohms law.

Duration:  28 Hours					Theory: 7 Hours						Practical: 21 Hour

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Make series circuit and measure voltage and verify KVL. | Trainee would be able to - Construct series circuit with different resistors on breadboard and give supply voltage. - Measure the voltage across each resistor. - Note the value of voltage against each resistor. - Sum the voltage of each resistor and verify is it equal to applied voltage. | - Describe characteristics of series circuit. - Describe relation between current, voltage and resistance - Explain Kirchhoff’s Voltage law (KVL). - Define Rheostat. - State laws of resistance. Practical Activity: Practically verify Kirchhoff's Voltage Law (KVL). | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Multi meters. - Resistors of different values. - DC power supply. - Connecting wires. - Rheostat. - Incandescent lamp of different watts. | Class Room and Workshop |
| LU2. Make parallel circuit and measure current and verify KCL. | Trainee would be able to - Construct parallel circuit with different resistors on bread board. - Measure the total current drawn by circuit. - Measure the value of current through each resistor individually. - Verify the law by putting all measured value into equation. | - Describe characteristics of series circuit. - Describe relation between current, voltage and resistance - Explain Kirchhoff’s current law (KCL). - Define Rheostat. - State laws of resistance Practical Activity: Practically verify Kirchhoff's Current Law (KCL). | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different watts. | Class Room and Workshop |
| LU3.Verify resistance of a resistor. | Trainee would be able to - Select the 4 colour band resistor. - Determine the nominal value of resistor using colour code including tolerances value. - Connect Ohm meter across the resistor and measure the value of resistance. - Compare both reading. | - Describe colour code and its use. - Describe the purpose of Ohm meter. - Describe how current, voltage and resistor interact with one another. - Define Rheostat. - State laws of resistance. Practical Activity: Practically verify Ohms law. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different watts. | Class Room and Workshop |
| LU4.Verify Ohm’s Law | Trainee would be able to - Construct the complex circuit using different resistors. - Find I, V with proper meter and determine R using Ohms law. - Find I, R with proper meter and determine V using Ohm law. - Find R, V with proper meter and determine I using Ohm law. | - Explain Current, Voltage and resistance quantities - Explain Ohm’s law. - Describe mathematical expression of ohm’s law. Practical Activity: Calculate unknown values of I,V & R and verify Ohms law. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different watts. | Class Room and Workshop |
| LU5.Measure resistance of Rheostat. | Trainee would be able to - Take sliding Rheostat of 17 Ω and Ohmmeter. (0 to 20 or more than 17 Ω) which is available. - Adjust zero with shortening the Ohmmeter leads. - Take reading of rheostat when slide is full placed at end,(whole resistance of rheostat)with the Ohmmeter like diagram shown above. - Note this result 17 Ω in your note book. - Repeat these reading when sliding exact in half of rheostat, and note this reading. | - Define Rheostat. - Explain procedure for measuring resistance of rheostat Practical Activity: Practically verify Ohms law by measuring resistance of Rheostat. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different watts. | Class Room and Workshop |
| LU6.Measure resistance by using multimeter. | Trainee would be able to - Take multimeter of 0 to 10 M-Ohms or any range of multimeter, which is available at your lab store. - Adjust zero with shortening the multimeter leads. - Take the reading of multimeter like shown above, and note it on your note book. | - Describe the use of multi meter. - State laws of resistance. Practical Activity: Measure resistance by using multimeter. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different watts. | Class Room and Workshop |
| LU7.Measure resistance of incandescent lamp. | Trainee would be able to - Take 40 W, lamp and multimeter. - Adjust zero with shortening the Ohmmeter leads. - Connect the Ohmmeter with its terminal and note its resistance. - Repeat this method for 60 & 100 W lamp. | - Describe characteristics of series circuit. - Describe characteristics of parallel circuit. - Describe colour code and its use. - Describe the purpose of Ohm meter. - Describe how current, voltage and resistor interact with one another. - Define Rheostat. - State laws of resistance. Practical Activity: Measure resistance of incandescent lamp by using Ohm meter. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different watts. | Class Room and Workshop |

### @Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

Objective:This competency standard covers the skills and knowledge required to Measure single phase and poly-phase electrical para meters using Volt-Ampere meter, watt meter & Measure consumed energy with Energy meter.

Duration:  28 Hours						Theory: 7 Hours						Practical: 21Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure power using Multimeter method | Trainee would be able to - Construct Select the load to calculate the power and give supply. - Measure the voltage and current by using Multimeter. - Find power using power formula. | - Define AC/D.C power. - Explain how to measure power of the circuit with meter. - Differentiate between electrical and mechanical power - Describe the use of Volt meter and Ampere meter. Practical Activity: Measure single phase apparent power using Volt-Ampere meter method. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Voltmeter. - Ammeter. - Connecting Leads. - Power Supply. - Screw Driver & Pliers. - Single Phase Load or 100 watt lamp. | Class Room and Workshop |
| - Measure power using watt meter | Trainee would be able to - Construct Select a load to calculate the power and connect watt meter across the load. - Power-ON the supply and measure the power. | - Define watt meter - Explain construction of watt meter - Describe procedure of measuring power through watt meter Practical Activity: Calculate real power of single phase load with the help of watt meter. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Wattmeter. - Connecting Leads. - Power Supply. - Screw Driver & Pliers. - Single Phase Load or 100 watt lamp. | Class Room and Workshop |
| - Measure single phase AC Power | Trainee would be able to - Select a load to calculate the power and appropriate supply. - Measure the voltage and current using Multimeter. - Calculate the power using power. | - Explain single phase network - Describe calculation of measuring power by power formula - Explain measurement procedure of voltage and current with multimeter Practical Activity: Demonstrate the two wattmeter method of measuring the power in single phase networks. | Total 4 Hrs. Theory: 1.0 Hrs. Practical: 3 Hrs. | - Voltmeter - Ampere meter - Multimeter - Combination plier - Phase tester - Cutter plier | Class Room and Workshop |
| - Measure three phase AC Power. | Trainee would be able to - Select a load to calculate the power with appropriate supply. - Measure the voltage, current using Multimeter for each phase. - Calculate the power using power formula. | - Explain three phase network - Describe calculation of measuring power by power formula - Explain measurement procedure of voltage and current with multimeter. Practical Activity: Demonstrate the two wattmeter method of measuring the power in 3-phase networks. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Wattmeter. - Connecting Leads. - Power Supply. - Screw Driver & Plier. - Three Phase Load (3 phase Motor) | Class Room and Workshop |
| - Measure consumed energy with Energy meter | Trainee would be able to - Connect phase line of AC supply to the energy meter. - Connect AC load to the output terminals of energy meter. - Power up the supply and take reading of energy in terms of unit after few minutes form display. | - Explain energy meter in detail - Explain the procedure for measuring consumed energy with energy meter - Describe formula to calculate energy Practical Activity: Connect energy meter with domestic load of single story building | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Connecting Leads. - Power Supply. - Screw driver & pliers. - Energy meter | Class Room and Workshop |
| - Measure power factor with voltmeter, ampere meter and watt meter | Trainee would be able to - Connect voltmeter & ampere meter with inductive load (Motor) - Connect watt meter and measure. - Connect AC supply and take the readings of volt, ampere and watt meter. - Utilize the above readings and calculate power factor using power formula P=VI Cos Ø | - Explain power factor - Describe formula for calculating power factor - Explain procedure for calculating power factor with voltmeter, Ampere meter and watt meter Practical Activity: Measure power factor by connecting voltmeter, ampere meter and watt meter. | Total 4 Hrs. Theory: Practical:3 Hrs. | - Power factor meter - Voltmeter - Ampere meter - Watt meter - Load (Lamp) - Connecting leads - Power Source | Class Room and Workshop |
| - Measure power factor with power factor meter. | Trainee would be able to - Connect current coil of power factor meter in series to the load. - Voltage coil of power factor meter in parallel to the load. - Power the supply and measure the value of power factor form the meter. | - Explain the reasons of power factor lagging - Describe procedures to improve power factor of an A.C inductive circuit - Explain the leading power factor - Explain function of power factor meter - Explain procedure for calculating power factor with power factor meter. Practical Activity: Measure power factor with power factor meter. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Power factor meter - Voltmeter - Ampere meter - Watt meter - Load (Lamp) - Connecting leads - Power Source | Class Room and Workshop |

### @Construct Electromagnet to See Various Effects & Verify Faradays Law

Objective::This competency standard covers the skills and knowledge related to Implement Electromagnet, Implement circuit to determine the effect on current carrying conductor in magnetic field, Determine the effect on conductor by varying the current with the help of rheostat, plot magnetic lines of forces of bar magnet, Verify Faradays law by moving magnet in side coil, Verify Faradays law by moving coil near the magnet field, Verify EMF through induction.

Duration:  28 Hours						Theory: 7 Hours						Practical: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Implement Electromagnet. | Trainee would be able to - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil wound on the iron nail. - Bring iron nail near the iron pieces and demonstrate the observation. | - Define magnet and magnetism. - Define electromagnet. - Define function of iron core. - Define magnetic lines of force. - Define magnetic field. Practical Activity: Observe how current can create a magnetic field. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Iron nail as core. - Thin coated copper wire. - Rheostat. - Current carrying conductor. - Horse shoe magnet. - Dry cell battery. | Class Room and Workshop |
| - Implement circuit to determine the effect on current carrying conductor in magnetic field. | Trainee would be able to - Take copper rod (5cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horse shoe magnet. - Observe the rod movement. - Change the polarity of supply and observe the rod again. | - Explain Fleming’s left-hand rule. - Define the term induction - Explain Faraday’s law of electro-magnetic induction. Practical Activity: Calculate the magnetic force on a current-carrying conductor. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Iron nail as core - Thin coated copper wire - Rheostat - Current carrying conductor - Horse shoe magnet. - Dry cell battery - Magnet. - Connecting leads. - Transformer - Compass needle - Coil - Bar Magnet - Paper - Lead pencil. - Galvanometer | Class Room and Workshop |
| - Determine the effect on conductor by varying the current with the help of rheostat. | Trainee would be able to - Reduce the rheostat resistance. - Record the effect on copper rod. - Increase the rheostat resistance. - Record the effect on copper rod. | - Define function of iron core. - Describe procedure of making magnetic strength of electromagnet. Practical Activity: Measure the effects of a magnetic field on a current carrying conductor with the help of rheostat. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Rheostat. - Current carrying conductor. | Class Room and Workshop |
| - Plot magnetic lines of forces of bar magnet. | Trainee would be able to - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet. - Mark points on paper where the compass needle stops. - Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for 3 to 4 | - Explain Fleming’s left-hand rule - Explain the direction of magnetic lines of forces Practical Activity: Observe the magnetic field associated with a bar magnet and construct the magnetic field lines. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs.. | - Iron nail as core - Thin coated copper wire - Rheostat - Current carrying conductor - Horse shoe magnet. - Dry cell battery - Magnet. - Connecting leads. - Transformer. - Compass needle. - Coil. - Bar Magnet. - Paper. - Lead pencil | Class Room and Workshop |
| - Verify Faradays law by moving magnet in side coil. | Trainee would be able to - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. | - Explain procedure of connecting galvanometer with the coil - Explain Faraday’s law of electromagnetic induction Practical Activity: Verify Faradys law with the help of galvanometer. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Iron nail as core. - Thin coated copper wire. - Rheostat. - Current carrying conductor. - Horse shoe magnet. - Dry cell battery. - Magnet. - Connecting leads. - Transformer. - Compass needle. - Coil. - Bar Magnet. - Paper. - Lead pencil. - Galvanometer. | Class Room and Workshop |
| - Verify Faraday’s law by moving coil near the magnet field. | Trainee would be able to - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvanometer with coil. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer. | - Explain Lenz’s Law - Explain procedure of moving coil near static magnetic field Practical Activity: Determine the direction ofthe current induced in a coil as a result of a change in the magnetic flux inside thecoil. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Iron nail as core. - Thin coated copper wire. - Rheostat. - Current carrying conductor. - Horse shoe magnet. - Dry cell battery - Magnet. - Connecting leads. - Transformer. - Compass needle. - Coil. - Bar Magnet. - Paper. - Lead pencil. - Galvanometer. | Class Room and Workshop |
| - Verify EMF throughinduction. | Trainee would be able to - Take step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat. - Continuously vary the rheostat and observe the reading on the Galvanometer. | - Define term induction - Explain Lenz’s Law for the direction of induced EMF - Explain reason of high and low EMF induction in coil. Practical Activity: Verify Farady’s law by changing magnetic flux and induced current associated with Faraday’s Law of Induction. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Iron nail as core. - Thin coated copper wire. - Rheostat. - Current carrying conductor. - Horse shoe magnet. - Dry cell battery. - Magnet. - Connecting leads. - Transformer. - Compass needle. - Coil. - Bar Magnet. - Paper. - Lead pencil. - Galvanometer. | Class Room and Workshop |

### @Verify Series and Parallel Combination of Capacitors & Determine Break down Voltage of Capacitor

Objective:This competency standard covers the skills and knowledge related to Implement a series circuit of capacitors, implement a parallel circuit of capacitors, and determines the breakdown voltage of low voltage capacitor.

Duration:  24 Hours					Theory: 6 Hours							Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Implement a series circuit of capacitors. | Trainee would be able to - Assemble a series circuit of capacitors with 3 different values. - Connect the battery and apply 10 voltages to the combination of capacitors - Measure the voltage across each capacitor connected in series - Add the voltage of each capacitor and verify is it equal to applied voltage. - Use voltage of each capacitor and its value to find the charge on each capacitor. | - Define the capacitance of capacitors. - Explain the principles of capacitance. - Describe the basic units of capacitance. - Explain different types of capacitors. - Determine total capacitance in series circuits. Practical Activity: Connect up-to 3 capacitors in series and find the value of total capacitance | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - Capacitors, 4.7 μF - Capacitors, 10, 100 & 8.2 μF. - Voltmeter. - D.C. power supply unit. - Bridging plugs. - Connecting leads. - Multimeter. (With option of Capacitance measurement). | Class Room and Workshop |
| - Implement a parallel circuit of capacitors. | Trainee would be able to - Assemble a parallel circuit of capacitors with 3 different values. - Connect the battery and apply 10 voltages to the combination of capacitors. - Use applied voltage and value of capacitor to find the charge on each capacitor. - Use total voltage and total capacitance to verify the net charge on the capacitor. | - Define the capacitance of capacitors. - Explain the principles of capacitance. - Describe the basic units of capacitance. - Explain different types of capacitors. - Determine total capacitance in parallel circuits. Practical Activity: Connect up-to 3 capacitors in parallel and find the value of total capacitance | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - Capacitors, 4.7 μF - Capacitors, 10, 100 & 8.2 μF. - Voltmeter. - D.C. power supply unit. - Bridging plugs. - Connecting leads. - Multimeter. (With option of Capacitance measurement). | Class Room and Workshop |
| - Determine the breakdown voltage of low voltagecapacitor. | Trainee would be able to - Take a capacitor of 6.3 rated volts. - Connect it across a 0-30 volts D.C variable supply - Very slowly increased the voltage of variable supply from zero to rated 6.3v, and then higher than rated, and observed carefully the capacitor and to voltmeter reading. - After slowly increasing at critical position of voltage, the capacitor will be burnt, and smoked. Note that | - Explain breakdown voltage of capacitors? - Explain factor can affect the life of a capacitor? - Define charging of capacitor? - Explain factors affecting capacitance. Practical Activity: Construct a RC circuit to measure breakdown voltage | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - Capacitors, 4.7 μF - Capacitors, 10, 100 & 8.2 μF. - Voltmeter. - Connecting leads. - D.C. power supply unit. - Bridging plugs - Multimeter. (With option of Capacitance measurement). - Variable D.C powers supply 0-30 Voltas. - Capacitor (Sanyo Operated Volts 6.3 V, SE 30) - D.C Analog or Preferably Digital voltmeter Range 0-50V. - L.E.D (3 to 9 V) Optional. | Class Room and Workshop |

### @Maintain Tools & Equipment.

Objective: This competency standard covers the skills and knowledge related to arrange tools/equipment, maintain tool box, insulate tools/equipment, calibrate measuring tools and manage proper inventory of used/unused tools/equipment. Trainee will be expected to follow the procedures to maintain the tools/equipment.

Duration: 15 Hours					Theory: 5 Hours					Practical: 10 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Arrange Tools and Equipment | Trainee would be able to: - Identify tools and equipment. - Interpret job card. - Prepare list of tools and equipment as per requirement. - Collect tools and equipment from store. | - Define tool usage w.r.t job. - Differentiate similar tools as per assigned job. - Explain the proper protection of tools before and after use. Practical Activity: List the necessary tools for wire cutting and jointing. | Total 03 Hrs. Theory: 01Hrs. Practical: 02 Hrs | - Cables. - Wires. - Wire stripper. - Soldering Iron - Solder wire. - Soldering paste - Plier. - Nose plier. - Insulation remover. | Class Room and Workshop |
| LU2. Maintain Tool Box | Trainee would be able to: - Check physical conditions of tools and equipment before use - Perform preventive maintenance as per standards - Perform corrective maintenance of tools as per requirements - Clean tools and equipment after use - Place tools and equipment at appropriate place. | - Define the cleaning method for metallic/ nonmetallic tools. - Define the parameter considered before using sharp tools. - Define the criteria for handling electric test equipment. - Define the storage method for sensitive test equipment’s Practical Activity: Inspect the malfunction issue in the meters used in the defined job | Total 03 Hrs. Theory: 01 Hrs. Practical: 02Hrs | - Cables. - Wires. - Wire stripper. - Soldering Iron - Solder wire. - Soldering paste - Plier. - Nose plier. - Insulation remover. - PCB Kit | Class Room and Workshop |
| LU3. Insulate Tools and Equipment | Trainee would be able to: - Select insulated tools and equipment - Apply insulated tools and equipment as per standards. | - Define the importance of insulation. - Explain the material used in insulation of tools. Practical Activity: Insulate damaged insulated tools available in workshop. | Total: 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs | - CombinationPlier. - Cutter Plier - Nose plier. | Class Room and Workshop |
| LU4. Calibrate measuring tools | Trainee would be able to: - Check calibration status of the measuring tools - Perform calibration of measuring tools as per standards - Record calibration test results. | - Define the term calibration. - Define calibration process in measurement tools. - Define the method for result keeping as record. Practical Activity: Test the measuring instrument’s calibration for accurate measurement. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs | - Analog Multimeter - Digital Multimeter - Wattmeter - Voltmeter - Ammeter | Class Room and Workshop |
| LU5. Manage Inventory of tools and equipment | Trainee would be able to: - Check tools and equipment as per record - Report for faulty tools and equipment to supervisor - Generate demand for deficit tools and equipment - Maintain all records of tools and Equipment. | - Define the format (performa) for faulty tool reporting. - Explain the method for presenting the demand/replacement for/of new tool. - List the inventory of tools as per record. Practical Activity: Prepare a list of old and new tool as merge list and discard the old tool. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs | - Wire Stripper - Solder wire - Soldering paste - CombinationPlier. - Cutter Plier - Nose plier - Multimeter | Class Room and Workshop |

### @Make Cable/Wire Joints

Objective: This competency standard covers the skills and knowledge related to make joints of cable during wiring and breakage of wires, Make Cross/Twist joint, Make Straight/Married joint, Make T- Joint, Make Rat tail joint, Make Britannia joint.

Duration: 15 Hours					Theory: 5 Hours						Practical: 10Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Make Cross/Twist joint | Trainee would be able to: - Select the cable. - Strip the wire according to 50mm. - Twist the conductors. - Solder the conductors - Insulate the joint | - Define joint and its application. - Define the term wire insulation. - Define the uses of Cross/Twistjoint. - Explain the soldering techniques. - Explain the importance of wire insulation. Practical Activity: Make Cross/Twist joint and insulate it. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs | - Cables - Wires. - Wire stripper. - Solder wire. - Soldering paste - Combination Plier - Cutter Plier - Nose plier. - Solder Iron | Class Room and Workshop |
| LU2. Make Straight/Married joint | Trainee would be able to: - Select the cable. - Strip wire to 75mm according to joint requirement. - Intermingle the conductors to 60mm into each other. - Twist 60mm conductors leaving behind 15mm of each cable. - Solder the conductor. - Insulate the joint. | - Explain straight / married joint and its application. - Define the term wire insulation. - Define the uses ofstraight/Marriedjoint. - Explain the soldering techniques for strengthen the joint. - Explain the importance of wire insulation. Practical Activity: Make the straight / Married joint and insulate it. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs | - Cables - Wires. - Wire stripper. - Solder wire. - Soldering paste - Combination Plier. - Cutter Plier - Nose plier. - Solder Iron | Class Room and Workshop |
| LU3. Make T-Joint | Trainee would be able to: - Select the cable - Remove the insulation of cable 1 to 50mm from where a connection is required - Separate conductors of cable 1 equally - Take another 12 mm stripped wire 2 - Insert between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable - Solder the joint. | - ExplainMake T-Joint and its application. - Define the term wire insulation. - Define the uses of T-Joint joint. - How soldering strength, the joint. - Highlight the importance of wire insulation in terms of protection. Practical Activity: Make the T-joint and insulate it. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs | - Cables - Wires. - Wire stripper. - Solder wire. - Soldering paste - Plier. - Nose plier. - Insulation remover. - Solder | Class Room and Workshop |
| LU4. Make Rat-tail joint | Trainee would be able to: - Select the single conductor wires - Strip both the wires to 5mm - Twist the conductor - Insulate the joint - Solder the joint | - ExplainRat tail joint and its application. - Explain the term wire insulation. - Define the uses of rat tail joint. - Explain soldering strength of the joint. Practical Activity: Makethe Rat tail joint and insulate it. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs | - Cables - Wires. - Wire stripper. - Solder wire. - Soldering paste - CombinationPlier - Cutter Plier - Nose plier. - Solder Iron | Class Room and Workshop |
| LU5. Make Britannia joint | Trainee would be able to: - Select the cable. - Strip both cables to 75mm. - Bend the tips of both cable right angle about to 6mm. - Hold the two cables overlap 50mm with tips in opposite direction. - Take another bare conductor of 1mm and wrap around in both directions to 6mm. - Solder the joint. - Insulate the joint. | - Explain Britannia joint and its application. - Define the wire insulation. - Define the uses of Britannia joint. Practical Activity: Make the Make Britannia jointand insulate it. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs | - Cables - Wires. - Wire stripper. - Solder wire. - Soldering paste - CombinationPlier - Cutter Plier - Nose plier. - Solder Iron | Class Room and Workshop |

### @Prepare and Install Distribution Boards

Objective: This competency standard covers the skills and knowledge required to prepare and install distribution board according to provide protection and load division to wiring, prepare estimate for wiring material, Prepare Distribution Board, Install Distribution Board and wiring.

Duration: 21 Hours					Theory: 3 Hours						Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Prepare estimate for wiring material. | Trainee would be able to: - Develop Basic Drawing - Perform Estimation of Materials - Calculate Labour Cost | - Define the importance of wiring plan (drawing). - Define the term cost estimation. - Define the attributes of Labour cost. - Explain calculation chart for labour cost Practical Activity: Analyze the available resource and estimate the cost of wiring material as per given budget. | Total 7 Hrs. Theory: 01 Hrs. Practical: 06 Hrs | - Cables - Wires. - Minature Circuit Breaker (MCB) - Molded Case Circuit Breaker (MCCB) - Resudual Current Device (RCD) - Earth Leakage Circuit Breaker (ELCB) - Distribution Board (DB) - Socket and switches | Class Room and Workshop |
| LU2.Prepare Distribution Board | Trainee would be able to - Select Distribution board w.r.t. size, current rating, voltage, No. of C.Bs and phases - Select Main Incoming Residual Current Circuit Breaker (RCCB) having minimum sensitivity - Select outgoing Miniature Circuit Breaker (MCB) according to load - Provide space for in DB for further MCB - Select Voltmeter, ampere meter and indicator according to load and phases - Select wire according to load and for wiring in Distribution Board (DB) - Make neutral common for all load - Connect accessories according to the circuit diagram | - Define the types of circuit breakers. - Differentiate between RCCB/MCB. - Explain different types of distribution board. - Explain the uses of different types of indicating instrument. - Explain importance of Neutral wire in distribution board. - Explain different accessories use in distribution wiring. Practical Activity: Inspect the overall requirement for as electrical distribution system as per the designated 3 phase load. | Total 7 Hrs. Theory: 01 Hrs. Practical: 06 Hrs | - Cables - Wires. - Wire stripper. - Solder wire. - Soldering paste - CombinationPlier - Nose plier. - Insulation remover. - Solder. - MCB - MCCB - RCD - ELCB - DB - Socket and switches | Class Room and Workshop |
| LU3.Install Distribution Board and wiring | Trainee would be able to - Fix the distribution board - Take wire from DB to load without joint - Distribute load equally on all phases | - Define the DB installation with earthling and insulation. - Explain the method to expand the wiring distribution in home or office from the distribution board. - Define how to distribute the load. Practical Activity: Develop the single phase distribution board for designated work place. | Total 7 Hrs. Theory: 1 Hrs. Practical: 6 Hrs | - Cables - Wires. - Wire stripper. - Combination Plier - Cutter Plier - Nose plier.. - MCB - MCCB - RCD - ELCB - DB - Socket and switches | Class Room and Workshop |

### @Carryout Basic Electrical Installation

Objective:Thiscompetency standard covers the skills and knowledge required to lay cables, perform single & three phase connections, basic electric wiring and wiring test for carrying out basic electrical Air Condition (AC) installation. The trainee will be required to follow company guidelines as well as the procedure for carrying out basic electrical AC installation.

Duration: 15 Hours						Theory: 5 Hours						Practical: 10 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Basic Electrical wiring | Trainee would be able to - Measure cables as per requirement - Connect cables - Perform joints - Insulate Joints | - Define different types of cable. - Explain simple electrical circuit - Define the difference between cable and wire. - Discuss different electrical wiring diagrams. Practical Activity: Develop the single phase distribution wiring network for a single floor building. | Total 3 Theory: 01 Hrs. Practical: 2 Hrs | - Cables - Solder - Wire stripper - Plier - Wires - Nose plier - Solder wire - Soldering paste - Insulation remover | Class Room and Workshop |
| - Conduct wiring Test | Trainee would be able to - Operate multi-meter for voltage and current - Perform continuity test - Perform polarity test - Perform earthling test - Perform insulation test - Record test results | - Define multi meter analog/ DMM. - Describe the use of megger and earth tester. - Define the importance of meter precision. - Define the process of insulation test. Practical Activity: Test the following attributes of the wire - Insulation test. - Current voltage test. - Earthing test. | Total 3 Theory: 01 Hrs. Practical: 2 Hrs | - Cables - Wire stripper - Plier - Nose plier - Solder wire - Soldering paste - Megger | Class Room and Workshop |
| - Lay Cables | Trainee would be able to - Interpret electrical drawing / document - Identify cables - Lay cable. - Perform earthling | - Define different types of cable - Define key features of earthling with examples. - Explain the cable laying process in different ground conditions. - Explain the types of earthling w.r.t service area (domestic, industrial, commercial) Practical Activity: Setup an earth pit for 120 yards house. | Total 3 Theory: 01 Hrs. Practical: 2 Hrs | - Cables - Solder - Wire stripper - Plier - Wires - Nose plier - Solder wire - Soldering paste - Insulation remover - Earth plate. | Class Room and Workshop |
| - Perform single-phase Connection | Trainee would be able to - Select cable gauge - Select cables colours - Connect cables - Insulate Joints | - Define the AC fundamentals. - Define the wire gauge and its calculation. - Explain the cable connection and its joints. - Explain the process of insulation of cable. Practical Activity: Develop a cable joint for wire connection in the distribution board. | Total 3 Theory: 01 Hrs. Practical: 2 Hrs | - Cables - Solder - Wire stripper - CombinationPlier - Nose plier - Solder wire - Soldering paste | Class Room and Workshop |
| - Perform three phase Connection | Trainee would be able to - Select cable Gauge - Select cables colours - Connect cables - Insulate Joints | - Define the AC fundamentals. - Define the wire gauge and its calculation. - Explain the difference in single and three phase circuits of electrical distribution. - Explain the three phase 3 wire and three phase 4 wire circuits. - Define the difference between cable and wire. Practical Activity: Connect a 3 phase Motor with a contactor using RYB phase configuration. | Total 3 Theory: 01 Hrs. Practical: 2 Hrs | - Cables - Solder - Wire stripper - Plier - Wires - Nose plier - Solder wire - Soldering paste - Insulation remover | Class Room and Workshop |

### @Install Single Phase Electrical Wiring

Objective:This competency standard covers the skills and knowledge required to make single pole switch circuit, make single pole switch socket circuit, make two-way switch circuit, make series/test lamp circuit. Make intermediate switch circuit. Make of Tunnel circuit, Make of impulse switch circuit.

Duration: 30 Hours						Theory: 6 Hours						Practical: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Make single pole switch circuit. | Trainee would be able to - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply | - Define the term pole and throw in a switch. - Define the various types of switch working and applications. - Define various types of wire types and ducting’s w.r.t specification. - Name various connectors and lug used in cable connections. - Explain various types of electrical supply standards. Practical Activity: Test and install the electrical wire/ cable as per given cable plan with distribution board. | Total 05 Hrs. Theory: 01 Hrs. Practical: 04 Hrs. | - Electrician Tool kit. - PVC Pipe/Duct. - PVC clamp. - Wooden/PVC board. - PVC wire according to load - Screw - Single pole switch. - Bulb holder - Bulb - AVO meter. - Test Indicator | Class Room and Workshop |
| - Make single pole switch socket circuit. | Trainee would be able to - Draw wiring diagram of single pole switch socket circuit. - Draw Current path diagram of single pole switch socket circuit. - Draw layout diagram of single pole switch socket circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply | - Define the phase, neutral, earth in a switch and socket. - Define various types of wire types and ducting’s w.r.t specification. - Define connectors used in switch socket connections. - Define various types of electrical supply standards for 2/3 pin switch sockets. - Explain the connections of multiple switch socket arrangements in an electrical board as per current requirement. Practical Activity: Test and install the electrical wire/ cable as per given cable plan and mentioned specifications for an electrical working board based on multi socket arrangement. | Total 05 Hrs. Theory: 01 Hrs. Practical: 04 Hrs. | - Electrician Tool kit. - PVC Pipe/Duct. - PVC clamp. - Wooden/PVC board. - PVC wire according to load - Circuit Breaker. - Screw - Single pole switch. - Lamp holder - Lamp - AVO meter. - Test Indicator | Class Room and Workshop |
| - Make two-way switch circuit. | Trainee would be able to - Draw wiring diagram of two-way switch circuit. - Draw Current path diagram of two-way switch circuit. - Draw layout diagram of two-way switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply | - Define the various types of switch working and applications. - Define structure of Two way switch with application and operation. Practical Activity: Develop a start case wiring for 2 floors. | Total 05 Hrs. Theory: 01 Hrs. Practical: 04 Hrs. | - Electrician Tool kit. - PVC Pipe/Duct. - PVC clamp. - Wooden/PVC board. - PVC wire according to load - Circuit Breaker. - Screw - Single pole switch. - Lamp holder - Lamp - AVO meter. - Test Indicator | Class Room and Workshop |
| - Make series/test lamp circuit. | Trainee would be able to - Draw wiring diagram of series circuit. - Draw Current path diagram of series circuit. - Draw layout diagram of series circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply | - Define series circuit. - Define parallel circuit - Discuss the difference between series and parallel circuit. - Explain the test procedure of Series circuit. - Selection of resistive load for series circuit. - Define the series circuit installation for testing purpose. Practical Activity: Develop a series test lap for testing purpose. | Total 05 Hrs. Theory: 01 Hrs. Practical: 04 Hrs. | - Electrician Tool kit. - PVC Pipe/Duct. - PVC clamp. - Wooden/PVC board. - PVC wire according to load - Circuit Breaker. - Screw - Single pole switch. - Lamp holder - Lamp - AVO meter. - Test Indicator | Class Room and Workshop |
| - Make intermediate switch circuit. | Trainee would be able to - Draw wiring diagram of intermediate switch circuit. - Draw Current path diagram of intermediate switch circuit. - Draw layout diagram of intermediate switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply | - Define the various types of switch working and applications. - Define structure of intermediate switch with application and operation. - Explain multi stage intermediate switch with applications. Practical Activity: Develop an electrical system to operate load from various location using intermediate switch. | Total 05 Hrs. Theory: 01 Hrs. Practical: 04 Hrs. | - Electrician Tool kit. - PVC Pipe/Duct. - PVC clamp. - Wooden/PVC board. - PVC wire according to load - Circuit Breaker. - Screw - Single pole switch. - Lamp holder - Lamp - AVO meter. - Test Indicator | Class Room and Workshop |
| - Make of Tunnel circuit | Trainee would be able to - Draw wiring diagram of tunnel circuit. - Draw Current path diagram of tunnel circuit. - Draw layout diagram of tunnel circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply | - Define the term pole and throw in an electrical switch. - Define the various types of switch working and applications. - Explain the function and operation of tunnel wiring with applications. - Explain multi location tunnel circuit with applications Practical Activity: Design a demonstration circuit for mine engineers. | Total 05 Hrs. Theory: 01 Hrs. Practical: 04 Hrs. | - Electrician Tool kit. - PVC Pipe/Duct. - PVC clamp. - Wooden/PVC board. - PVC wire according to load - Circuit Breaker. - Screw - Single pole switch. - Lamp holder - Lamp - AVO meter. - Test Indicator | Class Room and Workshop |

### @Perform Testing of Electrical Wiring

Objective:This competency standard covers the skills and knowledge required to Perform Earth leakage Test, Perform Open Circuit Test, Perform Short Circuit Test, Perform Continuity/Loop Test, Perform Visual Test, Perform Insulation Test, Perform Polarity Test, Perform Earth Resistance Test, Perform Murray Loop Test, and Perform BLAVIER & Earth Loop Test.

Duration: 48 Hours						Theory: 8 Hours						Practical: 40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Open Circuit Test | Trainee would be able to - Disconnect the supply source, neutral conductor and earth conductor from distribution fuse box. - Connect ohmmeter between starting and ending terminals of live conductor. - Ensure the reading must be near to 0 ohm. - Repeat P2 to P3 for neutral and earth conductor. | - Define open circuit. - Define open circuit attributes. - Explain the ohm resistance in the open circuit. Practical Activity: Test the cable for open circuit using earth tester. | Total 6 Theory: 1 Hrs. Practical: 5 Hrs | - Sockets - Loads - Supply source - Distribution Box - Ohmmeter - Continuity - Tester. - Breakers | Class Room and Workshop |
| - Perform Short Circuit Test | Trainee would be able to - Disconnect the supply source, neutral conductor and earth conductor from distribution fuse box. - Connect one terminal of ohmmeter with live conductor. - Connect the other terminals of ohmmeter with neutral conductor. - Ensure the reading must be infinity (No reading). - Connect ohmmeter between live conductor and earth conductor. - Ensure the reading must be infinity (No reading) | - Define short circuit. - Define short circuit attributes. - Explain the ohm resistance in the short circuit. Practical Activity: Test the cable for short circuit using earth tester. | Total 6 Theory: 1 Hrs. Practical: 5 Hrs | - Sockets - Loads - Supply source - Distribution Box - Ohmmeter - Continuity - Tester. - Breakers | Class Room and Workshop |
| - Perform Continuity/Loop Test | Trainee would be able to - Connect all metal clad switches, metal parts, conduits with earth. - Open the main switch. - Turn on all the switches. - Connect earth continuity tester with conduit and earth. - Measure the resistance value with tester which must not be more than 1 ohm. | - Define the term circuit continuity. - Define short circuit attributes. - Explain the ohm resistance and circuit continuity or loop test. Practical Activity: Test the transmission link for ensuring continuity of the cable using earth tester. | Total 6 Theory: 1 Hrs. Practical: 5 Hrs | - Sockets - Loads - Supply source - Distribution Box - Ohmmeter - Continuity Tester. - Breakers | Class Room and Workshop |
| - Perform Visual Test | Trainee would be able to - Check the wires joints. - Check if there is any spark. - Check over heating of wires. - Check all the appliances are on or fluctuating. - Check all indications of meters. - Check Voltages on all phases. - Check the DB and Breakers. - Check the main supply is coming properly. | - Define the cable visual test methods. - Define the physical state of the damage cable. - Observe structural change in the cable due to spark or heat. - Define the cause of increase in resistance due to damage in the cable. - Define the importance of cable grounding. Practical Activity: A fan was operating at its maximum speed. It stops and get out of order. Perform the visual test for fault finding. | Total 6 Theory: 1 Hrs. Practical: 5 Hrs | - Sockets - Loads - Supply source - Distribution Box - Ohmmeter - Continuity - Tester. - Breakers | Class Room and Workshop |
| - Perform Insulation Test | Trainee would be able to - Switch off the main breaker. - Short circuit all lighting connections. - Connect merger between line and earth. - Rotate the merger to generate the voltage. - Read the merger dial and ensure that the insulation resistance should not be less than 50M ohm of any single circuit. | - Define the need of insulation testing. - Define the loss caused due to weak insulation. - Explain various insulation testing method. Practical Activity: Test a cable for its insulation integrity using insulation tester. | Total 6 Theory: 1 Hrs. Practical: 5 Hrs | - Sockets - Loads - Supply source - Distribution Box - Ohmmeter - Continuity Tester. - Breakers - Megger | Class Room and Workshop |
| - Perform Polarity Test | Trainee would be able to - Switch off the main switch. - Disconnect all loads. - Switch on all circuit control switches. - Connect ohm meter between main line and all the terminal of sockets and load points one by one. - Ensure meter reading value must be less than 1 ohm. | - Define the term polarity. - Explain the importance of circuit polarity. - Define the visual inspection method to test polarity. - Define the continuity test method to test polarity. - Define the ohmic effect on polarity. Practical Activity: Test the polarity of a lighting circuit using ohm meter. | Total 6 Theory: 1 Hrs. Practical: 5 Hrs | - Sockets - Loads - Supply source - Distribution Box - Ohmmeter - Continuity Tester. - Breakers | Class Room and Workshop |
| - Perform Earth Resistance Test | Trainee would be able to - Place 3 electrodes in earth at distance of 10m apart between every electrode. - Connect terminal E of earth resistance tester to first electrode. - Connect terminal P of earth resistance tester to the second electrode. - Connect terminal C of the earth resistance tester with third electrode. - Measure the resistances after applying specific voltage. - Repeat P1 to P5 by interacting earthling regions. - Ensure 3 readings must be equal. | - Define the term earth resistance. - Define the function of electrodes. - Define working of Earth tester with example. - Define the standard range of earth resistance. - Define connection procedure of earth resistance tester. Practical Activity: An electricity lab is being setup for research purpose. Find the earth resistance to insulate the lab by making earthling pit at the location using earth resistance tester. | Total 6 Theory: 1 Hrs. Practical: 5 Hrs | - Sockets - Loads - Supply source - Distribution Box - Ohmmeter - Continuity Tester. - Breakers - Megger | Class Room and Workshop |
| - Perform Earth leakage Test. | Trainee would be able to - Ensure the socket must be at least 13A for RCD plug in. - Adjust the sensitivity of RCD. - Ensure tripping time must not exceed than 200msec. - Plug the RCD in socket and switch on socket outlet. - Ensure P-N and - P-E light up. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec. - Move the selection switch to 180o. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec. - Move selection switch to 00. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec | - Define earthling and its purpose. - Explain the operation of Earth leakage tester. - Define the indicators used in earth tester. - Explain the procedural steps involve in earth leakage testing. Practical Activity: Measure the earth leakage current of the cable using earth tester. | Total 6 Theory: 1 Hrs. Practical: 5 Hrs | - Sockets - RCD - ELCB - Loads - Supply source - Distribution Box - Ohmmeter - Earth resistance tester - Continuity - Tester. - Breakers - Megger | Class Room and Workshop |

### @Operate Single Phase Transformer

Objective: This Module covers the knowledge and skills required to Identify types of transformer, Measure transformation ratio of single phase transformer, Make transformer winding, Determine voltage regulation of single phase transformer with different load and Perform parallel operation of single phase transformer.

Duration:  30 Hours					Theory: 10 Hours					Practical: 20 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Identify types of transformer. | Trainee would be able to - Identify the number of windings in the primary and secondary side of transformer. - Identify the types of transformer cores. - Identify connection of the windings. - Separate single phase, three phase and auto transformer. | - Explain construction of transformer - Explain working principle of transformer - Explain types of transformer - Explain connection procedure of transformer with load Practical Activity: Identify primary and secodary winding of transformer | Total 06 Hrs. Theory: 02 Hrs. Practical: 04 Hrs. | - Different types of Transformer - Hand Gloves | Class Room and Workshop |  |
| - Measure transformation ratio of single phase transformer. | Trainee wouldbe able to: - Connect volt meters on both side of the transformer. - Apply rated voltage to the primary. - Note the volt meter reading. - Calculate transformation ratio using ratio formula. - Repeat 1st step to 2nd step for different secondary tapping. | - Explain procedure of connecting voltmeter with transformer both windings - Explain transformer ratio. Practical Activity: Connect volt meter with transformer and observe the voltage. | Total 06 Hrs. Theory: 02 Hrs. Practical: 04 Hrs. | - Single phase transformer - Voltmeter - Ampere meter - Hand Gloves | Class Room and Workshop |  |
| - Make transformer winding | Trainee would be able to: - Design the core of transformer according to the transformer rating. - Select the wire for primary and secondary winding. - Perform winding according to require turns with the help of winding machine. - Cover the winding with insulating paper. - Adjust the core pieces into the winding. - Remove the insulation of enameled wire. - Cover the ends of wire with sleeve. - Perform continuity test with ohm meter. | - Explain Core of the transformer - Explain primary and secondary winding of transformer - Explain importance of insulation of winding in transformer - Explain continuity test procedure in transformer Practical Activity: Design core of transformer according to voltage rating | Total 06 Hrs. Theory: 02 Hrs. Practical: 04 Hrs. | - Core material of transformer - Winding wire of transformer - Multimeter - Hand Gloves |  | Class Room and Workshop |
| - Determine voltage regulation of single phase transformer with different load. | Trainee would be able to - Connect watt meter, ampere meter, and volt meter to the primary side of transformer. - Connect the voltmeter to secondary side of transformer. - Connect resistive load to secondary side. - Apply rated voltage to primary side of transformer. - Calculate voltage drop and voltage regulations (%) from meter readings. - Repeat from step1 to step5 for inductive load. - Repeat from step1 to step5 for capacitive load. | - Explain procedure of connecting voltmeter, ammeter, watt meter with the primary and secondary winding - Explain procedure to calculate voltage drop of transformer Practical Activity: Connect secondary winding of transformer with load | Total 06 Hrs. Theory: 02 Hrs. Practical: 04 Hrs. | - Single phase transformer - Voltmeter - Ampere meter - Watt meter - Hand Gloves |  | Class Room and Workshop |
| - Perform parallel operation of single phase transformer. | Trainee would be able to - Identify two transformer having same rated voltage, frequency and transformation ratio. - Mark the terminals with the help of polarity test. - Connect secondary sides of both transformers in parallel. - Connect secondary sides of both transformers with load bus bar having ampere meters between them. - Connect the primary side of both transformers in parallel. - Connect primary side of transformer with generation bus bar. - Adjust the ammeter readings to zero by adding inductor and resistor in series with secondary side of the transformer. | - Explain procedure of connecting secondary winding of transformer with load bus - Explain procedure to test polarity of terminals Practical Activity: Connect transformer secondary winding with load bus bar | Total 06 Hrs. Theory: 02 Hrs. Practical: 04 Hrs. | - Single phase transformer - Hand Gloves - Frequency Meter |  | Class Room and Workshop |

### @Perform Tests on Single Phase Transformer

Objective: This Module covers the knowledge and skills required to Perform polarity test of single phase transformer, Perform Open circuit test of transformer, Perform Short circuit Test of Transformer, Calculate efficiency of transformer by direct method and Calculate efficiency of transformer by Back to Back test.

Duration:  15 Hours						Theory: 5 Hours						Practical: 10 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform polarity test of single phase transformer. | Trainee would be able to - Connect the circuit according to the diagram. - Note the reading of both voltmeter. - Identify additive and subtractive polarity. - Mark the terminal according to the additive and subtractive polarity. | - Explain the Additive and subtractive polarity of single phase transformer - Explain marking procedure of transformer terminal Practical Activity: Identify additve and subtractive polarity of single phase transformer. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs. | - Single phase Transformer - Volt meter | Class Room and Workshop |
| - Perform Open circuit test of transformer. | Trainee would be able to: - Connect watt meter, ampere meter, and volt meter to the lower voltage side of transformer. - Open the high voltage side of transformer. - Apply rated voltage to lower voltage side of transformer. - Read the instrument /meters reading. - Calculate iron losses and excitation branch component (Ro, Xo). | - Explain open circuit test and its importance - Explain types of losses - Explain Iron losses Practical Activity: Perform open circuit test of single phase transformer | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs. | - Single phase transformer - Volt meter - Ampere meter - Watt meter | Class Room and Workshop |
| - Perform Short circuit Test of Transformer. | Trainee would be able to: - Connect watt meter, ampere meter, and volt meter to the higher voltage side of transformer. - Connect lower voltage side terminal with each other (Short Circuit) - Apply 10 % to 15 % of rated voltage to higher voltage side of transformer. - Read the instrument/meters readings. - Calculate copper losses and winding component (Ro1, Xo1) | - Explain short circuit test and its importance. - Explain types of losses - Explain copper losses Practical Activity: Perform short circuit test of single phase transformer | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs. | - Single phase transformer - Volt meter - Ampere meter - Watt meter | Class Room and Workshop |
| - Calculate efficiency of transformer by direct method. | Trainee would be able to - Calculate iron losses and copper losses from above tests. - Calculate input power and output power. - Calculate efficiency from calculated powers. | - Describe the formula for calculating power of single phase transformer - Describeformula for calculating efficiency of single phase transformer Practical Activity: - Calculate efficiency of single phase transformer | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs. | - Single phase transformer | Class Room |
| - Calculate efficiency of transformer by Back to Back test. | Trainee would be able to - Identify two transformer of same rating. - Connect both transformer in parallel. - Connect another variable voltage transformer to secondary side of parallel transformers. - Connect a switch, watt meter and ammeter between variable voltage transformer and parallel transformer. - Connect voltmeter and watt meter to primary side of transformer parallel transformer. - Apply rated voltage to the primary side of the parallel transformers. - Open the switch. - Calculate iron losses. - Close the switch - Adjust the voltage of variable voltage transformer until the full load current passes from secondary side of parallel transformers. - Calculate copper losses. - Calculate efficiency from above calculated losses. | - Explain Procedure of connecting 2 transformer in parallel - Explain procedure to perform Back to Back test of single phase transformer. Practical Activity: Connect 2 transformer in parallel for Back to Back test. | Total 03 Hrs. Theory: 01 Hrs. Practical: 02 Hrs. | - Single phase transformer - Volt meter - Ampere meter - Watt meter | Class Room and Workshop |

- Metrology

### @Take measurements with graduated tools

Objective: This competency standard covers the skills and knowledge required to take measurements Steel rule, Take measurements Hook rule, Take measurements Folding rule and Take measurements with Trammels.

Duration: 16 Hours			Theory: 4 Hours			Practice: 12 Hours			Credit Hours: 1.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurements Steel rule. | Trainee would be able to - Place the ruler on various work pieces. - Take the dimension and record the multiple readings on each of work piece. - Take an average of readings. - Record the results. | - Explain basic terms in measurements. - Explain Dimensions and quantities. - Classify linear measurement tools. - Describe types of measurements and its units. - Explain the basic principles of design of linear measuringinstruments. - Explain area and volume of geometric shapes. - Describe line, arc and circle measurements. Practical Activity: Measure the linear dimensions of work piece with the help of Steel rule. | Theory: 1 Hrs. Practical: 03 Hrs Total 4 | - Steel rule. - Work piece. - Surface plate. - Steps and collars. | Class Room and Workshop |
| - Take measurements Hook rule. | Trainee would be able to - Place hook ruler at the given work piece. - Take the dimension and record multiple readings on each work piece. - Take averages of the recorded measurements or results. - Record the results. | - Explain measurements. - Explain Dimensions and quantities. - Explain linear measurement tools. - Describe types of measurements and its units. - Explain Measurement techniques. - Explain of area, volume of geometric shapes. - Describe line, arc and circle measurements. Practical Activity: Measure the linear dimensions of work piece with the help of Hook rule. | Theory: 1 Hrs. Practical: 03 Hrs Total 4 | - Hook rule. - Work piece. - Surface plate. - Steps and collars. | Class Room and Workshop |
| - Take measurements Folding rule. | Trainee would be able to - Take the suitable work piece for measurement. - Measure the dimensions of work pieces. - Compute surface area, volume, of given work pieces. - Record the results in data table. | - Explain measurements. - Explain Dimensions and quantities. - Explain linear measurement tools. - Describe types of measurements and its units. - Explain Measurement techniques. - Explain of area, volume of geometric shapes. - Describe line, arc and circle measurements. Practical Activity: Measure the linear dimensions of work piece with the help of Hook rule and calculate and surface area & volume. | Theory: 1 Hrs. Practical: 03 Hrs Total 4 | - Work piece. - Surface plate. - Steps and collars. - Folding rule | Class Room and Workshop |
| - Take measurements with Trammels. | Trainee would be able to - Open the trammel according to required dimension. - Draw circle or arc by fixing one point in the dot and rotate. | - Explain measurements. - Explain Dimensions and quantities. - Explain linear measurement tools. - Describe types of measurements and its units. - Explain Measurement techniques. - Explain of area, volume of geometric shapes. - Describe line, arc and circle measurements. Practical Activity: Draw circle &arc by fixing one point in the dot and rotate trammels to bisect lines and angles as per required dimensions on a given work piece. | . Theory: 1 Hrs. Practical 03 Hrs TOTAL 4 | - Trammel. - Work piece. - Surface plate. - Steps and collars. | Class Room and Workshop |

### @Take measurements with combination set

Objective: This competency standard covers the skills and knowledge required to Take Measurement with Square head, Perform levelling with square head as spirit level, Measure depth with square head as depth gauge, Measure height with square head as height gauge, Perform levelling /measure angle of surfaces with protractor Head, Perform cantering with centre head and Find diameter of round bar.

Duration: 28 Hours			Theory: 7 Hours			Practice: 21 Hours			Credit Hours: 2.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take Measurement with Square head. | Trainee would be able to - Place the combination set on given drawing sheet. - Draw layout of a component with the help of combination square. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Measurement techniques. - Explain procedures of dimensioning of drawings and layouts. - Explain Measurement techniques. Practical Activity: Measure depth and height with square head and draw layout of a component with the help of combination square. | Theory: 1 Practical: 6 Hrs Total 7 | - Combination set. - Scriber. - Tri square. - Divider. | Class Room and Workshop |
| - Perform leveling with square head as spirit level. | Trainee would be able to - Place the spirit level on horizontal work piece. - Check the deviation of bubbles. - Level the surface by inserting shims. - Place square head on vertical work piece. - Note the deviation and correct the level. - Check the other geometrical shapes. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedures ofdimensioning of drawings and layouts. Practical Activity: Correct the level ofsurfaces of a given work piece by using spirit level of combination sets. | Theory: 1 Practical: 6 Hrs Total 7 | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. | Class Room and Workshop |
| - Measure depth with square head as depth gauge. | Trainee would be able to - Place the square head on work piece. - Apply force at the head to retain contact with the component. - Set square head and record the depth. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts. Practical Activity: Measure depth of a given work piece with square head of combination set. | Theory: 1 Practical: 6 Hrs Total 7 | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Measure height with square head as height gauge. | Trainee would be able to - Place the square head on work piece. - Measure height of work piece with and without the help of height gauge attachment. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts. Practical Activity: Measure height of a given work piece by using square head of combination set. | Theory: 1 Practical: 6 Hrs Total 7 | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Perform leveling /measure angle of surfaces with Protractor head. | Trainee would be able to - Place protractor head of combination set on work piece. - Loose screws of revolving circular disk and adjust the bubble of level to minimize deviation. - Study the angle between slopes of a component. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Perform leveling of a given work piece and measure angle of surfaces with the help of Protractor head of combination set. | Theory: 1 Practical: 6 Hrs Total 7 | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Perform centering with center head. | Trainee would be able to - Place the center head on round stock. - Trace the line with scriber. - Revolve the center head by 90o and draw another line. - Find center of round stock by bisecting point of lines. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Locate centre of a bar stock or acircular job and perform centering of a given work piece by using center Head of combination set. | Theory: 1 Practical: 6 Hrs Total 7 | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Find diameter of round bar. | Trainee would be able to - Place the center head on round stock. - Note the reference point and measure point by avoiding parallax. - Find the difference between reference point and measure point. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Determine the diameter of round barof a circular job held against the centre Head of combination set. | Theory: 1 Practical: 6 Hrs Total 7 | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |

### @Perform Leveling.

Objective: This competency standard covers the skills and knowledge required to Exercise of cast iron, granite and glass surface plate, Check level of surfaces with straight edge and spirit level, Check level by Engineer’s level and measurements with Engineer’s parallel.

Duration: 16 Hours			Theory: 4 Hours			Practice: 12 Hours			Credit Hours: 1.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Exercise of cast iron, granite and glass surface plate. | Trainee would be able to - Place the work piece with angle plate on surface plate. - Check squareness and parallelism of components. - Check the squareness of angle plate with dial indicator. | - Explain squareness. - Explain parallelism. - Angularmeasurement techniques. - Descirbegeometric shapes. - Describe measurements system of units (SI / British). - Explain conversion factors. Practical Activity: Check squareness and parallelism of surface plate of different materials. | Theory: 1 Hrs. Practical: 3. Hrs Total 4 | - Surface plates. | Class Room and Workshop |
| LU2.Check level of surfaces with straight edge and spirit level. | Trainee would be able to - Place straight edge diagonally on the surface plate. - Place spirit level on the straight edge. - Check the level with bubble. - Adjust the level by feeler gauge. | - Explain squareness. - Explain parallelism. - Angular measurement techniques. - Descirbegeometric shapes. - Describe measurements system of units (SI / British). - Explain conversion factors. Practical Activity: Determine flatness &straightness of surfaces for measuring alignment ofmachine parts, by using straight edge and spirit level. . | Theory: 1 Hrs. Practical: 3. Hrs Total 4 | - Straight edge and spirit level. | Class Room and Workshop |
| LU3.Check level by Engineer’s level. | Trainee would be able to - Clean the level and surface. - Place the level on work piece. - Record the difference by changing the position of level. - Calculate mean or average value. - Maintain the level of plane by providing packing as per requirement. | - Angularmeasurement techniques. - Describe geometric shapes. - Describe measurements system of units (SI / British). - Explain conversion factors. Practical Activity: Check level of planeby changing the position of levelon given work piece by using Engineer’s level. | Theory: 1 Hrs. Practical: 3. Hrs Total 4 | - Engineer’s level. | Class Room and Workshop |
| LU4.Measurements with Engineer’s parallel. | Trainee would be able to - Insert the engineer’s parallel in work piece. - Measure the taper or diameter of cylindrical surface. | - Angularmeasurement techniques. - Describegeometric shapes. - Describe measurements system of units (SI / British). - Explain conversion factors. Practical Activity: Measure taper and diameter of cylindrical surface of a given work piece by using Engineer’s parallel. | Theory: 1 Hrs. Practical: 3. Hrs Total 4 | - Engineer’s parallel. | Class Room and Workshop |

### @Take measurements with gauges

Objective:This competency standard covers the skills and knowledge required to take measurement with fixed gauge, plug gauge, adjustable gauge, small hole gauge and telescope gauge.

Duration: 16 Hours			Theory: 4Hours			Practice: 12 Hours			Credit Hours: 1.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with fixed gauge and plug gauge. | Trainee would be able to - Check / Compare the dimension of provided work piece against the reference standard with the help of fixed gauge. - Insert the plug gauge into the work piece to determine either dimension is of acceptable size or over size. | - Explain procedure of dimensioning and measurement techniques. - Describe standards and references. - Explain inner dia. outer dia. and wall thickness. Practical Activity: Check the dimension of provided work piece against reference standard with the help of fixed gauge. | Theory: 1 Hrs. Practical: 3. Hrs Total 4 | - Fix gauge. - Surface plate. - Surface gauge. - Dial indicator. - Plug guage | Class Room and Workshop |
| - Take measurement with adjustable gauge. | Trainee would be able to - Insert both “go” and “not go” gauges in the work piece. - Check the size and record the results. | - Explain procedure of dimensioning and measurement techniques. - Explain go and not go gauges. - Describe standards and references. - Explain inner dia. outer dia. and wall thickness. Practical Activity: Check the size of a given work piece by using “go” and “not go” gauges and record the results. | Theory: 1 Hrs. Practical: 3. Hrs Total 4 | - Surface plate. - Adjustable gauge. | Class Room and Workshop |
| - Take measurement with small hole gauge. | Trainee would be able to - Insert the small hole gauge of require size into the work piece. - Record the result. | - Explain procedure of dimensioning and measurement techniques. - Describe standards and references. - Explain inner dia. outer dia. and wall thickness. Practical Activity: Determine the dimensions of small holes of a given work piece with the help of with small hole gauge. | Theory: 1 Hrs. Practical: 3. Hrs Total 4 | - Surface plate. - Hole gauge. | Class Room and Workshop |
| - Take measurement with telescope gauge. | Trainee would be able to - Insert the telescope gauge into the work piece. - Adjust the gauge and determine the size of work piece. | - Explain procedure of dimensioning and measurement techniques - Describe standards and references. - Explain inner dia. outer dia. and wall thickness. Practical Activity: Determine the size of a given work piece by taking measurements with the help of telescope gauge. | Theory: 1 Hrs. Practical: 3. Hrs Total 4 | - Surface plate - Telescope gauge | Class Room and Workshop |

### @Perform measurements through Micrometer

Objective:This competency standard covers the skills and knowledge required to take measurement with outside micro-meter, inside micrometer, depth micrometer, Measure threads with micrometer and Vernier micrometer.

Duration: 15Hours			Theory: 5 Hours			Practice: 10 Hours			Credit Hours: 1.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with outside micro-meter. | Trainee would be able to - Clean the gauge and surface of work piece. - Open slightly larger than the part to be measured. - Set anvil squarely against reference surface of part. - Using ratchet, slowly until it click once. - Record the reading and calculate the average. | - Describe how Abbe’s law applies to micrometer measurement. - Explain principle of design of outside micrometer. - Describe types of micrometers. - Classify measurement techniques. - Describe geometric shapes thickness. Practical Activity: Take measurements of a given work piece with outside micrometer. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Outside micrometer. | Class Room and Workshop |
| - Take measurement with inside micrometer. | Trainee would be able to - Clean the gauge and surface of work piece. - Open gauge slightly smaller than the part to be measured. - Set anvil squarely against reference surface of part with ratchet, slowly until it click once. - Record readings and calculate the average. | - Explain principle of design of inside micrometer. - Describe types of Micrometers. - Classify measurement techniques. - Geometric shapes thickness. Practical Activity: Take measurements of a given work piece with inside micrometer. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Inside Micrometer. | Class Room and Workshop |
| - Take measurement with depth micrometer. | Trainee would be able to - Clean the gauge and surface of work piece. - Access the depth of groove by steel rule. - Insert the suitable insertion Rod. - Place the depth micrometer gauge on the deeper groove and record the reading. | - Explain principle of design of depth micrometer. - Classify measurement techniques. - Describe geometric shapes thickness. Practical Activity: Access the depth of groove of a given work piece with depth micrometer gauge. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Depth micrometer. | Class Room and Workshop |
| - Measure threads with micrometer. | Trainee would be able to - Clean the gauge and the threads of work piece. - Use the suitable set of thread shape tips according to thread size which is to be measured. - Record the reading. | - Explain design and angles of screw threads. - Explain basic principles of measurement of screw threads. - Describe geometry of screw threads. - Explain measurement principles of major diameter, minordiameter, effective diameter, pitch, angle, and form of screwthreads. - Explain tools usage. Practical Activity: Measure the dimensions of threads on a given work piece by using micrometer. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs | - Inside Micrometer. - Outside micrometer. - Depth micrometer. | Class Room and Workshop |
| - Take measurement with Vernier micrometer. | Trainee would be able to - Clean the surface. - Place specimen on the surface plate. - Take reading accurately. - Calculate mean reading. | - Explain principle of measurement of a Vernier micrometer - Classify measurement techniques. - Geometric shapes thickness. - Explain tools usage. Practical Activity: Take measurements of a given specimen withVernier micrometer. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Vernier micrometer. | Class Room and Workshop |

### @Measure dimensions with Vernier tools

Objective:This competency standard covers the skills and knowledge required to take measurement with Vernier calliper, height gauge and Vernier depth gauge.

Duration: 18 Hours			Theory: 6 Hours			Practice: 12 Hours			Credit Hours: 1.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with Vernier caliper | Trainee would be able to - Clean the gauge and surface of work piece. - Place the work piece on surface plate. - Open the lock screws. - Slide the jaws up to the width or size of jaws. - Open slightly larger than the part to be measured. - Set anvil squarely against reference surface of part. - Maintain the proper pressure on the jaws with screw. - Lock the movable jaw with knurled screw. - Measure and record the dimensions. | - Classification of Vernier instruments. - Design of linear measurement instruments. - Explain basic principles of design of Vernier calipers. - Understand dimensioning. Practical Activity: Take measurements on a given work piece with Vernier caliper. | Total 6.0 Hrs. Theory: 2.0 Hrs. Practical: 4.0 Hrs. | - Vernier Caliper. | Class Room and Workshop |
| - Take measurement with height gauge | Trainee would be able to - Clean the gauge and surface of work piece. - Hold the work piece on angle plate. - Open the lock screws. - Set the height gauge on stud to check it with fine adjusting screw. - Lock the measuring head. - Record the reading. | - Classification of Vernier instruments. - Design of linear measurement instruments. - Explain basic principles of design of Vernier height gauge. - Understand dimensioning. Practical Activity: Measure height on a given work piece by using Vernier depth gauge. | Total 6.0 Hrs. Theory: 2.0 Hrs. Practical: 4.0 Hrs. | - Height Vernier. | Class Room and Workshop |
| - Take measurement with Vernier depth gauge | Trainee would be able to - Clean the gauge and surface of work piece. - Hold the work piece on sample plate. - Open the lock screws. - Insert the gauge inside the work piece with fine adjusting screw. - Lock the measuring head. - Record the reading. | - Classification of Vernierinstruments. - Design of linear measurement instruments. - Explain basic principles of design of Vernier depth gauge. - Understand dimensioning. Practical Activity: Measure depth on a given work piece by using Vernier depth gauge. | Total 6.0 Hrs. Theory: 2.0 Hrs. Practical: 4.0 Hrs. | - Depth Vernier. | Class Room and Workshop |

### @Measure angles with angle measuring Instruments.

Objective:This competency standard covers the skills and knowledge required to take measurement with Bevel protractor, Measure angles with Vernier Bevel protractor, Dial protractor, Steel protractor and Sine bar.

Duration: 15 Hours			Theory: 5 Hours			Practice: 10 Hours			Credit Hours: 1.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with Bevel protractor. | Trainee would be able to - Clean the instrument and work piece. - Set the work piece and protractor as per requirement. - Make adjustment of the protractor for accurate measurement. - Record and tabulate the results. | - Explain the Angles and their supplements. - Describe Angular measurement techniques. - Explain basic requirements of angular measurements. - Classify instruments available for Angular Measurement. - Explain basic principle of a protractor and its extension as the universal bevel protractor. Practical Activity: Measure the angle of a work pieceusing a universal bevel protractor in clockwise direction, by reading angle directly from the scale | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Bevel protractor. | Class Room and Workshop |
| - Measure angles with Vernier Bevel protractor. | Trainee would be able to - Set the protractor on work piece properly. - Read the Vernier scale. - Measure and record the angle for clock wise and counter clock wise directions. | - Explain basic principle of vernierbevel protractor. - Explain basic requirements of angular measurements. - Describe use angle gauges to set them accurately to the required angle. Practical Activity: Measure the angle ona given work piece with Vernier Bevel protector. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Vernier bevel protractor. | Class Room and Workshop |
| - Measure angles with Dial protractor. | Trainee would be able to - Set the protractor on work piece properly. - Read the dial scale. - Measure the angle accurately. | - Explainbasic principle of Dial protractor. - Explain Angularmeasurement techniques. - Define Angles. Practical Activity: Measure the angle on given work piece with Dial protectors. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Dial protractor. | Class Room and Workshop |
| - Measure angles with Steel protractor. | Trainee would be able to - Set the protractor on work piece. - Measure the angle. | - Explain basic principle of steel protractor. - Explain Angular measurement techniques. - Define Angles. Practical Activity: Measure the angle on a given work piece with steel protectors. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Steel protractor. | Class Room and Workshop |
| - Measure angles with Sine bar. | Trainee would be able to - Construct a coil with hollow iron. Select proper size Sine Bar. - Set the Sine Bar with gauge block to measure taper. - Calculate tapper angle accurately. | - Describe measure of angles using the sine principle. - Describe Measuring of Unknown Angles with Sine Bar. - Explain Sine Blocks, Sine Plates, and Sine Tables - Explain the use ofsine bar, sine block, sine plate, and sine centre. Practical Activity: What is the maximum recommended angleto which a Sine bar can be set? Measure angle with sine barand discuss therelationship between the angle being set and the error of measurement for a Sine bar. | Total 3.0 Hrs. Theory: 1.0 Hrs. Practical: 2.0 Hrs. | - Sine bar. - Sine block. - Sine plate. - Sine table. - Sinecentre. | Class Room and Workshop |

### @Measure Threads and Gears

Objective:This competency standard covers the skills and knowledge required to Measure threads with thread gauges, Measure gear dimensions with gear tooth calliper and Measure gear dimensions with gear testing machine.

Duration: 18 Hours			Theory: 6 Hours			Practice: 12 Hours			Credit Hours: 1.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure threads with thread gauges. | Trainee would be able to - Place suitable sized wire in thread of work piece. - Take the dimensions. - Note pitch of thread and calculate depth of thread by using formula. | - Define Threads and their types. - Explain basic principles of measurement of screw threads. - Describe geometry of screw threads. - Explain measurement principles of major diameter, minor diameter, effective diameter, pitch, angle, and form of screwthreads. - Describe thread-measuring instruments such as benchmicrometer, floating carriage micrometer, and pitch-measuring machine. - Discuss types and use of thread gauges for screw threadinspection. - Explain formulas for thread depth Practical Activity: Inspect a metric screw thread by using two-wire method in order to measureits effective diameter, pitch, diameter of thebest-size wire and distance over the wires. Determine the effectivediameter of the screw thread. | Total 6.0 Hrs. Theory: 2.0 Hrs. Practical: 4.0 Hrs. | - Thread gauge. - Plug screw gauges. - Ring screw gauges. - Working gauges. - Inspection gauges. - Master gauges. | Class Room and Workshop |
| - Measure gear dimensions with gear tooth caliper. | Trainee would be able to - Adjust the jaws of calliper into the tooth of gear work piece. - Measure the tooth thickness, cordials addendum and cordials thickness. - Compare the measured values with theoretical results. | - Describe measurement of tooth thickness. - Explain basic principles of measurement of gears. - Describe geometry of spur gears. - Explain techniques used for the measurement of spur gears. - Classify gear-measuring instruments/ testers. - Explain Gears, terminologies related to gears and its types. Practical Activity: Determine chordal height of a gear for which number of teeth, addendumcircle diameter, andpressure angle data is given: by using gear tooth caliperduring measurement. | Total 6.0 Hrs. Theory: 2.0 Hrs. Practical: 4.0 Hrs. | - Gear tooth calliper. | Class Room and Workshop |
| - Measure gear dimensions with gear testing machine. | Trainee would be able to - Identify components of gear inspection machines and draw schematic. - Measure the dimensions of spur, bevel, helical with gear testing machine. | - Describe geometry of gears. - Composite method of gear inspection. - Classify gear-measuring instruments/ testers. - Explain Gears, terminologies related to gears and its types. Practical Activity: Use The Parkinson gear tester (i.e. a composite gear testing machine) to measure the composite errors, such as: errors due to run out, tooth-to-tooth spacing, and profile variations. | Total 6.0 Hrs. Theory: 2.0 Hrs. Practical: 4.0 Hrs. | - Gear Inspection machine. - Gear testing machine. - Parkinson Gear Tester. | Class Room and Workshop |

## Basic Engineering Drawing

| - Construct multi view drawing of Simple Bearing. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of simple bearing - Draw front view of simple bearing - Draw side view of simple bearing | - Explain Multi view drawing of Simple Bearing. - Define object symbols and schematic. Practical Activity - Perform to construct multi view drawing of Simple Bearing. | Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| --- | --- | --- | --- | --- | --- |
| - Construct multi view drawing of Open Bearing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of open bearing - Draw front view of open bearing - Draw side view of open bearing | - Explain Multi view drawing of Simple Bearing. - Define object symbols and schematic. Practical Activity - Perform to construct multi view drawing of open Bearing. | Total: 05Hrs Theory: 01Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch prism | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch prism | - Explain Prism, Cone and pyramid. - Define 3D pattern development through parallel line method. Practical Activity - Perform to construct a 3D pattern of prism on given dimensions i.e. L x W x H. | Total: 03Hrs Theory: 01Hrs Practical: 02Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch cone | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Start with a horizontal oval - draw the two sides of a triangle which meets at a common point | - Explain Prism, Cone and pyramid. - Define 3D pattern development through radial line method. Practical Activity - Perform to construct a 3D pattern of cone on given dimensions i.e. Base Dia x H. | Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Draw pyramid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch pyramid | - Explain Prism, Cone and pyramid. - Define 3D pattern development through radial line method. Practical Activity - Perform to construct a 3D pattern of cone on given dimensions i.e. Base size x H. | Total: 03Hrs Theory: 01Hrs Practical: 02Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |

## Microprocessor and Microcontroller

4.	List of Personal Protective Equipment

(FOR A CLASS OF 25 STUDENTS)

| Name of Trade |  | Mobile Phone Technician |  |
| --- | --- | --- | --- |
| Duration of Course |  | 12 Months |  |
| Sr. # |  | Description |  | Quantity |
| 1. |  | Safety gloves, |  | 30 |
| 2. |  | Appropriate safety glasses, |  | 30 |
| 3. |  | Breathing apparatus, |  | 30 |
| 4. |  | Fire extinguishers, |  | 30 |
| 5. |  | Fire blankets, |  | 30 |
| 6. |  | Respirators, masks, |  | 30 |
| 7. |  | Fire hoses, |  | 04 |
| 8. |  | First aid kits, stretchers and other related tools and gear |  | 10 |

# 5. Complete List of Tools, Equipment, Machines and Consumables

Tools

| S# | Description | Quantity |
| --- | --- | --- |
|  | Precision screw driver | 25 |
|  | Screw driver kit | 25 |
|  | Tweezers | 25 |
|  | Brushes | 25 |
|  | Blade cutter | 25 |
|  | Nose cutter | 25 |
|  | Point cutter | 25 |
|  | Cutter plier | 25 |
|  | Long Nose plier | 25 |
|  | PCB holder | 25 |
|  | Hard tweezers | 25 |
|  | Magnifying glass | 25 |
|  | Opener’s kit | 25 |
|  | Suction openers | 25 |
|  | Magnifying lamp | 25 |
|  | Wrist wire | 25 |
|  | Clipping tools(clips) | 25 |
|  | seizer | 25 |
|  | Steel wire | 25 |
|  | Rubber pads | 25 |
|  | Glass openers | 25 |

Equipment for Hardware

| S# | Description | Quantity |
| --- | --- | --- |
|  | Digital Multi meter | 25 |
|  | Oscilloscope | 5 |
|  | Soldering Iron | 25 |
|  | Soldering Station/SMD work station | 25 |
|  | Digital variable Power supply | 5 |
|  | B. G. A. Kit | 25 |
|  | Ultrasonic PCB Cleaner | 5 |

Equipment for Display repair

| S# | Description | Quantity |
| --- | --- | --- |
|  | Hot gun | 25 |
|  | Smoke Absorber | 5 |
|  | Hot air blower | 5 |
|  | Hot plate display separator | 5 |
|  | Freezer for separation of OLED | 5 |
|  | Flex binding machine | 5 |
|  | Mini electric grinder kit (EMC) | 5 |
|  | UV lamp | 5 |
|  | Glass laminating machine | 5 |
|  | De-bubbler machine | 5 |
|  | PCB Owen | 5 |

Equipment for Software

| S# | Description | Quantity |
| --- | --- | --- |
|  | Octopus Box (with cable Samsung + LG) | 5 |
|  | J Tag | 5 |
|  | EFT Dongle | 5 |
|  | CM2 Dongle | 5 |
|  | Sigma Key | 5 |
|  | ATF Dongle | 5 |
|  | ZXW (Hardware) Schematic diagram Dongle | 5 |

Consumables

| S# | Description | Quantity |
| --- | --- | --- |
|  | Soldering wire | 25 roll |
|  | Gloves | 25 Nos |
|  | Jumper wire |  |
|  | IC Paste |  |
|  | Solder paste |  |
|  | Cleaning sponge | 25 |
|  | Paste flux |  |
|  | Cleaning cloth |  |
|  | Chemical for washing mobile phone |  |
|  | UV Gum |  |
|  | Double tape |  |
|  | Heat resistance tape |  |
|  | Adhesive Glue |  |
|  | Thinner |  |
|  | CTC |  |
|  | Shiner |  |
|  | Rubber adhesive glue |  |
|  | WD-40 |  |
|  | Thinner |  |
|  | Cotton |  |

List of Stationary

| S# | Description | Quantity |
| --- | --- | --- |
|  | Handbooks | 5 |
|  | Design books | 5 |
|  | Pencils | 30 |
|  | Rubber | 30 |
|  | Sharpeners | 30 |
|  | Paper Cutter | 5 |
|  | Seizers | 5 |
|  | Colors | 5 each color (Read, Green, Yellow, Blue, white and Black) |
|  | White charts | 50 Nos |
|  | Brown sheets | 100 Nos |
|  | White board markers | 5 Doz |
|  | Permanent markers | 2 Dozen |
|  | File cover and files | 50 Nos. |

Learning hours in training provider premises,

Training workshop, laboratory and on-the-job workplace