National Vocational Certificate Level -03 in Mechatronics Technology

(Curriculum)

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1. 	Introduction	8

1.1.	Purpose of the training program:	9

1.2.	Overall objectives of training program	9

1.3.	Competencies to be gained after completion of course	10

1.4.	Entry level of trainees	20

1.5.	Minimum qualification for teachers	20

1.6.	Medium of instruction	20

1.7.	Duration of the course:	21

1.8.	Description and structure of the course	22

2.	Overview of the Curriculum for Mechatronics Technology	30

3.	Detail of Modules.	32

3.1. 	Maintain Occupational Health and Safety:	32

3.1.1.	Maintain Health and Safety	32

3.1.2.	Perform safety practices at workplace	37

3.1.3.	Develop professionalism.	41

3.2.	Basic Machining	46

3.2.1.	Perform Metal / Bench Work	46

3.2.2.	Perform general machining	51

3.2.3.	Perform cutting on Metal Circular/Power Heck Saw	54

3.2.4.	Perform tool sharpening	56

3.2.5.	Perform Basic Lathe Machine Operations	59

3.2.6.	Perform Drilling Machine Operations	67

3.2.7.	Perform Shaper, Planar and Slotter Machining Operations	70

3.2.8.	Perform Milling Operations	75

3.3.	Electrical Essentials & Networks	87

3.3.1.	Operate Measuring Instruments.	87

3.3.2.	Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.	90

3.3.3.	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence	93

3.3.4.	Construct Electromagnet to See Various Effects & Verify Faradays Law	96

3.3.5.	Verify Series and Parallel Combination of Capacitors & Determine Break down Voltage of Capacitor	100

3.3.6.	Maintain Tools & Equipment.	102

3.3.7.	Make Cable/Wire Joints	104

3.3.8.	Prepare and Install Distribution Boards	107

3.3.9.	Carryout Basic Electrical Installation	110

3.3.10.	Install Single Phase Electrical Wiring	113

3.3.11.	Perform Testing of Electrical Wiring	118

3.3.12.	Operate Single Phase Transformer	123

3.3.13.	Perform Tests on Single Phase Transformer	126

3.4.	Metrology	129

3.4.1.	Take measurements with graduated tools	129

3.4.2.	Take measurements with combination set	132

3.4.3.	Perform Leveling.	135

3.4.4.	Take measurements with gauges	137

3.4.5.	Perform measurements through Micrometer	139

3.4.6.	Measure dimensions with Vernier tools	142

3.4.7.	Measure angles with angle measuring Instruments.	144

3.4.8.	Measure Threads and Gears	147

3.5.	Basic Engineering Drawing	150

3.5.1.	Perform Basic Manual Drawing.	150

3.5.2.	Construct different Engineering Curves	159

3.5.3.	Construct Multi-View Drawings.	171

3.5.4.	Draw basic drawing using CAD.	180

3.5.5.	Develop 2D using CAD software.	182

3.5.6.	Develop 3D drawing using CAD software.	184

3.6.	Welding	187

3.6.1.	Interpret Part and Assembly Drawing	187

3.6.2.	Evaluate the Use of Hand and Power Tools	190

3.6.3.	Perform Pre-Welding Operations	193

3.6.4.	Perform Oxy Acetylene Welding	196

3.6.5.	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)	198

3.6.6.	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)	202

3.6.7.	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	205

3.6.8.	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	212

3.6.9.	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	219

3.6.10.	Flux Cored Arc Welding (FCAW)	226

3.6.11.	Perform Soldering and Brazing Operations	230

3.7.	Microprocessor and Microcontroller	232

3.7.1.	Microprocessor applications	232

3.7.2.	Identify Microcontroller Types and its Architecture	234

3.7.3.	Interface Microcontroller with system	236

3.7.4.	Carryout various Microcontroller Applications	239

3.7.5.	Apply Microcontroller to build Control System	242

3.8.	Analog Electronics	244

3.8.1.	Identify Basic Electronics Components	244

3.8.2.	Design Rectifier using Diode	247

3.8.3.	Carry out Diode Application	249

3.8.4.	Implement Bipolar Junction Transistor (BJT) in Different Applications	250

3.8.5.	Implement Field Effect Transistor (FET) in Different Applications	255

3.8.6.	Implement Thyristor Family in Various Application	258

3.8.7.	Application of Operational Amplifier	260

3.9.	Computer Programming	263

3.9.1.	Installation of C++, IDE for Arduino and Python	263

3.9.2.	Carryout Basic C++ Programming	266

3.9.3.	Carryout Basic Programming using python	269

3.9.4.	Perform basic programming using microcontrollers	273

3.10.	Computer Integrated Machines	276

3.10.1.	Perform CNC Machine Operations	276

3.10.2.	Perform CNC Lathe Operations	279

3.10.3.	Perform CNC Milling Operations	286

3.11.	Machines and Mechanism	293

3.11.1.	Demonstrate Simple Mechanisms	293

3.11.2.	Perform Kinematic Analysis of Simple Mechanisms	296

3.11.3.	Demonstrate Mechanical Power Transmission Elements.	298

3.11.4.	Perform Kinematic Analysis of Mechanical Power Transmission Elements.	300

3.12.	AC & DC machines	303

3.12.1.	Verify Basic Laws of Electrical Machines.	303

3.12.2.	Analyze an Alternator	309

3.12.3.	Analyze Single Phase Motors	313

3.12.4.	Analyze Special Purpose Motors	316

3.12.5.	Analyze Three Phase Motors.	319

3.12.6.	Analyze Dc Generators	330

3.12.7.	Perform Tests on DC Generators	335

3.12.8.	Analyze Dc Motors.	340

3.12.9.	Perform Tests On DC Motors.	344

3.13.	Digital Electronics	353

3.13.1.	Verify Truth Tables of Digital Gates	353

3.13.2.	Construct & Verify Combinational Logic Circuit	357

3.13.3.	Construct and Verify Function of Flip Flops	361

3.13.4.	Construct Multi vibrator with 555 IC	363

3.13.5.	Construct Shift Registers and Counters with the Help of Flip Flops	365

3.14.	Sensor & Actuators	367

3.14.1.	Test Sensors and Transducers.	367

3.14.2.	Calibrate Sensors and Transducers.	376

3.14.3.	Interface Sensors and Transducers.	394

3.14.4.	Test Actuators	399

3.14.5.	Interface and control Actuator with sensors.	404

3.15.	Programmable logic controllers (PLCs)	409

3.15.1.	Install PLC software and Simulator	409

3.15.2.	Interface PLC with system (1)	416

3.15.3.	Program PLC using Ladder Logic	418

3.15.4.	Commissioning and Testing PLC	422

3.15.5.	Commissioning and Testing PLC	427

3.15.6.	Carry out Industrial Automation on PLC	430

3.15.7.	Interface PLC with system (2)	434

3.16.	Designing of Mechanical Members	438

3.16.1.	Calculate Stresses in Machine Parts	438

3.16.2.	Calculate diameter of cylinder for hoop and longitudinal stresses	441

3.16.3.	Calculate thickness and diameter of spherical shell for circumferential stresses	443

3.16.4.	Design welded joints for transverse and parallel fillet under static and fatigue loading	447

3.16.5.	Calculate stresses due to initial tightening and external load on screws	454

3.16.6.	Design dimension of square and rectangular keys	458

3.16.7.	Design shafts subjected to twisting moment	462

3.16.8.	Design flange coupling for specific torque	469

3.17.	Industrial Process Control & Instrumentation	472

3.17.1.	Design & Implement a process control	472

3.17.2.	Install Transducer & Transmitters used in Industrial Applications	475

3.17.3.	Install the process Regulators	478

3.17.4.	Install the Variable Frequency Drive (VFD)	481

3.17.5.	Install Pneumatic & Hydraulic systems	483

3.18.	Robotics	486

3.18.1.	Identify Various Parts of Robots	486

3.18.2.	Identify Various Parts of Robot Arm	490

3.18.3.	Develop Robot program	493

3.18.4.	Develop robotic arm structure	496

3.18.5.	Test the Robot	499

3.19.	Mechatronic Projects	502

3.19.1.	Access Control System using RFID Reader RC522	502

3.19.2.	Finger Print Sensor Based Biometric Attendance System	504

3.19.3.	Humidity and Temperature Monitoring System	506

3.19.4.	Smart Stick for Blinds	508

3.19.5.	Perform Line Following Robot	510

3.19.6.	Perform Smart Phone Control Robot Using Bluetooth	512

3.19.7.	Perform IoT Controlled based home automation system	515

3.19.8.	Perform Smart Phone Control Door Lock System	518

3.19.9.	Perform Security System Using Motion Detection	521

3.19.10.	Perform Water-level Detection in Tank	523

3.19.11.	Perform Mini CNC 2D Plotter	525

3.19.12.	Perform Voice Controlled Robot	527

3.20.	Hydraulics & Pneumatics	529

3.20.1.	Operate Hydraulic Bench and Its Functions	529

3.20.2.	Calibrate Bourdon Tube and Diaphragm Pressure Gauge	532

3.20.3.	Operate Hydraulic Press	534

3.20.4.	Verify Bernoulli’s Equation	537

3.20.5.	Analyze the performance of Pumps	540

3.20.6.	Construct circuit for Double Acting Hydraulic Cylinder	542

3.20.7.	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder	545

3.20.8.	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking	547

3.20.9.	Set Speed and Direction of Hydraulic Motor	552

3.20.10.	Operate Pressure Measuring Instruments	554

3.20.11.	Operate Flow Regulating Devices	560

4.	List of Personal Protective Equipment	565

5. Complete List of Tools, Equipment, Machines and Consumables	566

# 1. 	Introduction

Mechatronics is an interdisciplinary field that combines elements of engineering, machining, information technology, robotics, and electronics. It synchronize these diverse fields for the purpose of designing, fabrication and field testing of the production lines and manufacturing processes in order to optimize their performance and efficiencies. In this field several mechanical systems such as hydraulic, pneumatic, camera, electric motor and engines are interface, configure and combine in order to perform the tasks with increase efficiency and performance. Since several different mechanical, electrical and optical systems are inference, the continual operational maintenance, configuration and upgradation is required which need trained main power in the field of mechatronics in order to run the industrial processes and machines with any system failure or breakdown.

Typically, a mechatronics technologist need to perform several task such as standalone operation / testing of the mechanical and electronics systems in order to diagnose the faulty parts. These task may attribute to the checking, installation and configuration of electronic circuits, motor drives, hydraulics/pneumatic moving system. The industry is lacking the expert technologist who has sound knowledge and command over all mechanical and electronics systems especially when these are interface and working in the close loop environment.

Mechatronics technologists can work in several fields, but they are commonly found in the manufacturing and automation sector. Several industrial process and production lines can be optimize by implementing fine mechanical precision in environmental conditions that may be dangerous or inefficient  for human hands and can be perform optimally by adding mechatronic solutions.Mechatronics has had a measurable impact both on the speed with which new products can be assembled.

The professionals that are skilled in this area of mechatronics are in high demand on production lines that range from projects as large as automotive vehicle manufacturing to the smallest microelectronic products. For instance, blood pressure monitoring device has pneumatic pump, a sensor to sense the blood pressure and electronics / microprocessor based circuit to measure the blood pressure. Similarly, several home appliances such as washing machines, ovens and refrigerators are now equipped with sophisticated mechatronics systems in order to optimize their products in terms of performance and electrical power consumption. Therefore, to repair and maintain such home appliances, the traditional technician are fail to work because of their training explicitly in the field of mechanical, electrical or electronics. Being conscious of these facts, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechatronics Technology under National Vocational Qualifications Framework (NVQF). Followings are the competency standards which are   developed by the Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Mechatronics industries from public and private sector, Engineering Universities and Technical Education of the country.

  - Purpose of the training program:

The purpose of thetraining is to set the high professional standards for the Mechatronics technological sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of the skills attained through formal or informal pathways

- Improve the quality and effectiveness of the training and assessment for Mechatronics Technological sector

- Enabling / helping / facilitating the existing workforce to indulge themselves in new technologies and methods

  - Overall objectives of training program

The Mechatronics qualifications of level 5 consists of both the theoretical and practical details and having the following Occupations

1. Health & Safety

2. Basic Machining

3. Electrical Essentials & Networks

4. Metrology

5. Basic Engineering Drawings

6. Welding

7. Microprocessor and Microcontroller

8. Analog Electronics

9. Computer Programming

10. Computer Integrated Machines

11. Machines and Mechanism

12. AC & DC machines

13. Digital Electronics

14. Sensor & Actuators

15. Programmable logic controllers(PLCs)

16. Designing of Mechanical Members

17. Industrial Process Control & Instrumentation

18. Robotics

19. Mechatronic Projects

20. Hydraulics & Pneumatics

## Competencies to be gained after completion of course

Detail of the competency standards included in qualification are given below:

Health and Safety

0714E&A1.	Maintain Health and Safety

0714E&A2.	Perform safety practices at work place

0714E&A3.	Develop Professionalism

Basic Machining

0714E&A4.	Perform Metal / Bench Work

0714E&A5	Perform general machining

0714E&A6	Perform cutting on Metal Circular/Power Heck Saw

0714E&A7	Perform tool sharpening

0714E&A8	Perform Basic Lathe Machine Operations

0714E&A9	Perform Drilling Machine Operations

0714E&A10	Perform Shaper, Planar and Slotter Machining Operations

0714E&A11	Perform Milling Operations

Electrical Essentials & Networks

0714E&A12	Operate Measuring Instruments

0714E&A13	Verify Ohm’s & Kirchhoff’s Laws by Implementing Series/Parallel Circuits

0714E&A14	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

0714E&A15	Construct Electromagnet to See Various Effects & Verify Faradays Law.

0714E&A16 Verify Law of Combination of Capacitor & Determine Break Down Voltage of Capacitor

0714E&A17	Maintain Tools & Equipment

0714E&A18	Make Cable/Wire Joints

0714E&A19	Prepare and Install Distribution Boards

0714E&A20	Carryout Basic Electrical Installations

0714E&A21	Install Single Phase Electrical Wiring

0714E&A22	Perform Testing of Electrical Wiring

0714E&A23	Operate Single Phase Transformer

0714E&A24	Perform Tests on Single Phase Transformer

Digital Skills

0714E&A25	Install computer operating systems and hardware

0714E&A26	Operate digital media technology

0714E&A27	Perform computer operations

0714E&A28	Use computer applications

0714E&A29	Create user documentation

0714E&A30	Create technical documentation

0714E&A31	Create basic databases

0714E&A32	Use social media tools for collaboration and engagement

0714E&A33	E-Commerce- SEO (Search Engine Optimization)

0714E&A34	E-Commerce- SCM (Supply Chain Management)

0714E&A35	E-Commerce- Social Media Marketing

0714E&A36	Use digital devices

0714E&A37	Operate word-processing applications

0714E&A38	Operate spreadsheet applications

0714E&A39	Operate presentation packages

0714E&A40	Perform writing and editing tasks

Metrology

0714E&A41	Take measurements with graduated tools

0714E&A42	Take measurements with combination set

0714E&A43	Perform Levelling

0714E&A44	Take measurements with gauges

0714E&A45	Perform measurements through Micrometer

0714E&A46	Measure dimensions with Vernier tools

0714E&A47	Measure angles with angle measuring Instruments

0714E&A48	Measure Threads and Gears

Basic Engineering Drawings

0714E&A49	Perform Basic Manual Drawing

0714E&A50	Construct different Engineering Curves

0714E&A51	Construct multi-view drawings

0714E&A52	Develop 2D CAD drawings

0714E&A53	Develop 3D CAD drawing

Welding

0714E&A54	Interpret Part and Assembly Drawing

0714E&A55	Evaluate the Use of Hand and Power Tools

0714E&A56	Perform Pre-Welding Operations

0714E&A57	Perform Oxy Acetylene Welding

0714E&A58	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)

0714E&A59	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)

0714E&A60	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A61	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A62	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A63	Flux Cored Arc Welding (FCAW)

0714E&A64	Perform Soldering and Brazing Operations

Microprocessor and Microcontroller

0714E&A65.Design Microprocessor applications

0714E&A66	Identify Microcontroller Types and its Architecture

0714E&A67	Interface Microcontroller with system

0714E&A68	Carryout various Microcontroller Applications

0714E&A69	Apply Microcontroller to build Control System

Analogue Electronics

0714E&A70	Identify Basic Electronics Components

0714E&A71	Design a Rectifier Using Diode

0714E&A72	Carry Out Diode Application

0714E&A73	Implement Bipolar Junction Transistor (BJT) in Different Applications

0714E&A74	Implement Field Effect Transistors (FET) In Different Applications

0714E&A75	Implement (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in various application

0714E&A76	Design Operation Amplifier

Soft Skills

0714E&A77.	Develop workplace policy and procedures for sustainability

0714E&A78.	Manage meetings

0714E&A79.	Manage recruitment selection and induction processes

0714E&A80.	Manage personal work priorities and professional development

0714E&A81.	Manage workforce planning

0714E&A82.	Undertake project work

0714E&A83.	Identify and communicate trends in career development

0714E&A84.	Apply specialist interpersonal and counseling interview skills

0714E&A85.	Work safely in an office environment

0714E&A86.	Develop workplace documents

0714E&A87.	Prepare and implement negotiation

0714E&A88.	Maintain professionalism in the workplace

0714E&A89.	Organize schedules

Computer Programming

0714E&A90.	Installation of C++, IDE for Arduino and Python

0714E&A91.	Carryout Basic C++ Programming

0714E&A92.	Carryout Basic Programming using python

0714E&A93.	Perform basic programming using microcontrollers

Computer Integrated Manufacturing (CIM)

0714E&A94.	Perform CNC Machine Operations

0714E&A95.	Perform CNC Lathe Operations

0714E&A96.	Perform CNC Milling Operations

Machines and Mechanism

0714E&A97.	Demonstrate Simple Mechanisms

0714E&A98.	Perform Kinematic Analysis of Simple Mechanisms

0714E&A99.	Demonstrate Mechanical Power Transmission Elements

0714E&A100.	Perform Kinematic Analysis of Mechanical Power Transmission Elements

AC & DC Machines

0714E&A101.	Verify Basic Laws of Electrical Machines

0714E&A102.	Analyse an Alternator

0714E&A103.	Analyse Single Phase Motors

0714E&A104.	Analyse Special Purpose Motors

0714E&A105.	Analyse Three Phase Motors.

0714E&A106.	Analyze Dc Generators

0714E&A107.	Perform Tests on DC Generators

0714E&A108.	Analyze Dc Motors.

0714E&A109.	Perform Tests On DC Motors.

Digital Electronics

0714E&A110.	Verify Truth Tables of Digital Gates

0714E&A111.	Construct & Verify Combinational Logic Circuit

0714E&A112.	Construct and Verify Function of Flip Flops

0714E&A113.	Construct Multivibrator with 555 IC

0714E&A114.	Construct Shift Registers and Counters with the Help of Flip Flops

Sensors and Actuators

0714E&A115.	Test Sensors and Transducers

0714E&A116	Calibrate Sensors and Transducers

0714E&A117	Interface the Sensor & Transducers

0714E&A118	Test Actuators

0714E&A119	Interface and control Actuator with sensors

Programmable Logic Controllers (PLCS)

0714E&A120	Install PLC software and Simulator

0714E&A121	Interface PLC with system

0714E&A122	Program PLC using Ladder Logic

0714E&A123	Design and Test the PLC

0714E&A124	Carry out Industrial Automation on PLC

Designing of Mechanical Members

0714E&A125	Calculate Stresses in Machine Parts

0714E&A126	Calculate diameter of cylinder for hoop and longitudinal stresses

0714E&A127	Calculate thickness and diameter of spherical shell for circumferential stresses

0714E&A128	Design welded joints for transverse and parallel fillet under static and fatigue loading

0714E&A129	Calculate stresses due to initial tightening and external load on screws

0714E&A130	Design dimension of square and rectangular keys

0714E&A131	Design shafts subjected to twisting moment only

0714E&A132	Design flange coupling for specific torque

Industrial Process Control & Instrumentation

0714E&A133	Design & Implement a process control

0714E&A134	Install Transducer & Transmitters used in Industrial Applications

0714E&A135	Install the process Regulators

0714E&A136	Install the Variable Frequency Drive (VFD)

0714E&A137	Install Pneumatic & Hydraulic systems

Robotics

0714E&A138	Identify Various Parts of Robots

0714E&A139	Identify Various Parts of Robot Arm

0714E&A140	Develop Robot program

0714E&A141	Develop robotic arm structure

0714E&A142	Test the Robot

Mechatronic Projects

0714E&A143	Access Control System using RFID Reader RC522

0714E&A144	Finger print Sensor Based Biometric Attendance System

0714E&A145	Humidity and Temperature Monitoring System

0714E&A146	Smart Stick for Blinds

0714E&A147	Perform Line Following Robot

0714E&A148	Perform Smart Phone Control Robot Using Bluetooth

0714E&A149	Perform IoT Controlled based home automation system

0714E&A150	Perform Smart Phone Control Door Lock System

0714E&A151	Perform Security System Using Motion Detection

0714E&A152	Perform Water-level Detection in Tank

0714E&A153	Perform Mini CNC 2D Plotter

0714E&A154	Perform Voice Controlled Robot

Hydraulic and Pneumatics

0714E&A155	Operate Hydraulic Bench and Its Functions

0714E&A156	Calibrate Bourdon Tube and Diaphragm Pressure Gauge

0714E&A157	Operate Hydraulic Press

0714E&A158	Verify Bernoulli’s Equation

0714E&A159	Analyse the performance of Pumps

0714E&A160	Construct circuit for Double Acting Hydraulic Cylinder

0714E&A161	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder

0714E&A162	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking

0714E&A163	Set Speed and Direction of Hydraulic Motor

0714E&A164	Operate Pressure Measuring Instruments

0714E&A165	Operate Flow Regulating Devices

Entrepreneurial Skill

0714E&A166	Develop Entrepreneurial Skills

0714E&A167	Apply project information management and communications techniques

0714E&A168	Apply project human resources management approaches

0714E&A169	Direct human resources management of a project program

0714E&A170	Develop a project management plan

0714E&A171	Maintain business resources

0714E&A172	Develop a sales plan

0714E&A173	Plan and implement business-to-business marketing

0714E&A174	Address customer needs

0714E&A175	Manage personal finances

0714E&A176	Solve problems which jeopardize safety and security

0714E&A177	Coordinate a work team

0714E&A178	Lead small teams

0714E&A179	Plan and organize work

0714E&A180	Develop teams and individuals

0714E&A181	Apply problem solving techniques in the workplace using critical thinking

0714E&A182	Manage human resource services

  - Entry level of trainees

The entry for D.A. E National Certificate level 5, in Mechatronics Technology are

- A person having National Vocational Certificate level 4, in Mechatronics Technology.

- A person having Matric certificate with Science subjects

## Minimum qualification for teachers

- B.Sc. / B.E. in Mechatronics Engineering.

- 4 Years B. Tech in Mechatronics Technology with 3 Years teaching experience.

- D. A. E. in Mechatronics Technology with 7 Years teaching experience

- Must be able to communicate effectively

## Medium of instruction

English, and Urdu.

## Duration of the course:

The proposed curriculum is composed of 182 modules that will be covered in 4,927Learning hours. Duration of the course is proposed to be Three years.

Level - 2 = 6 months.

Level - 3 = 6 months

Level - 4 = 1 Year

Level - 5 = 1 Year

The distribution of contact hours is given below:

Total.		4927 hours.

Theory.	1347hours (27.34 %)

Practical.	3580 hours (72.66 %)

## Description and structure of the course

Following is the structure of the course:

| LEVEL 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Install Computer Operating Systems And Hardware | 3 | Generic | 1 | 24 | 25 | 2.5 |
| Operate Basic computer functions | 3 | Generic | 1 | 24 | 25 | 2.5 |
| Develop Computer Application skills | 3 | Generic | 1 | 24 | 25 | 2.5 |
| Perform word-processing applications | 3 | Generic | 1 | 24 | 25 | 2.5 |
| Operate Spreadsheet Application | 3 | Generic | 1 | 24 | 25 | 2.5 |
| Operate Presentation Packages | 3 | Generic | 1 | 24 | 25 | 2.5 |
| Perform writing and editing skills | 3 | Generic | 1 | 24 | 25 | 2.5 |
| Perform Basic Manual Drawing | 3 | Technical | 7 | 21 | 28 | 2.8 |
| Construct different Engineering Curves | 3 | Technical | 10 | 30 | 40 | 4 |
| Construct multi-view drawings | 3 | Technical | 8 | 32 | 40 | 4 |
| Develop 2D CAD drawings | 3 | Technical | 12 | 18 | 30 | 3 |
| Develop 3D CAD drawings | 3 | Technical | 9 | 27 | 36 | 3.6 |
| Interpret Parts and Assembly Drawing | 3 | Technical | 4 | 12 | 16 | 1.6 |
| Evaluate the Use of Hand and Power Tools | 3 | Technical | 12 | 18 | 30 | 3 |
| Perform Pre-Welding Operations | 3 | Technical | 4 | 12 | 16 | 1.6 |
| Perform Oxy Acetylene Welding | 3 | Technical | 9 | 21 | 30 | 3 |
| Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting) | 3 | Technical | 4 | 12 | 16 | 1.6 |
| Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F) | 3 | Technical | 9 | 27 | 36 | 3.6 |
| Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 3 | Technical | 6 | 12 | 18 | 1.8 |
| Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 3 | Technical | 3 | 18 | 21 | 2.1 |
| Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 3 | Technical | 6 | 12 | 18 | 1.8 |
| Flux Cored Arc Welding (FCAW) | 3 | Technical | 6 | 12 | 18 | 1.8 |
| Perform Soldering and Brazing Operations | 3 | Technical | 12 | 18 | 30 | 3 |
| TOTAL |  |  | 128 | 470 | 598 | 59.8 |
| PERCENTAGE |  |  | 21.40468 | 78.59532 |  |  |

# Overview of the Curriculum for Mechatronics Technology

| Module Title and Aim | Learning Units | Theory Days/hours | Workplace Days/hours | Timeframe of modules |
| --- | --- | --- | --- | --- |

- Detail of Modules.

## 3.1. Maintain Occupational Health and Safety:

### Perform Basic Manual Drawing.

This module covers the skills and knowledge required to Draw single stroke capital vertical lettering, Draw single stroke capital inclined lettering, Draw horizontal, vertical and inclined lines, Use of compass, circles, half circles, radius, Drawing Centre   lines, centres, curves, and crossing of lines, Construction of parallel-lines, perpendicular, bisects line, angles and equal division of lines, Draw round corners, circles elements, quadrilaterals inside and outside circle and Construction of angles and triangles.

Duration:  28 Hours					Theory: 07Hours						Practical: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Draw single stroke capital vertical lettering | - Prepare Drawing sheet. - Select the tools. - Use Proper pencil for lettering with holding techniques. - Draw Boundaries lines as per standards. - Make title bar - Draw upper and lower lines for lettering according to standards. - Start Writing Vertical Lettering with different style like Gothic, Roman and free hand lettering. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. Practical Activity - Perform to draw single stroke capital vertical lettering. | Total:4Hrs Theory:1 Hrs Practical:3 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-Square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw single stroke capital inclined lettering. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Draw upper and lower lines for lettering according to standards. - Start Writing inclined Lettering with different style like Gothic, Roman and free hand lettering. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. Practical Activity: - Perform to draw single stroke capital inclined lettering. | Total:4Hrs Theory:1 Hrs Practical:3 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-Square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw horizontal, vertical and inclined lines. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw lines at 30°, 45°, 60°, 90° and 120° angles. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. Practical Activity: - Perform to construct vertical, horizontal, angular lines. | Total:4Hrs Theory:1 Hrs Practical:3 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw circles, half circles, radius with compass | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia meter circles and half circles. | - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Define General rules for lettering - Describe drawing equipment’s - Define Basic lines - Describe Introduction to geometry - Define Introduction to sketching techniques. - Describe Techniques of sketching straight lines in different directions. Practical Activity: - Perform to draw circles, half circles, radius with compass | Total:4Hrs Theory:1 Hrs Practical:3 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw Lines | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Centre lines, - Draw parallel-lines, - Draw perpendicular & bisects line, - Draw equal division of lines - Make different angle curves. - Draw crossing line | - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. - Define Application of lines. - Define Introduction to sketching techniques. - Describe Techniques of sketching straight lines in different directions. Practical Activity: - Construct a different weight lines and different types of lines. | Total:4Hrs Theory:1 Hrs Practical:3 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw round corners, circles elements, quadrilaterals inside and outside circle. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia circles. - Make inside and outside different types of diagrams that touch the circles at the tangent points. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. - Define Application of lines. - Describe Introduction to geometry - Define Introduction to sketching techniques. - Describe Techniques of sketching straight lines in different directions. Practical Activity: - Construct a hexagon with inscribed circle or circumscribed circle method. | Total:4Hrs Theory:1 Hrs Practical:3 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Construct angles and triangles | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Equilateral Triangle, Isosceles triangle, Scalene Triangle, Right Triangle, Obtuse Triangle, Acute Triangle. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. - Define Application of lines. - Describe Introduction to geometry - Define Introduction to sketching techniques. - Describe Techniques of sketching straight lines in different directions. - Define Triangles, Quadrilateral, and Polygons. Practical Activity: - Perform to construct angles and triangles. | Total:4Hrs Theory:1 Hrs Practical:3 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |

### Construct different Engineering Curves

This module covers the skills and knowledge required to Construct inscribe and circumscribe figures, Construct a pentagon, Hexagon and Octagon by circumscribe method, Construct a pentagon, Hexagon and Octagon by inscribe method, Construct a Tangents of circles (Inside & Outside) When the centre of the given circle is known and when the circle of centre is not known, Construct an Ellipse by Concentric Circle Method, Rectangle Method, Oblong Method, Arcs of Circle Method, Rhombus Method and Basic Locus Method, Construct a parabola curve by Rectangle Method, Method of Tangents( Triangle Method) and Basic Locus Method, Construct a hyperbola curve, Construct a Archimedean Spiral curve, Construct a involutes curve of square rectangle hexagon and circle and Construct of cycloid, epicycloids, and hypocycloid.

Duration:  40Hours					Theory: 10 Hours						Practical: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Construct inscribe and circumscribe figures. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw square, triangle and hexagon according to dimension. | - Describe techniques of sketching straight lines in different directions - Define Triangles, Quadrilateral and Polygons - Describe circular arc using different line method - Describe circular arc - Define types of Geometric Shape Practical Activity - Perform to construct pentagon, Hexagon and Octagon by circumscribe method. | Total: 4 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct a pentagon, Hexagon and Octagon by circumscribe method. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon | - Describe techniques of sketching straight lines in different directions - Define Triangles, Quadrilateral and Polygons - Describe circular arc using different line method - Describe circular arc - Define types of Geometric Shape - Define two-dimensional shapes - Define three-dimensional shapes - Define types of Geometric Shape Practical Activity - Perform to construct pentagon, Hexagon and Octagon by circumscribe method. | Total: 4 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct pentagon, Hexagon and Octagon by inscribe method | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon. | - Describe techniques of sketching straight lines in different directions - Define Triangles, Quadrilateral and Polygons - Describe circular arc using different line method - Describe circular arc - Define types of Geometric Shape - Define two-dimensional shapes - Define three-dimensional shapes - Define types of Geometric Shape Practical Activity - Perform to Construct pentagon, Hexagon and Octagon by inscribe method | Total: 4 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct Tangents of circles (Inside & Outside) | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Tangents Inside of a circle when the center of the circle is known. - Draw Tangents Inside of a circle When the center of the circle is unknown - Draw Tangents outside of a circle When the centre of the circle is known - Draw Tangents outside of a circle When the centre of the circle is unknown | - Define methods of drawing Tangents & Normal - Define regular Polyhedrons - Define methods of drawing Tangents & Normal - Describe ellipse - Describe different methods of sketching ellipse - Describe parabola - Describe different methods of parabola - Describe hyperbola curve - Describe different methods of hyperbola curve. Practical Activity - Construct tangent on circle. | Total: 4 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct Ellipse | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw an Ellipse by Concentric Circle. - Draw an Ellipse by Rectangle Method - Draw an Ellipse by Oblong Method - Draw an Ellipse by Arcs of Circle Method - Draw an Ellipse by Rhombus Method. - Draw an Ellipse by Basic Locus Method | - Describe spiral curve - Describe involute curve - Describe cycloid - Describe epicycloids - Describe hypocycloid - Describe ellipse Practical Activity - Perform to Construct Ellipse. | Total: 4 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct a parabola curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a parabola curve by Rectangle - Draw a parabola curve by Method of Tangents( Triangle Method) - Draw a parabola curve by Basic Locus Method | - Describe parabola - Describe different methods of parabola - Describe spiral curve - Describe involute curve Practical Activity - Perform to construct parabolacurve. | Total: 04Hrs Theory: 01 Hr Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct a hyperbola curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a hyperbola curve. | - Describe hyperbola curve - Describe different methods of hyperbola curve. - Describe spiral curve - Describe involute curve Practical Activity - Perform to construct hyperbola curve. | Total: 4 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct a Archimedean Spiral curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw spiral curve. | - Describe cycloid - Describe epicycloids - Describe hypocycloid Practical Activity - Perform to Construct spiral curve. | Total: 04Hrs Theory: 01 Hr Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct involute curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw involute curve by square - Draw involute curve by rectangle - Draw involute curve by hexagon - Draw involute curve by circle. | - Describe cycloid - Describe epicycloids - Describe hypocycloid Practical Activity - Perform to construct involute curve. | Total: 4 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct of cycloid, epicycloid, and hypocycloid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw the generating circle and the base line equal to the circumference of the generating circle - Divide the circle and the base line in to equal number of parts - Complete the cycloid, epicycloids, and hypocycloid. | - Describe cycloid - Describe epicycloids - Describe hypocycloid Practical Activity - Perform to construct cycloid, epicycloid, and hypocycloid. | Total: 4 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |

### Construct Multi-View Drawings.

Objective: This module covers the skills and knowledge required to Sketch Orthographic projection 1st angle, Sketch Orthographic projection 3rd angle, Sketch Oblique Drawing, Construct multi view drawing of Simple Bearing, Construct multi view drawing of Open Bearing, Sketch prism, Sketch cone and Draw pyramid.

Duration:  40 Hours						Theory: 08 Hours						Practice: 32 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required |  |
| --- | --- | --- | --- | --- | --- |
| - Sketch Orthographic projection in 1st angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view | - Explain Orthographic projection 1st angle. - Define how to show hidden features in projection drawing. - Describe Front, Side and top views and its placement in 1st angle projection. - Describe 1st angle projection symbol. Practical Activity - Perform to draw 1st angle projection drawing on any given object/part. | Total: 5 Hrs Theory: 1 Hrs Practical: 4 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch Orthographic projection 3rd angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view | - Explain Orthographic projection 3rd angle. - Define how to show hidden features in projection drawing. - Describe Front, Side and top views and its placement in 3rdt angle projection. - Describe 3rd angle projection symbol. Practical Activity - Perform to draw 3rd angle projection drawing on any given object/part. | Total: 5 Hrs Theory: 1 Hrs Practical: 3Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch Oblique Drawing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts - Draw the front or side view of the object. | - Explain Oblique Drawing. - Define types of oblique drawing. - Describe uses of oblique drawing. - Explain general oblique. Practical Activity - Perform to draw oblique view drawing on any given object/part. | Total: 5 Hrs Theory: 1 Hrs Practical: 4 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct multi view drawing of Simple Bearing. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of simple bearing - Draw front view of simple bearing - Draw side view of simple bearing | - Explain Multi view drawing of Simple Bearing. - Define object symbols and schematic. Practical Activity - Perform to construct multi view drawing of Simple Bearing. | Total: 5 Hrs Theory: 1 Hrs Practical: 4 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct multi view drawing of Open Bearing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of open bearing - Draw front view of open bearing - Draw side view of open bearing | - Explain Multi view drawing of Simple Bearing. - Define object symbols and schematic. Practical Activity - Perform to construct multi view drawing of open Bearing. | Total: 05Hrs Theory: 01Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch prism | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch prism | - Explain Prism, Cone and pyramid. - Define 3D pattern development through parallel line method. Practical Activity - Perform to construct a 3D pattern of prism on given dimensions i.e. L x W x H. | Total: 5 Hrs Theory: 1 Hrs Practical: 4 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch cone | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Start with a horizontal oval - draw the two sides of a triangle which meets at a common point | - Explain Prism, Cone and pyramid. - Define 3D pattern development through radial line method. Practical Activity - Perform to construct a 3D pattern of cone on given dimensions i.e. Base Dia x H. | Total: 5 Hrs Theory: 1 Hrs Practical: 4 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Draw pyramid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch pyramid | - Explain Prism, Cone and pyramid. - Define 3D pattern development through radial line method. Practical Activity - Perform to construct a 3D pattern of cone on given dimensions i.e. Base size x H. | Total: 5 Hrs Theory: 1 Hrs Practical: 4 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |

## Develop 2D CAD Drawing

Objective: This module covers the skills and knowledge required to Develop and prepares 2D objects

Duration:  30 Hours					Theory: 12 Hours						Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Develop 2D Objects | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Save CAD drawing files in different file formats (DWG/IGS/etc., PDF, and JPG). - Create 2D Objects with given measurements - Edit 2D Objects to meet set standards | - Describe Basic Drawing Settings - Describe Unit setting - Describe Limits setting - Define User coordinate system Workspace setting - Describe Object Snap Settings - Describe Basic Commands and Concepts Angles and lines in CAD. - Define Differentiate between absolute, relative and polar system - Define DIMSTYLE and MTEXT commands - Describe HATCHING concepts in CAD - Describe Differentiate between CHAMFER and FILLET command - Describe Types of Array - Define OFFSET, CIRCLE and ROTATE short commands Practical Activity. - Create 2D drawing of given object/partby using CAD software. | Total: 15Hrs Theory: 06Hrs Practical:9Hrs | - Computer System - Computer table - CAD software. |  | Class Room or Demonstration Room Workshop |
| - Prepare Final Set of 2D Drawings | - Use appropriate command and tools to develop 2D Drawing - Develop 2D Drawing with given project specifications and measurements - Create title block layout as required - Plot drawing on scale according to required size and orientation | - Describe Zooming options - Describe Tools palettes window - Describe Design center - Define Scale and paper sizes Practical Activity. - Develop 2D Drawing with given project specifications and measurements | Total: 15Hrs Theory: 06Hrs Practical:9Hrs | - Computer System - Computer table - CAD software |  | Class Room or Demonstration Room Workshop |

## Develop 3D CAD drawing

Objective: This module covers the skills and knowledge required to Prepare to load machine and attachments, Load or assist with loading machine and attachments, Assist with securing machine and attachments, Unload or assist with unloading machine and attachments, Prepare rubber-tired machine for road travel and Drive rubber tired machine on public roads.

Duration:  36 Hours				Theory: 9 Hours				Practical: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - LU1. Develop 3D Objects | - Setup & save 3D drawing interface for required specifications. - Setup 3D user interface settings for required specifications. - Create 3D objects with given measurements. | - Describe 3D modeling in CAD - Describe 3D solids, - Describe surfaces, meshes, and Wireframe objects. Practical Activity - Develop 3D Objects on given object/part by using CAD software. | Total:12Hrs Theory: 03Hrs Practical: 09Hrs | - Computer System - Computer table - CAD software |  | Class Room or Demonstration Room Workshop |
| - LU2. Manipulate 3D objects using 3D Editing Tools | - Modify 3D objects in line with the requirements. - Make customized 3D models according to the requirement of given job. - Convert 3D Face objects into a single mesh objects. | - Describe Difference between Surface Modeling and Solid Modeling. - Define 3D face and Edges - Define Boolean operation concepts - Describe Subtraction - Describe Intersection - Describe Union - Define 3D Navigate control Practical Activity - Manipulate 3D objects by using 3D Editing Tools | Total:12Hrs Theory: 03Hrs Practical: 09Hrs | - Computer System - Computer table - CAD software |  | Class Room or Demonstration Room Workshop |
| LU3. Render 3D Model | - Apply material to required 3D Model as per given specification - Apply lights to get the requisite scene of required 3D model - Assign cameras to execute different views of required 3D Model. - Render and print the 3D model according to required size & orientation. - Apply texture to 3D model as per given specification. | - Describe functions of different camera settings. - Define Importance of scene creation - Describe preset views such as isometric, top, bottom, front, left, etc. - Describe perspective projection and parallel projection - Define Walk - Describe Constrained Orbit - Describe Material and light control - Describe Planner mapping - Describe Texture map - Describe Opacity control - Define Render context - Describe Render sampling Practical Activity - Perform Render a 3D Model | Total:12Hrs Theory: 03Hrs Practical: 09Hrs | - Computer System - Computer table - CAD software |  | Class Room or Demonstration Room Workshop |

- Welding

### ‘Interpret Part and Assembly Drawing

Objective: This competency standard covers the skills and knowledge required to Read and Understand Manufacturing Drawings, Interpret Welding Symbols, Recognize Material Specifications and Bill of Material (BOM)

Duration: 16 Hours			Theory:  4 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Interpret Manufacturing Drawings | Trainee will be able to: - Recognize basics of lines used in engineering drawings - Describe uses of lines in the engineering drawings - Recognize and explain orthographic and isometric views of a drawing - Identify the manufacturing requirements according to the drawings - Prepare job layout according to the manufacturing requirements | - Describe Basic elements of engineering drawing - Describe Drawing symbols - Explain the concept of Views and Projection System - Explain multi-view Projection System | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs |  | Class Room and workshop |
| - Interpret Welding Symbols | Trainee will be able to: - Recognize the basic and supplementary welding symbols used in manufacturing drawings - Identify and differentiate between the types of welds and joints - Identify the welding requirements according to the welding symbols given in the manufacturing drawings | - Explain Dimensioning techniques - Describe welding symbols - Define General/Dimensional tolerance - Define Angular tolerance - Define Geometric tolerance | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs |  | Class Room and workshop |
| - Recognize Material Specifications and Bill of Material (BOM) | Trainee must be able to: - Identify the material specifications according to manufacturing drawing - Identify the bill of material (BOM) according to the manufacturing drawing | - Explain Perspective View - Explain Exploded view - Explain Hidden view technique - Explain First angle projections - Explain Third angle projections | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs |  | Class Room and workshop |
| - Interpret assembly drawings | Trainee must be able to - Interpret the dimensional tolerances according to the manufacturing drawing - Identify the parts in an assembly drawing. | - Explain Perspective - Explain Exploded view - Explain Hidden view technique - Explain First angle projections - Explain Third angle projections | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs |  | Class Room and workshop |

### Evaluate the Use of Hand and Power Tools

Objective: This competency standard covers the skills and knowledge required to Put on handtools and Put on power tools

Duration: 30 Hours			Theory: 12 Hours				Practice:18  Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Maintain hand tools | Trainee will be able to: - Select the hand tools appropriate to the task requirements. - Use the hand tools to produce desired outcomes to the job specifications which may include finish, tension, size or shape. - Adhere all safety requirements to before, during and after use. - Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use. - Maintain the tools, including hand sharpening according to the standard operational procedures, principles and techniques. - Store the hand tools safely in the appropriate location according to the standard operational procedures and manufacturers recommendations. | - Applications of different hand and power tools in a general engineering context - Common faults and/or defects in hand and power tools - Procedures for marking unsafe or faulty tools for repair - Routine maintenance requirements for a range of hand and power tools - Storage location and procedures for a range of hand and power tools | Theory-06 Hrs Practical-09 Hrs Total- 15 Hrs |  | Class Room and workshop |
| - Maintain power tools | Trainee will be able to: - Select the power tools appropriate to the task requirements. - Use the power tools for a determined sequence of operations-which may include clamping, alignment and adjustment to produce desired outcomes-to job specifications which may include finish, size or shape. - Adhere all safety requirements to before, during and after use. - Identify and mark the unsafe or faulty tools before starting to use or work according to the designated procedures - Repeat the step P4 during and after completing the use - Maintain the tools, including hand sharpening according to standard operational procedures. - Store power tools safely in the appropriate location according to standard operational procedures and manufacturers’ recommendations. | - Benefits and limits of cutting and shaping metal with auxiliary equipment - Environmental benefits of maintaining auxiliary equipment - Clamping /securing methods - Adjustment / alignments to a range of power tools - Tool sharpening techniques for a range of power tools - Hazards and control measures associated with using hand and power tools | Theory-06 Hrs Practical-09 Hrs Total- 15 Hrs |  | Class Room and workshop |

    - Perform Pre-Welding Operations

Objective: This competency standard covers the skills and knowledge required to Set Welding Equipment, Prepare materials for welding, Cut and Prepare Edge/s of Base Materials and Prepare Welding Consumables

Duration: 16 Hours			Theory: 04 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set Welding Equipment | Trainee will be able to: - Adjust the pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch Make carburizing flame by increasing acetylene gas quantity - Make a neutral flame by adjusting both gases at same quantity - Make an oxidizing flame by increasing the oxygen gas quantity - Adjust pressure of both gas cylinders with the help of regulators - Select the correct size of the nozzle - Set the flame of welding torch as per standard | - Describe about welding torch - Identify Gas pressure regulators | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs |  | Class Room and workshop |
| - Prepare materials for welding | Trainee will be able to: - Select and obtain the required material/s as per job requirements - Select appropriate marking tools as per job requirements - Mark the area to be cut as per drawing / job requirements | - Explain temperature and its units - Describe pre heating - Explain importance of pre heating - Explain metal properties - Describe malleability | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs |  | Class Room and workshop |
| - Cut and Prepare Edge(s) of Base Materials | Trainee must be able to: - Select appropriate cutting equipment as per job requirements - Set-up cutting equipment as per manufacturer’s instructions / job requirements - Cut the base material as per job specifications and dimensions provided in the drawing - Prepare edges of the base materials as per drawing / WPS - Check dimensions of the prepared edges as per drawing / WPS - Select proper tools and chemicals for cleaning - Clean the edges of the base materials as per job requirements | - Describe types of grinder - Explain use of tri square | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs |  | Class Room and workshop |
| - Prepare Welding Consumables | Trainee must be able to - Select the relevant welding consumables as per job requirements/WPS - Prepare the consumables in accordance with required specifications | - Describe importance of filing - Describe the filler rod - Describe electrode baking oven - Describe purpose of flux | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs |  | Class Room and workshop |

    - Perform Oxy Acetylene Welding

Objective: his competency standard covers the skills and knowledge required to Practice of making Tee Joint, Practice of making Lap Joint and Practice of Making Butt Joint

Duration: 30 Hours			Theory: 09 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice of making Tee Joint | Trainee will be able to: - Take Work piece as per drawing - Straiten it with the help of hammer and anvil - Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other - Place the Bottom piece on work table and place the Top plate at 90 degree to each other as per drawing - Set the flame of welding torch as per standard - Complete the bead as per standard | - Explain positions of electrode according to work - Knowledge of setting the current on welding machine - Describe motion of electrode in ARC welding - Explain importance of gap between electrode and base metal | Theory-03 Hrs Practical-07 Hrs Total- 10 Hrs |  | Class Room and workshop |
| - Practice of making Lap Joint | Trainee will be able to: - Take Work piece as per drawing - Straiten it with the help of hammer and anvil - Grind the work pieces on grinding machine to prepare the edges flat - and parallel to each other - Place the Bottom piece on work table and place the Top plate along the marked line - Set the flame of welding torch as per standard - Complete the bead as per standard | - Describe use of tri square | Theory-03 Hrs Practical-07 Hrs Total- 10 Hrs |  | Class Room and workshop |
| - Practice of Making Butt Joint | Trainee must be able to: - Take Work piece as per drawing - Straiten it with the help of hammer and anvil - Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other - Place the work pieces parallel to each other - Set the flame of welding torch as per standard - Complete the bead as per standard | - Describe importance of cleanliness of surface to be welded | Theory-03 Hrs Practical-07 Hrs Total- 10 Hrs |  | Class Room and workshop |

### Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)

Objective: This competency standard covers the skills and knowledge required to cut metal by oxyacetylene Welding Manually, Cut metal by oxy-acetylene Welding Automatically, Cut metal by Grinding Manually and Cut metal by Grinding Automatically

Duration: 16 Hours			Theory: 04 Hours			Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Cut metal by oxy-acetylene Welding Manually | Trainee will be able to: - Take Work piece as per drawing - Straighten it with the help of hammer and anvil if required - Mark the work piece for cut - Set the flame of welding torch in oxidizing flame as per standard - Start cutting for one side of work piece - Maintain standard distance between welding torch nozzle and work piece - Complete the cut as per standard | - Explain metal marking gauges. - Explain various types of cutting processes. - Explain Gas cutting process. - Explain various cutting positions | Theory-01 Hrs Practical-03 Hrs Total- 4 Hrs | - Work piece - Hammer and Anvil - Marking tool / gauge - Oxygen cylinder - Acetylene gas cylinder - Pressure regulators - Cylinder key - Welding torch - Rubber hose pipe - Back fire arrester - Flash back arrester | Class Room and workshop |
| - Cut metal by oxyacetylene Welding Automatically | Trainee will be able to: - Take Work piece as per drawing - Straiten it with the help of hammer and anvil if required - Mark the work piece for cut - Place the work piece in auto feed machine - Set the flame of welding torch in oxidizing flame as per standard - Set cutting torch in Auto Feed machine - Set Feed Rate of machine - Start cutting for one side of work piece - Maintain standard distance between welding torch nozzle and work piece - Complete the cut as per standard | - Describe pre-heating - Describe motion of welding torch | Theory-01 Hrs Practical-03 Hrs Total- 4 Hrs | - Work piece - Hammer and Anvil - Marking tool / gauge - Oxygen cylinder - Acetylene gas cylinder - Pressure regulators - Cylinder key - Welding torch - Back fire arrester - Flash back arrester - Auto feed Machine | Class Room and workshop |
| - Cut metal by Grinding Manually | Trainee must be able to: - Take Work piece as per drawing - Straiten it with the help of hammer and anvil if required - Perform marking on work piece where cutting is required - Set disk grinder on grinding machine - Set r.p.m of disk grinding machine as per standard - Cut on marked line with disk grinder by hand | - Explain importance of gap between torch and base metal - Describe importance of cleanliness of surface to be cut. | Theory-01 Hrs Practical-03 Hrs Total- 4 Hrs | - Grinder Machine - Cutting grinding disks | Class Room and workshop |
| - Cut metal by Grinding Automatically | Trainee must be able to - Take Work piece as per drawing - Straiten it with the help of hammer and anvil if required - Perform marking on work piece where cutting is required - Set disk grinder on grinding machine - Set r.p.m of disk grinding machine as per standard - Set the guide blade of disk grinding machine as per requirement - Set feed of disk grinding machine - Cut on marked line with disk grinding machine | - Explain various cutting process by grinding. - Describe cutting disk types and grades. - Explain speed of grinder and its effects | Theory-01 Hrs Practical-03 Hrs Total- 4 Hrs | - Grinder Machine - Cutting grinding disks | Class Room and workshop |

### Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)

Objective: This competency standard covers the skills and knowledge required to Gas Welding on Mild Steel Plates (1F), Gas Welding on Mild Steel Plates (2F) and Gas Welding on Mild Steel Plates (3F)

Duration: 36 Hours			Theory: 9Hours			Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Gas Welding On Mild Steel Plates (1F) | Trainee will be able to: - Open the gas cylinder with the help of cylinder key - Adjust the pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch - Ignite acetylene gas with help of spark lighter - Open oxygen gas knob of welding torch - Set the work piece as per standard - Perform fore hand welding method - Perform the post welding operations | - Explain various types of GAS welding processes - Explain advantages of Gas welding - Describe the principle of Gas welding - Explain various welding positions - List Personal Protective Equipment - Demonstrate the method to correctly wear PPE - Explain safe working practices | Theory-03 Hrs Practical-09 Hrs Total- 12 Hrs | - Work piece - Marking tool / gauge - Oxygen cylinder - Acetylene gas cylinder - Pressure regulators - Cylinder key - Welding torch - Rubber house pipe - Back fire arrester - Flash back arrester | Class Room and workshop |
| - Gas Welding On Mild Steel Plates (2F) | Trainee will be able to: - Open the gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch - Ignite acetylene gas with help of spark lighter - Open oxygen gas knob of welding torch - Set work piece as per standard - Perform the welding in right hand welding method - Perform post welding operations | - Identify hazards associated with Gas welding and take remedial measures - Describe Welding procedure specifications (WPS) - Describe Method of Pre- heating of base metal - Describe Fillet lap joint - Describe Tee-fillet joint | Theory-03 Hrs Practical-09 Hrs Total- 12 Hrs | - Work piece - Marking tool - Oxygen cylinder - Acetylene gas cylinder - Pressure regulators - Cylinder key - Welding torch - Rubber house pipe - Back fire arrester - Flash back arrester | Class Room and workshop |
| - Gas Welding On Mild Steel Plates (3F) | Trainee must be able to: - Open gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch - Ignite acetylene gas with help of spark lighter - Open oxygen gas knob of welding torch - Set work piece as per standard - Perform welding in downward welding method - Perform post welding operations | - Describe Corner joint - Describe Butt joint - Describe Double J joint - Define Visual welding defects - Describe Welding codes and standards | Theory-03 Hrs Practical-09 Hrs Total- 12 Hrs | - Work piece - Marking tool - Oxygen cylinder - Acetylene gas cylinder - Pressure regulators - Cylinder key - Welding torch - Back fire arrester - Flash back arrester | Class Room and workshop |

### Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

Objective: This competency standard covers the skills and knowledge required to Prepare Welding Machine and Accessories for GMAW, Make Fillet Welds on Carbon Steel Plate, Make Groove Welds on Carbon Steel Plate, Make Fillet Welds on Carbon Steel Plate, Make Groove Welds on Carbon Steel Plate and Perform Post Welding Operations

Duration:  			18 Hours			Theory: 	06 Hours			Practice: 	12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Welding Machine and Accessories for GMAW and MIG | Trainee will be able to: - Identify the welding requirements from the job, welding procedure specifications and/or technical drawings - Prepare the GMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer instructions - Connect the welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications | - Explain various types of welding processes - Explain advantages of GMAW and MIG welding - Describe the principle of GMAW and MIG welding - List Personal Protective Equipment required for MIG welding and state their use - Demonstrate the method to correctly wear PPE | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode holder - Glass eye protector - Electrodes with flux - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Cut metal by oxyacetylene Welding Automatically | Trainee will be able to: - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain the gap between electrode and base metal as per standard practices - Carry out welding in Flat (1F) and Horizontal (2F) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Explain Specifications/ classification of electrode/s required for the job - Explain safe working practices to be followed while carrying out MIG welding - Identify hazards associated with MIG welding and take remedial measures - Define Electrical parameters like (voltage, current etc.) and their effects on weld | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode holder - Glass eye protector - Electrodes with flux - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Groove Welds on Carbon Steel Plate | Trainee must be able to: - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Flat (1G) and Horizontal (2G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check the root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of the visual welding defects | - Explain Welding techniques as per WPS/instruction sheet - Describe Welding procedure specifications (WPS) | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode holder - Glass eye protector - Electrodes with flux - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Fillet Welds on Carbon Steel Plate | Trainee must be able to - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain the gap between electrode and base metal as per standard practices - Carry out welding in Vertical (3F) and Overhead (4F) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Describe Method of Pre- heating of base metal - Explain Polarity setting according to standard specifications | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode holder - Glass eye protector - Electrodes with flux - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Groove Welds on Carbon Steel Plate | Trainee must be able to - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Vertical (3G) and Overhead (4G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Explain the factors to be considered in TIG welding like type and thickness of the base metal, current type and polarity, type of shielding gas to be used - Define Visual welding defects - Describe Welding codes and standards | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode holder - Glass eye protector - Electrodes with flux - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber |  |
| - Perform Post Welding Operation | Trainee must be able to - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required Knowledge & Understanding - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools / equipment / consumable materials in accordance with organization guidelines | - State the purpose of using shielding gas in TIG welding - Identify various gases / combination of gases for shielding | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode holder - Glass eye protector - Electrodes with flux - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber |  |

### Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

Objective: This competency standard covers the skills and knowledge required to Prepare Welding Machine and Accessories for GTAW, Make Fillet Welds on Carbon Steel Plate, Make Groove Welds on Carbon Steel Plate, Make Fillet Welds on Carbon Steel Plate, Make Groove Welds on Carbon Steel Plate and Perform Post Welding Operations

Duration:  			18 Hours			Theory: 	06 Hours				Practice: 	12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Welding Machine and Accessories for GTAW | Trainee will be able to: - Identify the welding requirements from the job, welding procedure specifications and/or technical drawings - Prepare the GTAW welding machine in with accordance with welding procedure specifications/ manufacturer´s instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions - Connect the welding machine to an independent power - supply - Set the polarity indicated in the welding procedure - specifications | - Explain the various types of welding processes - Explain advantages of GTAW - Describe the principle of TIG welding - Explain various welding positions - List Personal Protective Equipment required for the TIG welding and state their use | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Tungsten Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Fillet Welds on Carbon Steel Plate | Trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Flat (1F) and Flat (1G) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Demonstrate the method to correctly wear PPE - Explain Specifications / classification of electrode / s required for the job - Explain the safe working practices to be followed while carrying out TIG welding | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Tungsten Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Groove Welds on Carbon Steel Plate | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Horizontal (2F) and Horizontal (2G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Identify hazards associated with TIG welding and take remedial measures - Define Electrical parameters like (voltage, current etc.) and their effects on weld - Explain Welding techniques as per WPS/instruction sheet | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Tungsten Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Fillet Welds on Carbon Steel Plate | Trainee must be able to - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Vertical (3F) and Vertical (3G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Describe Welding procedure specifications (WPS) - Describe Method of Pre- heating of base metal - Describe Welding codes and standards | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Tungsten Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Groove Welds on Carbon Steel Plate | Trainee must be able to - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Overhead (4F) and Overhead (4G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure/job specifications requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - State the purpose of using shielding gas in TIG welding - Identify various gases/combination of gases for shielding | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Tungsten Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber |  |
| - Perform Post Welding Operations | Trainee must be able to - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines | - State the purpose of using shielding gas in TIG welding - Identify various gases / combination of gases for shielding | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Tungsten Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber |  |

### Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

Objective: This Competency Standard is designed to gain basic knowledge and skills required to perform Shielded Metal Arc Welding (SMAW) operations in Flat (1F, 1G) and Horizontal (2F, 2G) positions at workplace. The standard covers specific knowledge of performing the Shielded Metal Arc Welding (SMAW) by selecting and setting up welding equipment, installing consumables, adjusting welding parameters and making fillet and groove welds in Flat (1F, 1G) and Horizontal (2F, 2G) positions of plate. The standard also covers post welding operations comprising cleaning, measuring, inspecting and repairing welds at workplace

Duration:  			18 Hours			Theory: 	06 Hours				Practice: 	12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Welding Machine and Accessories for SMAW | Trainee will be able to: - Identify the welding requirements from the job, welding procedure specifications and/or technical drawings - Prepare the SMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer instructions - Connect the welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications | - Understanding of technical drawings - Electrical supply AC and DC - Specifications / classification of electrode/s required for the job | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode holder - Glass eye protector - Electrodes with flux - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Fillet Welds on Carbon Steel Plate | Trainee will be able to: - Adjust the welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld Maintain gap between electrode and base metal as per standard practices - Carry out the welding in Flat (1F) and Flat (1G) positions according to the standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS / instruction sheets | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/scrubber | Class Room and workshop |
| - Make Groove Welds on Carbon Steel Plate | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Horizontal (2F) and Horizontal (2G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Welding procedure specifications (WPS) - Method of Pre- heating of base metal | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/scrubber | Class Room and workshop |
| - Make Fillet Welds on Carbon Steel Plate | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Horizontal (3F) and Horizontal (3G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Polarity setting according to standard specifications - Visual welding defects | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/scrubber | Class Room and workshop |
| - Make Groove Welds on Carbon Steel Plate | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Horizontal (4F) and Horizontal (4G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Explain Welding codes and standards - Explain electrode classification number - Identification of Welding electrodes as per standards code | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/scrubber |  |
| - Perform Post Welding Operation | Trainee must be able to - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required Knowledge & Understanding - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools / equipment / consumable materials in accordance with organization guidelines | - State the purpose of using shielding gas in SMAW welding - Identify various shielding materials and their uses | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Electrode - Glass eye protector - Filler metal - Shielding Gas - Power cable - Hand gloves - Work piece cleaner/scrubber |  |

### Flux Cored Arc Welding (FCAW)

Objective: This competency standard covers the skills and knowledge required to Prepare Welding Machine and Accessories for FCAW, Make Fillet Welds on Carbon Steel Plate, Make Groove Welds on Carbon Steel Plate, Make Fillet Welds on Carbon Steel Plate, Make Groove on Carbon Steel Plate and Preform Post Welding Operations

Duration: 18Hours			Theory: 	06 Hours				Practice: 	12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Welding Machine and Accessories for FCAW | Trainee will be able to: - Identify the welding requirements from the job, welding procedure specifications and/or technical drawings - Prepare the FCAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer instructions - Connect the welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications | - Explain application of flux - Explain types of flux - Describe flux grades | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Flux cored Electrode - Glass eye protector - Filler Metal - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Fillet Welds on Carbon Steel Plate | Trainee will be able to: - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld Maintain gap between electrode and base metal as per standard practices - Carry out welding in in Flat (1F) and Horizontal (2F) Positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Classify flux electrodes as per required for the job - Explain Electrical parameters like (voltage, current etc.) and their effects on weld - Teach Welding techniques as per WPS/instruction sheet | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Flux cored Electrode - Glass eye protector - Filler Metal - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Groove Welds on Carbon Steel Plate | Trainee must be able to: - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Flat (1G) and Horizontal (2G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check the root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of the visual welding defects | - Welding procedure specifications (WPS) - Polarity setting according to standard specifications | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Flux cored Electrode - Glass eye protector - Filler Metal - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Fillet Welds on Carbon Steel Plate | Trainee must be able to - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain the gap between electrode and base metal as per standard practices - Carry out welding in Vertical (3F) and Overhead (4F) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Visual welding defects - Welding codes and standards | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Flux cored Electrode - Glass eye protector - Filler Metal - Power cable - Hand gloves - Work piece cleaner/ scrubber | Class Room and workshop |
| - Make Groove Welds on Carbon Steel Plate | Trainee must be able to - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Vertical (3G) and Overhead (4G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - State the purpose of using Flux cored Electrode in FCAW welding | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Flux cored Electrode - Glass eye protector - Filler Metal - Power cable - Hand gloves - Work piece cleaner/ scrubber |  |
| - Perform Post Welding Operation | Trainee must be able to - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required Knowledge & Understanding - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools / equipment / consumable materials in accordance with organization guidelines | - Identify various welding defects and perform repair work | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Work piece - Marking tool / gauge - ARC welding unit - Flux cored Electrode - Glass eye protector - Filler Metal - Power cable - Hand gloves - Work piece cleaner/ scrubber |  |

### Perform Soldering and Brazing Operations

Objective: This competency standard covers the skills and knowledge required to Soldering Operation and Brazing Operation

Duration:  			30 Hours			Theory: 	12 Hours			Practice: 		18Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Soldering Operation | Trainee will be able to: - Perform marking as per drawing - File work piece before soldering - Use copper filler rod as filler metal - Perform soldering operation as per standard | - Describe soldering - Describe sheet metal gauge - Explain disadvantages of soldering | Total: 15Hrs Theory: 06Hrs Practical:9Hrs | - Solder gun - Solder wire | Class Room and workshop |
| - Brazing Operation | Trainee will be able to: - Perform marking as per drawing - Cut the metal sheet according to drawing using shearing machine - Straighten the material with help of hammer - File work piece before soldering - Open gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Select the correct size of the nozzle - Set flame to carburizing flame as per standard - Use copper filler rod as filler metal - Perform brazing as per standard | - Describe brazing - Define carburizing flame - Define neutral flame - Define oxidizing flame | Total: 15Hrs Theory: 06Hrs Practical:9Hrs | - Brazing Torch - Oxygen cylinder - Acetylene gas cylinder - Pressure regulators - Cylinder key - Welding torch - Rubber house pipe - Back fire arrester - Flash back arrester - Marking tools - Copper Filler rod - Spark lighter - Steel wire brush | Class Room and workshop |

## Digital Skills

### Install computer operating systems and hardware

Objective: This module describes the performance outcomes, skills and knowledge required to select, configure and use computer operating systems and basic computer hardware.

Duration: 25 Hours							Theory: 01 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify operating system and hardware components | Trainee will be able to: - Determine ICT organizational requirements and specifications - Identify and select operating system - Identify appropriate external hardware components - Identify internal hardware components | - Basic knowledge of current industry-accepted operating system, hardware and software products Practical activity - Identify which operating system can be installed on the given hardware system | Theory-01 Hrs Practice-09 Hrs Total- 10 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Install and configure operating system and application software with hardware components | Trainee will be able to: - Install and configure operating system to meet organizational requirements - Identify the functions associated with the operating system and associated boot process - Configure power-management settings to minimize power consumption as an environmentally sustainable measure - Use both the graphical user interface and the command line interface to perform basic tasks - Install or upgrade application software onto the operating system and hardware configuration - Determine the relationship between an application program, the operating system and hardware - Identify general differences between the different computer platforms and their respective operating systems | - Compatibility of an operating system, in respect to other versions Practical activity - Install Microsoft office application in the given system. | Theory-0 Hrs Practice-09 Hrs Total- 9hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Optimize operating system and hardware components | - Optimize operating system using included tools or third-party utilities - Customize the graphical user interface - Use techniques unique to the command line interface - Set up and configure external hardware components and check functionality - Install drivers as appropriate and check functionality | - Function of single-user and multi-user operating systems - Interoperability between operating systems Practical activity Check compatibility of the required drivers, install and check their functionality. | Theory-0 Hrs Practice-06 Hrs Total- 06 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Operate Basics Computer Functions

Objective:This module covers the knowledge, skills and attitudes and values needed to perform basic computer operations which include inputting, accessing, producing and transferring data using the appropriate hardware and software.

Duration: 25 Hours							Theory: 01 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes |  | Learning Elements |  |  | Duration | Materials Required |  |  | Learning Place |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Plan and prepare for task to be undertaken |  | Trainee will be able to: - Requirements of task are determined as per standard - operating procedures - Appropriate hardware and software is selected according to task assigned and required outcome - Task is planned to ensure |  | - Main types of computers and basic features of different operating systems - Main Parts of computer - Calculating computer capacity Practical activity - Select the appropriate hardware & software as per the requirement of task. | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs |  |  | Desktop Computer/Laptop | Computer Lab Class room |  |
| - Input data into computer |  | Trainee will be able to: - Data are entered into the computer using appropriate program/application in accordance with company procedures - Accuracy of information is checked and information is saved in accordance with standard operating procedures - Inputted data are stored in storage media according to requirements - Work is performed within ergonomic guidelines |  | - Storage devices and basic categories of memory - Basic ergonomics of computer use Practical activity - Select storage devices to store input data | Theory-0 Hrs Practice-03 Hrs Total- 3 Hrs |  |  | Desktop Computer/Laptop | Computer Lab Class room |  |
| - Access information using computer |  | Trainee will be able to: - Correct program/application is selected based on job requirements - Program/application containing the information required is accessed according to company procedures - Desktop icons are correctly selected, opened and closed for navigation purposes - Keyboard techniques are carried out in line with OH & S requirements for safe use of keyboards |  | - Relevant types of software - OH & S principles and responsibilities - Basic ergonomics of keyboard use Practical activity - Operate the computer and its different application | Theory-0Hrs Practice-06 Hrs Total- 6 Hrs |  |  | Desktop Computer/Laptop | Computer Lab Class room |  |
| - Produce/output data using computer system |  | Trainee will be able to: - Entered data are processed using appropriate software commands - Data are printed out as required using computer hardware/peripheral devices in accordance with standard operating procedures - Files and data are transferred between compatible systems using computer software, hardware/ peripheral devices in accordance with standard operating procedures |  | - Relevant types of software Practical activity - Develop a required documents in MS word and take out print. | Theory-0 Hrs Practice-06 Hrs Total- 6 Hrs |  |  | Desktop Computer/Laptop | Computer Lab Class room |  |
| - Maintain computer equipment and systems |  | Trainee will be able to: - Systems for cleaning, minor maintenance and replacement of consumables are implemented - Procedures for ensuring security of data, including regular back-ups and virus checks are implemented in accordance with standard operating procedures - Basic file maintenance procedures are implemented in line with the standard operating procedures |  | - General security - Viruses Practical activity - Create a proper backup of computer data and install anti-virus software | Theory-0 Hrs Practice-06 Hrs Total- 06 Hrs |  |  | Desktop Computer/Laptop | Computer Lab Class room |  |

### Develop Computer Application skills

Objective: This module describes the performance outcomes, skills and knowledge required to identify, select and operate three commercial software packages, including a word-processing, a spreadsheet and presentation application package.

Duration: 25 Hours							Theory: 01 Hours				Practice: 24Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Use appropriate OHS office work practices | Trainee will be able to: - Use safe work practices to ensure ergonomic, work organization, energy and resource conservation requirements are addressed - Use wrist rests and document holders where appropriate - Use monitor anti-glare and radiation reduction screens where appropriate | - OHS principles and responsibilities for ergonomics, such as work periods and breaks Practical activity - Identify potential hazards in computer lab and use appropriate actions to mitigate them* | Theory-1 Hrs Practice-03 Hrs Total- 4 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Install and remove software | Trainee will be able to: - Select software to be installed - Follow installation instructions - Delete unrequired software | - Application software packages used by the organization - Basic knowledge of system usage - Basic knowledge of install and remove software - Import and export software functions Practical activity - Identify the required software, install and uninstall these software from the computer. | Theory-0 Hrs Practice-03 Hrs Total- 3 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Use appropriate word-processing software | Trainee will be able to: - Select word-processing software appropriate to perform activity - Identify document purpose, audience and presentation requirements, and clarify with personnel as required - Identify organizational requirements for text-based business documents and design document structure and layout to ensure consistency of style and image - Match document requirements with software functions to provide efficient production of documents - Use technical functions, other data and formatting to finalize documents - Ensure the naming and storing of documents in appropriate directories or folders and the printing of documents to the required specifications | - Current business practices related to Practical activity - using software to prepare reports - Functions and uses of word processing | Theory-0 Hrs Practice-06 Hrs Total- 6 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Use appropriate spreadsheet software | Trainee will be able to: - Select spreadsheet software appropriate to perform activity - Identify document purpose, audience and presentation requirements, and clarify with personnel as required - Enter simple formulas and functions using cell referencing where required - Customize spreadsheet settings and format documents to meet requirements - Ensure the naming and storing of documents in appropriate directories or folders and the printing of documents to the required specifications | - Functions and uses of Spreadsheet Practical activity - Prepare a spreadsheet using different functions and formulas. | Theory-0 Hrs Practice-06 Hrs Total- 6 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Use appropriate presentation software | Trainee will be able to: - Select software application package appropriate to perform activity - Identify purpose, audience and presentation requirements, and clarify with personnel as required - Use technical functions, other data and formatting to finalize documents - Ensure documents are named and stored in appropriate directories or folders and printed to required specifications - Make a presentation | - Functions and uses of presentation software Practical activity - Prepare a presentation using Microsoft power point application. | Theory-0 Hrs Practice-06 Hrs Total- 6 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Perform word-processing applications

Objective:This unit describes the skills and knowledge required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

Duration: 25 Hours							Theory: 01 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply workplace health and safety (WHS) practices | Trainee will be able to: - Use workplace ergonomic work practices and strategies - Organize work area to ensure an ergonomic work environment | - OHS principles and responsibilities for ergonomics, such as work periods and breaks Practical activity - Identify potential hazards in computer lab and use appropriate actions to mitigate them* | Theory-01 Hrs Practice-3 Hrs Total- 4 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Create documents | Trainee will be able to: - Open word-processing application, create document and add data according to information requirements - Use document templates as required - Use simple formatting tools when creating the document - Save document to directory | - Create and save new file - Use of appropriate template as required Practical activity - Develop a documents in MS word using format tools as per the requirement of the task. | Theory-0 Hrs Practice-2 Hrs Total- 2Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Customize basic settings to meet page layout conventions | Trainee will be able to: - Adjust page layout to meet information requirements - Open and view different toolbars - Change font format to suit document purpose - Change alignment and line spacing according to document information requirements - Modify margins to suit the document purpose - Open and switch between several documents | - Understand and edit page layout Practical activity - Open MS word and change select appropriate layout of the page as per the requirement adding header and footer in the document. | Theory-0 Hrs Practice-3 Hrs Total- 3 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Format documents | Trainee will be able to: - Use formatting features and styles as required - Highlight and copy text from another area in the document or from another active document - Insert headers and footers to incorporate necessary data - Save document in another file format - Save and close document to a storage device | Practical activity - Formatting word document | Theory-0 Hrs Practice-3 Hrs Total- 3 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Create tables | Trainee will be able to: - Insert standard table into document - Change cells to meet information requirements - Insert and delete columns and rows as necessary - Use formatting tools according to style requirements | Practical activity - Create and edit table | Theory-0 Hrs Practice-4 Hrs Total- 4 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Add images | Trainee will be able to: - Insert appropriate images into document and customize as necessary - Position and resize images to meet document formatting needs | Practical activity - Insert, position and resize images | Theory-0 Hrs Practice-4 Hrs Total- 5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Print documents | Trainee will be able to: - Preview document in print preview mode - Select basic print settings - Print document or part of document from printer | Practical activity - Printing document according to organization need | Theory-0 Hrs Practice-5 Hrs Total- 5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Operate Spreadsheet Application

Objective:This module describes the skills and knowledge required to operate Spreadsheet and perform basic operations, including creating and formatting spreadsheet, creating tables, incorporating chart and object in it, and printing labels.

Duration: 25Hours							Theory: 01 Hours				Practice: 24Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Create spreadsheets | Trainee will be able to: - Open the spreadsheet application, create spreadsheet files and enter numbers, text and symbols into cells according to information requirements - Enter simple formulas and functions using cell referencing when required - Correct formulas when error messages occur - Use a range of common tools during spreadsheet development - Edit columns and rows within the spreadsheet - Use the auto-fill function to increment data where required - Save the spreadsheet to a folder on a storage device | Practical activity - basic technical terminology related to reading help files and prompts - purpose, use and function of spreadsheet applications - create, edit and save spread sheet | Theory-01 Hrs Practice-03Hrs Total- 04 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Customize basic settings | - Adjust page layout to meet user requirements or special needs - Open and view different toolbars - Change font settings so they are appropriate for the document purpose - Change alignment options and line spacing according to spreadsheet formatting features - Format cell to display different styles as required - Modify margin sizes to suit the purpose of the spreadsheets - View multiple spreadsheets concurrently | Practical activity - Formatting spread sheet | Theory-0 Hrs Practice-03 Hrs Total- 3 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Format spreadsheet | Trainee will be able to: - Use formatting features as required - Copy selected formatting features from another cell in the spreadsheet or from another active spreadsheet - Use formatting tools as required within the spreadsheet - Align information in a selected cell as required - Insert headers and footers using formatting features - Save spreadsheet as another file type - Save to storage device and close spreadsheet | Practical activity - effect of formatting and appearance on the readability and usability of spreadsheets | Theory-0 Hrs Practice-06 Hrs Total- 06 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Incorporate object and chart in spreadsheet | - Import an object into an active spreadsheet - Manipulate imported object by using formatting features - Create a chart using selected data in the spreadsheet - Display selected data in a different chart - Modify chart using formatting features | Practical activity - Inserting and editing object in spread sheet - Inserting and modifying chart in spreadsheet | Theory-0 Hrs Practice-06 Hrs Total- 06 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Print spreadsheet | - Preview spreadsheet in print preview mode - Select basic printer options - Print spreadsheet or selected part of spreadsheet - Submit the spreadsheet to appropriate person for approval or feedback | Practical activity - Preview and Print spread sheet | Theory-0 Hrs Practice-06 Hrs Total- 06 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Operate Presentation Packages

Objective:This module describes the skills and knowledge required to operate power point processing applications and perform basic operations, including creating and formattingpresentations, adding slide show effects and printing presentations and notes. It applies to individuals in the workplace using fundamental knowledge of PowerPoint processing under direct supervision or with limited responsibility.

Duration: 25 Hours							Theory: 01 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Create Presentations | Trainee will be able to: - Open presentation package and create a simple design for a presentation according to organizational requirements - Open blank presentation and add text and graphics - Apply existing styles within a presentation - Use presentation template and slides to create a presentation - Use various tools to improve the look of the presentation - Save presentation to the appropriate storage device and folder | Practical activity - Basic technical terminology to read help files and prompts | Theory-1 Hrs Practice-3Hrs Total- 04 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Customize basic settings | Trainee will be able to: - Adjust display to meet user requirements - Open and view different toolbars to view options - Ensure font settings are appropriate for the presentation purpose - View multiple slides at once | Practical activity - effect of formatting and appearance on the readability and usability of presentation | Theory-0 Hrs Practice-3 Hrs Total- 03 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Format presentations | Trainee will be able to: - Use and incorporate organizational charts and bulleted lists, and modify as required - Add objects and manipulate to meet presentation purposes - Import objects and modify for presentation purposes - Modify slide layout, including text and colors, to meet presentation requirements - Use formatting tools as required within the presentation - Duplicate slides within and across a presentation - Reorder sequence of slides and delete slides for presentation purposes - Save presentation in another format - Save to storage device and close presentation | Practical activity - Insert objects and charts | Theory-0 Hrs Practice-06 Hrs Total- 06 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Add slide show effects | Trainee will be able to: - Incorporate pre-set animation and multimedia effects into presentation as required to enhance the presentation - Add slide transition effects to presentation to ensure smooth progression through the presentation - Test presentation for overall effect - Use onscreen navigation tools to start and stop slide show or move between different slides as required | Practical activity - suitable presentation effects for different audiences | Theory-0 Hrs Practice-5 Hrs Total- 5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Print presentation and notes | Trainee will be able to: - Select appropriate print format for presentation - Select preferred slide orientation - Add notes and slide numbers - Preview slides and run spell check before presentation - Print selected slides and submit presentation to appropriate person for feedback | Practical activity - Preview and print presentations | Theory-0 Hrs Practice-6 Hrs Total- 6Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Perform writing and editing skills

Objective:This module describes the skills and knowledge required to apply the conventions of plain English to writing and editing tasks of different forms. It also includes editing and proofreading techniques. It applies to individuals in various writing contexts who write and edit texts using appropriate language, style, grammar, spelling, and standard conventions for editing and proofreading.

Duration: 25 Hours							Theory: 1 Hours				Practice: 24Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply clear and appropriate language and style to writing and editing tasks | Trainee will be able to: - Use safe work practices including addressing ergonomic requirements when undertaking writing tasks - Use clear, concise and plain English in writing and editing tasks - Apply appropriate paragraph structure to written material to ensure clarity of meaning and ease of reading - Make clear and logical connections between sentences, paragraphs and sections - Determine and incorporate the language and style of the audience | Practical activity - Main features of clear, concise and plain English language for written material | Theory-1 Hrs Practice-6 Hrs Total- 07 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Apply the appropriate voice, tone and tense | Trainee will be able to: - Determine appropriate voice, tone and tense of the written materials according to audience requirements - Maintain consistent voice, tone and tense throughout written material | Practical activity - Appropriate use of tenses required in written material | Theory-0. Hrs Practice-06 Hrs Total- 06 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Apply appropriate grammar, spelling and punctuation | Trainee will be able to: - Apply appropriate grammar conventions to a range of written contexts including use of numbers, quotations, and tables - Apply appropriate spelling and punctuation conventions in writing and editing tasks. | Practical activity - Grammar, punctuation and spelling conventions that meet the task requirements | Theory-0 Hrs Practice-06 Hrs Total- 6Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Perform editing and proofreading tasks to meet requirements | Trainee will be able to: - Edit written material to ensure clear meaning through language and paragraphs, consistent voice, tone and tense - Copyedit written material by checking grammar, spelling and punctuation using standard editing conventions - Proofreading using style guides and by monitoring written material for errors | Practical activity - Editing conventions used in substantive editing and copyediting of written material - Basic software used to write and collect feedback | Theory-0 Hrs Practice-6Hrs Total- 6 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

# 4.	List of Personal Protective Equipment

(FOR A CLASS OF 25 STUDENTS)

| Name of Trade |  | Mobile Phone Technician |  |
| --- | --- | --- | --- |
| Duration of Course |  | 12 Months |  |
| Sr. # |  | Description |  | Quantity |
| 1. |  | Safety gloves, |  | 30 |
| 2. |  | Appropriate safety glasses, |  | 30 |
| 3. |  | Breathing apparatus, |  | 30 |
| 4. |  | Fire extinguishers, |  | 30 |
| 5. |  | Fire blankets, |  | 30 |
| 6. |  | Respirators, masks, |  | 30 |
| 7. |  | Fire hoses, |  | 04 |
| 8. |  | First aid kits, stretchers and other related tools and gear |  | 10 |

# 5. Complete List of Tools, Equipment, Machines and Consumables

Tools

| S# | Description | Quantity |
| --- | --- | --- |
|  | Precision screw driver | 25 |
|  | Screw driver kit | 25 |
|  | Tweezers | 25 |
|  | Brushes | 25 |
|  | Blade cutter | 25 |
|  | Nose cutter | 25 |
|  | Point cutter | 25 |
|  | Cutter plier | 25 |
|  | Long Nose plier | 25 |
|  | PCB holder | 25 |
|  | Hard tweezers | 25 |
|  | Magnifying glass | 25 |
|  | Opener’s kit | 25 |
|  | Suction openers | 25 |
|  | Magnifying lamp | 25 |
|  | Wrist wire | 25 |
|  | Clipping tools(clips) | 25 |
|  | seizer | 25 |
|  | Steel wire | 25 |
|  | Rubber pads | 25 |
|  | Glass openers | 25 |

Equipment for Hardware

| S# | Description | Quantity |
| --- | --- | --- |
|  | Digital Multi meter | 25 |
|  | Oscilloscope | 5 |
|  | Soldering Iron | 25 |
|  | Soldering Station/SMD work station | 25 |
|  | Digital variable Power supply | 5 |
|  | B. G. A. Kit | 25 |
|  | Ultrasonic PCB Cleaner | 5 |

Equipment for Display repair

| S# | Description | Quantity |
| --- | --- | --- |
|  | Hot gun | 25 |
|  | Smoke Absorber | 5 |
|  | Hot air blower | 5 |
|  | Hot plate display separator | 5 |
|  | Freezer for separation of OLED | 5 |
|  | Flex binding machine | 5 |
|  | Mini electric grinder kit (EMC) | 5 |
|  | UV lamp | 5 |
|  | Glass laminating machine | 5 |
|  | De-bubbler machine | 5 |
|  | PCB Owen | 5 |

Equipment for Software

| S# | Description | Quantity |
| --- | --- | --- |
|  | Octopus Box (with cable Samsung + LG) | 5 |
|  | J Tag | 5 |
|  | EFT Dongle | 5 |
|  | CM2 Dongle | 5 |
|  | Sigma Key | 5 |
|  | ATF Dongle | 5 |
|  | ZXW (Hardware) Schematic diagram Dongle | 5 |

Consumables

| S# | Description | Quantity |
| --- | --- | --- |
|  | Soldering wire | 25 roll |
|  | Gloves | 25 Nos |
|  | Jumper wire |  |
|  | IC Paste |  |
|  | Solder paste |  |
|  | Cleaning sponge | 25 |
|  | Paste flux |  |
|  | Cleaning cloth |  |
|  | Chemical for washing mobile phone |  |
|  | UV Gum |  |
|  | Double tape |  |
|  | Heat resistance tape |  |
|  | Adhesive Glue |  |
|  | Thinner |  |
|  | CTC |  |
|  | Shiner |  |
|  | Rubber adhesive glue |  |
|  | WD-40 |  |
|  | Thinner |  |
|  | Cotton |  |

List of Stationary

| S# | Description | Quantity |
| --- | --- | --- |
|  | Handbooks | 5 |
|  | Design books | 5 |
|  | Pencils | 30 |
|  | Rubber | 30 |
|  | Sharpeners | 30 |
|  | Paper Cutter | 5 |
|  | Seizers | 5 |
|  | Colors | 5 each color (Read, Green, Yellow, Blue, white and Black) |
|  | White charts | 50 Nos |
|  | Brown sheets | 100 Nos |
|  | White board markers | 5 Doz |
|  | Permanent markers | 2 Dozen |
|  | File cover and files | 50 Nos. |

Learning hours in training provider premises,

Training workshop, laboratory and on-the-job workplace