National Certificate Level -5 in Mechatronics Technology

(Curriculum)

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1. 	Introduction	8

1.1.	Purpose of the training program:	9

1.2.	Overall objectives of training program	9

1.3.	Competencies to be gained after completion of course	10

1.4.	Entry level of trainees	20

1.5.	Minimum qualification for teachers	20

1.6.	Medium of instruction	20

1.7.	Duration of the course:	21

1.8.	Description and structure of the course	22

2.	Overview of the Curriculum for Mechatronics Technology	30

3.	Detail of Modules.	32

3.1. 	Maintain Occupational Health and Safety:	32

3.1.1.	Maintain Health and Safety	32

3.1.2.	Perform safety practices at workplace	37

3.1.3.	Develop professionalism.	41

3.2.	Basic Machining	46

3.2.1.	Perform Metal / Bench Work	46

3.2.2.	Perform general machining	51

3.2.3.	Perform cutting on Metal Circular/Power Heck Saw	54

3.2.4.	Perform tool sharpening	56

3.2.5.	Perform Basic Lathe Machine Operations	59

3.2.6.	Perform Drilling Machine Operations	67

3.2.7.	Perform Shaper, Planar and Slotter Machining Operations	70

3.2.8.	Perform Milling Operations	75

3.3.	Electrical Essentials & Networks	87

3.3.1.	Operate Measuring Instruments.	87

3.3.2.	Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.	90

3.3.3.	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence	93

3.3.4.	Construct Electromagnet to See Various Effects & Verify Faradays Law	96

3.3.5.	Verify Series and Parallel Combination of Capacitors & Determine Break down Voltage of Capacitor	100

3.3.6.	Maintain Tools & Equipment.	102

3.3.7.	Make Cable/Wire Joints	104

3.3.8.	Prepare and Install Distribution Boards	107

3.3.9.	Carryout Basic Electrical Installation	110

3.3.10.	Install Single Phase Electrical Wiring	113

3.3.11.	Perform Testing of Electrical Wiring	118

3.3.12.	Operate Single Phase Transformer	123

3.3.13.	Perform Tests on Single Phase Transformer	126

3.4.	Metrology	129

3.4.1.	Take measurements with graduated tools	129

3.4.2.	Take measurements with combination set	132

3.4.3.	Perform Leveling.	135

3.4.4.	Take measurements with gauges	137

3.4.5.	Perform measurements through Micrometer	139

3.4.6.	Measure dimensions with Vernier tools	142

3.4.7.	Measure angles with angle measuring Instruments.	144

3.4.8.	Measure Threads and Gears	147

3.5.	Basic Engineering Drawing	150

3.5.1.	Perform Basic Manual Drawing.	150

3.5.2.	Construct different Engineering Curves	159

3.5.3.	Construct Multi-View Drawings.	171

3.5.4.	Draw basic drawing using CAD.	180

3.5.5.	Develop 2D using CAD software.	182

3.5.6.	Develop 3D drawing using CAD software.	184

3.6.	Welding	187

3.6.1.	Interpret Part and Assembly Drawing	187

3.6.2.	Evaluate the Use of Hand and Power Tools	190

3.6.3.	Perform Pre-Welding Operations	193

3.6.4.	Perform Oxy Acetylene Welding	196

3.6.5.	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)	198

3.6.6.	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)	202

3.6.7.	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	205

3.6.8.	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	212

3.6.9.	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	219

3.6.10.	Flux Cored Arc Welding (FCAW)	226

3.6.11.	Perform Soldering and Brazing Operations	230

3.7.	Microprocessor and Microcontroller	232

3.7.1.	Microprocessor applications	232

3.7.2.	Identify Microcontroller Types and its Architecture	234

3.7.3.	Interface Microcontroller with system	236

3.7.4.	Carryout various Microcontroller Applications	239

3.7.5.	Apply Microcontroller to build Control System	242

3.8.	Analog Electronics	244

3.8.1.	Identify Basic Electronics Components	244

3.8.2.	Design Rectifier using Diode	247

3.8.3.	Carry out Diode Application	249

3.8.4.	Implement Bipolar Junction Transistor (BJT) in Different Applications	250

3.8.5.	Implement Field Effect Transistor (FET) in Different Applications	255

3.8.6.	Implement Thyristor Family in Various Application	258

3.8.7.	Application of Operational Amplifier	260

3.9.	Computer Programming	263

3.9.1.	Installation of C++, IDE for Arduino and Python	263

3.9.2.	Carryout Basic C++ Programming	266

3.9.3.	Carryout Basic Programming using python	269

3.9.4.	Perform basic programming using microcontrollers	273

3.10.	Computer Integrated Machines	276

3.10.1.	Perform CNC Machine Operations	276

3.10.2.	Perform CNC Lathe Operations	279

3.10.3.	Perform CNC Milling Operations	286

3.11.	Machines and Mechanism	293

3.11.1.	Demonstrate Simple Mechanisms	293

3.11.2.	Perform Kinematic Analysis of Simple Mechanisms	296

3.11.3.	Demonstrate Mechanical Power Transmission Elements.	298

3.11.4.	Perform Kinematic Analysis of Mechanical Power Transmission Elements.	300

3.12.	AC & DC machines	303

3.12.1.	Verify Basic Laws of Electrical Machines.	303

3.12.2.	Analyze an Alternator	309

3.12.3.	Analyze Single Phase Motors	313

3.12.4.	Analyze Special Purpose Motors	316

3.12.5.	Analyze Three Phase Motors.	319

3.12.6.	Analyze Dc Generators	330

3.12.7.	Perform Tests on DC Generators	335

3.12.8.	Analyze Dc Motors.	340

3.12.9.	Perform Tests On DC Motors.	344

3.13.	Digital Electronics	353

3.13.1.	Verify Truth Tables of Digital Gates	353

3.13.2.	Construct & Verify Combinational Logic Circuit	357

3.13.3.	Construct and Verify Function of Flip Flops	361

3.13.4.	Construct Multi vibrator with 555 IC	363

3.13.5.	Construct Shift Registers and Counters with the Help of Flip Flops	365

3.14.	Sensor & Actuators	367

3.14.1.	Test Sensors and Transducers.	367

3.14.2.	Calibrate Sensors and Transducers.	376

3.14.3.	Interface Sensors and Transducers.	394

3.14.4.	Test Actuators	399

3.14.5.	Interface and control Actuator with sensors.	404

3.15.	Programmable logic controllers (PLCs)	409

3.15.1.	Install PLC software and Simulator	409

3.15.2.	Interface PLC with system (1)	416

3.15.3.	Program PLC using Ladder Logic	418

3.15.4.	Commissioning and Testing PLC	422

3.15.5.	Commissioning and Testing PLC	427

3.15.6.	Carry out Industrial Automation on PLC	430

3.15.7.	Interface PLC with system (2)	434

3.16.	Designing of Mechanical Members	438

3.16.1.	Calculate Stresses in Machine Parts	438

3.16.2.	Calculate diameter of cylinder for hoop and longitudinal stresses	441

3.16.3.	Calculate thickness and diameter of spherical shell for circumferential stresses	443

3.16.4.	Design welded joints for transverse and parallel fillet under static and fatigue loading	447

3.16.5.	Calculate stresses due to initial tightening and external load on screws	454

3.16.6.	Design dimension of square and rectangular keys	458

3.16.7.	Design shafts subjected to twisting moment	462

3.16.8.	Design flange coupling for specific torque	469

3.17.	Industrial Process Control & Instrumentation	472

3.17.1.	Design & Implement a process control	472

3.17.2.	Install Transducer & Transmitters used in Industrial Applications	475

3.17.3.	Install the process Regulators	478

3.17.4.	Install the Variable Frequency Drive (VFD)	481

3.17.5.	Install Pneumatic & Hydraulic systems	483

3.18.	Robotics	486

3.18.1.	Identify Various Parts of Robots	486

3.18.2.	Identify Various Parts of Robot Arm	490

3.18.3.	Develop Robot program	493

3.18.4.	Develop robotic arm structure	496

3.18.5.	Test the Robot	499

3.19.	Mechatronic Projects	502

3.19.1.	Access Control System using RFID Reader RC522	502

3.19.2.	Finger Print Sensor Based Biometric Attendance System	504

3.19.3.	Humidity and Temperature Monitoring System	506

3.19.4.	Smart Stick for Blinds	508

3.19.5.	Perform Line Following Robot	510

3.19.6.	Perform Smart Phone Control Robot Using Bluetooth	512

3.19.7.	Perform IoT Controlled based home automation system	515

3.19.8.	Perform Smart Phone Control Door Lock System	518

3.19.9.	Perform Security System Using Motion Detection	521

3.19.10.	Perform Water-level Detection in Tank	523

3.19.11.	Perform Mini CNC 2D Plotter	525

3.19.12.	Perform Voice Controlled Robot	527

3.20.	Hydraulics & Pneumatics	529

3.20.1.	Operate Hydraulic Bench and Its Functions	529

3.20.2.	Calibrate Bourdon Tube and Diaphragm Pressure Gauge	532

3.20.3.	Operate Hydraulic Press	534

3.20.4.	Verify Bernoulli’s Equation	537

3.20.5.	Analyze the performance of Pumps	540

3.20.6.	Construct circuit for Double Acting Hydraulic Cylinder	542

3.20.7.	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder	545

3.20.8.	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking	547

3.20.9.	Set Speed and Direction of Hydraulic Motor	552

3.20.10.	Operate Pressure Measuring Instruments	554

3.20.11.	Operate Flow Regulating Devices	560

4.	List of Personal Protective Equipment	565

5. Complete List of Tools, Equipment, Machines and Consumables	566

# 1. 	Introduction

Mechatronics is an interdisciplinary field that combines elements of engineering, machining, information technology, robotics, and electronics. It synchronize these diverse fields for the purpose of designing, fabrication and field testing of the production lines and manufacturing processes in order to optimize their performance and efficiencies. In this field several mechanical systems such as hydraulic, pneumatic, camera, electric motor and engines are interface, configure and combine in order to perform the tasks with increase efficiency and performance. Since several different mechanical, electrical and optical systems are inference, the continual operational maintenance, configuration and upgradation is required which need trained main power in the field of mechatronics in order to run the industrial processes and machines with any system failure or breakdown.

Typically, a mechatronics technologist need to perform several task such as standalone operation / testing of the mechanical and electronics systems in order to diagnose the faulty parts. These task may attribute to the checking, installation and configuration of electronic circuits, motor drives, hydraulics/ pneumatic moving system. The industry is lacking the expert technologist who has sound knowledge and command over all mechanical and electronics systems especially when these are interface and working in the close loop environment.

Mechatronics technologists can work in several fields, but they are commonly found in the manufacturing and automation sector. Several industrial process and production lines can be optimize by implementing fine mechanical precision in environmental conditions that may be dangerous or inefficient  for human hands and can be perform optimally by adding mechatronic solutions. Mechatronics has had a measurable impact both on the speed with which new products can be assembled.

The professionals that are skilled in this area of mechatronics are in high demand on production lines that range from projects as large as automotive vehicle manufacturing to the smallest microelectronic products. For instance, blood pressure monitoring device has pneumatic pump, a sensor to sense the blood pressure and electronics / microprocessor based circuit to measure the blood pressure. Similarly, several home appliances such as washing machines, ovens and refrigerators are now equipped with sophisticated mechatronics systems in order to optimize their products in terms of performance and electrical power consumption. Therefore, to repair and maintain such home appliances, the traditional technician are fail to work because of their training explicitly in the field of mechanical, electrical or electronics. Being conscious of these facts, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechatronics Technology under National Vocational Qualifications Framework (NVQF). Followings are the competency standards which are   developed by the Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Mechatronics industries from public and private sector, Engineering Universities and Technical Education of the country.

  - Purpose of the training program:

The purpose of the training is to set the high professional standards for the Mechatronics technological sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of the skills attained through formal or informal pathways

- Improve the quality and effectiveness of the training and assessment for Mechatronics Technological sector

- Enabling / helping / facilitating the existing workforce to indulge themselves in new technologies and methods

  - Overall objectives of training program

The Mechatronics qualifications of level 5 consists of both the theoretical and practical details and having the following Occupations

1. Health & Safety

2. Basic Machining

3. Electrical Essentials & Networks

4. Metrology

5. Basic Engineering Drawings

6. Welding

7. Microprocessor and Microcontroller

8. Analog Electronics

9. Computer Programming

10. Computer Integrated Machines

11. Machines and Mechanism

12. AC & DC machines

13. Digital Electronics

14. Sensor & Actuators

15. Programmable logic controllers (PLCs)

16. Designing of Mechanical Members

17. Industrial Process Control & Instrumentation

18. Robotics

19. Mechatronic Projects

20. Hydraulics & Pneumatics

## Competencies to be gained after completion of course

Detail of the competency standards included in qualification are given below:

Health and Safety

0714E&A1.	Maintain Health and Safety

0714E&A2.	Perform safety practices at work place

0714E&A3.	Develop Professionalism

Basic Machining

0714E&A4.	Perform Metal / Bench Work

0714E&A5	Perform general machining

0714E&A6	Perform cutting on Metal Circular/Power Heck Saw

0714E&A7	Perform tool sharpening

0714E&A8	Perform Basic Lathe Machine Operations

0714E&A9	Perform Drilling Machine Operations

0714E&A10	Perform Shaper, Planar and Slotter Machining Operations

0714E&A11	Perform Milling Operations

Electrical Essentials & Networks

0714E&A12	Operate Measuring Instruments

0714E&A13	Verify Ohm’s & Kirchhoff’s Laws by Implementing Series/Parallel Circuits

0714E&A14	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

0714E&A15	Construct Electromagnet to See Various Effects & Verify Faradays Law.

0714E&A16 Verify Law of Combination of Capacitor & Determine Break Down Voltage of Capacitor

0714E&A17	Maintain Tools & Equipment

0714E&A18	Make Cable/Wire Joints

0714E&A19	Prepare and Install Distribution Boards

0714E&A20	Carryout Basic Electrical Installations

0714E&A21	Install Single Phase Electrical Wiring

0714E&A22	Perform Testing of Electrical Wiring

0714E&A23	Operate Single Phase Transformer

0714E&A24	Perform Tests on Single Phase Transformer

Digital Skills

0714E&A25	Install computer operating systems and hardware

0714E&A26	Operate digital media technology

0714E&A27	Perform computer operations

0714E&A28	Use computer applications

0714E&A29	Create user documentation

0714E&A30	Create technical documentation

0714E&A31	Create basic databases

0714E&A32	Use social media tools for collaboration and engagement

0714E&A33	E-Commerce- SEO (Search Engine Optimization)

0714E&A34	E-Commerce- SCM (Supply Chain Management)

0714E&A35	E-Commerce- Social Media Marketing

0714E&A36	Use digital devices

0714E&A37	Operate word-processing applications

0714E&A38	Operate spreadsheet applications

0714E&A39	Operate presentation packages

0714E&A40	Perform writing and editing tasks

Metrology

0714E&A41	Take measurements with graduated tools

0714E&A42	Take measurements with combination set

0714E&A43	Perform Levelling

0714E&A44	Take measurements with gauges

0714E&A45	Perform measurements through Micrometer

0714E&A46	Measure dimensions with Vernier tools

0714E&A47	Measure angles with angle measuring Instruments

0714E&A48	Measure Threads and Gears

Basic Engineering Drawings

0714E&A49	Perform Basic Manual Drawing

0714E&A50	Construct different Engineering Curves

0714E&A51	Construct multi-view drawings

0714E&A52	Develop 2D CAD drawings

0714E&A53	Develop 3D CAD drawing

Welding

0714E&A54	Interpret Part and Assembly Drawing

0714E&A55	Evaluate the Use of Hand and Power Tools

0714E&A56	Perform Pre-Welding Operations

0714E&A57	Perform Oxy Acetylene Welding

0714E&A58	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)

0714E&A59	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)

0714E&A60	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A61	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A62	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A63	 Flux Cored Arc Welding (FCAW)

0714E&A64	Perform Soldering and Brazing Operations

Microprocessor and Microcontroller

0714E&A65. Design Microprocessor applications

0714E&A66	Identify Microcontroller Types and its Architecture

0714E&A67	Interface Microcontroller with system

0714E&A68	Carryout various Microcontroller Applications

0714E&A69	Apply Microcontroller to build Control System

Analogue Electronics

0714E&A70	Identify Basic Electronics Components

0714E&A71	Design a Rectifier Using Diode

0714E&A72	Carry Out Diode Application

0714E&A73	Implement Bipolar Junction Transistor (BJT) in Different Applications

0714E&A74	Implement Field Effect Transistors (FET) In Different Applications

0714E&A75	Implement (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in various application

0714E&A76	Design Operation Amplifier

Soft Skills

0714E&A77.	Develop workplace policy and procedures for sustainability

0714E&A78.	Manage meetings

0714E&A79.	Manage recruitment selection and induction processes

0714E&A80.	Manage personal work priorities and professional development

0714E&A81.	Manage workforce planning

0714E&A82.	Undertake project work

0714E&A83.	Identify and communicate trends in career development

0714E&A84.	Apply specialist interpersonal and counseling interview skills

0714E&A85.	Work safely in an office environment

0714E&A86.	Develop workplace documents

0714E&A87.	Prepare and implement negotiation

0714E&A88.	Maintain professionalism in the workplace

0714E&A89.	Organize schedules

Computer Programming

0714E&A90.	Installation of C++, IDE for Arduino and Python

0714E&A91.	Carryout Basic C++ Programming

0714E&A92.	Carryout Basic Programming using python

0714E&A93.	Perform basic programming using microcontrollers

Computer Integrated Manufacturing (CIM)

0714E&A94.	Perform CNC Machine Operations

0714E&A95.	Perform CNC Lathe Operations

0714E&A96.	Perform CNC Milling Operations

Machines and Mechanism

0714E&A97.	Demonstrate Simple Mechanisms

0714E&A98.	Perform Kinematic Analysis of Simple Mechanisms

0714E&A99.	Demonstrate Mechanical Power Transmission Elements

0714E&A100.	Perform Kinematic Analysis of Mechanical Power Transmission Elements

AC & DC Machines

0714E&A101.	Verify Basic Laws of Electrical Machines

0714E&A102.	Analyse an Alternator

0714E&A103.	Analyse Single Phase Motors

0714E&A104.	Analyse Special Purpose Motors

0714E&A105.	Analyse Three Phase Motors.

0714E&A106.	Analyze Dc Generators

0714E&A107.	Perform Tests on DC Generators

0714E&A108.	Analyze Dc Motors.

0714E&A109.	Perform Tests On DC Motors.

Digital Electronics

0714E&A110.	Verify Truth Tables of Digital Gates

0714E&A111.	Construct & Verify Combinational Logic Circuit

0714E&A112.	Construct and Verify Function of Flip Flops

0714E&A113.	Construct Multivibrator with 555 IC

0714E&A114.	Construct Shift Registers and Counters with the Help of Flip Flops

Sensors and Actuators

0714E&A115.	Test Sensors and Transducers

0714E&A116	Calibrate Sensors and Transducers

0714E&A117	Interface the Sensor & Transducers

0714E&A118	Test Actuators

0714E&A119	Interface and control Actuator with sensors

Programmable Logic Controllers (PLCS)

0714E&A120	Install PLC software and Simulator

0714E&A121	Interface PLC with system

0714E&A122	Program PLC using Ladder Logic

0714E&A123	Design and Test the PLC

0714E&A124	Carry out Industrial Automation on PLC

Designing of Mechanical Members

0714E&A125	Calculate Stresses in Machine Parts

0714E&A126	Calculate diameter of cylinder for hoop and longitudinal stresses

0714E&A127	Calculate thickness and diameter of spherical shell for circumferential stresses

0714E&A128	Design welded joints for transverse and parallel fillet under static and fatigue loading

0714E&A129	Calculate stresses due to initial tightening and external load on screws

0714E&A130	Design dimension of square and rectangular keys

0714E&A131	Design shafts subjected to twisting moment only

0714E&A132	Design flange coupling for specific torque

Industrial Process Control & Instrumentation

0714E&A133	Design & Implement a process control

0714E&A134	Install Transducer & Transmitters used in Industrial Applications

0714E&A135	Install the process Regulators

0714E&A136	Install the Variable Frequency Drive (VFD)

0714E&A137	Install Pneumatic & Hydraulic systems

Robotics

0714E&A138	Identify Various Parts of Robots

0714E&A139	Identify Various Parts of Robot Arm

0714E&A140	Develop Robot program

0714E&A141	Develop robotic arm structure

0714E&A142	Test the Robot

Mechatronic Projects

0714E&A143	Access Control System using RFID Reader RC522

0714E&A144	Finger print Sensor Based Biometric Attendance System

0714E&A145	Humidity and Temperature Monitoring System

0714E&A146	Smart Stick for Blinds

0714E&A147	Perform Line Following Robot

0714E&A148	Perform Smart Phone Control Robot Using Bluetooth

0714E&A149	Perform IoT Controlled based home automation system

0714E&A150	Perform Smart Phone Control Door Lock System

0714E&A151	Perform Security System Using Motion Detection

0714E&A152	Perform Water-level Detection in Tank

0714E&A153	Perform Mini CNC 2D Plotter

0714E&A154	Perform Voice Controlled Robot

Hydraulic and Pneumatics

0714E&A155	Operate Hydraulic Bench and Its Functions

0714E&A156	Calibrate Bourdon Tube and Diaphragm Pressure Gauge

0714E&A157	Operate Hydraulic Press

0714E&A158	Verify Bernoulli’s Equation

0714E&A159	Analyse the performance of Pumps

0714E&A160	Construct circuit for Double Acting Hydraulic Cylinder

0714E&A161	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder

0714E&A162	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking

0714E&A163	Set Speed and Direction of Hydraulic Motor

0714E&A164	Operate Pressure Measuring Instruments

0714E&A165	Operate Flow Regulating Devices

Entrepreneurial Skill

0714E&A166	Develop Entrepreneurial Skills

0714E&A167	Apply project information management and communications techniques

0714E&A168	Apply project human resources management approaches

0714E&A169	Direct human resources management of a project program

0714E&A170	Develop a project management plan

0714E&A171	Maintain business resources

0714E&A172	Develop a sales plan

0714E&A173	Plan and implement business-to-business marketing

0714E&A174	Address customer needs

0714E&A175	Manage personal finances

0714E&A176	Solve problems which jeopardize safety and security

0714E&A177	Coordinate a work team

0714E&A178	Lead small teams

0714E&A179	Plan and organize work

0714E&A180	Develop teams and individuals

0714E&A181	Apply problem solving techniques in the workplace using critical thinking

0714E&A182	Manage human resource services

  - Entry level of trainees

The entry for D.A. E National Certificate level 5, in Mechatronics Technology are

- A person having National Vocational Certificate level 4, in Mechatronics Technology.

- A person having Matric certificate with Science subjects

## Minimum qualification for teachers

- B.Sc. / B.E. in Mechatronics Engineering.

- 4 Years B. Tech in Mechatronics Technology with 3 Years teaching experience.

- D. A. E. in Mechatronics Technology with 7 Years teaching experience

- Must be able to communicate effectively

## Medium of instruction

English, and Urdu.

## Duration of the course:

The proposed curriculum is composed of 182 modules that will be covered in 4,927 Learning hours. Duration of the course is proposed to be Three years.

Level - 2 = 6 months.

Level - 3 = 6 months

Level - 4 = 1 Year

Level - 5 = 1 Year

The distribution of contact hours is given below:

Total.		4927 hours.

Theory.	1347 hours (27.34 %)

Practical.	3580 hours (72.66 %)

## Description and structure of the course

Following is the structure of the course:

| LEVEL 4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total | CH |
| Microprocessor applications | 4 | Technical | 5 | 18 | 23 | 2.3 |
| Identify Microcontroller Types and its Architecture | 4 | Technical | 4 | 24 | 28 | 2.8 |
| Interface Microcontroller with system | 4 | Technical | 6 | 36 | 42 | 4.2 |
| Carryout various Microcontroller Applications. | 4 | Technical | 4 | 30 | 34 | 3.4 |
| Apply Microcontroller to build Control System. | 4 | Technical | 2 | 18 | 20 | 2 |
| Develop workplace policy and procedures for sustainability | 4 | Generic | 1 | 18 | 19 | 1.9 |
| Manage meetings with customers | 4 | Generic | 1 | 18 | 19 | 1.9 |
| Manage and select your team (workforce) according job requirement | 4 | Generic | 1 | 18 | 19 | 1.9 |
| Manage workforce planning | 4 | Generic | 1 | 18 | 19 | 1.9 |
| Undertake project work | 4 | Generic | 1 | 18 | 19 | 1.9 |
| Prepare and implement negotiation | 4 | Generic | 1 | 18 | 19 | 1.9 |
| Maintain professionalism in the workplace | 4 | Generic | 1 | 18 | 19 | 1.9 |
| Installation of C++, IDE for Arduino and Python | 4 | Functional | 6 | 18 | 24 | 2.4 |
| Carryout Basic C++ Programming | 4 | Functional | 12 | 36 | 48 | 4.8 |
| Carryout Basic Programming using python | 4 | Functional | 7 | 42 | 49 | 4.9 |
| Perform basic programming using microcontrollers | 4 | Functional | 4 | 24 | 28 | 2.8 |
| Perform CNC Machine Operations | 4 | Technical | 4 | 12 | 16 | 1.6 |
| Perform CNC Lathe Operations | 4 | Technical | 10 | 30 | 40 | 4 |
| Perform CNC Milling Operations | 4 | Technical | 10 | 30 | 40 | 4 |
| Demonstrate Simple Mechanisms | 4 | Technical | 6 | 18 | 24 | 2.4 |
| Perform Kinematic Analysis of Simple Mechanisms | 4 | Technical | 3 | 9 | 12 | 1.2 |
| Demonstrate Mechanical Power Transmission Elements | 4 | Technical | 6 | 18 | 24 | 2.4 |
| Perform Kinematic Analysis of Mechanical Power Transmission Elements | 4 | Technical | 6 | 18 | 24 | 2.4 |
| Verify Basic Laws of Electrical Machines | 4 | Technical | 12 | 36 | 48 | 4.8 |
| Analyze an Alternator | 4 | Technical | 6 | 18 | 24 | 2.4 |
| Analyze Single Phase Motors | 4 | Technical | 6 | 18 | 24 | 2.4 |
| Analyze Special Purpose Motors | 4 | Technical | 8 | 24 | 32 | 3.2 |
| Analyze Three Phase Motors | 4 | Technical | 15 | 45 | 60 | 6 |
| Analyze Dc Generators | 4 | Technical | 7 | 21 | 28 | 2.8 |
| Perform Tests on DC Generators | 4 | Technical | 7 | 21 | 28 | 2.8 |
| Analyze Dc Motors. | 4 | Technical | 6 | 18 | 24 | 2.4 |
| Perform Tests On DC Motors. | 4 | Technical | 10 | 30 | 40 | 4 |
| Verify Truth Tables of Digital Gates | 3 | Technical | 7 | 21 | 28 | 2.8 |
| Construct & Verify Combinational Logic Circuit | 4 | Technical | 9 | 27 | 36 | 3.6 |
| Construct and Verify Function of Flip Flops | 4 | Technical | 8 | 24 | 32 | 3.2 |
| Construct Multi vibrator with 555 IC | 4 | Technical | 6 | 18 | 24 | 2.4 |
| Construct Shift Registers and Counters with the Help of Flip Flops | 4 | Technical | 8 | 24 | 32 | 3.2 |
| Test Sensors and Transducers | 4 | Technical | 7 | 21 | 28 | 2.8 |
| Calibrate Sensors and Transducers | 4 | Technical | 8 | 24 | 32 | 3.2 |
| Interface the Sensor & Transducers | 4 | Technical | 10 | 15 | 25 | 2.5 |
| Test Actuators | 4 | Technical | 10 | 15 | 25 | 2.5 |
| Interface and control Actuator with sensors | 4 | Technical | 8 | 12 | 20 | 2 |
|  |  |  | 260 | 939 | 1199 | 119.9 |
|  |  |  | 22% | 78% |  |  |

# Overview of the Curriculum for Mechatronics Technology

| Module Title and Aim | Learning Units | Theory Days/hours | Workplace Days/hours | Timeframe of modules |
| --- | --- | --- | --- | --- |

- Detail of Modules.

### Microprocessor applications

Objective: This Module covers the knowledge and skills required to Identify microprocessors, Identify function of microprocessors and their associated pins, Program a microprocessor using basic instruction set, Program a microprocessor to perform basic arithmetic and logical operation, Program a microprocessor to perform special operations

Duration:  23 Hours			Theory: 05 Hours			Practical: 18 Hours			Credit Hours: 3

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Identify microprocessors. | Trainee would be able to - Identify microprocessor ICs - Compare and contrast the characteristics of Microprocessor & Microcontroller | - Define microprocessor - Explain architecture of microprocessor in detail. - Explain microprocessor IC and pin configuration. - Describe applications of microprocessor Practical Activity: Identify the microprocessor ICs from cluster of ICs. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Different types of Microprocessor IC’s. - Instructional Manual - Data sheet | Class Room and Workshop |  |
| LU2 Identify function of microprocessors and their associated pins | Trainee would be able to: - List down the memory functions and their allocation - Identify pin configuration of microprocessor ICs using data sheet | - Describe Arithmetic and Logic units - Explain the bus structure of microprocessor - Understand the IOs and their application Practical Activity: List down pin configuration of microprocessor ICs by using data sheet | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Different types of Microprocessor IC’s. - Instructional Manual - Data sheet | Class Room and Workshop |  |
| LU3.Program a microprocessor using basic instruction set | Trainee would be able to: - List down Basic instruction set. - Practice on assembly language - Make a simple program using basic instruction set using assembly language | - Explain basic instruction set - Explain addressing mode - Explain data transfer via accumulator Practical Activity: Make a simple program using basic instruction set | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Different types of Microprocessor IC’s. - Instructional Manual - Data sheet - Assembler - Microprocessor development Board |  | Class Room and Workshop |
| LU4.Program a microprocessor to perform basic arithmetic and logical operation | Trainee would be able to List Down and practice arithmetic and logical operation. - Addition & Subtraction - Multiplication & Division - AND, OR, NOT operation - Jump operation | - Explain programming concepts - Explain arithmetic and logic operation | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Different types of Microprocessor IC’s. - Instructional Manual - Data sheet - Assembler - Microprocessor development Board |  | Class Room and Workshop |
| LU5. Program a microprocessor to perform special operations | Trainee would be able to - List Down and practice special function operations - Loops - Input / Output - Subroutines and interrupt - Explain Debugging and its procedure | - Explain programming concepts - Explain following special function operation - Loops - Input / Output - Subroutines and interrupt | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs.. | - Instructional Manual - Data sheet - Assembler - Microprocessor development Board |  | Class Room and Workshop |

### Identify Microcontroller Types and its Architecture

Objective: This Module covers the knowledge and skills required to Introduction to Microcontroller (Arduino, Raspberry Pi, etc.), Introduction to Arduino IDE, Program Arduino Microcontroller by using conditional and loop instructions and Perform programing using Timer, and Interrupt.

Duration:  28 Hours			Theory: 04 Hours			Practical: 24 Hours			Credit Hours: 2.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Introduction to Microcontroller (Arduino, Raspberry Pi, etc.) | Trainee would be able to - Identify the various Microcontrollers and their models. - Identify RAM, SRAM, RAM, ROM, EEPROM, EPROM. - Identify Timer, Counters, Clock/processing speed of Microcontrollers - Identify busses and Processing Speed (4, 8, 16, 32, 64 bits) - Identify number of Analog to digital converters (ADCs) and Digital to analog converter (DAC), - Identify Analog Input and Output Pins. - Identify Digital Input and Output Pins. - Identify the PWM Pins. - Identify the UART Pins. - Identify the I2C pins. | - Describe PIC Microcontroller - Explain Architecture of 8051 Microcontroller - Describe microprocessors and microcontrollers - Differentiate between micro-processor and micro controllers - Define the components of a micro-processing system (Central Processing Unit, Input Output Interfaces and Memory) - Explain the functions of various registers. - Understand interrupt structure of 8051. - Explain Analog to digital converters (ADCs) and Digital to analog converter (DAC), Practical Activity: Identify different types of microcontroller | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Different types of Microcontroller - Arduino Kit and shields - Raspberry kit and shields | Class Room and Workshop |  |
| LU2. Introduction to Arduino IDE | Trainee would be able to: - Perform the Arduino IDE Installations. - Perform Arduino Sketch/Programming - Use Arduino supporting libraries and functions - Write down sketch for Arduino in IDE. - Identify the Shields, Accessories, and Sensors for Arduino | - Explain Arduino software installation procedure - Explain different accessories for Arduino - Describe Arduino supporting libraries and functions Practical Activity: Install Arduino software. | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields | Class Room and Workshop |  |
| LU3. Program Arduino Microcontroller by using conditional and loop instructions. | Trainee would be able to: - Use Boolean Expressions - Use If, If-Else, Nested if Statements - Identify Logical Operators - Identify Simple Recursion - Use While Loop - Use for loop | - Explain Boolean expressions - Explain different loops used in Arduino programming Practical Activity: Make simple program using for loop of Arduino software | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields |  | Class Room and Workshop |
| LU4. Perform programing using Timer, and Interrupt. | Trainee would be able to - Identify Timers in microcontroller - Identify Interrupts in microcontroller - Differentiate Internal and External interrupts - Identify Interrupt priority - Use While Loop - Use for loop | - Define interrupt - Explain types of interrupts - Explain interrupts priority mechanism Practical Activity: - Make simple program using while loop of Arduino software | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields |  | Class Room and Workshop |

### Interface Microcontroller with system

Objective: This Module covers the knowledge and skills required to Interface Analog Sensor with microcontroller, Interface Digital Sensors with microcontroller, Interface 7 segment display and LCD display with microcontroller, Interface Servomotor with microcontroller, Interface Serial (UART) based communication and Interface I2C based  communication.

Duration:  42 Hours			Theory:  06 Hours			Practical: 36 Hours			Credit Hours: 3.0

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Interface Analog Sensor with microcontroller. | Trainee would be able to - Identify Analog Sensors and pin configurations - Open Arduino IDE software. - Select and connect analog sensor to Analog Input Pin. - Make a source code/sketch. - Compile and Debug - Run Program. - Get results (Analog values) | - Describe interfacing - Describe five senses: (vision, hearing, smell, taste, and touch) - Describe input and output of sensors and actuators. - Describe Acceleration, Shock and Vibration Sensors - Explain SCADA as interfacing techniques using ADC and DAC Practical Activity: Identify different types of microcontroller | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Microcontroller - Analog Sensor - Arduino IDE Software | Class Room and Workshop |  |
| LU2. Interface Digital Sensors with microcontroller. | Trainee would be able to: - Identify Digital Sensors and pin configurations - Open Arduino IDE software. - Select and connect digital sensor to Analog Input Pin. - Make a source code/sketch. - Compile and Debug - Run Program. - Get results (Digital values 0 or 1) | - Explain digital sensor - Proximity sensor - Limit Switch - Pressure switch - Level Switch - Thermostat - Explain installation procedure for interfacing of digital sensors with microcontroller Practical Activity: Design elevator control system | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields - Proximity sensor - Limit Switch - Pressure switch - Level Switch - Thermostat | Class Room and Workshop |  |
| LU3. Interface 7 segment display and LCD display with microcontroller. | Trainee would be able to: - Identify pin configurations - Open Arduino IDE software. - Select and connect LCD display Module to Pin.? - Make a source code/sketch to display the number 3 on 7 segment display, and display text PAKISTAN on LCD display Module. - Compile and Debug - Run Program. - Get results (PAKISTAN, 3) | - Explain 7 segment LCD display - Explain Arduino software interfacing technique in detail - Explain source code programming concept Practical Activity: Show the elevator level on LCD display | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs.. | - Arduino Kit and shields - 7 Segment - LCD display |  | Class Room and Workshop |
| LU4. Interface Servomotor with microcontroller | Trainee would be able to - Identify pin configurations - Open Arduino IDE software. - Select and connect Servo motor to desired Pin. - Make a source code/sketch to rotate about 45 degrees. - Compile and Debug - Run Program. - Get results (Rotation value 45o) | - Explain servo motor - Explain servo motor interfacing technique with microcontroller - Explain Arduino software interfacing technique in detail - Explain source code programming concept Practical Activity: - Control conveyor belt with the help of sero motor | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields - Servo motor |  | Class Room and Workshop |
| LU5. Interface Serial (UART) based communication | Trainee would be able to - Open Arduino IDE software. - Connect module serial out TX pin with Arduino board serial RX pin and connect module serial RX pin with Arduino board TX pin. - Make a source code and set baud rate. - Compiling and Debugging. - Run Program. | - Explain Universal Asynchronous Reciever transmitter (UART) - Explain Universal Asynchronous Reciever transmitter (UART) interfacing technique with microcontroller - Explain Arduino software interfacing technique in detail - Explain source code programming concept Practical Activity: - Program the Arduino to show the speed of servo motor via Serial interface Universal Asynchronous Reciever transmitter (UART) | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields - Universal Asynchronous Reciever transmitter (UART) |  | Class Room and Workshop |
| LU6. Interface I2C based communication | Trainee would be able to - Open Arduino IDE software. - Connect two wires of sensor with SCL and SDA pins of Arduino board. - Make a source code using two wire (I2C) libraries. - Compiling and Debugging. - Run Program | - Explain I2C Board - Explain I2C protocol - Explain I2C Board interfacing technique with microcontroller - Explain Arduino software interfacing technique in detail - Explain source code programming concept Practical Activity: - Interface the LCD display via I2C protocol | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields - Microcontroller Board - LCD display - Resistor |  | Class Room and Workshop |

### Carryout various Microcontroller Applications

Objective: This Module covers the knowledge and skills required to Perform a task by connecting level/floating sensor and ON/OFF relay with microcontroller, Perform a task by connecting light sensor and servomotor with microcontroller, Perform a task by connecting pressure sensor and pneumatic actuator with microcontroller and Perform a task by connecting potentiometer and Servomotor with microcontroller.

Duration:  34 Hours			Theory: 04 Hours			Practical: 30 Hours			Credit Hours: 3

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Perform a task by connecting level/floating sensor and ON/OFF relay with microcontroller | Trainee would be able to - Identify level sensors to detect the level of water. - Identify relay to ON/OFF the water pump. - Identify the Input and Output pins. - Identify the Digital, Analog and PMW pins. - Make a layout and connect components - Write, compile, debug and upload a program to microcontroller. - RUN and Get results (ON/OFF depends on level) | - Explain level sensors - Explain relay - Explain input and output pins of microcontroller Practical Activity: Design water level control of tank by using floating sensor and ON/OFF relay | Total 10 Hrs. Theory: 01 Hrs. Practical: 09 Hrs. | - Microcontroller - Arduino IDE Software - Float switch - Water tank with Accessories - LEDs - Resistors - Water Pump - Relay - Power Supply | Class Room and Workshop |  |
| LU2. Perform a task by connecting light sensor and servomotor with microcontroller | Trainee would be able to: - Identify light sensors to detect the resistivity of light. - Identify servo motor to rotate/position the system. - Identify the Input and Output pins. - Identify the Digital, Analog and PMW pins. - Make a layout and connect components - Write, compile, debug and upload a program to microcontroller. - RUN and Get results (Position depends on resistivity) | - Explain function of light sensor - Explain function of servo motor - Describe Arduino programming Practical Activity: Design Solar panel steering system with help of light sensor and servo motor | Total 10 Hrs. Theory: 01 Hrs. Practical: 09 Hrs . | - Arduino Kit and shields - Arduino IDE Software - Servo Motor - Servo Motor drive - Servo Motor Manual - Light Sensor Module - Power Supply | Class Room and Workshop |  |
| LU3. Perform a task by connecting pressure sensor and pneumatic actuator with microcontroller | Trainee would be able to: - Identify pressure sensors. - Identify pneumatic actuator to apply force in system. - Identify the Input and Output pins. - Identify the Digital, Analog and PMW pins. - Make a layout and connect components - Write, compile, debug and upload a program to microcontroller. - RUN and Get results (Force depends on pressure) | - Explain function of pressure sensor - Explain function of pneumatic actuator - Describe Arduino programming Practical Activity: Design pressure safety system in a pneumatic line | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields - Arduino IDE Software - Pressure Sensor - Pnuematic Actuator - Solenoid - Relay - Pnuematic Power Supply - Pnuematic Pipe |  | Class Room and Workshop |
| LU4. Perform a task by connecting potentiometer and Servomotor with microcontroller. | Trainee would be able to - Identify potentiometer to detect the resistance. - Identify servomotor to position the system. - Identify the Input and Output pins. - Identify the Digital, Analog and PMW pins. - Make a layout and connect components - Write, compile, debug and upload a program to microcontroller. - RUN and Get results (Angle depends on Potentiometer) | - Explain function of potentiometer - Explain digital, Analog and PMW pins - Describe Arduino programming Practical Activity: - Control the speed of servo motor with the help of potentiometer | Total 07 Hrs. Theory: 01 Hrs. Practical: 06 Hrs. | - Arduino Kit and shields - Arduino IDE Software - Potentiometer - Servo Motor - Servo Motor drive - Servo Motor Manual - Electrical Power Supply |  | Class Room and Workshop |

### Apply Microcontroller to build Control System

Objective: This Module covers the knowledge and skills required to Build a Access Control System Using RFID Reader and Build a System to Control Light Using Smart Phone

Duration:  20 Hours			Theory: 2 Hours			Practical: 18 Hours			Credit Hours: 2.4

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Build a Access Control System Using RFID Reader | Trainee would be able to - Identify RFID (radio frequency identification devices). - Identify tag, read/write devices for RFID - Identify host system for data collection, processing, and transmission. - Identify the Digital, Analog and PMW pins. - Make a layout and connect components - Write, compile and debug a program for writing - Write, compile and debug a program for reading - Upload a program to microcontroller for writing a valid key. - Upload a program to microcontroller for reading key and unlock the door if key is valid. | - Explain function of Radio Frequency Identification reader (RFID) - Explain debugging process - Describe Arduino programming Practical Activity: Design Access control system using Radio Frequency Identification RFID reader | Total 10 Hrs. Theory: 01 Hrs. Practical: 09 Hrs. | - Microcontroller - Arduino IDE Software - RFID Reader - RFID Card - LEDs - Resistors | Class Room and Workshop |
| LU2. Build a System to Control Light Using Smart Phone | Trainee would be able to: - Identify light sensors to detect the resistivity, Intensity of light. - Identify servomotor to rotate/position the system. - Identify the Input and Output pins. - Identify the Digital, Analog and PMW pins. - Make a layout and connect components - Write, compile, debug and upload a program to microcontroller. - RUN and Get results (Position depends on resistivity) | - Explain function of light sensor - Explain resistivity and intensity of light terminology - Describe Arduino programming Practical Activity: Design home automation system using smart phone | Total 10 Hrs. Theory: 01 Hrs. Practical: 09 Hrs. | - Microcontroller - Arduino Kit and shields - Arduino IDE Software - Lamp - Relay - Smart Phone - Smart Phone App | Class Room and Workshop |

    - @Installation of C++, IDE for Arduino and Python

Objective: This module covers the knowledge and skills required to install Turbo-C, Arduino IDE and Python IDE software and configure them.

Duration:  		24 Hours			Theory:      06 Hours				Practice: 	18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Install Turbo C++ on Windows | Trainee will be able to: - Create Turbo-C directory inside C drive and extract the tc3. - unzip inside c:\turboc - Double click on install.exe file - Click on the TC application file located inside c:\TC\BIN to - Write the C program. | - System Software architecture - C++ as a language - OOP Concept - Interpreter and Script Mode - Turbo-C layout (GUI), menus and settings. PRACTICAL - Install the Turbo-C software - Configure Directories of Turbo-C | Theory-02 Hrs Practical-06 Hrs Total- 08 Hrs | - Computer or Laptop System - Turbo C++ Software | Computer Lab |
| - Install Arduino IDE on Windows | Trainee will be able to: - Download the Arduino Software (IDE) - RUN setup file and choose components to install. - Choose the installation directory and complete installation. - Proceed with board specific instructions (Uno, Mega, etc.) - Open software to view IDE. | - Arduino microcontroller - IDE working concept - Arduino IDE Software - Describe the software installation procedure - Specify the Arduino boards with their specifications - Arduino IDE layout (GUI), menus and settings. PRACTICAL - Install the Arduino IDE software - Configure given board at Arduino IDE | Theory-02 Hrs Practical-06 Hrs Total- 08 Hrs | - Computer or Laptop - Arduino Software - Arduino Development Board - SD Card | Computer Lab |
| - Install Python IDE on Windows | Trainee must be able to: - Download the Python 3 Installer - Run the installer - Download the python IDE - Identify Interpreter vs Script Mode - Install VNC Server - Configure VNC Server graphically - Configure VNC Server at the command line - Connecting to your Raspberry Pi with VNC Viewer - Establishing a direct connection - Establishing a cloud connection - Authenticating to VNC Server - Creating a virtual desktop | - Raspberry Pi microcontroller - Remote Connection and its types - VNC Server and client - Describe the syntax and semantics. - Differentiate Programs and Algorithms. - Cloud computing and its connection - Describe authentication system - Define virtual desktop PRACTICAL - Install the Python IDE software - Install VBC Server and Client - Configure remote Connection using VNC - Connect to Cloud | Theory-02 Hrs Practical-06 Hrs Total- 08 Hrs | - Computer or Laptop - Python Software - VNC Software - Cloud Service - Raspberry Pi Development Board - SD Card | Computer Lab |

    - @Carryout Basic C++ Programming

Objective: This module covers the knowledge and skills required to write programs for arithmetic, trigonometric operations. Also utilize counters, conditional statements, recursion and array structure in the code for complex applications.

Duration:     42	Hours			Theory:     06	    Hours				Practice:     36     Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop a computer program for mathematic functions | Trainee will be able to: - Open turbo C++ software - Create new file - Write a program for addition / subtraction / multiplication /Division. - Save and run the program - Identify the error in compiler - Remove the error if required | - Explain program files - Program and output file creation - Bare minimum code - Main program concept - Program compilation, debugging and execution - Calculation and output representation PRACTICAL - Create a C++ code for Simple calculator | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Turbo-C software | Computer Lab |
| - Develop a computer program for trigonometric functions | Trainee will be able to: - Open turbo C++ software - Create new file - Write a program for sin / cosine / tan - Save and run the program - Identify the error in compiler - Remove the error if required | - Header files - Program and output file - Debugging and execution - Calculation and output representation PRACTICAL - Create a C++ code for Scientific calculator | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Turbo-C software | Computer Lab |
| - Develop a computer simple program for given problem. | Trainee must be able to: - Analyze the given problem - Open the IDE for coding - Code a simple program - Compile a code - Debug the code (in case of error) - Run a code | - Knowledge of IDE - Application of software in real world. - Debugging and execution of program - Representation of output PRACTICAL - Create a C++ code for given problem and debug it | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Turbo-C software | Computer Lab |
| - Develop a program based on control structures | Trainee must be able to: - Identify the keywords and identifiers - Perform comments, indentation and statements - Recognize variables and data types - Check standard Input and Output - Use Operators - Use of IF statement - Use of IF ELSE statement - Use of SWITCH statement | - Control flow structure - Data types - Comments, indentation and statement - Application of conditions - Conditional operators PRACTICAL - Create a C++ code for student mark sheet, Display result after passing the given criteria | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Turbo-C software | Computer Lab |
| - Develop program using loop structures | Trainee must be able to: - Use of FOR loop - Use of WHILE loop - Use of DO-WHILE loop - Use of nested loop - Use Break and Continue | - Loops structure - Types of loops - Nested loops - Program break PRACTICAL - Create a C++ code to calculate prime number between 0 to 1000 | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Turbo-C software | Computer Lab |
| - Develop a program using Arrays | Trainee must be able to: - Use of Arrays - Use of one dimension array declaration. - Use of Array declaration. - Use of Array initialization. - Use of Array elements accessing. - Use of Two dimension array declaration. | - Array structure - One dimension and poly dimension arrays - Array declaration, initialization and access types. - Application of arrays - Types of arrays PRACTICAL - Create a C++ code for to perform 2D-array addition and subtraction | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Turbo-C software | Computer Lab |

    - @Carryout Basic Programming using python

Objective: This module covers the knowledge and skills required to utilize Linux Terminal commands, handle file/folder processing in Linux environment. It also covers python programming to write code which includes Arithmetic & String operations, Conditional statements, recursion, functions, data structures and file handling operation.

Duration:     49	Hours			Theory:     07	    Hours				Practice:     42     Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Introduction to LINUX commands | Trainee will be able to: - Opening Linux Terminal - Perform Basics of Linux Terminal - Use Commands to display directory - Create file on the directory - Use Commands to run the file on LINUX | - Intro to Linux - Intro to terminal - Basic Terminal commands - Knowledge of Directory Structure - Browsing, file/folder creation and execute commands PRACTICAL - Open Linux terminal and create a new file in new directory. - Open Linux terminal and append the given file | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Linux OS | Computer Lab |
| - Develop a program based on control structures | Trainee will be able to: - Identify Basic Data Types - Use Assignment Operators - Use print() function - Use Comments in Python - Use input() functions - Identify the Code Blocks and Indentation - Identify Reserved words in Python - Use Assignment Operators and Expressions - Use Arithmetic Operations - Identify Precedence and Associativity - Identify Doc string | - Data types used in linux - Assignment operator, print, input, utilization - Working of comments, code blocks, keywords - Development of code via operators and expressions - Use of arithmetic operators - Use of precedence and associativity in expression - Implementation and importance of Doc string PRACTICAL Create a Python code for calculator | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Linux OS - Python IDE | Computer Lab |
| - Develop program Using Conditional Structures and Operators | Trainee must be able to: - Use Boolean Expressions - Use If Statement - Use If-Else - Use Nested-If - Use If-Elif-Else - Identify Membership Operators - Identify Logical Operators - Use Bitwise Operators - Identify Simple Recursion - Use Factorial with Recursion - Program to display Fibonacci sequence using recursion - Use Indirect Recursion | - Basics of Boolean expression - Conditional program flow - Utilization of If, If-Else, Nested-If and If-Elif-If - Logical operator and their utilization - Knowledge of Bitwise operation - Intro to recursion - Implementation of recursion - Knowledge and implementation of Indirect recursion PRACTICAL Create a Python code for Elevator | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Linux OS - Python IDE | Computer Lab |
| - Develop program using loop | Trainee must be able to - Use While Loop - Use While-break - Use While-Continue - Use for-list - Use for-range - Use for-continue-break - Program to display prime numbers in an interval - Calculate factorial - Compute Fibonacci Series | - While loop implementation - Break implementation - Continue implementation - List basic and its utilization - Range basics and its utilization PRACTICAL Create a Python code to record student database (15 students) | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Linux OS - Python IDE | Computer Lab |
| - Develop a program using Functions | Trainee must be able to - Use of simple FUNCTIONS. - Use of Function Declaration. - Use of Function Prototype. - Use of Function Arguments - Use of Function definition. - Use of Function Call. - Use of passing parameters to Function. - Use of Function Arbitrary Arguments | - Function basics - Function structure and its types - Implementation of Parameter passing functions - Knowledge of sub function PRACTICAL Create a Python code to retrieve information of student from database (15 students) | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Linux OS - Python IDE | Computer Lab |
| - Develop a program using Python Data Structure | Trainee must be able to - Declare Strings - Create Lists in python - Apply Lists methods - Constructing Tuples in python - Apply Tuples methods - Create Sets - Use Operators on Sets - Create Dictionary - Methods to query information from dictionaries - Create Strings - Formatters and Operator on String | - String Structure basics - Use of string in lists - List functions utilization - Tuple Basics - Use of string in Tuples - Tuple functions utilization - Set basics - Use of String in Sets - Sets functions utilization - Knowledge and implementation of string formats - String functions PRACTICAL - Create a Python code to make a dictionary software | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Linux OS - Python IDE | Computer Lab |
| - Python file handling | Trainee must be able to - Create a Text File - Append Data to a File - Read a File - Read a File line by line - Identify File Modes in Python - Handling of CSV file - Handling of Excel File | - Text file handling - File Creation, Deletion, and append function in python - CSV file handling and operations - Excel file handling and operation PRACTICAL - Create a Python code to retrieve data from CSV - Create a Python code to retrieve data from Excel | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Linux OS - Python IDE | Computer Lab |

### @Perform basic programming using microcontrollers

Objective: This module covers the knowledge and skills required to write programs for Arduino micro controller to implement mathematical, trigonometric, logical   and recursive operations.

Duration:     28	Hours			Theory:     4	    Hours				Practice:     24 	     Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Introduction to Arduino IDE | Trainee will be able to: - Perform the Arduino IDE Installations. - Perform Arduino Sketch / Programming - Use Arduino supporting libraries and functions - Write down the sketch for Arduino in IDE. - Identify the Shields, Accessories, and Sensors for Arduino | - Knowledge of Arduino and its specification - Knowledge of Arduino IDE and its details - Application of support libraries - Utilization of boards and shields in Code. PRACTICAL Create a Arduino Code and configure the board on IDE. | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Arduino Software | Computer Lab |
| - Program Arduino Microcontroller for mathematical function | Trainee will be able to: - Open Arduino IDE software. - Connect Display with Arduino - Make a source code/sketch for addition / subtraction / - Multiplication / Division. - Compile and Debug - Run Program. - Display results | - Understanding of Arduino IDE - Understanding of connection to PC - Application of arithmetic operation in code. - Debugging of arithmetic program - Execution of Code - Presentation of arithmetic operation’s result PRACTICAL Program Arduino Microcontroller to make calculator | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Arduino Software - Arduino Development Board | Computer Lab |
| - Program Arduino Microcontroller for Invers Trigonometric function | Trainee must be able to: - Open Arduino IDE software. - Connect LCD Display with Arduino - Make a source code/sketch for inverse sine / cosine / tan - Compile and Debug - Run Program. - Display the results | - Information about LCD module - LCD Display information - Understanding of LCD commands - LCD Connection - Application of trigonometric operation in code. - Debugging of trigonometric program - Execution of Code - Presentation of arithmetic operation’s result PRACTICAL Program Arduino Microcontroller to calculate and display angle on LCD with help of sides | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Arduino Software - Arduino Development Board | Computer Lab |
| - Program Arduino Microcontroller by using conditional and loop instructions. | Trainee must be able to - Use Boolean Expressions - Use If, If-Else, Nested if Statements - Identify Logical Operators - Identify Simple Recursion - Use While Loop - Use for loop - Display values from 1 to 100 as per given instructions. | - Basics of Boolean expression and logic levels - Understanding Conditional program flow - Program flow chart creation - Understanding of increment and decrement operators - Implementation of logical operators - Knowledge of Recursion basics and its types - Implementation of recursion in code PRACTICAL Program Arduino Microcontroller to make visitor counter according to given criteria | Theory-01 Hrs Practical-06 Hrs Total- 07 Hrs | - Computer or Laptop - Arduino Software - Arduino Development Board - LEDs - Breadboard - Dupont wires - Resistors - 7 Segment | Computer Lab |

## Computer Integrated Machines

### @Perform CNC Machine Operations

Objective: This module covers the knowledge and  skills  required to Comply with occupational health, safety and environmentally sustainable work practice and Set CNC machine according to job requirements.

Duration:  16	Hours					Theory:  04 Hours					Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Comply with occupational health and safety and environmentally sustainable work practice | Trainee will be able to: - Follow the safety rules and work place instruction according to the provided manuals. - Report and identify the workplace hazards and accidents - Follow and identify the emergency procedures - Identify the current resources in connection with environmental issues. - Comply with the environmental regulations. - Seek the opportunities to improve the environmental risks and resources in order to increase the efficiency. | - Knowledge of workplace safety manuals - Knowledge of hazards associated with workplace - Knowledge of emergency procedures - Knowledge of environmental regulations Practical Activity: Demonstrate the first aid emergency procedures. | Theory-02 Hrs Practical-6 Hrs Total- 08 Hrs | - Workshop safety manuals - Hazard Awareness Chart - Personal Protective Equipment (PPE’s) | Class Room and workshop |
| - Set CNC machine according to job requirements | Trainee will be able to: - Follow / observe the safety and workplace instructions in order to avoid injuries and accidents. - Mount and set the cutting tool and work-piece according to procedures. - Set up and adjust the machine’s parameters according to the work specification. - Report the concerned person or incharge regularly about the uncertainties / deviations /errors which were face / observe during the operation. . | - Basic knowledge of CNC Machine - Machine’s process standards and functions. - Methods and techniques for adjustment of the operating parameters of machine. - Use of control panel. - Basic knowledge of machine margins and alignments. Practical Activity: Individually set up the CNC machine according to the requirement. | Theory-02 Hrs Practical-6 Hrs Total- 08 Hrs | - CNC Machine - Work piece | Workshop |

### @Perform CNC Lathe Operations

Objective: This module covers the knowledge and  skills required to Mount the job, Generate the program, Run simulation, Feed the program and Perform CNC lathe operations.

Duration:  40	Hours					Theory: 10 Hours					Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Mount the job | Trainee will be able to: - Mount the work-piece according to the working capacity of the machine along with the drawing / design of the job. - Select the appropriate work holding device(s) in order to achieve the dimensional accuracy Enter - Clamp the job firmly in the work holding device according to the standard procedure. - Attain the proper alignment of the tool / cutter and work-piece e.g. concentricity of rotating jobs as per set / standard practice - Follow / Keep the safety measures while mounting the work-piece so that the unwanted operation by machine may not be initiated | - Work place safety and health considerations - Work holding device - Use of Turret / Magazine and their sequence of tool mounting - Possible accidents and their counteractions Practical Activity: Mount the work piece as per instructions | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | CNC Lathe Work piece | Class Room and workshop |
| - Generate the Program | Trainee will be able to: - Interpret the job requirements, i.e. calculation of the offsets, errors and new reference point during the removal of materials, Define the absolute or incremental coordinates system, tool path strategies, machining features and tool compensation for generating the tool path as per standard procedure - Use the appropriate part programming credentials (Sequence, G-codes, M-codes, coordinates, feed, speed, tooling information etc.) according to the control parameters of the CNC machine - Keep the record of generated part program in soft / hard form in order to feed into a machine control unit as per standard procedures | - Reading Drawing - Mechanism of working of CNC lathe machine - Use of control panel and commands - Program debugging techniques - Use of Simulation software - G codes and M codes - Methods of calculating Coordinates techniques - Feed and speed concepts in Lathe Machine Practical Activity: Program the CNC Machine as per requirement | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | Computer system with CNC simulation software | Class Room and workshop |
| - Run Simulation | Trainee must be able to: - Feed the program of the drawing for the generated part into an appropriate simulation software’s platform and run the simulation for checking the tool gouge according to safety measures - Run the simulation, observe the movements of tool / cutter and verify the measurement with the defined the sequences - Identify the occurrence of errors and modify the program as per defined procedures and sequences | - Use of Simulation software - Use of control panel and commands - Program debugging techniques Practical Activity: Simulate the above program (i.e. LU2) | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | Coomputer system with CNC simulation software CNC lathe machine | Class Room and workshop |
| - . Feed the Program | Trainee must be able to - Ensure the proper synchronization between the machine’s control unit with the program of the part’s drawing as per standard operating procedures - Switch the machine in the receiving mode and feed the program of the desired part drawing / file into the machine control unit for execution as per standard operating procedures - Select the program’s file for the desired part for execution as per standard operating procedures | - Use of Simulation software - Methods of calculating Coordinates techniques - G codes and M codes - Feed and speed concepts in Lathe Machine - Mechanism of working of CNC lathe machine - Use of Lathe tools and their types with respect to operations and materials Practical Activity: Feed the program into the machine control unit for execution as per standard operating procedures | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs |  | Class Room and workshop |
| - . Perform CNC Lathe Operations | Trainee must be able to - Ensure and check the safety control operation of the CNC machines as per standards before executing the program file. - Control the feeds, speeds and override of the machine before execution according to the prescribed procedures - Switch the machine to execution mode (single block or auto) and press cycle start to run the machining sequence as per prescribed method. | - G codes and M codes - Work place safety and health considerations. - Use of Personal Protective Equipment - Mechanism of working of CNC lathe machine - Use of control panel and commands - Use of portable devices for CNC lathe - Coolant types along with benefits and uses - Possible accidents and their counteractions - Use of Turret / Magazine and their sequence of tool mounting - Use of Lathe tools and their types with respect to operations and materials - Feed and speed concepts in Lathe Machine Practical Activity: Individually perform the CNC lathe operations as per instructions | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | CNC lathe machine Work piece Different types of tools used in CNC lathe machine | Class Room and workshop |

### @Perform CNC Milling Operations

Objective: This module covers the knowledge and  skills required to Mount the job, Generate the program, Run simulation, Feed the program and Perform CNC milling operations.

Duration:  40	Hours					Theory: 10 Hours					Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Mount the job | Trainee will be able to: - Mount the work-piece by considering the working capacity of machine as well as job requirement according to the drawing/design - Select appropriate work holding device(s) in order to achieve dimensional accuracy and clamp the job firmly as per standard practice - Attain proper alignment of tool/ cutter and work-piece e.g. co-axiality, concentricity of rotating jobs as per set practice - Keep safe measures while mounting the work-piece so that unwanted operation by machine - may not be initiated as per safety precautions | - Work place safety and health considerations - Work holding device - Work piece mounting procedures - Possible accidents and their counteractions Practical Activity: Individually mount the workpiece as per Instructions | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | CNC milling Work piece | Class Room and workshop |
| - Generate the Program | Trainee will be able to: - Select appropriate CAM software according to the machine control unit and import 3D model into it as per standard procedure Rewrite and edit all according to the previous - Define reference point and apply material/stock for machining to the design/model as per job requirements - Apply machining feature(s), toolpath strategies and leads/links as per prescribed procedure - Generate part program file against the applied machining sequence according to the post processor of CNC machine - Use appropriate part programming credentials (Sequence, G-codes, M-codes, Coordinates, Feed, Speed, Tooling Information etc.) according to the CNC machine control unit - Keep record of generated part program file in order to feed into machine control unit as per standard procedure | - Reading Drawing - Mechanism of working of CNC lathe machine - Use of control panel and commands - Program debugging techniques - Use of Simulation software - G codes and M codes - Methods of calculating Coordinates techniques - Feed and speed concepts in milling Machine Practical Activity: Program the CNC Machine as per requirement | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | Computer system with CNC simulation software | Class Room and workshop |
| - Run Simulation | Trainee must be able to: - Feed the generated part program into appropriate simulation platform and run simulation for checking the tool gouge according to safety measures - Run simulation and verify movements of tool/cutter to get same results as per defined sequence Identify occurrence of errors and modify the program as per defined procedure | - Use of Simulation software - Use of control panel and commands - Program debugging techniques Practical Activity: Simulate the above program (i.e. LU2) | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | Coomputer system with CNC simulation software CNC milling machine | Class Room and workshop |
| - .Feed the Program into CNC milling | Trainee must be able to Ensure proper synchronization between machine control unit and part program file as per standard operating procedure P2. Switch machine to receiving mode and feed the desired part program file into machine control unit for further execution as per standard operating procedure - P3. Select the desired part program file for execution as per standard operating procedure Rewrite and edit all according to the previous | - Use of Simulation software - Methods of calculating Coordinates techniques - G codes and M codes - Feed and speed concepts in Lathe Machine - Mechanism of working of CNC lathe machine - Use of Lathe tools and their types with respect to operations and materials Practical activity: Feed the program into the machine control unit for execution as per standard operating procedures | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | CNC milling machine | Class Room and workshop |
| - . Perform CNC milling Operations | Trainee must be able to Ensure to control the safe operation of working on CNC machines before executing part program according to the safety measures Control the feeds, speeds and override of machine before operating according to the prescribed procedure Switch machine to execution mode (single block or auto) and press cycle start to run the machining sequence as per prescribed method Compare the block-wise movements of machining sequence thoroughly during operating of machine according to the part program file Complete the job and inspect its accuracy and precision according to the drawing/design | - G codes and M codes - Work place safety and health considerations. - Use of Personal Protective Equipment - Mechanism of working of CNC lathe machine - Use of control panel and commands - Use of portable devices for CNC lathe - Coolant types along with benefits and uses - Possible accidents and their counteractions - Use of Turret / Magazine and their sequence of tool mounting - Use of Lathe tools and their types with respect to operations and materials - Feed and speed concepts in Lathe Machine Practical Activity: Individually perform the CNC milling operations as per instructions | Theory-02 Hrs Practical-6 Hrs Total-08 Hrs | CNC milling machine Work piece Different types of tools used in CNC milling machine | Class Room |

## Machines and Mechanism

### @Demonstrate Simple Mechanisms

Objective: This competency standard covers the skills and knowledge required to Identify Commonly Used Links and    Joints, Identify simple mechanisms, Analyze simple mechanisms & Perform kinematic inversion of mechanisms

Duration:  32	Hours					Theory:  08 Hours					Practice:  24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Commonly Used Links and Joints | Trainee will be able to: - Recognize the pin joints. - Recognize sliding joints. - Identify cam joints. - Identify gear joints. - Differentiate the simple and complex links | Kinematics principles Knowledge of different types of joints Knowledge of machine elements Practical Activity: Individually identify the various types of provided joints | Theory-2 Hrs Practical-6 Hrs Total-8 Hrs | Different types of joints including sliding joints, cam joints, gear joints | Class Room and workshop |
| - Identify simple mechanisms | Trainee will be able to: - Identify the four bar mechanisms. - Identify Crank Rocker Mechanism - Identify Slider Crank Mechanism. - Identify quick return Mechanism. | Simple Mechanism and their types Terminology of mechanism Knowledge of crank rocker & slider cranck mechanism Practical Activity: List down all types of mechanism. | Theory-2 Hrs Practical-6 Hrs Total-8 Hrs | Crank & rocker Mechanism Simple Bar Mechanism Slider crank mechanism | Class Room and workshop |
| - Analyze simple mechanisms | Trainee must be able to: - Draw free hand sketch of simple and complex mechanism - Draw kinematic diagram of simple mechanisms. - Interpret kinematic diagram of simple mechanisms - Identify degrees of freedom in mechanisms | Simple Mechanism and their types Terminology of mechanism Kinematic diagram Techniques of Mechanism Analysis Practical Activity: Draw the kinematic diagram for simple machine. | Theory-2 Hrs Practical-6 Hrs Total-8 Hrs | Different types of simple mechanism Chart papers | Class Room and workshop |
| - . Perform kinematic inversion of mechanisms | Trainee must be able to - Perform Kinematic Inversion of four bar mechanisms - Perform Kinematic Inversion of Crank Rocker Mechanism - Perform Kinematic Inversion of Slider Crank Mechanism. - Perform Kinematic Inversion of Quick Return Mechanism | Kinematic inversion Techniques of Mechanism Analysis Kinematic Diagrams Practical Activity: Perform the kinematic inversion of any mechanism | Theory-2 Hrs Practical-6 Hrs Total-8 Hrs | Chart papers | Class Room and workshop |

### @Perform Kinematic Analysis of Simple Mechanisms

Objective: This module covers the knowledge and  skills  required to develop computer models of simple mechanisms, perform displacement  analysis and perform Velocity Analysis.

Duration:  12	Hours					Theory: 03 Hours					Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop computer models of simple mechanisms | Trainee will be able to: - Build Model of Slider Crank Mechanism Correct according to the previous suggestions - Simulate model of Slider Crank Mechanism - Build Model of Gear Mechanism - Simulate Gear Mechanism | - Knowledge of various Mechanisms - Procedure of simulation - Knowledge of computer and allied software | Theory-1 Hrs Practical- 3Hrs Total- 4 Hrs | - Computer - Related Software | Class Room and workshop |
| - LU2. Perform Displacement Analysis | Trainee will be able to: - Define a position of a point in a mechanism with a position vector. - Determine Linear Displacement. - Determine Angular Displacement | Describe linear displacement Describe angular displacement | Theory-1 Hrs Practical- 3Hrs Total- 4 Hrs | Notebook | Class Room and workshop |
| - Perform Velocity Analysis | Trainee will be able to: - Determine Liner Velocity of a point on a link. - Determine Angular Velocity of a point on a link. - Relate Linear and Angular velocities of a point on a link. | Explain linear and angular velocity. | Theory-1 Hrs Practical- 3Hrs Total- 4 Hrs | Notebook | Class Room and workshop |

### @Demonstrate Mechanical Power Transmission Elements.

Objective: This module covers the knowledge and  skills  required to cover the various methods of mechanical power transmission methods of belts, gears and chain drives.

Duration:  24 Hours						Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Interpret Belt Drives | Trainee will be able to: - Identify the Belt Types. - Identify the Pulley Types - Draw Belt Drives Mechanism. | - Belt drive & different types of belts - Pulley drive & different types of pulleys - Geometry of belt & pulleys Practical Activity: Draw belt drives mechanism | Theory-2Hrs Practical-6Hrs Total- 08Hrs | - Different types of belts - Different types of Pulleys | Class Room and workshop |
| LU2. Interpret Gears | Trainee will be able to: - Interpret Gears - Identify the gear types - Interpret Spur Gear Terminology. | - Different types of gears and their applications - Different types of terminologies used in gears Practical Activity: Interpret gears | Theory-2Hrs Practical-6Hrs Total- 08Hrs | - Different types of gears | Class Room and workshop |
| - Interpret Chain Drives | Trainee will be able to: - Select chain types. - Sketch single start chain | - Types of chains - Terminologies used in chains Practical Activity: Interpret chain drives | Theory-2Hrs Practical-6Hrs Total- 08Hrs | - Different types of chains | Class Room and workshop |

### @Perform Kinematic Analysis of Mechanical Power Transmission Elements.

Objective: This module covers the knowledge and  skills  required to perform the kinematic analysis of the various methods of mechanical power transmission methods of belts, gears and chain drives.

Duration:  24	Hours					Theory: 06 Hours						Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Calculate belt speed | Trainee will be able to: - Select the belts size - Determine the diameter of driver and driven pulleys. - Calculate velocity ratio for belts - Calculate the belt speed. | Knowledge of different types of belts and pulleys Knowledge of Belt Drives Geometry Knowledge of Belt Drives Kinematics Calculation procedure of belt size requirement Calculation procedure of velocity ratio | Theory-2 Hrs Practical-6 Hrs Total-08 Hrs | - Flat belts - V-belts - Timing belt - Pulleys | Class Room and workshop |
| - LU2. Calculate the velocity ratio of Gears | Trainee will be able to: - Determine the rotational velocities of driver and driven gears. - Calculate Velocity Ratio of Gears. | Knowledge of different types of gears Knowledge of Gear Geometry Knowledge of Gear Kinematics Calculation procedure of velocity ratio of gears | Theory-2 Hrs Practical-6 Hrs Total-08 Hrs | - Spur Gears - Helical Gears - Bevel Gears - Worm gear set | Class Room and workshop |
| - Calculate chain speed | Trainee will be able to: - Determine the Velocity Ratio for Chains. - Calculate Chain Speed | Knowledge of different types of chains and sprockets Knowledge of Chain Drive Geometry Knowledge of Chain Drive Kinematics Calculation procedure of velocity ration of chains Calculation procedures of selection of chain | Theory-2 Hrs Practical-6 Hrs Total-08 Hrs | - Sprockets - Chains | Class Room and workshop |

## AC & DC machines

### @Verify Basic Laws of Electrical Machines.

Objective: This module covers the knowledge and  skills required to verify Faraday’s law by moving permanent magnet inside the coil, verify Faraday’s law by moving coil near the magnet field, verify Faraday’s law using relative motion of coil and magnet, verify Faraday’s Law using simple loop generator, verify EMF through induction, verify Torque induce in a current carrying loop and verify Mutual induction.

Duration:  48	Hours			Theory: 12 Hours				Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Verify Faraday’s law by moving permanent magnet inside the coil. | Trainee will be able to: - Construct a coil. - Connect a Galvanometer with coil. - Move permanent magnet inside the coil fast and slow. - Record the effect of movement of magnet on reading of Galvanometer. - Hold the magnet inside the coil and keep it stationary Record the effect on reading of?? Galvanometer again. | - Describe coil. - Explain Galvanometer. - Explain permanent magnet with coil. - Explain Faradays law of electromagnetic induction. - Explain the direction of induced emf? - Explain induction. Practical Activity: Construct a coil, move permanent magnent between it and observe effect on galvanometer. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Galvanometer - Magnet. - Connecting leads. - Transformer - Rheostat - Loop - Voltmeter - Coil - DC source - Connecting leads. - Electrical kit | Class Room and workshop |
| - Verify Faraday’s law by moving coil near the magnet field. | Trainee will be able to: - Construct a coil. - Connect Galvanometer with coil. - Move the coil with various speeds (i.e. from slow to the fast) inside / outside the permanent magnet which is set at the stationary position. - Record the effect coil on via Galvanometer. - Hold the coil near the magnetic field and keep them stationary. - Record the effect via Galvanometer. | - Explain permanent magnet with coil. - Explain Faradays law of electromagnetic induction. - Explain induction. - Explain Galvanometer. - Explain permanent magnet near coil or in side coil stationary and moving condition. Practical Activity: Construct a coil, move permanent magnet near coil and observe effect on galvanometer. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Galvanometer - Magnet. - Connecting leads. - Transformer - Rheostat - Loop - Voltmeter - Coil - DC source - Connecting leads. - Electrical kit | Class Room and workshop |
| - Verify Faraday’s law using relative motion of coil and magnet. | Trainee will be able to: - Construct a coil. - Connect the Galvanometer with coil. - Make relative motion of coil and magnet. - Record the effect of movement on the coil via Galvanometer. | - Explain permanent magnet with coil. - Explain Faradays law of electromagnetic induction. - Explain Galvanometer. Explain permanent magnet near coil or in side coil stationary and moving condition. Practical Activity: Construct a coil, move coil around permanent magnent and observe effect on galvanometer. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Galvanometer - Magnet. - Connecting leads. - Transformer - Rheostat - Loop - Voltmeter - Coil - DC source - Connecting leads. - Electrical kit | Class Room and workshop |
| - Verify Faraday’s Law using simple loop generator. | Trainee will be able to: - Identify the single loop generator and its parts. - Select field winding. - Connect Galvanometer with single loop coil. - Rotate the single loop coil of generator in the field - Measure the voltage induces in the loop generator. - Record the effect via Galvanometer. | - Explain permanent magnet with coil of single loop - Explain Faradays law of electromagnetic induction. - Explain Galvanometer. - Explain permanent magnet near coil or in side coil stationary and moving condition. Practical Activity: Construct loop of wire, rotate it under permanent magnetic field and observe its effect on voltmeter | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Galvanometer - Magnet. - Connecting leads. - Transformer - Rheostat - Loop - Voltmeter - Coil - DC source - Connecting leads. - Electrical kit | Class Room and workshop |
| - Verify Torque induce in a current carrying loop | Trainee will be able to: - Construct a coil. - Apply DC voltage to the coil. - Place a current carrying loop in the coil. - Check the direction of force to verify the torque produce not clear? | - Why force produce on current carrying conductor. - Explain torque Explain permanent magnet with coil. - Explain Faradays law of electromagnetic induction. - Explain Galvanometer. Explain permanent magnet near coil or in side coil stationary and moving condition. Practical Activity: Construct loop of wire, rotate it under permanent magnetic field and verify torque of loop. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Galvanometer - Magnet. - Connecting leads. - Transformer - Rheostat - Loop - Voltmeter - Coil - DC source - Connecting leads. - Electrical kit | Class Room and workshop |
| - Verify Mutual induction | Trainee will be able to: - Construct two coils on the two different legs of the similar core. - Apply AC voltage to either one of the coil (act as primary winding). - Check the volts induce in the coil via voltmeter. | - Explain permanent magnet with coil. - Why emf induced in 2nd coil, when voltage is applied to 1st coil. - Explain mutual induction - Explain Faradays law of electromagnetic induction. - Explain Galvanometer. - Explain permanent magnet near coil or in side coil stationary and moving condition. Practical Activity: Construct two coils of different number of turns, apply voltage in one coil and observe emf produce in second coil. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Galvanometer - Magnet. - Connecting leads. - Transformer - Rheostat - Loop - Voltmeter - Coil - DC source - Connecting leads. - Electrical kit | Class Room and workshop |

### @Analyze an Alternator

Objective: This module covers the knowledge and  skills required to measure the stator winding resistance with DC test, perform no load test, perform short circuit test, measure the efficiency of an alternator, measure the effect of field current on terminal voltage, measure the effect of speed changes on alternator frequency and active power and perform parallel operation of Alternators.

Duration:  24	Hours			Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure the stator winding resistance with DC test. | Trainee will be able to: - Connect DC Ampere meter in series with armature. - Apply DC voltage to either one of the two phases. - Measure current and voltage via Multimeter. - Measure the resistance per phase by ohm’s law. | - Explain DC ampere meter, - Explain the procedure measuring current, voltage and resistance with Multimeter. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - DC ampere meter. - Coil - Dc volt meter. - Multimeter - Connecting leads. | Class Room and workshop |
| - Perform no load test. | Trainee will be able to: - Make connection according to diagram. - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Measure Iron losses from watt meter. | - Explain Alternator - Explain Iron loss | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Alternator. - Multimeter. - prime mover - connection leads. | Class Room and workshop |
| - Perform short circuit test. | Trainee will be able to: - Make connection according to diagram. - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage 10 to 15 % to its rated value. - Measure the copper loss from the watt meter. | - Explain alternator - Explain copper loss - Explain procedure to measure copper loss. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Alternator. - Multimeter. - prime mover - connection leads - Tachometer . | Class Room and workshop |
| - Measure the efficiency of an alternator. | Trainee will be able to: - Perform no load test to measure Iron losses. - Perform short circuit test to measure copper losses. - Apply formula to find efficiency. | - Explain procedure to measure iron loss. - Explain procedure of no load test. - Explain procedure of short circuit test. - Describe formula to calculate efficiency of alternator. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Alternator. - Multimeter. - prime mover - Connection leads. | Class Room and workshop |
| - Measure the effect of field current on terminal voltage. | Trainee will be able to: - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Draw no load curve between field current and terminal voltage. | - Explain procedure to run the alternator at rated speed. - Explain no load curve between field current and terminal voltage. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Alternator. - Multimeter. - prime mover - connection leads. - Dc ampere meter. - DC voltmeter. | Class Room and workshop |
| - Measure the effect of speed changes on alternator frequency and active power. | Trainee will be able to: - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Apply electrical load on alternator. - Measure the effects of load on alternator speed and frequency. - Adjust the prime mover speed to its rated value. - Measure the effect of this change on alternator speed and frequency. | - Explain active, reactive and apparent power. - Explain frequency. - Explain RMS value. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Alternator. - Multimeter. - prime mover - Connection leads. - Frequency meter. - Watt meter | Class Room and workshop |

### @Analyze Single Phase Motors

Objective: This module covers the knowledge and  skills required to measure the stator winding resistance with DC test, perform no load test, perform short circuit test, measure the efficiency of an alternator, measure the effect of field current on terminal voltage, measure the effect of speed changes on alternator frequency and active power and perform parallel operation of Alternators.

Duration:  24	Hours			Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Operate split phase single phase AC motor. | Trainee will be able to: - Make the connections according to circuit diagram. - Apply rated voltage to the stator. - Measure RPM and direction of rotation. - Disconnect the auxiliary winding from centrifugal switch. - Check the effect of disconnecting auxiliary winding of split phase motor. | - Expalin split phase single phase motor. - Explain working principle of single phase motor - Explain procedure of using tachometer. | Theory-2 Hrs Practical-6 Hrs Total- 08 Hrs | - Split phase motor. - AC ampere meter. - AC voltmeter - Tachometer - Connecting leads. | Class Room and workshop |
| - Operate capacitor start and Capacitor run single phase AC motor. | - Make connections according to circuit diagram. - Apply rated voltage to the stator. - Measure the RPM. - Remove the starting capacitor with centrifugal switch. - Measure the RPM again. - / Analyze / observe the effect of starting capacitor on motor’s torque. | - Explain function of capacitor in split phase motor.. - Explain torque. - Explain centrifugal switch. | Theory-2 Hrs Practical-6 Hrs Total- 08 Hrs | - Split phase motor. - AC ampere meter. - AC voltmeter - Tachometer - Connecting leads. | Class Room and workshop |
| - Operate shaded pole single phase motor. | - Make connections according to circuit diagram. - Apply rated voltage to the stator. - Change the applied voltage to measure the effect on motor’s speed. | - Explain shaded pole motor. - Explain working principle of shaded pole motor. | Theory-2 Hrs Practical-6 Hrs Total- 08 Hrs | - Shaded pole motor - AC ampere meter. - AC voltmeter - Tachometer - Connecting leads. | Class Room and workshop |

### @Analyze Special Purpose Motors

Objective: This module covers the knowledge and  skills required to operate and speed control of AC series motor, operate miniature (reluctance and hysteresis) single phase AC motors, construct and operate stepper motor and construct and operate and develop control circuit with the help of servo motor.

Duration:  32	Hours			Theory: 08 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Operate and speed control of AC series motor. | Trainee will be able to: - Connect the armature and field winding in series. - Apply rated voltage to the motor. - Measure and analyze the effect on motor’s speed by change in applied voltage. | - Explain series motor - Explain field winding. - Explain working principle of series motor. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - AC series motor. - AC ampere meter. - AC voltmeter - Tachometer - Connecting leads. | Class Room and workshop |
| - Operate miniature (reluctance and hysteresis) single phase AC motors. | Trainee will be able to: - Connect the stator of motor with single phase supply. - Apply mechanical load by hand - Measure the effect of load change on motor speed. - Draw torque speed curves. | - Explain miniature AC motor. - Explain hysteresis effect. - Explain load effect of motor with or without external force | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Split phase motor. - AC ampere meter. - AC voltmeter - Tachometer - Connecting leads. | Class Room and workshop |
| - Construct and operate stepper motor. | Trainee will be able to: - Connect the stator of the motor with control circuit. - Adjust the control voltage of control unit. - Measure the RPM. - Readjust the control voltage of control unit. - Measure the RPM again. - Analyze and distinguish the effect of Control voltage and RPM. | - Explain stepper motor. - Describe the function of control module. - Explain working principle of stepper motor. - Describe stepper motor application. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Stepper motor - DC ampere meter. - DC voltmeter - Tachometer - Connecting leads. | Class Room and workshop |
| - Construct, Operate and develop control circuit of servo motor. | Trainee will be able to: - Pair the servo motor with some encoder to provide position and speed feedback. - Compare the measured position to external input to the controller. - Check the effect of feedback on motor position. | - Describe servo motor. - Explain working principle of servo motor. - Explain encoder. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Servo motor module - DC ampere meter. - DC voltmeter - Connecting leads. | Class Room and workshop |

### @Analyze Three Phase Motors.

Objective: This module covers the knowledge and  skills required to identify and operate 3 phase Squirrel cage motor, verify rotating magnetic field , measure the starting and running current of three phase induction motor, draw torque slip curve of three phase induction motor, determine the slip of 3 phase induction motor by stroboscope, start 3 phase Induction motor with direct on line, start 3 phase Induction motor with Auto transformer starter, start 3 phase Induction motor with Star Delta starter, make connection to reverse the direction of induction motor, determine the efficiency of three phase induction motor, control the speed of three phase induction motor by changing primary voltage, control the speed of three phase induction motor by rotor resistance control method, operate two motors in cascade, start the synchronous motor using external prime mover and identify the effect of field current on three phase synchronous motor.

Duration:  60	Hours			Theory: 15 Hours				Practice: 45 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify and operate 3 phase Squirrel cage motor. | Trainee will be able to: - Identify and select three 3 phase squirrel cage motor. - Make connection as per diagram. - Connect the supply and interpret the result. | - Describe three phase squirrel cage motor. - Explain working principle of Squirrel cage motor. - Explain Squirrel cage motor in application. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phase Squirrel cage motor. - Three phase supply. - Electrical tool kit. | Class Room and workshop |
| - Verify rotating magnetic field | Trainee will be able to: - Connect the three terminals of stator winding with each other and with ground. - Apply three phase supply to the other terminals. - Place a compass between the stator and note the direction of rotating magnetic field. - Change the two phases with each other. - Observe / Note the direction of rotating magnetic field. | - Explain rotating magnetic field. - Explain three phase motor stator winding. - Describe of magnetic field with compass. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phase motor. - Three phase supply. - Compass - Earthing rod for grounding purpose. - Electrical tool kit. | Class Room and workshop |
| - Measure the starting and running current of three phase induction motor. | Trainee will be able to: - Connect the stator winding in star. - Connect an ammeter in series with stator. - Note down the ammeter reading at instant of just starting. - Note down the reading of ammeter at running condition. | - Explain Star connection on three phase motor. - Explain delta connection in three phase motor. - Explain ammeter working principle. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phase motor. - Three phase supply. - Compass - Earthing rod for grounding purpose. - Ampere meter. - Electrical tool kit. | Class Room and workshop |
| - Draw torque slip curve of three phase induction motor | Trainee will be able to: - Connect three phase induction motor with electro dynamo meter with Belt. - Start the motor and run at rated speed - Measure the RPM. - Change the torque with electro dynamo meter and note the speed - Calculate slip with given values of speed. - Draw graph between slip and torque. | - Explain torque - Define slip - Explain electro dynamo meter. - Define three phase. Dynamo. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phase motor. - Three phase supply. - Compass - Earthing rod for grounding purpose. - Ampere meter. - Electrical tool kit. - Techno meter | Class Room and workshop |
| - Determine the slip of three 3 phase induction motor by stroboscope | Trainee will be able to: - Connect the stator of three phase induction motor in star - Apply single phase AC supply to the stroboscope. - Mark dot on motor shaft with marker. - Apply three phase supply to the motor - Note the RPM of the motor with the help of stroboscope. - Calculate slip. | - Explain three phase motor stator. - Explain stroboscope. - Explain working of slip in three phase. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phase motor. - Three phase supply. - Earthing rod for grounding purpose. - Ampere meter. - Electrical tool kit. - Techno meter - stroboscope | Class Room and workshop |
| - Start three 3 phase Induction motor with direct on line. | Trainee will be able to: - Draw line diagram of direct-on three 3-phase motor Starter. - Identify and select 3 phase Induction motor. - Make Connection of motor with DOL as per diagram. - Switch on the 3 phase breaker and interpret the result. | - Explain DOL in three phases. - Explain three phase supply. - Explain three phase motor starter. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phase motor. - Three phase supply. - Earthing rod for grounding purpose. - Ampere meter. - Electrical tool kit. - Techno meter | Class Room and workshop |
| - Start 3 phases Induction motor with Auto transformer starter. | Trainee will be able to: - Connect power circuit with the help of diagram. - Connect control circuit with the help of diagram. - Apply rated voltage to the motor. - Gradually reduce the taping of auto transformer with the help of control circuit. | - Explain three phase auto transformer. - Explain three phase auto transformer control circuit. Describe working principle of auto transformer. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phase motor. - Three phase supply. - Earthing rod for grounding purpose. - Three phase auto transformer. - Electrical tool kit. |  |
| - Start 3 phases Induction motor with Star Delta starter. | Trainee will be able to: - Draw line diagram of a starter-delta starter. - Identify and select 3 phase Induction motor. - Make Connection of motor with star –delta- starter as per diagram. - Switch / Power on via three 3 phase breaker and interpret the result. | - Explain star and delta. - Describe working principle of star delta. - Explain connecting procedure of star and delta. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Contactors. - MCCB - Three phase motor. - Three phase supply. - Earthing rod for grounding purpose. - Three phase auto. - Electrical tool kit. |  |
| - Make connection to reverse the direction of induction motor. | Trainee will be able to: - Draw line diagram to reverse the direction of induction motor - Identify and select 3 phase Induction motor. - Make Connection of motor through 3 pole 2 way switch as per diagram. - Move the handle of 3 Pole 2 Way switch from off position to position 1. - Move the handle of 3 Pole 2 Way switch from Position 1 to off position and wait until the motor is stopped. - Move the handle of 3 Pole 2 way switch from off position to position 2 and interpret the result. | - Explain three phase motor forward and reverse direction connection. - Describe three phase forward and reverse direction two way switch application. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs rs | - Changeover. - MCCB - Three phase motor. - Three phase supply. - Earthing rod for grounding purpose. - Three phase auto. - Electrical tool kit. |  |
| - Determine the efficiency of three phase induction motor | Trainee will be able to: - Connect wattmeter, ammeter with the stator of three phase induction motor. - Run the motor at rated speed. - Measure the reading of wattmeter and voltmeter. - Stop the motor - Couple three phase induction motor and electro dynamo meter with belt. - Run the motor. - Supply single phase supply to the dynamo meter. - Adjust the supply of electro dynamo meter so that the maximum current passes through the motor. - Read the meters reading. - Calculate the efficiency of the motor by using these reading. | - Explain watt meter. - Explain ampere meter. - Explain voltmeter - Describe efficiency of three phase motor. - Explain electro dynamo meter. | Theory-0 Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Contactors. - MCCB - Three phase motor. - Three phase supply. - Earthing rod for grounding purpose. - Single phase supply. - Electro dynamo meter - Three phase auto. - Ampere meter. - Voltmeter - Watt meter. - Electrical tool kit. |  |
| - Control the speed of three phase induction motor by changing primary voltage | Trainee will be able to: - Connect the stator of the motor in star connection. - Insert rheostat in series with stator of the motor. - Set the rheostat resistance to its max value. - Run the motor - Change rheostat resistance and note the effect of speed changes. | - Explain three phase induction motor primary and secondary voltage. - Explain rheostat resistance. - Describe working of rheostat. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three pahse motor. - Three phase supply. - Rheostat - Connection wire. - Electrical tool kit |  |
| - Control the speed of three phase induction motor by rotor resistance control method | Trainee will be able to: - Select wound rotor induction motor. - Connect the stator of motor in star connection. - Connect a star connected starter with motor rotor. - Set rotor resistance to its max value. - Run the motor. - Change the rotor resistance and note the effect of speed change. | - Explain wound rotator induction motor. - Explain stator of wound motor. - Describe working principle of resistive control circuit. | Theory-0.4 Hrs Practical-1.6 Hrs Total- 2 Hrs | - Three phase wound motor. - Three phase supply. - MCCB - Contactors. - Earth rod for grounding. - Electrical tool kit. |  |
| - Operate two motors in cascade | Trainee will be able to: - Select two wound rotor induction motor. - Couple shaft of the both motors. - Connect the stator of motor 2 with the rotor of motor 1. - Connect the motor 2 rotor in star connection. - Apply three phase voltage to the stator of motor 1. - Control the speed of both motors by changing the rotor resistance of motor 2. | - Explain wound rotator induction motor. - Explain stator of wound motor. - Describe working principle of resistive control circuit in cascaded form . | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phase wound motor. - Three phase supply. - MCCB - Contactors. - Earth rod for grounding. - Electrical tool kit. |  |
| - Start the synchronous motor using external prime mover. | Trainee will be able to: - Couple the shaft of the synchronous motor with external prime mover (DC Shunt motor) - Connect the stator winding of synchronous motor in star connection. - Apply rated voltage to the stator. - Run the prime mover, which results in locking of rotor with stator magnetic field. | - Explain external prime mover. - Explain DC shunt motor. - Explain magnetic field. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Synchronous motor. - Dc shunt motor. - Three phase supply. - MCCB - Contactors. - Earth rod for grounding. - Electrical tool kit. |  |
| - Identify the effect of field current on three phase synchronous motor. | Trainee will be able to: - Apply rated voltage to stator. - Excite the field winding. - Change the field winding voltage and measure the effect on Armature current. - Draw the V curve between Armature Current and Field current. | - Explain three phase synchronous motor. - Explain field current and its effect on rotor speed. - Explain armature current. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Three phsae Synchronous motor. - Dc shunt motor. - Three phase supply. - MCCB - Contactors. - Earth rod for grounding. - Electrical tool kit. |  |

### @Analyze Dc Generators

Objective: This Module covers the knowledge and skills required to Calculate different terms used in armature winding of DC machines, Develop Single Layer Lap Winding, Develop Double Layer Wave Winding, Measure the resistance of field winding, armature winding and terminal polarity, Identify DC shunt generator, Identify DC series generator and Identify DC compound generator.

Duration:  28 Hour                               Theory: 07 Hours                       Practical: 21 Hours			Credit Hours: 2.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Calculate different terms used in armature winding of DC machines | Trainee would be able to - Count the number of slots (S) of armature. - Calculate pole pitch (Y). - Calculate Coil span / pitch. - Calculate Back pitch (Yb) - Calculate Front pitch (Yf) - Calculate Resultant Pitch (YR) - Calculate commutator pitch (Yc) | - Explain Pole pitch (Y) - Explain Back pitch (Yb) - Explain front pitch (Yf) - Explain Resultant pitch (YR) - Explain Commutator pitch (Yc) Practical Activity: Calaculate pole pitch, back pitch and commutator pitch | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Machine | Class Room |  |
| - Develop Single Layer Lap Winding | Trainee would be able to: - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commutator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Subtract the value of back pitch from the added value. - Connect added value from front side to subtracted value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. | - Explain types of winding - Explain Lap winding Practical Activity: Perform Layer Lap winding | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Machine - Armature wire - Winding machine | Class Room and Workshop |  |
| - Develop Double Layer Wave Winding | Trainee would be able to: - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentators pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Add the value of front pitch in previous added value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. | - Explain types of winding - Explain wave winding Practical Activity: Perform Double Layer wave winding | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Machine - Armature wire - Winding machine |  | Class Room and Workshop |
| - Measure the resistance of field winding, armature winding and terminal polarity | Trainee would be able to - Open the terminal box. - Connect the ohm meter with the terminal of both winding. - Read down ohm meter reading. - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. - Rotate the generator slowly by hand. - Touch the probe of galvanometer with each winding. - Identify armature with the deflection of galvanometer pointer. - Mark the polarity of terminals with the deflection of galvanometer. | - Explain the resistivity - Explain the procedure of measuring resistance of field winding of DC machine - Explain function of Galvanometer Practical Activity: Measure value of resistance of field winding with ohm meter | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Machine - Ohm Meter - Galvanometer |  | Class Room and Workshop |
| - Identify DC shunt generator | Trainee would be able to - Measure the terminal resistance of generator with ohm meter. - Identify DC shunt generator from measured value of resistance. - Disconnect both windings. - Identify the shunt field winding of shunt generator. - Identify armature of shunt generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. | - Explain DC shunt generator - Differentiate armature winding and field winding of shunt generator Practical Activity: Identify armature winding and field winding of DC shunt generator | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Shunt Generator - Ohm meter |  | Class Room and Workshop |
| - Identify DC series generator | Trainee would be able to - Measure the terminal resistance of generator with ohm meter. - Identify DC series generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series generator. - Identify armature of series generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. | - Explain DC Series generator - Differentiate armature winding and field winding of Series generator Practical Activity: - Identify armature winding and field winding of DC Series generator | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Series Generator - Ohm meter |  | Class Room and Workshop |
| - Identify DC compound generator | Trainee would be able to - Measure the terminal resistance of generator with ohm meter. - Identify DC Compound generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of generator. - Identify the shunt field winding of generator. - Identify armature of shunt generator. - Measure and record the resistance of both winding separately. - Measure and Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. | - Explain DC Compound generator - Differentiate armature winding and field winding of Compound generator Practical Activity: - Identify armature winding and field winding of DC Compound generator | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Compound Generator - Ohm meter |  | Class Room and Workshop |

### @Perform Tests on DC Generators

Objective: This Module covers the knowledge and skills required to Plot the Open circuit characteristics of a Separately excited D.C  generator, Plot the Open circuit characteristics of a D.C shunt generator, Plot the load characteristics of D.C shunt generator, Plot the load characteristics of D.C series generator, Plot the load characteristics of D.C compound generator for under, flat and over compounding, Perform parallel operation of two DC shunt generators and Determine copper, iron & friction losses of shunt generator by actual loading.

Duration:  28 Hours			Theory: 07 Hours			Practical: 21 Hours			Credit Hours: 2.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plot the Open circuit characteristics of a Separately excited D.C generator | Trainee would be able to - Connect DC supply to field coil through rheostat and ampere meter. - Rotate armature at constant speed. - Connect voltmeter across armature terminal. - Decrease the resistance of rheostat in steps and measure and record the relevant output voltage of armature and field current. - Repeat the step2 to step 4 till voltage of armature reach at its saturation point. - Plot the graph between field current and armature voltage from the recorded values. | - Explain Open circuit characteristics of a Separately excited D.C generator - Explain function of rheostat Practical Activity: Connect rheostat with Armature field, vary the value of resistance and observe voltage and current of armature field | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Separately Excited DC Generator - Rheostat - Voltmeter - Ampere meter - Connecting leads | Class Room and Workshop |
| - Plot the Open circuit characteristics of a D.C shunt generator | Trainee would be able to: - Connect DC supply to field coil through rheostat and ampere meter. - Rotate armature at constant speed. - Connect voltmeter across armature terminal. - Decrease the resistance of rheostat in steps and measure and record the relevant output voltage of armature and field current. - Repeat the step2 to step3 till voltage of armature reach at its saturation point. - Plot the graph between field current and armature voltage from the recorded values. | - Explain Open circuit characteristics of a D.C shunt generator - Explain function of rheostat Practical Activity: Connect rheostat with Armature field, vary the value of resistance and observe voltage and current of armature field | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC shunt Generator - Rheostat - Voltmeter - Ampere meter - Connecting leads | Class Room and Workshop |
| - Plot the load characteristics of D.C shunt generator. | Trainee would be able to: - Select DC shunt generator and connect variable load. - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. | - Explain load characteristics of a D.C shunt generator - Explain Armature of DC Shunt generator Practical Activity: Connect voltmeter and ampere meter with armature to calculate the values in DC Shunt Generator | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Shunt Generator - Volt meter - Ampere meter - Connecting leads - Load | Class Room and Workshop |
| - Plot the load characteristics of D.C series generator. | Trainee would be able to - Select DC series generator and connect variable load. - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Increase the load in steps unless specific reduction in terminal voltage will observed. - Plot the graph between load current and terminal voltage from the recorded values. | - Explain load characteristics of a D.C Series generator - Explain Armature of DC Series generator Practical Activity: - Connect voltmeter and ampere meter with armature to calculate the values in DC Series Generator | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs.. | - Single phase transformer | Class Room |
| - Plot the load characteristics of D.C compound generator for under, flat and over compounding | Trainee would be able to - Select DC compound generator and connect variable load. Correct as the previous - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Adjust field diverter on zero ohm. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. - Such off the load. - Adjust the field diverter at such position that the terminal voltages are equal to no load voltages. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. - Such off the load. - Adjust the field diverter at such position that the terminal voltages are greater than no load voltages. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. | - Explain load characteristics of a D.C Compound generator - Explain Armature of DC Compound generator Practical Activity: Connect voltmeter and ampere meter with armature to calculate the values in DC Compound Generator | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Single phase transformer - Volt meter - Ampere meter - Watt meter | Class Room and Workshop |
| - Perform parallel operation of two DC shunt generators. | Trainee would be able to - Connect the circuit according to the circuit diagram. - Place all switches in off position. - Run generator 1 at rated speed. - Reduce field resistance of generator 1 - Connect generator with bus bar after attaining rated voltage. - Run generator 2 with prime mover. - Reduce the field resistance of generator 2 to attain its rated voltage. - Close the switches for parallel operation. - Calculate and record the reading of voltage and current on different loads. | - Explain procedure of connecting 2 DC shunt Generator in Parallel - Explain procedure to reduce the field resistance of generator Practical Activity: - Connect 2 DC shunt generator in parallel and connect them to load and record the voltage and current readings | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Shunt Generator - Volt meter - Ampere meter - Load - Connecting leads - Rheostat | Class Room and Workshop |
| - Determine copper, iron & friction losses of shunt generator by actual loading. | Trainee would be able to - Connect ammeter in series with both winding. - Connect voltmeter in parallel with both winding. - Apply 10-20 DC volt. - Record the readings of voltmeter and ammeter. - Calculate Ra and Rsh. - Run the generator at rated speed with prime mover. - Turn on the load switch and record the meter reading. - Calculate input power and output power. - Calculate total losses (Copper loss, Friction loss, Iron loss) | - Explain types of losses - Copper Losses - Iron Losses - Friction Losses - Explain Shunt resistance and Armature resistance Practical Activity: Connect Load with DC Generator and calculate the losses. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - DC Shunt Generator - Volt meter - Ampere meter - Load - Connecting leads | Class Room and Workshop |

### @Analyze Dc Motors.

Objective: This module covers the knowledge and  skills required to identifies the competencies required to identify DC shunt motor, identify DC series motor and identify DC compound motor.

Duration:  24	Hours			Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify DC shunt motor | Trainee will be able to: - Measure the terminal resistance of motors with ohm meter. - Identify DC shunt motor from measured value of resistance. - Disconnect both windings. - Identify field winding of shunt motor. - Identify armature of shunt motor. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. | - Explain DC shunt motor. - Describe the resistance of dc motor winding. - Explain the working principle of DC shunt motor. - Explain armature and field winding. | Theory-2 Hrs Practical-6 Hrs Total- 08 Hrs | - DC shunt motor. - DC Ohm meter. - DC voltmeter - Connecting leads. | Class Room and workshop |
| - Identify DC series motor | Trainee will be able to: - Measure the terminal resistance of DC motors with ohm meter. - Identify DC series motor from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series motor. - Identify armature of series motor. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. | - Explain DC shunt motor. - Describe series field winding. - Describe armature winding. | Theory-2 Hrs Practical-6 Hrs Total- 08 Hrs | - DC series motor. - DC Ohm meter. - DC voltmeter - Connecting leads. . | Class Room and workshop |
| - Identify DC compound motor | Trainee will be able to: - Measure the terminal resistance of DC motor with ohm meter. - Identify DC Compound motor from measured value of resistance. - Disconnect both windings. - Identify the series field winding of motor. - Identify the shunt field winding of motor. - Identify armature of compound motor. - Measure and record the resistance of both winding separately. - Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. | - Explain Dc compound motor. - Describe armature winding, series field winding. - Describe shunt field winding | Theory-2 Hrs Practical-6 Hrs Total- 08 Hrs | - DC compound motor. - DC Ohm meter. - DC voltmeter - Connecting leads. | Class Room and workshop |

### @Perform Tests On DC Motors.

Objective: This module covers the knowledge and  skills required to Plot the load characteristics of D.C shunt motor, plot the load characteristics of D.C series motor, plot the load characteristics of D.C compound motor, control the speed of DC series motor, control the speed of DC shunt motor, start  DC shunt motor with 3 point starter, identify the effect of back EMF in DC motor, calculate BHP of motor by Brake test, calculate efficiency of the motor by Swinburne Test and perform Regenerative / Hopkinson/ Back to back Test

Duration:  40	Hours			Theory: 10 Hours				Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plot the load characteristics of D.C shunt motor | Trainee will be able to: - Couple electro dynamo meter to the DC motor with belt - Connect shunt motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter’s control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics for the DC shunt motor. | - Describe DC shunt motor. - Describe Electro dynamo meter. - Explain characteristic of DC shunt motor. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - DC shunt motor. - Electro dynamo meter. - DC power supply. - Motor belt. - Techno meter. | Class Room and workshop |
| - Plot the load characteristics of D.C series motor. | Trainee will be able to: - Select DC series motor and achieve connection as per diagram. - Turn on DC power supply and increase the load to full value. - Record the value of speed and current on each load. - Draw the graph between load current and speed |  | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Dc series motor. - DC power supply. - Motor belt. - Techno meter. | Class Room and workshop |
| - Plot the load characteristics of D.C compound motor* | Trainee will be able to: - Couple electro dynamo meter to the DC motor with belt - Connect compound motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC compound motor.. | - Explain DC compound motor. - Describe characteristic of DC compound motor. - Explain Dynamo meter. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - DC compound motor. - Electro dynamo meter. - DC power supply. - Motor belt. - Techno meter | Class Room and workshop |
| - Control the speed of DC series motor. | Trainee will be able to: - Make the connection as per circuit diagram. - Insert external resistances in series to armature and field coil. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different supply voltages. - Make graph between speed and applied voltage. | - Explain DC series control module. - Describe DC series motor load characteristic. - Describe procedure to control the speed DC series motor. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Dc series motor. - DC series Control module - DC power supply. - Motor belt. - Techno meter. | Class Room and workshop |
| - Control the speed of DC shunt motor. | Trainee will be able to: - Make the connection as per circuit diagram. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different field current. - Make graph between speed and field current - Fixed the field current and very armature current - Record the speed at different armature current. - Make graph between speed and armature current | - Explain DC shunt motor control module. - Describe DC Shunt motor load characteristic. - Describe procedure to control the speed DC shunt motor. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Dc shunt motor. - DC shunt Control module - DC power supply. - Motor belt. - Techno meter | Class Room and workshop |
| - Start DC shunts motor with 3 point starter. | Trainee will be able to: - Make the connection as per circuit diagram. - Switch on the supply and move starter handle from off position to position 1. - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. | - Explain DC shunt motor control module with three point starter. - Describe DC Shunt motor load characteristic. - Describe procedure to control the speed DC shunt motor with three point. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Dc shunts motor. - DC shunt Control module - DC power supply. - Motor belt. - Techno meter. | Class Room and workshop |
| - Identify the effect of back EMF in DC motor. | Trainee will be able to: - Select DC series motor and make connection as per diagram. - Turn on DC power supply and increase the load 0 to full value. - Record the value of speed and current on each load. - Calculate back EMF by using the recoded values and appropriate formula. - Make graph between speed and back EMF and interpret the result. | - Explain back EMF. - Describe DC series Motor. - Describe the effect of back EMF. - Explain Self-induction. - Explain Mutual induction. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - DC motor. - DC Control module - DC power supply. - Motor belt. - Techno meter. - DC ampere meter. - Dc voltmeter. |  |
| - Calculate Brake Horse Power (BHP) of motor by Brake test. | Trainee will be able to: - Connect the spring balance with one end of the rope. - Connect other end of rope with weight (W1). - Measure the radius of the pulley. - Hang the weight on the pulley of the motor and fix the spring balance end with earth. - Run the motor. - Measured the RPM of the motor. - Adjust the weight to pass full load current from motor. - Calculate spring balance weight and hanged weight (W1) - Calculate shaft torque. - Calculate BHP of motor by shaft torque. | - Explain brake horse power of motor. - Explain Torque. - Describe shaft torque. - Describe horse power conversion into Watt power. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - DC motor. - DC Control module - DC power supply. - Motor shaft pulley. - Rope with weight. - Techno meter. - DC ampere meter. - Dc voltmeter. |  |
| - Calculate efficiency of the motor by Swinburne Test | Trainee will be able to: - Perform DC test to find the value armature resistance and field resistance - Run the motor at rated voltage without load. - Adjust the value of shunt regulator to attain rated speed. - Measure the Field and no load current via ampere meter. - Calculate total copper losses and iron losses. - Calculate input and output power. - Calculate efficiency from calculated power. | - Explain efficiency. - Describe armature resistance. - Describe field resistance. - Explain copper loss. - Describe shunt regulator. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - DC shunt & compound motor. - DC Control module - DC power supply. - Techno meter. - DC ampere meter. - DC voltmeter. - Watt meter. |  |
| - Perform Regenerative / Hopkinson/ Back to back Test | Trainee will be able to: - Identify two shunt machines. - Couple the shafts of both machines. - Connect the machines electrically in such a way that one machine work as a motor and other as a generator. - Connect external power supply to the machines to overcome the machine losses. - Run both the machines at no load. - Reduce the field excitation of one machine which work as a motor. - Increase the field excitation of one machine which work as a generator. - Adjust the field winding in a way that voltage of generator becomes equal to the applied voltage which result the machines runs in parallel - Calculate the efficiency by using voltmeter and ammeter. | - Explain Hokinson test. - Explain shunt machine. - Describe working principle of shunt machine. - Explain machine losses. - Explain excitation of machine as a generator. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - DC shunt machine - DC Control module - Coupling & shaft - DC power supply. - Techno meter. - DC ampere meter. - DC voltmeter. - Watt meter. |  |

## Digital Electronics

### @Verify Truth Tables of Digital Gates

Objective: This Module covers the knowledge and skills required to Verify the truth table of AND gate, Verify the truth table of OR gate, Verify the truth table of NOT gate, Verify the truth table of NAND gate, Verify the truth table of NOR gate, Verify the truth table of XOR gate and Verify the truth table of XNOR gate

Duration:  28 Hours			Theory: 07 Hours			Practical: 21 Hours			Credit Hours: 2.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Verify the truth table of AND gate | Trainee would be able to - Identify the symbol of logic gate, IC & logic function. - Place (AND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Define Gate and its functions - Explain types of Gate - Explain gates symbol - Explain function of logic gate IC - Explain Function of AND gate - Explain truth table of AND gate Practical Activity: Construct AND gate and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Logic AND gate IC - Bread Board - Connecting wires - Digital Multimeter - LEDs - DC Supply | Class Room and Workshop |  |
| LU2 Verify the truth table of OR gate | Trainee would be able to: - Identify the symbol of logic gate, IC & logic function. - Place (OR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Define Gate and its functions - Explain types of Gate - Explain gates symbol - Explain function of logic gate IC - Explain Function of OR gate - Explain truth table of OR gate Practical Activity: Construct OR gate and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs : 02 Hrs. | - Logic OR gate IC - Bread Board - Connecting wires - Digital Multimeter - LEDs - DC Supply | Class Room and Workshop |  |
| LU3. Verify the truth table of NOT gate | Trainee would be able to: - Identify the symbol of logic gate, IC & logic function. - Place (NOT gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Define Gate and its functions - Explain types of Gate - Explain gates symbol - Explain function of logic gate IC - Explain Function of NOT gate - Explain truth table of NOT gate Practical Activity: Construct NOT gate and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Logic NOT gate IC - Bread Board - Connecting wires - Digital Multimeter - LEDs - DC Supply |  | Class Room and Workshop |
| LU4. Verify the truth table of NAND gate | Trainee would be able to - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Define Gate and its functions - Explain types of Gate - Explain gates symbol - Explain function of logic gate IC - Explain Function of NAND gate - Explain truth table of NAND gate Practical Activity: Construct NAND gate and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Logic NAND gate IC - Bread Board - Connecting wires - Digital Multimeter - LEDs - DC Supply |  | Class Room and Workshop |
| LU5. Verify the truth table of NOR gate | Trainee would be able to - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Define Gate and its functions - Explain types of Gate - Explain gates symbol - Explain function of logic gate IC - Explain Function of NOR gate - Explain truth table of NOR gate Practical Activity: Construct NOR gate and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Logic NOR gate IC - Bread Board - Connecting wires - Digital Multimeter - LEDs - DC Supply |  | Class Room and Workshop |
| LU6. Verify the truth table of XOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Define Gate and its functions - Explain types of Gate - Explain gates symbol - Explain function of logic gate IC - Explain Function of XOR gate - Explain truth table of XOR gate Practical Activity: Construct XOR gate and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Logic XOR gate IC - Bread Board - Connecting wires - Digital Multimeter - LEDs - DC Supply |  | Class Room and Workshop |
| LU7. Verify the truth table of XNOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Define Gate and its functions - Explain types of Gate - Explain gates symbol - Explain function of logic gate IC - Explain Function of XNOR gate - Explain truth table of XNOR gate Practical Activity: Construct XNOR gate and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Logic XNOR gate IC - Bread Board - Connecting wires - Digital Multimeter - LEDs - DC Supply |  | Class Room and Workshop |

### @Construct & Verify Combinational Logic Circuit

Objective: This Module covers the knowledge and skills required to Apply Karnaugh mapping & Boolean algebra to simplify logic expressions, Construct & verify the truth table of Half adder, Construct & verify the truth table of Full adder, Construct & verify the truth table of Half Subtraction, Construct & verify the truth table of Full Subtraction, Verify Decoder, Operate seven segment display with seven segment decoder, Verify Encoder and Verify multiplexer and DE- multiplexer.

Duration:  36 Hours			Theory: 09 Hours			Practical: 27 Hours			Credit Hours: 3.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Apply Karnaugh mapping & Boolean algebra to simplify logic expressions | Trainee would be able to - Identify the SOP & POS - Apply Boolean algebra & Karnaugh mapping to simplify SOP & POS. - Construct logic circuits with simplified SOP & POS. | - Describe the laws and rules of Boolean algebra. - Understanding of commutative. • and distributive expiration. That is, A • (B + C) = (A • B) + (A • C) and A + (B • C) = (A + B) • (A + C). - Explain combinational logic circuit. (Half adder, Full adder, Half subtraction, Full subtraction, Binary Multiplier, Magnitude Comparator) - Explain Product-of-Sums & Sum Of Product(SOP) Simplification - Expalin Karnaugh Map of four Variables) and Knowledge of Don't-Care Conditions. Practical Activity: Verify SOP and POS expressions with the help of logic gates | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Different logic gate ICs - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |
| LU2. Construct & verify the truth table of Half adder | Trainee would be able to: - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a half-adder circuit using one XOR gate and two NAND gates. | - Explain half adder circuit. - Describe the procedure of constructing half adder circuit. - Explain the procedure for verification of truth table. Practical Activity: Construct Half adder circuit and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs . | - AND, XOR and NAND gate ICs - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |
| LU3. Construct & verify the truth table of Full adder | Trainee would be able to: - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a full-adder circuit using two ICS 7486 and 7400. | - Explain full adder circuit. - Describe the procedure of constructing full adder circuit. - Explain the procedure for verification of truth table. Practical Activity: Construct full adder circuit and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - AND, XOR and gate ICs - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |
| LU4. Construct & verify the truth table of Half Subtraction | Trainee would be able to - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input | - Explain half subtraction circuit. - Describe the procedure of constructing half subtraction circuit. - Explain the procedure for verification of truth table. Practical Activity: - Construct Half subtractor circuit and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - AND, NOT and XOR gate ICs - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |
| LU5. Construct & verify the truth table of Full Subtraction | Trainee would be able to - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. | - Explain full subtraction circuit. - Describe the procedure of constructing full subtraction circuit. - Explain the procedure for verification of truth table. Practical Activity: Construct full subtractor circuit and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - AND, NOT and XOR gate ICs - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  |
| LU6. Verify Decoder | Trainee would be able to - Place (Decoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. | - Explain Decoder function. - Describe procedure for inserting Decoder IC on breadboard. - Explain procedure for verification of Truth Table. Practical Activity: Construct decoder circuit and verify its truth table | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Decoder IC - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  |
| LU7. Operate seven segment display with seven segment decoder. | Trainee would be able to - Insert (7 segment decoder IC) and 7 segment display on bread board. - Identify the input, output, Vcc and ground pin. - Connect seven segment display with seven segment decoder input output pins. - Record & verify the output result against each given input. | - Explain function of Seven segment Display - Describe the function of Seven Segment decoder IC. - Explain complete procedure for connecting Seven Segment display IC with Seven Segment Decoder. - Explain the procedure for verification of Truth Table. Practical Activity: Construct a circuit with the help of 7 segment display and 7 segment decoder | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Decoder IC - 7 Segment display IC - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  |
| LU8. Verify Encoder | Trainee would be able to - Place (Encoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. | - Explain Encoder function. - Describe procedure for inserting Decoder IC on breadboard. - Explain procedure for verification of Truth Table Practical Activity: Construct a encoder circuit and verify its output | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Encoder IC - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  |
| LU9. Verify multiplexer and DE- multiplexer | Trainee would be able to - Implement following function with multiplexer F(ABC)=∑ (0,2,3,4,5,6): - Implement 4-to-1 mux and one 2-to-1 mux. - Implement 1-to-4 de mux using 1-to-2 de mux. | - Explain Multiplexer and De-Multiplexer circuit in detail. - Describe procedure for implantation of MUX and DEMUX. - Explain procedure for verification of Truth Table Practical Activity: Construct a Multiplexer and DE-Multiplexer circuit and verify its output | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Multiplexer and DE-Multiplexer IC - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  |

### @Construct and Verify Function of Flip Flops

Objective: This Module covers the knowledge and skills required to Construct and verify the truth table of RS latch using NAND gate, Construct and verify the truth table of clocked RS latch using NAND gate and Verify function of D flip flop, Verify function of JK/T flip flop.

Duration:  32 Hours			Theory:  08 Hours			Practical: 24 Hours			Credit Hours: 3

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Construct and verify the truth table of RS latch using NAND gate | Trainee would be able to - Place the NAND gate IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to the outputs pins. - Apply various logic inputs to Record & verify the output result against each given input. | - Define term Latch - Explain SR latch - Explain NAND gate IC and its pin diagram - Explain procedure for verification of truth table Practical Activity: Construct SR latch with NAND gate IC and verify its truth table. | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - SR Latch - NAND gate IC - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |  |
| LU2. Construct and verify the truth table of clocked RS latch using NAND gate | Trainee would be able to: - Place the NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply various logic inputs to Record & verify the output result against each given input | - Define term Latch - Explain clocked SR latch - Explain NAND gate IC and its pin diagram - Explain procedure for verification of truth table Practical Activity: Construct clocked SR latch with NAND gate IC and verify its truth table. | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs.. | - Clocked SR Latch - NAND gate IC - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |  |
| LU3. Verify function of D flip flop. | Trainee would be able to: - Insert the D flip flop IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to the outputs pins. - Apply various logic inputs to Record & verify the output result against each given input. | - Explain the function of Flip Flop IC - Explain the function of D-Flip Flop IC - Explain D-Flip Flop IC and its pin diagram Practical Activity: Design D-Flip Flop and verify its output | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs.. | - D-Flip Flop IC - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  | Class Room and Workshop |
| LU4. Verify function of JK/T flip flop | Trainee would be able to - Insert 74112 (JK flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply various logic inputs to Record & verify the output result against each given input. | - Explain the function of Flip Flop IC - Explain the function of JK&T-Flip Flop IC - Explain JK&T-Flip Flop IC and its pin diagram Practical Activity: - Design JK&T-Flip Flop and verify its output | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - JK-Flip Flop IC - T-Flip Flop IC - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  | Class Room and Workshop |

### @Construct Multi vibrator with 555 IC

Objective: This Module covers the knowledge and skills required to Construct 555 IC as a stable Multi vibrator, Construct 555 IC as Mono-stable Multi vibrator, Construct 555 IC as Bi-stable Multi vibrator and verify its set and reset conduction. Construct 555 IC as A-stable, mono-stable &bi-stable Multi vibrator and observe their outputs.

Duration:  24 Hours			Theory: 06 Hours			Practical: 18 Hours			Credit Hours: 2.4

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Construct 555 IC as Astable Multivibrator | Trainee would be able to - Draw circuit diagram for Actable Multivibrator - Place 555 IC on bread board / trainer - Make connection as per diagram. - Apply voltage to circuit. - Recode the output signal wave shape from oscilloscope. | - Explain 555 timer IC - Define Multivibrator - Explain types of Multivibrator - Explain function of Astable Multivibrator - Describe the procedure of connecting 555 timer IC with oscilloscope Practical Activity: Construct 555 timer IC as Astable multivibrator and verify its output signal on oscilloscope | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - 555 timer IC - Oscilloscope - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |  |
| LU2. Construct 555 IC as Mono-stable Multivibrator | Trainee would be able to: - Draw circuit diagram for Mono-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. | - Explain 555 timer IC - Define Multivibrator - Explain types of Multivibrator - Explain function of Mono-stable Multivibrator - Describe the procedure of connecting 555 timer IC with oscilloscope Practical Activity: Construct 555 timer IC as mono-stable multivibrator and verify its output signal on oscilloscope | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - 555 timer IC - Oscilloscope - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |  |
| LU3.Construct 555 IC as Bi-stable Multivibrator and verify its set and reset conduction | Trainee would be able to: - Draw circuit diagram for Bi-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. | - Explain 555 timer IC - Define Multivibrator - Explain types of Multivibrator - Explain function of Bi-stable Multivibrator - Describe the procedure of connecting 555 timer IC with oscilloscope Practical Activity: Construct 555 timer IC as Bi-stable multivibrator and verify its output signal on oscilloscope | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - 555 timer IC - Oscilloscope - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  | Class Room and Workshop |

### @Construct Shift Registers and Counters with the Help of Flip Flops

Objective: This Module covers the knowledge and skills required to Construct a 4 bit shift register by Using Flip Flops, Construct a 4-bit binary counter Using Flip Flops, Construct 4-bitsynchronous Counter with D flip-Flops and Repair & Troubleshoot  combinational logic circuits.

Duration:  32 Hours			Theory: 08 Hours			Practical: 24 Hours			Credit Hours: 3

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Construct a 4 bit shift register by Using Flip Flops | Trainee would be able to - Draw circuit diagram 4-bit shift register. - Make connection of D-Flip Flop as per diagram to construct 4-bit shift register. - Apply data at the input of register and give clock pulse. - Record the output according to the input. | - Define Register - Explain Register and its types - Explain function of 4 bit shift Register - Explain function of D-Flip Flop Practical Activity: Design 4 bit shift register using D-Flip Flop and verify its output | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs.. | - D-Flip Flop - DLD Trainer Kit - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |
| LU2. Construct a4-bit binary counter Using JK- Flip Flops | Trainee would be able to: - Draw circuit diagram a 4 bit counter. - Make connection of JK-Flip Flop as per diagram to construct 4-bit binary counter. - Connect LEDs to the outputs pins. - Apply the clock pulse and record the output. | - Define Counter - Explain Counter and its types - Explain function of 4 bit Binary counter - Explain function of JK-Flip Flop Practical Activity: Design 4 bit Binary counter using JK-Flip Flop and verify its output. | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs. | - JK-Flip Flop - DLD Trainer Kit - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source | Class Room and Workshop |
| LU3. Construct 4-bit synchronous Counter with JK Flip-Flops | Trainee would be able to: - Draw circuit diagram synchronous counter. - Make connection of JK-Flip Flop as per diagram to construct 4-bit synchronous counter. - Connect LEDs to the output pins. - Apply the clock pulse and record the output. | - Define Counter - Explain synchronous Counter and its types - Explain function of 4 bit synchronous counter - Explain function of JK-Flip Flop Practical Activity: Design 4 bit synchronous counter using JK-Flip Flop and verify its output. | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs.. | - JK-Flip Flop - DLD Trainer Kit - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  |
| LU4. Repair & Troubleshoot combinational logic circuits | Trainee would be able to: - Identify principles and operations of types of logic gates ICs - Check the grounding system in electronic equipment. - Locate external and internal digital IC faults | - Explain Troubleshooting techniques for combination logic circuit - Describe digital IC’s fault and its types - Explain the procedure to locate fault in digital ICs Practical Activity: Diagnose the digital circuit and locate fault. | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs.. | - Different Logic gate ICs - DLD Trainer Kit - Logic tester - Digital Multimeter - Bread Board - LEDs - Connecting leads - DC Power Source |  |

## Sensor & Actuators

### @Test Sensors and Transducers.

Objective: This module covers the knowledge and  skills required to Identify Various types of Sensors, Identify Various types of Transducers, Identify Mode of Various Sensors and Transducers, Operate Oscilloscope for testing sensors and transducers, Test sensor / transducer with Oscilloscope, Test Temperature   Sensor and Test the IR sensor.

Duration:  		28 Hours			Theory: 	07 Hours				Practice: 		21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Various types of Sensors | Trainee must be able to: - Identify temperature sensors. - Identify sound sensors. - Identify proximity sensors. - Identify pressure sensors. - Identify light sensors. - Identify position sensors. - Identify voltage sensors. - Identify current sensors. - Identify the vision sensors. - Identify infrared (IR) sensors. - Identify power requirement for each sensor. Shift down in last | - Describe the Sensor and its types - Describe the Range, Span, error, resolution, linearity of sensor. - Describe Proximity Sensor (Magnetic, Inductive, Capacitive, Optical) | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - log and Digital Oscilloscope - Arduino Development board. - Voltage probe (provided with oscilloscope) / BNC cables. - Jumpers / wires. - LM35 Temperature Sensor. - Sharp IR proximity sensor - Proximity Sensor (Inductive) | Class Room and workshop |
| - Identify Various types of Transducers | Trainee must be able to: - Identify Temperature transducers - Identify Displacement transducers - Identify Motion transducers (Speed, Velocity, Acceleration) - Identify Force/ Torque transducer - Identify Strain transducer - Identify Vibration transducer - Identify Sound transducer - Identify Flow rate transducer - 0714E&A Identify Flow transducer - Identify Level transducer - Identify Pressure transducer - Identify Light transducer - Identify Humidity transducer - Identify Power requirement for each transducer. | - Describe the Transducer and its types. - Describe Tacho-generator Transducer - Describe Photoelectric Speed transducer. - Describe LM35 temperature sensors. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - log and Digital Oscilloscope - Arduino Development board. - Voltage probe (provided with oscilloscope) / BNC cables. - Jumpers / wires. - LM35 Temperature Sensor. - Sharp IR proximity sensor - Proximity Sensor (Inductive) . | Class Room and workshop |
| - Identify Mode of Various Sensors and Transducers | Trainee must be able to: - Identify energy mode (Active / Passive) - Identify Signal mode (Analog /Digital) - Identify contact / non-contact mode - Identify Null / deflection mode | - Describe Photoelectric Speed transducer - Describe LM35 temperature sensors, - Describe PIR Motion detector module - Describe Proximity Sensor (Magnetic, Inductive, Capacitive, Optical) - Describe LVDT transducer | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - log and Digital Oscilloscope - Arduino Development board. - Voltage probe (provided with oscilloscope) / BNC cables. - Jumpers / wires. - LM35 Temperature Sensor. - Sharp IR proximity sensor - Proximity Sensor (Inductive) | Class Room and workshop |
| - Operate Oscilloscope for testing sensors and transducers | Trainee must be able to: - Identify components of oscilloscope, i.e. Display, power supply, etc - Identify Basic controls and functions of oscilloscope - Measure voltage and frequency of signal. - Identify signals in oscilloscope Measure time dependent electrical signals from sensors and transducers | - Differentiate Programs and Algorithms. - Oscilloscope functions and control - Describe LM35 temperature sensors. - Describe contactors, relays and timers | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - log and Digital Oscilloscope - Arduino Development board. - Voltage probe (provided with oscilloscope) / BNC cables. - Jumpers / wires. - LM35 Temperature Sensor. - Sharp IR proximity sensor - Proximity Sensor (Inductive) | Class Room and workshop |
| - Test sensor / transducer with Oscilloscope | Trainee must be able to: - Identify the pins (Power, Signal) of sensor/transducer - Apply required Power to transducer/sensor power pins. - Connect signal pin with oscilloscope. - Write down the obtained data. - Compare the data with transducer/sensor datasheet. | - Differentiate Programs and Algorithms. - Oscilloscope functions and control - Describe LM35 temperature sensors. - Describe contactors, relays and timers. - Describe phototransistor (converts light energy into electric energy) | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - log and Digital Oscilloscope - Arduino Development board. - Voltage probe (provided with oscilloscope) / BNC cables. - Jumpers / wires. - LM35 Temperature Sensor. - Sharp IR proximity sensor - Proximity Sensor (Inductive) | Class Room and workshop |
| - Test Temperature Sensor | Trainee must be able to: - Open Arduino IDE. - Connect the Arduino board to PC using USB cable. - Program the Arduino board with given code - Connect the LM35 sensor to output pin of Arduino - Power ON the oscilloscope - Connect the Channel 1 probe of the oscilloscope to output pin of Arduino - Acquire the signal(s) from circuit on oscilloscope Measure the RMS voltage at different temperatures (25, 26, 27, 28, 29, 30, 31, 32). - Tabulate the Temperature versus measured RMS voltage. - 714E&A Plot to verify linear relationship between Temperature and RMS voltage. | - Describe LM35 temperature sensors, - Describe PIR Motion detector module - Describe the Program languages - Describe the syntax and semantics. - Differentiate Programs and Algorithms. - Oscilloscope functions and control | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - log and Digital Oscilloscope - Arduino Development board. - Voltage probe (provided with oscilloscope) / BNC cables. - Jumpers / wires. - LM35 Temperature Sensor. - Sharp IR proximity sensor - Proximity Sensor (Inductive) | Class Room and workshop |
| - Test the IR sensor | Trainee must be able to: - Open Arduino IDE. - Connect the Arduino board to PC using USB cable. - Program the Arduino board with given code - Connect the IR sensor to output pin of Arduino - Power ON the oscilloscope - Connect the Channel 1 probe of the oscilloscope to output pin of Arduino - Acquire the signal(s) from circuit on oscilloscope - Keep a white body at a distance of 5 cm from the sensor. - Keep moving the object away from the sensor by 1cm and note the MAXIMUM measured voltage - 0714E&A Vary the distance between object and IR sensor (5 to 40 cms with the step of 2 cms) and measure the voltage - Tabulate the distance versus measured MAXIMUM voltage. - Plot the voltage V/S distance graph and check its validation with the datasheet and sensor specification. | - Describe PIR Motion detector module - Describe Proximity Sensor (Magnetic, Inductive, Capacitive, Optical) - Describe the Program languages - Describe the syntax and semantics. - Differentiate Programs and Algorithms. - Oscilloscope functions and control | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - log and Digital Oscilloscope - Arduino Development board. - Voltage probe (provided with oscilloscope) / BNC cables. - Jumpers / wires. - LM35 Temperature Sensor. - Sharp IR proximity sensor - Proximity Sensor (Inductive) | Class Room and workshop |

### @Calibrate Sensors and Transducers.

Objective: This module covers the knowledge and  skills required to Calibrate Capacitive Transducer for Displacement measurement, Calibrate Linear Variable Differential Transformer (LVDT) Transducer for Displacement measurement, Calibrate Resistance Temperature Detector (RTD) for Temperature measurement, Calibrate Thermocouple for Temperature Measurement, Calibrate thermistor for temperature measurements, Calibrate Pressure cell for pressure Measurements, Calibrate strain gauge for temperature measurement, Calibrate Stroboscope for Speed Measurement and Calibrate rotameter for flow measurement

Duration: 32 Hours			Theory: 08 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Calibrate Capacitive Transducer for Displacement measurement | - Trainee will be able to: - Check connection made to the instrument - Allow the instrument in ON position for 10 minutes for initial warm up - Pull the top plate to Zero position - Adjust the ZERO potentiometer so that the display reads ‘000’ - Move the plate in step of 5 to 10 mm and note down the reading in the tabular column till 50mm. | - Describe the calibration of Transducer and Sensor - Describe the Calibration Curves. - Describe Hysteresis. - Describe Errors in Sensor Measurement. - Describe Calibration Process - Describe Error due to Improper Zero Reference. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Oscilloscope - Voltage probe (provided with oscilloscope). - Capacitive transducer. | Class Room and workshop |
| - Calibrate LVDT Transducer for Displacement measurement | Trainee will be able to: - Connect the power supply chord at the rear panel to the 230V 50Hz supply. - Switch on the instrument by pressing down the toggle switch. The display glows to indicate the instrument is ON. - Allow the instrument in ON position for 10 minutes for initial warm-up. - Rotate the micrometer until it shows “20.0”. - Adjust the CAL potentiometer at the front panel so that the display reads “10.0” - Rotate the core of micrometer until the micrometer shows reads “10.0” and adjust the ZERO potentiometer till the display reads “00.0”. - Rotate back the micrometer core upto 20.0 and adjust once again CAL Potentiometer till the display read (Now the instrument is calibrated for +/- 10.0 mm range. As the core of LVDT moves the display reads the displacement in mm.) - Rotate the core of the micrometer in steps of 1 or 2 mm and tabulate the readings. The micrometer will show the exact displacement given to the LVDT core the display will read the displacement sensed by the LVDT. - 14E&A Tabulate the readings and Plot the graph Actual V/s indicator readings | - Calibrate LVDT transducer - Describe Error due to Mechanical Wear or Damage - Describe the Calibration Curves. - Describe Calibration Process. - Describe Error due to Improper Zero Reference | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - LVDT transducer. - Photoelectric Speed transducer. - Oscilloscope - Voltage probe (provided with oscilloscope | Class Room and workshop |
| - Calibrate RTD for Temperature measurement | Trainee must be able to: - Check connection made and Switch ON the instrument by rocker switch at the front panel (The display glows to indicate the instrument is ON.) - Allow the instrument in ON Position for 10 minutes for initial warm-up. - Pore around 3/4th full of water to the kettle and place sensors and thermometer inside the kettle. - Note down the Initial water temperature from the thermometer. - Select the sensor on which the experiment to be conducted through selection switch on the front panel. - Adjust the Initial set Potentiometer in the front panel till the display shows / reads initial water temperature. - Switch the power and wait until the water boils, note down the reading in the thermometer and set Final set potentiometer?? till the display reads boiling water temperature. - Remove the sensor from the boiling water immerse in it I ?? the cold water. Set the cold water temperature using initial set potentiometer. - Repeat the process till the display reads exact boiling water and cold water temperature. - 0714E&A Change the water in the kettle with and re heat the water. Now the display starts showing exact temperature raise in the kettle. - Experiment can be repeated for all the three sensors. - Temperature in the thermometer and the indicator readings in steps of 10o C can??be tabulated | - Describe Hysteresis. - Describe Offset - Describe Gain - Describe Error due to Improper Zero Reference. - Describe Calibration Process - Describe the Calibration Curves. - Describe Hysteresis. - Describe Offset - Describe Gain | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Oscilloscope - Voltage probe (provided with oscilloscope. - RTD instrument. | Class Room and workshop |
| - Calibrate thermistor for temperature measurements | Trainee must be able to - Check connection made and Switch ON the instrument by rocker switch?? At the front panel. - The display glows to indicate the instrument is ON. - Allow the instrument in ON Position for 10 minutes for initial warm-up. - Pore around 3/4th full of water to the kettle and place sensors and thermometer inside the kettle. - Note down the Initial water temperature from the thermometer. - Select the sensor on which the experiment to be conducted through selection switch on the front panel. - Adjust the Initial set Potentiometer in the front panel till the display reads initial water temperature. - Switch on the kept and wait till the water boils note down the reading in the thermometer and set Final set potentiometer till the display reads boiling water temperature. - 0714E&A Remove the sensor from the boiling water immerse it??I the cold water. Repeat the corrections as above - Set the cold-water temperature using initial set potentiometer. - Repeat the process till the display reads exact boiling water and cold-water temperature. - Change the water in the kettle with and re heat the water. Now the display starts showing exact temperature raise in the kettle. Experiment can be repeated for all the three sensors. - Temperature in the thermometer and the indicator readings in steps of 10 C can be tabulated. | - Describe Hysteresis. - Describe Offset - Describe Gain - Describe Error due to Improper Zero Reference. - Describe Calibration Process - Describe the Calibration Curves. - Describe Hysteresis. - Describe Offset - Describe Gain | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Voltage probe (provided with oscilloscope) / BNC cables. - Analog and Digital Oscilloscope. - Jumpers / wires. - Thermistor types enclosed sensor. | Class Room and workshop |
| - Calibrate Pressure cell for pressure Measurements . | Trainee must be able to: - Check connection made and switch ON the instrument by rocker switch at the front panel. The display glows to indicate the instrument is ON. Repeat the corrections as above - Allow the instrument in ON Position for 10 minutes for initial warm-up. - Adjust the Potentiometer in the front panel till the display reads “000” - Apply pressure on the sensor using the loading arrangement provided. - The instrument reads the pressure coming on the sensor and display through LED. - Readings the data. - Tabulate the data and % error of the instrument, linearity can be calculated. . | - Describe Hysteresis. - Describe Offset - Describe Gain - Describe Error due to Improper Zero Reference. - Describe Calibration Process - Describe the Calibration Curves. - Describe Hysteresis. - Describe Offset - Describe Gain. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Voltage probe (provided with oscilloscope) / BNC cables. - Analog and Digital pressure module or Oscilloscope. - Jumpers / wires | Class Room |
| - Calibrate strain gauge for temperature measurement. | Trainee must be able to - Check connection made and Switch ON the instrument by toggle switch at the back of the box. Repeat the corrections as above - The display glows to indicate the instrument is ON. - Allow the instrument in ON Position for 10 minutes for initial warm-up. - Adjust the ZERO Potentiometer on the panel till the display roads ‘OOP’. - Apply load on the sensor using the loading arrangement provided in steps of 100g up to 1 Kg. - The instrument display exact micro-strain strained by the cantilever beam. - Note down the readings in the tabular column. Percentage error in the readings. - Hysteresis and Accuracy of the instrument can be calculated by comparing with the theoretical values | - Describe LM35 temperature sensors, - Describe RTD and Thermocouple. - Describe signal conditioning circuits of RTD and Thermocouple | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - LM35 Temperature Sensor. - Voltage probe (provided with oscilloscope) / BNC cables. - Analog and Digital Oscilloscope. - Jumpers / wires. |  |
| - Calibrate Stroboscope for Speed Measurement | Trainee must be able to - Check the connections of the equipment. Repeat the corrections as above - Select the mode of operation on Xenon flash lamp (Hi/lo). - Make a mark on the rotating wheel. - Now switch on the Xenon flash lamp and adjust that the light exactly projected on the rotating wheel. - Adjust the speed of the motor such that ONE STATIONARY mark is visible on the wheel. Note down the reading in the tabular column. - Now adjust the sped of the motor such that Two Stationary marks are visible and note the readings. - Similarly for Three and more marks and note the readings. |  | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Sharp IR proximity sensor. - Proximity Sensor (Inductive. - Voltage probe (provided with oscilloscope) / BNC cables. - Analog and Digital Oscilloscope. - Jumpers / wires. |  |
| - Calibrate rotameter for flow measurement | Trainee must be able to: - Connect the turbine flow sensor with indicator marked as flow sensor input. Repeat the corrections as above - Connect the two pin of the motor to the instrument. - Vary the flow control potentiometer to any required set level. - Compare the Rotameter reading and digital reading with set reading. - Take reading for different set of flows rate. - Plot the graph of Rotameter Reading with Digital Indicator Reading. | - Describe the calibration of rota meter and Sensor - Describe the Calibration Curves. - Describe Hysteresis. - Describe Errors in Sensor Measurement. - Describe Calibration Process - Describe Error due to Improper Zero Reference | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Micrometer. - Digital or analog flow meter. - Voltage probe (provided with oscilloscope) / BNC cables. - Analog and Digital Oscilloscope. - Jumpers / |  |

### @Interface Sensors and Transducers.

Objective: This module covers the knowledge and  skills required to Interface LVDT with microcontroller for sensing linear displacement, Interface temperature transducer with microcontroller for sensing temperature, Interface rotary flow transducer with microcontroller for sensing flow rate, interface Proximity sensor to sense the speed of DC Motor and Interface MQ135 Gas Sensor with Arduino

Duration: 25 Hours			Theory: 10 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Interface LVDT with microcontroller for sensing linear displacement | Trainee must be able to: - Open Arduino IDE. - Connect the Arduino board to PC using USB cable. - Program the Arduino board with given code - Connect the LVDT to Arduino - Acquire the data (voltages) form LVDT due to change in resistance. - Convert the data (voltages) to linear displacement by equation | - Describe the Range, Span, error, resolution, linearity of sensor - Describe PIR Motion detector module - Describe the Program languages - Describe the syntax and semantics. - Differentiate Programs and Algorithms. | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Interfacing Communication cable. - Microcontroller. - Sensor with linear displacement. | Class Room and workshop |
| - Interface temperature transducer with microcontroller for sensing temperature. | Trainee must be able to: - Open Arduino IDE. - Connect the Arduino board to PC using USB cable. - Program the Arduino board with given code - Connect the temperature transducer (LM35) to Arduino - Acquire the data (voltages) form LM35 due to change in resistance. - Convert the data (voltages) to temperature by equation | - Describe the Transducer - Describe RTD and Thermocouple - Describe signal conditioning circuits of RTD and Thermocouple - Describe LVDT transducer - Describe the Program languages - Describe the syntax and semantics. - Differentiate Programs and Algorithms | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Temperature transducer - Microcontroller with communication cable. - Sensor with module. | Class Room and workshop |
| - Interface rotary flow transducer with microcontroller for sensing flow rate | Trainee must be able to: - Open Arduino IDE. - Connect the Arduino board to PC using USB cable. - Program the Arduino board with given code - Connect the rotary flow transducer to Arduino - Acquire the data (rpm) form rotary flow transducer due to flow. - Convert the data (rpm) to linear displacement by equation. | - Describe the Transducer - Describe signal conditioning circuits of RTD and Thermocouple - Describe LVDT transducer - Describe Photoelectric Speed transducer - Describe the Program languages - Describe the syntax and semantics. - Differentiate Programs and Algorithms | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Analog or digital flow meter. - Microcontroller with communication cable. - Sensor with module. | Class Room and workshop |
| - Interface Proximity sensor to sense the speed of DC Motor | Trainee must be able to: - Open Arduino IDE. - Connect the Arduino board to PC using USB cable. - Program the Arduino board with given code - Connect the DC Motor to Arduino - Attach a rotating (hollow) wheel to DC Motor shaft. - Connect the inductive proximity sensor to Arduino - Attach proximity sensor near rotating wheel of DC motor - Acquire the data form sensor | - Describe the Sensor - Describe the Range, Span, error, resolution, linearity of sensor - Describe contactors, relays and timers. - Describe Proximity Sensor (Magnetic, Inductive, Capacitive, Optical) - Describe the Program languages - Describe the syntax and semantics. - Differentiate Programs and Algorithms | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - DC motor. - Proximity sensor. - Sensor with module. - Microcontroller with communication cable. | Class Room and workshop |
| - Interface MQ135 Gas Sensor with Arduino | Trainee must be able to: - Identify pin configurations - Open Arduino IDE software. - Connect MQ135 gas sensor to desired Pin. - Make a source code / sketch to sense the gas. - Compile and Debug - Run Program. - Acquire the data form sensor - Assign a task on sensor data. . | - Describe the Sensor - Describe the Range, Span, error, resolution, linearity of sensor - Describe contactors, relays and timers. - Describe Proximity Sensor (Magnetic, Inductive, Capacitive, Optical) - Describe the Program languages - Describe the syntax and semantics. - Differentiate Programs and Algorithms | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Microcontroller with communication cable. - Gas flow meter with sensor. - Sensor module | Class Room. workshop |

### @Test Actuators

Objective: This module covers the knowledge and  skills required to Identify various types of Actuators, Leak test for Pneumatic actuators/tubes, Leak test Hydraulic actuators/tubes, Force/Power test and Piezoelectric actuator test

Duration:  25	Hours			Theory: 10 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify various types of Actuators | Trainee must be able to: - Identify Mechanical Actuators (Spring, Gear) - Identify Electrical Actuators - Identify Hydraulic Actuators - Identify Pneumatic Actuators. - Identify Electromechanical Actuators - . Identify Electromagnetic Actuators - Identify Manual Actuators - Identify power sources of actuators | - Describe the Actuators types. - Describe the Pneumatic actuators - Describe the Hydraulic actuators - Describe solenoids as actuator - Describe the Pneumatic actuators components. - Describe the Hydraulic actuators components | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Air Compressor - Pneumatic Actuators. - Pneumatic valves and accessories. - Pressure gauges - Hydraulic pump - Hydraulic Actuators. . | Class Room and workshop |
| - Leak test for Pneumatic actuators/tubes | Trainee must be able to: - Select test type. (Bubble leak test, Chemical Reactivity test, Pressure drop) - Apply high pressure in actuator/tubes for bubble leak test. - Make soap water - Apply soap water on joints and seals. - Mark the places of bubbles. - Make a report and suggest maintenance or repairing. | - Describe the Actuators types. - Describe the Pneumatic actuators. - Describe the Pneumatic actuators components. | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Air Compressor - Pneumatic Actuators. - Pneumatic valves and accessories. - Pressure gauges . | Class Room and workshop |
| Leak test Hydraulic actuators/tubes | Trainee must be able to: - Select test type. (Bubble leak test, Chemical Reactivity test, Pressure drop) - Attach pressure gauges to actuator for pressure drop test. - Apply high pressure in actuator/tubes and seal it. - Note down the pressure and time. - Note down the pressure after each 5 minutes. - Make a report and suggest maintenance or repairing. | - Describe the Actuators types. - Describe the Hydraulic actuators - Describe the types of leak tests (Bubble leak test, Chemical Reactivity test, Pressure drop). - Describe the Hydraulic actuators components | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Air Compressor. - Pressure gauges - Hydraulic pump - Hydraulic Actuators. - Hydraulic valves and accessories. | Class Room and workshop |
| - Force/Power test | Trainee must be able to: - Select actuator type. (Pneumatic, Hydraulic) - Attach a load cell with actuator as given. - Apply controlled pressure in actuator. - Note down the reading on load cell. - Increase the pressure step by step of 5 psi and take data. - Tabulate the data. | - Describe contactors, relays and timers. - Describe the Pneumatic actuators components. | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Air Compressor - Pneumatic Actuators. - Pneumatic valves and accessories. - Pressure gauges - Hydraulic pump - Hydraulic Actuators. - Hydraulic valves and accessories. | Class Room and workshop |
| - Piezoelectric actuator test | Trainee must be able to: - Select a piezoelectric actuator. - Identify power range for piezoelectric actuator. - Select displacement sensor. - Install displacement sensor. - Apply the voltage of step 0.1 volts from 0 to maximum range. - Take the results from displacement sensor at step of 0.1 volts - Tabulate the data . | - Describe solenoids as actuator - Describe piezoelectric actuators. - Describe contactors, relays and timers. | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Air Compressor. - Pressure gauges - Piezoelectric actuators - Load cell | Class Room. workshop |

### @Interface and control Actuator with sensors.

Objective: This module covers the knowledge and  skills required to Identify motion of Actuators, Control extension length of pneumatic actuator, Control the room temperature with relay (actuator) and Control angle of Servomotor with potentiometer.

Duration:  20	Hours			Theory: 08 Hours				Practice: 12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify motion of Actuators | Trainee must be able to: - Identify Linear Actuators - Identify Rotary Actuators - Identify Binary Actuators - Identify Continues Actuators. | - Describe the types of actuator motion. - Describe potentiometer as manual rotary actuator. - Describe relay as binary actuator. - Describe pneumatic actuator as linear actuation. | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Pneumatic actuator. - Binary actuators. - Rotatory actuator. - Linear actuators | Class Room and workshop |
| - Control extension length of pneumatic actuator | Trainee must be able to: - Select pneumatic actuator - Select displacement sensor - Select a solenoid valve with relay to actuate pneumatic cylinder - Select controller (Arduino) - Make a wiring diagram for connection of sensor, relay and controller. - Open Arduino IDE. - Connect the Arduino board to PC using USB cable. - Program the Arduino board to ON/OFF the relay for minimum and maximum extension of actuating rod. - 0714E&A Perform reciprocating motion of rod. | - Describe the Control of system. - Describe potentiometer as manual rotary actuator. - Describe relay as binary actuator. - Describe pneumatic actuator as linear actuator. | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Pneumatic actuators. - Solenoid valve - Relays - Air compressor. - Microcontroller - Controller with modules - Wiring cables. - Pneumatic pipes . | Class Room and workshop |
| - Control the room temperature with relay (actuator) | Trainee must be able to: - Select temperature sensor - Select heater or air conditioner - Select a relay (actuator) for operating heater or air conditioner - Select controller (Arduino). - Make a wiring diagram for connection of heater or air conditioner, sensor, relay and controller. - Open Arduino IDE. - Connect the Arduino board to PC using USB cable. - Program the Arduino board to ON/OFF the relay to maintain temperature. | - Describe the types of actuator motion. - Describe the Pressure force relationship - Describe the Control of system - Describe open loop systems - Describe close loop systems. - Describe Servo motor as rotary actuator. | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Air Compressor. - Pressure gauges. - Relays - Microcontroller - Sensors. - Temperature related condition relays with controller. | Class Room and workshop |
| - Control angle of Servomotor with potentiometer | Trainee must be able to: - Identify potentiometer (manual actuator) to detect the resistance/angle. - Identify servomotor (rotary actuator) to position the system. - Identify the Input and Output pins. - Identify the Digital, Analog and PMW pins. - Make a layout and connect components - Write, compile, debug and upload a program to microcontroller. - RUN and Get results (Angle depends on Potentiometer). | - Describe the types of actuator motion. - Describe the Pressure force relationship - Describe the Control of system - Describe open loop systems - Describe close loop systems. - Describe relay as binary actuator. - Describe Servo motor as rotary actuator | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | - Air Compressor - Pneumatic Actuators. - Pneumatic valves and accessories. - Pressure gauges. - Servo motor with potentiometer.. - PMW module. - Microcontroller | Class Room and workshop |

## Module: Develop workplace policy and procedures for sustainability.

Objective: After the completion of this competency standard, the Trainee will be able to develop and implement a workplace
sustainability policy and to modify the policy to suit changed circumstances. It applies to individuals with managerial responsibilities who undertake work developing approaches to create, monitor and improve strategies and policies within workplaces and engage with a range of relevant stakeholders and specialists.

Duration:  19 Hours			Theory:  1 Hours			Practice:  18 Hours          	        Credit Hours:  0.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop workplace sustainability policy | Trainee will be able to: - Define scope of sustainability policy - Gather information from a range of sources to plan and develop policy - Identify and consult stakeholders as a key component of the policy development process - Include appropriate strategies in policy at all stages of work for minimizing resource use, reducing toxic material and hazardous chemical use and employing life cycle management approaches - Make recommendations for policy options based on likely effectiveness, timeframes and cost - Develop policy that reflects the organization s commitment to sustainability as an integral part of business planning and as a business opportunity - Agree to appropriate methods of implementation, outcomes and performance indicators | - Outline the environmental or sustainability legislation, regulations and codes of practice applicable to the organization identify internal and external sources of information and - Explanation of plan and development of the organization sustainability policy | Theory- 1 Hrs Practical-03 Hrs Total- 4 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Communicate workplace sustainability policy | Trainee will be able to: - Promote workplace sustainability policy, including its expected outcome, to key stakeholders. - Inform those involved in implementing the policy about expected outcomes, activities to be undertaken and assigned responsibilities. | - Outline organizational systems and procedures that relate to sustainability | Theory- 0Hrs Practical-06 Hrs Total- 6 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Implement workplace sustainability policy | Trainee will be able to: - Develop and communicate procedures to help implement workplace sustainability policy - Implement strategies for continuous improvement in resource efficiency - Establish and assign responsibility for recording systems to track continuous improvements in sustainability approaches | - Outline typical barriers to implementing policies and procedures in an organization and possible strategies to address them. | Theory- 1 Hrs Practical-03 Hrs Total- 3 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU4. Review workplace sustainability policy implementation | Trainee will be able to: - Review workplace sustainability policy implementation. - Investigate successes or otherwise of policy. - Monitor records to identify trends that may require remedial action and use to promote continuous improvement of performance. - Modify policy and or procedures as required to ensure improvements are made | - Explain workplace policy development processes and practices | Theory- 0 Hrs Practical-6 Hrs Total- 6 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Manage meetings with customers

Objective: After the completion of this competency standard, the Trainee will be able to manage a range of meetings including
overseeing the meeting preparation processes, chairing meetings, organizing the minutes and reporting meeting outcomes. It applies to individuals employed in a range of work environments who are required to organize and manage meetings within their workplace, including conducting or managing administrative tasks in providing agendas and meeting material. They may work as senior administrative staff or may be individuals with responsibility for conducting and chairing meetings in the workplace

Duration:  19 Hours			Theory:  1 Hours			Practice:  18 Hours          	        Credit Hours:  0.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Plan and Prepare for meetings | Trainee will be able to: - Develop agenda in line with stated meeting purpose - Ensure style and structure of meeting are appropriate to its purpose - Identify meeting participants and notify them in accordance with organizational procedures - Confirm meeting arrangements in accordance with requirements of meeting - Dispatch meeting papers to participants within designated timelines | - Outline responsibilities of the chairperson and explain group dynamics in relation to managing meetings | Theory- 1 Hrs Practical- 6Hrs Total- 7 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Conduct meetings | Trainee will be able to: - Chair meetings in accordance with organizational requirements, agreed conventions for type of meeting and legal and ethical requirements - Conduct meetings to ensure they are focused, time efficient and achieve the required outcomes - Ensure meeting facilitation enables participation, discussion, problem solving and resolution of issues - Brief minute-taker on method for recording meeting notes in accordance with organizational requirements and conventions for type of meeting | - Describe options for meetings including face-to-face, tele-conferencing, web-conferencing and using webcams | Theory- 0 Hrs Practical- 6Hrs Total- 6 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Follow up meeting’s agenda for implementation | Trainee will be able to: - Check transcribed meeting notes to ensure they reflect a true and accurate record of the meeting and are formatted in accordance with organizational procedures and meeting conventions - Distribute and store minutes and other follow-up documentation within designated timelines, and according to organizational requirements - Report outcomes of meetings as required, within designated timelines | - Devise meeting terminology, structures, arrangements as per organizational procedures | Theory- Hrs Practical-6 Hrs Total- 6 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Manage and select your team (workforce) according job requirement

Objective: After the completion of this competency standard, the Trainee will be able to manage all aspects of recruitment
selection and induction processes in accordance with organizational policies and procedures. It applies to individuals or human resource personnel who take responsibility for managing aspects of selecting new staff and orientating those staff in their new positions. It is not assumed that the individual will be directly involved in the selection processes themselves, although this may well be the case.

Duration:  19 Hours			Theory:  1 Hours			Practice:  18 Hours          	        Credit Hours:  0.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop team selection policies and procedures | Trainee will be able to: - Analyze strategic and operational plans and policies to identify relevant policies and objectives - Develop recruitment, selection and induction policies and procedures and supporting documents - Review options for technology to improve efficiency and effectiveness of recruitment and selection process - Obtain support for policies and procedures from senior managers - Trial forms and documents supporting policies and procedures and make necessary adjustments - Communicate policies and procedures to relevant staff and provide training if required | - Describe recruitment and selection methods, including assessment centers - Describe the purpose of employee contracts and industrial relations - Summarize relevant legislation, regulations, standards and codes of practice that may affect recruitment, selection and induction | Theory- 1 Hrs Practical- 06Hrs Total- 07 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Select team according to job | Trainee will be able to: - Determine future human resource needs in collaboration with relevant managers and sections - Ensure current position descriptors and person specifications for vacancies are used by managers and others involved in recruitment, selection and induction processes - Provide access to training and other forms of support to all persons involved in recruitment and selection process - Ensure advertising of vacant positions complies with organizational policy and legal requirements - Utilize specialists where necessary - Ensure selection procedures are in accordance with organizational policy and legal requirements - Ensure processes for advising applicants of selection outcome are followed - Ensure job offers and contracts of employment are executed promptly, and new appointments are provided with advice about salary, terms and conditions | - Explain the concept of outsourcing - Explain why terms and conditions of employment are an important aspect of recruitment | Theory- Hrs Practical-06 Hrs Total- 6 Hr | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Manage staff induction | Trainee will be able to: - Provide access to training and ongoing support for all persons engaged in staff induction - Check induction processes are followed across the organization - Oversee management of probationary employees and provide them with feedback until their employment is confirmed or terminated - Obtain feedback from participants and relevant managers on extent induction process is meeting its objectives - Make refinements to induction policies and procedures | Explain the relevance of psychometric and skills testing programs to recruitment. | Theory- Hrs Practical-06 Hrs Total- 6 Hr | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Manage workforce planning

Objective: After the completion of this competency standard, the Trainee will be able to manage planning in relation to an
organization’s workforce including researching requirements, developing objectives and strategies, implementing initiatives and monitoring and evaluating trends. It applies to individuals who are human resource managers or staff members with a role in a policy or planning unit that focuses on workforce planning.

Duration:  19 Hours			Theory:  1 Hours			Practice:  18 Hours          	        Credit Hours:  0.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Research workforce requirements | Trainee will be able to: - Review current data on staff turnover and demographics - Assess factors that may affect workforce supply - Establish the organization’s requirements for a skilled and diverse workforce | Outline industrial relations relevant to the specific industry | Theory- 1 Hrs Practical- 06Hrs Total- 7 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Develop workforce objectives and strategies | Trainee will be able to: - Review organizational strategy and establish aligned objectives for modification or retention of the workforce - Consider strategies to address unacceptable staff turnover, if required - Define objectives to retain required skilled labor - Define objectives for workforce diversity and cross-cultural management - Define strategies to source skilled labor - Communicate objectives and rationale to relevant stakeholders - Obtain agreement and endorsement for objectives and establish targets - Develop contingency plans to cope with extreme situations | Explain current information about external labor supply relevant to the specific industry or skill requirements of the organization | Theory- Hrs Practical-06 Hrs Total- 6 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Implement initiatives to support workforce planning objectives | Trainee will be able to: - Implement action to support agreed objectives for recruitment, training, redeployment and redundancy - Develop and implement strategies to assist workforce to deal with organizational change - Develop and implement strategies to assist in meeting the organization’s workforce diversity goals - Implement succession planning system to ensure desirable workers are developed and retained - Implement programs to ensure workplace is an employer of choice | - Strategies to assist workforce to deal with organizational change & work environment | Theory- 0Hrs Practical- 3Hrs Total- 3 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU4. Monitor and evaluate workforce trends | Trainee will be able to: - Review workforce plan against patterns in exiting employee and workforce changes - Monitor labor supply trends for areas of over- or under-supply in the external environment - Monitor effects of labor trends on demand for labor - Survey organizational climate to gauge worker satisfaction - Refine objectives and strategies in response to internal and external changes and make recommendations in response to global trends and incidents - Regularly review government policy on labor demand and supply - Evaluate effectiveness of change processes against agreed objectives | Describe labor force analysis and forecasting techniques | Theory- 0Hrs Practical- 3Hrs Total- 3 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Undertake project work

Objective: After the completion of this competency standard, the Trainee will be able to undertake a straightforward project or
a section of a larger project. It covers developing a project plan, administering and monitoring the project, finalizing the project and reviewing the project to identify lessons learned for application to future projects. This unit applies to individuals who play a significant role in ensuring a project meets timelines, quality standards, budgetary limits and other requirements set for the project.

Duration:  19 Hours			Theory:  1 Hours			Practice:  18 Hours          	        Credit Hours:  1.9

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Define project | Trainee will be able to: - Access project scope and other relevant documentation - Define project stakeholders - Seek clarification from delegating authority of issues related to project and project parameters - Identify limits of own responsibility and reporting requirements - Clarify relationship of project to other projects and to the organization’s objectives - Determine and access available resources to undertake project | Give examples of project management tools and how they contribute to a project Explain the organization’s procedures and processes that are relevant to managing a project including: lines of authority and approvals quality assurance human resources budgets and finance recordkeeping reporting | Theory- 1 Hrs Practical- 06Hrs Total- 7 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Develop project plan | Trainee will be able to: - Develop project plan in line with the project parameters - Identify and access appropriate project management tools - Formulate risk management plan for project, including Work Health and Safety (WHS) - Develop and approve project budget - Consult team members and take their views into account in planning the project - Finalize project plan and gain necessary approvals to commence project according to documented plan | - Outline types of documents and other sources of information commonly used in defining the parameters of a project | Theory- Hrs Practical- 06Hrs Total- 6Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Administer and monitor project | Trainee will be able to: - Take action to ensure project team members are clear about their responsibilities and the project requirements - Provide support for project team members, especially with regard to specific needs, to ensure that the quality of the expected outcomes of the project and document time lines are met - Establish and maintain required recordkeeping systems throughout the project - Implement and monitor plans for managing project finances, resources and quality - Complete and forward project reports as required to stakeholders - Undertake risk management as required to ensure project outcomes are met - Achieve project deliverables | - Outline the organization’s mission, goals, objectives and operations and how the project relates to them | Theory- Hrs Practical- 03Hrs Total- 03 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU4. Finalize project | Trainee will be able to: - Complete financial recordkeeping associated with project and check for accuracy - Ensure transition of staff involved in project to new roles or reassignment to previous roles - Complete project documentation and obtain necessary sign-offs for concluding project | - Explain processes for identifying and managing risk in a project | Theory- Hrs Practical- 03Hrs Total- 03 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Prepare and implement negotiation

Objective: After the completion of this competency standard, the Trainee will be able to prepare for and participate in a process of negotiation, Coordinate support services, Restore order, Provide leadership direction and guidance to the work group.

Duration:  19 Hours			Theory:  1 Hours			Practice:  18 Hours          	        Credit Hours:  1.9

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Prepare for the negotiation | Trainee will be able to: - Identify objectives and preferred outcome of the negotiation and determine minimum acceptable outcome - Understand in relation to what can be offered and what is needed from the other party - Gather information regarding the other party – objectives, needs, preferences, resources, what they want to achieve – in order to determine best negotiating points - List and rank the issues to consider concessions that may be made. - Find examples and refine negotiation argument - Check information to ensure it is correct and up-to-date. - Develop a negotiation plan that includes information about the other party and its interests and a set of responses and strategies to the anticipated tactics. - Prepare an agenda in advance, which includes discussion topics, participants, location and schedule | - Explain organization’s policies, guidelines and procedures related to control and surveillance, safety and preventing and responding to incidents and breaches of orders covered in the range of variables. | Theory- 0 Hrs Practical- 03Hrs Total- 03 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Participate in negotiations | Trainee will be able to: - Analyze all aspects of the incident for degree of hazard, priorities, optional outcomes and appropriate strategies - Analyze and determine strategies and priorities on the incident sought from a range of sources - Assess long term objectives against resources and priorities - Apply a range of communication techniques to make and maintain contact with the key people - Provide clear and factual information to enable an honest and realistic assessment of the interests of the key people and their positions - Resolve the conflict and express their likely consequences clearly and do an analysis of the benefits - Reassess points of disagreements for common positive positions | - Outline the principles of effective communication | Theory- Hrs Practical- 03Hrs Total- 03 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Coordinate support services | Trainee will be able to: - Assess the need for support services in terms of the determined strategies and priorities - Negotiate the resources of support services according to established procedures and availability - Provide information on strategies to support services and maintain the communication - Delegate roles and responsibilities according to expertise and resources | - Explain organization’s management and accountability systems | Theory- Hrs Practical- 03Hrs Total- 03 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU4. Restore order | Trainee will be able to: - Assess the incidents for degree of risk and take appropriate action to reduce and remove the impact of the incident and restore order - Take action designed to minimize risk and the preserve the safety and security of all involved - Take action to prevent the escalation of the incident appropriate to the circumstances and agreed procedures. - Carry out the use of force for the restoration of control and the maintenance of security in the least restrictive manner. - Complete reports accurately and clearly provided to the appropriate authority promptly - Review, evaluate and analyze the incident and the organizational response to it and report it promptly and accurately. | - Describe teamwork principles and strategies | Theory- Hrs Practical- 03Hrs Total- 03 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU5. Provide leadership direction and guidance to the work group | Trainee will be able to: - Link between the function of the group and the goals of the organization - Participate in decision making routinely to develop, implement and review work of the group and to allocate responsibilities where appropriate - Give opportunities and encouragement to others to develop new and innovative work practices and strategies - Identify conflict and resolve with minimum disruption to work group function - Provide staff with the support and supervision necessary to perform work safely and without risk to health - Allocate tasks within the competence of staff and support with appropriate authority, autonomy and training - Supervise appropriately the changing priorities and situations and takes into account the different needs of individuals and the requirements of the task | - Outline the guidelines for use of equipment and technology - Explain code of conduct | Theory- 01Hrs Practical- 06Hrs Total- 07 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Maintain professionalism in the workplace

Objective: After the completion of this competency standard, the Trainee will be able to maintain a professional image in the
workplace, including behaving ethically, demonstrating motivation, respecting timeframes and maintaining personal appearance.

Duration:  19 Hours			Theory:  1 Hours			Practice:  18 Hours          	        Credit Hours:  1.9

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Respect work timeframes | Trainee will be able to: - Demonstrate punctuality in meeting, set working hours and times. - Utilize working hours only for working and follow company regulations. - Complete work tasks within deadlines according to order of priority - Supervisors are informed of any potential delays in work times or projects | - Explain application of good manners and right conduct - Outline the company code of conduct/values | Theory- 1 Hrs Practical- 03Hrs Total- 04 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Maintain personal appearance and hygiene | Trainee will be able to: - Clean hair, body and nails regularly. - Wear suitable cloths for the workplace, and respect local and cultural contexts - Meet specific company dress code requirements | - Describe common products used for oral and personal hygiene | Theory- Hrs Practical- 3Hrs Total- 3 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Maintain adequate distance with colleagues and clients | Trainee will be able to: - Respect personal space of colleagues and clients with reference to local customs and cultural contexts. - Keep sufficient distance from others - Avoid cross transmission of infections (especially through respiration) | - State workplace hygiene standards - Explain basic practices for oral and personal hygiene | Theory- Hrs Practical- 3Hrs Total- 3 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Mulmediaprojector | Class Room |
| LU4. Work in an ethical manner | Trainee will be able to: - Follow company values/ethics codes of ethics and/or conduct, policies and guidelines. - Use company resources in accordance with company ethical standards. - Conduct personal behavior and relationships in accord with ethical standards and company policies. - Undertake work practices in compliance with company ethical standards, organizational policy and guidelines. - Instruct co-workers on ethical, lawful and reasonable directives. - Share company values /practices with co-workers using appropriate behavior and language. - Report work incidents /situations and/or resolved in accordance with company protocol /guidelines. | - Outline the Company regulations, performance and ethical standards - Explain work responsibilities/job functions - Describe communication skills | Theory- Hrs Practical- 9Hrs Total- 9 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

# 4.	List of Personal Protective Equipment

(FOR A CLASS OF 25 STUDENTS)

| Name of Trade |  | Mobile Phone Technician |  |
| --- | --- | --- | --- |
| Duration of Course |  | 12 Months |  |
| Sr. # |  | Description |  | Quantity |
| 1. |  | Safety gloves, |  | 30 |
| 2. |  | Appropriate safety glasses, |  | 30 |
| 3. |  | Breathing apparatus, |  | 30 |
| 4. |  | Fire extinguishers, |  | 30 |
| 5. |  | Fire blankets, |  | 30 |
| 6. |  | Respirators, masks, |  | 30 |
| 7. |  | Fire hoses, |  | 04 |
| 8. |  | First aid kits, stretchers and other related tools and gear |  | 10 |

# 5. Complete List of Tools, Equipment, Machines and Consumables

Tools

| S# | Description | Quantity |
| --- | --- | --- |
|  | Precision screw driver | 25 |
|  | Screw driver kit | 25 |
|  | Tweezers | 25 |
|  | Brushes | 25 |
|  | Blade cutter | 25 |
|  | Nose cutter | 25 |
|  | Point cutter | 25 |
|  | Cutter plier | 25 |
|  | Long Nose plier | 25 |
|  | PCB holder | 25 |
|  | Hard tweezers | 25 |
|  | Magnifying glass | 25 |
|  | Opener’s kit | 25 |
|  | Suction openers | 25 |
|  | Magnifying lamp | 25 |
|  | Wrist wire | 25 |
|  | Clipping tools(clips) | 25 |
|  | seizer | 25 |
|  | Steel wire | 25 |
|  | Rubber pads | 25 |
|  | Glass openers | 25 |

Equipment for Hardware

| S# | Description | Quantity |
| --- | --- | --- |
|  | Digital Multi meter | 25 |
|  | Oscilloscope | 5 |
|  | Soldering Iron | 25 |
|  | Soldering Station/SMD work station | 25 |
|  | Digital variable Power supply | 5 |
|  | B. G. A. Kit | 25 |
|  | Ultrasonic PCB Cleaner | 5 |

Equipment for Display repair

| S# | Description | Quantity |
| --- | --- | --- |
|  | Hot gun | 25 |
|  | Smoke Absorber | 5 |
|  | Hot air blower | 5 |
|  | Hot plate display separator | 5 |
|  | Freezer for separation of OLED | 5 |
|  | Flex binding machine | 5 |
|  | Mini electric grinder kit (EMC) | 5 |
|  | UV lamp | 5 |
|  | Glass laminating machine | 5 |
|  | De-bubbler machine | 5 |
|  | PCB Owen | 5 |

Equipment for Software

| S# | Description | Quantity |
| --- | --- | --- |
|  | Octopus Box (with cable Samsung + LG) | 5 |
|  | J Tag | 5 |
|  | EFT Dongle | 5 |
|  | CM2 Dongle | 5 |
|  | Sigma Key | 5 |
|  | ATF Dongle | 5 |
|  | ZXW (Hardware) Schematic diagram Dongle | 5 |

Consumables

| S# | Description | Quantity |
| --- | --- | --- |
|  | Soldering wire | 25 roll |
|  | Gloves | 25 Nos |
|  | Jumper wire |  |
|  | IC Paste |  |
|  | Solder paste |  |
|  | Cleaning sponge | 25 |
|  | Paste flux |  |
|  | Cleaning cloth |  |
|  | Chemical for washing mobile phone |  |
|  | UV Gum |  |
|  | Double tape |  |
|  | Heat resistance tape |  |
|  | Adhesive Glue |  |
|  | Thinner |  |
|  | CTC |  |
|  | Shiner |  |
|  | Rubber adhesive glue |  |
|  | WD-40 |  |
|  | Thinner |  |
|  | Cotton |  |

List of Stationary

| S# | Description | Quantity |
| --- | --- | --- |
|  | Handbooks | 5 |
|  | Design books | 5 |
|  | Pencils | 30 |
|  | Rubber | 30 |
|  | Sharpeners | 30 |
|  | Paper Cutter | 5 |
|  | Seizers | 5 |
|  | Colors | 5 each color (Read, Green, Yellow, Blue, white and Black) |
|  | White charts | 50 Nos |
|  | Brown sheets | 100 Nos |
|  | White board markers | 5 Doz |
|  | Permanent markers | 2 Dozen |
|  | File cover and files | 50 Nos. |

Learning hours in training provider premises,

Training workshop, laboratory and on-the-job workplace