National Certificate Level

- 05 in Mechatronics Technology

(Curriculum)

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1. 	Introduction	8

1.1.	Purpose of the training program:	9

1.2.	Overall objectives of training program	9

1.3.	Competencies to be gained after completion of course	10

1.4.	Entry level of trainees	20

1.5.	Minimum qualification for teachers	20

1.6.	Medium of instruction	20

1.7.	Duration of the course:	21

1.8.	Description and structure of the course	22

2.	Overview of the Curriculum for Mechatronics Technology	30

3.	Detail of Modules.	32

3.1. 	Maintain Occupational Health and Safety:	32

3.1.1.	Maintain Health and Safety	32

3.1.2.	Perform safety practices at workplace	37

3.1.3.	Develop professionalism.	41

3.2.	Basic Machining	46

3.2.1.	Perform Metal / Bench Work	46

3.2.2.	Perform general machining	51

3.2.3.	Perform cutting on Metal Circular/Power Heck Saw	54

3.2.4.	Perform tool sharpening	56

3.2.5.	Perform Basic Lathe Machine Operations	59

3.2.6.	Perform Drilling Machine Operations	67

3.2.7.	Perform Shaper, Planar and Slotter Machining Operations	70

3.2.8.	Perform Milling Operations	75

3.3.	Electrical Essentials & Networks	87

3.3.1.	Operate Measuring Instruments.	87

3.3.2.	Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.	90

3.3.3.	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence	93

3.3.4.	Construct Electromagnet to See Various Effects & Verify Faradays Law	96

3.3.5.	Verify Series and Parallel Combination of Capacitors & Determine Break down Voltage of Capacitor	100

3.3.6.	Maintain Tools & Equipment.	102

3.3.7.	Make Cable/Wire Joints	104

3.3.8.	Prepare and Install Distribution Boards	107

3.3.9.	Carryout Basic Electrical Installation	110

3.3.10.	Install Single Phase Electrical Wiring	113

3.3.11.	Perform Testing of Electrical Wiring	118

3.3.12.	Operate Single Phase Transformer	123

3.3.13.	Perform Tests on Single Phase Transformer	126

3.4.	Metrology	129

3.4.1.	Take measurements with graduated tools	129

3.4.2.	Take measurements with combination set	132

3.4.3.	Perform Leveling.	135

3.4.4.	Take measurements with gauges	137

3.4.5.	Perform measurements through Micrometer	139

3.4.6.	Measure dimensions with Vernier tools	142

3.4.7.	Measure angles with angle measuring Instruments.	144

3.4.8.	Measure Threads and Gears	147

3.5.	Basic Engineering Drawing	150

3.5.1.	Perform Basic Manual Drawing.	150

3.5.2.	Construct different Engineering Curves	159

3.5.3.	Construct Multi-View Drawings.	171

3.5.4.	Draw basic drawing using CAD.	180

3.5.5.	Develop 2D using CAD software.	182

3.5.6.	Develop 3D drawing using CAD software.	184

3.6.	Welding	187

3.6.1.	Interpret Part and Assembly Drawing	187

3.6.2.	Evaluate the Use of Hand and Power Tools	190

3.6.3.	Perform Pre-Welding Operations	193

3.6.4.	Perform Oxy Acetylene Welding	196

3.6.5.	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)	198

3.6.6.	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)	202

3.6.7.	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	205

3.6.8.	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	212

3.6.9.	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	219

3.6.10.	Flux Cored Arc Welding (FCAW)	226

3.6.11.	Perform Soldering and Brazing Operations	230

3.7.	Microprocessor and Microcontroller	232

3.7.1.	Microprocessor applications	232

3.7.2.	Identify Microcontroller Types and its Architecture	234

3.7.3.	Interface Microcontroller with system	236

3.7.4.	Carryout various Microcontroller Applications	239

3.7.5.	Apply Microcontroller to build Control System	242

3.8.	Analog Electronics	244

3.8.1.	Identify Basic Electronics Components	244

3.8.2.	Design Rectifier using Diode	247

3.8.3.	Carry out Diode Application	249

3.8.4.	Implement Bipolar Junction Transistor (BJT) in Different Applications	250

3.8.5.	Implement Field Effect Transistor (FET) in Different Applications	255

3.8.6.	Implement Thyristor Family in Various Application	258

3.8.7.	Application of Operational Amplifier	260

3.9.	Computer Programming	263

3.9.1.	Installation of C++, IDE for Arduino and Python	263

3.9.2.	Carryout Basic C++ Programming	266

3.9.3.	Carryout Basic Programming using python	269

3.9.4.	Perform basic programming using microcontrollers	273

3.10.	Computer Integrated Machines	276

3.10.1.	Perform CNC Machine Operations	276

3.10.2.	Perform CNC Lathe Operations	279

3.10.3.	Perform CNC Milling Operations	286

3.11.	Machines and Mechanism	293

3.11.1.	Demonstrate Simple Mechanisms	293

3.11.2.	Perform Kinematic Analysis of Simple Mechanisms	296

3.11.3.	Demonstrate Mechanical Power Transmission Elements.	298

3.11.4.	Perform Kinematic Analysis of Mechanical Power Transmission Elements.	300

3.12.	AC & DC machines	303

3.12.1.	Verify Basic Laws of Electrical Machines.	303

3.12.2.	Analyze an Alternator	309

3.12.3.	Analyze Single Phase Motors	313

3.12.4.	Analyze Special Purpose Motors	316

3.12.5.	Analyze Three Phase Motors.	319

3.12.6.	Analyze Dc Generators	330

3.12.7.	Perform Tests on DC Generators	335

3.12.8.	Analyze Dc Motors.	340

3.12.9.	Perform Tests On DC Motors.	344

3.13.	Digital Electronics	353

3.13.1.	Verify Truth Tables of Digital Gates	353

3.13.2.	Construct & Verify Combinational Logic Circuit	357

3.13.3.	Construct and Verify Function of Flip Flops	361

3.13.4.	Construct Multi vibrator with 555 IC	363

3.13.5.	Construct Shift Registers and Counters with the Help of Flip Flops	365

3.14.	Sensor & Actuators	367

3.14.1.	Test Sensors and Transducers.	367

3.14.2.	Calibrate Sensors and Transducers.	376

3.14.3.	Interface Sensors and Transducers.	394

3.14.4.	Test Actuators	399

3.14.5.	Interface and control Actuator with sensors.	404

3.15.	Programmable logic controllers (PLCs)	409

3.15.1.	Install PLC software and Simulator	409

3.15.2.	Interface PLC with system (1)	416

3.15.3.	Program PLC using Ladder Logic	418

3.15.4.	Commissioning and Testing PLC	422

3.15.5.	Commissioning and Testing PLC	427

3.15.6.	Carry out Industrial Automation on PLC	430

3.15.7.	Interface PLC with system (2)	434

3.16.	Designing of Mechanical Members	438

3.16.1.	Calculate Stresses in Machine Parts	438

3.16.2.	Calculate diameter of cylinder for hoop and longitudinal stresses	441

3.16.3.	Calculate thickness and diameter of spherical shell for circumferential stresses	443

3.16.4.	Design welded joints for transverse and parallel fillet under static and fatigue loading	447

3.16.5.	Calculate stresses due to initial tightening and external load on screws	454

3.16.6.	Design dimension of square and rectangular keys	458

3.16.7.	Design shafts subjected to twisting moment	462

3.16.8.	Design flange coupling for specific torque	469

3.17.	Industrial Process Control & Instrumentation	472

3.17.1.	Design & Implement a process control	472

3.17.2.	Install Transducer & Transmitters used in Industrial Applications	475

3.17.3.	Install the process Regulators	478

3.17.4.	Install the Variable Frequency Drive (VFD)	481

3.17.5.	Install Pneumatic & Hydraulic systems	483

3.18.	Robotics	486

3.18.1.	Identify Various Parts of Robots	486

3.18.2.	Identify Various Parts of Robot Arm	490

3.18.3.	Develop Robot program	493

3.18.4.	Develop robotic arm structure	496

3.18.5.	Test the Robot	499

3.19.	Mechatronic Projects	502

3.19.1.	Access Control System using RFID Reader RC522	502

3.19.2.	Finger Print Sensor Based Biometric Attendance System	504

3.19.3.	Humidity and Temperature Monitoring System	506

3.19.4.	Smart Stick for Blinds	508

3.19.5.	Perform Line Following Robot	510

3.19.6.	Perform Smart Phone Control Robot Using Bluetooth	512

3.19.7.	Perform IoT Controlled based home automation system	515

3.19.8.	Perform Smart Phone Control Door Lock System	518

3.19.9.	Perform Security System Using Motion Detection	521

3.19.10.	Perform Water-level Detection in Tank	523

3.19.11.	Perform Mini CNC 2D Plotter	525

3.19.12.	Perform Voice Controlled Robot	527

3.20.	Hydraulics & Pneumatics	529

3.20.1.	Operate Hydraulic Bench and Its Functions	529

3.20.2.	Calibrate Bourdon Tube and Diaphragm Pressure Gauge	532

3.20.3.	Operate Hydraulic Press	534

3.20.4.	Verify Bernoulli’s Equation	537

3.20.5.	Analyze the performance of Pumps	540

3.20.6.	Construct circuit for Double Acting Hydraulic Cylinder	542

3.20.7.	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder	545

3.20.8.	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking	547

3.20.9.	Set Speed and Direction of Hydraulic Motor	552

3.20.10.	Operate Pressure Measuring Instruments	554

3.20.11.	Operate Flow Regulating Devices	560

4.	List of Personal Protective Equipment	565

5. Complete List of Tools, Equipment, Machines and Consumables	566

# 1. 	Introduction

Mechatronics is an interdisciplinary field that combines elements of engineering, machining, information technology, robotics, and electronics. It synchronize these diverse fields for the purpose of designing, fabrication and field testing of the production lines and manufacturing processes in order to optimize their performance and efficiencies. In this field several mechanical systems such as hydraulic, pneumatic, camera, electric motor and engines are interface, configure and combine in order to perform the tasks with increase efficiency and performance. Since several different mechanical, electrical and optical systems are inference, the continual operational maintenance, configuration and upgradation is required which need trained main power in the field of mechatronics in order to run the industrial processes and machines with any system failure or breakdown.

Typically, a mechatronics technologist need to perform several task such as standalone operation / testing of the mechanical and electronics systems in order to diagnose the faulty parts. These task may attribute to the checking, installation and configuration of electronic circuits, motor drives, hydraulics/ pneumatic moving system. The industry is lacking the expert technologist who has sound knowledge and command over all mechanical and electronics systems especially when these are interface and working in the close loop environment.

Mechatronics technologists can work in several fields, but they are commonly found in the manufacturing and automation sector. Several industrial process and production lines can be optimize by implementing fine mechanical precision in environmental conditions that may be dangerous or inefficient  for human hands and can be perform optimally by adding mechatronic solutions. Mechatronics has had a measurable impact both on the speed with which new products can be assembled.

The professionals that are skilled in this area of mechatronics are in high demand on production lines that range from projects as large as automotive vehicle manufacturing to the smallest microelectronic products. For instance, blood pressure monitoring device has pneumatic pump, a sensor to sense the blood pressure and electronics / microprocessor based circuit to measure the blood pressure. Similarly, several home appliances such as washing machines, ovens and refrigerators are now equipped with sophisticated mechatronics systems in order to optimize their products in terms of performance and electrical power consumption. Therefore, to repair and maintain such home appliances, the traditional technician are fail to work because of their training explicitly in the field of mechanical, electrical or electronics. Being conscious of these facts, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechatronics Technology under National Vocational Qualifications Framework (NVQF). Followings are the competency standards which are   developed by the Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Mechatronics industries from public and private sector, Engineering Universities and Technical Education of the country.

  - Purpose of the training program:

The purpose of the training is to set the high professional standards for the Mechatronics technological sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of the skills attained through formal or informal pathways

- Improve the quality and effectiveness of the training and assessment for Mechatronics Technological sector

- Enabling / helping / facilitating the existing workforce to indulge themselves in new technologies and methods

  - Overall objectives of training program

The Mechatronics qualifications of level 5 consists of both the theoretical and practical details and having the following Occupations

1. Health & Safety

2. Basic Machining

3. Electrical Essentials & Networks

4. Metrology

5. Basic Engineering Drawings

6. Welding

7. Microprocessor and Microcontroller

8. Analog Electronics

9. Computer Programming

10. Computer Integrated Machines

11. Machines and Mechanism

12. AC & DC machines

13. Digital Electronics

14. Sensor & Actuators

15. Programmable logic controllers (PLCs)

16. Designing of Mechanical Members

17. Industrial Process Control & Instrumentation

18. Robotics

19. Mechatronic Projects

20. Hydraulics & Pneumatics

## Competencies to be gained after completion of course

Detail of the competency standards included in qualification are given below:

Health and Safety

0714E&A1.	Maintain Health and Safety

0714E&A2.	Perform safety practices at work place

0714E&A3.	Develop Professionalism

Basic Machining

0714E&A4.	Perform Metal / Bench Work

0714E&A5	Perform general machining

0714E&A6	Perform cutting on Metal Circular/Power Heck Saw

0714E&A7	Perform tool sharpening

0714E&A8	Perform Basic Lathe Machine Operations

0714E&A9	Perform Drilling Machine Operations

0714E&A10	Perform Shaper, Planar and Slotter Machining Operations

0714E&A11	Perform Milling Operations

Electrical Essentials & Networks

0714E&A12	Operate Measuring Instruments

0714E&A13	Verify Ohm’s & Kirchhoff’s Laws by Implementing Series/Parallel Circuits

0714E&A14	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

0714E&A15	Construct Electromagnet to See Various Effects & Verify Faradays Law.

0714E&A16 Verify Law of Combination of Capacitor & Determine Break Down Voltage of Capacitor

0714E&A17	Maintain Tools & Equipment

0714E&A18	Make Cable/Wire Joints

0714E&A19	Prepare and Install Distribution Boards

0714E&A20	Carryout Basic Electrical Installations

0714E&A21	Install Single Phase Electrical Wiring

0714E&A22	Perform Testing of Electrical Wiring

0714E&A23	Operate Single Phase Transformer

0714E&A24	Perform Tests on Single Phase Transformer

Digital Skills

0714E&A25	Install computer operating systems and hardware

0714E&A26	Operate digital media technology

0714E&A27	Perform computer operations

0714E&A28	Use computer applications

0714E&A29	Create user documentation

0714E&A30	Create technical documentation

0714E&A31	Create basic databases

0714E&A32	Use social media tools for collaboration and engagement

0714E&A33	E-Commerce- SEO (Search Engine Optimization)

0714E&A34	E-Commerce- SCM (Supply Chain Management)

0714E&A35	E-Commerce- Social Media Marketing

0714E&A36	Use digital devices

0714E&A37	Operate word-processing applications

0714E&A38	Operate spreadsheet applications

0714E&A39	Operate presentation packages

0714E&A40	Perform writing and editing tasks

Metrology

0714E&A41	Take measurements with graduated tools

0714E&A42	Take measurements with combination set

0714E&A43	Perform Levelling

0714E&A44	Take measurements with gauges

0714E&A45	Perform measurements through Micrometer

0714E&A46	Measure dimensions with Vernier tools

0714E&A47	Measure angles with angle measuring Instruments

0714E&A48	Measure Threads and Gears

Basic Engineering Drawings

0714E&A49	Perform Basic Manual Drawing

0714E&A50	Construct different Engineering Curves

0714E&A51	Construct multi-view drawings

0714E&A52	Develop 2D CAD drawings

0714E&A53	Develop 3D CAD drawing

Welding

0714E&A54	Interpret Part and Assembly Drawing

0714E&A55	Evaluate the Use of Hand and Power Tools

0714E&A56	Perform Pre-Welding Operations

0714E&A57	Perform Oxy Acetylene Welding

0714E&A58	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)

0714E&A59	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)

0714E&A60	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A61	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A62	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions

0714E&A63	 Flux Cored Arc Welding (FCAW)

0714E&A64	Perform Soldering and Brazing Operations

Microprocessor and Microcontroller

0714E&A65. Design Microprocessor applications

0714E&A66	Identify Microcontroller Types and its Architecture

0714E&A67	Interface Microcontroller with system

0714E&A68	Carryout various Microcontroller Applications

0714E&A69	Apply Microcontroller to build Control System

Analogue Electronics

0714E&A70	Identify Basic Electronics Components

0714E&A71	Design a Rectifier Using Diode

0714E&A72	Carry Out Diode Application

0714E&A73	Implement Bipolar Junction Transistor (BJT) in Different Applications

0714E&A74	Implement Field Effect Transistors (FET) In Different Applications

0714E&A75	Implement (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in various application

0714E&A76	Design Operation Amplifier

Soft Skills

0714E&A77.	Develop workplace policy and procedures for sustainability

0714E&A78.	Manage meetings

0714E&A79.	Manage recruitment selection and induction processes

0714E&A80.	Manage personal work priorities and professional development

0714E&A81.	Manage workforce planning

0714E&A82.	Undertake project work

0714E&A83.	Identify and communicate trends in career development

0714E&A84.	Apply specialist interpersonal and counseling interview skills

0714E&A85.	Work safely in an office environment

0714E&A86.	Develop workplace documents

0714E&A87.	Prepare and implement negotiation

0714E&A88.	Maintain professionalism in the workplace

0714E&A89.	Organize schedules

Computer Programming

0714E&A90.	Installation of C++, IDE for Arduino and Python

0714E&A91.	Carryout Basic C++ Programming

0714E&A92.	Carryout Basic Programming using python

0714E&A93.	Perform basic programming using microcontrollers

Computer Integrated Manufacturing (CIM)

0714E&A94.	Perform CNC Machine Operations

0714E&A95.	Perform CNC Lathe Operations

0714E&A96.	Perform CNC Milling Operations

Machines and Mechanism

0714E&A97.	Demonstrate Simple Mechanisms

0714E&A98.	Perform Kinematic Analysis of Simple Mechanisms

0714E&A99.	Demonstrate Mechanical Power Transmission Elements

0714E&A100.	Perform Kinematic Analysis of Mechanical Power Transmission Elements

AC & DC Machines

0714E&A101.	Verify Basic Laws of Electrical Machines

0714E&A102.	Analyse an Alternator

0714E&A103.	Analyse Single Phase Motors

0714E&A104.	Analyse Special Purpose Motors

0714E&A105.	Analyse Three Phase Motors.

0714E&A106.	Analyze Dc Generators

0714E&A107.	Perform Tests on DC Generators

0714E&A108.	Analyze Dc Motors.

0714E&A109.	Perform Tests On DC Motors.

Digital Electronics

0714E&A110.	Verify Truth Tables of Digital Gates

0714E&A111.	Construct & Verify Combinational Logic Circuit

0714E&A112.	Construct and Verify Function of Flip Flops

0714E&A113.	Construct Multivibrator with 555 IC

0714E&A114.	Construct Shift Registers and Counters with the Help of Flip Flops

Sensors and Actuators

0714E&A115.	Test Sensors and Transducers

0714E&A116	Calibrate Sensors and Transducers

0714E&A117	Interface the Sensor & Transducers

0714E&A118	Test Actuators

0714E&A119	Interface and control Actuator with sensors

Programmable Logic Controllers (PLCS)

0714E&A120	Install PLC software and Simulator

0714E&A121	Interface PLC with system

0714E&A122	Program PLC using Ladder Logic

0714E&A123	Design and Test the PLC

0714E&A124	Carry out Industrial Automation on PLC

Designing of Mechanical Members

0714E&A125	Calculate Stresses in Machine Parts

0714E&A126	Calculate diameter of cylinder for hoop and longitudinal stresses

0714E&A127	Calculate thickness and diameter of spherical shell for circumferential stresses

0714E&A128	Design welded joints for transverse and parallel fillet under static and fatigue loading

0714E&A129	Calculate stresses due to initial tightening and external load on screws

0714E&A130	Design dimension of square and rectangular keys

0714E&A131	Design shafts subjected to twisting moment only

0714E&A132	Design flange coupling for specific torque

Industrial Process Control & Instrumentation

0714E&A133	Design & Implement a process control

0714E&A134	Install Transducer & Transmitters used in Industrial Applications

0714E&A135	Install the process Regulators

0714E&A136	Install the Variable Frequency Drive (VFD)

0714E&A137	Install Pneumatic & Hydraulic systems

Robotics

0714E&A138	Identify Various Parts of Robots

0714E&A139	Identify Various Parts of Robot Arm

0714E&A140	Develop Robot program

0714E&A141	Develop robotic arm structure

0714E&A142	Test the Robot

Mechatronic Projects

0714E&A143	Access Control System using RFID Reader RC522

0714E&A144	Finger print Sensor Based Biometric Attendance System

0714E&A145	Humidity and Temperature Monitoring System

0714E&A146	Smart Stick for Blinds

0714E&A147	Perform Line Following Robot

0714E&A148	Perform Smart Phone Control Robot Using Bluetooth

0714E&A149	Perform IoT Controlled based home automation system

0714E&A150	Perform Smart Phone Control Door Lock System

0714E&A151	Perform Security System Using Motion Detection

0714E&A152	Perform Water-level Detection in Tank

0714E&A153	Perform Mini CNC 2D Plotter

0714E&A154	Perform Voice Controlled Robot

Hydraulic and Pneumatics

0714E&A155	Operate Hydraulic Bench and Its Functions

0714E&A156	Calibrate Bourdon Tube and Diaphragm Pressure Gauge

0714E&A157	Operate Hydraulic Press

0714E&A158	Verify Bernoulli’s Equation

0714E&A159	Analyse the performance of Pumps

0714E&A160	Construct circuit for Double Acting Hydraulic Cylinder

0714E&A161	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder

0714E&A162	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking

0714E&A163	Set Speed and Direction of Hydraulic Motor

0714E&A164	Operate Pressure Measuring Instruments

0714E&A165	Operate Flow Regulating Devices

Entrepreneurial Skill

0714E&A166	Develop Entrepreneurial Skills

0714E&A167	Apply project information management and communications techniques

0714E&A168	Apply project human resources management approaches

0714E&A169	Direct human resources management of a project program

0714E&A170	Develop a project management plan

0714E&A171	Maintain business resources

0714E&A172	Develop a sales plan

0714E&A173	Plan and implement business-to-business marketing

0714E&A174	Address customer needs

0714E&A175	Manage personal finances

0714E&A176	Solve problems which jeopardize safety and security

0714E&A177	Coordinate a work team

0714E&A178	Lead small teams

0714E&A179	Plan and organize work

0714E&A180	Develop teams and individuals

0714E&A181	Apply problem solving techniques in the workplace using critical thinking

0714E&A182	Manage human resource services

  - Entry level of trainees

The entry for D.A. E National Certificate level 5, in Mechatronics Technology are

- A person having National Vocational Certificate level 4, in Mechatronics Technology.

- A person having Matric certificate with Science subjects

## Minimum qualification for teachers

- B.Sc. / B.E. in Mechatronics Engineering.

- 4 Years B. Tech in Mechatronics Technology with 3 Years teaching experience.

- D. A. E. in Mechatronics Technology with 7 Years teaching experience

- Must be able to communicate effectively

## Medium of instruction

English, and Urdu.

## Duration of the course:

The proposed curriculum is composed of 182 modules that will be covered in 4,927 Learning hours. Duration of the course is proposed to be Three years.

Level - 2 = 6 months.

Level - 3 = 6 months

Level - 4 = 1 Year

Level - 5 = 1 Year

The distribution of contact hours is given below:

Total.		4927 hours.

Theory.	1347 hours (27.34 %)

Practical.	3580 hours (72.66 %)

## Description and structure of the course

Following is the structure of the course:

| LEVEL 5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total | CH |
| Install PLC software and Simulator | 5 | Technical | 12 | 36 | 48 | 4.8 |
| Interface PLC with system | 5 | Technical | 6 | 18 | 24 | 2.4 |
| Program PLC using Ladder Logic | 5 | Technical | 12 | 24 | 36 | 3.6 |
| Commisioning and Testing the PLC | 5 | Technical | 12 | 24 | 36 | 3.6 |
| Carry out Industrial Automation on PLC | 5 | Technical | 12 | 24 | 36 | 3.6 |
| Calculate Stresses in Machine Parts | 5 | Technical | 6 | 18 | 24 | 2.4 |
| Calculator diameter of cylinder for hoop and longitudinal stresses | 5 | Technical | 6 | 18 | 24 | 2.4 |
| Calculate thickness and diameter of spherical shell for circumferential stresses | 5 | Technical | 8 | 24 | 32 | 3.2 |
| Design welded joints for transverse and parallel fillet under static and fatigue loading | 5 | Technical | 10 | 30 | 40 | 4 |
| Calculate stresses due to initial tightening and external load on screws | 5 | Technical | 8 | 12 | 20 | 2 |
| Design dimension of square and rectangular keys | 5 | Technical | 5 | 15 | 20 | 2 |
| Design shaft subjected to twisting moment only | 5 | Technical | 7 | 21 | 28 | 2.8 |
| Design flange coupling for specific torque | 5 | Technical | 3 | 9 | 12 | 1.2 |
| Design & Implement a process control | 5 | Technical | 8 | 12 | 20 | 2 |
| Install Transducer & Transmitters in Industrial Applications | 5 | Technical | 6 | 18 | 24 | 2.4 |
| Install the process Regulators | 5 | Technical | 8 | 24 | 32 | 3.2 |
| Install the Variable Frequency Drive (VFD) | 5 | Technical | 6 | 12 | 18 | 1.8 |
| Install Pneumatic & Hydraulic systems | 5 | Technical | 6 | 18 | 24 | 2.4 |
| Identify Various Parts of Robots | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Identify Various Parts of Robot Arm | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Develop Robot program | 5 | Technical | 9 | 18 | 27 | 2.7 |
| Develop robotic arm structure | 5 | Technical | 4 | 18 | 22 | 2.2 |
| Test the Robot | 5 | Technical | 4 | 18 | 22 | 2.2 |
| Access Control System using RFID Reader RC522 | 5 | Technical | 5 | 9 | 14 | 1.4 |
| Finger print Sensor Based Biometric Attendance System | 5 | Technical | 4 | 9 | 13 | 1.3 |
| Humidity and Temperature Monitoring System. | 5 | Technical | 3 | 9 | 12 | 1.2 |
| Smart Stick for Blinds | 5 | Technical | 4 | 9 | 13 | 1.3 |
| Perform Line Following Robot | 5 | Technical | 5 | 9 | 14 | 1.4 |
| Perform Smart Phone Control Robot Using Bluetooth | 5 | Technical | 5 | 9 | 14 | 1.4 |
| Perform IoT Controlled based home automation system | 5 | Technical | 8 | 12 | 20 | 2 |
| Perform Smart Phone Control Door Lock System | 5 | Technical | 6 | 12 | 18 | 1.8 |
| Perform Security System Using Motion Detection | 5 | Technical | 6 | 12 | 18 | 1.8 |
| Perform Water- Detection in Tank | 5 | Technical | 6 | 12 | 18 | 1.8 |
| Perform Mini CNC 2D Plotter | 5 | Technical | 5 | 12 | 17 | 1.7 |
| Perform Voice Controlled Robot | 5 | Technical | 7 | 12 | 19 | 1.9 |
| Operate Hydraulic Bench and Its Functions | 5 | Technical | 4 | 12 | 16 | 1.6 |
| Calibrate Bourdon Tube and Diaphragm Pressure Gauge | 5 | Technical | 2 | 6 | 8 | 0.8 |
| Operate Hydraulic Press | 4 | Technical | 2 | 6 | 8 | 0.8 |
| Verify Bernoulli’s Equation | 5 | Technical | 6 | 9 | 15 | 1.5 |
| Analyse the performance of Pumps | 5 | Technical | 4 | 12 | 16 | 1.6 |
| Construct circuit for Double Acting Hydraulic Cylinder | 4 | Technical | 4 | 6 | 10 | 1 |
| Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder | 5 | Technical | 4 | 12 | 16 | 1.6 |
| Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking | 5 | Technical | 8 | 12 | 20 | 2 |
| Set Speed and Direction of Hydraulic Motor | 5 | Technical | 2 | 6 | 8 | 0.8 |
| Operate Pressure Measuring Instruments | 5 | Technical | 14 | 21 | 35 | 3.5 |
| Operate Flow Regulating Devices | 5 | Technical | 6 | 18 | 24 | 2.4 |
| Develop Entrepreneurial Skills | Level 5 | Entrepreneurial | 15 | 3 | 18 | 1.8 |
| Apply management and communications techniques | Level 5 | Entrepreneurial | 15 | 3 | 18 | 1.8 |
| Create Human Resources Management Plan | Level 5 | Entrepreneurial | 15 | 3 | 18 | 1.8 |
| Plan And Implement Business-To-Business Marketing | Level 5 | Entrepreneurial | 15 | 3 | 18 | 1.8 |
| Manage Finances | Level 5 | Entrepreneurial | 20 | 3 | 23 | 2.3 |
| Develop a Project Management Plan | Level 5 | Entrepreneurial | 20 | 6 | 26 | 2.6 |
| Develop a Sales Plan | Level 5 | Entrepreneurial | 20 | 6 | 26 | 2.6 |
| Implement new Green Energy Technologies | Level 5 | Green Skills | 27 | 9 | 36 | 3.6 |
| To design automation system for the conservation of Energy | Level 5 | Green Skills | 25 | 9 | 34 | 3.4 |
| TOTAL |  |  | 462 | 738 | 1200 | 120 |
|  |  |  | 39% | 62% |  |  |

# Overview of the Curriculum for Mechatronics Technology

| Module Title and Aim | Learning Units | Theory Days/hours | Workplace Days/hours | Timeframe of modules |
| --- | --- | --- | --- | --- |

- Detail of Modules.

# Entrepreneurship

### Module: Develop Entrepreneurial Skills

Objective:

After the completion of this competency standard, the Trainee will be expected to develop a business plan, collect information regarding funding sources, develop a marketing plan and develop basic business communication skills. Trainee’ss underpinning knowledge regarding entrepreneurial skills will be sufficient to provide you the basis for your work.

Duration:  18 Hours			Theory:  15 Hours		Practice:  03 Hours          	        Credit Hours: 1.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop a business plan | Trainee will be able to: Conduct a market survey to collect following information: Customer /demand Tools, equipment, machinery and furniture with rates Raw material Supplier Credit / funding sources Marketing strategy Market trends Overall expenses Profit margin Select the best option in terms of cost, service, quality, sales, profit margin, overall expenses Compile the information collected through the market survey, in the business plan format | Main elements of business plan Filling the business plan format Enlist specific business terms used in the industry Describe 7Cs of business communication | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU2. Collect information regarding funding sources | Trainee will be able to: Identify the available funding sources based on their terms and conditions, maximum loan limit, payback time, interest rate Choose the best available option according to investment requirement Prepare documents according to the loan agreement requirement Include the information of funding sources in the business plan | Enlist the available funding sources Explain how to get loan to start a new business Explain market survey and its tools e.g.: questionnaire, interview, observation etc | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU3. Develop a marketing plan | Trainee will be able to: Make a marketing plan for the business including product, price, placement, promotion, people, packaging and positioning Include the information of marketing plan in the business plan | 7ps of marketing including product, price, placement, promotion, people, packaging and positioning | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU4. Develop basic business communication skills | Trainee will be able to: Communicate with internal customers e.g.: labor, partners and external customers e.g.: suppliers, customers etc., using effective communication skills Use different modes of communication to communicate internally and externally e.g.: presentation, speaking, writing, listening, visual representation, reading etc. Use specific business terms used in the market | Description of the market trends for specific product offering Different modes of communication and their application in the industry | Theory- 03 Hr Practical- 0Hrs Total- 03 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |

### Apply management and communications techniques

Objective:

After the completion of this competency standard, the Trainee will be able to contribute to communications planning, Conduct information management activities, Communicate project information, Contribute to assessing effectiveness of communication.

Duration:  18 Hours		Theory:  15Hours			Practice:  03 Hours          	        Credit Hours: 1.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Contribute to communications planning | Trainee will be able to: Identify, source and contribute relevant information requirements to initial project documentation. Contribute to developing and implementing the project communications plan and communications networks. | Outline methods of reviewing communication outcomes | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU2. Conduct information management activities | Trainee will be able to: Act on and process project information according to agreed procedures as directed, to aid decision-making processes throughout project life cycle. Maintain information to ensure data is secure and auditable. | Summarize models and methods of communications management in context of project life cycle and other project management functions | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU3. Communicate project information | Trainee will be able to: Communicate with clients and other stakeholders during project using agreed networks, processes and procedures to ensure flow of necessary information. Ensure reports are prepared and released according to authorization, or produced for release by others. Seek information and advice from appropriate project authorities as required. | Identification of Project Information, policies and procedures relevant Summarize models and methods of communications management in context of project life cycle and other project management functions | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU4. Contribute to assessing effectiveness of communication | Trainee will be able to: Assist in ongoing review of project outcomes to determine effectiveness of communications-management activities. Report communications-management issues and responses to higher project authorities for application of lessons learned to future projects. | Assessment of effectiveness of project communication | Theory- 03 Hr Practical- 0Hrs Total- 03 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |

### Module: Create Human Resources Management Plan

Objective:

After the completion of this competency standard, the Trainee will be able to develop a plan for a telecommunications project, including assessing project requirements and planning for all stages to completion and final documentation. It applies to individuals who are likely to have responsibility for the provision of installations, maintenance, upgrades and new services.

Duration:  18 Hours			Theory:  15 Hours		Practice:  03 Hours          	        Credit Hours: 1.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Prepare project management plan | Trainee will be able to: Evaluate and assess project brief and related documents. Produce document on project tasks and associated timelines, including installation processes and test requirements. Assess and produce document on resource requirements to assist allocation of appropriate resources. Produce training plan assessing training needs and associated timelines for efficient project implementation. Determine and document budgetary requirements. Discuss roles of all identified parties associated with project to ensure their involvement. Produce project verification document, including monitoring and control processes, and review processes such as quality audits. Consult with all relevant parties prior to finalizing draft plan and make changes as appropriate. | Preparation of Project Plans and their management Evaluation and assessment of project related documentation and tasks Project Budget preparation | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU2. Develop and evaluate management plan | Trainee will be able to: Produce preliminary plan for consultation, including identified factors that may impact on realization of project and observance of relevant legislation, codes, regulation and standards. Consult with client and clarify any amendments. Develop final plan with recommendations. | Development and evaluation of project management plans Client dealing for clarification and amendments in project plans Finalization of plans | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU3. Communicate project information | Trainee will be able to: Produce and document final plan to include implementation details and training needs. Present plan to client and obtain sign off. | Presentation of Project plan to client | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU4. Contribute to assessing effectiveness of communication | Trainee will be able to: Assist in ongoing review of project outcomes to determine effectiveness of communications-management activities. Report communications-management issues and responses to higher project authorities for application of lessons learned to future projects. | Review of project outcomes and asses effectiveness of business communication | Theory- 03 Hr Practical- 0Hrs Total- 03 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |

### Module: Plan And Implement Business-To-Business Marketing

Objective:

After the completion of this competency standard, the Trainee will be able to plan and implement business-to-business (B2B)
marketing. It applies to individuals who work in a supervisory capacity in a team environment, who possess a sound theoretical knowledge base and demonstrate a range of managerial skills to ensure business activities are conducted effectively. In this role, individuals may work in small, medium or large enterprises across a variety of industries.

Duration:  18 Hours			Theory:  15 Hours		Practice:  03 Hours          	        Credit Hours: 1.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify and evaluate business -to-business marketing strategies | Trainee will be able to: Identify B2B markets in an industry context Research characteristics of business markets in an industry context Identify and analyze factors influencing business buyers in an industry context Analyze the business buying process and its implications in the industry context Research and analyze a range of B2B marketing strategies appropriate for the organization Identify key personnel in buying decision process in the organization s business markets | Identification of Business to Business markets | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU2. Select business-to business marketing strategies | Trainee will be able to: Analyze trends within business markets and identify B2B marketing opportunities for the organization Identify and analyze success of the organization s previous B2B marketing strategies Select most appropriate B2B marketing strategies and activities that fit with organization s strategic & marketing plans | Preparation of business to business strategies | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU3. Plan and develop business-to-business marketing activities | Trainee will be able to: Record B2B marketing objectives and purpose Calculate costs of B2B marketing activities with assistance of appropriate personnel Select methods to report and measure effectiveness of B2B marketing activities Assign responsibilities to team members for B2B marketing activities Record B2B marketing plan and present to relevant stakeholders Assemble required resources to implement B2B marketing plan | Plan the business to business strategies | Theory- 04 Hr Practical- 01Hrs Total- 05 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU4. Implement and monitor business-to business marketing plan | Trainee will be able to: Schedule work on each B2B marketing campaign element, according to lead times required and marketing plan Brief staff and suppliers on their budgets, timelines, roles and responsibilities, and legal and ethical requirements Plan implementation of B2B marketing activities according to marketing plan Identify & use evaluation criteria and evaluation methods to determine effectiveness of marketing plan Analyze success indictors of B2B marketing plan and record performance according to organizational reporting requirements | Trends in B2B marketing, corporate sector and business sector and preparation of plans | Theory- 03 Hr Practical- 0Hrs Total- 03 Hr | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |

### Module: Manage  Finances

Objective:

After the completion of this competency standard, the Trainee will be able to develop, implement and monitor a personal
budget in order to plan regular savings and manage debt effectively.

Duration:  18 Hours			Theory:  15 Hours		Practice:  03 Hours          	        Credit Hours: 1.8

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop a personal budget | Trainee will be able to: Calculate current living expenses using available information to prepare a personal budget. Keep a record of all income and expenses for a short period of time to help estimate ongoing expenses. Subtract total expenses from total income to determine a surplus or deficit budget for the specified period. Find reasons for a deficit budget and ways to reduce expenditure identified. Identify ways to increase income, if possible | Preparing cash flow statement Basic concepts of Time value of money Understanding of current and fixed assets Understanding of elements of equity and liabilities (long term and short term) Identification of available financial resources Step by step budget plan preparation Budget review techniques | Theory- 04 Hrs Practical- 1Hrs Total- 5 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU2. Develop longer term personal budget | Trainee will be able to: Analyze income and expenditure and set longer term personal, work and financial goals. Develop a longer-term budget based on the outcomes of short-term budgeting, and adjust to meet living, work and future career requirements. Identify obstacles that might affect finances such as job loss, sickness or unexpected expenses contingency savings Formulate a regular savings plan based on budget, using secure savings products and services. Monitor expenditure against budget and identify areas of possible expenditure saving | Develop a personal budget based on analysis of expenditure and income Monitor expenditure for a period of up to 2 weeks. | Theory- 04 Hrs Practical- 1Hrs Total- 5 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU3. Identify ways to maximize future finances | Trainee will be able to: Determine sources and ways to maximize personal income, including from work, investments or available government payments/allowances. Get further education or training to maintain or improve future income. Identify the need for debt to finance living and other expenses, and determine the appropriate levels of debt and repayment. Consolidate existing debt, where possible, to minimize interest costs and fees. Seek professional money management services, where available, to ensure financial plans are effective and achievable | Formulate goals and identify financial contingency plans | Theory- 07 Hrs Practical- 1Hrs Total- 5 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |

### Module: Develop A Project Management Plan

Objective:

After the completion of this competency standard, the Trainee will be able to develop a plan for a telecommunications project, including assessing project requirements and planning for all stages to completion and final documentation. It applies to individuals who are likely to have responsibility for the provision of installations, maintenance, upgrades and new services.

Duration:  26 Hours		Theory:  20 Hours			Practice:  06 Hours          	        Credit Hours: 2.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Prepare project management plan | Trainee will be able to: Evaluate and assess project brief and related documents. Produce document on project tasks and associated timelines, including installation processes and test requirements. Assess and produce document on resource requirements to assist allocation of appropriate resources. Produce training plan assessing training needs and associated timelines for efficient project implementation. Determine and document budgetary requirements. Discuss roles of all identified parties associated with project to ensure their involvement. Produce project verification document, including monitoring and control processes, and review processes such as quality audits. Consult with all relevant parties prior to finalizing draft plan and make changes as appropriate. | Preparation of Project Plans and their management Evaluation and assessment of project related documentation and tasks Project Budget preparation | Theory- 05 Hrs Practical- 01Hrs Total- 06 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU2. Develop and evaluate management plan | Trainee will be able to: Produce preliminary plan for consultation, including identified factors that may impact on realization of project and observance of relevant legislation, codes, regulation and standards. Consult with client and clarify any amendments. Develop final plan with recommendations. | Development and evaluation of project management plans Client dealing for clarification and amendments in project plans Finalization of plans | Theory- 05 Hrs Practical- 01Hrs Total- 06 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU3. Communicate project information | Trainee will be able to: Produce and document final plan to include implementation details and training needs. Present plan to client and obtain sign off. | Presentation of Project plan to client | Theory- 5 Hrs Practical- 02Hrs Total- 07 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU4. Contribute to assessing effectiveness of communication | Trainee will be able to: Assist in ongoing review of project outcomes to determine effectiveness of communications-management activities. Report communications-management issues and responses to higher project authorities for application of lessons learned to future projects. | Review of project outcomes and asses effectiveness of business communication | Theory- 5 Hrs Practical- 02Hrs Total- 07 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |

### Module: Develop A Sales Plan

Objective:

After the completion of this competency standard, the Trainee will be able to develop a sales plan for a product or service for a team covering a specified sales territory based on strategic objectives and in accordance with established performance targets. It applies to individuals working in a supervisory or managerial sales role who develop a sales plan for a product or service.

Duration:  26 Hours		Theory:  20 Hours			Practice:  06 Hours          	        Credit Hours: 2.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify organizational strategic direction | Trainee will be able to: Obtain and analyze assessment of market needs and strategic planning documents Review previous sales performance and successful approaches to identify factors affecting performance Analyze information on market needs, new opportunities, customer profiles and requirements as a basis for decision making | Assessment of market needs and strategic planning documents preparation Knowledge of business opportunities, customer profiles and decision making | Theory- 05 Hrs Practical- 01Hrs Total- 06 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU2. Establish performance targets | Trainee will be able to: Determine practical and achievable sales targets Establish realistic time lines for achieving targets Determine measures to allow for monitoring of performance Ensure objectives of the sales plan and style of the campaign are consistent with organizational strategic objectives and corporate image | Statistical techniques for analyzing sales and market trends | Theory- 05 Hrs Practical- 01Hrs Total- 06 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU3. Develop a sales plan for a product | Trainee will be able to: Determine approaches to be used to meet sales objectives Identify additional expertise requirements and allocate budgetary resources accordingly Identify risks and develop risk controls Develop advertising and promotional strategy for product Identify appropriate distribution channels for product Prepare a budget for the sales plan Present documented sales plan to appropriate personnel for approval | Principles and techniques for selling Sources of internal and external information Identification of organizational strategic direction and developing a product sales plan Preparation of sales plan and its approval | Theory- 5 Hrs Practical- 02Hrs Total- 07 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |
| LU4. Identify support requirements | Trainee will be able to: Identify and acquire staff resources to implement sales plan Develop an appropriate selling approach Train staff in the selling approach selected Develop and assess staff knowledge of product to be sold | Resources requirement for sales plan | Theory- 5 Hrs Practical- 02Hrs Total- 07 Hrs | Internet Laptop/Computer White board Projector screen Multimedia projector | Classroom |

### Install PLC software and Simulator

Objective: The module covers the knowledge and skill required to Install PLC software and Simulator, Program a PLC using simulator, Analyze user requirements and specifications, Identify error codes and functionality testing, Identify HMI Software and programming, Identification and configuration of SCADA Systems.

Duration: 48 Hours				Theory: 12 Hours						Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Install PLC software and Simulator | Trainee will be able to: - Install the PLC Programming - Software as per PLC manufacturer such as RS Logic 5000 and RS links classic etc. - Select the best and most frequently used Simulator for programming - Select the module - Input the generic value as per universal settings (check from internet/helped) - Start The Simulator. - Open PLC programming software. - Install the HMI software as per manufacturer such as Win CC flexible and Easy Builder etc. | - Define simulator. - Differentiate between simulator and emulator. - Define HMI working. - Define communication Protocol Practical Activity Install PLC programming software along with HMI editor | Theory- 2 Hrs Practical-6 Hrs Total- 8 Hrs | - PLC CPU - PLC Software and hardware - Helping Manual - User guide - Simulator - HMI software and hardware - Communication modules - PC - PLC programming cable. | Class Room and workshop |
| LU2. Program a PLC using simulator | Trainee will be able to: - Open the programming software as per PLC manufacturer - Ensure that the Simulator is connected and is in ON condition - Create a basic ladder logic program for ON/OFF of a bulb using Examine ON and Examine OFF switch - Create a basic ladder logic program for Simple Start/Stop Ladder Logic Relay - Create a basic ladder logic program for Single Push Button On/Off Ladder Logic - Create a basic ladder logic program for with On Delay Timer - Create a basic ladder logic program for with Off Delay Timer - Create a basic ladder logic program for Up Counter - Create a basic ladder logic program for Down Counter - Create a basic ladder logic program for Motor Control - Create a basic ladder logic program for Traffic signal lights - Create a basic ladder logic program for Elevator Control. | - Define NO and NC in Ladder logic. - Define relay operation in PLC. - Differentiate between momentary switch and toggle switch. - Define different types of timer and its operation. - Define different types of counter operation. Practical Activity Develop a PLC logic for Motor Direction and jog control circuit. | Theory- 2 Hrs Practical-6 Hrs Total- 8 Hrs | - PLC CPU - PLC Software and hardware - Helping Manual - User guide - Simulator - HMI software and hardware - Communication modules - PC - PLC programming cable. | Class Room and workshop |
| LU3. Analyze user requirements and specifications | Trainee will be able to: - Draw the general value chain of the end user industry - Highlight the set of activities that a firm operating in a specific industry performs in the value chain drawing - Enlist the equipment / gauges / sensors / actuators / transducers used in different stages of the process - Identify critical stages in the process - Identify the safety aspect required in the critical stages of the process - Analyze the possible automation in the existing processes and global trends in automation - Analyze the client requirement at broad level from the proposal - Generate a report of various industrial processes involved in industry. - Collect the required specification of the equipment (if already prepared by the user) and clarify the technical specification | - Define the method of analyzing the customer requirements. Practical Activity Develop and report for analyzing the user requirements for acquiring PLC based automation system. | Theory- 2 Hrs Practical-6 Hrs Total- 8 Hrs |  | Class Room and workshop |
| LU4. Identify error codes and functionality testing | Trainee will be able to: - Use error codes to identify programming errors - Determine program scan time in milliseconds - Use a force function to verify an input - Use a force function to verify an output - Perform measurement of status/values at IOs and variables used for logical operation using programming software | - Define various types of error codes used in programming. - Define force function and its effectiveness. Practical Activity Test the hardware for the unusual operation using force function to test the system robustness. | Theory- 2 Hrs Practical-6 Hrs Total- 8 Hrs | - PLC CPU - PLC Software and hardware - Helping Manual - User guide - Simulator - HMI software and hardware - Communication modules - PC - PLC programming cable. | Class Room and workshop |
| LU5. Identify HMI Software and programming. | Trainee will be able to: - Open software. - Make a program of graphical representation of tasks on CU2 (P3 – P12). - Program of HMI software to communicate with PLC. - Download the software on HMI and Run. - Create the connection between two units of PLCs. - Identify the communications between two units. - Convey the data of both units on HMI. | - Define the difference between upload and download of PLC program on HMI. - How to select the appropriate PLC from the software? - Define PLC networking. - Define HMI OFF line simulation. - Define HMI ON line simulation. Practical Activity - Design an HMI interface of Motor control. - Integrate the HMI with PLC for ON Line HMI operation of Motor. | Theory- 2 Hrs Practical-6 Hrs Total- 8 Hrs |  | Class Room and workshop |
| LU6. Identification and configuration of SCADA Systems | Trainee will be able to: - Create the connection between two units of PLCs. - Define the programming. - Identify the communications between two units. - Convey the data of both units on HMI. | - Define the types of PLC Network connections. - Define Network Protocols. - Define key features of SCADA System. - Define the process of PLC and HMI configuration on SCADA. | Theory- 2 Hrs Practical-6 Hrs Total- 8 Hrs | - PLC CPU. - HMI software and hardware - SCADA software - Communication modules - Communication Cable. | Class Room and workshop |

### Interface PLC with system

Objective: The module covers the knowledge and skill required to: Differentiate Modules of control systems, Install / Modify PLC and its components, Install field instruments

Duration: 24 Hours				Theory:06 Hours						Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Differentiate Modules of control systems | Trainee will be able to: - Identify the standalone/ module - Demonstrate its functions & capabilities - Extract information from device part number | - Define the basic control system components. - Define the specifications of modules. - Name different types of modules used with the system. - Explain the various module operations w.r.t. applications. - Demonstrate the datasheet reading methods Practical Activity - Select and demonstrate the module as per the required parameters and specification mentioned in the user guide. | Theory- 2 Hrs Practical-6 Hrs Total-08 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC editing software. - Personal Computer. - Power source. - Test bench - Connection cords. - Hand tools. - Din rail. - Screw driver set. - Cutter. - Player | Class Room and workshop |
| LU2. Install / Modify PLC and its components | Trainee will be able to: - Implement hardware configuration as per given design - Disassemble the hardware in accordance user’s manual | - Define PLC hardware and its associated module. - Define the dissembling process of the PLC modules and associated hardware. Practical Activity Demonstrate the process of assembling and dissembling of modules. | Theory- 2 Hrs Practical-6 Hrs Total-08 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC editing software. - Personal Computer. - Power source. - Test bench - Connection cords. - Hand tools. - Din rail. - Screw driver set. - Cutter. - Player | Class Room and workshop |
| LU3. Install field instruments | Trainee will be able to: - Identify field instruments - Install the wiring from Module to instrument location - Install the instrument - Perform field testing and calibration (if required) - Generate the field-testing reports | - Explain the process of field instrument installation. - Explain the Instrument parameter under consideration operation. Practical Activity Install the provided instrument as per the SO.P. | Theory- 2 Hrs Practical-6 Hrs Total-08 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC editing software. - Personal Computer. - Power source. - Test bench - Connection cords. - Hand tools. - Screw driver set. - Cutter. - Player | Class Room and workshop |

### Program PLC using Ladder Logic

Objective: The module covers the knowledge and skill required to: Installation of PLC programming software, Design Basic logic program on Ladder logic, Apply timers on Ladder logic program, Apply counters on Ladder logic program

Duration: 36 Hours				Theory:12 Hours						Practical: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Installation of PLC programming software . | Trainee will be able to: - Identify PLC languages a) Instructions List (IL) a1) Structured Text (ST) b) Graphical language b1) Ladder Diagrams (LD) b2) Function Block Diagram (FBD) b3) Sequential Function Chart (SFC) - Download and open setup file for software installation on windows PC. - Configure installation setup. - Install all parts of software. - Complete installation and restart the PC. | - Explain the various programming languages of PLC. - Explain various PLC software’s from different brands. - Explain the procedure of software installation with port setup. - Configure the data rate for PLC to PC communication. Practical Activity - Perform the installation procedure for PLC programming software on personal computers. - Establish communication between PLC and Program editor. | Theory- 3 Hrs Practical-6 Hrs Total- 9 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC Communication cable. - PLC editing software. - Push buttons. - Light Indicators. - Personal Computer. - Power source. - Test bench. - Table chair. - Stationary. | Class Room and workshop |
| LU2. Design Basic logic program on Ladder logic | Trainee will be able to: - Open software GUI. - Design basic logical ON/OFF operation in Ladder logic programming. - Run simulator software to verify program. - Upload that program on PLC hardware | - Define PLC Programming interface with its various tabs. - Explain the process of using Simulator for testing the logic. - Define the PLC addressing for handling the PLC I/O modules. - Demonstrate the PLC uploading and downloading process. Practical Activity - Develop a PLC ON/OFF logic for digital Ii/O (using any language) on PLC editing software. - Simulate the program on PLC editor for error testing and debugging. . | Theory- 3 Hrs Practical-6 Hrs Total- 9 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC Communication cable. - PLC editing software. - Personal Computer - Power supply. - Test bench - Table. - Chair. - Stationary. | Class Room and workshop |
| LU3. Apply timers on Ladder logic program . | Trainee will be able to: - Open software GUI. - Design program and insert timers. - Configure timer block according to requirement. - Run simulator software to verify this program. - Upload this program and run on PLC. | - Define timers and their function. - Explain different types of timer application. - Explain ON Delay and OFF Delay timers functions. - Explain time retention function in PLC. - Explain various modes and timer resolution. Practical Activity Develop a timer logic to operate motor Stat Delta control system using timer. | Theory- 3 Hrs Practical-6 Hrs Total- 9 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC Communication cable. - PLC editing software. - Personal Computer - Power supply. - Test bench - Table. - Chair. - Stationary. | Class Room and workshop |
| LU4. Apply counters on Ladder logic program | Trainee will be able to: - Open software GUI. - Design program and insert counters. - Configure counter block according to requirement. - Run simulator software to verify this program. - Upload this program and run on PLC | - Define counters and their function. - Explain UP/ Down counter function. - Explain different types of counter application. Practical Activity Develop a logic to operate a car parking management system having capacity of 100 cars using counter function. | Theory- 3 Hrs Practical-6 Hrs Total- 9 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC Communication cable. - PLC editing software. - Personal Computer - Power supply. - Test bench - Table chair. - Stationary. | Class Room and workshop |

### Commissioning and Testing the PLC

Objective: The module covers the knowledge and skill required to: Prepare work plan, Design and program PLC, Installation and Commissioning, Test the PLC.

Duration: 36 Hours				Theory:12 Hours						Practical: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Prepare work plan | Trainee will be able to: - Suggest globally practiced and accepted automation systems if the user is not aware of the technical specifications - List down the sub systems that are involved in the process - List down sensors and actuators requirement. - Collect information on process logic - Collect information for operator station screens - Decide on whether the system can be developed as per the user requirement - Support the project manager in calculating the time required for each stage to ensure completion of project - Assist in preparing the work plan with deliverables and timelines - Explain the expected output to the user - Calculate the number of days needed for commissioning of the panel at site - Summarize the user requirement | - Define the sub systems that are involved in the process. - Define sensors and actuators requirement. - Define the process and its flow. - Define customer requirement and customization in work plan. - Define system Imperilment and planning implications. - Explain time estimation in project planning. - Explain how to ensure customer satisfaction. - Explain the methods to meet deadlines. - Explain the time line estimation for process control. Practical Activity - Design the work plan sheet for project implementation. - Develop a generic work sheet for implementing the work plan as plan | Theory- 3 Hrs Practical-6 Hrs Total- 09 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC Communication cable. - PLC editing software. - Relays. - Push buttons. - Light Indicators. - Personal Computer. - Pen - Pencil - Eraser - Marker - Power source. - Test bench | Class Room and workshop |
| LU2. Design and program PLC. | Trainee will be able to: - Develop PLC application as per user requirement by following the standard operating procedure (SOP) of the organization - Apply approved engineering concepts, processes and principles in developing the PLC application - Install organization approved software (system and application software) to develop the system - Identify the requirement of indications, switchgears and accessories - Develop the control circuit drawing - Prepare wiring plans - Integrate the main process system with the sub-systems as per the user requirement (e.g., using communication protocol) - Ensure that safety aspect of the process is captured in the design plan - Program PLC as per FDF - Program SCADA Application - PLC-SCADA Communication - Create backup copies of all designs developed for control panel and store in a secure location - Prepare a product manual and store them for future references. | - Define Programming. - Define program standardization - Explain PLC programming for customized control system. - Explain key features of standard programming in any PLC language. - Describe essential of system design. - Describe hardware design for any control system for PLC. Practical Activity - Develop a folio for hardware required for PLC based control system. - Design a PLC program to automate the system as per requirement. | Theory- 3 Hrs Practical-6 Hrs Total- 09 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC Communication cable. - PLC editing software. - Associated hardware (limit switch, actuators ) hardware. - Push buttons. - Light Indicators. - Personal Computer - Power supply. - Hand tools. - Screwdrivers. - Tape. - Player. - Test bench | Class Room and workshop |
| LU3 Installation and Commissioning | Trainee will be able to: - Check availability of panel and tools required for installation - Check the internal panel wiring and ensure that it is in accordance with the design drawing - Carry out insulation check of internal panel wiring and devices within the panel - Check if batteries and chargers have been assembled in accordance with the manufacturers recommended procedures - Identify the conductors size and capacity for installation - Ensure that the panel is positioned as prescribed, following safety norms - Make connections to socket outlets, switches and protective conductors - Perform settings as per customer requirements on the equipment in each of the panels - Test all control system interlocks - Check each digital control point by comparing the command at the control panel and status of the device that it controls - Ensure that fuses, switches and other protective devices are labeled correctly - Follow the grounding and earthling procedures while commissioning - Put danger and warning notices, (if necessary) - Test continuity, insulation resistance, functions of all devices, etc., after completion of installation | - Define personal safety standards during work. - Define the term Installation. - Define the term commissioning. - Differentiate between installation and commissioning. - Explain the working of necessary tools needed for the job. - Explain electrical drawing reading and editing process for the assigned task. - Describe the necessity of Pannel insulation. - Explain the importance of insulation safety. - Explain safety standards adopted during installation and commissioning process. - Describe the backup power and its essentials. - Describe the checking procedures to ensure smooth operation. - Define wiring standards and its selection criteria. - Define panel standards and essentials. - Explain the importance of interlocking in an electrical system. - Describe cable tagging/ labeling and its importance in fault detection. - Explain the participation of earthling in plant protection. - Explain the pre and post installation test for leakage current. - Explain the importance of hazard sign during plant erection Practical Activity Install and commission the given electrical and PLC panels as per instruction. Keeping the safety standard in line with the process till smooth operating conditions achieved. | Theory- 3 Hrs Practical-6 Hrs Total- 09 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC Communication cable. - PLC editing software. - Associated hardware (limit switch, actuators ) hardware. - Push buttons. - Light Indicators. - Personal Computer - Power supply. - Hand tools. - Screwdrivers. - Dinrail - Pannel - Wires - Cables - Connectors - Player. - Test bench | Class Room and workshop |
| LU4. Test the PLC | Trainee will be able to: - Locate field devices and their interface to PLC - Test the system in off line mode using simulator - Test the gauges independently for integration of main system with the sub-systems (if applicable) - Verify that the system conforms with all the user specifications during testing - Rework if there are any issues found and fix them - Send the test report for review to the customer - Perform Factory Acceptance Test (FAT) - Perform site acceptance test plan | - Describe the pre start procedures of PLC testing. - Describe PLC static (before operation) Input and output wiring check. - Describer the control program review. - Explain the Dynamic system checkout (Program logic review). - Explain Factory Acceptance Test (FAT) and Site Acceptance Test (SAT) for PLC. Practical Activity Perform the Factory Acceptance Test (FAT) of a PLC. Adopt necessary procedure to test the PLC operation. | Theory- 3 Hrs Practical-6 Hrs Total- 09 Hrs | - PLC Power supply. - PLC CPU. - PLC Interfacing module. - PLC Communication cable. - PLC editing software. - Associated hardware (limit switch, actuators) hardware. - Push buttons. - Light Indicators. - Personal Computer - Power supply. - Hand tools. - Screwdriver - Test bench |  |

### Carry out Industrial Automation on PLC

Objective: The module covers the knowledge and skill required to: Implement industrial application using instruments and communication system, Analyze user requirements and specifications, Implement DC Motor control with PLC,  Proximity and Limit Switches, Maintenance and Troubleshooting

Duration: 36 Hours				Theory: 12 Hours						Practical: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Implement industrial application using instruments and communication system | Trainee will be able to: - Sketch the process of application (Flowchart, Pseudo code) - Identify process data and communication protocols and their specification - Implement open/close loop motor control - Implement a communication BUS system to monitor remote data | - Explain the use of Flow chart for process elaboration/demonstration. - Explain the difference between Pseudo code and flowchart. - Describe various communication protocols (Modbus, CNET, PPI, MPI….) used in PLC for industrial communication. - Describe DCS and SCADA communication protocol. - Explain Remote Terminal Unit RTU uses and application. - Describe the Industrial Ethernet communication. Practical Activity - Establish a communication between PC and PLC using MPI communication cable with desired parameters. - Establish a communication between PC and HMI using RS232 communication cable with desired parameters. - Establish a communication between PLC and HMI using communication cable with desired parameters | Theory- 3 Hrs Practical-6 Hrs Total- 09 Hrs | - PLC Software - Communication Module - Simulator - Helping Manual - User guide - Communication driver. | Class Room and workshop |
| LU2. Analyze user requirements and specifications | Trainee will be able to: - Draw the general value chain of the end user industry - Highlight the set of activities that a firm operating in a specific industry performs in the value chain drawing - Enlist the equipment / gauges / sensors / actuators / transducers used in different stages of the process - Identify critical stages in the process - Identify the safety aspect required in the critical stages of the process - Analyze the possible automation in the existing processes and global trends in automation - Analyze the client requirement at broad level from the proposal - Generate a report of various industrial processes involved in industry - Collect the required specification of the equipment (if already prepared by the user) and clarify the technical specification. | - Describe the general requirements of a project. - Describe the frequently required parameters considered by the user in PLC based system. - Explain the critical stages of a project for quick response. - Describe the methods for selecting the appropriate hardware or software as per user need. - Describe the basic project management skill needed to complete the project without omitting the necessary needs. - Describe the importance of supportive (exemplary) documents for justifying the selection of components and software. Practical Activity - Implement the PLC based project from user requirement till finalization stage with fulfilling all necessary user and technical needs | Theory- 3 Hrs Practical-6 Hrs Total- 09 Hrs | - Stationary. - Need Assessment Form. - Hardware manuals. - Manufacturer list. - Cost analysis tool. - Varinty analysis. - MTBF evaluation. - Project management software. | Class Room and workshop |
| LU3 Implement DC Motor control with PLC, Proximity and Limit Switches. | Trainee will be able to: - Enlist the required hardware and tool used in different stages of the process. - Draw a circuit connection diagram - Draw control strategy of system - Make physical connection of PLC, DC motor, Proximity switch and limit switch as per drawing. - Power up the circuit. - Write and upload the code - Write report | - Explain the method to generate requirement list (hardware / software). - Explain the circuit connection diagram for elaboration. - Explain the flow control of the operating logic of PLC. - Describe the wiring connection layout of loads and control circuit associated with PLC system. - Describe the operation of sensor and transducers associated with the PLC system. - Describe the control logic of the system systematic operation. - Explain the report generation methods for given tasks. Practical Activity A part is places on the conveyer at S1. A start push button is pressed which moves the part on the conveyer to the spray painting booth. As the part enter the booth sensor SN1 Enable paint sprayer. As the conveyer moves on senxor SN2 sense the part and stop the spray motor. When the part reaches at location S2 the conveyer stops automatically. The system may be stop at any time by pressing stop button. | Theory- 3 Hrs Practical-6 Hrs Total- 09 Hrs | - PLC Software - DC Motor - Proximity Switches - Limit Switches - Screw drivers - Simulator - Helping Manual - User guide - Wiring and Connectors - Power supply. - PPE | Class Room and workshop |
| LU4. Maintenance and Troubleshooting | Trainee will be able to: - Identify status of LEDs on Power Supply, CPU, I/O Cards - Match status LEDs within troubleshooting tables defined in manufacturer manual - Monitoring of Input and Output status using Programming interface - Carry out replacement of faulty cards as per manufacturer instructions - Perform replacement of faulty back up battery on PLC - Identify problems in control circuit for I/O by loop testing, contact resistance, signal electrical parameters | - Describe Error codes w.r.t manufactures specification. - Explain the uses of troubling table - Explain the hardware troubleshooting methods for associated components (Diode, transistor, LED, CPU, Modules). - Explain basic troubleshooting technique may incuse short circuit test, continuity test, power break up test. Practical Activity Troubleshoot the power supply of the PLC for fault detection and elimination. | Theory- 3 Hrs Practical-6 Hrs Total- 09 Hrs | - PLC . - Power Supply. - Power Source. - Test and measuring equipment. - Troubleshooting manual. - PPE |  |

### Calculate Stresses in Machine Parts

Objective: This competency standard covers the skills and knowledge required to Calculate Tensile Loads, Calculate Compressive load and Calculate Shear load.

Duration:  24			Hours			Theory: 06	Hours				Practice: 	18	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Calculate Tensile loads | Trainee will be able to: - Select the attachments for tensile load. - Select the specimen as per machine manual description and capacity. - Calculate Dimensions of the specimen. - Mount specimen properly. - Perform tensile load test. - Prepare the dimension’s table with loaded and unloaded part. - Calculate the Stresses, strain, modulus of elasticity, %percentage elongation, % percentage reduction in the area and factor of safety. | - Stress and strain, - Modulus of elasticity, - Relationship of %age elongation, - Relationship of %age reduction in area, - Factor of safety for simple machine parts. - Stress-Strain Diagrams Practical Activity: Perform an activity individually to perform tensile load test. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - UTM - Specimen for tensile load - Micrometer Screw gauge - Vernier-Caliper | Class Room and workshop |

| - Calculate Compressive load | Trainee will be able to: - Select the attachments for compressive load. - Select specimen as per machine manual description and capacity. - Calculate Dimensions of the specimen. - Mount specimen properly. - Perform compressive load test. - Prepare dimensions’ table with loaded and unloaded part. - Calculate Stresses, strain, modulus of elasticity, % percentage age elongation, % percentage reduction in area and factor of safety. | - Stress and strain, - Modulus of elasticity, - Relationship of %age elongation, - Relationship of %age reduction in area, - Factor of safety for simple machine parts. - Stress-Strain Diagrams Practical Activity: Perform an activity individually to perform Compressive load test. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - UTM - Specimen for tensile load - Micrometer Screw gauge - Vernier-Caliper | Class Room and workshop |
| --- | --- | --- | --- | --- | --- |
| - Calculate Shear load | Trainee must be able to: - Select attachments for shear load. - Select specimen as per machine manual description and capacity. - Calculate Dimensions of the specimen. - Mount specimen properly. - Perform shear load test. - Prepare dimensions table. - Calculate Stresses, strain, modulus of elasticity, %age elongation, %age reduction in area and factor of safety. | - Stress and strain, - Modulus of elasticity, - Relationship of %age elongation, - Relationship of %age reduction in area, - Factor of safety for simple machine parts. - Stress-Strain Diagrams Practical Activity: Perform an activity individually to perform Shear load test. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - UTM - Specimen for tensile load - Micrometer Screw gauge - Vernier-Caliper | Class Room and workshop |

### Calculate diameter of cylinder for hoop and longitudinal stresses

Objective: This competency standard covers the skills and knowledge required to Differentiate between thin and thick shells, Identify Hoop and longitudinal stress in cylindrical shells and Calculate Transverse and longitudinal failure of pressure vessel.

Duration:  24			Hours			Theory: 06	Hours				Practice: 	18	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Differentiate between thin and thick shells. | Trainee will be able to: - Select the work-piece - Calculate thickness and diameter - Apply the criteria of thin and thick. | - Cylindrical part. - Basic measurements - Thin and thick cylinders’ relationships. - Introduction of strain gauges and pressure gauges Practical Activity: Perform an activity individually to measure thickness and diameter of a cylinder. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Micrometer Screw gauge - Vernier-Caliper - Measuring Gauges - Pressure Gauges | Class Room and workshop |
| - Identify Hoop and longitudinal stress in cylindrical shells | Trainee will be able to: - Select thin wall pressure vessel - Apply air testing - Calculate the hoop and longitudinal stresses. | - Thin and thick cylinders’ relationships. - Hoop and longitudinal stress analysis - Relationship of hoop and longitudinal stresses - Introduction of strain gauges and pressure gauges - Use air compressor. Practical Activity: Perform an activity individually to identify hoop stress. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Micrometer Screw gauge - Vernier-Caliper - Measuring Gauges - Pressure Gauges - Air compressor with safety valve - Strain Gauges (for hoop and longitudinal strains and stresses) | Class Room and workshop |
| - Calculate Transverse and longitudinal failure of pressure vessel | Trainee must be able to: - Select thin wall pressure vessel - Apply air testing - Calculate the hoop and longitudinal - Apply failure criteria. | - Relationship of hoop and longitudinal stresses - Introduction of strain gauges and pressure gauges - Use air compressor. - Relationship of failure criteria for pressure vessel Practical Activity: Perform an activity individually to calculate Transverse and longitudinal failure. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Micrometer Screw gauge - Vernier-Caliper - Measuring Gauges - Pressure Gauges - Air compressor with safety valve - Strain Gauges (for hoop and longitudinal strains and stresses) | Class Room and workshop |

### Calculate thickness and diameter of spherical shell for circumferential stresses

Objective: This competency standard covers the skills and knowledge required to Calculate Lame’s equations for brittle materials and Calculate Different cases of thickness of thick shells of brittle material, Measure thickness of spherical shell for circumferential stresses and Measure diameter of spherical shell for circumferential stresses.

Duration:  16		Hours			Theory: 08	Hours				Practice: 	24	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Calculate thickness using Lame’s equation for brittle materials for given values. | Trainee will be able to: - Select pressure vessel part - Choose measurement tool - Measure Pressure and Diameter - Measure dimensions of the cylindrical part - Solve the distributed load profiles. - Identify lame’s equation - Identify given values for lame’s equation. - Calculate the thickness of tank. | - Thick cylinder derivations - Measuring techniques - Basic knowledge of stresses inside the thick wall - Understanding about Lame’s equation - Knowledge of geometric - Analyze Stresses in thick wall cylinder - d=200mm, P=10 N/mm2, =18 N/mm2. Practical Activity: Perform an activity individually to measure thickness and diameter of a cylinder. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Measuring tools (Micrometer Screw gauge, Vernier-Caliper Measuring Gauges) - Cylindrical part of thick cylinder of the brittle materials - Thick cylinder catalogues - Basic measuring tools - Formula catalogues | Class Room and workshop |
| - Calculate thickness using different cases of thick shells of brittle material | Trainee will be able to: - Select pressure vessel part - Choose measurement tool - Measure dimensions of the cylindrical part - Solve the distributed load profiles. - Measure Pressure and Diameter - Identify maximum shear stress equation - Identify given values for maximum shear stress equation. - Calculate the thickness of tank. | - Thick cylinder derivations - Measuring techniques - Basic knowledge of stresses inside the thick wall - Understanding about Maximum Shear stress equation - Knowledge of geometric - Analyze Stresses in thick wall cylinder - d=200mm, P=10N/mm2, =18 N/mm2. =/2 Practical Activity: Perform an activity individually to measure thickness of thick shells of brittle material. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Measuring tools (Micrometer Screw gauge, Vernier-Caliper Measuring Gauges) - Cylindrical part of thick cylinder of the brittle materials - Thick cylinder catalogues - Basic measuring tools - Formula catalogues | Class Room and workshop |
| - Measure thickness of spherical shell for circumferential stresses | Trainee must be able to: - Identify thick wall pressure vessel - Measure thickness of spherical part - Calculate Stresses on thick spherical shells. | - Thick cylinder derivations - Measuring techniques - Basic knowledge of stresses inside the thick wall - Understanding about Lame’s equation - Knowledge of geometric - Analyze Stresses in thick wall cylinder Practical Activity: Perform an activity individually to measure thickness of a sphere. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Measuring tools (Micrometer Screw gauge, Vernier-Caliper Measuring Gauges) - Cylindrical part of thick cylinder of the brittle materials - Thick cylinder catalogues - Basic measuring tools - Formula catalogues | Class Room and workshop |
| - Measure diameter of spherical shell for circumferential stresses | Trainee must be able to: - Identify thick wall pressure vessel - Measure diameter of spherical part - Calculate Stresses on inner side of spherical shells. | - Thick cylinder derivations - Measuring techniques - Basic knowledge of stresses inside the thick wall - Understanding about Lame’s equation - Knowledge of geometric - Analyze Stresses in thick wall cylinder Practical Activity: Perform an activity individually to measure diameter of spherical shell. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Measuring tools (Micrometer Screw gauge, Vernier-Caliper Measuring Gauges) - Cylindrical part of thick cylinder of the brittle materials - Thick cylinder catalogues - Basic measuring tools - Formula catalogues | Class Room and workshop |

### Design welded joints for transverse and parallel fillet under static and fatigue loading

Objective: This competency standard covers the skills and knowledge required to Produce transverse fillet weld, Produce parallel fillet weld, Calculate Tensile stresses in transverse fillet weld, Calculate Shear stresses in transverse fillet weld, Calculate Tensile stresses in parallel fillet weld, Calculate Shear stresses in parallel fillet weld, Calculate Transverse fillet weld under static loadings, Calculate Parallel fillet weld under fatigue loading, Calculate Parallel fillet weld under static loadings and Calculate Parallel fillet weld under fatigue loadings.

Duration:  40	Hours			Theory: 10	Hours				Practice: 	30	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Produce Transverse fillet weld. | Trainee will be able to: - Identify the weld types - Select Electrode and weld machine as per material requirement - Perform transverse fillet Weld - Observe tensile and shear stresses in the transverse fillet weld. | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to produce transverse fillet weld. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Produce Parallel fillet weld | Trainee will be able to: - Identify weld types correct as the above - Select Electrode, weld machine as per material requirement - Perform parallel fillet Weld - Observe tensile and shear stresses in transverse fillet weld. | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to produce Parallel fillet weld. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Calculate Tensile stresses in transverse fillet weld. | Trainee must be able to: - Identify weld types correct as the above - Select Electrode, weld machine as per material requirement - Perform transverse fillet Weld - Analyze tensile and transverse fillet weld | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to calculate Tensile stresses in transverse fillet weld. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Calculate Shear stresses in transverse fillet weld. | Trainee must be able to: - Identify weld types correct as the above - Select Electrode, weld machine as per material requirement - Perform transverse fillet Weld - Analyze shear stresses in transverse fillet weld. | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to calculate Shear stresses in transverse fillet weld. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Identify Tensile stresses in parallel fillet weld. | Trainee must be able to: - Identify weld types - Select Electrode, weld machine as per material requirement - Identify Tensile stresses parallel in fillet Weld. - Observe tensile stresses in parallel fillet weld. | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to identify Tensile stresses in parallel fillet weld. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Identify Shear stresses in parallel fillet weld | Trainee must be able to: - Identify weld types - Select Electrode, weld machine as per material requirement - Identify Shear stresses in parallel fillet weld - Observe shear stresses in parallel fillet weld. | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to identify Shear stresses in parallel fillet weld. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Identify transverse fillet weld under static loadings | Trainee must be able to: - Identify weld types correct as the above - Select Electrode, weld machine as per material requirement - Identify load types - Observe transverse fillet weld under static loadings. | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to Identify transverse fillet weld under static loadings . | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Identify parallel fillet weld under fatigue loading | Trainee must be able to: - Identify weld types correct as the above - Select Electrode, weld machine as per material requirement - Identify load types on parallel fillet Weld - Observe parallel fillet weld under fatigue loading | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to Identify parallel fillet weld under fatigue loading. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Identify parallel fillet weld under static loadings | Trainee must be able to: - Identify weld types correct as the above - Select Electrode, weld machine as per material requirement - Identify load types on parallel fillet weld - Observe parallel fillet weld under static loadings | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to Identify parallel fillet weld under static loadings. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |
| - Identify parallel fillet weld under fatigue loadings | Trainee must be able to: - Identify weld types correct as the above - Select Electrode, weld machine as per material requirement - Identify load types parallel fillet weld. - Observe parallel fillet weld under fatigue loadings | - Understanding of various welding types - Recognize material types as per requirements. - Knowledge about the current and voltage knowledge of welding transformers. - Familiar with the types of loads. Practical Activity: Perform an activity in group of three students to Identify parallel fillet weld under fatigue loadings. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Fixture and hand vice - Welding Safety Equipment - Chipping hammer - Power supply - Measuring Tools as per requirement. - Cutting equipment and accessories - Grinding equipment and accessories | Class Room and workshop |

### Calculate stresses due to initial tightening and external load on screws

Objective: this competency standard covers the skills and knowledge required to measure stress area of a screw, develop relation with core diameter and nominal diameter of a screw thread, perform initial tightening and its specific values and calculate different cases of external load raised by different bolts.

Duration:  20	Hours			Theory: 08	Hours				Practice: 	12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure Stress area of a screw. | Trainee will be able to: - Choose the bolt - Select bolt types w.r.t. threads categories - Compare the of Nut with bolt thread - Calculate stresses on the threaded part by using mathematical relationships - Compare stresses with given standards in tables. | - Difference between nut, bolts and Screws - Thread types - Calculations of the stresses inside the bolts - Weak areas of the screw - Measurements of the diameters core and nominal - Failure criteria of the screw w.r.t. diameters - Various types of loads - Analyzation of the stresses - Standard tables and graphs for the specific values - Types of bolts - Material of bolts. Practical Activity: Perform an activity in group of two students to measure Stress area of a screw. | Theory-3 Hr Practical-2 Hrs Total- 5 Hrs | - Measuring tools(Scales, Micrometer Screw Gauges) - Thread pitch gauge - Safety gloves - Nut bolt assembly - Torque wrench | Class Room and workshop |
| - Develop relation with core dia. and nominal dia. of a screw thread | Trainee will be able to: - Choose correct bolt. - Select bolt types w.r.t. threads categories - Measure core and nominal diameters of the bolt. - Develop mathematical relationships between the diameters. - Compare stresses with given standard tables. | - Thread types - Calculations of the stresses inside the bolts - Measurements of the diameters core and nominal - Analyzation of the stresses - Standard tables and graphs for the specific values - Types of bolts - Material of bolts. Practical Activity: Perform an activity in group of three students to produce transverse fillet weld. | Theory-3 Hr Practical-2 Hrs Total- 5 Hrs | - Measuring tools(Scales, Micrometer Screw Gauges) - Thread pitch gauge - Safety gloves - Nut bolt assembly - Torque wrench | Class Room and workshop |
| - Perform Initial tightening and its specific values | Trainee must be able to: - Measure tensile stresses due to stretching of the bolt correct as the above - Calculate torsional shear stress due to frictional resistance at the threads. - Identify shear stress across threads - Identify compressive or crushing stress on the threads - Identify bending stress | - Calculations of the stresses inside the bolts - Failure criteria of the screw - Weak areas of the screw - Failure criteria of the screw w.r.t. diameters - Various types of loads - Analyzation of the stresses - Torque theory and calibration - Understanding about different cases of external load - Types of bolts - Material of bolts. Practical Activity: Perform an activity in group of three students to produce transverse fillet weld. | Theory-2 Hr Practical-3 Hrs Total- 5 Hrs | - Measuring tools(Scales, Micrometer Screw Gauges) - Thread pitch gauge - Safety gloves - Nut bolt assembly - Torque wrench | Class Room and workshop |
| - Identify different types of external load raised by different bolts | Trainee must be able to: - Measure stresses due to stretching of the bolt correct as the above - Calculate torsional shear stress due to frictional resistance at the threads. - Identify shear stress across threads - Identify compressive or crushing stress on the threads - Identify bending stress - Perform same procedure for different cases. | - Calculations of the stresses inside the bolts - Failure criteria of the screw - Weak areas of the screw - Measurements of the diameters core and nominal - Various types of loads - Analyzation of the stresses - Torque theory and calibration - Understanding about different cases of external load - Types of bolts - Material of bolts. Practical Activity: Perform an activity in group of three students to produce transverse fillet weld. | Theory-2 Hr Practical-3 Hrs Total- 5 Hrs | - Measuring tools (Scales, Micrometer Screw Gauges) - Thread pitch gauge - Safety gloves - Nut bolt assembly - Torque wrench | Class Room and workshop |

### Design dimension of square and rectangular keys

Objective: This competency standard covers the skills and knowledge required to Recognise all types of sunk keys, Understand sizes of keys proportional to the shaft diameter, Select length of a sunk key for same material with shaft and equal strength with shaft, Check torque transmitted by rectangular and square keys against shearing as well as crushing and Calculate length of a sunk key when torque transmitted dia. of shaft, stress (shear & compressive) and width of key is given

Duration:  20	Hours			Theory: 05	Hours				Practice: 	15	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize all types of sunk keys. | Trainee will be able to: - Select any key - Measure thickness of the key - Measure width of the key - Identify the type of the key. | - Types of keys - Measurement techniques - Categories of the keys - Key dimensions Practical Activity: Perform an activity individually to recognize types of sunk keys. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Various types of sunk keys - Measuring instruments - Various slotted shafts - Various type of other keys - Measuring tools | Class Room and workshop |
| - Identify sizes of keys proportional to the shaft diameter | Trainee will be able to: - Select a key according to the shaft requirement. - Identify the type of the key - Perform shaft and key integration. | - Types of keys - Measurement techniques - Categories of the keys - Key dimensions - Shaft slot dimension - Coupling mechanism - Shaft key-way dimension - Assembly of both parts. Practical Activity: Perform an activity individually to assemble key and shaft. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Various types of sunk keys - Measuring instruments - Various slotted shafts - Various type of other keys - Measuring tools | Class Room and workshop |
| - Select length of a sunk key for same material with shaft and equal strength with shaft | Trainee must be able to: - Select sunk key - Measure thickness of the key - Measure width of the key - Measure width of the key-way inside the shaft - Measure Length of the key-way inside the shaft - Measure the depth of the key-way inside the shaft | - Types of keys - Measurement techniques - Categories of the keys - Key dimensions - Shaft slot dimension - Coupling mechanism - Shaft key-way dimension - Assembly of both parts. - Strength of Key material - Torque transmission - Shearing stresses, - Torsional stresses - Crushing stresses. Practical Activity: Perform an activity in group of two students to identify key and measure the depth of the key-way. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Various types of sunk keys - Measuring instruments - Various slotted shafts - Various type of other keys - Measuring tools - Tables and Graphs of key standards | Class Room and workshop |
| - Check torque transmitted by rectangular and square keys against shearing as well as crushing | Trainee must be able to: - Select rectangular key - Measure thickness of the key - Measure width of the key - Measure width of the key-way inside the shaft - Measure Length of the key-way inside the shaft - Measure the depth of the key-way inside the shaft - Apply torque as per given condition - Analyze the stresses at various different condition - Perform the same procedure for square key | - Types of keys - Measurement techniques - Categories of the keys - Key dimensions - Shaft slot dimension - Coupling mechanism - Shaft key-way dimension - Assembly of both parts. - Strength of Key material - Torque transmission - Shearing stresses, - Torsional stresses - Crushing stresses. Practical Activity: Perform an activity in group of two students to measure torque transmitted by rectangular and square keys. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Various types of sunk keys - Measuring instruments - Various slotted shafts - Various type of other keys - Measuring tools - Tachometer - Torque wrench | Class Room and workshop |
| - Calculate length of a sunk key when torque transmitted, diameter and width of key is given. | Trainee must be able to: - Select sunk key correct as the above - Measure thickness of the key - Measure width of the key - Measure width of the key-way inside the shaft - Measure Length of the key-way inside the shaft - Measure depth of the key-way inside the shaft - Apply torque as per given condition - Analyze the stresses on different condition - Calculate tangential forces inside the key - Apply failure criteria to find out the torsional, shear and compression stresses. | - Types of keys - Measurement techniques - Categories of the keys - Key dimensions - Shaft slot dimension - Coupling mechanism - Shaft key-way dimension - Assembly of both parts. - Strength of Key material - Torque transmission - Shearing stresses, - Torsional stresses - Crushing stresses. Practical Activity: Perform an activity in group of two students to calculate tangential forces inside the key. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Various types of sunk keys - Measuring instruments - Various slotted shafts - Various type of other keys - Measuring tools - Tachometer - Torque wrench | Class Room and workshop |

### Design shafts subjected to twisting moment

Objective: This competency standard covers the skills and knowledge required to understand twisting and bending moment on solid shaft, Perform torsion and bending equation for strength of shaft, calculate diameters of shaft under torsion when torque to be transmitted and torsional shear stress is given, calculate diameter of shafts subjected to combine bending and twisting moments Understand twisting moment and bending moment on hollow shaft, Knowledge of torsion and bending equation and Calculate dia. of hollow shaft (inside & outside dia.) when bending moment, twisting moment and stresses are given:

Duration:  28	Hours			Theory: 07	Hours				Practice: 	21	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform twisting and bending moment on solid shaft. | Trainee will be able to: - Select the required solid shaft - Measure dimensions of the shaft - Recognize the reaction forces which produce bending moment and twisting. | - Measurements (Diameter and length) of the solid shaft - Moment types. - Difference between the twisting moment and bending moment. - Twisting behavior of the shaft - Bending moment of the shaft - Stiffness of the solid shaft - Modulus of the elasticity of the solid shaft - Shearing moment of the shaft - Measure the diameters (Inner, outer or thickness.) - Twisting and bending. Practical Activity: Perform an activity individually to perform twisting of shaft. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Solid shaft - Measuring instruments - Safety equipment - Fixture and hand vice | Class Room and workshop |
| - Perform torsion and bending equation for strength of shaft | Trainee will be able to: - Select required solid shaft - Measure dimensions of the shaft - Measure the force to produce moment along the dia. - Calculate the reaction forces which produce bending moment. - Calculate the moment of inertia of shaft - Produce the relationship to find-out the strength of the shaft. | - Measurements (Diameter and length) of the solid shaft - Moment types. - Difference between the twisting moment and bending moment. - Twisting behavior of the shaft - Bending moment of the shaft - Stiffness of the solid shaft - Modulus of the elasticity of the solid shaft - Shearing moment of the shaft - Measure the diameters (Inner, outer or thickness.) - Twisting and bending. Practical Activity: Perform an activity individually to perform torsion and bending. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Solid shaft - Measuring instruments - Safety equipment - Fixture and hand vice | workshop |
| - Calculate diameters of shaft under torsion when torque to be transmitted and torsional shear stresses are given | Trainee must be able to: - Select required solid shaft - Identify the type of shaft - Measure dimensions of the shaft - Apply torque to produce torsion along the dia. - Calculate the reaction forces which produce shear moment. - Calculate the moment of inertia of the shaft - Produce the relationship to find-out the diameter of the shaft under torsional loading | - Measurements (Diameter and length) of the solid shaft - Moment types. - Difference between the twisting moment and bending moment. - Twisting behavior of the shaft - Bending moment of the shaft - Stiffness of the solid shaft - Modulus of the elasticity of the solid shaft - Shearing moment of the shaft - Measure the diameters (Inner, outer or thickness.) - Twisting and bending. Practical Activity: Perform an activity individually to measure diameter under applied torque. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Solid shaft - Measuring instruments - Safety equipment - Fixture and hand vice | Class Room and workshop |
| - Calculate diameter of shafts subjected to combine bending and twisting moments | Trainee must be able to: - Select required solid shaft - Identify the type of shaft - Measure dimensions of the shaft - Apply torque to produce torsion along the dia. - Calculate the reaction forces which produce shear moment. - Calculate the moment of inertia of the shaft - Produce the relationship to find-out the diameter of the shaft under torsional loading - Calculate the max. normal stress in the shaft - Calculate Min. normal stress in the shaft - Bending stress induce in the shaft in the bending moment - Calculate the shear stress induced in the shaft due to twisting moment - Calculate the equivalent bending moment - Calculate the max. shear stress | - Measurements (Diameter and length) of the solid shaft - Moment types. - Difference between the twisting moment and bending moment. - Twisting behavior of the shaft - Bending moment of the shaft - Stiffness of the solid shaft - Modulus of the elasticity of the solid shaft - Shearing moment of the shaft - Measure the diameters (Inner, outer or thickness.) - Twisting and bending. Practical Activity: Perform an activity individually to measure diameter under applied bending and twisting. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Solid shaft - Measuring instruments - Safety equipment - Fixture and hand vice | Class Room and workshop |
| - Perform twisting moment and bending moment on hollow shaft | Trainee must be able to: - Select required hollow shaft - Measure dimensions of the shaft - Fix it Fixture - Apply twisting - Apply bending - Observe bending and twisting moment of the hollow shaft. | - Measurements (Diameter and length) of the solid shaft - Moment types. - Difference between the twisting moment and bending moment. - Twisting behavior of the shaft - Bending moment of the shaft - Stiffness of the solid shaft - Modulus of the elasticity of the solid shaft - Shearing moment of the shaft - Measure the diameters (Inner, outer or thickness.) - Twisting and bending. Practical Activity: Perform an activity individually to observe bending and twisting moment of the hollow shaft. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Hollow shaft - Measuring instruments - Safety equipment - Fixture and hand vice | Class Room and workshop |
| - Calculate torsion and bending if diameter is given | Trainee must be able to: - Select required hollow shaft - Measure inner and outer diameter. - Identify equation - Put diameter values. - Observe bending and twisting moment of the hollow shafts. | - Measurements (Diameter and length) of the solid shaft - Moment types. - Difference between the twisting moment and bending moment. - Twisting behavior of the shaft - Bending moment of the shaft - Stiffness of the solid shaft - Modulus of the elasticity of the solid shaft - Shearing moment of the shaft - Measure the diameters (Inner, outer or thickness.) - Twisting and bending. Practical Activity: Perform an activity individually to measure form a bending equation. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Solid shaft - Measuring instruments - Safety equipment - Fixture and hand vice | Class Room and workshop |
| - Calculate dia. of hollow shaft (inside & outside dia.) when bending moment, twisting moment and stresses are given | Trainee must be able to: - Select required hollow shaft - Measure Torque - Measure the bending - Calculate inner dia - Calculate outer dia - Observe bending and twisting moment of the hollow shaft. | - Measurements (Diameter and length) of the solid shaft - Moment types. - Difference between the twisting moment and bending moment. - Twisting behavior of the shaft - Bending moment of the shaft - Stiffness of the solid shaft - Modulus of the elasticity of the solid shaft - Shearing moment of the shaft - Measure the diameters (Inner, outer or thickness.) - Twisting and bending. Practical Activity: Perform an activity individually to measure dimension of hollow shaft. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Hollow shaft - Measuring instruments - Safety equipment - Fixture and hand vice | Class Room and workshop |

### Design flange coupling for specific torque

Objective: This competency standard covers the skills and knowledge required to un-protected flange coupling, Knowledge empirical size of flange coupling and design assembly (hub, keys, flange and bolts) of unprotected type flange coupling:

Duration:  12	Hours			Theory: 03	Hours				Practice: 	09	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Un-protected flange coupling. | Trainee will be able to: - Identify the flange from the different machine parts like Hub, keys. Flywheel, bolts etc. - Differentiate various type of the flanges from Marine, Unprotected and Protected flange coupling | - Understanding of the machine parts - Measurement techniques - Knowledge about the sizes of flange - Measurement techniques - Knowledge about all the parts of the flange assembly - Knowing about the unprotected flange application. - Able to check the errors in fitting. Practical Activity: Perform an activity individually to Identify the flanges. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Flange - Bolts - Keys - Hubs - Flywheels - Measuring instruments - Leveling instruments | Class Room and workshop |
| - Identify empirical size of flange coupling | Trainee will be able to: - Choose the flange coupling - Measure diameter of the shaft - Measure outside diameter of the shaft - Measure Length of hub/ Effective length of key - Measure Thickness of output flange - Measure Pitch circular diameter - Measure Thickness of Protective rim - Measure Diameter of pin - Measure Number of pins. | - Understanding of the machine parts - Measurement techniques - Knowledge about the sizes of flange - Measurement techniques - Knowledge about all the parts of the flange assembly - Knowing about the unprotected flange application. - Able to check the errors in fitting. Practical Activity: Perform an activity individually to measure the dimensions of flanges. | Theory-1 Hr Practical-3 Hrs Total- 4 Hrs | - Flange - Bolts - Keys - Hubs - Flywheels - Measuring instruments - Leveling instruments | Class Room and workshop |
| - Design assembly (hub, keys, flange and bolts) of unprotected type flange coupling | Trainee must be able to: - Measure and select hub as per requirement - Identify keys and measure dimensions for required unprotected type flange coupling - Identify un-protected flange type - Select the bolts as per requirements - Assemble shaft inside the flange - Insert key and check fitting - Assemble bolt and tight nuts - Check the torque of the nut bolt assembly. | - Understanding of the machine parts - Measurement techniques - Knowledge about the sizes of flange - Measurement techniques - Knowledge about all the parts of the flange assembly. - Knowing about the unprotected flange application. - Able to check the errors in fitting. Practical Activity: Perform an activity individually to assemble flanges with system. | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | - Flange - Bolts - Keys - Hubs - Flywheels - Measuring instruments - Leveling instruments | Class Room and workshop |

## Industrial Process Control & Instrumentation

### Design & Implement a process control

Objective: This module covers the knowledge and  skills required to design an open loop process Control circuit, Design a closed loop process control circuit, Implement the ON/OFF Control Mode and Implement the PID Control Mode.

Duration:  		20	Hours			Theory: 08	Hours				Practice: 12		 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design an Open Loop Process Control Circuit | Trainee will be able to: - Specify the open loop system which is to be analyzed. - Select the type of the “Process variable” need to be measured, such as, pressure, temperature, level, flow, velocity, etc. - Specify the input to the system. - Select type of instrument which is required to measure the Process variable. - Construct the open-loop process control circuit. - Generate and display the output reading. | - Definition of Process Control. - Definition of Open loop Process Control System. - Industrial applications of open loop system by using different instruments. - Draw a block diagram of open loop process control system. - Definition of Process Variable. Practical Activity - Design an open loop control system for silo filling. | Theory-02 Hrs Practical-03 Hrs Total-05 Hrs | - Water Pump - Contactor - On/Off Switch - Wires/Cables - Small Scale Silo - Digital Multimeter - Electrical Tool kit - Power Source | Class Room and workshop |
| - Design a Closed Loop Process Control Circuit | Trainee will be able to: - Specify the close loop system which is to be analyzed. Repeat the same correction as above. - Select the type of process variable for the measurement, such as, pressure, temperature, level, flow, velocity, etc. - Specify the input to the system. - Select type of instrument for process variable measurement. - Construct the closed-loop process control circuit. - Generate and display the output reading. | - Definition of closed loop process control circuit. - Comparison of closed loop and open loop circuits. - Industrial applications of close loop system by using different instruments. - Block diagram of close loop process control system. Practical Activity - Design a close loop control system for silo filling system. | Theory-02 Hrs Practical-03 Hrs Total-05 Hrs | - Water Pump - Contactor - On/Off Switch - Wires/Cables - Small Scale Silo - Limit Switches - Digital Multimeter - Electrical Tool kit - Power Source | Class Room and workshop |
| - Implement the ON/OFF Control Mode | Trainee must be able to: - Specify the ON/OFF type of control mode to be installed Repeat the same correction as above. - Select the appropriate type of control mode. - Install the desired control mode type as per the installation Instructions. - Perform a test run on the ON/OFF control mode. - Generate the Output reading. | - Method of On/Off control mode. - Neutral Zone in On/Off Control Mode. - Industrial applications of On/Off control mode - Set Point and Process Variable - Pros & Cons of On/Off control Mode. Practical Activity - Design a domestic temperature control system. | Theory-02 Hrs Practical-03 Hrs Total-05 Hrs | - Thermostat - Heater - Contactor - On/Off Switch - Wires/Cables - Digital Multimeter - Electrical Tool kit - Power Source | Class Room and workshop |
| - Implement the PID Control Mode | Trainee must be able to - Specify the closed loop system which is to be analysed. Repeat the same correction as above. - Select the type of process variable for the measurement, such as, pressure, temperature, level, flow, velocity, etc. - Select the appropriate type of control mode. - Install the desired control mode type as per the installation instructions. - Perform a test run on the P control mode - Perform a test run on the PI control mode - Perform a test run on the PID control mode - Generate the Output report | - Definition of PID - Working principle of PID - PID Control Loop - Difference between PID & On/Off Control modes. - Advantages of PID Control Mode - Practical Industrial application - Purpose of P, PI and PD Control mode. - Implement PID and operate an actuator by controlling process variable Practical Activity - Implement PID and control the water pump speed (RPM) to maintain the outlet pressure. | Theory-02 Hrs Practical-03 Hrs Total-05 Hrs | - Water Pump - Pressure Transmitter - Contactor - On/Off Switch - Wires/Cables - Digital Multimeter - Electrical Tool kit - Power Source | Class Room and workshop |

### Install Transducer & Transmitters used in Industrial Applications

Objective: This module covers the knowledge and  skills required to Apply the P/I and I/P Transducers, Install the Differential Pressure Transmitters and install the Pneumatic transmitter in industry.

Duration:  		24	Hours			Theory: 06	Hours				Practice: 18		 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply the P/I and I/P Transducers | Trainee will be able to: - Specify the type of transducer to be installed - Install the desired type of transducer as per the installations manual. - Perform the post installation measures. - Generate an output report for the transducer. | - Working principle of I/P transducer. - Working principle of P/I transducer. - Applications of I/P and P/I Transducers in industry. - Installation Criteria of I/P and P/I Transducers. - Calibration of I/P & P/I Transducers. - Identification and planning of Tools/Equipment required for installation Practical Activity - Calibrate I/P & P/I Transducers | Theory-02 Hrs Practical-06 Hrs Total-08 Hrs | - I/P Transducer - P/I Transducer - I/P & P/I Calibrator - Wires/Cables - Digital Multimeter - Electrical Tool kit - Power Source | Class Room and workshop |
| - Install the Differential Pressure Transmitters | Trainee will be able to: - Analyze the type of DP Transmitter to be installed - Locate the point of installation for DP transmitter - Install the DP Transmitter as per the installations manual. - Perform Post Installation measures. - Generate an output report for the transmitter | - Definition of Differential Pressure. - Working principle of Orifice Plate. - How to find delta pressure? - Industrial applications for Differential Pressure Transmitter. - Types of Differential Pressure Transmitters. - Importance of differential pressure monitoring in industries. - Installation requirements of DP Transmitter. Practical Activity - Installation of DP Transmitters as per manual. | Theory-02 Hrs Practical-06 Hrs Total-08 Hrs | - Differential Pressure Transmitter - Wires/Cables - Digital Multimeter - Electrical Tool kit - Wrench set - Mechanical Tool Kit - Power Source | Class Room and workshop |
| - Install the Pneumatic Transmitter in Industry | Trainee must be able to: - Analyze the type of Pneumatic Transmitter to be installed. - Locate the point of installation for Pneumatic transmitter - Install the Pneumatic Transmitter as per the installations manual. - Perform Post Installation measures. - Generate an output report for the transmitter. | - Working principle of different pneumatic transmitters. - Identification of different kinds of Pneumatic Transmitters. - Selection of appropriate Pneumatic transmitter - Installation procedures of Pneumatic Transmitters. - Pneumatic piping - Understanding of P&ID Drawing - Calibration procedure of Pneumatic Transmitter. Practical Activity - Select and install a pneumatic transmitter in a given system | Theory-02 Hrs Practical-06 Hrs Total-08 Hrs | - Pneumatic Piping - Pneumatic Transmitter - P&ID Drawing - Wires/Cables - Digital Multimeter - Electrical Tool kit - Wrench set - Mechanical Tool Kit - Power Source | Class Room and workshop |

### Install the process Regulators

Objective: This module covers the knowledge and  skills required to install the Regulator and Flow Control Actuators in application, analyse the Power Controlling Devices and Install the Motor in Application

Duration:  		32	Hours			Theory: 08	Hours				Practice: 24		 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Install the regulator in application | Trainee will be able to: - Specify the type of regulator to be installed - Select the appropriate type of regulator - Install the desired regulator type as per the installation instructions - Perform a test run on the regulator installed - Generate the Output report | - Definition of Regulator - Types of Regulator - Working Principle of Regulator - Identification of regulator - Installation procedure for different types of regulators - How to read installation manual Practical Activity - Adjust the oil flow by selecting and installing appropriate regulator. | Theory-02 Hrs Practical-04 Hrs Total- 06 Hrs | - Oil Flow Regulator - Installation Manual - Wrench set - Mechanical Tool Kit | Class Room and workshop |
| - Install the Flow Control Actuators in application | Trainee will be able to: - Specify the type of actuator to be installed - Select the appropriate type of actuator - Install the desired actuator type as per the installation instructions - Perform a test run on the installed actuator - Generate the Output report | - Definition of Actuator - Types of Actuators - Working Principle of Flow Control Actuators. - Identification of different actuators provided by the instructor - Definition of Flow - Calibration techniques of Actuator - Installation method as per manual Practical Activity - Install air flow actuator of a fan to control its pressure outlet | Theory-02 Hrs Practical-04 Hrs Total- 06 Hrs | - Air Flow Actuator - Pressure Transmitter - Wrench set - Mechanical Tool Kit | Class Room and workshop |
| - Analyze the Power Controlling Devices | Trainee must be able to: - Specify the type of Power Control Device to be installed - Select the appropriate type of Power Control Device - Install the desired Power Control Device type as per the installation instructions - Perform a test run on the Power Control Device - Generate the Output report | - Explain Power Control Devices. - Types of Power Control Devices - Selection of Power Control Devices - Power Distribution circuit building - Power Control circuit building Practical Activity - Select appropriate Power Control Devices to run a motor as per motor kW. | Theory-03 Hrs Practical-06 Hrs Total-09 Hrs | - Circuit Breakers - Contactors - Relays - Overload Protection Relay - Fuses - Multimeter | Class Room and workshop |
| - Install the Motor in Application | Trainee must be able to: - Specify the type of motor to be installed - Select the appropriate type of motor - Install the desired motor type as per the installation instructions - Perform a test run on the motor - Generate the Output report | - Working Mechanism of Motor - Types of motors - Rotor & Stator identification and understanding - Commutator identification and understanding - Brush & Brushless motors differences. - Safeties of a motor in a power circuit - Control and Power Circuits to run a motor - Star & Delta Connection understanding Practical Activity - Install and control a three phase motor direction | Theory-03 Hrs Practical-06 Hrs Total-09 Hrs | - Three Phase Motor - Contactor - Circuit Breaker - Relay - Overload Protection Relay - Fuses - Wires/Cables - Power Source - Electrical Tool Kit - Mutimeter - Installation Manual - Wrench set - Mechanical Tool Kit | Class Room and workshop |

### @Install the Variable Frequency Drive (VFD)

Objective: This module covers the knowledge and  skills required to install the Variable Frequency Drive and perform the Parameterization process.

Duration:  		18	Hours			Theory: 06	Hours				Practice: 12		 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Install the Variable Frequency Drive | Trainee will be able to: - Select/specify the type of VFD to be installed - Select the appropriate type of VFD - Install the desired VFD type as per the installation instructions - Perform a test run on the VFD - Generate the output reports | - Definition of Frequency - Definition of Variable Frequency Drive - Types of VFD - Working principle of VFD - Importance of VFD - VFD Installation procedure - What is the criteria to select VFD Practical Activity - Select and install VFD as per the motor current rating. | Theory-03 Hrs Practical-06 Hrs Total- 09 Hrs | - Variable Frequency Drive - Installation Manual - Wrench set - Mechanical Tool Kit | Class Room and workshop |
| - Perform the Parameterization process | Trainee will be able to: - Specify the required parameter, such as, current, voltage, rpm. - Select the appropriate parameter - Perform parameter downloading - Tune the selected parameters with test run to adjust the output - Generate the output report. | - Select and unselect the parameters - Manual and Remote Mode - How to set the ranges for different parameters (such as; RMP, Voltage, Current, Process Value and Set Point) - Selection of Analog Signal format - Configuration of VFD, to communicate with PLC/Controller via Field Protocol BUS. Practical Activity - Configure the VFD as per the specification provided by the instructor. | Theory-03 Hrs Practical-06 Hrs Total- 09 Hrs | - Variable Frequency Drive - Installation Manual - Wrench set - Mechanical Tool Kit - Power Source - Programming Cable - Computer/Laptop - Programming Software | Class Room and workshop |

### Install Pneumatic & Hydraulic systems

Objective: This module covers the knowledge and  skills required to install hydraulic equipment, perform equipment maintenance and install pneumatic equipment.

Duration:  		24	Hours			Theory: 06	Hours				Practice: 	18	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Install Hydraulic Equipment | Trainee will be able to: - Specify the type of Hydraulic system to be installed, such as Jack, press, pump, etc - Select the appropriate type of hydraulic system - Install or mount the desired hydraulic type as per the installation instructions - Perform a test run on the hydraulic equipment - Generate the Output report | - What is Hydraulic system? - Basic components of Hydraulic system - Working principle of Hydraulic operation. - Types of Hydraulic system - Practical applications of Hydraulic system - Pre and Post Installation procedures. - System testing procedures Practical Activity - Install Hydraulic Jack | Theory-02 Hrs Practical-06 Hrs Total-08 Hrs | - Hydraulic Jack - Hydraulic fittings - Solenoid Valve - Pressure Gauge - Hydraulic Station (pump) - Hoses - Installation Manual - Wrench set - Mechanical Tool Kit | Class Room and workshop |
| - Perform equipment maintenance | Trainee will be able to: - Analyze the type of hydraulic maintenance required - Select the appropriate procedure of hydraulic maintenance - Inspect the Pressure Seal, Spring, Piston and oil level - Service the equipment as per the servicing manual | - Types of hydraulic maintenance - Importance of fluid selection, level, temperature and filtration. - Fluid Power principles - Things to monitor during preventive maintenance. - Cleaning and service of components - Importance of Oil and filter changes - Methods for checking pressure Practical Activity - Change the hydraulic filter | Theory-02 Hrs Practical-06 Hrs Total-08 Hrs | - Hydraulic Filter - Hydraulic fittings - Pressure Gauge - Hydraulic Station (pump) - Oil Level - Hoses - Service Manual - Hydraulic Oil - Hydraulic O Rings & Seals - Wrench set - Mechanical Tool Kit | Class Room and workshop |
| - Install Pneumatic Equipment | Trainee must be able to: - Specify the type of pneumatic system to be installed, such as valve, jack, cylinder, etc - Select the appropriate type of Pneumatic - Install the desired Pneumatic type equipment as per the installation instructions - Perform a test run on the Pneumatic equipment - Generate the Output report | - What is Pneumatic system? - Basic components of Pneumatic system - Working principle of Pneumatic operation. - Types of Pneumatic system - Practical applications of Pneumatic system - Working Mechanism of Pneumatic Valve - Pre and Post Installation procedures. - System testing procedures Practical Activity - Install and operate pneumatic paper cutting machine | Theory-02 Hrs Practical-06 Hrs Total-08 Hrs | - Solenoid Valve - Pressure Gauge - Pneumatic Cylinder - Air Compressor - Air Filter - Installation Manual - Mechanical Tool Kit | Class Room and workshop |

## Robotics

### Identify Various Parts of Robots

Objective: This competency standard covers the skills and knowledge required to Identify Various types of Sensors, Identify Various Controllers/ Microprocessors and Identify Designing and Development Software.

Duration:  24			Hours			Theory: 06	Hours				Practice: 	18	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Various types of Sensors | Trainee will be able to: - Identify temperature sensors. - Identify sound sensors. - Identify proximity sensors. - Identify pressure sensors. - Identify light sensors. - Identify position sensors. - Identify voltage sensors. - Identify current sensors. - Identify the vision sensors. - Identify power supply of each sensor | - Robot Introduction - Difference between Machine and robot - Description of robot Parts. - function of Following sensors: (Temperature, sound, proximity, pressure, light, position, voltage, current and vision sensors). - Power Supply Functions Practical Activity: Perform an activity individually and identify the type of sensors. | Theory-2 Hrs Practical-6Hrs Total- 8 Hrs | Different types following of Sensors: - Temperature. - Sound - Proximity - Light Sensors - Position - Voltage - Current - Vision - Power supply for each sensor | Class Room and workshop |

| - Identify Various Controllers/ Microprocessors | Trainee will be able to: - Identify the open loop control systems. - Identify the closed loop control systems. - Recognize the algorithms for both control systems. - Recognize the components of both control systems. - Draw circuit diagram for both control systems. - Draw the block diagram and flow charts of both control systems. - Recognize the power supply for control systems. | - Definition of open and close loop systems. - Algorithms for open and close loop systems. - Draw diagram for open and close loop systems. - Robot programing languages - Program development architecture - Robot software tools - Power Supply Functions Practical Activity: Perform an activity in group of three students to identify loops and recognized the components of loops. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | - Manipulator links and joints - Measuring tool kit - Trainer - Controllers - Mounting accessories - End Effectors - Mounting accessories - Actuators - Multi-meter - Electrical Wiring | Class Room and workshop |
| --- | --- | --- | --- | --- | --- |
| - Identify Designing and Development Software. | Trainee must be able to: - Identify the programming languages. - Identify the program development architecture for the robot. - Identify the software tools for the different types of controllers. | - Robot programing languages - Program development architecture - Robot software tools Practical Activity: Perform an activity individually to identify the software and architecture. | Theory-2 Hrs Practical-6Hrs Total- 8 Hrs | - Computer System - Software - Base for the manipulator - Electrical Wiring | Class Room and workshop |

### Identify Various Parts of Robot Arm

Objective: This competency standard covers the skills and knowledge required to Assemble the different types of Manipulators or Rovers, Set up Various Types of End Effectors with the Robot and Assemble Various Actuators / Drives with the Robot.

Duration: 24 			Hours			Theory: 06	Hours				Practice: 	18	 Hours 2.4

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Assemble the different types of Manipulators or Rovers. | Trainee will be able to: - Identify parts and main body and of the manipulator. - Identify the links and joints. - Assemble links, joints and overall assembly of the manipulator. - Identify the kinematics (movement) of a manipulator. | - Robot arm Introduction - Degrees of freedom - Explanation of robot arm links. - Description of robot arm (linear, rotary, both) joints. - Robot assembly - Kinematic diagram of manipulator Practical Activity: Perform an activity in group of three students to identify the parts, main body, manipulator links and joints | Theory-2 Hrs Practical-6Hrs Total- 8 Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit. - Trainer - Manipulator links and joints - End Effectors - Mounting accessories - Actuators - Multi-meter | Class Room and workshop |
| - Set up Various Types of End Effectors with the Robot | Trainee will be able to: - Identify the types of End Effectors. - Select the End Effectors as given per requirements. - Assemble the End Effectors with the manipulator. - Identify Drives for the End Effector. - Identify the kinematics (movement) of the end effectors. | - Kinematic diagram of manipulator - Explanation of end effector and its types. - Difference between manipulator and end effector - Description of end effector drives Practical Activity: - Perform an activity individually to identify types of End Effectors and its drives. | Theory-2 Hrs Practical-6Hrs Total- 8 Hrs | - Manipulator links and joints - Measuring tool kit - Trainer - Mounting accessories - End Effectors - Drives for end effector - Mounting accessories - Actuators - Multi-meter - Electrical Wiring | Class Room and workshop |
| - Assemble Various Actuators / Drives with the Robot. | Trainee must be able to: - Identify Hydraulic Actuators. - Identify Electrical Actuators. - Identify Pneumatic Actuators. - Identify Mechanical Actuators. - Identify Power Drives. - Identify the input power for each actuator. - Assemble actuators with manipulators and end effectors. | - Robot assembly - Actuator Introduction and its types (Hydraulic Actuators, Electrical Actuators, Pneumatic Actuators, Mechanical Actuators.) Practical Activity: Perform an activity individually to identify the actuators and drives | Theory-2 Hrs Practical-6Hrs Total- 8 Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit. - Trainer - End Effectors - Mounting accessories - Actuators - Base for the manipulator | Class Room and workshop |

### Develop Robot program

Objective: This competency standard covers the skills and knowledge required to Develop program to interface temperature sensor LM35 with Arduino, Develop program to operate DC motor with PWM based speed control, Develop program to operate SONAR and measure distance with Arduino.

Duration:  	27		Hours			Theory: 09	Hours				Practice: 	18	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop program to interface temperature sensor LM35 with Arduino. | Trainee will be able to: - Identify the pin configuration of Arduino and LM 35 temperature sensor - Make connection of power, ground and analog pin of LM 35 with Arduino - Connect Arduino with computer(PC) and open Arduino IDE and select port and Devices - Write and upload the code of LM35 into Arduino - Observe the value of temperature in serial monitor Screen in PC - Measure the temperature of ice and calibrate reading by tuning in programming. | - Introduction of Robot program - Explanation of hardware required for Robot program - Definition of interfacing - Configuration - Definition of controller. - Introduction of pins (Analog and logical) - Explanation of LM39, - Definition of Robot code - Introduction of PWM signal Practical Activity: Perform an activity in group of two students to interface LM35 sensor with Arduino and observes values. | Theory-3 Hrs Practical-6 Hrs Total- 9 Hrs | - Trainer - Arduino Mega - LM35 - Computer system - Complete design of the Robot - Robotic fastening tool kit | Class Room and workshop |
| - Develop program to operate DC motor with PWM based speed control | Trainee will be able to: - Identify the components and connection of circuits - Connect the potentiometer to analog pin and connect the MOSFET to digital Pin of Arduino - Connect the DC motor with separate Power Supply unit with MOSFET as switch - Write code to take value of voltage and make switch case statements to generate PWM signal of multiple Duty cycle - Upload the code and run DC motor. | - Configuration - Definition of MOSFET. - Introduction of pins (Analog and logical) - Explanation of DC Motor, - Definition of Robot code - Introduction of PWM signal Practical Activity: Perform an activity in group of two students to interface DC Motor with Arduino and observes Speed. | Theory-3 Hrs Practical-6 Hrs Total- 9 Hrs | - Trainer - Arduino Mega - DC Motor - MOSFET - Computer system - Measuring tool kit - Basic Robotic components - Complete design of the Robot - Robotic fastening tool kit | Class Room and workshop |
| - Develop program to operate SONAR and measure distance with Arduino. | Trainee must be able to: - Identify the Pin configuration of Sonar and Arduino - Make connection according to circuit diagram - Connect Arduino with PC and open Arduino IDE - Write and upload the code in Arduino - Run the code and measure value at serial monitor - Calibrate measure value of distance with Scale. | - PIN Configuration - Definition of controller. - Introduction of pins (Analog and logical) - Explanation of SONAR, Practical Activity: Perform an activity in group of two students to interface SONAR with Arduino and observes Distance. | Theory-3 Hrs Practical-6 Hrs Total- 9 Hrs | - Trainer - Arduino Mega - SONAR - Computer system - Measuring tool kit - Basic Robotic components - Complete design of the Robot - Robotic fastening tool kit | Class Room and workshop |

### Develop robotic arm structure

Objective: This competency standard covers the skills and knowledge required to Design the Robotic Arm and Develop the structure of Robotic Arm.

Duration:  	22		Hours			Theory: 04	Hours				Practice: 	18	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design the Robotic Arm. | Trainee will be able to: - Identify the types of robotic arms. - Select the links and joints type for robotic arm. - Plan the axes of motion (rotation and straight line movement) of robotic arm. - Specify the degrees of freedom for end effector. - Specify the reach envelope for robotic arm. - Identify the speed and payload for robotic arm. - Design the model of links, joints and overall assembly of the manipulator. | - Introduction of Robotic arm - Types of Robotic arm. - Definition of plan axes of motion - Definition of links and joints - Description of degrees of freedom - Definition of reach envelope of robotic arm - Flow chart of manipulator assembly - Description of Manipulators components. - Difference between End Effectors and the manipulator - Power distribution - Definition of forward kinematics - Definition of inverse kinematics. Practical Activity: Perform an activity to design a model of links, joints and overall assembly of the manipulator. | Theory-2 Hrs Practical-9 Hrs Total- 11 Hrs | - Manipulator links and joints - Measuring tool kit - Trainer - Manipulator links and joints - Computer system - Mounting accessories | Class Room and workshop |
| - Develop the structure of Robotic Arm | Trainee will be able to: - Assembly of the parts of manipulators. - Assembly of the End Effectors with the manipulator. - Assemble actuators with manipulators and end effectors. - Assemble Controller with actuators and power supply | - Assembly and Configuration of robot - Definition of plan axes of motion - Definition of links and joints - Description of degrees of freedom - Definition of reach envelope of robotic arm - Flow chart of manipulator assembly - Power distribution Practical Activity: Perform an activity in group of three students and assemble a manipulator | Theory-2 Hrs Practical-9 Hrs Total- 11 Hrs | - Manipulator links and joints KIT - Actuators - Controllers - Measuring tool kit - Robot Kit - Robotic fastening tool kit. - Trainer - Manipulator links and joints - Computer system - Mounting accessories - Complete design of the sensor assembly | Class Room and workshop |

### Test the Robot

Objective: This competency standard covers the skills and knowledge required to Configure the Robotic Arm and Test the Robotic Arm.

Duration:  22			Hours			Theory: 04	Hours				Practice: 	18	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Configure the Robotic Arm. | Trainee will be able to: - Identify of Working space. - Identify of Controller. - Identify of Manipulator. - Identify of End Effector. - Identify of Actuators. - Identify of Joints. - Identify of Software. - Identify of power supply. - Identify of connecting wiring. - Identify the components (Resistor, Capacitor and Potentiometer). - Draw the circuit diagram. - Develop the algorithm flowchart to implement on controller. - Identify the power distribution. | - Work space introduction. - Robot controller - Manipulator - End effector - Actuator - Joints - Software - power supply - Wiring diagram - Flow chart - Power distribution - Forward kinematics - Inverse kinematics - Explanation of movement detection test - Robotic arm System feasibility - Manipulability for end effectors. Practical Activity: Perform an activity individually to Draw/check circuit diagram and develop flowchart. | Theory-2 Hrs Practical-9 Hrs Total- 11 Hrs | - Manipulator links and joints - Electronic Components - End effectors - Measuring tool kit - Robotic Kit - Robotic fastening tool kit. - Trainer - Actuator - Computer system - Mounting accessories - Complete design of the sensor assembly | Class Room and workshop |
| - Test the Robotic Arm | Trainee will be able to: - Identify movement of robotic arm. - Perform the arm movement detection test - Check the robotic arm System feasibility and manipulability for end effectors. - Check of Robotic arm operating conditions. - Check all the connections of the components. - Recognize the program algorithm for the robotic arm. | - Work space introduction. - Robot controller - Manipulator - End effector - Actuator - Joints - Software - power supply - Wiring diagram - Flow chart - Power distribution - Forward kinematics - Inverse kinematics - Explanation of movement detection test - Robotic arm System feasibility - Manipulability for end effectors. Practical Activity: Perform an activity individually to check the operating condition of robotic arm and workspace | Theory-2 Hrs Practical-9 Hrs Total- 11 Hrs | - Measuring tool kit - Robotic Kit - Trainer - Software - Actuator - Manipulator links and joints - Computer system - Mounting accessories - Complete design of the sensor assembly | Class Room and workshop |

## Mechatronic Projects

### Access Control System using RFID Reader RC522

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  14			Hours			Theory: 05	Hours				Practice: 	09	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop an access control system using RFID reader RC522 | Trainee will be able to: - Identify Components of system. - Identify Components ratings. - Identify the reader to read the RFID (Radio Frequency Identification) - Sketch the model and overall assembly of the system. - Select the Controller - Recognize the power supply distribution for controller and components. - Assemble the parts of system - Draw the circuit diagram - Electric wiring of sensors, switches and controller - Design the algorithm flowchart to implement on controller. - Program the controller to read a key from RFID to Allow or Block the Access. - Upload code/Program to controller. - RUN the controller to read key from RFID. | - Project Introduction - Definition of access control system. - Description of required components. - function of RFID reader. - Power Supply Functions | Theory-04 Hrs Practical-26 Hrs Total- 30 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - RFID reader - RFID Writer - Computer system - Different types of switches - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Finger Print Sensor Based Biometric Attendance System

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  13			Hours			Theory: 04	Hours				Practice: 	09	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop Finger Print Sensor Based Biometric Attendance System | Trainee will be able to: - Identify Components of system. - Identify Components ratings. - Sketch the model and overall assembly of the system. - Select the Controller - Recognize the power supply distribution for controller and components. - Assemble the parts of system - Draw the circuit diagram - Connect the sensors, switches and controller - Design the algorithm flowchart to implement on controller. - Program the controller to scan the fingerprints and save in database. - Upload code/Program to controller. - RUN the controller to scan the fingerprints. | - Project Introduction - Description of finger print sensor. - Definition of attendance system. - Description of required components. - Power Supply Functions | Theory-04 Hrs Practical-09 Hrs Total- 13 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Finger print Sensor - Computer system - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Humidity and Temperature Monitoring System

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  12			Hours			Theory: 03	Hours				Practice: 	09	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop a Humidity and Temperature Monitoring System | Trainee will be able to: - Identify Components of system. - Identify Components ratings. - Identify Sensors and their ratings. - Sketch the model and overall assembly of the system. - Select the Controller - Recognize the power supply distribution for controller, components and sensors. - Assemble the parts of system - Draw the circuit diagram - Connect the sensors, switches and controller - Design the algorithm flowchart to implement on controller. - Program the controller to read the values form sensors. - Upload code/Program to controller. - RUN the controller to read the values. - Display the values on LCD | - Project Introduction - Definition of system. - Description of required components. - function of Humidity and Temperature sensor. - Power Supply Functions | Theory-03 Hrs Practical-09 Hrs Total- 12 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Humidity Sensor - Temperature Sensor - LCD - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Smart Stick for Blinds

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  13			Hours			Theory: 04	Hours				Practice: 	09	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop Smart Stick for Blinds | Trainee will be able to: - Identify Components of system. - Identify Components ratings. - Sketch the model and overall assembly of the system. - Select the Controller - Recognize the power supply distribution for controller, components and sensors. - Assemble the parts of system - Draw the circuit diagram - Connect the sensors, switches and controller - Design the algorithm flowchart to implement on controller. - Program the controller to read the values form sensors. - Upload code/Program to controller. - RUN the controller to read the values. | - Project Introduction - Definition of system. - Description of required components. - function of smart system. - Power Supply Functions | Theory-04 Hrs Practical-09 Hrs Total- 13 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Stick - Speaker/Beep - Touch Sensor - Force Sensor - Ultrasonic sensor - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Perform Line Following Robot

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  14			Hours			Theory: 05	Hours				Practice: 	09	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop a Line Following Robot | Trainee will be able to: - Specify the types of line following robot. - Identify mobile base of line following robot. - Identify the actuators for of line following robot. - Select the components for line following robot. - Identify the sensors to detect the path. - Design the model and overall assembly of the line following robot. - Assembly of the parts of line following robot - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for controller and actuator. - Recognize the program algorithm for the line following robot - Program the controller to change direction of robot according to the receive signal. - Identification of movement of the line following robot. - Preform the robot movement detection test - Check the line following robot feasibility and manipulability. - Check of line following robot operating conditions - Check the condition of each component and overall assembly of the line following robot. - Check all the connections of the components. . | - Project Introduction - Definition of system. - Description of required components. - function of line following system. - Power Supply Functions | Theory-05 Hrs Practical-09 Hrs Total- 14 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Color sensor - Robot car - Black tape - Ultrasonic sensor - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Perform Smart Phone Control Robot Using Bluetooth

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  14			Hours			Theory: 05	Hours				Practice: 	09	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop Smart Phone Control Robot Using Bluetooth | Trainee will be able to: - Specify the types of Smart Phone Control Robot Using Bluetooth. - Identify mobile base. - Identify the actuators. - Select the components. - Identify the sensors to detect the Bluetooth signals. - Design the model and overall assembly of the robot. - Assembly of the parts - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for controller and actuator. - Recognize the program algorithm for the Smart Phone Control Robot Using Bluetooth. - Program the controller to change direction of robot according to the receive signal from smart phone android application via Bluetooth. - Download and Install the android application on smartphone. - Connect the Bluetooth with Bluetooth module on controller. - Identification of movement of the Smart Phone Control Robot Using Bluetooth. - Preform the robot movement detection test - Check the robot feasibility and manipulability. - Check the robot all possible operating conditions. - Check the Bluetooth range and its connectivity. - Check the condition of each component and overall assembly of the robot. - Check all the connections of the components. | - Project Introduction - Definition of system. - Description of required components. - function of smart control. - Function of bluetooth - Power Supply Functions | Theory-06 Hrs Practical-30 Hrs Total- 36 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Bluetooth sensor - Robot car - Ultrasonic sensor - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Perform IoT Controlled based home automation system

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  20			Hours			Theory: 08	Hours				Practice: 	12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop IoT Controlled based home automation system | Trainee will be able to: - Specify the types of IoT Controlled based home automation system. - Identify the specification of home appliances and things to automate. - Select the components for each part of the automation system. - Identify the sensors to detect light, movement, temperature and other. - Perform the Internet module connection with controller. - Connect the LCD display with controller. - Design the model and overall assembly on single control unit. - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller. - Design the algorithm flowchart to implement on controller for each sensor. - Recognize the power supply distribution for sensors, controller and actuator. - Recognize the program algorithm for each sensor. - Setup the cloud for getting the sensors data on cloud. - Program the controller to automate the door, turn light on/off from smart phone, control the Fan from smart phone and display the temperature and humidity on screen. - Download and Install the android application on smartphone. - Connect the Internet with Internet module on controller. - Check the robot feasibility. - Check of line robot operating conditions - Check the data acquisition accuracy. | - Project Introduction - Definition of system. - Description of required components. - function of home automation. - Power Supply Functions | Theory-08 Hrs Practical-12 Hrs Total- 20 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Relays - Wifi Shield - LCD - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Perform Smart Phone Control Door Lock System

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  18			Hours			Theory: 06	Hours				Practice: 	12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop a Smart Phone Control Door Lock System | Trainee will be able to: - Specify the types Of Smart Phone Control Door Lock System. - Identify the specification of door lock. - Select the components. - Identify the actuators. - Identify the module to detect the Bluetooth signals. - Perform the Bluetooth module connection with controller. - Connect the LCD display with controller. - Design the model and overall assembly on single control unit. - Select the Controller - Draw the circuit diagram. - Electric wiring of sensors, actuator and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for sensors, controller and actuator. - Program the Algorithm for controller to unlock the door according to the received signals. - Download and Install the android application on smartphone. - Connect the Bluetooth with Bluetooth module on controller. - Identification of door activity according to pin code provide to unlock the door. - Preform the Door Lock System error detection test. - Check the robot feasibility and manipulability. - Check the Door Lock System all possible operating conditions. - Check the Bluetooth range and its connectivity. - Check the condition of each component. - Check all the connections and wiring of the components. | - Project Introduction - Definition of system. - Description of required components. - function of door lock system. - Power Supply Functions | Theory-06 Hrs Practical-30 Hrs Total- 36 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Relays - Wifi Shield - Bluetooth sensor - LCD - Door lock - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Perform Security System Using Motion Detection

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  18			Hours			Theory: 06	Hours				Practice: 	12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop a Security System Using Motion Detection | Trainee will be able to: - Specify the types of Security System Using Motion Detection. - Identify the specification of Security System. - Select the components. - Identify the actuators. - Identify the sensor to detect the Motion. - Select the Controller. - Design the model and overall assembly on single control unit. - Draw the circuit diagram. - Electric wiring of sensors, actuator and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for sensors, controller and actuator. - Program the Algorithm to detect the unwanted motion and ring buzzer when detected. - Preform the movement detection test on the Security System. - Check the robot feasibility and manipulability. - Check the Security System for all possible operating conditions. - Check the buzzer range. - Check the condition of each component. - Check all the connections and wiring of the components. | - Project Introduction - Definition of system. - Description of required components. - function of motion detector. - Power Supply Functions | Theory-06 Hrs Practical-12 Hrs Total- 18 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Relays - Motion detection sensor - Bluetooth sensor - LCD - Door lock - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Perform Water-level Detection in Tank

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  18			Hours			Theory: 06	Hours				Practice: 	12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop a Water-level Detection system for Tank | Trainee will be able to: - Specify the types of Water-level Detection in Tank. - Identify the specification of Water-level Detection Tank. - Select the components. - Identify the actuators. - Identify the sensor to detect the Water-level. - Select the Controller. - Design the model and overall assembly on single control unit. - Draw the circuit diagram. - Electric wiring of sensors, actuator and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for sensors, controller and actuator. - Program the Algorithm to detect the Water-level in tank and display reading on the LCD. - Preform the Water-level Detection test on different ranges of sensor with varying the level of water. - Check the Water-level Detection System feasibility and manipulability. - Check the Water-level Detection System for all possible operating conditions. - Check the sensor range. - Check the condition of each component. - Check all the connections and wiring of the components. | - Project Introduction - Definition of system. - Description of required components. - function of water level sesnor. - Power Supply Functions | Theory-06 Hrs Practical-12 Hrs Total- 18 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Relays - Level sensor - Bluetooth sensor - LCD - Door lock - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Perform Mini CNC 2D Plotter

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  17			Hours			Theory: 05	Hours				Practice: 	12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop a Mini CNC 2D Plotter | Trainee will be able to: - Specify the types of 2D CNC Plotter. - Identify static base of 2D CNC Plotter. - Identify the actuators. - Select the components. - Identify the sensors. - Design the model and overall assembly of the robot. - Assemble the parts - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for controller and actuator. - Recognize the program algorithm for the 2D CNC Plotter. - Program the controller to trace the input image on the page. - Identification of movement of the 2D CNC Plotter. - Preform the robot movement detection test - Check the 2D CNC Plotter feasibility and manipulability. - Check of 2D CNC Plotter operating conditions. - Check the condition of each component and overall assembly of the 2D CNC Plotter. - Check all the connections of the components. | - Project Introduction - Definition of system. - Description of required components. - function of plotter. - Power Supply Functions | Theory-05 Hrs Practical-12 Hrs Total- 17 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Keypad - Microcontroller - Relays - Level sensor - Bluetooth sensor - LCD - Door lock - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

### Perform Voice Controlled Robot

Objective: This Project is designed to introduce the student to some application of robotics in industry.

Duration:  19			Hours			Theory: 07	Hours				Practice: 	12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Project: to develop a Voice Controlled Robot | Trainee will be able to: - Specify the types of Voice Controlled Robot. - Identify mobile base of robot. - Identify the actuators for the robot. - Select the components for robot. - Identify the sensors to recognize the voice instructions. - Design the model and overall assembly of the robot. - Assemble of the parts of line following robot - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for controller and actuator. - Recognize the program algorithm for the Voice Controlled Robot. - Program the controller to change direction of robot according to the voice command. - Identification of movement of the Voice Controlled Robot. - Preform the robot movement detection test - Check the robot feasibility and manipulability. - Check of line robot operating conditions - Check the condition of each component and overall assembly of the robot. - Check all the connections of the components. | - Project Introduction - Definition of system. - Description of required components. - function of voice sensor. - Power Supply Functions | Theory-07 Hrs Practical-12 Hrs Total- 19 Hrs | - Soldering Air Station. - Soldering Sucker. - Screw driver set - Microcontroller - Voice sensor - Bluetooth sensor - LCD - Computer System - Mounting accessories - Complete design and assembly drawing - Basic Electronics components - Software - Sensors - Wiring and accessories | workshop |

## Hydraulics & Pneumatics

### Operate Hydraulic Bench and Its Functions

Objective: This module covers the knowledge and skills required to identify and operator Hydraulic Bench with various functions to acquire hydraulic measurements.

Duration:  		16 Hours			Theory:      04 Hours				Practice: 	12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify parts of hydraulic bench | Trainee will be able to: - Enlist all parts of a Hydraulic Bench - Identify the Control valve, Drain valve and Dump valve. - Identify the Sump Tank and Volumetric Tank. - Identify the Open Channel and Vertical Pipe. - Identify the Centrifugal Pump - Identify the Actuators | - Purpose of hydraulic press - Working of hydraulic press - Knowledge of centrifugal pump - Understanding of control valves and its types - Knowledge of sump tank and volumetric tank - Knowledge of channels and its types - Understanding of actuators and its types - Understanding of hydraulic diagram - Knowledge of level indicator, Stilling baffle, overflow and starter PRACTICAL - Identify the parts of hydraulic press as per given criteria | Theory-02 Hrs Practical-06 Hrs Total- 08 Hrs | - Hydraulic bench - Control Valve - Centrifugal pump - Sump tank - Vertical pipe - Channel - Drain valve - Volumetric tank - Water Level Indicator - Dump valve - Actuators | Class Room and workshop |
| - Identify/perform Various Functions of/on Hydraulic Bench | Trainee will be able to: - Ensure the pump is switched off. - Set up the device to be tested. - Attach a water supply for the open channel flow or a close conduct device to the water inlet. - Ensure that all connections are secure. - Ensure that supply valve is closed, and dump valve is open. - Turn ON the power of the pump - Open the valve slowly to allow the water to circulate the bench. - Make an appropriate measurement for the testing device for the recommended setting of the supply valve, e.g. pressure reading, water depth etc. - Close the supply of the valve after completion of the test. - Disconnect the device which was used for testing. | - Pump Operation - Pump circuit - Hydraulic & Electrical connection of bench - Pressure adjustment Procedure - Safe SOP for hydraulic bench - Hydraulic Testing procedure - working of level indicator, Stilling baffle, overflow and starter - Working of valves - Working of actuators PRACTICAL - Measure the input or output flow measurement of testing equipment - Monitor the water leakage in an equipment with the help of hydraulic bench | Theory-02 Hrs Practical-06 Hrs Total- 08 Hrs | - Hydraulic bench - Control Valve - Centrifugal pump - Sump tank - Vertical pipe - Channel - Drain valve - Volumetric tank - Water Level Indicator - Dump valve - Actuators | Class Room and workshop |

### Calibrate Bourdon Tube and Diaphragm Pressure Gauge

Objective: This module covers the knowledge and skills required to record the working principle of mechanical Pressure gauges and incorporate the Calibration Methods for Mechanical Gauges.

Duration:  		08 Hours			Theory:      02 Hours				Practice: 	06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Calibrate Bourdon Tube using dead weight Pressure gauge and master gauge Calibrator | Trainee will be able to: - Connect the Bourdon Tube with the dead weight pressure gauge. - Fill the cylinder of dead weight pressure gauge with water. - Place a known weight on the plunger of the dead weight pressure gauge. - Note the pressure reading of the gauge to be calibrated. - Add more weights of the known values and note corresponding pressure readings. - Compare the readings with the values provided in the table. - Compare the dead weight gauge with the master gauge. | - Working principle of Bourdon Tube - Knowledge of dead weight calibrator - Understanding of Pascal law - Verification of Pascal law - Calibration procedure of bourdon tube - Relationship of weight and pressure PRACTICAL - Calibrate the bourdon tube with help of dead weight calibrator | Theory-01 Hrs Practical-03Hrs Total- 04 Hrs | - Deadweight gauge Tester - Weights - Master Gauge Calibrator - Bourdon Tube - Hydraulic tubing - Hydraulic fittings | Class Room and workshop |
| - Calibrate Diaphragm Pressure Gauge using dead weight pressure gauge and master gauge Calibrator | Trainee will be able to: - Connect the Diaphragm Pressure Gauge with the dead weight pressure gauge. - Fill the cylinder of dead weight pressure gauge with water. - Place a known weight on the plunger of the dead weight pressure gauge. - Note the pressure reading of the gauge which is needed to be calibrated. - Add more weights of known values and note corresponding pressure readings. - Compare these readings with the values provided in the table. - Compare the dead weight gauge with the master gauge. | - Knowledge of Diaphragm - Know of Diaphragm pressure gauge - Calibration procedure of Diaphragm pressure gauge - Application of diaphragm for pressure testing - comparison procedure of Master gauge with test gauge PRACTICAL - Calibrate the Diaphragm with help of dead weight calibrator | Theory-01 Hrs Practical-03Hrs Total- 04 Hrs | - Deadweight gauge Tester - Weights - Master Gauge Calibrator - Diaphragm Pressure Gauge - Hydraulic tubing - Hydraulic fittings | Class Room and workshop |

### Operate Hydraulic Press

Objective: This module covers the knowledge and skills required to operate a hydraulic press and observe power required to drive it.

Duration:  		08 Hours			Theory:      02 Hours				Practice: 	06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Operate hydraulic press | Trainee will be able to: - Prepare the work piece as per size requirement. - Ensure / inspect the press machine for oil level and other necessary elements. - Turn on the hydraulic press and select the load according to the thickness of the material. - Place the work piece between the die and bench. - Exert a force on the work piece with the help of punch. This would prepare the work piece to fit in the die. - Turn off the hydraulic press and note the observations and calculate the power required to drive the hydraulic press. | - Knowledge and purpose of hydraulic Press - Construction of hydraulic press - Working of hydraulic press - Pascal Law - Safe operating procedure of hydraulic press - Inspection procedure of hydraulic press - Operation of hydraulic press - Relationship of force and pressure PRACTICAL - Identify the parts of hydraulic press as per given criteria - Operate the hydraulic press and explain the working | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - Hydraulic Press - Control Valve - Plunger - Ram - Chamber - Weight - Hydraulic tubing - Hydraulic fittings | Class Room and workshop |
| - Observe the power requirement to drive a Hydraulic press | Trainee will be able to: - Prepare the work piece as per size requirement. Correct as previous. - Inspect the press for oil level and other necessary elements. - Turn on the hydraulic press and select the load according to the thickness of the material. - Place the work piece between the die and bench. - Exert a force on the work piece with the using the punch of the press. This would prepare the work piece to fit in the die. - Turn off the hydraulic press and note all observations. - Enter values of Area of Ram, Area of Plunger, length of the stroke of the Plunger, loaded arm and weight lifted by the arm in the Formula to calculate the power required. | - Relationship of pressure to force - Power calculation required to apply specific pressure - Working of Die, bench, Ram, Plunger, Loaded Arm and Weight. - Cooling mechanism in hydraulic press - Waste management procedure. PRACTICAL - Operate and observe the operation of hydraulic press and calculate the power requirement for given object. | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - Hydraulic Press - Control Valve - Plunger - Ram - Chamber - Weight - Hydraulic tubing - Hydraulic fittings - Pressure gauge | Class Room and workshop |

### Verify Bernoulli’s Equation

Objective: This module covers the knowledge and skills required to verify Velocity Head, Pressure head and Datum Head as well as Law of Conservation of Energy by measuring pressure and velocity at various points.

Duration:  		15 Hours			Theory:     06 Hours				Practice: 	09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Verification of Velocity Head, Pressure Head and Datum Head | Trainee will be able to: - Open inlet valve of hydraulic bench supply tank to allow the water to flow out of the tank. Apply Correction procedure. - Adjust the valves to obtain continuous flow at inlet and outlet valve. - Ensure continuous pressure head in the piezometer tube and note the reading. - Determine the water quantity collected in the measuring tank during the time noted by stop watch. - Determine the cross-sectional area of piezometer tube. - Calculate the velocity head and pressure head. - Change input and output supply and monitor them. | - Basic physics of fluids - Concept of Bernoulli’s Theorem - Definition of Velocity head, Pressure head and Datum head - Knowledge of piezoelectric effect - Relationship of pressure and velocity - Flow and Pressure measurement procedure PRACTICAL - Proof Bernoulli’s theorem with help of spray mechanism. | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Hydraulic Bench - Tapered inclined pipe fitted with number of piezometer tube points - Measuring tank - Scale - Stop watch - Pipe - Differential U-tube manometer - Piezometer tube - Flow circuit of G.I. Pipes of different fittings via Large bend, Small bend - Sudden enlargement from 25 mm to 50 mm diameter - Sudden contraction from 50 mm to 25 mm diameter. | Class Room and workshop |
| - Observe the performance of friction pipe apparatus | Trainee will be able to: - Select a pipe of suitable size as per available equipment for the experiment. Apply correction procedure. - Start the flow of water in the pipe with the help of hydraulic bench. - Observe the change in mercury level of U-tube manometer due to water flow in the pipe. - Monitor h1 and h2. - Monitor the time from stop watch to fill the water in the tank up to the height of 5 cm. - Change the flow and repeat the procedure. - Calculate coefficient of discharge. | - Definition of friction pipe - Effects of friction on flow - Relationship of height and flow at friction pipe apparatus - Calculation related to flow rate - Working principle of U-Tube manometer - Calculation of coefficient of discharge - Assembly of pipes - Working of hydraulic bench PRACTICAL - Measure the flow rate of line with help of U-Tube manometer. - Also calculate and verify losses. | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Hydraulic Bench - Tapered inclined pipe fitted with number of piezometer tube points - Measuring tank - Scale - Stop watch - Pipe - Differential U-tube manometer - Piezometer tube - Friction Pipe | Class Room and workshop |
| - Examine loss of head due to sudden enlargement, Contraction and entrance in a pipe | Trainee will be able to: - Measure the length and diameter of the pipe present between hydraulic bench and pressure tank. Correct according to the previous suggestions - Keep the pressure tank, end of the pipe open and the other end closed. - Operate the power supply of the hydraulic bench. - Start the flow of water in the circuit. - Adjust the flow up to maximum level. - Determine the total discharge in the circuit with the help of left and right limbs of U-tube manometer. | - Knowledge of Darcy’s friction factor - Losses associated with flow - Losses associated with head pressure - Calculation of coefficient of loss - Assembly of pipes - Working of hydraulic bench - Impact of sudden change in pipe area to flow rate and pressure PRACTICAL - Minimize the losses in given line due to sudden enlargement or contraction of entrance | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Hydraulic Bench - Tapered inclined pipe fitted with number of piezometer tube points - Measuring tank - Scale - Stop watch - Pipe - Differential U-tube manometer - Piezometer tube - Friction Pipe | Class Room and workshop |

### Analyze the performance of Pumps

Objective: This module covers the knowledge and skills required to observe and analyse centrifugal pump as well as reciprocating pump.

Duration:  		16 Hours			Theory:      04 Hours				Practice: 	12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Analyze the performance of Centrifugal Pump | Trainee will be able to: - Prepare pump for operation - Open the gate valve 1 or 2 rotations - Start the motor and set the vacuum gauge reading to the required head - Monitor the readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x - Set the vacuum gauge reading to the other heads - Monitor G, V, T, and t readings | - Purpose of centrifugal Pump - Working of centrifugal Pump - Parts of centrifugal pump - Working of suction and delivery valve - Working of Piston - Types and working of impeller - Definition of pump priming - Procedure of pump priming - Relationship of pressure and vacuum - Stages of centrifugal pump PRACTICAL - Construct a fire protection sprinkler using centrifugal pump | Theory-02 Hrs Practical-06 Hrs Total- 08 Hrs | - Centrifugal Pump - Test Bench - Pipe Hoses - Voltmeter - Ammeter - Reciprocating Pump - Tachometer - Stop watch - Pressure gauge - Energy meter | Class Room and workshop |
| - Analyze the performance of Reciprocating Pump | Trainee will be able to: - Prime the pump Correct according to the previous suggestions - Open the gate valve 1 or 2 rotations - Start the motor and set the vacuum gauge reading to the required head - Note down the following readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x And then set the vacuum gauge reading to the other heads - Monitor G, V, T, and t readings | - Construction of Reciprocating pump - Working of reciprocating pump - Parts of reciprocating pump - Types of reciprocating pump - Relationship of pressure and vacuum - Difference between centrifugal and reciprocating pump PRACTICAL - Construct a water transfer system of two tanks using reciprocating pump | Theory-02 Hrs Practical-06 Hrs Total- 08 Hrs | - Centrifugal Pump - Test Bench - Pipe Hoses - Voltmeter - Ammeter - Reciprocating Pump - Tachometer - Stop watch - Pressure gauge - Energy meter | Class Room and workshop |

### Construct circuit for Double Acting Hydraulic Cylinder

Objective: This module covers the knowledge and  skills required to actuate double acting hydraulic cylinder by switch and draw a circuit diagram and actuate double acting cylinder by oneway throttle valve.

Duration:  		10	Hours					Theory: 04	Hours					Practice: 06	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Actuate double acting hydraulic cylinder by switch and draw a circuit diagram | Trainee will be able to: - Connections of hydraulic circuit as per drawing - Turn on the supply of fluid to operate double acting cylinder - Move the piston to initiate the supply from forward stroke - Turn on the second push button switch after the piston reaches top dead center. This will start supply in the opposite direction - Observe the functions from control valve by varying its speed after the completion of supply in the circuit - Make observation sheet/table/chart. | - Understanding of Hydraulic Cylinder mechanism - Understanding of Double acting hydraulic circuit. - Configurations of hydraulic cylinder - Types of double acting cylinder - Components used in double acting cylinder - Working principle of Double acting hydraulic cylinder - Push Button function - Direction Control Valve - Usage of Double acting hydraulic cylinder - Benefits of actuating double acting hydraulic cylinder by switch - Industrial applications for actuating double acting hydraulic cylinder by switch. Practical Activity - Construct an item separator on production line using double acting cylinder and solenoid. | Theory-02 Hrs Practical-03 Hrs Total-05 Hrs | - Double acting cylinder - Pipe hoses And fittings - Pressure gauges - Flow control valve - Direction control valve - Push button - Hydraulic bench - Double ting cylinder - Pipe hoses and fittings - Pressure gauges - One-way Throttle valve - Direction control valve | Class Room and workshop |
| - Actuate double acting hydraulic cylinder by oneway throttle valve | Trainee will be able to: - Prepare the hydraulic circuit as per drawing - Ensure all accessories are working properly - Ensure the throttle valve on return line of the circuit. - Observe quick return of cylinder. - Understand the cylinder speed at various positions of the valve. - Make observation sheet/table/chart. | - Throttle Valve function - Understanding of One-way throttle Valve - Advantages and disadvantages of actuating double acting hydraulic cylinder by one-way throttle valve. - Adjusting cylinder advancing and retracing speed - Role of cylinder speed Practical Activity - Construct an item separator on production line using double acting cylinder and throttle valve. | Theory-02 Hrs Practical-03 Hrs Total-05 Hrs | - Double acting cylinder - Pipe hoses And fittings - Pressure gauges - Flow control valve - Direction control valve - Push button - Hydraulic bench - Double ting cylinder - Pipe hoses and fittings - Pressure gauges - One-way Throttle valve - Direction control valve | Class Room and workshop |

### Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder

Objective: This module covers the knowledge and  skills required to Set-up pressure device on a double acting cylinder using pressure reducing valve and hold specific load using double acting cylinder.

Duration:  		16	Hours					Theory: 04	Hours					Practice: 12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set-up pressure device on a double acting cylinder using pressure reducing valve | Trainee will be able to: - Understand the hydraulic circuit as per given drawing - Check if all devices/ components are working properly - Ensure that fluid has started flowing and the initial adjusted pressure is passed on to the circuit while passing through the relief valve. - Observe and note the pressure reading at various pressure gauges. - Change direction with the help of DCV and ensure control of double acting cylinder according to the direction. | - Pressure devices for Hydraulic system - Types of Hydraulic Cylinder - Pressure control valve - Types of pressure control valves - Pressure reducing valve - Working of pressure reducing valve in hydraulic system - Direction controlling of double acting cylinder - Working principle and uses of Directional Control Valve (DCV) - Construction of double acting hydraulic cylinder circuit Practical Activity - Construct a hydraulic station of given pressure specification with help of pressure reducing valve. | Theory-02 Hrs Practical-06 Hrs Total-08 Hrs | - Hydraulic Bench - Double ACTING Cylinder - Pressure reading valve - DCV pipe hoses - Hydraulic bench - Double acting cylinder - Pilot operated check valve - Direction control valve - Various loads - Pipe hoses - Pressure gauges | Class Room and workshop |
| - Hold specific load using double acting cylinder | Trainee will be able to: - Build the hydraulic circuit as per given drawings. - Check if all devices are working properly - Adjust pressure according to desired/required load. - Ensure fluid flow from port A to uplift the load. - Observe the downward motion of loaded ram. | - Hydraulic Circuit testing - Types of actuators used to hold the load - Pressure adjustment - Direction control using DCV - Principle of Hydraulic ram Practical Activity - Build a Car Lifter using double acting cylinder | Theory-02 Hrs Practical-06 Hrs Total-08 Hrs | - Hydraulic Bench - Double ACTING Cylinder - Pressure reading valve - DCV pipe hoses - Hydraulic bench - Double acting cylinder - Pilot operated check valve - Direction control valve - Various loads - Pipe hoses - Pressure gauges | Class Room and workshop |

### Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking

Objective: This module covers the knowledge and  skills required to construct a hydraulic circuit for double acting hydraulic cyclinder for mechanical interlocking with switch, circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch, hydraulic circuit using Accumulator stored Energy by DAC and LU4.	Measure pressure at various connections in hydraulic circuits.

Duration:  		20	Hours					Theory: 08	Hours					Practice: 12	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Construct a hydraulic circuit for double acting hydraulic cyclinder for mechanical interlocking with switch | Trainee will be able to: - Complete the hydraulic circuit as per drawing. - Ensure working of all devices. - Operate hydraulic circuit - Try lift some weight - Drop the pressure using pressure gauge before the piston starts moving - Construct Hydraulic Circuit Diagram - Construct electric circuit diagram | - Construction of hydraulic circuit as per drawing - Functions of hydraulic circuit - Construction of electric circuit drawing - Mechanical interlocking function - Mechanical switches functions and applications - Fluid circulation Practical Activity - Construct a hydraulic press machine with 2 limit switches safety feature to protect the operator hand. | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Double acting cylinder - DC pressure Gauges - Double acting cylinder - 3/2 push button - 5/2 double acting side pilot operated DCV - Contacting tubes - Hydraulic bench - Push button - Limit switch - Pressure gauges - Hydraulic Bench - Double Acting Cylinder - Accumulator Flow Control - Valve - DCV - Pipe Hoses - Pressure control valve - Hydraulic pump | Class Room and workshop |
| - Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch | Trainee will be able to: - Prepare simulation circuit of double acting cylinder as per drawing. Apply correction procedure. - Fix the push button and limit switch at their designated place as per drawing - Turn on the circuit power supply and ensure proper working of all equipment. - Press the push button to start fluid supply and will enable the cylinder for forward stroke supply. - Observe that as the push rod touches limit switch the supply of coil would disconnect and the piston would stop. - Press the second push button to complete backward stroke of piston, this will enable the supply of side coil and the backward stroke of piston would be completed and as it enters the surroundings of limit switches the supply would be disconnected. - Change 5/2 DCV and complete the opposite circuit. - Observe this changed circuit and note down all the observations. | - 3/2 push button function and working - Component placing as per drawing - Interlocking hydraulic cylinder Practical Activity - Construct an automatic door opener via double acting cylinder and pressure reducing valve. | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Double acting cylinder - DC pressure Gauges - Double acting cylinder - 3/2 push button - 5/2 double acting side pilot operated DCV - Contacting tubes - Hydraulic bench - Push button - Limit switch - Pressure gauges - Hydraulic Bench - Double Acting Cylinder - Accumulator Flow Control - Valve - DCV - Pipe Hoses - Pressure control valve - Hydraulic pump | Class Room and workshop |
| - Construct hydraulic circuit using Accumulator stored Energy by DAC | Trainee must be able to: - Complete the hydraulic circuit as per drawing. - Turn on the motor power supply and ensure proper working of all devises and equipment. - Use flow control valve to ensure fluid flow. - Observe the movement of piston rod. - When the circuit is burnt/completed the turn off the power supply. - Observe that due to accumulator the double acting cylinder completes its stroke and doesn’t stop immediately after the burning of circuit. | - Construction of accumulator - Working principle of accumulator - Application of accumulators - Working of flow control devices Practical Activity - Construct hydraulic lifting machine (for heavy load) using accumulator and monitor the behavior of jack with accumulator. | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Double acting cylinder - DC pressure Gauges - Double acting cylinder - 3/2 push button - 5/2 double acting side pilot operated DCV - Contacting tubes - Hydraulic bench - Push button - Limit switch - Pressure gauges - Hydraulic Bench - Double Acting Cylinder - Accumulator Flow Control - Valve - DCV - Pipe Hoses - Pressure control valve - Hydraulic pump | Class Room and workshop |
| - Measure pressure at various connections in hydraulic circuits | Trainee must be able to: - prepare the hydraulic circuit as per drawing - Install pressure gauges on the points where pressure is to be noted. - Turn on the hydraulic circuit. - Ensure fluid flow through the whole circuit. - Complete the circuit and note the readings of all gauges. - Enter all readings in observation table | - Verification of bernoulli’s theorem - Understanding of hydraulic network - Understanding of pressure change in a hydraulic circuit. - Pressure gauge application - Understanding of types of hydraulic couple Practical Activity - Monitor the pressure at different points of Lab apparatus. | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Double acting cylinder - DC pressure Gauges - Double acting cylinder - 3/2 push button - 5/2 double acting side pilot operated DCV - Contacting tubes - Hydraulic bench - Push button - Limit switch - Pressure gauges - Hydraulic Bench - Double Acting Cylinder - Accumulator Flow Control - Valve - DCV - Pipe Hoses - Pressure control valve - Hydraulic pump | Class Room and workshop |

### Set Speed and Direction of Hydraulic Motor

Objective: This module covers the knowledge and  skills required to set hydraulic motor R.P.M using flow control valve and direction by using direction control valve.

Duration:  	08		Hours			Theory: 02	Hours				Practice: 06		 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set hydraulic motor R.P.M using flow control valve | Trainee will be able to: - Construct the hydraulic circuit as per drawing. - Turn on the motor power supply and ensure proper working of all devices and equipment. - Fix the tachometer with the rotor to note motor RPM - Use flow control valve to control the flow, this would also control the motor RPM. - Observe the speed during this process. - Note RPM at different flow rates and analyze them. | - Hydraulic Motor - Operating principle of Hydraulic Motor - Flow Control Valve - Tachometer working principle - Hydraulic Motor RPM using Flow Control Valve Practical Activity - Construct a variable speed drill machine using hydraulic motor | Theory-02 Hrs Practical-03 Hrs Total-05 Hrs | - Hydraulic bench - Hydraulic motors - Tachometer - Flow control valve - Direction control valve - Pipe hoses - Pressure gauges | Class Room and workshop |
| - Set hydraulic motor direction by using direction control valve | Trainee will be able to: - Complete the hydraulic circuit as per drawing. - Install motor power supply and ensure proper working of all devices and equipment. - Use DCV to rotate the motor Clockwise and Anti Clockwise. - Observe the direction during this process. | - Hydraulic Motor direction using Directional Control Valve - Installation of DCV to set motor direction - Installation of Motor Power Supply - Hydraulic motor direction control method - Dual direction of rotor through direction control valve Practical Activity - Construct a power steering system using hydraulic motor | Theory-01 Hrs Practical-02 Hrs Total-04 Hrs | - Hydraulic bench - Hydraulic motors - Tachometer - Flow control valve - Direction control valve - Pipe hoses - Pressure gauges | Class Room and workshop |

### Operate Pressure Measuring Instruments

Objective: This module covers the knowledge and  skills required to U Tube Manometer, Inclined Manometer, pressure measurement in the Mercury Barometer, Borden Tube, Piezo Metric Tube and Elastic Diaphragms.

Duration:  		35	Hours			Theory: 14	Hours				Practice: 21		 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - U Tube Manometer | Trainee will be able to: - Set up the equipment. - Monitor measurement from U Tube Manometer - Connection of U Tube Manometer with the main line - Read the Find the Differential pressure using U Tube Manometer - Compute the pressure difference between the pipe inlet and outlet. | - Pressure and its units - Pressure measuring methods - Manometer and its types - Working principle of U Tube Manometer - Industrial applications of U Tube Manometer - Functions of U Tube Manometer - Pros and cons of U Tube Manometer - U Tube differential manometer - Installation method for U Tube Manometer Practical Activity - Calibrate the faulty pressure gauge with help of U-Tube Manometer | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - U Tube Manometer - Hydraulic Bench - Air Compressor - Scale - Tubes and piping | Class Room and workshop |
| - Inclined manometer | Trainee will be able to: - Set up the equipment. Connect one leg of inclined manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. - Observe the difference in head between the two legs of the manometer and record them. - Compute the pressure difference between the pipe inlet and outlet. | - Inclined Manometer working principle - Functions of inclined manometer - Advantages of inclined manometer - Measurement from inclined manometer - Inclined Manometer physical connection - Industrial applications of inclined manometer Practical Activity - Calibrate the faulty gauge with help of Inclined manometer Manometer | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Hydraulic Bench - Air Compressor - Scale - Inclined Manometer - Tubes and piping | Class Room and workshop |
| - Micro Manometer | Trainee must be able to: - Observe the enlarged ends of both tubes of the manometer - Adjust the instrument initially as p1 = p2. - Record / Note down the reading of the micrometer - Apply the unknown pressure difference - Observe the meniscus to move off the hairline, which can be restored to the initial position by raising or lowering the well (mercury sump). - Note down the difference in the initial and final micrometer readings - Note down the height of the mercury column and hence the pressure. | - Definition of Micro Manometer - Working principle of micro manometer - Functions of micro manometer - Pros and cons of micro manometer - Capillary effect and meniscus reading errors. - Measuring techniques of micro manometer. Practical Activity - Calibrate the faulty pressure gauge with help of micro Manometer | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Micro Manometer - Hydraulic Bench - Air Compressor - Scale - Tubes and piping | Class Room and workshop |
| - Pressure measurement in the Mercury Barometer | Trainee must be able to: - Take a reservoir and fill it with any fluid - Put a glass tube in the reservoir in the inverted state - Make the top portion of glass tube as air tight while the internal surface as complete vacuum - When the glass tube is dipped in the tank, an improper balanced pressure is created - Observe the water from the tank rises in the tube - Note that the water will rise to particular height/ limit - Note down the point where the water will stop lifting up | - The Mercury Barometer - Working principle of Mercury Barometer - Functions of Mercury Barometer - Uses of Mercury Barometer - Atmospheric Pressure - Measurement of atmospheric pressure using Mercury Barometer - Height of a Mercury Barometer on atmospheric pressure - Set up a Mercury Barometer. Practical Activity - Calibrate the faulty pressure gauge with help of Mercury Manometer | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Mercury Barometer - Hydraulic Bench - Air Compressor - Scale - Tubes and piping | Class Room and workshop |
| - Bourdon Tube | Trainee must be able to: - Observe the device consisting of a metallic tube of elliptical section closed at one end. - Measure the pressure at the other point (B) which is being fitted to the gauge point. - Insert the fluid in the tube. - Observe the tube tending to straighten. - Observe a pinion-sector arrangement pivoted with the pointer needle and already calibrated markings. - Note the small elastic deformation of the tube - Calibrate the device by subjecting to various known pressures. | - Bourdon Tube Working principle - Bourdon Tube functions - Uses of Bourdon Tube - Advantages of Bourdon Tube - Bourdon Tube elliptical shape - Function of a Bourdon Gauge - Industrial applications of Bourdon Tube - Shapes of Bourdon Tube - Pressure Measurement through Bourdon Tube Practical Activity - Calibrate the faulty pressure gauge with help of Bourdon Tube | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Bourdon Tube - Hydraulic Bench - Air Compressor - Scale - Pressure Gauge - Tubes and piping | Class Room and workshop |
| - Piezo Metric Tube | Trainee must be able to: - Shut off the valves of the tubes - Fill the tank until the lower part of the piezo metric tube is in contact with water. - Connect the pump to the upper plug of the tank with the non-return valve in the correct direction and pressurize the tank. - Observe the water head in the piezo metric tube rises. - Observe the level whether. | - Piezo Metric Tube - Bernoulli's Equation - Piezo Metric Tube functions - Working principle of Piezo metric tube - Industrial applications of Piezo metric tube - Pros and cons of Piezo Metric Tube Practical Activity - Calibrate the faulty gauge with help of Piezo metric Tube | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Bourdon Tube - Hydraulic Bench - Air Compressor - Scale - Pressure Gauge - Tubes and piping | Class Room and workshop |
| - Elastic Diaphragms | Trainee must be able to: - Observe the diaphragm is fixed in a tubular - Observe the initial pressure on a diaphragm - Apply pressure from one side - Observe a direct connection between diaphragm and pinion-sector - Observe a pinion-sector arrangement pivoted with the pointer needle - Note that the small elastic deformation of the diaphragm is communicating with the pointer | - Elastic Diaphragm Gauges - Construction of Elastic Diaphragm gauges - Working principle of Elastic Diaphragm - Functions of Elastic Diaphragm - Advantages of Elastic Diaphragm - Causes of deformation of elastic diaphragm - Parts of Elastic Diaphragm gauges Practical Activity - Calibrate the faulty pressure gauge with help of Elastic Diaphrams | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | - Diaphragm gauge - Hydraulic Bench - Air Compressor - Scale - Pressure Gauge - Tubes and piping | Class Room and workshop |

### Operate Flow Regulating Devices

Objective: This module covers the knowledge and  skills required to Ball Valve, Butterfly Valve, Gate Valve, Diaphragm Valve, Non Return Valve and Pressure Relief Valve.

Duration:  		24	Hours			Theory: 06	Hours				Practice: 18		 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Ball Valve | Trainee will be able to: - Identify the various parts of the ball valve including Handle, shaft, Ball, Seats and valve body - Observe a ball with a hole drilled through the center swivel mounted within the valve body - Observe the hole orientation in the ball. - Observe / Note that whether the hole in the ball is smaller in diameter than bore of the pipe | - Functions of valves - Types of Valves - Ball Valve functions and working principle - Uses of Ball Valve - Advantages and disadvantages of ball valve hole orientation with respect to the pipe and flow of the medium. - Hole to bore ratio. - Industrial applications of ball valve - Safety procedures to operate and install ball valve. Practical Activity - Control the pressure off a line via Ball Valve | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - Ball Valve - Section view drawing of Ball Valve - Tubes and piping - Pressure gauge | Class Room and workshop |
| - Butterfly Valve | Trainee will be able to: - Identify the gear box, handle, butterfly and the body of the butterfly valve. - Observe a circular disc in the middle of fluid pipe. - Ensure that the butterfly is oriented in the same direction as the pipe in order to allow the full flow. - Orient the butterfly away from the direction of the pipe, the flow rate will be restricted by the increased area of obstruction to the fluid. - Keep the butterfly orienting away from the fluid until 90 degrees to completely restrict the flow. | - Butterfly Valve definition - Functions and working mechanism of butterfly valve - Purposes of Butterfly valve - Types of Butterfly valve - Advantages and disadvantages of butterfly valves - Industrial applications of butterfly valves - Installation and selection of butterfly valve - Safety procedures to operate and install butterfly valve. Practical Activity - Control the pressure off a line via Butterfly Valve | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - Butterfly Valve - Section view drawing of Butterfly Valve - Tubes and piping - Pressure gauge | Class Room and workshop |
| - Gate Valve | Trainee must be able to: - Identify gate and body of the gate valve Correct according to the previous suggestions - Identify the gate in the valve - Identify the shape of the gate with respect to the pipe diameter - Identify the direction of movement of the gate - Ensure the complete upward motion of the gate for full flow of the fluid - Ensure complete downward motion of the gate until the opposite end for complete restriction to the fluid flow. | - Definition of Gate Valve - Isolation valve - Functions and working mechanism of Gate valve - Purposes of gate valve - Types of gate valve - Advantages and disadvantages of gate valves - Industrial applications of gate valves - Installation and selection of gate valve - Safety procedures to operate and install gate valve. Practical Activity - Control the pressure off a line via Gate Valve | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - Gate Valve - Section view drawing of Gate Valve - Tubes and piping - Pressure gauge | Class Room and workshop |
| - Diaphragm Valve | Trainee must be able to: - Identify bonnet, diaphragm/ flexible sheet, seat and main body of the valve - Identify whether the valve is manual or pneumatic actuated - Observe the fluid flow rates from the pipe as the diaphragm is pushed towards the seat for complete obstruction to the flow | - Working principle of Diaphragm valve - Functions of Diaphragm Valve - Diaphragm valve relation with fluid flow rate - Identification of manual and pneumatic valve - Industrial applications of Diaphragm valve - Advantages and disadvantages of diaphragm valve Practical Activity - Control the pressure off a line via Diaphragm Valve | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - Diaphragm Valve - Section view drawing of Diaphragm Valve - Tubes and piping - Pressure gauge | Class Room and workshop |
| - Non Return Valve | Trainee must be able to: - Identify the following parts in a non-return/ check valve: body, cover plate, disc, seats, hinge, hinge pin, nut, cotter pin, stud bolt, gasket, washer Correct according to the previous suggestions - Observe the fluid flow as the disk is released - Observe the disk being fixed as the flow direction is reversed | - Non return Valve - Functions and working principle of non-return valve - Identification of parts of non-return valve - Limiting the fluid flow rate - Understanding of fluid flow direction Practical Activity - Control the pressure off a line via Non Return Valve (NRV) | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - NRV Valve - Section view drawing of NRV Valve - Tubes and piping - Pressure gauge | Class Room and workshop |
| - Pressure Relief Valve | Trainee must be able to: - Identify the following parts in pressure relief valve: set pressure adjusting screw, spring, disk holder, seat disk, nozzle, bonnet, bonnet plug and body Correct according to the previous suggestions - Identify various types of pressure relief valve - Observe system pressure and spring pressure - Observe the valve opening as the system pressure increases then spring pressure | - Pressure Relief Valve - Working principle of pressure relief valve - Identification of parts of pressure relief valve - Types of pressure relief valve - Understanding of system pressure - Understanding of spring pressure - Advantages and disadvantages of pressure relief valve - Industrial applications of pressure relief valve Practical Activity - Control the pressure off a line via Pressure relief valve (PRV) | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - PRV Valve - Section view drawing of PRV Valve - Tubes and piping - Pressure gauge | Class Room and workshop |

# 4.

# List of Personal Protective Equipment

(FOR A CLASS OF 25 STUDENTS)

| Name of Trade |  | Mobile Phone Technician |  |
| --- | --- | --- | --- |
| Duration of Course |  | 12 Months |  |
| Sr. # |  | Description |  | Quantity |
| 1. |  | Safety gloves, |  | 30 |
| 2. |  | Appropriate safety glasses, |  | 30 |
| 3. |  | Breathing apparatus, |  | 30 |
| 4. |  | Fire extinguishers, |  | 30 |
| 5. |  | Fire blankets, |  | 30 |
| 6. |  | Respirators, masks, |  | 30 |
| 7. |  | Fire hoses, |  | 04 |
| 8. |  | First aid kits, stretchers and other related tools and gear |  | 10 |

# 5. Complete List of Tools, Equipment, Machines and Consumables

Tools

| S# | Description | Quantity |
| --- | --- | --- |
|  | Precision screw driver | 25 |
|  | Screw driver kit | 25 |
|  | Tweezers | 25 |
|  | Brushes | 25 |
|  | Blade cutter | 25 |
|  | Nose cutter | 25 |
|  | Point cutter | 25 |
|  | Cutter plier | 25 |
|  | Long Nose plier | 25 |
|  | PCB holder | 25 |
|  | Hard tweezers | 25 |
|  | Magnifying glass | 25 |
|  | Opener’s kit | 25 |
|  | Suction openers | 25 |
|  | Magnifying lamp | 25 |
|  | Wrist wire | 25 |
|  | Clipping tools(clips) | 25 |
|  | seizer | 25 |
|  | Steel wire | 25 |
|  | Rubber pads | 25 |
|  | Glass openers | 25 |

Equipment for Hardware

| S# | Description | Quantity |
| --- | --- | --- |
|  | Digital Multi meter | 25 |
|  | Oscilloscope | 5 |
|  | Soldering Iron | 25 |
|  | Soldering Station/SMD work station | 25 |
|  | Digital variable Power supply | 5 |
|  | B. G. A. Kit | 25 |
|  | Ultrasonic PCB Cleaner | 5 |

Equipment for Display repair

| S# | Description | Quantity |
| --- | --- | --- |
|  | Hot gun | 25 |
|  | Smoke Absorber | 5 |
|  | Hot air blower | 5 |
|  | Hot plate display separator | 5 |
|  | Freezer for separation of OLED | 5 |
|  | Flex binding machine | 5 |
|  | Mini electric grinder kit (EMC) | 5 |
|  | UV lamp | 5 |
|  | Glass laminating machine | 5 |
|  | De-bubbler machine | 5 |
|  | PCB Owen | 5 |

Equipment for Software

| S# | Description | Quantity |
| --- | --- | --- |
|  | Octopus Box (with cable Samsung + LG) | 5 |
|  | J Tag | 5 |
|  | EFT Dongle | 5 |
|  | CM2 Dongle | 5 |
|  | Sigma Key | 5 |
|  | ATF Dongle | 5 |
|  | ZXW (Hardware) Schematic diagram Dongle | 5 |

Consumables

| S# | Description | Quantity |
| --- | --- | --- |
|  | Soldering wire | 25 roll |
|  | Gloves | 25 Nos |
|  | Jumper wire |  |
|  | IC Paste |  |
|  | Solder paste |  |
|  | Cleaning sponge | 25 |
|  | Paste flux |  |
|  | Cleaning cloth |  |
|  | Chemical for washing mobile phone |  |
|  | UV Gum |  |
|  | Double tape |  |
|  | Heat resistance tape |  |
|  | Adhesive Glue |  |
|  | Thinner |  |
|  | CTC |  |
|  | Shiner |  |
|  | Rubber adhesive glue |  |
|  | WD-40 |  |
|  | Thinner |  |
|  | Cotton |  |

List of Stationary

| S# | Description | Quantity |
| --- | --- | --- |
|  | Handbooks | 5 |
|  | Design books | 5 |
|  | Pencils | 30 |
|  | Rubber | 30 |
|  | Sharpeners | 30 |
|  | Paper Cutter | 5 |
|  | Seizers | 5 |
|  | Colors | 5 each color (Read, Green, Yellow, Blue, white and Black) |
|  | White charts | 50 Nos |
|  | Brown sheets | 100 Nos |
|  | White board markers | 5 Doz |
|  | Permanent markers | 2 Dozen |
|  | File cover and files | 50 Nos. |

Learning hours in training provider premises,

Training workshop, laboratory and on-the-job workplace