# Written Assessment Guide

for

“Mix Sound”

Level-1 & 2

Date:12-11-2019

National Vocational & Technical Training Commission

| Title of Qualification: Professional Video Editor | CS Code: | Level: 1 & 2 | Version: 1 |
| --- | --- | --- | --- |
| Competency Standard Title: Mix Sound | Assessment Date (DD/MM/YY): |  |  |

| Guidance for Candidate | To complete your assessment for this Competency Standard, you need to answer the questions on the following pages successfully. |
| --- | --- |

Assessors Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: ..................................... ............................................................. |
| --- | --- |
| Written Assessment Outcome | COMPETENT  NOT YET COMPETENT  Name of the Assessor: Assessor’s code: Signature of the Assessor: ....................................................... ............................................................. |

Mix Sound	Page | 1

| Title of Qualification: Professional Video Editor | CS Code: | Level: 1 & 2 | Version: 1 |
| --- | --- | --- | --- |
| Competency Standard Title: Mix Sound | Assessment Date (DD/MM/YY): |  |  |

WRITTEN ASSESSMENT

| Question | Candidate’s answer |
| --- | --- |
| 1. What Is an Audio Interface? | An audio interface is a piece of hardware that expands and improves the sonic capabilities of a computer. Some audio interfaces give you the ability to connect professional microphones, instruments and other kinds of signals to a computer, and output a variety of signals as well. Basically, it is an external sound card |
| 2. What are the basic format of sound? | - MP3 The most popular audio format, (also known as MPEG-3) MP3 is a lossy format playable on many devices and programs. However, due to numerous involved patents, some programs, especially open source or independently developed programs, cannot handle MP3 files as the developers do not have the resources to pay the necessary royalties. - WAV The default uncompressed audio format on Windows, also playable in most programs. WAV is a higher quality format than MP3, so it uses more storage. |
| 3. What is bit rate? | Bit Rate refers to the audio quality of the stream. It is measured in Kilo bits per sec (kbps or k). Bit rate is no of bits (data) encoded per second or the no. of bits transmitted or received per second. Higher the bit rate with more sampling rate, requires high bandwidth and produces good audio quality. Low bit rates refer to smaller file size and less bandwidth with a drop in audio quality. For good quality music usually 64–128 kbps (96kbps+ recommended) bit rate is preferred. |

| Question | Candidate’s answer |
| --- | --- |
| 4. What is console? | In sound recording and reproduction, and sound reinforcement systems, a mixing console is an electronic device for combining sounds of many different audio signals. frames |
| 5. What is Sound and What is Noise? | Sound is what we hear. Noise is unwanted sound. The difference between sound and noise depends upon the listener and the circumstances. ... Sound is produced by vibrating objects and reaches the listener's ears as waves in the air or other media. |
| 6. What are effects? | Sound effects or audio effects are artificially created or enhanced sounds, or sound processes used to emphasize artistic or other content of movies, video games, music, or other media. |
| 7. What is audio mixing? | Audio mixing is the process by which multiple sounds are combined into one or more channels. In the process, a source's volume level, frequency content, dynamics, and panoramic position are manipulated or enhanced. This practical, aesthetic, or otherwise creative treatment is done in order to produce a finished version that is appealing to listeners. Audio mixing is practiced for music, film, television and live sound. The process is generally carried out by a mixing engineer operating a mixing console or digital audio workstation. |
| 8. What is voice over recording? | A voice over recording (often just “voice over”) is a person speaking (but not seen) during a video — often describing, highlighting, explaining, or providing additional context to what a viewer sees. It's often confused (or equated) with narration. And, while they are similar, they are not quite the same thing. |
| 9. Write name of 4 Audio recording software. | Adobe Audition, Mix Craft 8, FL Studio, CuBase, Reaper, Luvento |