National Vocational Certificate in  (level-4)

Microgrid Technician Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Level -4 Microgrid Technician. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed module needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	4

2.	Scheme of Studies	5

3.	Overview of the Curriculum for Creative Motion Design	6

4.	Details of Modules	10

0713E&E3701: Maintain Safe Working Environment of Microgrid	10

0713E&E3702: Maintain Equipment/ Machines at Microgrid station	15

0713E&E3703: Operate Microgrid in line with the SOP’s	21

0713E&E3704: Control Operation of Microgrid	26

0713E&E3705: Perform Data Monitoring of Microgrid  	33

0713E&E3706: Manage Power Generation	39

0713E&E3707: Manage Load dispatching	50

0713E&E3708: Manage Load Stability	53

0713E&E3709: Maintain Inventory of Microgrid System	57

0713E&E3710: Perform trouble shooting of Microgrid station	62

0713E&E3711: Develop Entrepreneurship/Business	65

0713E&E3712: On-Job Training/Project	76

5.	List of Tools and Equipment Required for the Qualification	77

6.	List of Consumable Items Required for the Qualification	82

7.	Members of Qualification Development Committee	86

- Introduction

This training course is designed for the occupation of (level-4) Microgrid Technician’ in the energy sector. A Microgrid Technician is responsible for the installation, operation, maintenance, control and troubleshooting of Microgrid systems, ensuring efficient and reliable energy distribution. The specific duties of a Microgrid Technician depend on the type of Microgrid system and its scale. Technicians are responsible for monitoring and maintaining electrical and control systems, integrating renewable energy sources, managing battery storage, and ensuring compliance with safety and regulatory standards. Their duties also include diagnosing faults, conducting performance assessments, optimizing system efficiency, and coordinating with engineers and other stakeholders. Additionally, they are responsible for training junior technicians and ensuring adherence to operational standards. Microgrid Technicians play a crucial role in maintaining resilient and sustainable energy solutions. To meet the growing demand for skilled professionals in this field, NAVTTC, in collaboration with GIZ, has introduced Competency-Based Training and Assessment (CBT&A) programs, ensuring the development of a competent workforce aligned with industry needs.

| 1.1 Date of Development | 19th -23rd May, 2025 |
| --- | --- |
| - Date of Review and Validation |  |
| 1.3. Entry Requirement | - General Electrician -Level-3, - Industrial Electrician- Level-3, - Solar PV Electrician -Level-3 OR - Or any other Equivalent qualification |
| 1.4. Minimum qualification for trainers/ teachers | - BS/BSc/BE Engineering (Electrical/ Electronics) - B.Tech (Honour)/BS Technology (Electrical/ Electronics) - Diploma of Associate Engineers (DAE in Electrical/ Electronic )with 2 years’ experience |
| 1.5. Codes of Qualification (ISCED) | Code: 0713E&E37 Description: Microgrid Technician (Level-4) |
| 1.6. Medium of instruction | English/Urdu |
| 1.7. Duration of Course | Total. 840 hours. Theory. 80 hours Practical. 520 hours On Job Training. 240 hours |

  - Overall objectives of training program

0713E&E3701: Maintain Safe Working Environment of Microgrid

0713E&E3702: Maintain Equipment/ Machines at Microgrid station

0713E&E3703: Operate Microgrid in line with the SOP’s

0713E&E3704: Control Operation of Microgrid

0713E&E3705: Perform Data Monitoring of Microgrid

0713E&E3706: Manage Power Generation

0713E&E3707: Manage Load dispatching

0713E&E3708: Manage Load Stability

0713E&E3709: Maintain Inventory of Microgrid System

0713E&E3710: Perform trouble shooting of Microgrid station

0713E&E3711: Develop Entrepreneurship/Business

0713E&E3712: On-Job Training/Students Project

- Scheme of Studies

| Module Code | Name of Competency | NVQF Level | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. | Total |  |
| 0713E&E3701 | Maintain Safe Working Environment of Microgrid | 4 | Functional | 5 | 50 | 55 | 3 |
| 0713E&E3702 | Maintain Equipment/ Machines at Microgrid station | 4 | Technical | 6 | 68 | 74 | 4 |
| 0713E&E3703 | Operate Microgrid in line with the SOP’s | 4 | Technical | 9 | 62 | 71 | 4 |
| 0713E&E3704 | Control Operation of Microgrid | 4 | Technical | 9 | 42 | 51 | 3 |
| 0713E&E3705 | Perform Data Monitoring of Microgrid | 4 | Technical | 7 | 46 | 53 | 3 |
| 0713E&E3706 | Manage Power Generation | 4 | Technical | 7 | 46 | 53 | 3 |
| 0713E&E3707 | Manage Load dispatching | 4 | Technical | 9 | 22 | 31 | 2 |
| 0713E&E3708 | Manage Load Stability | 4 | Technical | 8 | 24 | 32 | 2 |
| 0713E&E3709 | Maintain Inventory of Microgrid System | 4 | Technical | 9 | 62 | 71 | 4 |
| 0713E&E3710 | Perform trouble shooting of Microgrid station | 4 | Technical | 6 | 68 | 74 | 4 |
| 0713E&E3711 | Develop Entrepreneurship/Business | 4 | Generic | 5 | 30 | 35 | 2 |
| 0713E&E3712 | On-Job Training/Students Project | 4 | Industrial engagement /Students Project | 240 |  |  | 6 |
| Total |  |  |  | 80 | 520 | 840 | 40 |

- Overview of the Curriculum for Microgrid Technician (Level-4)

| Module Title | Learning Units | Theory hours | Workplace hours | Time frame of modules |
| --- | --- | --- | --- | --- |
| 0713E&E3701 Maintain Safe Working Environment of Microgrid | - Analyze potential risk from Microgrid System & its requirements - Categorize potential health Risk and safety of the Microgrid working environment - Install Safety Systems - Monitor Safety Protocols - Repair/ Replace faulty components of safety systems - Maintain Proper Signage and Warning Labels | 5 | 50 | 55 |
| 0713E&E3702 Maintain Equipment/ Machines at Microgrid station | - Perform Regular Inspections of machines - Perform Preventive Maintenance - Detect the faulty Components/equipment - Perform Corrective Maintenance - Carryout Run-Test of the repaired/ replaced Component/ equipment - Carry out tagging-in/out of machines/equipment - Prepare Maintenance Report | 6 | 68 | 74 |
| 0713E&E3703 Operate Microgrid in line with the SOP’s | - Start Generation system - Shift load on the generator - Maintain frequency of Microgrid for smooth operation - Operate switchgears for shutdown | 9 | 62 | 71 |
| 0713E&E3704 Control Operation of Microgrid | - Forecast load of Microgrid - Check hydrology/ wind speed/ solar Irradiation - Assess load pattern for 24 hours - Plan schedule as per load pattern - Implement the planned schedule - Manage variation in term of load and generation | 9 | 42 | 51 |
| 0713E&E3705 Perform Data Monitoring of Microgrid | - Check Grid Parameters as per Grid Code/SOPs - Adjust the Grid Parameter Variation - Record Variation in Grid Parameters - Generate Grid Parameters Report | 7 | 46 | 53 |
| 0713E&E3706 Manage Power Generation | - Carry out Megawatt Control through water flow - Carry out Megawatt Control through Solar Radiation - Perform Megawatt Control through wind speed - Perform Mega VAR (MVAR) through Excitation in hydel Power Generation System - Perform Mega VAR (MVAR) through in/out filters for solar and wind system - Maintain the desired (50Hz) Frequency. | 7 | 46 | 53 |
| 0713E&E3707 Manage Load dispatching | - Monitor Power Demand & Supply - Optimize Power Distribution - Implement Load management/ Shedding Strategies - Coordinate with Generation Sources - Perform Communication with Operators & Stakeholders | 9 | 22 | 31 |
| 0713E&E3708 Manage Load Stability | - Monitor Voltage & Frequency Levels - Handle emergency situation of Microgrid - Perform Balancing of Load and Generation. - Integrate Battery Energy Storage Systems (BESS) - Implement Automatic Voltage Regulation (AVR) | 8 | 24 | 32 |
| 0713E&E3709 Maintain Inventory of Microgrid System | - Maintain Tools, Equipment & spares Inventory - Maintain Faults Record - Maintain Shift Record - Maintain Supplier and Procurement Records - Implement Barcode/RFID System | 9 | 62 | 71 |
| 0713E&E3710 Perform trouble shooting of Microgrid station | - Diagnose Faults - Isolate Faulty Components - Test Faulty Components - Rectify the Fault - Analyze power quality and load behavior after rectification - Prepare report | 6 | 68 | 74 |
| 0713E&E3711 Develop Entrepreneurship/Business | - Communicate effectively in a professional environment - Prepare reports and documentation - Develop customer relationship and service skills - Analyze business fundamentals in the energy sector - Apply financial management skills - Follow entrepreneurial best practices - Implement safety and regulatory compliance - Identify business opportunities in the Energy sector - Develop structure of the new Energy business - Develop strategy to attract customers - Create portfolios on Facebook, Instagram and Linkedin | 5 | 30 | 35 |
| 0713E&E3712 On-Job Training/Project | Industrial Engagement/Students Project | - | - | 240 |
| Total |  | 80 | 520 | 840 |

- Detail of Modules

0713E&E3701: Maintain Safe Working Environment of Microgrid

Objective: This Module covers the underpinning knowledge and skills required for a student/trainee to be competent in maintaining a safe working environment for a Microgrid system. After completing this module, the students/Trainees will be able to perform all the tasks covered in (LU1 to LU6) in accordance with the essence of the Competency-Based Training System.

Duration: 55 Hours	Theory: 5 Hours	Practice: 50 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required |  | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LU1. Analyze potential risk from Microgrid System & its requirements | - Identify Microgrid Components & Operations - Assess Potential Risks in Microgrid Systems - Evaluate Safety & Compliance Requirements - Follow risk Mitigation Strategies | - Identification of components & its operation procedures - Recognition of potential risks - Safety requirements - Implementation of SOP’s - Risk Mitigation Strategies Practical Activity Conduct a Safety Risk Assessment and Develop a Mitigation Strategy for a Microgrid System following below given steps: - Component Identification - Hazard Identification - Mitigation Strategy by using PPE, improved signage or labeling,isolation procedures and Safety training requirements | Theory: 1Hr Practical: 4Hrs Total: 5Hrs |  | Consumable Items: - Pen - Pencil - Register Non Consumable Items: - PPE’s |  | - Well-equipped classroom /Work shop |
| LU2. Categorize potential health Risk and safety of the Microgrid working environment | - Identify Health Risks in the Microgrid Working Environment - Assess Safety Hazards in Microgrid Operations - Comply with Health and Safety Regulations - Categorize potential Risks | - Health risks at working environment - Safety hazards - Health and safety regulations. - Categorization of potential risks Practical Activity: Ensure safe working environment at micro grid through categorization of potential health risks and safety measures. - Safety Briefing and Introduction - On-Site Walk , - identify hazards (e.g. exposed wiring, poor ventilation, battery leakage signs etc.) - Risk assessment | Theory: 1Hr Practical: 5Hrs Total: 6Hrs |  | Consumable Items: - Notebook - Pen Non Consumable Items: - PPEs |  | - Well-equipped classroom / Work shop |
| LU3. Install Safety Systems | - Select the required Safety system - Assess Safety System Requirements - Prepare Safety Equipment - Implement Fire and Environmental Safety Measures - Set Up Security and Emergency System - Verify and Test Installed Safety System | - Recognition of safety system - Assessment of Safety System Requirements - Selection of safety systems - Installation techniques of safety equipment - Verification and testing techniques of safety system Practical Activity: Install and test the required safety system | Theory: 1 Hrs. Practical: 12Hrs Total: 13Hrs |  | Non Consumable Items: - Tools and equipment - PPEs - Safety systems |  | - Lab setup |
| LU4. Monitor Safety Protocols | - Establish Safety Monitoring Procedures - Conduct Routine Safety Inspections - Comply with Safety Regulations - Identify Safety Hazards - Report Safety Hazards - Implement Safety Protocols | - Safety SOP’s and regulations - Safety Protocols inspection technique Monitoring of Safety SOP’s - Saftey Hazards - Implementation of Safety Protocols - Reporting procedures of safety Protoc0ls Practical Activity: Identify and report safety hazards in Micro grid system through: - Inspection of Personal Protective Equipment (PPE’s) - Conduct of Safety Walk for identification of safety and security equipment. - Practicing proper lockout/ tagout procedure. - Reporting Safety Hazards | Theory: 0.5Hrs Practical: 10Hrs Total 10.5Hrs |  | Consumable Items: - Notebook - Pen Non Consumable Items: - PPEs |  | - Well-equipped classroom - Lab setup |
| LU5. Repair/ Replace faulty components of safety systems | - Inspect the components of faulty system for fault detection - Identify Faulty Components in Safety System - Evaluate Repair or Replacement Needs - Repair/ Replace the faulty components - Test and Verify System for smooth functioning | - Physical Inspection of system - Detection of faulty components - Identification of tool equipment/consumable for rectification - Evaluation of repair/replacement needs - Repairing/replacement techniques - Testing and verification of system for smooth functioning Practical Activity Perform repair/replacement of the faulty components as per assigned task and carry out testing and verification of system for smooth functioning. | Theory: 1Hrs Practical: 12Hrs Total: 13Hrs |  | Consumable Items: - Notebook - Pen Non Consumable Items: - PPEs - Tools and equipment's |  | - Well-equipped classroom - Lab setup/ Work shop |
| LU6. Maintain Proper Signage and Warning Labels | - Identify Signage and Warning Label Requirements - Assess Existing Signage - Check Hazardous Areas and Equipment for Proper Labeling - Install/ Replace Safety Signs and Labels - Verify Compliance and Employee Awareness - Maintain Signage Records and Updates | - Signage and warning label - Labeling of hazardous areas and equipment's - Installation/Replacement techniques of safety signs and labels - Safety Awareness campaigns - Signage and labels inventory Practical Activity Install/ Replace Safety Signs and Labels after Checking Hazardous Areas and Equipment for Proper Labeling | Theory: 0.5Hr Practical: 7Hrs Total: 7.5Hrs |  | Consumable Items: - Notebook - Pen Non Consumable Items: - PPEs - Tools and equipment's - Warning Label |  | - Well-equipped classroom - Lab setup /Work shop |

0713E&E3702: Maintain Equipment/ Machines at Microgrid station

Objective: This Module covers the under-pinning knowledge and skills required for a student/trainee to be competent in Maintaining equipment/ Machines at Microgrid station. After completing the Module, students/Trainees will be able to perform all the tasks  covered in (LU1 to LU7) in accordance with the essence of the Competency-Based Training System.

Duration: 74 Hours	 Theory: 6 Hours	Practice: 68 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Perform Regular Inspections of machines | - Prepare plan for Machine Inspections - Conduct Visual and Physical Inspection - Test Machine Performance and Safety Systems - Prepare Inspection Notes - Comply with Inspection Standards | - Identification of Machines/equipment - Importance of periodic inspection and maintenance schedule. - Testing procedure of machine - Reporting procedure Practical Activity Role play: You are a microgrid technician. One day you noticed loose wiring during your daily inspection visit of installed equipment that may cause a tripping of critical equipment. Prepare inspection report that highlights complete details of your inspection findings and recommend the remedial actions. | Theory: 1Hrs. Practical: 4Hrs. Total: 5Hrs. | Consumable Items: - Stationary Non-Consumable Items: - PPE’s | - Well-equipped classroom - Lab setup/Work shop |
| LU2. Perform Preventive Maintenance | - Plan and Schedule Preventive Maintenance Activities - Conduct Routine Inspection and Servicing - Test System Performance and Calibration - Address Potential Issues - Document Maintenance Activities - Comply with Safety and Operational Standards for Preventive Maintenance | - Importance of preventive Maintenance - Preventive maintenance Schedule - Safety and Operational Standards for Preventive Maintenance - Testing and calibration techniques - Documentation and Communication Practical Activity The battery bank in the microgrid energy storage system is scheduled for preventive maintenance. Your task is to prepare a maintenance plan, perform the required preventive maintenance and document the entire process following the operational and safety standards. | Theory: 1Hrs. Practical: 14Hrs. Total: 15Hrs. | Consumable Items: - Notebook - Pen Non-Consumable Items - Battery Bank - Multimeter - IR thermometer - Maintenance checklist and report form - Gloves - Goggles - Safety shoes - Cleaning material (brush cloth, terminal grease) | - Well-equipped classroom - Lab setup / Work shop |
| LU3. Detect the faulty Components/equipment | - Identify Signs of Faulty Components/Equipment - Apply Diagnostic Tools and Techniques - Analyze System Logs and Error Codes - Isolate Faulty Components for Further Examination - Fix the fault | - Symptoms of faulty components and equipment failures - Diagnostic tools and techniques for fault identification - System data logging and error messages - Handling procedure of faulty equipment - Fault fixing - Preventive techniques of recurring issues Practical Activity You are a Microgrid Technician on duty. The inverter in your Microgrid station has shown abnormal behavior, such as reduced output and intermittent alarms. Your task is to carry out fault detection using visual checks, diagnostic tools, and log analysis. You are required to detect and isolate the faulty components. | Theory: 1Hrs. Practical: 12Hrs. Total: 13 Hrs. | Consumable Items: - Notebook - Pen Non-Consumable Items: - Inverter system - Digital multimeter - IR thermometer - Fault code chart/ inverter troubleshooting guide - Gloves, safety glasses - Tagging material (labels, lockout devices) | - Well-equipped classroom - Lab setup/ Work shop |
| LU4. Perform Corrective Maintenance | - Diagnose Equipment Malfunctions - Plan and Prepare for Corrective Maintenance - Repair/Replace Faulty Components - Test and Verify Restored Equipment Functionality - Document corrective Maintenance Activities - Comply with Safety and Operational Standards for Corrective Maintenance | - Corrective maintenance - Malfunctions in Microgrid system. - Maintenance checklist - Corrective Maintenance procedure and techniques. - Post corrective maintenance testing and verifying procedure - Logs maintenance and reporting. Practical Activity: You are a Microgrid Technician on duty. A solar sub system in the Microgrid has been reported to be overcharging the battery bank. The charge controller appears to be malfunctioning and is not regulating the charging current correctly. Your task is to safely isolate the charge controller from main system, apply necessary protection tags and perform corrective maintenance of the equipment and compile a report. | Theor: 1Hrs. Practical: 12Hrs. Total: 13 Hrs. | Non Consumable material - PV Panel - Solar charge controller - Battery bank - Multimeter - Connecting wires and test leads - Screw driver set - Charge controller user manual - Fault cards/ Scenarios | - Well-equipped classroom - Lab setup / Work shop |
| LU5. Carryout Run-Test of the repaired/ replaced Component/ equipment | - Prepare for Run-Test - Conduct Initial Functional Check - Execute Full Load Testing - Analyze Test Results and Identify Issues - Document Run-Test Activities | - Importance of Run-test of equipment - Procedure to carry out Run-test - Importance of full load testing of equipment - Interpretation of test results of run test. Practical Activity: Find out a repaired electrical inverter and conduct a full load test to check its performance and document the result | Theor: 0.5Hrs. Practical: 12Hrs. Total: 12.5 Hrs. | Non consumable material - Electrical Inverter - Digital Multimeter - Clamp meter - Resistive load bank (100-watt incandescent lamps in series) - Test bench | - Well-equipped classroom - Lab setup / Work shop |
| LU6. Carry out tagging-in/out of machines/equipment | - Detect the faulty machines/equipment for Tagging - Perform Tagging-Out for Faulty or Under-Maintenance Equipment - Verify Compliance with Safety Measures - Perform Tagging-In After Maintenance - Document and Communicate Tagging Action | - Types of lock out and tag out - Lockout / Tagout (LOTO) kits - Importance of LOTO in creating safe working environment - Tagging in/Tagging out procedure - Tagging in/Tagging out documentation Practical Activity You are working as a Microgrid Technician during a routine inspection. You discover that an inverter is overheating and emitting unusual noise, indicating a potential internal fault. You must carry out Tagging-Out to prevent its use, ensure compliance with safety procedures, and later on Tag-In the inverter after rectification and testing. | Theor: 0.5Hrs. Practical: 10Hrs. Total: 10.5 Hrs. | Non-consumable material - Electrical Inverter with test bench - LOTO KIT - Safety signs - Simulated fault checklist | - Well-equipped classroom - Lab setup / Work shop |
| LU7. Prepare Maintenance Report | - Collect Maintenance Data - Compile the Maintenance Report - Channelize the Report | - Maintenance data collection procedure. - Introduction to technical report writing. - Importance of effective communication. Practical Activity Generate a detailed maintenance report regarding preventive maintenance of a battery bank in the Microgrid system. | Theory: 1Hrs. Practical: 4Hrs. Total: 5 Hrs. | Consumable Items: - Stationary - Paper Non-consumable Items: - Personal computer - Printer | - Well-equipped classroom |

0713E&E3703: Operate Microgrid in line with the SOP’s

Objective: This Module outlines the necessary skills and knowledge required to efficiently operate a Microgrid system while adhering to Standard Operating Procedures (SOPs). It covers system startup, load management, frequency regulation, and shutdown procedures to ensure the reliability, safety, and efficiency of power distribution. After completing the Module students/Trainees will be able to perform all the tasks covered in (LU1 to LU4) in accordance with the essence of the Competency-Based Training System

Duration: 71 Hours		Theory: 9 Hours		Practice: 62Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Start Generation System | - Identify operational requirements for power generation based on system demand - Check the availability of energy sources (renewable or hybrid) - Perform pre-startup safety checks and ensure protection systems are active - Start up the Generation system following SOPs and synchronize with the microgrid - Monitor generation parameters | - Electrical Power Generation Concepts. - Types of generators. - Difference b/w Grid Vs Island mode. - Types of Energy source for power generation. - System demand and its assessment. - Advantages and disadvantages of renewable and non-renewable energy sources. - Importance of pre-startup safety checks. - Types and Role of protection systems in Microgrid. - Starting procedure for Generation system (hydel/solar/wind) and safety checks. - Function of synchro-scope and synchro check relay and principle of synchronization. - Generation parameters (Frequency, terminal voltage, Power factor, current, active power, reactive power) and reading of their concerned meters. Practical Activity Draw and label a simple diagram of hydroelectric power station and their main parts to understand basic components of generation system | Theory: 2Hrs. Practical: 16 Hrs. Total: 18 Hrs. | Consumable Items: - Pen - Notebook Non-Consumable Items: - Multimeter / Clamp meter - Synchroscope / Synchro-check relay - Torque wrench - Generator models/simulators (Diesel, Solar, Wind) - Frequency and voltage meter - Digital tachometer - Hand tools (spanners, insulated screwdrivers, pliers) - First aid kit (reusable box with consumable contents) - PPEs | - Well-equipped classroom - Lab setup / Work shop |
| LU2. Shift load on the generator | - Assess load demand and distribution within the Microgrid - Transfer loads to the generator based on capacity and priority - Perform load shifting provided that it does not cause voltage fluctuations or frequency instability - Coordinate with relevant personnel to ensure smooth transitions | - Types of loads (Critical load, non-critical load, Resistive load, inductive load and capacitive load) - Peak and off peak load and its demand forecasting. - Difference b/w generator capacity and availability. - Procedure for close generator circuit breaker. - Monitoring of generator and system parameters i.e. voltage, frequency and load stability or misbalancing from local panel or HMI. - Coordination with relevant personnel/stakeholders. Practical Activity Perform load shifting test on AC/DC generator in Lab following SOP and gradually increase load/Demand to check fluctuation in generator parameters(frequency, voltage, current) | Theory: 2Hrs. Practical: 15 Hrs. Total: 17 Hrs. | Consumable Items: - Pen - Notebook Non-Consumable Items: - Multimeter / Clamp meter - Synchro-scope / Synchro-check relay - Torque wrench - Generator models/simulators (Diesel, Solar, Wind) - Frequency and voltage meter - Digital tachometer - Hand tools (spanners, insulated screwdrivers, pliers) - First aid kit (reusable box with consumable contents) | - Well-equipped classroom - Lab setup |
| LU3. Maintain frequency of Microgrid for smooth operation | - Monitor frequency levels using system indicators and control mechanisms - Adjust generation output to balance demand and maintain stable frequency - Respond to fluctuations caused by load changes or system disturbances - Implement frequency control measures such as load shedding/load management or generator adjustments - Record frequency variations and corrective actions taken | - Equipment used for real time monitoring of frequency and control mechanism. - Impact of freq deviations on equipment and operations - Standard frequency levels (e.g., 50 Hz or 60 Hz depending on region - Sources of frequency disturbance in microgrids (Load fluctuation, Generation instability, Renewable intermittency(solar/wind)) - Concept of Governor (Droop control) and Automatic Generation Control (AGC) to maintain frequency stable - Implementation of frequency control measures such as load shedding/load management or generator adjustments Practical Activity Adjust generation output of Microgrid to balance demand and maintain stable frequency and voltages. | Theory: 3Hrs Practical: 16Hrs. Total: 19 Hrs. | Consumable Items: - Pen - Notebook Non-Consumable Items: - Multimeter / Clamp meter - Synchroscope / Synchro-check relay - Torque wrench - Generator models/simulators (Diesel, Solar, Wind) - Frequency and voltage meter - Digital tachometer - Hand tools (spanners, insulated screwdrivers, pliers) - First aid kit (reusable box with consumable contents) | - Well-equipped classroom - Lab setup / Work shop |
| LU4. Operate switchgears for shutdown | - Follow SOPs for shutting down Microgrid components safely - Operate circuit breakers, disconnect switches, and protective devices in sequential order - Confirm system shutdown and communicate with relevant stakeholders - Record documentation of shutdown procedures and system status | - Difference b/w planned and unplanned shutdown. - Function and types of switchgear and its components - Importance of circuit breakers, disconnected switches, earthing, grounding and protection relay (Distance relay, overcurrent relay, breaker failure relay, under voltage relay and auto re-closer) - Pre-shutdown inspection and system status confirmation procedure. - Sequence of operation of switchgear and order of disconnection to prevent back feeding. - Communication during shutdown with relevant. stakeholder’s/control teams. - Compliance on Logging all switchgear operations in accordance with protocols. Practical Activity Prepare a detailed report on Safe and coordinated Shutdown of Microgrid systems using switchgear. | Theory: 2 Hrs. Practical: 15Hrs. Total: 17 Hrs. | Consumable Items: - Pen - Notebook Non-Consumable Items: - Multimeter / Clamp meter - Synchro-scope / Synchro-check relay - Torque wrench - Generator models/simulators (Diesel, Solar, Wind) - Frequency and voltage meter - Digital tachometer - Hand tools (spanners, insulated screwdrivers, pliers) | - Well-equipped classroom - Lab setup / Work shop |

0713E&E3704: Control Operation of Microgrid

Objective: This Module outlines the essential skills and knowledge required to control operation of a Microgrid effectively. It covers real-time monitoring, forecasting energy demand, assessing renewable energy availability, scheduling operations, and managing variations in load and generation. The goal is to ensure stable, efficient, and safe Microgrid operation while optimizing energy use. After completing the Module, students/trainees will be able to perform all the tasks covered in (LU1 to LU6) in accordance with the essence of the Competency-Based Training System.

Duration: 51 Hours			Theory: 9 Hours				Practice: 42 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Forecast load of Microgrid | - Analyze historical load data to predict future demand - Check seasonal, daily, and hourly load variations - Assess factors influencing load demand (weather, industrial use, etc.) | - Load forecasting and its methods - Importance of accurate forecasting to prevent system overload or under-utilization . - Impacts of inaccurate forecasting on power generation efficiency and operational costs - Types of load data (peak, average, interval) - Identification of Tools for collecting and analyzing historical consumption - Techniques for identifying patterns and trends - Simple prediction methods (moving average, trend analysis) - Identification of key demand influencers: - time of day (peak vs. off-pea - seasonal variations - Industrial and commercial activity patterns - Social and behavioral impacts (holidays, working hours) - user consumption trends Practical activity Collect data of load demand of any micro grid from internet for any two days and identify and analyze the impact and importance of key demand influencers that is time of day seasonal variations etc. on load forecasting | Theory: 1.5Hrs. Practical: 4Hrs. Total: 5.5Hrs | Consumable Items: - Printed worksheet - Graph papers - Stationery Non-Consumable Items: - Personal Computer - Printer - Whiteboard - Internet access. | - Well-equipped classroom - Computer Lab |
| LU2. Check hydrology/ wind speed/ solar Irradiation | - Monitor weather conditions affecting renewable energy sources - Apply hydrological data to estimate hydroelectric power generation potential - Measure wind speed and direction to assess wind power availability - Analyze solar radiation levels to determine solar energy output | - Understanding Weather variables affecting renewable energy (rainfall, solar irradiance, wind speed/direction) - Tools and instruments for weather monitoring (anemometers, pyrometers, hydrological sensors) - Use of weather stations and real-time data feeds (manual and digital) - Understanding flow rate, head, and water levels. Measuring wind speed and direction with anemometers and solar radiation with pyrometers or solar data loggers - Interpreting rainfall data, river flow records, and reservoir levels. wind data charts and identifying viable wind power conditions. daily and seasonal solar patterns - Calculation of potential hydroelectric output using basic formulas (e.g., Q x H x g x η) - Estimating wind power using standard equations (P = ½ ρ A V³ η) - Measuring solar radiation and Estimating photovoltaic output based on irradiation and panel characteristics Practical activity - Select a water tank somewhere in your college and consider it as dam analyze the live storage, put some liter of water note the level rise and use some of its water identify head of your tank P= ρgh - Check the wind speed of your city on internet for different times Estimating wind power using standard equations (P = ½ ρ A V³ η) | Theory: 2Hrs. Practical: 4Hrs. Total: 6Hrs | Consumable Items: - Printed worksheet - Graph papers - Stationery Non-Consumable Items: - Personal Computer - Projector. - Whiteboard - Internet access - Microgrid Trainer - Solar Irradiance Meter - Flowmeter - Anemometer - Weather Station (manual or digital) | - Well-equipped classroom/ Work shop - Lab setup |
| LU3. Assess load pattern for 24 hours | - Collect data regarding past energy consumption - Evaluate past energy consumption trends over a 24-hour cycle - Check peak and off-peak demand periods. - Assess Grid stability based on demand fluctuations - Adjust energy storage and generation accordingly | - Types of energy consumption data (interval data, hourly usage, total kWh) - Techniques for recording and organizing 24-hour load data using smart meters, energy meter or manual logs - 24-hour load curves to identify trends and consumption behaviors across time blocks (night, morning, business hours, evening) - Importance of peak demand identification and defining peak off-peak hours, shoulder periods - Impact of pricing or operational conditions influence peak off-peak hours, shoulder periods - Impacts of load fluctuations on voltage, frequency, and grid reliability - Impact of load trends on Forecasting and pre-scheduling storage actions - Concepts of base load, peak load, and load variability Practical Activity - Select the energy meter of your college note the energy reading in KHW (units) for every hour, note the reading for 5 hours. Draw load curve show trends and consumption behaviors across time blocks. - Asses the load for the same time on next day on the bases of today’s data. (you can also take into account comparison of temperature and humidity for both days) | Theory: 1Hrs. Practical: 4Hrs. Total: 5Hrs | Consumable Items: - Printed worksheet - Graph papers - Stationery - Report Templates/Forms Non-Consumable Items: - Personal Computer - Projector - Whiteboard - Internet access - Microgrid Trainer - Energy Meter / Smart Meter - Data Logger | - Well-equipped classroom - Lab setup / Work shop |
| LU4. Plan schedule as per load pattern | - Develop energy dispatch schedule to match demand and generation - Allocate power sources based on availability and efficiency - Confirm backup power availability for emergency scenarios - Coordinate with stakeholders for schedule approval and execution | - Purpose of energy dispatch scheduling in micro grid - Benefits of proper scheduling: efficiency, cost reduction, load balancing - Energy demand fluctuation throughout the day - Reviewing 24-hour load patterns to determine demand profiles and track variation - Types of consumption patterns - {residential, commercial, and industrial) - Matching of energy sources (renewable, storage, conventional) to demand periods by considering their availability, efficiency, cost, emissions, and operational characteristics such as intermittency, ramp-up time, and maintenance schedules - Planning for handling source failure or demand spikes and identify backup systems (generators, batteries, load-shedding) Practical Activity Plan and justify energy schedule aligned with actual or simulated demand, available generation capacity, and operational constraints of any micro grid taken from internet. Prepare schedule table showing: - Time - Load demand - Source(s) used - Reserve/backup strategy Justify scheduling decisions on the bases of efficiency, cost, reliability | Theory: 1.5Hrs. Practical: 4Hrs. Total: 5.5Hrs | Consumable Items: - Printed worksheet - Graph papers - Stationery - Report Templates/Forms - Printed Case Studies or Load Scenarios Non-Consumable Items: - Personal Computer - Spreadsheet Software (Excel) - Projector - Whiteboard - Internet access - Load Profile Analysis Software | - Well-equipped classroom - Computer Lab |
| LU5. Implement the planned schedule | - Execute power distribution and generation as per the approved plan - Monitor deviations in real-time. - Verify smooth transition during load shifting - Document system performance for future improvements | - Implementation of approved energy dispatch schedules and operational procedures (SOPs). - Monitoring of real-time system parameters (voltage, current, frequency) using energy management tools. - Detection and response to deviations in generation and load, understanding scenarios requiring load shifting - Avoiding voltage/frequency instability or supply disruption during transitions - System performance data recording procedure to support future improvements and audits. - Implementation of safety protocols during load adjustments Practical Activity Operate a micro grid simulator or training board to run the dispatch plan. Monitor parameters and log any deviations (e.g., generation shortfall, load surge).Simulate a source failure (e.g., solar drop) and perform a smooth shift to backup power | Theory: 1.5Hrs. Practical: 12Hrs. Total: 13.5Hrs | - Consumable Items: - Logbooks or Printed Data Sheets - Graph papers - Stationery - Report Templates/Forms - Fuses or small circuit protection components (for replacement during training) - Non-Consumable Items: - Personal Computer - Real-time Display Units (for voltage, frequency meter) - Digital Multimeter - Internet access - Micro grid Trainer - Data Monitoring Software - Simulated Load Devices | - Well-equipped classroom - Lab setup / Work shop |
| LU6. Manage variation in term of load and generation | - Respond to unexpected fluctuations in demand and supply. - Implement load-shedding if required. - Optimize the use of energy storage systems (batteries, pumped storage, etc.). - Maintain communication with grid operators and stakeholders for coordinated adjustments | - Working of micro grid control systems, including key components like generators, loads, and storage systems. - Identification of common causes of load and generation fluctuations (e.g., sudden demand, weather changes, faults). - Impacts of mismatched supply and demand, such as voltage or frequency instability. - Implementation of real-time monitoring and manual/automated responses to balance the system. - Description of load-shedding when and how to perform safe, prioritized load-shedding. - Identifying critical vs. non-critical loads and basic principles of demand-side management - Use of grid frequency regulation techniques (automatic voltage regulation, reactive power compensation). - Communication and coordinated actions using messaging tools. - Adjustment techniques of power supply. - Compliance of emergency procedures to stabilize power and log events for review. Practical Activity Create a simulated scenario where energy demand or generation changes unexpectedly. Monitor the system in real time and take corrective actions such as load-shedding. keep a log of actions taken | Theory: 1.5Hrs. Practical: 14Hrs. Total: 15.5Hrs | - Consumable Items: - Log sheets - Printer paper - Electrical safety tags / labels - Stationery - Whiteboard & marker - Fuses or small circuit protection components (for replacement during training) - Non-Consumable Items: - Personal Computer - Real-time Display Units (for voltage, frequency meter) - Digital Multimeter - Communication devices - Smart meter - Micro grid Trainer - Data Monitoring Software or EMS - Simulation Software - Protective relay setup | - Well-equipped classroom - Lab setup / Work shop |

0713E&E3705: Perform Data Monitoring of Microgrid

Objective:  This Module is about to keep an eagle eye regarding ‘Perform data monitoring of Microgrid’ and to observe that on how much power is being made and used, as well as checking the energy storage levels, voltage, and frequency. This helps make sure everything is running smoothly. Technicians use special tools to track this data in real time, looking for any problems like power surges or low energy levels. If something goes wrong, they can fix it right away. Automated alerts also help warn them if something isn’t working properly, ensuring the system stays stable and provides reliable energy. After completing the module, the students/Trainees will be able to perform all the tasks covered in (LU1 to LU4) in accordance with the essence of the Competency-Based Training System.

Duration: 53 Hours        	Theory: 7 Hours 	Practice: 46 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Check Grid Parameters as per Grid Code/SOPs | - Inspect panels for recoding real-time grid data. - Configure monitoring systems. - Manage settings on equipment to match grid parameters. - Document grid data in logs & reports accurately. - Verify the grid data as per standard. - Fix variations by adjusting the system. - Record and report Grid data. | - Grid code standards and their importance - Importance of monitoring for stability and compliance - Types of monitoring equipment (digital meters, EMS systems, data loggers) - Procedure of configuring and calibrating monitoring systems - Abnormal reading limits - Interpretation of real-time data from control panels or screens - Cross-checking data with standard grid code or SOPs - Adjustment techniques of voltage or frequency for correcting power factor or imbalance - Maintenance and reporting of daily operation sheets. Practical activity Monitor the control panel for recording the current grid parameter such as voltage, frequency, and power factor of WAPDA supply in your college and Compare the readings with the standard values provided in the grid code by National electric power Regulatory Authority (NEPRA) ( grid code can be downloaded from NEPRA site) | Theory: 2Hrs. Practical: 15Hrs. Total: 17Hrs. | Consumable Items: - Log Sheets / Printed Record Forms - Stationery (pens, markers, notepads) - Electrical Safety Tags / Labels - Cleaning Materials for Panels and Equipment - Spare Fuses or Relays Non-Consumable Items - Digital Multimeter - Power Quality Analyzer - Standard Operating Procedure (SOP) - Data Logger or Monitoring Software - HMI - Alarm or Notification Systems - Calibration Kit - Voltage & Frequency meter - Energy management system (EMS) software | - Well-equipped classroom - Lab setup / Work shop |

| LU2. Adjust the Grid Parameter Variation | - Monitor the grid parameters. - Identify the cause of the grid parameter variation. - Inspect control systems or equipment to adjust the grid parameters. - Modify the settings to bring the grid parameters back within acceptable limits. - Check that the adjustments are working by observing real-time data. - Confirm that the grid parameters are stable and meet required standards. | - Key grid parameters: voltage, frequency, current, power factor - Acceptable limits of Grid parameters - Common causes of variation (e.g., load imbalance, faulty equipment, generation mismatch) - Tools/equipment for monitoring real-time parameters - Critical variations in Grid values. - Adjustment of system settings to correct grid parameters - Rechecking real-time data to confirm corrections - Comparing values against SOP or national grid standards - Importance of electrical safety protocols before making adjustments Practical Activity Monitor the current grid parameter such as voltage at your college using electrical system of WAPDA and identify the voltage parameter that is outside the acceptable range mentioned in grid code that is 230V for domestic use. Identify the possible reason of variation. Adjust the necessary setting to correct the variation. After adjustment, observe the real-time data to make sure the issue is fixed and the voltage parameters are stable. Report your actions to the instructor. | Theory: 1.5Hrs. Practical: 20Hrs. Total: 21.5Hrs. | Consumable Items: - Cable ties / Tags / Labels - Log sheets / Record books. - Stationery - Cleaning materials Non-Consumable Items - Multimeter / Clamp meter - Power Quality Analyzer - Digital panel meters - Protective relays a - Computer/Laptop - Adjustable tap changers or voltage regulators - Personal Protective Equipment. | - Well-equipped classroom - Lab setup / Work shop |
| --- | --- | --- | --- | --- | --- |
| LU3. Record Variation in Grid Parameters | - Identify any variations in the grid parameters that are outside the normal range. - Log the time and details of the variations in a record-keeping system. - Document the type and extent of the variation in an easy-to-read format. - Verify the accuracy of the recorded variation by double-checking the data. - Update the records as needed with any new information about the variations. | - Normal operating ranges according to SOPs/grid code - Tools for detection of real-time changes: (EMS, multimeters, control panels) - Causes of variations - Data recording procedure - Cross-checking of logged data - Reporting procedure and channels Practical Activity Monitor the current grid parameter such as frequency of your college using electrical system of WAPDA and record the variation in frequency for 5 minutes on the following pattern - Date - Time - Parameter - Type - Measured value - Expected value | Theory: 1.5Hrs. Practical: 6Hrs. Total: 7.5 Hrs. | Consumable Items: - Sticky Notes / Flags - Cable ties / Tags - Log sheets / Record books. - Stationery - Printer ink and paper Non-Consumable Items - Multimeter / Clamp meter - Power Quality Analyzer - Protective relays Computer - Energy management system (EMS) software | - Well-equipped classroom / Work shop |
| LU4. Generate Grid Parameters Report. | - Document all grid parameter variations, including any corrective actions taken. - Summarize key findings in the report, highlighting any problems or unusual patterns. - Verify the accuracy of the report by double-checking the data and calculations. - Create the final report using the proper templates or formats for easy reading and understanding. - Submit the report to the relevant team or manager for review and further action. - Maintain records of the report for future reference or audits. | - Importance of accurate grid reporting to achieve operational efficiency, troubleshooting, and compliance. - Reporting procedure of parameter variations(timestamps, actions taken, affected systems, and possible causes). - Abnormal patterns - Formatting for readability: tables, bullet summaries, clear section headings. - Basic calculations or summaries (averages, deviations) to support findings. - Importance of timely submission to the correct personnel. - Storing and retrieving reports for audits or reference. Practical Activity Monitor the current grid parameter such as voltage and frequency of your college using electrical system of WAPDA and identify the voltage and frequency parameter that is outside the acceptable range mentioned in grid code that is 230V and 49.5-50.5HZ. Create the report as following - Time - Date - Parameter - Recorded value - Expected value - Action Taken if any | Theory: 2Hrs. Practical: 5Hrs. Total: 7Hrs | Consumable Items: - Log Sheets or Grid Record Forms - Stationery - Paper - Printer Ink/Toner - USB Drives/CDs Non-Consumable Items - Computer - Energy Management System (EMS) software - Digital Multimeter - Printer - Data Logger Device - Report Forms | - Well-equipped classroom / Work shop |

0713E&E3706: Manage Power Generation

Objective: The Module of Managing power generation covers that enough energy/electricity is produced to meet demand by balancing sources like solar, wind, and generators. Microgrid Technicians monitor energy use, adjust output, and maintain system stability. They quickly address issues like power drops or equipment failures to prevent disruptions. After completing the module, the students/Trainees will be able to perform all the tasks covered in (LU1 to LU6) in accordance with the essence of the Competency-Based Training System.

Duration: 53 Hours		                    Theory: 7 Hours		Practice: 46 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Carry out Megawatt Control through water flow | - Locate the water flow control valve and megawatt controller panel. - Check the current water flow rate and megawatt output readings. - Adjust the water flow valve to increase or decrease flow. - Monitor the changes in megawatt output. - Perform fine-tuning of water flow to maintain stable megawatt output. - Record the final water flow rate and megawatt output readings. | - Identification and function of the water flow control valve and megawatt controller panel - System schematics and flow diagrams - Monitoring of water flow rate and megawatt output reading - Adjustment of water flow valve, - Monitoring the changes in megawatt output - Optimization of water flow control for stable energy output. - Record keeping procedure of the final water flow rate and megawatt output reading Practical activity Perform fine-tuning of water flow to maintain stable megawatt output. | Theory: 1Hrs. Practical: 8Hrs. Total: 9Hrs. | Consumable Items: - log sheet - stationary Non-Consumable Items - Personal protective equipment (PPE) - Tools and equipment | - Well-equipped classroom - Lab setup / Work shop |
| Carry out Megawatt Control through Solar Radiation | - Identify the solar radiation sensor and megawatt control panel. - Check the current solar radiation levels and megawatt output. - Adjust the megawatt control settings based on solar radiation levels. - Monitor the changes in megawatt output as solar radiation fluctuation. - Perform fine-tuning of the megawatt control settings to optimize performance based on solar radiation. - Record the final solar radiation and megawatt output readings for reference | - Introduction to solar radiation and its effect on energy output - Monitoring solar energy inputs and output - Interpretation of solar irradiance levels, solar environmental conditions, using pyrometers or sensors - stabilization Techniques for megawatt output - Impacts of variable conditions such as fluctuations in sunlight (e.g. cloud cover, etc.) on Megawhat output. - Integration techniques of energy storage systems (e.g., batteries) to buffer output - Solar irradiance and the corresponding energy output levels are recorded in relation to control settings and system responses - Fine-tuning procedure of inverter, controller settings, and MPPT settings for optimized and stable megawatt output - Maintain record of final solar radiation and megawatt output Practical activity - Perform fine-tuning of the megawatt control settings to optimize performance based on solar radiation. | Theory: 1 Hrs. Practical: 6 Hrs. Total: 7Hrs. | Non-Consumable Items: - Solar panels (at least 1-2, capable of generating measurable power) - Charge controller - Energy meter/power meter to measure megawatt - PV Analyzer - Lux meter - Digital multi-meter - Solar radiation sensor (pyrometer) - Solar operated trainer or PV System, Helioscope, HOMER ProOff-grid inverter Consumable items: - Manual data sheet/logbook for recording observations. - Wires - Stationary | - Well-equipped classroom - Lab setup |
| Perform Megawatt Control through wind speed | - Locate the wind speed sensor and megawatt control panel. - Check the current wind speed and megawatt output readings. - Adjust the megawatt control settings based on the wind speed. - Monitor the changes in megawatt output as wind speed varies. - Perform fine-tuning of the megawatt control settings to maintain stable output as wind speed fluctuates. - Record the final wind speed and megawatt output readings. | - Introduction to wind turbine systems - Principles of wind energy conversion - Importance of wind speed on electrical power output - Key components of Wind turbine. - Principle and concepts of turbine control systems (pitch control, yaw control, braking systems) - Monitoring of wind speed and output - Identification of optimal wind speed ranges for optimized generation - Regulating/Adjustment techniques of Megawatt Output under varying wind speeds with safety consideration - Techniques of fine-tuning for turbine blade angle adjustments to balance turbine efficiency with grid demand requirements - Recording of wind speed and megawatt output readings accurately Practical activity - Identify the wind turbine blades, generator, anemometer, and the control panel/controller interface on the lab setup. - Use a fan to create a steady wind (e.g., at a moderate speed), Record the baseline wind speed and corresponding Megawatt output in their data sheet. Interpretation of megawatt output with respect to wind speed, and megawatt output relate to the turbine's rated power. Demonstrate how to modify relevant settings on the controller/simulator (e.g., pitch angle, power limiting, or simulated load resistance). Students will record the new settings, wind speed, and the resulting megawatt output. | Theory: 1 Hrs. Practical: 8 Hrs. Total: 9 Hrs. | Non-Consumable Items - Small-scale wind turbine (preferably with adjustable pitch or a controllable braking/loading mechanism). - Anemometer (wind speed sensor) with a display or data output. - Generator (part of the turbine system). - Power meter/energy meter to measure megawatt (or kilowatt) output. - Wind turbine controller/inverter (ideally with a user interface allowing parameter adjustments, or a simulated one). - Fan (large enough to simulate varying wind speeds for the miniature turbine). Consumable items: - Manual data sheet/logbook for recording observations. - Wires - Stationary | - Well-equipped classroom - Lab setup |
| Perform Mega VAR (MVAR) through Excitation in Hydel Power generation System | - Locate the excitation control panel and MVAR measurement system. - Check the current MVAR levels and excitation system settings. - Adjust the excitation control to increase or decrease the MVAR output. - Monitor the changes in MVAR output. - Perform fine-tuning of the excitation settings to maintain desired MVAR levels. - Record the final excitation settings and MVAR output for reference. | - Introduction to reactive power and Mega VAR (MVAR) , - Role reactive of power in power systems - Relationship between reactive power, voltage, and generator excitation . - importance of MVAR in voltage regulation and grid stability - Types of excitation systems (e.g., static, brushless) and their components - Understanding of control loops between excitation and MVAR output - Monitoring of MVAR output ,excitation settings and system parameters - Techniques of excitation adjustment for MVAR control - Impacts of excitation changes on overall generator performance - Precise tuning of excitation to maintain target reactive power levels - Maintaining and recording MVAR values and generator voltage levels under varying load conditions Practical activity - You are required to identify the main controls for excitation (e.g., field rheostat, AVR set point) and observe the MVAR meter. - You are required to gradually increase the excitation current/AVR set point- Over-excitation and record the excitation settings, the new MVAR output (should be positive/lagging), and the change in generator terminal voltage. Similarly decrease for Under-excitation/Absorb MVAR. - Students will adjust the excitation to achieve a near-unity power factor (MVAR output close to zero). - Record multiple data points showing wind speed, control settings, and MVAR output during these fluctuations. - Students record their final desired MVAR target, the excitation settings that achieved it, and the corresponding generator terminal voltage. | Theory: 1 Hrs. Practical: 8 Hrs. Total: 9 Hrs. | Non-Consumable Items - A robust power system simulation software (e.g., ETAP, DIgSILENT PowerFactory, PSS/E, or a dedicated generator control simulator). - Lab setup: Small synchronous generator (e.g., a lab machine typically used for electrical machines experiments). - Prime mover (e.g., a DC motor simulating a turbine, with speed control). - Excitation system for the synchronous generator (e.g., a DC power supply with variable output, simulating a static exciter). - Voltage regulator/AVR (manual or simulated). - Reactive power meter (MVAR meter) - Digital multimeter - Resistive/Inductive/Capacitive Load Bank (to simulate grid conditions or local loads). - PPEs Consumable items: - Stationary items | - Well-equipped classroom - Lab setup |
| - Perform Mega VAR (MVAR) through in/out filters for solar and wind system | - Identify the in/out filters and MVAR control panel for the solar and wind system. - Check the current MVAR levels and filter settings for both solar and wind systems. - Adjust the in/out filters to modify the MVAR output based on system requirements. - Monitor the MVAR levels and filter performance as adjustments are made. - Perform fine-tuning of the filter settings to achieve the desired MVAR output for both solar and wind systems. - Record the final filter settings and MVAR output for future reference. | - Role of in/out filters in reactive power compensation and voltage regulation with high penetration of solar and wind power - Identification of in/out filters used in solar and wind power systems (e.g., capacitor banks, inductive filters, STATCOMs) - Importance of MVAR control panels specific to solar and wind power systems - Monitoring filter settings and MVAR levels - Procedure to adjust filter banks or settings for MVAR regulation - Compensation techniques like inductive and capacitive device to control MVAR - Performing of real-time or simulated adjustments of in/out filters on solar and wind systems - Recording and observations of initial and final filter settings and corresponding MVAR values Practical activity - Students navigate the simulation software or physically identify the solar PV array, wind turbine(s), their inverters, the point of common coupling (PCC), and the reactive power compensation devices (e.g., capacitor banks, reactors, STATCOM models) at the PCC or within the plant, along with their associated control interfaces/panels. - Students will analyze baseline scenario (e.g., moderate solar radiation, moderate wind speed, and initial grid load), students record the overall plant MVAR output, voltage at the PCC, and the current settings of the in/out filters. - Students will check a condition where the voltage at the PCC drops, or the system needs to supply more MVAR (e.g., by increasing an inductive load on the grid or reducing the solar/wind active power output). (Voltage Sag/Low MVAR) - Students will set a condition where the voltage at the PCC rises, or the system needs to absorb MVAR (e.g., by reducing an inductive load or increasing capacitive load, or by increasing solar/wind active power output into a weak grid). (Voltage Swell/High MVAR) - Record the new filter settings, voltage, and MVAR output for both sag and swell. - Students will make real-time (or near real-time in a simulator) adjustments to the filter settings to maintain a target MVAR level or a target voltage level at the PCC, demonstrating power quality management. Furthermore, Observing the immediate impact on MVAR and voltage. - Students will set a condition where the voltage at the PCC rises, or the system needs to absorb MVAR (e.g., by reducing an inductive load or increasing capacitive load, or by increasing solar/wind active power output into a weak grid). (Voltage Swell/High MVAR) - Record the new filter settings, voltage, and MVAR output for both sag and swell. - Students will make real-time (or near real-time in a simulator) adjustments to the filter settings to maintain a target MVAR level or a target voltage level at the PCC, demonstrating power quality management. Furthermore, Observing the immediate impact on MVAR and voltage. | Theory: 2 Hrs. Practical: 8 Hrs. Total: 10 Hrs. | Non consumable: - Power System Simulation Software e.g., ETAP, DIgSILENT PowerFactory, PSS/E, PSCAD/EMTDC including components: - grid-connected PV arrays - wind turbines, - reactive power compensation - devices (e.g., switched capacitor banks, - shunt reactors, - STATCOMs/SVCs - Monitoring tools for MVAR, voltage, and power factor at various points. - Small-scale grid-tied solar PV inverter (with reactive power control capability if available). - Small-scale grid-tied wind turbine inverter (with reactive power control capability if available). - Physical switchable capacitor bank and/or inductor bank for MVAR compensation. - Multi-function power quality analyzer or MVAR meter capable of measuring reactive power, voltage, and power factor. Consumable Items: - Stationary items | - Well-equipped classroom - Lab setup |
| Maintain the desired (50Hz) Frequency | - Identify the frequency monitoring system and control panel. - Check the current system frequency and compare it with the desired 50Hz. - Adjust the system settings (e.g., generator load or input) to maintain the 50Hz frequency. - Monitor the frequency continuously to ensure it stays at 50Hz. - Perform Fine-tuning of system adjustments if the frequency deviates from 50Hz. | - Identification of frequency monitoring systems (e.g., digital meters, SCADA interfaces) - Procedures to check and record the current system frequency and comparing it with the target of 50Hz - Methods to control frequency through generator load adjustments or power input changes - Utilization of Automatic Generation Control (AGC) systems, governor control, and droop settings for frequency control - Techniques for fine-tuning system parameters to maintain or restore 50Hz - Importance of coordination with other power sources in a multi-generator environment to resolve transient events such as load surges or supply fluctuations - Monitoring techniques like alarms, trend data, and auto-logging tools to detect deviations early and implement real-time corrective actions Practical Activity: Students navigate the simulation software, locating the main system frequency display, individual generator control panels (governors), load control interface, and the AGC interface. Students will run predefined scenarios in the simulator. For each scenario: Scenario 1: Load Surge: Record the initial stable frequency, the load surge event, the minimum frequency reached, and the time taken for the primary response to stabilize it (even if not at 50Hz). Scenario 2: Generator Trip: Record the initial stable frequency, the generator trip event, the minimum frequency reached, and the immediate system response. Scenario 3: Excess Generation/Load Shed: Record the initial stable frequency, the event, and the maximum frequency reached. Students use the manual governor control (speeder gear) to bring the frequency back to 50Hz. They must make incremental adjustments, observing the frequency response. Record the steps taken, the adjustments made, and the resulting stable frequency. | Theory: 1 Hrs. Practical: 8 Hrs. Total: 9Hrs | Non consumable: - Advanced power system simulation software e.g., ETAP, DIgSILENT PowerFactory, PSS/E, PSCAD/EMTDC capable of modeling - synchronous generators, - prime movers with governors (with adjustable droop), - various load types, - simulated grid with frequency monitoring. - Lab setup: A physical small synchronous generator with a controllable prime mover (e.g., DC motor with speed control), a frequency meter, and a load bank can illustrate the basic relationship. However, complex multi-generator and AGC concepts are best done in simulation. Consumable Items: - Stationary items | - Well-equipped classroom - Lab setup |

0713E&E3707: Manage Load dispatching

Objective: This Module covers the under-pinning knowledge and skills required for a student/trainee to be competent in monitoring, implement load shedding strategies and communication. After completing the Module, the students/Trainees will be able to perform all the tasks covered in (LU1 to LU5) in accordance with the essence of the Competency-Based Training System

Duration: 31 Hours                                    	Theory: 9 Hours           	                	Practice: 22 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Monitor Power Demand & Supply | - Identify demand requirement with respect to load - Check the generation Capability - Adjust the difference between Demand & Supply - Record difference between Demand & Supply - Report recorded data. | - Demand requirement with respect to load. - Verification of operational status and output capacity of generation units. - Optimization of system performance by aligning energy supply with consumption levels - Fluctuations between energy supply and usage - Reporting procedure Practical activity Monitor power demand & supply using simulation software .Adjust difference between Demand & Supply. Record your finding | Theory: 2Hrs. Practical: 4Hrs. Total: 6Hrs. | Consumable items: - Notepad - Pen - Pencil Non-Consumable items: - Personal Computer - Internet Connection - Telephone or other communication device (carrier) | - Well-equipped classroom - Computer Lab |
| Optimize Power Distribution | - Check requirements for optimization - Evaluate load distribution - Monitor over-loading of system. - Adjust load variation as per demand. | - Evaluation of operational conditions to identify optimization needs - Evaluation of load distribution - Responding to overloading of system - Optimization of load variation with respect to current demand. Practical activity Adjust load variation as per demand. | Theory: 2Hrs. Practical: 6Hrs. Total: 8Hrs. | Consumable items: - Notepad - Pen - Pencil Non-Consumable items: - Personal Computer - Internet Connection - Telephone or other communication device (carrier) | - Well-equipped classroom - Computer Lab |
| Implement Load management/ Shedding Strategies | - Analyze excess demand of load - Check distribution load as per system - Select Areas for load shedding based on its importance - Implement load shedding Schedule | - Illustration of excess load demand - Evaluation of conditions causing excessive energy demand. - Load distribution techniques - Categorization criteria for load shedding - Framing & Implementation of load shedding schedule Practical activity Frame and Implement load shedding schedule for an area based on its importance | Theory: 2Hrs Practical: 4Hrs Total: 6Hrs | Consumable items: - Notepad - Pen - Pencil Non-Consumable items: - Personal Computer - Internet Connection - Telephone or other communication device (carrier) | - Well-equipped classroom - Computer Lab |
| Coordinate with Generation Sources | - Identify Power Generation Sources - Analyze Power Generation Capability - Monitor Power Supply Parameters - Perform Synchronization with Grid in case of multiple generation sources - Communicate Power Stations staff - Handle Power Contingencies - Maintain Coordination Logbook | - Power generation sources - Evaluation of power generation capability - Supervision of power supply conditions to maintain system performance - Synchronization of generation sources with the main grid - Communications with power station’s staff - Handling of unexpected power disruptions - Managing coordination logbook . Practical activity Generate and Analyze Report based on Coordination with Generation Sources to handle power Contingencies | Theory: 2Hrs. Practical: 4Hrs. Total: 6Hrs. | Consumable items: - Notepad - Pen - Pencil Non-Consumable items: - Personal Computer - Internet Connection - Telephone or other communication device (carrier) | - Well-equipped classroom - Computer Lab |
| Perform Communication with Operators & Stakeholders. | - Identify the cause of faults - Check the available sources for communication. - Communicate with the concerned person/Stakeholder. - Maintain information clarity at the floor - Interpret signage language at the workplace | - Types of sound files - Understanding of licensing agreements - Introduction to audio formats - Selecting music and sound effects according to the mood and tone in animation - Audio Layer key framing and transitions - Synchronizing Audio and Graphics for audio-visual harmony Practical activity Interpret signage language at the workplace to communicate regarding the faulty component/machine | Theory: 1Hrs Practical: 4Hrs Total: 5Hrs | Consumables Items: - Notepad - Pen - Pencil Non-Consumables items: - Personal Computer - Internet Connection - Telephone or other communication device (carrier) | - Well-equipped classroom - Computer Lab |

0713E&E3708: Manage Load Stability

Objective: This Module covers the under-pinning knowledge and skills required for a student/trainee to be competent in monitoring voltage frequency level, handling emergency level. Balance load and generation, implement AVR. After completing the Module, the students/Trainees will be able to perform all the tasks covered in (LU1 to LU6) in accordance with the essence of the Competency-Based Training System.

Duration: 32 Hours			Theory: 8 Hours				Practice: 24Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Monitor Voltage & Frequency Levels | - Record voltage level at Grid - Record frequency levels - Maintain variation of grid as per standard. - Maintain log-book for grid parameter | - Nominal voltage and frequency values (e.g., 230V, 415V, 50Hz, 60Hz) and their tolerance ranges. - Instruments used for measurement of Grid parameters. - Causes of voltage and Frequency fluctuations - Control techniques for stability of micro grid (Droop control, Battery system for frequency support, load shedding) - Data logging system. Practical activity Monitor voltage and frequency levels and record the findings. | Theory: 1Hrs. Practical: 4Hrs. Total: 5Hrs. | Consumables Items: - Notepad - Pen Non-Consumables items: - Computer/Simulator - PPE - Digital multimeters - Clamp meters (AC/DC) | - Classroom - Lab setup / Work shop |
| LU2. Handle Emergency situation of Microgrid | - Locate emergency fault - Identify nature of fault - Isolate emergency faulty component/equipment in grid - Perform emergency shutdown for fault clearance - Report the emergency incident. - Replace Faulty component in Grid - Test operation of grid after fault rectification | - Introduction to common emergency scenarios and risks in micro grids - Types of faults and protection systems in micro grids - Techniques for fault location and nature of fault - Faulty component isolation and emergency shutdown procedure - Importance of incident reporting and communication - Fault rectification techniques - Post-repair re-energizing and testing the system Practical Activity: You notice an emergency condition in micro grind. You are required to locate the fault, isolate the faulty component and perform emergency shutdown for rectification. | Theory: 2Hrs. Practical: 5Hrs. Total: 7Hrs. | Consumables Items: - Notepad - Pen Non-Consumables items: - Personal Computer - Internet Connection - PPEs - Tools and equipment's | - Classroom - Lab setup |
| LU3. Perform Balancing of Load and Generation. | - Analyze load demand as per generation capacity. - Match demand & supply as per real-time consumption. - Distribute load effectively | - Load balancing and its importance in microgrid stability. - Types of loads(resistive, inductive, constant, variable, critical and non-critical). - Introduction to active power (P), reactive power (Q), voltage (V), impedance (Z), current(I). - Definition of base load and peak load - Common factors influencing electricity demand (weather, time of day, industrial activity). - Role of system operators and electricity control centers. - Micro grid load balancing techniques (Manual/Automatic load shedding or switching off non-critical loads etc.) - Understanding of load shedding mechanisms and frequency regulation. Practical activity Balance generation and load of microgrid by manually switching on/off different loads | Theory: 2Hrs Practical: 5Hrs Total: 7Hrs | Consumables Items: - Notepad - Pen Non-Consumables items: - Computer/Simulator - PPE - Digital multimeters - Clamp meters (AC/DC) | - Classroom - Lab setup |
| LU4. Integrate Battery Energy Storage Systems (BESS) | - Configure battery energy storage systems based on grid demand. - Connect battery energy storage units to Microgrid. - Verify charge cycles for efficiency. - Verify discharge cycles for efficiency. | - Configuration and optimization of battery storage system according to grid demand - Integration of battery energy storage units into the micro grid - Verification of charge and discharge cycles for efficiency Practical activity Perform charging and discharging cycles to find efficiency of BESS system of a microgrid. | Theory: 2Hrs. Practical: 4Hrs. Total: 6Hrs. | Consumables Items: - Notepad - Pen Non-Consumables items: - Personal Computer - Internet Connection - Battery system | - Classroom - Lab setup |
| LU5. Implement Automatic Voltage Regulation (AVR) | - Evaluate AVR settings based on system voltage requirements. - Monitor voltage regulation performance. - Analyze AVR parameters for stable voltage. - Record AVR parameters | - Assessment of AVR setting based on system voltage requirements - Supervision and monitoring of voltage regulation to ensure stable performance. - Assessment/Examination of AVR parameters for reliable and stable voltage regulation. - Record keeping of AVR Parameters Practical activity Monitor AVR parameters of a microgrid system and record the findings. | Theory: 1Hrs Practical: 6Hrs Total: 7Hrs | Consumables Items: - Notepad - Pen Non-Consumables items: - Personal Computer - Internet Connection - AVR | - Classroom - Lab setup / Work shop |

0713E&E3709: Maintain Inventory of Microgrid System

Objective: This Module covers the underpinning knowledge and skills required for a student/trainee to be competent in Maintaining Inventory of Microgrid System. After completing the Module, the students/Trainees will be able to perform all the tasks covered in (CU1 to CU7) in accordance with the essence of the Competency-Based Training System

Duration: 71 Hours			Theory: 9 Hours				Practice: 62 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Maintain Tools, Equipment & spares Inventory | - Identify the tool/Equipment required for Microgrid. - Categorize the equipment/tools - Place / store all the tools/equipment at designated place. - Record the regular cleaning /sharpening /lubricating of the equipment/tools - Apply software tool for inventory management - Check smooth functioning of the tools/ equipment - Update Equipment Records Regularly - Generate Reports on Tools & Equipment Inventory as per SoP. | - Identification & classification of essential microgrid tools and equipment. - Identification of designated places of tools/equipment. - Storage protocol for battery operated tools and equipment. - Storage protocols for mechanical equipment requiring lubrication. - Introduction to inventory management software for effective resource management - Importance of record updating Practical Activity You have recently joined a microgrid maintenance team as a technician. Your are assigned the task to organize the maintenance workshop, verify all tools/equipment, ensure their proper functioning, and update the inventory records, including a report submission based on SOPs. | Theory: 2Hrs. Practical: 14Hrs. Total: 16Hrs. | Consumable items: - Pen - Notebook Non-Consumable items: - Personal Computer - MS Office | - Classroom - Computer Lab |
| LU2. Maintain Faults Record | - Enlist the number of Faults - Categorize the nature/intensity of faults - Follow Tracking System for each fault - Update Faults Records Regularly - Generate Reports on faults Inventory | - Types of common faults in Microgrid sysytem - Categorization of the nature/intensity of faults - Fault tracking system for monitoring and resolution. - Generation of fault inventory report. Practical Activity: Students are grouped as Microgrid Maintenance Teams. Each group receives a set of fault scenario cards that describe different equipment issues (realistic but simulated). Their task is to record, classify, track, and report the faults using the formats provided. | Theory: 2Hrs. Practical: 12Hrs. Total: 14Hrs. | Non-Consumable items: - Fault scenario cards | - Classroom - Computer Lab |
| LU3. Maintain Shift Record | - Maintain human resource record - Check generation & Transmission log book - Check system conditions - Identify low stock items - Record materials used - Maintain inventory storage of safety, labeling, and organizational standards. - Perform Handing / Taking over the shifts | - Introduction to human resource record management. - Introduction to shift logbook - Essential elements of logbook. - Material tracking and usage reporting techniques - Shift changeover protocols and operational continuity. - Maintaining inventory storage of safety, labeling, and organizational standards. Practical Activity Students will work in pairs or small teams to simulate a microgrid technician's duties during a shift. One group represents the outgoing shift, while another represents the incoming shift. They will use pre-filled log templates and scenario cards to practice reviewing, updating, and handing over shift data. | Theory: 1Hrs Practical: 12Hrs Total: 13Hrs | Non-Consumable items: - Shift logbook | - Classroom - Computer Lab |
| LU4. Maintain Supplier and Procurement Records | - Enlist the items received - Enlist the items consumed - Ensure that the faulty or damaged parts are dumped at designated places - Apply Green (Environmental friendly) practices in waste disposal and management process - Update the procurement inventory | - Inventory Management Processes - Material consumption and usage tracking system. - Introduction to Green ( Environmental friendly) practices - Adoption of effective/ Green waste disposal and management practices Practical Activity Students are assigned to work as inventory and maintenance assistants at a simulated microgrid warehouse. They are given inventory records, material request slips, and waste disposal situations. Their job is to track material usage, update records, and manage safe and Green disposal according to SOPs. | Theory: 2Hrs. Practical: 12Hrs. Total: 14Hrs. | Non-Consumable items: - Material consumption register - Material request slips - Material issuance register | - Classroom - Computer Lab |
| LU5. Implement Barcode/ RFID System | - Assess Barcode/RFID System Requirements - Plan Barcode/RIPD Implementation - Prepare Barcode/RIPD Implementation - Tag Inventory with Barcodes/RFID - Configure the System - Test the System - Monitor System Performance and Optimize Usage | - Introduction to Barcode and RFID technology - Barcode/ RFID system setup and configuration. - Data encoding and tagging methods - Techniques for effective use of Barcode in inventory management. - Troubleshooting of common Barcode/ RFID issues - Monitoring of System Performance and Optimize Usag Practical Activity you are , as an inventory assistant in a microgrid store, receive various tool/equipment samples (real or labeled boxes), each with Barcodes. Using a Barcode reader, scan items, check database entries, create new item labels, and simulate resolving scanning issues. | Theory: 2 Hrs. Practical: 12Hrs Total: 14Hrs | Non-Consumable items: - Barcode scanner - Electrical and mechanical tools - Barcode printer - Personal computer - MS Office | - Classroom - Lab setup / Work shop |

0713E&E3710: Perform trouble shooting of Microgrid station

Objective: This Module covers the under-pinning knowledge and skills required for a student/trainee to be competent to Perform trouble shooting of Microgrid station After completing the Module the students/Trainees will be able to perform all the tasks covered in (LU1 to LU6) in accordance with the essence of the Competency-Based Training System.

Duration: 74 Hours                                   	Theory: 6 Hours    	                                	Practice: 68 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Diagnose Faults | - identify symptoms of faults - Identify the required diagnostic tools and techniques - Interpret system data, logs, and alarms to locate faults - Apply troubleshooting procedures based on system design - Document diagnostic process, findings, and recommended actions | - Introduction to fault’s symptoms and system’s indicators - Identification of diagnostic tools, fault’s symptoms and system’s indicators - Techniques for diagnosing faults - Interpretation of system data, logs and alarms to trace fault origins. - Fault analysis using system schematics and design parameters - Maintaining structured record of troubleshooting and remedial recommendations Practical activity Perform physical inspections of Microgrid for fault detection | Theory: 1Hr. Practical: 8 Hrs. Total: 9Hrs. | Non-Consumable items: - Tools and equipment - PPEs | - Classroom - Lab setup / Work shop |
| Isolate Faulty Components | - Select necessary tool and equipment to isolate the faulty component - Follow standard procedures to safely isolate the faulty component - Handle the faulty component as per SOPs | - Identification of tools and equipment to isolate the faulty component - Safety guidelines for isolation of faulty section - Importance of handling faulty component as per SOP’s/ guidelines Practical activity Isolate Faulty Components following the SOPs and safety guidelines. | Theory: 1 Hr. Practical: 13Hrs. Total: 14Hrs. | Non-Consumable items: - Tools and equipment - Safety PPEs | - Classroom - Lab setup / Work shop |
| Test Faulty Components | - Conduct functional and electrical tests to confirm the fault - Record the diagnosed fault - Recommend necessary repairs or replacements based on findings | - Identification of testing tools/equipment - Fault verification techniques - Procedure for conduct of functional and electrical testing - Record keeping of the diagnosed faulty components. - Recommendations for repairs/replacements of faulty components Practical Activity Carryout functional and electrical tests of the component to confirm the fault | Theory: 1 Hrs. Practical: 13 Hrs. Total: 14Hrs. | Non-Consumable items: - Tools and equipment - Safety PPEs | - Classroom - Lab setup |
| Rectify the Fault | - Follow standard procedures and manufacturer guidelines - Rectify fault using the desired tools and equipment - Test the repaired or replaced component for functionality - Verify system stability after rectification - Restore the Microgrid to normal operation | - Standard Operating Procedures and Equipment Manuals for rectification of fault - Identification of the required tools/equipment - Rectification techniques for removal of fault. - Testing procedure for the repaired/replaced components for smooth functioning - Restoration of micro grid to normal operation/stability after rectification of fault. Practical activity Check system stability after rectification of fault to restore the micro grid to normal operation | Theory: 1 Hr. Practical: 13 Hrs. Total: 14Hrs. | Non-Consumable items: - Tools and equipment - Safety PPEs | - Classroom - Lab setup / Work shop |
| Analyze power quality and load behavior after rectification | - Assess load behavior under different operating conditions - Identify any remaining issues or inefficiencies in the system - Verify proper synchronization of Microgrid components - Analyze system response to varying loads and demand changes - Document findings and corrective measures taken - Recommend further adjustments or optimizations if needed | - Load performance evaluation across various operating scenarios - Identification of any unresolved problems or performance gaps in the system - Synchronization of Micro grid components - Monitoring of system response during load fluctuations and demand adjustments - Record maintenance of the identified faults and resolution steps - Recommendation for supplementary changes or upgrades. Practical activity Assess and report load behavior under different operating conditions after rectification of fault | Theory: 1 Hr. Practical: 13 Hrs. Total: 14 Hrs. | Non-Consumable items: - Computer - Power Analyzer Consumable item - Pen - notebook | - Classroom - Computer Lab |
| Prepare report | - Collect relevant data from diagnostics, testing, and rectification - Organize and summarize information in a clear and structured format - Maintain proper documentation for reference and compliance | - Data collection procedure - Data organization in the formal format - Maintaining record of results for future reference Practical activity Prepare report,regarding fault diagnosis, testing, and rectification of component as per the assigned task. | Theory: 1 Hrs. Practical: 8 Hrs. Total: 9Hrs. | Non-Consumable items: - Personal Computer Consumable item - Pen - notebook | - Classroom - Computer Lab |

0713E&E3711: Develop Entrepreneurship/Business

Objective: This Module covers the underpinning knowledge and skills required for a student/trainee to be competent in developing entrepreneurial skills and business operations within the energy sector. After completing this Module, the Trainee/student will be able to successfully perform effective communication, business structuring, financial management, customer engagement, and regulatory compliance in accordance with the essence of the Competency-Based Training System. After completing the module, the students/Trainees will be able to perform all the tasks covered in (LU1 to LU11) in accordance with the essence of the Competency-Based Training System.

Duration: 35 Hours			Theory: 5 Hours				Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Communicate effectively in a professional environment | - Enlist appropriate technical terminology in energy sector. - Deliver clear and concise messages to team members &supervisors. - Demonstrate active listening skills in discussions. - Deliver communication style based on the audience | - Key Technical terms in energy sector - Active listening and Message precision - Audience centred communication skills - Communication Tools and Channels Activity 1:· The team leader must deliver a 2-minute briefing about an energy system issue (e.g., power load imbalance, maintenance delay) using appropriate technical terms. | Theory: 2 Hr Practical: 0 Hrs Total: 3 Hrs | Chart papers Marker White board Projector Chart Paper Ball pen Note book Stop watch | - Class Room |
| Prepare reports and documentation | - Prepare daily operation log - Prepare document for maintenance records - Document the troubleshooting reports. - Maintain organized record-keeping system. | - Daily Operation Logs - Maintenance activity - Effective Record keeping system - Documentation accuracy Activity Task 1: Create a Daily Operations and Maintenance Logbook Activity 2: Write a Troubleshooting Report including: Problem Description Date/Time Reported Root Cause Steps Taken Final Outcome Technician's Remarks | Theory: 1 Hr Practical: 3 Hrs Total: 4 Hrs | Chart papers Marker White board Projector Chart Paper Ball pen Note book | - Computer Lab |
| Develop customer relationship and service skills | - Address and resolve customer concerns related to energy sector services. - Educate consumers about energy efficiency and sustainable solutions. - Maintain professionalism while interacting with clients and stakeholders. - Build long-term relationships by ensuring customer satisfaction and trust. | - Techniques to resolve customer concerns - Sustainable energy solutions - Importance of conduct in professional relationships - Strategies for long term customer satisfaction Activity 1: Design a brochure or flyer aimed at residential or commercial customers. Include: - Tips for saving energy at home/work - Benefits of using renewable energy - Contact info or support channels | Theory: 1 Hr Practical: 3 Hrs Total: 4 Hrs |  | - Classroom - Computer Lab |
| Analyse business fundamentals in the energy sector | - Identify the basics of energy business models and revenue generation. - Identify key stakeholders involved in energy projects. - Analyze cost and pricing structures for energy distribution. - Evaluate market trends and business opportunities within the energy sector. | - Energy models and revenue streams - Responsibilities of stake holders in energy projects - Cost structures - Pricing mechanism - Maeket trends evaluation - Business opportunities in energy sector Activity 1: Prepare a Cost Estimation for a given energy Project | Theory: 0 Hr Practical: 3 Hrs Total 3 Hrs | - Flip charts or whiteboards, markers, reference handouts | - Computer Lab |
| Apply financial management skills | - Prepare cost estimations for energy projects. - Track operational expenses and recommend cost-saving measures. - Identify basic financial terms related to energy businesses. - Develop budgeting strategies to enhance financial sustainability. | - Basic Financial terms - Cost estimations for energy projects - Methods to maintain on going cost - Techniques to anlayze expenses - Financial sustainability budgets Activity 1: Sketch a simple flow diagram showing how money, energy, and information move between stakeholders. | Theory: 1 Hr Practical: 3 Hrs Total: 4 Hrs | Chart papers Marker White board Projector Chart Paper Ball pen Note book | - Computer Lab - Media Lab - Classroom |
| Follow entrepreneurial best practices | - Identify market opportunities for energy services. - Develop strategies for business growth and customer acquisition. - Implement risk management techniques for sustainable business operations. - Enlist funding opportunities for energy projects. - Enlist investment opportunities for energy projects. | - Exploring market oppurtunites - Business growth stratgies - Techniques of risk management for business sustainability - Funding Avenues - Seizing investment opportunities Activity 1: Research local or regional energy challenges (e.g., load shedding, high energy cost, low access to renewable). Activity 2: Present the findings on a poster/chart and suggest one potential energy-based business idea that solves a key issue. | Theory: 0 Hr Practical: 3 Hrs Total: 3 Hrs | Chart papers Marker White board Projector Chart Paper Ball pen Note book | - Media Lab - Creative Studio - Outdoor Spaces |
| Implement safety and regulatory compliance | - Enlist the legal and safety requirements for energy operations. - Enlist government policies and licensing requirements. - Enlist occupational health and safety regulations. - Enlist environmental compliance and sustainable energy practices. | - legal and safety requirements - government policies and licensing requirements. - Occupational health and safety regulations - Sustainable energy enterprenueral practices Activity 1: Research and list: Relevant occupational health and safety (OHS) regulations. Environmental compliance obligations. Sustainable practices applicable to the chosen business. | Theory: 0 Hr Practical: 3 Hrs Total: 3 Hrs | Notebook Pens, markers, White board - chart papers | - Computer Lab - Classroom |
| Identify business opportunities in the Energy sector | - Conduct market research to identify potential energy business opportunities. - Analyze demand for decentralized energy solutions. - Evaluate the impact of renewable energy trends on energy businesses. - Develop a feasibility study for launching an energy service. | - Methods of market research - Needs of decentralized energy solutions - Businesses impact of renewable energy trends - Feasibility of Energy Sector Ventures Activity 1: Conduct a short market survey by interviewing at least 5 local households or small businesses. Activity 2: Collect information on: Energy usage patterns Current challenges with electricity supply | Theory: 0 Hr Practical: 3 Hrs Total: 3 Hrs | Survey questionnaire forms Clipboards pens Computer system Graph/ chart paper White board markers, | - Computer Lab - Seminar Room |
| Develop structure of the new Energy business | - Identify different business models for an energy company. - Establish roles and responsibilities within the energy enterprise. - Design an operational workflow for energy distribution. - Demonstrate compliance with industry standards and regulations. | - Organizational roles and responsibilities - Energy distribution work flow - Compliance with industry standards - Structure for sustainable business growth Activity 1: Create a mini business plan: A basic operational workflow diagram for energy distribution. Activity 2: Create a checklist of required standards and regulations that any energy company must follow (e.g., NEPRA, ISO standards, environmental laws). | Theory: 0 Hr Practical: 3 Hrs Total: 4 Hrs | Handouts of selected case studies, Computer system checklist templates. Ball pen Notebook | - Computer Lab - Interactive Classroom |
| Develop strategy to attract customers | - Create marketing and outreach campaigns to promote energy services. - Engage with potential customers through digital platforms and local events. - Build partnerships with stakeholders and community leaders. - Develop customer incentives to enhance service adoption. | - Energy services using various marketing technques - Digital Platforms for Customer Engagement - Strategic Partnerships with Stakeholders - Customer Incentives and Loyalty Programs Activity Task 1: Design a Marketing Campaign for an Energy Service Activity 2: Propose two types of incentives | Theory: 0 Hr Practical: 3 Hrs Total: 3Hrs | Chart paper poster boards ·Markers, pens, pencils, Computer Scissors, glue, stickers Printed handouts or templates | - Lab |
| Create portfolios on Facebook, Instagram and Linkedin | - Create a Facebook Page - Upload Portfolio Content on Facebook - Create a Facebook Group - Join already created Facebook groups for freelancers - Create Profile on Instagram - Upload Portfolio Content on Instagram - Create LinkedIn Profile, company page and showcase page - Create LinkedIn group - Join LinkedIn groups for freelancers | - Professional Presence on Facebook - Instagram - LinkedIn Identity Activity 1: Create a LinkedIn Group nd write a welcome message post. Activity 2: Upload 3 sample portfolio posts (images with captions) showcasing your project on both Facebook and Instagram. | Theory: 0 Hr Practical: 3 Hrs Total: 3 Hrs | Computer system Note book Ball pen | - Computer Lab |

0713E&E3712: On-Job Training/Project

Objective: The on-job training component of this qualification provides learners with the opportunity to apply their acquired skills and knowledge in real world environment. During this phase, students/Trainees must take on entry level or support roles such as Microgrid technician. This role span across diverse industries including Hydel, Wind, and Solar Power Generation Systems or any other relevant industry that enable learners to contribute to power generation while fulfilling the requirements of the qualifications.

As alternate arrangement, in case of non-availability of the concerned industry, the students/Trainees may fulfill requirements of the qualification through conduct of Students Project on the pattern of final year students/Trainees of Diploma of Associate Engineers (DAEs) or Engineering Universities. This would, no doubt, cover industrial aspect of the qualification.

- List of Tools and Equipment Required for the Qualification:

| S.No | Items | Quantity |
| --- | --- | --- |
| 1 | Personal Protective Equipment’s (PPEs) (Safety Gloves (upto rated 2kV&18kV), Safety Helmets, Safety Glasses, Arc Flash Suit, Safety Shoes, Full Body Harness, Safety Belt, Ear Plugs, Ear Moff, Apron, Safety Jacket) | 25 Sets |
| 2 | Fire Extinguishers (CO2, DCP) | 05 Nos each |
| 3 | Fire Hydrants | 02 Nos |
| 4 | Fire Ball Extinguisher | 02 Nos |
| 5 | Fire Blankets | 25 Nos |
| 6 | Sand Buckets | 25 Nos |
| 7 | First Aid Kit | 02 Nos |
| 8 | Safety Mask N95 | 25 Nos |
| 9 | Portable Temporary Ground | 25 Nos |
| 10 | Grounding Rods & Cables | 25 Nos |
| 11 | Lockout/Tagout (LOTO) Kits | 25 Nos |
| 12 | Safety Barriers & Cones | 25 Nos |
| 13 | Smoke & Gas Detectors | 05 Nos |
| 14 | Emergency Alarm Systems | 05 Nos |
| 15 | Digital Multimeter | 25 Nos |
| 16 | DC/AC Clamp Meter | 25 Nos |
| 17 | Power Quality Analyzer | 25 Nos |
| 18 | Infrared Thermometer | 25 Nos |
| 19 | Motor Generator Set | 25 Nos |
| 20 | Fuel Flow Meter (For Diesel Generators) | 25 Nos |
| 22 | Solar Irradiance Meter (For Renewable Systems) | 25 Nos |
| 23 | Lockout/Tagout (LOTO) Kit | 25 Nos |
| 24 | Grounding Rods & Clamps | 25 Nos |
| 25 | Insulation Resistance Tester | 25 Nos |
| 26 | Circuit Breaker Analyzer | 25 Nos |
| 27 | Synchroscope | 25 Nos |
| 28 | Magnetic Contactor 220V | 25 Nos |
| 30 | Magnetic Contactor 400V | 25 Nos |
| 31 | On-Off Push Buttons | 25 Nos |
| 32 | Rev-Stop-For Push Button | 25 Nos |
| 33 | Thermal Overload | 25 Nos |
| 34 | Phase Sequence Relay | 25 Nos |
| 35 | Current Transformer (CTs) | 25 Nos |
| 36 | Potential Transformers (PTs) | 25 Nos |
| 37 | Galvanometer | 25 Nos |
| 38 | Exciters | 25 Nos |
| 40 | Automatic Voltage Regulator | 25 Nos |
| 41 | Frequency Meter | 25 Nos |
| 42 | Single Phase Wattmeter | 25 Nos |
| 43 | Three Phase Wattmeter | 25 Nos |
| 44 | DC Wattmeter (100W) | 25 Nos |
| 45 | Digital Lux Meter | 25 Nos |
| 46 | Analog Lux Meter | 25 Nos |
| 47 | Solar Panels (180W, 585W) | 25 Nos each |
| 48 | Inverters (1000W, 2kW, 5kW) | 25 Nos |
| 49 | Controller Trainer | 25 Nos |
| 50 | Charge Controller PWM (20A, 30A) | 25 Nos each |
| 51 | Charge Controller MPPT (40A) | 25 Nos |
| 52 | Battery 100Ah | 5 Nos |
| 53 | Battery 200Ah | 5 Nos |
| 54 | Single Phase Energy meters (Analog and Digital) | 25 Nos each |
| 55 | Three Phase Energy meters (Analog and Digital) | 25 Nos each |
| 56 | Testing Board | 25 Nos |
| 57 | Single Pole Circuit Breaker (C6,C13,C63) | 25 Nos each |
| 58 | Two Pole Circuit Breaker (C6,C13,C63) | 25 Nos each |
| 59 | Three Pole Circuit Breaker (C6,C13,C63) | 25 Nos each |
| 60 | Power Factor meter | 25 Nos |
| 61 | kVAR meter | 25 Nos |
| 62 | Four Pole Circuit Breaker (C6,C13,C63) | 25 Nos |
| 63 | Barcode Scanners (For Equipment Tagging & Tracking) | 25 Nos |
| 64 | RFID Tags & Readers (For Asset Management) | 25 Nos |
| 65 | Handheld Label Printers (For Equipment & Component Labeling) | 25 Nos |
| 66 | Digital Weighing Scales (For Measuring Components & Materials) | 25 Nos |
| 67 | Log Books | 25 Nos |
| 68 | Microsoft Office 2024 | 05 Nos |
| 69 | Computer with Printer | 25 Nos |
| 70 | Telephone Set | 05 Nos |
| 71 | kVARs Meters | 25 Nos |

- List of Consumable Items Required for the Qualification:

| S. No. | Items | Quantity |
| --- | --- | --- |
|  | Cable (various gauges: 1.5mm², 2.5mm², 4mm², etc.) | 90m per type |
|  | Connectors & terminals (lug connectors, ring terminals) | 1000 pcs |
|  | Fuses (various ratings: 5A, 10A, 15A, 20A) | 100 each |
|  | Circuit breakers (MCB, MCCB, RCD) | 50 each |
|  | Batteries (for meters & analyzers, AA, AAA, 9V, lithium-ion) | 24 each |
|  | Insulation tape (red, black, yellow, green, blue) | 5 roll each |
|  | Heat shrinks tubing (assorted sizes) | 100m |
|  | Cable glands (assorted sizes) | 100 pcs |
|  | Cable lugs & ferrules | 1000 pcs |
|  | Safety gloves (electrical insulated, cut-resistant, nitrile) – | 50 pairs |
|  | Helmets | 50 pcs |
|  | Safety glasses | 50 pcs |
|  | Arc flash suits | 5 sets |
|  | Face masks (N95, surgical) | 300 pcs |
|  | Earplugs | 200 pairs |
|  | Face shields | 10 pcs |
|  | High-visibility vests | 50 pcs |
|  | Respirators | 10 pcs |
|  | Safety shoes (optional if required) | 50 pairs |
|  | Cable ties (various sizes) | 2000 pcs |
|  | Marking labels (self-adhesive, cable labels) | 500 pcs |
|  | Lockout/Tagout tags | 100 pcs |
|  | Industrial-grade lubricants & grease | 20 liters |
|  | Contact cleaner spray | 10 cans |
|  | Rust removers | 5 liters |
|  | Wire strippers & cutters (extra consumable blades) | 10 pcs |
|  | Calibration stickers & logbooks | 10 set |
|  | Ethernet cables (Cat 6, Cat 5e) | 1000m roll |
|  | RJ45 connectors | 500 pcs |
|  | Fiber optic cables | 500m |
|  | Fiber optic connectors (SC, LC, ST) | 100 pcs |
|  | Network switch replacement modules | 5 pcs |
|  | Power strips & surge protectors | 20 pcs |
|  | BNC connectors (for oscilloscopes) | 10 pcs |
|  | Fire extinguishers (CO₂, dry chemical, foam) | 2 pcs |
|  | Fire extinguisher refills | 5 sets |
|  | Fire blankets | 10 pcs |
|  | First aid kits | 10 kits |
|  | Emergency exit sign | 10 pcs |
|  | Emergency lights (batteries included) | 10 pcs |
|  | Eye wash station solution | 5 liters |
|  | Safety cones & barriers | 20 sets |
|  | Multimeter probes & leads | 50 sets |
|  | Clamp meter accessories (flexible CT coils, extra clamps) | 10 sets |
|  | Oscilloscope probes | 20 sets |
|  | Pressure gauge calibration kits | 5 sets |
|  | Infrared thermometer lens cleaning wipes | 50 packs |
|  | Battery analyzer test leads | 20 sets |
|  | Earth tester auxiliary electrodes | 10 sets |
|  | Whiteboard Markers | 50 sets |
|  | Whiteboard Cleaner | 10 bottles |
|  | Projector bulbs | 5 pcs |
|  | Presentation boards & flip charts | 5 pcs |
|  | Printed training manuals & datasheets | 25 copies |
|  | USB flash drives (for software & reports) | 50 pcs |
|  | Ethernet patch cables (for network devices) | 100 pcs |
|  | Barcode labels (for scanner testing) | 500 pcs |

- Members of Qualifications Development Committee

The following members participated in the qualification’s development and validation process:

| Qualification Development Committee |  |  |  |
| --- | --- | --- | --- |
| S. No. | Name |  | Organization |
|  |  | Engr. Abdul Maqsood | DACUM Facilitator KP-TEVTA |
|  |  | Engr. Hafiz Muhammad Asad | A. E.E, Tarbela Dam |
|  |  | Engr. Ahmad Wisal | A.E.E,Tarbela Dam |
|  |  | Engr. Waqas Niaz | SDO NPCC NTDC ,Islamabad |
|  |  | Engr. Abdul Wajid | AM Transmission NTDC, Bannu |
|  |  | Engr. Muhammad Basheer Khan | XEN Transmission 500KV NTDC Peshawar |
|  |  | Engr. Ghulam Hafeez | Lecturer GCT, Nowshera |
|  |  | Ms. Tabassum Feroz | Lecturer GTTC, Peshawar |