National Vocational Certificate level – 4 in Robotics Technology

(Robotics Supervisor)

Competency Standards

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

1.	Introduction	4

2.	Purpose of the Qualification	5

3.	Date of Validation	5

4.	Date of Review	5

5.	Codes of Qualifications	6

6.	Members of Qualification Development Committee	7

7.	Members of Qualification Validation Committee	8

8.	Entry Requirements	9

9.	Regulations for the Qualification and Schedule of Units	9

10.	Mapping of the Qualification	9

11.	Common Courses with respective levels	10

12.	Packaging of Qualifications (with respect to occupations)	10

13.	Summary of Competency Standards	11

14.	Detail of Competency Standards	13

Soft Skills	13

0714E&A1.	Develop workplace policy and procedures for sustainability	13

0714E&A2.	Maintain Professionalism in the Workplace	16

0714E&A3.	Manage personal work priorities and professional development	18

0714E&A4.	Manage workforce planning	20

0714E&A5.	Undertake project work	22

0714E&A6.	Prepare and implement negotiation	25

0714E&A7.	Organize schedules	28

0714E&A8.	Identify and communicate trends in career development	30

A.	Digital Electronics	32

0714E&A9.	Design Operation Amplifier	32

0714E&A10.	Verify Truth Tables of Digital Gates	34

0714E&A11.	Construct & Verify Combinational logic circuit	36

0714E&A12.	Construct and Verify function of Flip Flops	39

0714E&A13.	Use 555 IC as Multivibrator	41

0714E&A14.	Construct Shift Registers and Counters Used Flip Flops	43

B.	AC & DC machines (Motor and Generator)	45

0714E&A15.	Verify Basic Laws of Electrical Machines	45

0714E&A16.	Analyse Electrical Generators	48

0714E&A17.	Perform tests on DC Motors	52

0714E&A18.	Operate and test  stepper and servo motors	56

C.	Robotic Programming	58

0714E&A69: Object Oriented Programming-I	58

0714E&A70:Object Oriented Programming - II	60

Robotic System	62

0714E&A19.	Identify various parts of Robotics.	62

0714E&A20.	Assemble various Robotic parts.	64

0714E&A21.	Robotics Control Boards	66

0714E&A22.	Programmable Logic controller PLC	68

0714E&A23.	Field Programmable logic controller	69

0714E&A24.	Arduino Development Board	71

0714E&A25.	Develop Robot program.	74

0714E&A26.	Develop Robotic arm structure	76

0714E&A27.	Configure and test robotic arm	78

0714E&A28.	Develop Object Recognition System	80

0714E&A29.	Perform Assembly and Operation of Remotely Operated Robot	84

D.	Engineering Drawing	88

0714E&A30.	Perform Basic Manual Drawing	88

0714E&A31.	Construct different Engineering Curves	91

0714E&A32.	Construct multi-view drawings	95

0714E&A33.	Install  CAD software	98

0714E&A34.	Develop 2D drawings using CAD software	100

0714E&A35.	Develop 3D drawings using CAD software	102

# Introduction

This qualification gives an introduction to robotics so that the student will have the competence to supervise and implement robotic systems. This is done by presenting relevant topics from geometry, kinematics, dynamics, control systems, and Programing techniques. . Students will gain hands on experience in robotic assembling, synchronization with system, repair & maintenance along with the some fabrication and machining work.

The National Skills Strategy (NSS) aims at achieving a paradigm shift from time-bound and supply lead to competency-based and demand driven training in Pakistan. Competency-based training approach focuses on the demonstration of actual skills required in the workplace/industry. To achieve this strategy, NAVTTC in collaboration with GIZ is involved in the development process of CBT program to ensure competent skilled labor in demand driven vocational trades.

National Vocational Qualification Framework (NVQF) as designed in consultation with the stakeholders including academia, researchers, industry, chambers and TEVTAs. Its aim is to identify knowledge & skills required for a qualification, determine equivalence, provide guidelines for Recognition of Prior Learning (RPL) and assure quality of training. NVQF designed for the Vertical and horizontal progression for learners, recognition of qualification, meeting with the national and international standards, facilitate conversion of informal training to formal training through RPL, improvement in quality of training and increased options for learners for selecting training programs in different trades. Competency Standards are performance specification that identify the knowledge and competencies an individual need to succeed in the workplace.

The National Competency Standards could be used as a referral document for the development of curriculum to be used by training institutions.

# Purpose of the Qualification

This qualification will help student to supervise and implement robotic systems. The candidate will be able to perform fundamental operations of robot logic functions, systems, understand the concepts of various digital systems, their functions and designs, grasp concepts of various memory structures, register counter and their integration with various digital system and there troubleshooting. This qualification will help candidate in assembling, synchronization and construction of the robotic system, identify and implement certain robotic components, elaborate linkage between the components and their industrial workability & maintenance of various robotic devices.

This course enables the youth with greater employment opportunities and entrepreneurial skills.

# Date of Validation

The level 5 (Diploma) of National vocational qualification on “Robotics” has been validated by the Qualifications Development Committee (QDC) members on 22-24 August 2019 at Karachi and will remain valid for ten years i.e. 24 August,2029

# Date of Review

The level 5 (Diploma) of National vocational qualification on “Robotics” has been validated by the Qualifications Development Committee (QDC) members on 22-24 August 2019 at Karachi and shall be reviewed after three years from the date of validation i.e. 25 August,2022

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Robotics Technician level 5 |  |
| --- | --- |
| Code | Description |
| 0714E&A(4) | 4th Level D.A. E National Certificate of level-5, in “Robotics Technology” |

# Members of Qualification Development Committee

The following members participated in the Qualification development process.

| S# | Name | Organization |
| --- | --- | --- |
| 1 | Prof. Dr. Zeeshan Ali Memon (Chairman Qualification Development Committee) | Head of Industrial Engg. Deptt. Dawood University of Engg. & Tech. |
| 2 | Engr. Nimra Ghazal | Dawood University of Engg. & Tech. |
| 3 | Engr. Javed Larik | Mehran University of Engg. & Tech. |
| 4 | Engr. Muhammad Ismail | Dawood University of Engg. & Tech. |
| 5 | Dr. Abid Khan | APC&IC Lab, PCSIR |
| 6 | Mr. Muhammad Aasim | CASE School of Robotics. |
| 7 | Mr. Usama Khan | PCI, Automotive Pvt. Ltd. |
| 8 | Mr. Babar Sayeed | PSTC, PCSIR |
| 9 | Engr. Dr. Rana Javaid Masood | Usman Institute of Technology |
| 10 | Engr. Aurangzaib Ansari | Usman Institute of Technology |
| 11 | Engr. Talha Tariq | Dawood University of Engg. & Tech. |
| 12 | Dr. Arsalan Ansari | Head of Electronics Dawood University of Engg. & Tech. |
| 13 | Mr. Liaqat Ali Jamro | Director Academics, STEVTA |
| 14 | Mr. Shaikh Asim Qamar | Dacum Facilitator |
| 15 | Mr. Yasir | Assistant Director, NAVTTC |

# Members of Qualification Validation Committee

The following members participated in the Qualification development process.

| S# | Name | Organization |
| --- | --- | --- |
| 1 | Dr. Zeeshan Ali Memon (Chairman Qualification Development Committee) | Head of Industrial Engg. Deptt. Dawood University of Engg. & Tech. |
| 2 | Engr. Nimra Ghazal | Research Associate, Dawood University of Engg. & Tech. |
| 3 | Dr. Asif | Associate Professor, Ziauddin University Karachi |
| 4 | Mr. Liaqat Ali Jamro | Director Academics, STEVTA |
|  | Dr. Saifullah Samo | Assistant Professor, Mechatronics Department MUET |
| 5 | Mr. Ayaz Khan | Lecturer, Govt. Advance Technical Training Centre, KP(Tevta) |
| 6 | Mr. Syed Mansoor Ahmed | Assistant IT Manager, SBTE Karachi |
| 7 | Engr. Mustafa Mohiuddin | Lab Engineer, Dawood University of Engg. & Tech. |
| 8 | Mr. Muhammad Usman | Incharge & Instructor Mechatronics, GCT Taxila(P-TEVTA) |
| 9 | Engr. Aijaz Ahmed Zia | D&A Engineer/INTECH |
| 10 | Mr. Yasir | Assistant Director, NAVTTC/ DACUM Expert |

# Entry Requirements

The entry for National Vocational Certificate level 5, in “Robotics” is given below:

| Title | Entry Requirements |
| --- | --- |
| National Vocational Certificate level 5 | Entry into training institute for this qualification is candidate having Matriculation (Science) and or National Vocational Certificate level 4. |

# Regulations for the Qualification and Schedule of Units

Not Applicable.

# Mapping of the Qualification

# Common Courses with respective levels

LEVEL 4

# Packaging of Qualifications (with respect to occupations)

The National Vocational Qualifications have been packaged as detailed below:

Level 4

(Robot installation technician, Robotic System operator)

Robot installation technician

- Assemble various robotic parts

- Perform Robot Calibration

- Inspect Industrial sensors and actuators

- Test/Calibrate sensors, Stepper motor& Servo Motor

- Inspect the hazardous area for installation

- Perform Robot Installation and Maintenance Principles

- Perform Diagnose precise robot movement

- Perform Diagnose Coupling Parameters

- Implement the performance characteristics identified in ISO9283 standards

Robotic System operator

- Identify various parts of robot

- Perform Assembly and Operation of Remotely Operated Robot (ROR)

# Summary of Competency Standards

The National Vocational Qualifications have been packaged as detailed below:

| Level 4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Soft Skills |  |  |  |  |  |  |
| Develop Workplace Policy and Procedures for Sustainability | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Maintain Professionalism in the Workplace | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Manage Personal Work Priorities and Professional Development | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Manage Workforce Planning | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Undertake Project Work | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Prepare and Implement Negotiation | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Manage Meetings | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Organize Schedules | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Identify and Communicate Trends in Career Development | Level 4 | Generic | 4 | 16 | 20 | 2 |
| Digital Electronics |  |  |  |  |  |  |
| Design Operation Amplifier | Level 4 | Technical | 5 | 15 | 20 | 2 |
| Verify truth tables of digital gates | Level 4 | Technical | 10 | 28 | 38 | 3.8 |
| Construct & Verify Combinational logic circuit | Level 4 | Technical | 14 | 36 | 50 | 5 |
| Construct and Verify function of Flip Flops | Level 4 | Technical | 8 | 16 | 24 | 2.4 |
| Use 555 IC as Multi vibrator | Level 4 | Technical | 6 | 15 | 21 | 2.1 |
| Construct Shift Registers and Counters using Flip Flops | Level 4 | Technical | 7 | 16 | 23 | 2.3 |
| AC & DC machines (Motor and Generator) |  |  |  |  |  |  |
| Verify Basic Laws of Electrical Machines | Level 4 | Technical | 6 | 44 | 50 | 5 |
| Analyze Dc Generators | Level 4 | Technical | 7 | 43 | 50 | 5 |
| Perform Tests On Dc Generators | Level 4 | Technical | 8 | 60 | 68 | 6.8 |
| Operate and test stepper and servo motors | Level 4 | Technical | 3 | 18 | 21 | 2.1 |
| Robotic Programming |  |  |  |  |  |  |
| Object Oriented Programming-I | Level 4 | Technical | 20 | 90 | 110 | 11 |
| Object Oriented Programming-II | Level 4 | Technical | 20 | 90 | 110 | 11 |
| Robotic System |  |  |  |  |  |  |
| Identify various parts of robot | Level 4 | Technical | 6 | 20 | 26 | 2.6 |
| Assemble various robotic parts | Level 4 | Technical | 6 | 20 | 26 | 2.6 |
| Robotics control Board | Level 4 | Technical | 6 | 20 | 26 | 2.6 |
| Develop Robot program | Level 4 | Technical | 6 | 20 | 26 | 2.6 |
| Develop robotic arm structure | Level 4 | Technical | 5 | 20 | 25 | 2.5 |
| Configure and test robotic arm | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Develop Object recognation system | Level 4 | Technical | 5 | 25 | 30 | 3 |
| Perform Assembly and Operation of Remotely Operated Robot (ROR) | Level 4 | Technical | 5 | 31 | 36 | 3.6 |
| Engineering Drawing |  |  |  |  |  |  |
| Perform Basic Manual Drawing | Level 4 | Technical | 6 | 30 | 36 | 3.6 |
| Construct different Engineering Curves | Level 4 | Technical | 6 | 30 | 36 | 3.6 |
| Construct multi-view drawings | Level 4 | Technical | 6 | 30 | 36 | 3.6 |
| Install CAD Software | Level 4 | Technical | 6 | 30 | 36 | 3.6 |
| Develop 2D CAD drawings | Level 4 | Technical | 6 | 30 | 36 | 3.6 |
| Develop 3D CAD drawings | Level 4 | Technical | 6 | 30 | 36 | 3.6 |
| Level 4 Total |  |  | 231 | 969 | 1200 | 120 |

# Detail of Competency Standards

# Soft Skills

# Develop workplace policy and procedures for sustainability

Overview: This competency standard covers the skills and knowledge required to develop and implement a workplace sustainability policy and to modify the policy to suit changed circumstances. It applies to individuals with managerial responsibilities who undertake work developing approaches to create, monitor and improve strategies and policies within workplaces and engage with a range of relevant stakeholders and specialists.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Develop workplace sustainability policy | - Define scope of sustainability policy - Gather information from a range of sources to plan and develop policy - Identify and consult stakeholders as a key component of the policy development process - include appropriate strategies in policy at all stages of work for minimizing resource use, reducing toxic material and hazardous chemical use and employing life cycle management approaches - Make recommendations for policy options based on likely effectiveness, timeframes and cost - Develop policy that reflects the organization s commitment to sustainability as an integral part of business planning and as a business opportunity - Agree to appropriate methods of implementation, outcomes and performance indicators |
| - CU2. Communicate workplace sustainability policy | - Promote workplace sustainability policy, including its expected outcome, to key stakeholders - Inform those involved in implementing the policy about expected outcomes, activities to be undertaken and assigned responsibilities |
| - CU3. Implement workplace sustainability policy | - Develop and communicate procedures to help implement workplace sustainability policy - Implement strategies for continuous improvement in resource efficiency - Establish and assign responsibility for recording systems to track continuous improvements in sustainability approaches |
| - CU4. Review workplace sustainability policy implementation | - Review workplace sustainability policy implementation - Investigate successes or otherwise of policy - Monitor records to identify trends that may require remedial action and use to promote continuous improvement of performance - Modify policy and or procedures as required to ensure improvements are made |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define outline the environmental or sustainability legislation, regulations and codes of practice applicable to the organization identify internal and external sources of information and explain how they can be used to plan and develop the organization s sustainability policy

- Explain policy development processes and practices

- Describe organizational systems and procedures that relate to sustainability

- Describe typical barriers to implementing policies and procedures in an organization and possible strategies to address them.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to develop workplace policy and procedures for sustainability. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Scope and develop organizational policies and procedures that comply with legislative requirements and support the organization s sustainability goals covering at a minimum:

  - minimizing resource use

  - resource efficiency

  - reducing toxic material and hazardous chemical use

  - employing life cycle management approaches

  - continuous improvement

- Plan and implement sustainability policy and procedures including:

  - agreed outcomes

  - performance indicators

  - activities to be undertaken

  - assigned responsibilities

  - record keeping, review and improvement processes

- Consult and communicate with relevant stakeholders to generate engagement with sustainability policy development, implementation and continuous improvement

- Review and improve sustainability policies.

# Maintain Professionalism in the Workplace

Overview: This competency standard covers the skills and knowledge required to maintain a professional image in the workplace, including behaving ethically, demonstrating motivation, respecting timeframes and maintaining personal appearance.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Respect work timeframes | - Demonstrate punctuality in meeting, set working hours and times. - Utilize working hours only for working and follow company regulations. - Complete work tasks within deadlines according to order of priority - Supervisors are informed of any potential delays in work times or projects. |
| - CU2. Maintain personal appearance and hygiene | - Clean hair, body and nails regularly. - Wear suitable cloths for the workplace, and respect local and cultural contexts - Meet specific company dress code requirements |
| - CU3. Maintain adequate distance with colleagues and clients | - Respect personal space of colleagues and clients with reference to local customs and cultural contexts. - Keep sufficient distance from others - Avoid cross transmission of infections (especially through respiration). |
| - CU4. Work in an ethical manner | - Follow company values/ethics codes of ethics and/or conduct, policies and guidelines. - Use company resources in accordance with company ethical standards. - Conduct personal behavior and relationships in accord with ethical standards and company policies. - Undertake work practices in compliance with company ethical standards, organizational policy and guidelines. - Instruct co-workers on ethical, lawful and reasonable directives. - Share Company values/practices with co-workers using appropriate behaviour and language. - Report work incidents/situations and/or resolved in accordance with company protocol/guidelines. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define outline the environmental or sustainability legislation, regulations and codes of practice Explain application of good manners and right conduct

- Explain basic practices for oral and personal hygiene

- Describe common products used for oral and personal hygiene

- Describe the company code of conduct/values

- Describe the Company regulations, performance and ethical standards

- Explain work responsibilities/job functions

- Describe communication skills

- Describe workplace hygiene standards

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to maintain professionalism in the workplace the evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Clarify and affirm work values/ethics/concepts consistently in the workplace;

- Comply with required working times;

- Conduct work practices satisfactorily and consistently, in compliance with work ethical standards, organizational policy and guidelines;

- Develop suitable hygiene

Keep adequate distance while interacting with colleagues and clients.

# Manage personal work priorities and professional development

Overview: This competency standard covers the skills and knowledge required to create systems and process to organize information and prioritize tasks. It applies to individuals working in managerial positions who have excellent organizational skills. The work ethic of individuals in this role has a significant impact on the work culture and patterns of behaviour of others as managers at this level are role models in their work environment.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Establish personal work goals | - Serve as a positive role model in the workplace through personal work planning - Ensure personal work goals, plans and activities reflect the organization s plans, and own responsibilities and accountabilities - Measure and maintain personal performance in varying work conditions, work contexts and when contingencies occur |
| - CU2. Set and meet own work priorities | - Take initiative to prioritize and facilitate competing demands to achieve personal, team and organizational goals and objectives - Use technology efficiently and effectively to manage work priorities and commitments - Maintain appropriate work-life balance, and ensure stress is effectively managed and health is attended to |
| - CU3. Develop and maintain professional competence | - Assess personal knowledge and skills against competency standards to determine development needs, priorities and plans - Seek feedback from employees, clients and colleagues and use this feedback to identify and develop ways to improve competence - P3 Identify, evaluate, select and use development opportunities suitable to personal learning style/s to develop competence - Participate in networks to enhance personal knowledge, skills and work relationships - Identify and develop new skills to achieve and maintain a competitive edge |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain principles and techniques involved in the management and organization of:

- Define performance measurement

- Describe personal behavior, self-awareness and personality traits identification

- Describe personal development plan

- Describe personal goal setting

- Describe time management

- Describe management development opportunities and options for self

- Describe methods for achieving a healthy work-life balance

- Define organization policies, plans and procedures

- Explain types of learning style/s and how they relate to the individual

- Describe types of work methods and practices that can improve personal performance.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage personal work priorities and professional development. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Demonstrate use business technology to create and use systems and processes to organize and priorities tasks and commitments

- measure and maintain personal work performance including assessing competency against competency standards and seeking feedback

- maintain an appropriate work-life balance to manage personal health and stress

- participate in networks

- develop a personal development plan which includes career objectives and an action plan

- Develop new skills.

# Manage workforce planning

Overview: This competency standard covers the skills and knowledge required to manage planning in relation to an organization’s workforce including researching requirements, developing objectives and strategies, implementing initiatives and monitoring and evaluating trends. It applies to individuals who are human resource managers or staff members with a role in a policy or planning unit that focuses on workforce planning.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Research workforce requirements | - Review current data on staff turnover and demographics - Assess factors that may affect workforce supply - Establish the organization’s requirements for a skilled and diverse workforce |
| - CU2. Develop workforce objectives and strategies | - Review organizational strategy and establish aligned objectives for modification or retention of the workforce - Consider strategies to address unacceptable staff turnover, if required - Define objectives to retain required skilled labour - Define objectives for workforce diversity and cross-cultural management - Define strategies to source skilled labour - Communicate objectives and rationale to relevant stakeholders - Obtain agreement and endorsement for objectives and establish targets - Develop contingency plans to cope with extreme situations |
| - CU3. Implement initiatives to support workforce planning objectives | - Implement action to support agreed objectives for recruitment, training, redeployment and redundancy - Develop and implement strategies to assist workforce to deal with organizational change - Develop and implement strategies to assist in meeting the organization’s workforce diversity goals - Implement succession planning system to ensure desirable workers are developed and retained - Implement programs to ensure workplace is an employer of choice |
| - CU4. Monitor and evaluate workforce trends | - Review workforce plan against patterns in exiting employee and workforce changes - Monitor labour supply trends for areas of over- or under-supply in the external environment - Monitor effects of labour trends on demand for labour - Survey organizational climate to gauge worker satisfaction - Refine objectives and strategies in response to internal and external changes and make recommendations in response to global trends and incidents - Regularly review government policy on labour demand and supply - Evaluate effectiveness of change processes against agreed objectives. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain current information about external labor supply relevant to the specific industry or skill requirements of the organization

- Describe industrial relations relevant to the specific industry

- Describe labor force analysis and forecasting techniques

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage workforce planning. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Review and interpret information from a range of internal and external sources to identify:

  - current staff turnover and demographics

  - labor supply trends factors that may affect workforce supply

  - organization’s workforce requirements objectives and strategies

- Manage workforce planning including developing, implementing, monitoring and reviewing strategies to meet workforce needs

- Review relevant trends and supply and demand factors that will impact on an organization’s workforce

- Develop a workforce plan that includes relevant research and specific strategies to ensure access to a skilled and diverse workforce.

# Undertake project work

Overview: This competency standard covers the skills and knowledge required to undertake a straightforward project or a section of a larger project. It covers developing a project plan, administering and monitoring the project, finalizing the project and reviewing the project to identify lessons learned for application to future projects. This unit applies to individuals who play a significant role in ensuring a project meets timelines, quality standards, budgetary limits and other requirements set for the project.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Define project | - Access project scope and other relevant documentation - Define project stakeholders - Seek clarification from delegating authority of issues related to project and project parameters - Identify limits of own responsibility and reporting requirements - Clarify relationship of project to other projects and to the organization’s objectives - Determine and access available resources to undertake project |
| - CU2. Develop project plan | - Develop project plan in line with the project parameters - Identify and access appropriate project management tools - Formulate risk management plan for project, including Work Health and Safety (WHS) - Develop and approve project budget - Consult team members and take their views into account in planning the project - Finalize project plan and gain necessary approvals to commence project according to documented plan |
| - CU3. Administer and monitor project | - Take action to ensure project team members are clear about their responsibilities and the project requirements - Provide support for project team members, especially with regard to specific needs, to ensure that the quality of the expected outcomes of the project and documented time lines are met - Establish and maintain required recordkeeping systems throughout the project - Implement and monitor plans for managing project finances, resources and quality - Complete and forward project reports as required to stakeholders - Undertake risk management as required to ensure project outcomes are met - Achieve project deliverables |
| - CU4. Finalize project | - Complete financial recordkeeping associated with project and check for accuracy - P2 Ensure transition of staff involved in project to new roles or reassignment to previous roles - P3 Complete project documentation and obtain necessary sign-offs for concluding project |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe examples of project management tools and how they contribute to a project

- Describe types of documents and other sources of information commonly used in defining the parameters of a project

- Explain processes for identifying and managing risk in a project

- Describe the organization’s mission, goals, objectives and operations and how the project relates to them

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to undertake project work. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Define the parameters of the project including:

  - project scope

  - project stakeholders, including own responsibilities

  - relationship of project to organizational objectives and other projects

  - reporting requirements

  - resource requirements

- Use project management tools to develop and implement a project plan including:

  - deliverables

  - work breakdown

  - budget and allocation of resources

  - timelines

  - risk management

  - recordkeeping and reporting

- Consult and communicate with relevant stakeholders to generate input and engagement in planning, implementing and reviewing the project

- Provide support to team members to enable them to achieve deliverables and to transition them as appropriate at completion of the project

- Finalize the project including documentation, sign-offs and reporting

Review and document the project outcomes.

# Prepare and implement negotiation

Overview: This competency standard covers the skills and knowledge required to prepare for and participate in a process of negotiation.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Prepare for the negotiation | - Identify objectives and preferred outcome of the negotiation and determine minimum acceptable outcome - Understand in relation to what can be offered and what is needed from the other party - Gather information regarding the other party – objectives, - needs, preferences, resources, what they want to achieve - in - order to determine best negotiating points - List and rank the issues to consider concessions that may be made. - Find examples and refine negotiation argument. - Check information to ensure it is correct and up-to-date. - Develop a negotiation plan that includes information about the other party and its interests and a set of responses and strategies to the anticipated tactics. - Prepare an agenda in advance, which includes discussion topics, participants, location and schedule |
| - CU2. Participate in negotiations | - Analyse all aspects of the incident for degree of hazard, priorities, optional outcomes and appropriate strategies - Analyse and determine strategies and priorities on the incident sought from a range of sources - Assess long term objectives against resources and priorities - Apply a range of communication techniques to make and maintain contact with the key people - Provide clear and factual information to enable an honest and realistic assessment of the interests of the key people and their positions - Resolve the conflict and express their likely consequences clearly and do an analysis of the benefits - Reassess points of disagreements for common positive positions |
| - CU3. Coordinate support services | - Assess the need for support services in terms of the determined strategies and priorities - Negotiate the resources of support services according to established procedures and availability - Provide information on strategies to support services and maintain the communication - Delegate roles and responsibilities according to expertise and resources |
| - CU4. Restore order | - Assess the incidents for degree of risk and take appropriate action to reduce and remove the impact of the incident and restore order - Take action designed to minimize risk and the preserve the safety and security of all involved - Take action to prevent the escalation of the incident appropriate to the circumstances and agreed procedures. - Carry out the use of force for the restoration of control and the maintenance of security in the least restrictive manner. - Complete reports accurately and clearly provided to the appropriate authority promptly - Review, evaluate and analyse the incident and the organizational response to it and report it promptly and accurately. |
| - CU5. Provide leadership, direction and guidance to the work group | - Link between the function of the group and the - goals of the organization - Participate in decision making routinely to develop, implement and review work of the group and to allocate responsibilities where appropriate - Give opportunities and encouragement to others to develop new and innovative work practices and strategies - Identify conflict and resolve with minimum disruption to work group function - Provide staff with the support and supervision necessary to perform work safely and without risk to health - Allocate tasks within the competence of staff and support with appropriate authority, autonomy and training - Supervise appropriately the changing priorities and situations and takes into account the different needs of individuals and the requirements of the task |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

Explain organization’s policies, guidelines and procedures related to control and surveillance, safety and preventing and responding to incidents and breaches of orders covered in the range of variables.

- Explain organization’s management and accountability systems

- Describe teamwork principles and strategies

- Define the principles of effective communication

- Describe the guidelines for use of equipment and technology

- Explain code of conduct

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to solve problems which jeopardize safety and security. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- evidence of effective communication strategies including negotiation, counseling, mediation, advocacy demonstrated under pressure working effectively in a team environment

- evidence of knowledge and application of organizations policies, procedures and

- guidelines for critical incidents

- evidence of accurate and safe use of all emergency equipment

- evidence of managing effective outcomes using strategic planning

- team leadership and situational analysis

# Organize schedules

Overview: This competency standard covers the skills and knowledge required to manage appointments and diaries for personnel within an organization, using manual and electronic diaries, schedules and other appointment systems. It applies to individuals employed in a range of work environments who provide administrative support to teams and individuals.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Establish schedule requirements | - Identify organizational requirements and protocols for diaries and staff planning tools - Identify organizational procedures for different types of appointments - Determine personal requirements for diary and schedule items for individual personnel - Establish appointment priorities and clarify in discussion with individual personnel |
| - CU2. Manage schedules | - Identify recurring appointments and deadlines, and schedule these in accordance with individual and organizational requirements - Establish availability of attendees, and schedule new appointments in accordance with required timelines and diary commitments - Negotiate alternative arrangements and confirm when established appointments are changed - Record appointments and manage schedules in accordance with organizational policy and procedures |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Identify the key provisions of relevant legislation, standards and codes that affect aspects of business operations or the achievement of team goals

- Describe organizational requirements for managing appointments for personnel within the organization

- Summarize the range of appointment systems that could be used

- Outline important considerations when managing the schedules of others.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to organize schedules. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Appropriately manage the schedules of various individuals through a process of careful planning and negotiation.

# Identify and communicate trends in career development

Overview: This competency standard covers the skills and knowledge required to manage appointments and diaries for personnel within an organization, using manual and electronic diaries, schedules and other appointment systems. It applies to individuals employed in a range of work environments who provide administrative support to teams and individuals.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Establish schedule requirements | - Identify organizational requirements and protocols for diaries and staff planning tools - Identify organizational procedures for different types of appointments - Determine personal requirements for diary and schedule items for individual personnel - Establish appointment priorities and clarify in discussion with individual personnel |
| - CU2. Manage schedules | - Identify recurring appointments and deadlines, and schedule these in accordance with individual and organizational requirements - Establish availability of attendees, and schedule new appointments in accordance with required timelines and diary commitments - Negotiate alternative arrangements and confirm when established appointments are changed - Record appointments and manage schedules in accordance with organizational policy and procedures |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Identify the key provisions of relevant legislation, standards and codes that affect aspects of business operations or the achievement of team goals

- Describe organizational requirements for managing appointments for personnel within the organization

- Summarize the range of appointment systems that could be used

- Outline important considerations when managing the schedules of others.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to organize schedules. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Perform appropriately manage the schedules of various individuals through a process of careful planning and negotiation.

### Digital Electronics

# Design Operation Amplifier

Overview: This competency standard covers the skills and knowledge required to Construct a Non-inverting amplifier using operational amplifier, construct an Inverting amplifier using and Construct a differentiator circuit using operational amplifier. Construct Operation Amplifier.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Construct a Non-inverting amplifier using operational amplifier | - Identify the Operational amplifier and its terminals (Inverting, Non-inverting Inputs &Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of non-Inverting Op-Amp. - Select the components for Non-Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Non-Inverting Op-Amp circuit. - Measure the output frequency response & gain - Draw the characteristic curves of Op-Amp. - Generate the Output report |
| - CU2. Construct an Inverting amplifier using operational amplifier | - Identify the Operational amplifier and its terminals (Inverting, Non-inverting Inputs &Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of Inverting Op-Amp. - Select the components for Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Inverting Op-Amp circuit. - Measure the output frequency response & gain - Draw the characteristic curves of Op-Amp. - Generate the Output report |
| - CU3. Construct a differentiator circuit using operational amplifier | - Draw the Schematic diagram of differentiator circuit using Op-Amp. - Select the components for differentiator circuit. - Implement differentiator circuit. - Perform the operations of differentiator circuit. - Measure the output, frequency response & gain - Draw the characteristic curves of differentiator circuit. - Generate the Output report |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe the basics of Transistors, ICs, Capacitors, Resistors, Op-Amp & their applications in circuits

- Explain the basics of electronics & its applications

- Explain Multimeter, Oscilloscope

- Describe power Supply& their applications

- Describe the data sheets

- Explain the basics of electronics& its applications

- Explain the basics of electronics, Differentiator and their applications

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Construct a Non-inverting amplifier using operational amplifier

- Construct an Inverting amplifier

- Construct a differentiator circuit using operational amplifier.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Functions Generator |
|  | Datasheets |
|  | Digital Oscilloscope |
|  | Capacitors |
|  | Inductors |
|  | Op-Amp |
|  | Power supply |
|  | Trainer |
|  | Multimeter |
|  | Resistors |

# Verify Truth Tables of Digital Gates

Overview: This competency standard covers the skills and knowledge required to Verify the truth table of AND gate, Verify the truth table of OR gate, Verify the truth table of NOT gate, Verify the truth table of NAND gate, Verify the truth table of NOR gate, Verify the truth table of XOR gate and Verify the truth table of XNOR gate Verify the truth table of AND gate, OR gate, NOT gate, NAND gate, NOR gate, XOR gate and XNOR gate.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Verify the truth table of AND gate | - Identify the symbol of logic gate, IC & logic function. - Place (AND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| - CU2. Verify the truth table of OR gate | - Identify the symbol of logic gate, IC & logic function. - Place (OR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| - CU3. Verify the truth table of NOT gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOT gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU4. Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU5. Verify the truth table of NOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU6. Verify the truth table of XOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU7. Verify the truth table of XNOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe logic gates Logic gates. AND, OR, NAND, NOR, NOT, XOR and XNOR.

- Define the Boolean expression of AND, OR, NAND, NOR, NOT, XOR and XNOR, gate and its equivalent electrical circuit

- Define Universal gate and enlist its types.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Construct a Non-inverting amplifier using operational amplifier

- Construct an Inverting amplifier

- Construct a differentiator circuit using operational amplifier.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | AND gate (7408 2-input Quad) |
|  | OR gate (7432 2-input Quad) |
|  | NOT gate (7404 Hex) |
|  | NAND gate (7400 2-input Quad) |
|  | NOR gate (7402 2-input Quad) |
|  | X-OR gate (7486 2-input Quad) |
|  | X-NOR gate (74266 2-input Quad) |
|  | Bread board |
|  | DC supply (5 V) |
|  | LED |
|  | Connecting leads |

# Construct & Verify Combinational logic circuit

Overview: This competency standard covers the skills and knowledge required to Apply Karnaugh mapping & Boolean algebra to simplify logic expressions, Construct & verify the truth table of Half adder, Construct & verify the truth table of Full adder, Construct & verify the truth table of Half Subtraction,   Operate seven segment display with seven segment decoder, Construct & verify the truth table of Full Subtraction, Verify Encoder, Verify Encoder and Verify multiplexer and DE- multiplexer Construct & verify the truth table of Half adder, full adder and Half Subtraction.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Apply Karnaugh mapping & Boolean algebra to simplify logic expressions | - Identify the SOP & POS - Apply Boolean algebra & Karnaugh mapping to simplify SOP & POS. - Construct logic circuits with simplified SOP & POS. |
| - CU2. Construct & verify the truth table of Half adder | - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a half-adder circuit using one XOR gate and two NAND gates. |
| - CU3. Construct & verify the truth table of Full adder | - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a full-adder circuit using two ICS, &7486 and &7400. |
| CU4. Construct & verify the truth table of Half Subtraction | - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input |
| CU5. Construct & verify the truth table of Full Subtraction | - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| CU6. Verify Decoder | - Place (Decoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| CU7. Operate seven segment display with seven segment decoder. | - Insert (7 segment decoder IC) and 7 segment display on bread board. - Identify the input, output, Vcc and ground pin. - Connect segment display with seven segment decoder input output pins. - Record & verify the output result against each given input. |
| CU8. Verify Encoder | - Place (Encoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| CU9. Verify multiplexer and DE- multiplexer | - Implement following function with multiplexer F(ABC)=∑ (0,2,3,4,5,6): - Implement 4-to-1 mux and one 2-to-1 mux. - Implement 1-to-4 dmux using 1-to-2 dmux. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe the laws and rules of Boolean algebra.

- Define commutative. • And distributive expiration. That is, A • (B + C) = (A • B) + (A • C) and A + (B • C) = (A + B) • (A + C).

- Describe the combinational logic circuit. (Half adder, Full adder, Half subtraction, Full subtraction, Binary Multiplier, Magnitude, Comparator)

- Describe the Product-of-Sums & SOP Simplification

- Define Don't-Care Conditions

- Define Karnaugh Map of four Variable.

- Describe Decoders & Encoders&   Multiplexers.

- Define Pin configuration of iCs

- Describe 7 segment display.

- Explain pin 7 segment display and common cathode 7 segment display.

- Define limiting resistor

- Describe how to implement functions using multiplexers.

- Describe DE multiplexer

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Apply Karnaugh mapping & Boolean algebra to simplify logic expressions

- Operate seven segment display with seven segment decoder

- Construct & verify the truth table of Full Subtraction, Verify Encoder, and Verify Encoder.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | AND gate (7408 2-input Quad) |
|  | OR gate (7432 2-input Quad) |
|  | NOT gate (7404 Hex) |
|  | NAND gate (7400 2-input Quad) |
|  | NOR gate (7402 2-input Quad) |
|  | X-OR gate (7486 2-input Quad) |
|  | X-NOR gate (74266 2-input Quad) |
|  | Bread board |
|  | DC supply (5 V) |
|  | LED |
|  | Connecting leads |
|  | Bread board |
|  | Resistances (1K ohm) |
|  | Connecting leads |
|  | DC supply (5 V) |
|  | Mux KL-33006 block e |
|  | Seven segment display |
|  | 74LS47 IC |
|  | Mux KL-33006 block |

# Construct and Verify function of Flip Flops

Overview: This competency standard covers the skills and knowledge required to Construct and verify the truth table of RS latch using NAND gate, Construct and verify the truth table of clocked RS latch using NAND gate, verify function of D flip flop, verify function of JK/T flip flop construct and verify the truth table of RS latch using NAND gate, clocked RS latch using NAND gate, D flip flop and JK flip flop.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Construct and verify the truth table of RS latch | - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |
| - CU2. Construct and verify the truth table of clocked RS latch using NAND gat | - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |
| - CU3. Verify function of D flip flop. | - Insert (D flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |
| CU4. Verify function of JK/T flip flop | - Insert 74112 (JK flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Latch

- Define, How many inputs are given to SR Latch

- Define, which IC is used for NAND and NOR gate

- Define, what is clocked RS flip flop.

- Define, what is difference between Latch and flip flop

- Describe Symbols for Combinational Elements (Symbols for Flip-Flops.)

- Define D flip flop.

- Define clock pulse.

- Define, difference between synchronous & asynchronous input

- Define JK/T flip flop.

- Exercise to draw the symbol of JK flip flop

- Define, which IC is used for JK flip flop

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Construct and verify the truth table of RS latch using NAND gate

- Construct and verify the truth table of clocked RS latch using NAND gate

- verify the truth table of RS latch using NAND gate, clocked RS latch using NAND gate, D flip flop and JK flip flop.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | AND gate (7408 2-input Quad) |
|  | OR gate (7432 2-input Quad) |
|  | NOT gate (7404 Hex) |
|  | NAND gate (7400 2-input Quad) |
|  | NOR gate (7402 2-input Quad) |
|  | X-OR gate (7486 2-input Quad) |
|  | X-NOR gate (74266 2-input Quad) |
|  | Digital clock |
|  | DC supply (5 V) |
|  | LED |
|  | Connecting leads |

# Use 555 IC as Multivibrator

Overview: This competency standard covers the skills and knowledge required to Construct 555 IC as Actable Multivibrator, Construct 555 IC as Mono-stable Multivibrator, and Construct 555 IC as Bi-stable Multivibrator and verify its set and reset conduction. Construct 555 IC as Actable, nonstable & beatable Multivibrator and observe their outputs.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Construct Astable Multivibrator using 555 IC | - Draw circuit diagram for Actable Multivibrator - Place 555 IC on bread board/trainer - Make connection as per diagram. - Apply voltage to circuit. - Recode the output signal wave shape from oscilloscope. |
| - CU2. Construct Mono-stable Multivibrator using 555 IC | - Draw circuit diagram for Mono-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. |
| - CU3. Construct Bi-stable Multivibrator using 555 IC and verify its set and reset conduction | - Draw circuit diagram for Bi-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe basic elements of 555 timer IC.

- Describe Name pins of 555 timer IC.

- Describe function of voltage control input

- Define stable Multivibrator

- Define non stable Multivibrator

- Define beatable Multivibrator

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Construct 555 IC as Actable Multivibrator

- Construct 555 IC as Mono-stable Multivibrator

- Construct 555 IC as Bi-stable Multivibrator

- Construct 555 IC as Actable, nonstable & beatable Multivibrator and observe their outputs.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | DC supply (5 V |
|  | Connecting leads |
|  | 555 Timer IC |
|  | Capacitor 0.1µF |
|  | Resistors 10 KΩ |
|  | dual trace Oscilloscope 0-20MHZ |
|  | Capacitor 0.01µF |
|  | Bread board |

# Construct Shift Registers and Counters Used Flip Flops

Overview: This competency standard covers the skills and knowledge required to construct a 4-bit shift register by Using Flip Flops, construct a 4-bit binary counter Using Flip Flops, Construct 4-bit synchronous counter with D flip-Flops and Repair & Troubleshoot combinational logic circuits.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Construct a 4 bit shift register by Using Flip Flops | - Draw circuit diagram 4-bit register. - Make connection of D-Flip Flop as per diagram to construct 4-bit shift register. - Apply data at the input of register and give clock pulse - Recode the output according to the input. |
| - CU2. Construct a4-bit binary counter Using Flip Flops | - Draw circuit diagram counter. - Make connection of JK-Flip Flop as per diagram to construct 4-bit binary counter. - Connect LEDs to the outputs pins. - Apply the clock pulse and record the output. |
| - CU3. Construct 4-bitsynchronous Counter with D flip-Flops | - Draw circuit diagram synchronous counter. - Make connection of JK-Flip Flop as per diagram to construct 4-bit synchronous counter. - Connect LEDs to the output pins. - Apply the clock pulse and record the output. |
| - CU4. Repair & Troubleshoot - combinational logic circuits | - Identify principles and operations of types of logic gates - Locate drawing and diagrams. - Check the grounding system in electronic equipment. - Locate external and internal digital IC faults |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define symbols for Combinational Elements (Symbols for Registers, Counters, RAM.)

- Describe the function of registers, and counters in digital circuits

- Describe basic computer memories and its types.

- Explain precautions when handling components.

- Describe proper use of ESD equipment.

- Explain how to distinguish hardware from software issues

- Describe how pulsars are used for digital signal tracing and how logic probes are used to verify states in digital equipment.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify symbols for Combinational Elements (Symbols for Registers, Counters, RAM.)

- Explain basic computer memories and its types.

- Demonstrate safety precautions when handling components.

- Demonstrate proper use of ESD equipment.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Software Simulator and IDE’s. |
|  | DLD trainer |
|  | Multimeter |
|  | IC’s and Components |
|  | Logic tester |

### AC & DC machines (Motor and Generator)

# Verify Basic Laws of Electrical Machines

Overview: This competency standard covers the skills and knowledge required to verify Faraday’s law by moving permanent magnet inside the coil, verify Faraday’s law by moving coil near the magnet field, verify Faraday’s law using relative motion of coil and magnet, verify Faraday’s Law using simple loop generator, verify EMF through induction, verify Torque induce in a current carrying loop and verify Mutual induction.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Verify Faraday’s law by moving permanent magnet inside the coil. | - Construct a coil. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow. - Record the effect of movement of magnet on reading of Galvanometer. - Hold the magnet inside the coil and do not move. - Record the effect on reading of Galvanometer again. |
| - CU2. Verify Faraday’s law by moving coil near the magnet field. | - Construct a coil. - Connect Galvanometer with coil. - Fix permanent magnet and move the coil fast and slow on it. - Record the effect of movement of coil on reading of Galvanometer. - Hold the coil near the magnetic field and do not move. - Record the effect on reading of Galvanometer. |
| - CU3. Verify Faraday’s law using relative motion of coil and magnet. | - Construct a coil. - Connect Galvanometer with coil. - Make relative motion of coil and magnet. - Record the effect of movement on reading of Galvanometer. |
| - CU4. Verify Faraday’s Law using simple loop generator. | - Identify single loop generator and its parts. - Select field winding. - Connect Galvanometer with single loop coil. - Rotate the single loop coil of generator in the field - Measures the voltage induces in loop generator. - Record the effect of movement on reading of Galvanometer. |
| - CU5. Verify Torque induce in a current carrying loop | - Construct a coil. - Apply DC voltage to this coil. - Place a current carrying loop in this coil. - Check the direction of force to verify the torque produce. |
| - CU6. Verify Mutual induction | - Construct 2 coils on two different legs of single core. - Apply AC voltage to the any one coil. - Check the volts induce in other coil by voltmeter. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define first law of Faraday

- Define second law of Faraday

- Define to find the direction of induced emf

- Define the role of magnetic strength in Faraday’s Law

- Define the role of conductor’s length or turns in Faraday’s Law

- Define the Lenz’s Law

- Define the torque

- Define force produce on current carrying conductor.

- Define, what is core

- Define, what is induction

- Define Faraday’s first law of Electro-Magnetic Induction

- Define the meaning of coupled coils.

- Define the emf induced in 2nd coil, when voltage is applied to 1st coil.

- Define, what is mutual induction?

- Define, Which machine work on the principle of mutual induction

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Describe magnetic strength in Faraday’s Law

- Explain the Lenz’s Law

- Explain Faraday’s first law of Electro-Magnetic Induction

- Describe mutual induction

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Coil |
|  | Galvanometer. |
|  | Voltmeter |
|  | Transformer |
|  | Connecting leads. |
|  | Magnet. |
|  | Connecting leads. |
|  | Transformer |
|  | DC source |
|  | Rheostat |
|  | Coils |
|  | Loop |
|  | Voltmeter |
|  | Rheostat |
|  | Connecting leads |

# Analyse Electrical Generators

Overview: This competency standard covers the skills and knowledge required to calculate different terms used in armature winding of DC machines, develop Single Layer Lap Winding, develop Double Layer Wave Winding, measure the resistance of field winding, armature winding and terminal polarity, identify DC shunt generator, identify DC series generator and identify DC compound generator.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Calculate different terms used in armature winding of DC machines | - Count the number of slots (S) of armature. - Calculate pole pitch (Y). - Calculate Coil span/pitch. - Calculate Back pitch (Yb) - Calculate Front pitch (Yf) - Calculate Resultant Pitch (YR) - Calculate commentator pitch (Yc) |
| - CU2. Develop Single Layer Lap Winding | - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Subtract the value of back pitch from the added value. - Connect added value from front side to subtracted value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. |
| - CU3. Develop Double Layer Wave Winding | - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Add the value of front pitch in previous added value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. |
| - CU4. Measure the resistance of field winding, armature winding and terminal polarity | - Open the terminal box. - Connect the ohm meter with the terminal of both winding. - Read down ohm meter reading. - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. - Rotate the generator slowly by hand. - Touch the probe of galvanometer with each winding. - Identify armature with the deflection of galvanometer pointer. - Mark the polarity of terminals with the deflection of galvanometer. |
| - CU5. Identify DC shunt generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC shunt generator from measured value of resistance. - Disconnect both windings. - Identify the shunt field winding of shunt generator. - Identify armature of shunt generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |
| - CU6. Identify DC series generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC series generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series generator. - Identify armature of series generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |
| - CU7. Identify DC compound generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC Compound generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of generator. - Identify the shunt field winding of generator. - Identify armature of shunt generator. - Measure and record the resistance of both winding separately. - Measure and Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain the term armature slots.

- Define the pole pitch (Y).

- Define the coil span/pitch

- Define the Back pitch (Yb)

- Define the Front pitch (Yf)

- Define the Resultant Pitch (YR)

- Define the commutation pitch (Yc)

- Define the lap winding

- Define, How many parallel paths in lap winding

- Define the meaning by single layer winding

- Define the wave winding

- Define how many parallel paths in wave winding.

- Define the meaning by double layer winding

- Perform the reading on the scale of galvanometer

- Define the difference between field winding and armature winding

- Explain, which winding having low resistance and why

- Explain the technique to identify the terminal polarity

- Explain the number of winding in shunt generator

- Explain the different between field winding and armature winding

- Explain, which winding having low resistance and why.

- Explain, how to disconnect both winding of shunt generator

- Explain, how many winding in series generator

- Explain the different between field

- Explain winding and armature winding.

- Explain which winding having low resistance and why.

- Explain how to disconnect both winding of series generator.

- Explain which conduction you follow at the time of reconnection of both winding.

- Define the Type of DC machines

- Explain the principle of simple loop generator.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform Armature winding of DC machines

- Perform Single Layer Lap Winding

- Perform Double Layer Wave Winding

- Perform measure the resistance of field winding

- Identify DC series generator

- Identify DC compound generator

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Ohm meter |
|  | Galvanometer |
|  | DC machine |
|  | Connecting wires |
|  | Galvanometer |
|  | Sleeve |
|  | Armature |
|  | Winding machine |
|  | Insolating paper |
|  | Carbon brushes |
|  | Stator frame |
|  | Soldering wire |
|  | Soldering iron |
|  | Stator frame |

# Perform tests on DC Motors

Overview: This competency standard covers the skills and knowledge required to Plot the load characteristics of D.C shunt motor, plot the load characteristics of D.C series motor, plot the load characteristics of D.C compound motor, control the speed of DC series motor, control the speed of DC shunt motor, start  DC shunt motor with 3 point starter, identify the effect of back EMF in DC motor, calculate BHP of motor by Brake test, calculate efficiency of the motor by Swinburne Test and perform Regenerative / Hopkinson/ Back to back Test.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Plot the load characteristics of D.C shunt motor | - Couple electro dynamo meter to the DC motor with belt - Connect shunt motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC shunt motor.. |
| - CU2. Plot the load characteristics of D.C series motor. | - Select DC series motor and achieve connection as per diagram. - Turn on DC power supply and increase the load to full value. - Record the value of speed and current on each load. - Draw the graph between load current and speed |
| - CU3. Plot the load characteristics of D.C compound motor | - Couple electro dynamo meter to the DC motor with belt - Connect compound motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC compound motor.. |
| - CU4. Control the speed of DC series motor. | - Make the connection as per circuit diagram. - Insert external resistances in series to armature and field coil. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different supply voltages. - Make graph between speed and applied voltage. |
| - CU5. Control the speed of DC shunt motor. | - Make the connection as per circuit diagram. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different field current. - Make graph between speed and field current - Fixed the field current and very armature current - Record the speed at different armature current. - Make graph between speed and armature current |
| - CU6. Start DC shunt motor with 3 point starter. | - Make the connection as per circuit diagram. - Switch on the supply and move starter handle from off position to position 1. - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. |
| - CU7. Identify the effect of back EMF in DC motor. | - Select DC series motor and make connection as per diagram. - Turn on DC power supply and increase the load 0 to full value. - Record the value of speed and current on each load. - Calculate back EMF by using the recoded values and appropriate formula. - Make graph between speed and back EMF and interpret the result. |
| - CU8. Calculate Brake Horse Power (BHP) of motor by Brake test. | - Connect the spring balance with one end of the rope. - Connect other end of rope with weight (W1). - Measure the radius of the pulley. - Hang the weight on the pulley of the motor and fix the spring balance end with earth. - Run the motor. - Measured the RPM of the motor. - Adjust the weight to pass full load current from motor. - Calculate spring balance weight and hanged weight (W1) - Calculate shaft torque. - Calculate BHP of motor by shaft torque. |
| - CU9. Calculate efficiency of the motor by Swinburne Test | - Perform DC test to find the value armature resistance and field resistance - Run the motor at rated voltage without load. - Adjust the value of shunt regulator to attain rated speed. - Measure Field current and No load current with ampere meter. - Calculate total copper losses and iron losses. - Calculate input and output power. - Calculate efficiency from calculated power. |
| - CU10. Perform Regenerative / Hopkinson/ Back to back Test | - Identify two shunt machines. - Couple the shafts of both machines. - Connect the machines electrically in such a way that one machine work as a motor and other as a generator. - Connect external power supply to the machines to overcome the machine losses. - Run the both machine at no load. - Reduce the field excitation of one machine which work as a motor. - Increase the field excitation of one machine which work as a generator. - Adjust the field winding so that voltage of generator becomes equal to the applied voltage which result the machines runs in parallel - Calculate the efficiency by using reading of voltmeter and ammeter. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain the number of winding in compound generator

- Explain the different between field winding and armature winding

- Define, which winding having low resistance and why.

- Explain the technique to identify the terminal polarity.

- Explain how to disconnect both winding of series generator.

- Explain why series winding having low turns.

- Explain the effect of field current on motor speed.

- Explain the effect of load on motor speed.

- Explain why we never start DC series motor without load.

- Explain why series winding having low turns and shunt winding having high number of turns.

- Explain the effect of field current on motor speed.

- Explain the effect of load on motor speed.

- Describe the different between shunt, series and compound motor load characteristics?

- Explain speed control of DC motor with applied voltage.

- Describe why we control the speed of DC motor.

- Define the techniques to control the speed of DC shunt motor

- Explain the effect of field current changes on motor speed.

- Describe what is 3-point starter

- Explain what is back EMF

- Explain generator action in motor.

- Explain the effect of motor

- Explain the speed on back EMF

- Perform the RPM measurement of the motor

- Explain what is shaft torque

- Perform the calculation of BHP from shaft torque

- Explain the what is iron loss

- Explain the what is copper loss

- Explain which loss having maximum value and why.

- Explain why we connect external source with motor generator set.

- Explain what is mean by field excitation

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Plot the load characteristics of D.C shunt motor, load characteristics of D.C series motor, load characteristics of D.C compound motor

- Demonstrate the speed control of DC series motor or DC shunt motor

- Perform Regenerative / Hopkinson/ Back to back test.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | DC shunt motor |
|  | Tachometer |
|  | Voltmeter |
|  | Ammeter |
|  | Connecting wire |
|  | Wattmeter |
|  | Spring balance |
|  | Rope |
|  | DC series motor |
|  | Ohm meter |
|  | Belt |
|  | Electro dynamo meter |
|  | DC compound motor |

# Operate and test  stepper and servo motors

Overview: This competency standard covers the skills and knowledge required to operate and speed control of AC series motor, operate miniature (reluctance and hysteresis) single phase AC motors, construct and operate stepper motor and construct and operate and develop control circuit with the help of servo motor.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Operate and speed control of AC series motor. | - Connect the armature and field winding in series. - Apply rated voltage to the motor. - Measure the effect on motor speed by change in applied voltage. |
| - CU2. Operate miniature (reluctance and hysteresis) single phase AC motors. | - Connect the stator of motor with single phase supply. - Apply mechanical load by hand - Measure the effect of load change on motor speed. - Draw torque speed curves. |
| - CU3. Construct and operate stepper motor. | - Connect the stator of the motor with control circuit. - Adjust the control voltage of control unit. - Measure the RPM. - Readjust the control voltage of control unit. - Measure the RPM again. - Make comparison between Control voltage and RPM. |
| - CU4.. Construct, Operate and develop control circuit of servo motor. | - Pair the servo motor with some encoder to provide position and speed feedback. - Compare the measured position to external input to the controller. - Check the effect of feedback on motor position. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain the working of AC series motor.

- Explain the effect of applied voltage on speed of the motor.

- Explain the working of reluctance motor.

- Explain the working of hysteresis motor.

- Explain the working of stepper motor.

- Explain the working of control unit for stepper motor.

- Explain the working of servo motor.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain the effect of applied voltage on speed of the motor.

- Explain the working of stepper motor.

- Explain the working of control unit for stepper motor.

- Explain the working of servo motor.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | AC series motor |
|  | Tachometer |
|  | Connecting wires |
|  | Reluctance motor |
|  | Hysteresis motor |
|  | Tachometer |
|  | Connecting wires |
|  | Stepper motor |
|  | Control unit |
|  | Tachometer |
|  | Connecting wires |
|  | Servo motor |
|  | Control circuit |

### Robotic Programming

# 0714E&A69: Object Oriented Programming-I

Overview: This competency standard covers the skills and knowledge on C, Classes, Object and Inheritance..

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Developing object oriented programs (OOP) by making simple class. | - Open OOP IDE for coding - Code a simple program and create simple class - Create object of that class - Create variable of object - Compile a code - Debug the code (in case of error) - Run a code |
| - CU2. Develop OOP based program by using inheritance | - Open OOP IDE for coding - Create a class - Create inherited class. - Create object for inherited class - Utilize data type of inherited class - Utilize data type of parent class - Compile and run the code |
| - CU3. Develop a program using operator overloding | - Open OOP IDE for coding - Create class - Create object of that class - Create variable of object - Utilize overloading - Compile a code - Debug the code (in case of error) - Run a code |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define object oriented programming (OOP).

- Define the characteristics of OOP

- Differentiate between object and class

- Define a Class

- Define Object

- Define Constructors and Destructors by using programming examples

- Define operator overloading using different operators.

- Define access specifiers

- Define Inheritance

- Define types of inheritance

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Develop simple program using object oriented basic techniques

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer System |
|  | C language Packages |
|  | Object Oriented based C++ compiler |

# 0714E&A70:Object Oriented Programming - II

Overview: This competency standard covers the skills and knowledge on C, Classes, Object, Polymorphism, Encapsulation and Interfaces.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Develop a Class in OOPs using polymorphism technique | - Open OOP Compiler - Create class - Create Objects in classes - Create function in class for polymorphism - Call polymorphism functions with specific object - Compile and run the code |
| - CU2. Develop program using Encapsulation technique | - Open OOP Compiler - Create class - Create Objects in classes - Apply encapsulation on defining class - Call protected, public and private data with object - Compile and run the code |
| - CU3. Develop Interface | - Create the interface unit - Declare the reference variable of the object class - Make the methods for child object using encapsulation |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define object oriented programming (OOP).

- Define the characteristics of OOP

- Differentiate between different OOP techniques

- Define Polymorphism

- Define Parent class and child class

- Define and understand Encapsulation

- Define protected, public and private data.

- Define and understand interfaces

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Develop program using object oriented techniques ( polymorphism, encapsulation and interfaces)

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer System |
|  | C language Packages |
|  | Object oriented based C++ compiler |

### Robotic System

# Identify various parts of Robotics.

Overview: This competency standard covers the skills and knowledge related to fundamental of robots, Identification of sensors and controller used in robot design and the software development environment to program controller and interfacing of robot parts.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify Various types of Sensors | - Identify temperature sensors. - Identify sound sensors. - Identify proximity sensors. - Identify pressure sensors. - Identify light sensors. - Identify position sensors. - Identify voltage sensors. - Identify current sensors. - Identify the vision sensors. - Identify power rating of each sensor. - Identify the signal input/output of each sensor using datasheet. |
| - CU2. Identify Various Controllers/Microprocessors | - Identify the open loop controllers. - Identify the closed loop controllers. - Identify the algorithms for both controllers. - Identify the components of both controllers. - Draw circuit diagram for controllers. - Draw the block diagram and flow charts of both controllers. - Identify the power supply for controllers. |
| - CU3. Identify Designing and Development Software | - Identify the programming languages. - Identify the program development architecture for the robot. - Identify the software tools for the different types of controllers. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand the usage of temperature sensor

- Understand the operating principles of sound sensor.

- Understand the operation of proximity sensor.

- Understand the usage of robotic tool kit.

- Understand the movement position sensor.

- Learn to identify the types of controller

- Lean to identify requirement of power supply.

- Understand the value of programming software.

- Understand the usage of software tools.

- Understand software tools for different types of controller

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify the major parts and main body of the Robot.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Manipulator links and joints |
|  | Measuring tool kit |
|  | Robotic fastening tool kit. |
|  | Trainer |
|  | Manipulator links and joints |
|  | End Effectors |
|  | Mounting accessories |
|  | Actuators |
|  | Multimeter |
|  | Computer System |
|  | Software |
|  | Sensors |
|  | Base for the manipulator |
|  | Electrical Wiring |

# Assemble various Robotic parts.

Overview: This competency standard covers the skills and knowledge related to fundamental of robots, Identification of sensors and controller used in robot design and the software development environment to program controller and interfacing of robot parts.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Assemble the different types of Manipulators or Rovers. | - Identify parts and main body and of the manipulator. - Identify the links and joints. - Assemble links, joints and overall assembly of the manipulator. - Identify the kinematics (movement) of a manipulator. |
| - CU2. Set up Various Types of End Effectors with the Robot. | - Identify the types of End Effectors. - Assemble the End Effectors. - Assembly of the End Effectors with the manipulator. - Drives for the End Effector. - Identify the kinematics (movement) of the end effectors. |
| - CU3. Assemble various Actuators/Drives with the Robot. | - Identify Hydraulic Actuators. - Identify Electrical Actuators. - Identify Pneumatic Actuators. - Identify Mechanical Actuators. - Identify Locomotive Actuators. - Identify Manipulation Actuators. - Identify the power supply for each actuator. - Assemble actuators with manipulators and end effectors. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand the movement of an end effector.

- Learn the operation of hydraulic actuators.

- Learn the operation of electrical actuators.

- Learn the operation of pneumatic actuators.

- Learn the operation of mechanical actuators.

- Understand the circuit diagram of actuators.

- Learn and understand the concept of degree of freedom.

- Learn and understand the assembly of actuators with manipulators and end effectors.

- Learn the operating principles of the actuators.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform assemble/integrate the parts and the main body of the Robot.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Manipulator links and joints |
|  | Measuring tool kit |
|  | Robotic fastening tool kit. |
|  | Trainer |
|  | Manipulator links and joints |
|  | End Effectors |
|  | Mounting accessories |
|  | Actuators |
|  | Multimeter |
|  | Computer System |
|  | Software |
|  | Sensors |
|  | Base for the manipulator |
|  | Electrical Wiring |

# Robotics Control Boards

Overview: This competency standard covers the skills and knowledge required to install, run, interface and application.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform installation of operating system. | - Insert memory card in pc. - Download and mount Raspbian operating system on SD card. - Insert SD card in raspberry pi and turn it on. - Connect HDMI to LCD. - Run installation on Raspberry pi. - After completing installation restart raspberry pi/ |
| - Run Python on Raspberry pi | - Download and install Python on raspberry pi. - Open python shell compiler on Raspberry pi. - Design code on Python shell. - Compile code and see output. |
| - Control GPIO. | - Open python shell program on Raspberry pi operating system. - Design program on python to cntroll GPIO. - Run compiler and operate actuator. |
| - Interface sensors and actuators | - Open python shell program on Raspberry pi operating system. - Connect jumpers with GPIO pins and sensor/actuators. - Design program on python to control GPIO. - Run compiler and operate actuator. |

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Raspberry pi |
|  | Jumpers |
|  | SD card |
|  | LED |
|  | Actuators |
|  | Power Adaptor |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify components.

- Installation of Raspbian software.

- Run Raspberry pi and control GPIOs

- Interface sensors and actuators with GPIOs of Raspberry pi.

# Programmable Logic controller PLC

Overview: This competency standard covers the skills and knowledge required to install, run, simulate and use in real world application.

| Competency Units | Performance Criteria |
| --- | --- |
| - Installation of PLC programming software | - Download and open setup file for software installation on windows PC. - Configure installation setup. - Install all parts of software. - Complete installation and restart the PC |
| - Design Basic logic program on Ladder logic. | - Open software GUI. - Design basic logical ON/OFF operation in Ladder logic programming. - Run simulator software to verify program. - Upload that program on PLC hardware. |
| - Apply timers on ladder logic program | - Open software GUI. - Design program and insert timers. - Configure timer block according to requirement. - Run simulator software to verify this program. - Upload this program and run on PLC |
| - Apply counters on Ladder logic program. | - Open software GUI. - Design program and insert counters. - Configure counter block according to requirement. - Run simulator software to verify this program. - Upload this program and run on PLC |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify components.

- Installation of PLC software.

- Design ladder logic program.

- Interface sensors and actuators with PLC.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Programming logic controller |
|  | Computer |
|  | Programming software. |
|  | Simulation software |
|  | Communication cable. |
|  | Power Adaptor |

# Field Programmable logic controller

Overview: This competency standard covers the skills and knowledge required to install, run, simulate and apply on field work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Installation of Xilinx ISE programming software | - Download and open setup file for software installation on windows PC. - Configure installation setup. - Install all parts of software. - Complete installation and restart the PC |
| - Design Basic logic program Hardware description language. | - Open software GUI. - Design basic logical ON/OFF operation in hardware description language. - Synthesize program to verify it. |
| - Simulate timing diagram of designed program | - Open software GUI. - Design logical operation on hardware description language. - Synthesize program and apply pin mapping - Simulate timing diagram. - Apply input and set simulation time. |
| - Implement logic on hardware | - Open software GUI. - Design logical operation on hardware description language. - Synthesize program and apply pin mapping - Simulate timing diagram. - Apply input and set simulation time. - After varication start mapping, routing and placement process. - Connect FPGA hardware with PC. - Upload program in FPGA. |

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | FPGA |
|  | Computer |
|  | Programming software. |
|  | Simulation software |
|  | Communication cable. |
|  | Power Adaptor |

# Arduino Development Board

Overview: This competency standard covers the skills and knowledge required to install, run, simulate and apply on field work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Introduction to Arduino Development Board. | - Identify the different models. - Identify Analog Input and Output Pinouts. - Identify Digital Input and Output Pinouts. - Identify the PWM Pins. - Identify the UART Pins. - Identify the I2C pins. |
| - Introduction to Arduino IDE | - Performed Arduino IDE Installations. - Open software and - Introduce all libraries and functions. |
| - Interface Analog Sensor | - Open Arduino IDE software. - Connect analog sensor with any Analog Input Pin. - Make a source code - Compiling and Debugging - Run Program. |
| - Interface Digital Sensors. | - Open Arduino IDE software. - Connect Sensor with any Digital Input Pin. - Make a source code. - Compiling and Debugging. - Run Program. |
| - Interface Serial (UART) based communication | - Open Arduino IDE software. - Connect module serial out TX pin with Arduino board serial RX pin and connect module serial RX pin with Arduino board TX pin. - Make a source code and set baud rate. - Compiling and Debugging. - Run Program. |
| - Interface I2C based communication | - Open Arduino IDE software. - Connect two wires of sensor with SCL and SDA pins of Arduino board. - Make a source code using two wire library. - Compiling and Debugging. - Run Program. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify components.

- Installation of Arduino IDE software.

- Run Arduino IDE and design source code.

- Interface sensors and actuators.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Arduino development board |
|  | Computer |
|  | Programming software. |
|  | Simulation software |
|  | Communication cable. |
|  | Power Adaptor |

# Develop Robot program.

Overview:

This competency standard covers the skills and knowledge related to fundamental of

Programming and interfacing of sensor and actuators as they are essential in robot designing.

Programming involved interfacing SONAR, temperature sensors and generation of PWM signal that is used to control the speed of DC motor.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Develop program tointerface temperature sensor LM35 with Arduino | - Identify the pin configuration of Arduino and LM 35 temperature sensor - Make connection of power , ground and analog pin of LM 35 with Arduino - Connect Arduino with computer(PC) and open Arduino IDE and select port and Devices - Write and upload the code of LM35 into Arduino - Observe the value of temperature in serial monitor Screen in PC - Measure the temperature of ice and calibrate reading by tuning in programming. |
| - CU2. Develop program to operate DC motor with PWM based speed control | - Identify the components and connection of circuits. - Connections the potentiometer to analog pins and connects the MOSFET to digital Pin of Arduino. - Connect the DC motor with separate Power Supply unit with MOSFET as switch. - Write code to take value of voltage and make switch case statements to generate PWM signal of multiple Duty cycle. - Upload the code and run DC motor. |
| - CU3. Develop program to operate SONAR and measure distance with Arduino | - Identify the Pin configuration of Sonar and Arduino. - Make connection according to circuit diagram. - Connection Arduino with PC and open Arduino IDE. - Write and upload the code in Arduino. - Run the code and measure value at serial monitor. - Verify measured value of distance with Scale. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Learn and understand the connections and application of LM35

- Learn and understand the application of PWM technique

- Learn and understand the principle of SONAR sensor.

- Learn and understand the control data structure in different software.

- Learn and understand the packages, libraries and data handling in software.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify and Assemble parts of robot.

- Describe the programing Algorithm and Operation Logic with various sensors.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Trainer |
|  | Computer system |
|  | CAD software |
|  | Measuring tool kit |
|  | Basic Robotic components |
|  | Complete design of the Robot |
|  | Robotic fastening tool kit |
|  | Computer system |

# Develop Robotic arm structure

Overview: This competency standard covers the skills and knowledge related to fundamental of robots arm and assembling of robot the various End effector used to perform different kinds of mechanical jobs, and the development of robot structure.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Design the Robotic Arm | - Specify the types of robotic arms. - Select the links and joints type for robotic arm. - Plan the axes of motion (rotation and straight line movement) of robotic arm. - Specify the degrees of freedom for end effector. - Specify the reach envelope for robotic arm. - Identify the speed and payload for robotic arm. - Design the model of links, joints and overall assembly of the manipulator. |
| - CU2. Develop the structure of Robotic Arm | - Assemble manipulators. - Assemble End Effectors with the manipulators. - Assemble actuators with manipulators and end effectors. - Assemble Controller with actuators and power supply. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand the usage of different types of robotic arm.

- Understand the operating principles of a robotic arm.

- Understand the development of computer aided design (CAD) model.

- Understand the kinematic structure ad workspace    of robotic arm.

- Understand the movement and design of a manipulator.

- Learn to identify the type of manipulators for the Robotic Arm.

- Lean to identify the drive for the End Effector.

- Understand the development of computer aided design (CAD) model.

- Understand the usage of robotic fastening tool kit.

- Understand the movement of an end effector.

- Understand the movement of manipulator.

- Learn and understand the controller for robotics arm applications in the industries.

- Learn and understand the categories, components, and advantages of various controllers.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform the Integrate a part to form structure of Robot.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Manipulator links and joints |
|  | Measuring tool kit |
|  | Robotic fastening tool kit. |
|  | Trainer |
|  | Manipulator links and joints |
|  | Computer system |
|  | Mounting accessories |
|  | Complete design of the sensor assembly |

# Configure and test robotic arm

Overview: This competency standard covers the skills and knowledge related to configuration of Robotic Arm and selection and configuration controller for specific industrial process, it also includes testing of various parts of robot.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Select parts of Robotic Arm | - Select Controller. - Select Manipulator. - Select Effector. - Select Actuators. - Select Software. - Select power supply. - Select connecting wires. - Identify the components (Resistor, Capacitor and Potentiometer). - Identify electric wiring of actuators, controllers and power supply. |
| - CU2. Program the controller | - Assemble the system according to circuit diagram - Open IDE for controller. - Develop the algorithm flowchart to implement on controller - Upload the prepared code |
| - CU3. Test the Robotic Arm | - Identify movement of robotic arm. - Perform the arm movement detection test - Check the robotic arm System feasibility and manipulability for end effectors. - Check of Robotic arm operating conditions. - Check all the connections of the components. - Recognize the program algorithm for the robotic arm. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand types of controller.

- Understand types of manipulator.

- Understand types of End effector.

- Understand power supply and required power according to application.

- Learn and understand the problem solving techniques and error identification.

- Learn and understand robot System feasibility.

- Learn and understand program algorithm.

ss

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform the configure and test Robotic arm.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Manipulator links and joints |
|  | Measuring tool kit |
|  | Robotic fastening tool kit. |
|  | Trainer |
|  | Power supply and wires |
|  | Computer system |
|  | Mounting accessories |
|  | Complete design of the sensor assembly |
|  | Robot operating software |

# Develop Object Recognition System

Overview: This competency standard covers the skills and knowledge related to introduce the student to the machine vision system’s design and development for object recognition. Explore the extraction of object from video feed using artificial intelligence algorithm to command the actuator according to its application.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Develop the object recognition system | - Specify the types of object recognition system. - Identify camera for real time video feed. - Identify the actuators for object recognition system. - Select the components types. - Plan the maneuverability of end effectors. |
| - CU2. Implement Vision Algorithm | - Get the Video sequence - Identify the background subtraction from video by programming algorithm - Identify the Noise filtration from video by programming algorithm. - Calculate the field of view of vision system. - Identify the frame grabber for vision system. - Detect the object - Techniques used for 3D vision - Object location and orientation measurement results store it in memory - Data transmission to controller - Identify actuator to perform task according to object detection. |
| - CU3. Configure the object recognition system. | - Select the Controller. - Identify the components (Resistor, Capacitor and Potentiometer) - Draw the circuit diagram - Electric wiring of actuators and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply for controllers. |
| - CU4. Program the object recognition system. | - Program the controller to lift the object from one point and move it to another point. - Program the controller to read the values from potentiometer. - Program the controller to map the values to actuator to move manipulator and end effector. - Program the movements and positions of actuators. - Program the angle of end effector to determine grabbing of object. |
| - CU5. Test and Evaluate the object recognition system. | - Identity and check the proper data feed. - Recognize the data points for image recognition system. - Verify the pattern for image recognition. - Check the performance accuracy of trained module. - Verify the reliability, maintenance and safety measures for object recognition system. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand the usage of different types of object recognition system.

- Understand the operating principles of an object recognition system.

- Understand the workspace of object recognition system.

- Understand the design of an object recognition system.

- Understand and learn the application of sensors.

- Understand and learn the programming libraries and package for collecting data of video.

- Understand and learn the type of modules of camera for different purposes.

- Understand and learn the artificial intelligence algorithm.

- Understand and learn the neural network for the object recognition.

- Understand and learn  the principles for classification of object

- Understand and learn the testing and training of neural network model for specific object.

- Understand and learn the predictive model for object

- Understand the application of object recognition that is presence of object, localization of object, counting of object, determining the types of object, dimensions of object and tracking.

- Learn and understand the controller ROV applications in industry.

- Learn and understand the categories, components, and advantages of various controllers.

- Learn and Understand principles of algorithms control and describe how algorithms and flowcharts can be applied to design, problem-solving and troubleshooting techniques.

- Learn and understand the operating principles of the controllers.

- Learn and understand the connections and configuration of controller

- Learn and understand the connections and configuration of controller and sensors

- Learn and understand the programming principle languages.

- Learn and understand the various programming techniques associated with industrial object recognition system.

- Learn and understand the control data structure in different software.

- Learn and understand the control data acquisition principles.

- Learn and understand the control data transmission medium.

- Learn and understand the packages, libraries and data handling in software.

- Learn and understand the algorithm for the specified movement

- Learn and understand to check the proper data feed.

- Understand the data points for image recognition system.

- Learn and understand pattern for image recognition

- Learn and understand the performance accuracy measure of trained module.

- Learn and understand reliability, maintenance and safety measures for object recognition system.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Design an object recognition system and plan the maneuverability of end effectors.

- Perform Program for the controller to lift the object from one point and move it to another point.

- Explain the reliability, maintenance and safety measures for object recognition system.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Manipulator links and joints |
|  | Basic object recognition system components |
|  | Complete design of the object recognition system |
|  | Trainer• |
|  | Manipulator links and joints |
|  | Computer system |
|  | Mounting accessories |
|  | Complete design of the sensor assembly |
|  | Camera |
|  | Controller |
|  | Software |
|  | Sensors |
|  | Trainer |
|  | Wiring and accessories |

# Perform Assembly and Operation of Remotely Operated Robot

Overview: This competency standard covers the skills and knowledge related to introduce the student to the one most use category of robot that remotely operated vehicle’s design and development for specific purposes. This module covers operating principles of remotely operated vehicles and describes types of remotely operated vehicles found in industry. Explore the main consideration that’s is mobile base, actuator ,sensors, transmission medium for data, and assembling it with links and joints for remotely operated vehicle. Also introduce to the applications of remotely operated vehicle in different sectors.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Design the Remotely Operated Robot | - Specify the types of remotely operated mobile robot. - Identify mobile base of the ROR. - Identify the actuators for ROR. - Select the components type for ROR. - Plan the maneuverability (mobility and severability) of ROR. - Determine the sensor for positioning. - Identify the sensors to avoid obstacles. - Design the model and overall assembly of the ROV. |
| - CU2. Develop the structure of Remotely Operated Vehicle | - Identify the design of the ROV. - Assembly of the parts of ROV. - Assembly of the base with actuator. - Mount and assemble actuators with base and sensors. - Identify the fastener and accessories of ROV. |
| - CU3. Installation of Sensors | - Installation of positioning sensor - Installation of robot vision (camera). - Installation of distance sensors. - Setting the transmitter/receiver - Setting the monitor. - Shape and design of the sensors. - Types and application and sensors. |
| - CU4. Configure the ROR | - Identify and Select the Controller. - Identify the components (Resistor, Capacitor, and Motor Controller). - Data acquisition from sensor and display on monitor. - Draw the circuit diagram. - Electric wiring of sensors, camera, receiver, actuators and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply for controllers. |
| - CU5. Program the ROR | - Program the controller to change direction of robot according to the receive signal. - Program the controller to get the live camera display on monitor. - Get the values from positioning sensor to map the location and send it to monitor. - Program the distance measuring sensor for collision avoidance and distance measurement from object. - Program to display the all sensors values on monitor. |
| - CU6. Test and Evaluate the ROR | - Identification of movement of the ROR. - Preform the robot movement detection test - Check the ROR System feasibility and manipulability. - Check of ROR operating conditions - Check the condition of each component and overall assembly of the ROR. - Check all the connections of the components. - Recognize the program algorithm for the ROV. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand the usage of different types of remotely operated mobile robot

- Understand the operating principles of a ROV.

- Understand the development of computer aided design (CAD) model.

- Understand the kinematic structure ad workspace    of ROV.

- Understand the movement and design of a ROV.

- Understand the sensors.

- Learn to identify the type of manipulators for the ROV.

- Learn to identify the type of base/platforms for the ROV.

- Lean to identify drive of end effector for the ROV.

- Understand the development of computer aided design (CAD) model.

- Understand the usage of robotic fastening tool kit.

- Understand the movement of an end effector.

- Learn and understand the operation of each sensor.

- Learn and understand the use of each sensor for industrial operations.

- Learn and understand the circuit diagram of each sensor.

- Understand Multimeter& Power Supply

- Learn and understand the connections and assembly of sensors.

- Learn and understand the operating principles of the sensors.

- Learn and understand the controller for r ROV applications in industry.

- Learn and understand the categories, components, and advantages of various controllers.

- Learn and Understand principles of algorithms control and describe how algorithms and flowcharts can be applied to design, problem-solving and troubleshooting techniques.

- Learn and understand the operating principles of the controllers.

- Learn and understand the connections and configuration of controller

- Learn and understand the programming principle languages.

- Learn and understand the various programming techniques associated with industrial Remotely Operated Vehicle

- Learn and understand the control data structure in different software.

- Learn and understand the control data acquisition principles.

- Learn and understand the control data transmission medium.

- Learn and understand the packages, libraries and data handling in software.

- Learn and understand the algorithm for the specified movement

- Understand the performance measurement criteria.

- Learn and understand to check the proper data feed.

- Understand the Point-To-Point (PTP) control mode and Continuous-Path (CP) control mode.

- Learn and understand motion and tasks

- Learn and understand reliability, maintenance and safety measures for ROV.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Integrate the structure of  Remotely Operated Vehicle

- Determine the principles of algorithms control and describe how algorithms and flowcharts can be applied to design, problem-solving and troubleshooting technique.

- Program the Remotely Operated Vehicle for the distance measuring sensor from collision avoidance and measuring distance from object.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Manipulator links and joints |
|  | Measuring tool kit |
|  | Robotic fastening tool kit. |
|  | Trainer |
|  | Manipulator links and joints |
|  | Computer system |
|  | Mounting accessories |
|  | Complete design of the sensor assembly |
|  | Basic ROV components |
|  | Complete design of the ROV |
|  | Software |
|  | Sensors |
|  | Trainer |
|  | Wiring and accessories |

### Engineering Drawing

# Perform Basic Manual Drawing

Overview: This competency standard covers the skills and knowledge required to Draw single stroke capital vertical lettering, Draw single stroke capital inclined lettering, Draw horizontal, vertical and inclined lines, Use of compass, circles, half circles, radius, Drawing Centre   lines, centres, curves, and crossing of lines, Construction of parallel-lines, perpendicular, bisects line, angles and equal division of lines, Draw round corners, circles elements, quadrilaterals inside and outside circle and Construction of angles and triangles.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Draw single stroke capital vertical lettering | - Prepare Drawing sheet. - Select the tools. - Use Proper pencil for lettering with holding techniques. - Draw Boundaries lines as per standards. - Make title bar - Draw upper and lower lines for lettering according to standards. - Start Writing Vertical Lettering with different style like Gothic, Roman and free hand lettering. |
| - CU2. Draw single stroke capital inclined lettering. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Draw upper and lower lines for lettering according to standards. - Start Writing inclined Lettering with different style like Gothic, Roman and free hand lettering. |
| - CU3. Draw horizontal, vertical and inclined lines. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw lines at 30°, 45°, 60°, 90° and 120° angles. |
| - CU4. Draw circles, half circles, radius with compass | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia meter circles and half circles. |
| - CU5. Draw Lines | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Centre lines, - Draw parallel-lines, - Draw perpendicular & bisects line, - Draw equal division of lines - Make different angle curves. - Draw crossing line |
| - CU6. Draw round corners, circles elements, quadrilaterals inside and outside circle. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia circles. - Make inside and outside different types of diagrams that touch the circles at the tangent points. |
| - CU7. Construct angles and triangles | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Equilateral Triangle, Isosceles triangle, Scalene Triangle, Right Triangle, Obtuse Triangle, Acute Triangle. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Importance of Technical Drawing.

- Describe Language of engineering terminology.

- Define application of Technical Drawing

- Describe Type of Drawing

- Describe Drawing Pencil, their grading, sharpening and using techniques.

- Describe Style of letters.

- Define General rules for lettering

- Describe List of drawing equipment’s

- Define Basic lines

- Describe Importance of lines

- Describe Common Types of lines and correct line weightage.

- Define Application of lines.

- Describe Introduction to geometry

- Define Introduction to sketching techniques.

- Describe Techniques of sketching straight lines in different directions.

- Define Triangles, Quadrilateral, and Polygons.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform to draw single stroke capital vertical lettering

- Perform to draw single stroke capital inclined lettering

- Perform to draw  circles, half circles, radius with compass

- Perform to construct  angles and triangles

- Explain Different types of Technical Drawing methods and engineering terminology.

- Explain application of Technical Drawing

- Identify common Types of lines and correct line weightage.

- Identify Triangles, Quadrilateral, and Polygons.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Tool & Equipment |
|  | Graph and drawing sheet. |
|  | Drawing Board/Table. |
|  | T-Square |
|  | Set Square. |
|  | Templets. |
|  | Geometry Box. |
|  | Tee-square and compass. |
|  | Geometry Box. |

# Construct different Engineering Curves

Overview: This competency standard covers the skills and knowledge required to Construct inscribe and circumscribe figures, Construct a pentagon, Hexagon and Octagon by circumscribe method, Construct a pentagon, Hexagon and Octagon by inscribe method, Construct a Tangents of circles (Inside & Outside) When the centre of the given circle is known and when the circle of centre is not known, Construct an Ellipse by Concentric Circle Method, Rectangle Method, Oblong Method, Arcs of Circle Method, Rhombus Method and Basic Locus Method, Construct a parabola curve by Rectangle Method, Method of Tangents( Triangle Method) and Basic Locus Method, Construct a hyperbola curve, Construct a Archimedean Spiral curve, Construct a involutes curve of square rectangle hexagon and circle and Construct of cycloid, epicycloids, and hypocycloid.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Construct inscribe and circumscribe figures. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw square, triangle and hexagon according to dimension. |
| - CU2. Construct a pentagon, Hexagon and Octagon by circumscribe method. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon |
| - CU3. Construct pentagon, Hexagon and Octagon by inscribe method | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon. |
| - CU4. Construct Tangents of circles (Inside & Outside) | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Tangents Inside of a circle When the centre of the circle is known. - Draw Tangents Inside of a circle When the centre of the circle is unknown - Draw Tangents outside of a circle When the centre of the circle is known - Draw Tangents outside of a circle When the centre of the circle is unknown |
| - CU5. Construct Ellipse | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw an Ellipse by Concentric Circle. - Draw an Ellipse by Rectangle Method - Draw an Ellipse by Oblong Method - Draw an Ellipse by Arcs of Circle Method - Draw an Ellipse by Rhombus Method. - Draw an Ellipse by Basic Locus Method |
| - CU6. Construct a parabola curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a parabola curve by Rectangle - Draw a parabola curve by Method of Tangents( Triangle Method) - Draw a parabola curve by Basic Locus Method |
| - CU7. Construct a hyperbola curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a hyperbola curve. |
| - CU8. Construct a Archimedean Spiral curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw spiral curve. |
| - CU9. Construct involute curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw involute curve by square - Draw involute curve by rectangle - Draw involute curve by hexagon - Draw involute curve by circle. |
| - CU10. Construct of cycloid, epicycloid, and hypocycloid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw the generating circle and the base line equal to the circumference of the generating circle - Divide the circle and the base line in to equal number of parts - Complete the cycloid, epicycloids, and hypocycloid. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe techniques of sketching straight lines in different directions

- Define Triangles, Quadrilateral and Polygons

- Describe circular arc using different line method

- Describe circular arc

- Define types of Geometric Shape

- Define two-dimensional shapes

- Define three-dimensional shapes

- Define types of Geometric Shape

- Define regular Polyhedrons

- Define methods of drawing Tangents & Normal

- Describe ellipse

- Describe different methods of sketching ellipse

- Describe parabola

- Describe different methods of parabola

- Describe hyperbola curve

- Describe different methods of hyperbola curve.

- Describe spiral curve

- Describe involute curve

- Describe cycloid

- Describe epicycloids

- Describe hypocycloid

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain different techniques of sketching.

- Perform to Construct Ellipse.

- Perform to Construct  Tangent of circles (Inside & Outside)

- Perform to Construct  pentagon, Hexagon and Octagon by inscribe method

- Identify different types of 2D/3D geometric shapes.

- Explain different methods of sketching ellipse, parabola etc.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Drawing Board/Table |
|  | Set Square |
|  | Geometry Box |
|  | Graph and drawing sheet |
|  | Tea-Square |
|  | Templets Construct multi-view drawings |
|  | French Curve |

# Construct multi-view drawings

Overview: This competency standard covers the skills and knowledge required to Sketch Orthographic projection 1st angle, Sketch Orthographic projection 3rd angle, Sketch Oblique Drawing, Construct multi view drawing of Simple Bearing, Construct multi view drawing of Open Bearing, Sketch prism, Sketch cone and Draw pyramid.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Sketch Orthographic projection in 1st angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view |
| - CU2. Sketch Orthographic projection 3rd angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view |
| - CU3. Sketch Oblique Drawing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts - Draw the front or side view of the object. |
| - CU4. Construct multi view drawing of Simple Bearing. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of simple bearing - Draw front view of simple bearing - Draw side view of simple bearing |
| - CU5. Construct multi view drawing of Open Bearing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of open bearing - Draw front view of open bearing - Draw side view of open bearing |
| - CU6. Sketch prism | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch prism |
| - CU7. Sketch cone | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Start with a horizontal oval - draw the two sides of a triangle which meets at a common point |
| - CU8. Draw pyramid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch pyramid |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain Orthographic projection 1st angle.

- Explain Orthographic projection 3rd angle.

- Explain Oblique Drawing.

- Explain Multi view drawing of Simple Bearing.

- Explain Prism, Cone and pyramid

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform read and Construct multi-view drawings.

- Perform read and Construct Orthographic projection 1st angle.

- Perform read and Construct Orthographic projection 3rd angle.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Graph and drawing sheet. |
|  | Drawing Board/Table. |
|  | Tea-Square |
|  | Set Square. |
|  | Templets. |
|  | Geometry Box. |
|  | French Curve |

# Install  CAD software

Overview: This competency standard covers the skills and knowledge required to install CAD software and create new file and create basic drawing.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Install software and Create New File | - Install latest software version - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits |
| - CU2. Create Basic Drawings | - Select Coordinate System as per requirements - Draw a rectangle using line command - Draw an ARC - Draw a circle with given requirements - Draw a circle with 3-P touching outer corner of Equilateral Triangle - Use the Erase Command |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- This competency standard will provide knowledge related to

- Basic Drawing Settings

- Unit setting

- Limits setting

- User coordinate system Workspace setting

- Object Snap Settings

- Basic Commands and Concepts Angles and lines in CAD software.

- Differentiate between absolute, relative and polar system

- DIMSTYLE and MTEXT commands

- HATCHING concepts in CAD

- Differentiate between CHAMFER and FILLET command

- Types of Array

- OFFSET, CIRCLE and ROTATE short commands

- Zooming options

- Tools palettes window

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform Install CAD software and Create New File

- Perform Create Basic Drawings.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer with all accessories |
|  | AutoCAD software disk |
|  | Models |

# Develop 2D drawings using CAD software

Overview: This competency standard covers the skills and knowledge required to Develop and prepares 2D objects

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Develop 2D Objects | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Save CAD drawing files in different file formats (DWG/IGS/etc., PDF, and JPG). - Create 2D Objects with given measurements - Edit 2D Objects to meet set standards |
| - CU2. Prepare Final Set of 2D Drawings | - Use appropriate command and tools to develop 2D Drawing - Develop 2D Drawing with given project specifications and measurements - Create title block layout as required - Plot drawing on scale according to required size and orientation |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe Basic Drawing Settings

- Describe Unit setting

- Describe Limits setting

- Define User coordinate system Workspace setting

- Describe Object Snap Settings

- Describe Basic Commands and Concepts Angles and lines in CAD.

- Define Differentiate between absolute, relative and polar system

- Define DIMSTYLE and MTEXT commands

- Describe HATCHING concepts in CAD

- Describe Differentiate between CHAMFER and FILLET command

- Describe Types of Array

- Define OFFSET, CIRCLE and ROTATE short commands

- Describe Zooming options

- Describe Tools palettes window

- Describe Design center

- Define Scale and paper sizes

- Describe Modify dimension style and text size according to paper size

- Describe Backup file

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Create 2D drawing

- Develop 2D Drawing with given project specifications and measurements

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer with all accessories |
|  | AutoCAD software disk |
|  | Models |

# Develop 3D drawings using CAD software

Overview: This competency standard covers the skills and knowledge required to develop 3D objects, manipulate and Edit 3D objects and render 3D objects.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Develop 3D Objects | - Setup & save 3D drawing interface for required specifications. - Setup 3D user interface settings for required specifications. - Create 3D objects with given measurements. |
| - CU2. Manipulate 3D objects using 3D Editing Tools | - Modify 3D objects in line with the requirements. - Make customized 3D models according to the requirement of given job. - Convert 3D Face objects into a single mesh objects. |
| - CU2. Render 3D Model | - Apply material to required 3D Model as per given specification - Apply lights to get the requisite scene of required 3D model - Assign cameras to execute different views of required 3D Model. - Render and print the 3D model according to required size & orientation. - Apply texture to 3D model as per given specification. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe 3D modeling in CAD

- Describe 3D solids,

- Describe surfaces, meshes, and Wireframe objects.

- Describe Difference between Surface Modeling and Solid Modeling.

- Define 3D face and Edges

- Define Boolean operation concepts

- Describe Subtraction

- Describe Intersection

- Describe Union

- Define 3D Navigate control

- Describe functions of different camera settings.

- Define Importance of scene creation

- Describe preset views such as isometric, top, bottom, front, left, etc.

- Describe perspective projection and parallel projection

- Define Walk

- Describe Constrained Orbit

- Describe Material and light control

- Describe Planner mapping

- Describe Texture map

- Describe Opacity control

- Define Render context

- Describe Render sampling

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Develop 3D Objects

- Manipulate 3D objects using 3D Editing Tools

- Perform Render a 3D Model

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer with all accessories |
|  | AutoCAD software disk |
|  | Models |