National Vocational Qualification level – 5 Diploma

“Robotics”

(Curriculum)

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

1. 	Introduction	1

1.1.	Purpose of the training program:	1

1.2.	Overall objectives of training program:	2

1.3.	Competencies to be gained after completion of course:	3

1.4.	Possible available job opportunities available immediately and later in the future	9

1.5.	Entry level of trainees:	10

1.6.	Minimum qualification of trainer/instructor:	11

1.7.	Recommended trainer: trainee ratio	11

1.8.	Medium of instruction i.e. language of instruction:	11

1.9.	Duration of the course (Total time, Theory & Practical time):	11

1.10.	Description and structure of the course	12

2.	Overview of the Curriculum for Robotics Technology:	13

A-	Maintain Occupational Health and Safety:	21

0714E&A1: Maintain Health and Safety	21

0714E&A2: Perform safety practices at workplace	32

0714E&A3: Develop professionalism.	39

B-	Basic Electrical Wiring (Single Phase):	47

0714E&A8: Maintain Tools & Equipment.	47

0714E&A9: Make Cable/Wire Joints	49

0714E&A10:	Prepare and Install Distribution Boards	52

0714E&A11: Carryout Basic Electrical Installation	55

0714E&A12: Install Simple Electrical Wiring	58

0714E&A13: Perform Testing of Electrical Wiring	63

0714E&A14: Repair/ Maintenance of Electrical Installations	68

0714E&A15: Carry Out Electrical Planning and Estimation	70

C-	Basic Workshop Practice:	74

0714E&A12: Perform Metal/Bench Work	74

0714E&A13: Demonstrate general machining	87

0714E&A14: Perform cutting on Metal Circular/Power Heck Saw	95

0714E&A15: Perform Basic Lathe Machine Operations	100

0714E&A16: Perform Drilling Machine Operations	122

0714E&A17: Perform Milling Operations	129

D-	Welding:	155

0714E&A18: Interpret Drawing and Execute to Fabricate Parts	155

0714E&A19: Perform Pre-Welding Operations	161

0714E&A20: Evaluate the Use of Hand and Power Tools	169

0714E&A21: Perform Oxy Acetylene Welding	172

0714E&A22: Perform Soldering and Brazing Operations	178

E-	Metrology:	183

0714E&A37: Take measurements with graduated tools	183

0714E&A38: Take measurements with combination set	186

0714E&A39: Perform Leveling.	189

0714E&A40: Take measurements with gauges	191

0714E&A41: Perform measurements through Micrometer	193

0714E&A42: Measure dimensions with Vernier tools	195

F-	Basic Electrical Engineering:	202

0714E&A42: Operate Measuring Instruments.	202

0714E&A43: Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.	204

0714E&A44: Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence	207

0714E&A46: Implement Electromagnet to See Various Effects & Verify Faradays Law	210

0714E&A47: Verify Series and Parallel Combination of Capacitors & Determine Break Down Voltage of Capacitor	214

G-	Basic Electronics (Analog)	217

0714E&A47: Identify Basic Electronics Components	217

0714E&A48: Design Rectifier using Diode	220

0714E&A49: Carry out Diode Application	222

0714E&A50: Implement Bipolar Junction Transistor (BJT) in Different Applications	224

0714E&A51: Implement Field Effect Transistor (FET) in Different Applications	228

0714E&A52: Implement Thyristor Family in Various Application	232

0714E&A53: Application of Operational Amplifier	234

H-	Engineering Drawing	238

0714E&A54: Perform Basic Manual Drawing.	238

0714E&A55: Construct different Engineering Curves	247

0714E&A56: Construct Multi-View Drawings.	258

0714E&A57: Draw basic drawing using CAD.	267

0714E&A58: Develop 2D using CAD software.	269

0714E&A59: Develop 3D drawing using CAD software.	271

I-	Robotic Programming	273

0714E&A82: Basic Programming Skills-I	273

0714E&A83:: Basic ProgrammingSkills-II	275

J-	Digital Electronics	278

0714E&A72: Design Operational Amplifier.	278

0714E&A73: Verify truth table using digital gates.	281

0714E&A74: Construct and verify combinational logic circuits.	288

0714E&A75: Construct and verify functions of flip flop.	293

0714E&A76: Use 555 Timer IC as Multivibrator	296

0714E&A77: Construct shift registers and counters using Flip Flops	298

K-	AC & DC machines (Motor and Generator)	300

0714E&A78: Verify Basic Laws of Electrical Machines	300

0714E&A79: Analyze Dc Generator	304

0714E&A80: Perform Tests On Dc Generators	310

0714E&A81: Operate and test stepper and servo motors	317

L-	Robotic Programming	319

0714E&A60: Object Oriented Programming-I	319

0714E&A61: Object Oriented Programming-II	322

M-	Robotic System	324

0714E&A84: Identify various parts of Robotics.	324

0714E&A85: Assemble various Robotic parts.	326

0714E&A86: Robotics Control Boards	328

0714E&A87: Develop Robot program.	330

0714E&A88:  Develop robotic arm structure.	333

0714E&A89: Configure and test robotic arm	335

0714E&A90: Develop Object Recognition System	337

0714E&A91: Perform Assembly and Operation of Remotely Operated Robot	345

N-	Robotic Maintenance Technician	352

0714E&A92: Perform Robot Calibration	352

0714E&A93:	Inspect Industrial sensors and actuators	355

0714E&A94: Test/Calibrate sensors, Stepper motor& Servo Motor	360

0714E&A95: Ensure Product Quality as per standards	363

0714E&A96: Inspect the hazardous area for installation	365

O-	Robotic Programming	368

0714E&A101: Advanced Programming-I	368

0714E&A102: Advanced Programming-II	371

P-	Robotic System Evaluation	374

0714E&A103: Perform Robot Installation	374

0714E&A104: Diagnose precise robot movement	376

0714E&A105: Diagnose Coupling Parameter	379

0714E&A106: Implement the performance characteristics identified in ISO9283 Standard	382

Q-	Robotic Application	385

0714E&A107: Develop line following Robot	385

0714E&A108: Develop smart Phone Control Robot Using Bluetooth	388

0714E&A109: Develop IoT based Home automation	391

0714E&A110: Develop smart Phone Control Door Lock System	394

0714E&A111: Develop Security System Using Motion Detection	397

0714E&A112: Develop Water Level Detection in Tank	400

0714E&A113: Develop Mini CNC 2D Plotter	403

0714E&A114: Develop Voice Controlled Robot	407

R-	Industrial Electronics	411

0714E&A115: Implement diode and Thyristor in power control application.	411

0714E&A116: Construct single phase and Three phase Inverter	415

Objective: This module covers the skills and knowledge required to Construct single phase half bridge inverter and observe its output, construct single phase full bridge inverter and observe its output and Construct three phase bridge inverter and observe its output Construct single phase half bridge inverter, single phase full bridge, three phase bridge inverter and observe their outputs.	415

0714E&A117: Control Speed of DC/AC Motors	417

S-	Hydraulic & Pneumatic Machinery	420

0714E&A118: Operate Hydraulic Bench and Its Functions	420

0714E&A119: Analyze the performance of Pumps	422

0714E&A120: Working/operation directional control valves	425

0714E&A121: Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder	432

0714E&A122: Actuate Double Acting Hydraulic Cylinder	435

0714E&A123: Construct Circuit for Double Acting Hydraulic Cylinder for Mechanical interlocking	437

0714E&A124: Set Speed and Direction of a Hydraulic Actuators	441

0714E&A125: Perform Pressure measuring instruments operations	442

0714E&A126: Operate Flow Regulating Devices (Valves)	447

0714E&A127: Maintain Pneumatic System	451

List of Tools and Equipment	454

6.	Members of the Curriculum Development Committee	457

# 1. 	Introduction

This qualification gives an introduction to robotics so that the student will have the competence to design and implement robotic systems. This is done by presenting relevant topics from geometry, kinematics, dynamics, control systems, and computer vision. Python programming is an important part of the course to ensure that the student acquires skills in kinematics and computer vision. Systems for robot control using computer vision and contact force sensors will be presented.

Students will gain hands on experience in robotic assembling, synchronization with system, repair & maintenance along with the some fabrication and machining work.

The National Skills Strategy (NSS) aims at achieving a paradigm shift from time-bound   and supply led to competency-based and demand driven training in Pakistan. Competency-based training approach focuses on the demonstration of actual skills required in the workplace/industry. To achieve this strategy, NAVTTC in collaboration with GIZ is involved in the development process of CBT program to ensure competent skilled labor in demand driven vocational trades.

National Vocational Qualification Framework (NVQF) as designed in consultation with the stakeholders including academia, researchers, industry, chambers and TEVTAs. Its aim is to identify knowledge & skills required for a qualification, determine equivalence, provide guidelines for Recognition of Prior Learning (RPL) and assure quality of training. NVQF designed for the Vertical and horizontal progression for learners, recognition of qualification, meeting with the national and international standards, facilitate conversion of informal training to formal training through RPL, improvement in quality of training and increased options for learners for selecting training programs in different trades. Competency Standards are performance specification that identify the knowledge and competencies an individual need to succeed in the workplace.

The National Competency Standards could be used as a referral document for the development of curriculum to be used by training institutions.

## Purpose of the training program:

This qualification will help in the designing the robotic systems. The candidate will be able to perform fundamental operations of robotic logic functions, systems, understand the concepts of various digital systems, their functions and designs, grasp concepts of various memory structures, register counter and their integration with various digital system and there troubleshooting. This qualification will help candidate to assembling, synchronization and construct the robotic system, identify and implement certain robotic components, elaborate linkage between the components and their industrial workability& maintenance of various robotic devices.

This course enables the youth with greater employment opportunities and entrepreneurial skills.

  - Overall objectives of training program:

The overall objective of the Robotics program is to enable students to:

The Robotics technology qualification consists of level 2-5 theoretical and practical competences, which enabling trainee to learn operational techniques of Robotics technology.

The main objectives of the Robotics qualification are as follows:

- Maintain Occupational Health and Safety

- Basic Electrical Wiring (Single Phase)

- Basic Workshop Practices

- Welding

- Digital Skills

- Metrology

- Basic Electrical Engineering

- Basic Electronics (Analog)

- Engineering Drawing

- Robotic Programming (Basic)

- Soft Skills

- Digital Electronics

- AC & DC machines (Motor and Generator)

- Robotic Programming (Object Oriented)

- Robotic System

- Robotic Maintenance Technician

- Machine & Mechanisms

- Robotic Programming (Advance)

- Robotic System Evaluation

- Robotic Application

- Industrial Electronics

- Hydraulic & Pneumatic Machinery

- Develop Entrepreneur Skills

## Competencies to be gained after completion of course:

At the end of the course, the trainee has attained the following competencies:

National Qualification in  Robotics Technology level  2.

- Maintain Health and Safety

- Perform Safety Practices at Workplace

- Develop Professionalism.

- Maintain Tools & Equipment.

- Make Cable/Wire Joints

- Prepare and Install Distribution Boards

- Carryout Basic Electrical Installation

- Install Simple Electrical Wiring

- Perform Testing of Electrical Wiring

- Repair/ Maintenance of Electrical Installations

- Carryout Electrical planning and Installation

- Perform Metal/Bench Work

- Demonstrate general machining

- Perform cutting on Metal Circular/Power Heck Saw

- Perform Basic Lathe Machine Operations

- Perform Drilling Machine Operations

- Perform Milling Operations

- Interpret Drawing and Execute to Fabricate Parts

- Perform Pre-Welding Operations

- Evaluate the Use of Hand and Power Tools

- Perform Oxy Acetylene Welding

- Perform Soldering and Brazing Operations

National Qualification in  Robotics Technology level  3

- Install Computer Operating Systems And Hardware

- Operate Word-Processing Applications

- Operate Spreadsheet Applications

- Operate Presentation Packages

- Perform  Writing And Editing Tasks

- Perform Computer Operations

- Use Computer Applications

- Create User Documentation

- Create Technical Documentation

- Create Basic Databases

- Operate Digital Media Technology

- Use Social Media Tools For Collaboration And Engagement

- E-Commerce

- Use Digital Devices

- Take measurements with graduated tools

- Take measurements with combination set

- Take measurements through various gauges

- Perform measurements through Micrometer

- Measure dimensions with Vernier tools

- Perform different measurements

- Operate Measuring Instruments.

- Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.

- Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

- Implement Electromagnet to See Various Effects & Verify Faradays Laws.

- Verify Law of Combination of Capacitor and Determine Break down voltage of Capacitor.

- Identify Basic Electronics Components

- Design a Rectifier using Diode

- Carry out Diode Application

- Implement Bipolar Junction Transistor (BJT) in Different Applications

- Implement Field Effect Transistor (FET) in Different Applications

- Implement Thyristor Family in Various Application

- Applications of Operation Amplifier

- Perform Basic Manual Drawing

- Construct different Engineering Curves

- Construct multi-view drawings

- Install CAD Software

- Develop 2D CAD drawings

- Develop 3D CAD drawings

- Basic Programming skills-I

- Basic Programming skills-II

National Qualification in  Robotics Technology level 4

- Develop Workplace Policy and Procedures for Sustainability

- Maintain Professionalism in the Workplace

- Manage Personal Work Priorities and Professional Development

- Manage Workforce Planning

- Undertake Project Work

- Prepare and Implement Negotiation

- Manage Meetings

- Organize Schedules

- Identify and Communicate Trends in Career Development

- Apply Specialist Interpersonal and Counseling Interview Skills

- Design Operation Amplifier

- Verify truth tables of digital gates

- Construct & Verify Combinational logic circuit

- Construct and Verify function of Flip Flops

- Use 555 IC as Multi vibrator

- Construct Shift Registers and Counters using Flip Flops

- Verify Basic Laws of Electrical Machines

- Analyze Dc Generators

- Perform Tests On Dc Generators

- Operate and test stepper and servo motors

- Object Oriented Programming-I

- Object Oriented Programming-II

- Identify various parts of robot

- Assemble various robotic parts

- Robotics control Board

- Develop Robot program

- Develop robotic arm structure

- Configure and test robotic arm

- Develop Object recognition system

- Perform Assembly and Operation of Remotely Operated Robot (ROR)

National Qualification in  Robotics Technology level 5

- Perform Robot Calibration

- Inspect Industrial sensors and actuators

- Test/Calibrate sensors, Stepper motor& Servo Motor

- Ensure Product Quality as per standards

- Inspect the hazardous area for installation

- Demonstrate Simple Mechanisms

- Perform Kinematic Analysis

- Demonstrate Mechanical Power Transmission Elements

- Perform Kinematic Analysis of Mechanical power Transmission Elements

- Advanced Programming-I

- Advanced Programming-II

- Perform Robot Installation and Maintenance Principles

- Perform Diagnose precise robot movement

- Perform Diagnose Coupling Parameters

- Implement the performance characteristics identified in ISO9283 standards

- Develop Line Following Robot

- Develop smart phone control robot using Bluetooth

- Develop IoT Controlled i based home automation system

- Develop Smart Phone Control Door Lock System

- Develop Security System Using Motion Detection

- Develop  Water-level Detection in Tank

- Develop Mini CNC 2D Plotter

- Develop Voice Controlled Robot

- Implement diode and Thyristor in power control application.

- Implement single phase and Three phase Inverter

- Control speed of DC/AC Motor

- Operate Hydraulic Bench and Its Functions

- Analyze the performance of Pumps

- Working/operation directional control valves

- Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder

- Actuate Double Acting Hydraulic Cylinder

- Construct Circuit for Double Acting Hydraulic Cylinder

- Set Speed and Direction of a Hydraulic Actuators

- Perform Pressure measuring instruments operations

- Operate Flow Regulating Devices (Valves)

- Maintain Pneumatic System

- Investigate Microbusiness Opportunities

- Develop A Micro Business Proposal

- Develop A marketing Plan

- Develop And Review A Business Plan

- Organize Finances For The Micro Business

- Manage Human Resources

- Market Products And Services

- Monitor And Review Business Performance

- Negotiate For Resolving Business Issues

- Manage Personal Finances

- Coordinate A Work Team

- Lead Small Teams

## Possible available job opportunities available immediately and later in the future

Skilled personnel in the field of Robotics technology usually get hired in areas pertaining to the light, medium and heavy engineering sector. They can become:

- Robotic system supervisor

- Robot service technician

- Associate Robot programmer

- Robot installation technician

- Robotic System operator

- Lead Robotic Technician

- Entrepreneur

- Instructor/trainer (Level 4)

- Associate Robotic Engineer

Some experienced personnel in the field of Robotics technology achieve a high level of salaries. There are good prospects for travel both within Pakistan and abroad. The employment outlook in this occupation will be influenced by a wide variety of factors including:

- Trends and events affecting overall employment (especially in the manufacturing industry)

- Location in Pakistan

- Employment turnover (work opportunities generated by people leaving existing positions)

- Occupational growth (work opportunities resulting from the creation of new positions that never existed before)

- Size of the industry

- Flexibility of the applicant (concerning location and schedule of work)

## Entry level of trainees:

The entry for National Vocational Certificate level 2-5, in “Robotics Technology” are given below:

| Title | Entry requirements |
| --- | --- |
| National Vocational Certificate level 2, in Robotics Technology | Entry for assessment for this qualification is open. However, entry into formal training institutes, based on this qualification is candidate having Secondary level (8 grade) and or National Vocational Certificate level 1. |
| National Vocational Certificate level 3, in Robotics Technology | Entry for assessment for this qualification is open. However, entry into formal training institutes, based on this qualification is candidate having Secondary level (8 grade) and or National Vocational Certificate level 2. |
| National Vocational Certificate level 4, in Robotics Technology | Entry for assessment for this qualification is open. However, entry into formal training institutes, based on this qualification is candidate having Matriculation (Science) and or National Vocational Certificate level 3. |
| National Vocational Certificate level 5, in Robotics Technology | Entry for assessment for this qualification is open. However, entry into formal training institutes, based on this qualification is candidate having Matriculation (Science) and or National Vocational Certificate level 4. |

## Minimum qualification of trainer/instructor:

    - Must be a holder of Bachelor of Technology in Electronics (With three years of experience) or Bachelor of Engineering in Electronics or Robotics (with one years of experience).

    - Must be able to communicate effectively both orally and in written form.

    - Must be able to perform all competences, given in Robotics Technology Level 5.

## Recommended trainer: trainee ratio

The recommended maximum trainer: trainee ratio for this program is 1 trainer for 20 to 25 trainees.

## Medium of instruction i.e. language of instruction:

Instructions will be in Urdu/ English/ Local language.

## Duration of the course (Total time, Theory & Practical time):

This curriculum comprises 23 modules. The recommended delivery time is 3989 hours. Delivery of the course could therefore be full time, 5or 6 days a week, for 3 years. Training providers are at liberty to develop other models of delivery, including part-time and evening delivery.

The distribution of contact hours is given below:

Total		-	3989 hours.

Theory	-	1118 hours (28%)

Practical	-	2871 hours (72%)

## Description and structure of the course

Following is the structure of the course:

| Level 4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Soft Skills |  |  |  |  |  |  |
| Develop Workplace Policy and Procedures for Sustainability | Level 4 | Generic | 10 | 66 | 76 | 7.6 |
| Manage meetings with customers | Level 4 | Generic | 10 | 66 | 76 | 7.6 |
| Manage and select your team (workforce) according job requirement | Level 4 | Generic | 10 | 66 | 76 | 7.6 |
| Manage workforce planning | Level 4 | Generic | 10 | 66 | 76 | 7.6 |
| Undertake project work | Level 4 | Generic | 10 | 66 | 76 | 7.6 |
| Prepare and implement negotiation | Level 4 | Generic | 10 | 66 | 76 | 7.6 |
| Maintain professionalism in the workplace | Level 4 | Generic | 10 | 66 | 76 | 7.6 |
| Digital Electronics |  |  |  |  | 0 | 0 |
| Design Operation Amplifier | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Verify truth tables of digital gates | Level 4 | Technical | 11 | 33 | 44 | 4.4 |
| Construct & Verify Combinational logic circuit | Level 4 | Technical | 9 | 27 | 36 | 3.6 |
| Construct and Verify function of Flip Flops | Level 4 | Technical | 4 | 12 | 16 | 1.6 |
| Use 555 IC as Multi vibrator | Level 4 | Technical | 6 | 9 | 15 | 1.5 |
| Construct Shift Registers and Counters using Flip Flops | Level 4 | Technical | 8 | 12 | 20 | 2 |
| AC & DC machines (Motor and Generator) |  |  |  |  |  |  |
| Verify Basic Laws of Electrical Machines | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Analyze Dc Generators | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Perform Tests On Dc Generators | Level 4 | Technical | 10 | 30 | 40 | 4 |
| Operate and test stepper and servo motors | Level 4 | Technical | 4 | 12 | 16 | 1.6 |
| Robotic Programming |  |  |  |  | 0 | 0 |
| Object Oriented Programming-I | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Object Oriented Programming-II | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Robotic System |  |  |  |  |  |  |
| Identify various parts of robot | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Assemble various robotic parts | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Robotics control Board | Level 4 | Technical | 8 | 24 | 32 | 3.2 |
| Develop Robot program | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Develop robotic arm structure | Level 4 | Technical | 4 | 12 | 16 | 1.6 |
| Configure and test robotic arm | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Develop Object recognation system | Level 4 | Technical | 10 | 15 | 25 | 2.5 |
| Perform Assembly and Operation of Remotely Operated Robot (ROR) | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Engineering Drawing |  |  |  |  | 0 | 0 |
| Perform Basic Manual Drawing | Level 4 | Technical | 7 | 21 | 28 | 2.8 |
| Construct different Engineering Curves | Level 4 | Technical | 10 | 30 | 40 | 4 |
| Construct multi-view drawings | Level 4 | Technical | 8 | 24 | 32 | 3.2 |
| Install CAD Software | Level 4 | Technical | 4 | 12 | 16 | 1.6 |
| Develop 2D CAD drawings | Level 4 | Technical | 4 | 12 | 16 | 1.6 |
| Develop 3D CAD drawings | Level 4 | Technical | 6 | 30 | 36 | 3.6 |
| Level 4 Total |  |  | 243 | 957 | 1200 | 120 |
|  |  |  | 20% | 80% |  |  |

# Engineering Drawing

## 0714E&A54: @Perform Basic Manual Drawing.

Objective: This module covers the skills and knowledge required to Draw single stroke capital vertical lettering, Draw single stroke capital inclined lettering, Draw horizontal, vertical and inclined lines, Use of compass, circles, half circles, radius, Drawing Centre   lines, centres, curves, and crossing of lines, Construction of parallel-lines, perpendicular, bisects line, angles and equal division of lines, Draw round corners, circles elements, quadrilaterals inside and outside circle and Construction of angles and triangles.

Duration:  24 Hours					Theory: 07 Hours						Practical: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Draw single stroke capital vertical lettering | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Use Proper pencil for lettering with holding techniques. - Draw Boundaries lines as per standards. - Make title bar - Draw upper and lower lines for lettering according to standards. - Start Writing Vertical Lettering with different style like Gothic, Roman and free hand lettering. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. Practical Activity - Perform to draw single stroke capital vertical lettering. | Total: 3Hrs Theory: 0.5Hrs Practical:2.5Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-Square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw single stroke capital inclined lettering. | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Draw upper and lower lines for lettering according to standards. - Start Writing inclined Lettering with different style like Gothic, Roman and free hand lettering. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. Practical Activity: - Perform to draw single stroke capital inclined lettering. | Total: 4Hrs Theory: 0.5Hrs Practical: 3.5Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-Square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw horizontal, vertical and inclined lines. | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw lines at 30°, 45°, 60°, 90° and 120° angles. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. Practical Activity: - Perform to construct vertical, horizontal, angular lines. | Total: 03Hrs Theory: 1Hrs Practical: 2Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw circles, half circles, radius with compass | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia meter circles and half circles. | - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Define General rules for lettering - Describe drawing equipment’s - Define Basic lines - Describe Introduction to geometry - Define Introduction to sketching techniques. - Describe Techniques of sketching straight lines in different directions. Practical Activity: - Perform to draw circles, half circles, radius with compass | Total: 03Hrs Theory: 01 Hrs Practical: 02Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw Lines | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Centre lines, - Draw parallel-lines, - Draw perpendicular & bisects line, - Draw equal division of lines - Make different angle curves. - Draw crossing line | - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. - Define Application of lines. - Define Introduction to sketching techniques. - Describe Techniques of sketching straight lines in different directions. Practical Activity: - Construct a different weight lines and different types of lines. | Total: 04 Hrs Theory: 01 Hrs Practical: 03 Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Draw round corners, circles elements, quadrilaterals inside and outside circle. | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia circles. - Make inside and outside different types of diagrams that touch the circles at the tangent points. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. - Define Application of lines. - Describe Introduction to geometry - Define Introduction to sketching techniques. - Describe Techniques of sketching straight lines in different directions. Practical Activity: - Construct a hexagon with inscribed circle or circumscribed circle method. | Total: 03Hrs Theory: 1Hrs Practical: 02Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |
| - Construct angles and triangles | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Equilateral Triangle, Isosceles triangle, Scalene Triangle, Right Triangle, Obtuse Triangle, Acute Triangle. | - Define Importance of Technical Drawing. - Describe Language of engineering terminology. - Define application of Technical Drawing - Describe Type of Drawing - Describe Drawing Pencil, their grading, sharpening and using techniques. - Describe Style of letters. - Define General rules for lettering - Describe List of drawing equipment’s - Define Basic lines - Describe Importance of lines - Describe Common Types of lines and correct line weightage. - Define Application of lines. - Describe Introduction to geometry - Define Introduction to sketching techniques. - Describe Techniques of sketching straight lines in different directions. - Define Triangles, Quadrilateral, and Polygons. Practical Activity: - Perform to construct angles and triangles. | Total: 04Hrs Theory: 1Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets |  | Class Room or Demonstration Room Workshop |

## 0714E&A55: @Construct different Engineering Curves

This module covers the skills and knowledge required to Construct inscribe and circumscribe figures, Construct a pentagon, Hexagon and Octagon by circumscribe method, Construct a pentagon, Hexagon and Octagon by inscribe method, Construct a Tangents of circles (Inside & Outside) When the centre of the given circle is known and when the circle of centre is not known, Construct an Ellipse by Concentric Circle Method, Rectangle Method, Oblong Method, Arcs of Circle Method, Rhombus Method and Basic Locus Method, Construct a parabola curve by Rectangle Method, Method of Tangents( Triangle Method) and Basic Locus Method, Construct a hyperbola curve, Construct a Archimedean Spiral curve, Construct a involutes curve of square rectangle hexagon and circle and Construct of cycloid, epicycloids, and hypocycloid.

Duration:  40 Hours					Theory: 10Hours						Practical: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Construct inscribe and circumscribe figures. | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw square, triangle and hexagon according to dimension. | - Describe techniques of sketching straight lines in different directions - Define Triangles, Quadrilateral and Polygons - Describe circular arc using different line method - Describe circular arc - Define types of Geometric Shape Practical Activity - Perform to construct pentagon, Hexagon and Octagon by circumscribe method. | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct a pentagon, Hexagon and Octagon by circumscribe method. | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon | - Describe techniques of sketching straight lines in different directions - Define Triangles, Quadrilateral and Polygons - Describe circular arc using different line method - Describe circular arc - Define types of Geometric Shape - Define two-dimensional shapes - Define three-dimensional shapes - Define types of Geometric Shape Practical Activity - Perform to construct pentagon, Hexagon and Octagon by circumscribe method. | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct pentagon, Hexagon and Octagon by inscribe method | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon. | - Describe techniques of sketching straight lines in different directions - Define Triangles, Quadrilateral and Polygons - Describe circular arc using different line method - Describe circular arc - Define types of Geometric Shape - Define two-dimensional shapes - Define three-dimensional shapes - Define types of Geometric Shape Practical Activity - Perform to Construct pentagon, Hexagon and Octagon by inscribe method | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct Tangents of circles (Inside & Outside) | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Tangents Inside of a circle when the center of the circle is known. - Draw Tangents Inside of a circle When the center of the circle is unknown - Draw Tangents outside of a circle When the centre of the circle is known - Draw Tangents outside of a circle When the centre of the circle is unknown | - Define methods of drawing Tangents & Normal - Define regular Polyhedrons - Define methods of drawing Tangents & Normal - Describe ellipse - Describe different methods of sketching ellipse - Describe parabola - Describe different methods of parabola - Describe hyperbola curve - Describe different methods of hyperbola curve. Practical Activity - Construct tangent on circle. | Total: Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct Ellipse | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw an Ellipse by Concentric Circle. - Draw an Ellipse by Rectangle Method - Draw an Ellipse by Oblong Method - Draw an Ellipse by Arcs of Circle Method - Draw an Ellipse by Rhombus Method. - Draw an Ellipse by Basic Locus Method | - Describe spiral curve - Describe involute curve - Describe cycloid - Describe epicycloids - Describe hypocycloid - Describe ellipse Practical Activity - Perform to Construct Ellipse. | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct a parabola curve | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a parabola curve by Rectangle - Draw a parabola curve by Method of Tangents( Triangle Method) - Draw a parabola curve by Basic Locus Method | - Describe parabola - Describe different methods of parabola - Describe spiral curve - Describe involute curve Practical Activity - Perform to construct parabola curve. | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct a hyperbola curve | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a hyperbola curve. | - Describe hyperbola curve - Describe different methods of hyperbola curve. - Describe spiral curve - Describe involute curve Practical Activity - Perform to construct hyperbola curve. | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct a Archimedean Spiral curve | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw spiral curve. | - Describe cycloid - Describe epicycloids - Describe hypocycloid Practical Activity - Perform to Construct spiral curve. | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct involute curve | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw involute curve by square - Draw involute curve by rectangle - Draw involute curve by hexagon - Draw involute curve by circle. | - Describe cycloid - Describe epicycloids - Describe hypocycloid Practical Activity - Perform to construct involute curve. | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct of cycloid, epicycloid, and hypocycloid | Trainee would be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw the generating circle and the base line equal to the circumference of the generating circle - Divide the circle and the base line in to equal number of parts - Complete the cycloid, epicycloids, and hypocycloid. | - Describe cycloid - Describe epicycloids - Describe hypocycloid Practical Activity - Perform to construct cycloid, epicycloid, and hypocycloid. | Total: 1Hrs Theory: 03Hrs Practical: 04Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |

## 0714E&A56: @Construct Multi-View Drawings.

Objective: This module covers the skills and knowledge required to Sketch Orthographic projection 1st angle, Sketch Orthographic projection 3rd angle, Sketch Oblique Drawing, Construct multi view drawing of Simple Bearing, Construct multi view drawing of Open Bearing, Sketch prism, Sketch cone and Draw pyramid.

Duration:  32 Hours						Theory: 08 Hours						Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required |  |
| --- | --- | --- | --- | --- | --- |
| - Sketch Orthographic projection in 1st angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view | - Explain Orthographic projection 1st angle. - Define how to show hidden features in projection drawing. - Describe Front, Side and top views and its placement in 1st angle projection. - Describe 1st angle projection symbol. Practical Activity - Perform to draw 1st angle projection drawing on any given object/part. | Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch Orthographic projection 3rd angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view | - Explain Orthographic projection 3rd angle. - Define how to show hidden features in projection drawing. - Describe Front, Side and top views and its placement in 3rdt angle projection. - Describe 3rd angle projection symbol. Practical Activity - Perform to draw 3rd angle projection drawing on any given object/part. | Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch Oblique Drawing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts - Draw the front or side view of the object. | - Explain Oblique Drawing. - Define types of oblique drawing. - Describe uses of oblique drawing. - Explain general oblique. Practical Activity - Perform to draw oblique view drawing on any given object/part. | Total Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct multi view drawing of Simple Bearing. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of simple bearing - Draw front view of simple bearing - Draw side view of simple bearing | - Explain Multi view drawing of Simple Bearing. - Define object symbols and schematic. Practical Activity - Perform to construct multi view drawing of Simple Bearing. | Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Construct multi view drawing of Open Bearing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of open bearing - Draw front view of open bearing - Draw side view of open bearing | - Explain Multi view drawing of Simple Bearing. - Define object symbols and schematic. Practical Activity - Perform to construct multi view drawing of open Bearing. | Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch prism | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch prism | - Explain Prism, Cone and pyramid. - Define 3D pattern development through parallel line method. Practical Activity - Perform to construct a 3D pattern of prism on given dimensions i.e. L x W x H. | Total Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Sketch cone | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Start with a horizontal oval - draw the two sides of a triangle which meets at a common point | - Explain Prism, Cone and pyramid. - Define 3D pattern development through radial line method. Practical Activity - Perform to construct a 3D pattern of cone on given dimensions i.e. Base Dia x H. | Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |
| - Draw pyramid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch pyramid | - Explain Prism, Cone and pyramid. - Define 3D pattern development through radial line method. Practical Activity - Perform to construct a 3D pattern of cone on given dimensions i.e. Base size x H. | Total: 04Hrs Theory: 01Hrs Practical: 03Hrs | - Steel Scale - Drawing sheet - Drawing bench or board - Set Square - Compass - French curve - T-square - Pencils different grades - Erasers - sharpeners - Paper cutter - Scissors - Adhesive tape - Printer - Multimedia projector - Computer - White chart paper - White board - White board markers (red, blue, green, black) - Permanent markers (black) - File covers - Box files - Graphics and drawing sheets | Class Room or Demonstration Room Workshop |

## 0714E&A57: Install CAD software:

Objective: This module covers the skills and knowledge required to install CAD software and create new file and create basic drawing.

Duration:  16 Hours					Theory: 04Hours						Practical: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Install software and Create New File | - Install latest software version - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits | - Define CAD software installation procedure. - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system Workspace setting - Object Snap Settings Practical Activity - Perform Install CAD software and Create New File | Total: 02 Hrs Theory: 06 Hrs Practical: 08 Hrs | - Computer System - Computer table - CAD software. |  | Class Room or Demonstration Room Workshop |
| - Create Basic Drawings | - Select Coordinate System as per requirements - Draw a rectangle using line command - Draw an ARC - Draw a circle with given requirements - Draw a circle with 3-P touching outer corner of Equilateral Triangle - Use the Erase Command | - Basic Commands and Concepts Angles and lines in CAD software. - Differentiate between absolute, relative and polar system - DIMSTYLE and MTEXT commands - HATCHING concepts in CAD - Differentiate between CHAMFER and FILLET command - Types of Array - OFFSET, CIRCLE and ROTATE short commands - Zooming options - Tools palettes window Practical Activity - Perform to Create Basic Drawings on CAD software. | Total: 02 Hrs Theory: 06 Hrs Practical: 08 Hrs | - Computer System - Computer table - CAD software. |  | Class Room or Demonstration Room Workshop |

## 0714E&A58: @Develop 2D using CAD software.

Objective: This module covers the skills and knowledge required to Develop and prepares 2D objects

Duration:  16 Hours					Theory: 04 Hours						Practical: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Develop 2D Objects | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Save CAD drawing files in different file formats (DWG/IGS/etc., PDF, and JPG). - Create 2D Objects with given measurements - Edit 2D Objects to meet set standards | - Describe Basic Drawing Settings - Describe Unit setting - Describe Limits setting - Define User coordinate system Workspace setting - Describe Object Snap Settings - Describe Basic Commands and Concepts Angles and lines in CAD. - Define Differentiate between absolute, relative and polar system - Define DIMSTYLE and MTEXT commands - Describe HATCHING concepts in CAD - Describe Differentiate between CHAMFER and FILLET command - Describe Types of Array - Define OFFSET, CIRCLE and ROTATE short commands Practical Activity. - Create 2D drawing of given object/part by using CAD software. | Total: 02 Hrs Theory: 06 Hrs Practical: 08 Hrs | - Computer System - Computer table - CAD software. |  | Class Room or Demonstration Room Workshop |
| - Prepare Final Set of 2D Drawings | - Use appropriate command and tools to develop 2D Drawing - Develop 2D Drawing with given project specifications and measurements - Create title block layout as required - Plot drawing on scale according to required size and orientation | - Describe Zooming options - Describe Tools palettes window - Describe Design center - Define Scale and paper sizes Practical Activity. - Develop 2D Drawing with given project specifications and measurements | Total: 02 Hrs Theory: 06 Hrs Practical: 08 Hrs | - Computer System - Computer table - CAD software |  | Class Room or Demonstration Room Workshop |

## 0714E&A59: @Develop 3D drawing using CAD software.

Objective: This module covers the skills and knowledge required to Prepare to load machine and attachments, Load or assist with loading machine and attachments, Assist with securing machine and attachments, Unload or assist with unloading machine and attachments, Prepare rubber-tired machine for road travel and Drive rubber tired machine on public roads.

Duration:  24 Hours				Theory: 06 Hours				Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - LU1. Develop 3D Objects | - Setup & save 3D drawing interface for required specifications. - Setup 3D user interface settings for required specifications. - Create 3D objects with given measurements. | - Describe 3D modeling in CAD - Describe 3D solids, - Describe surfaces, meshes, and Wireframe objects. Practical Activity - Develop 3D Objects on given object/part by using CAD software. | Total: 02 Hrs Theory: 06 Hrs Practical: 08 Hrs | - Computer System - Computer table - CAD software |  | Class Room or Demonstration Room Workshop |
| - LU2. Manipulate 3D objects using 3D Editing Tools | - Modify 3D objects in line with the requirements. - Make customized 3D models according to the requirement of given job. - Convert 3D Face objects into a single mesh objects. | - Describe Difference between Surface Modeling and Solid Modeling. - Define 3D face and Edges - Define Boolean operation concepts - Describe Subtraction - Describe Intersection - Describe Union - Define 3D Navigate control Practical Activity - Manipulate 3D objects by using 3D Editing Tools | Total: 02 Hrs Theory: 06 Hrs Practical: 08 Hrs | - Computer System - Computer table - CAD software |  | Class Room or Demonstration Room Workshop |
| LU3. Render 3D Model | - Apply material to required 3D Model as per given specification - Apply lights to get the requisite scene of required 3D model - Assign cameras to execute different views of required 3D Model. - Render and print the 3D model according to required size & orientation. - Apply texture to 3D model as per given specification. | - Describe functions of different camera settings. - Define Importance of scene creation - Describe preset views such as isometric, top, bottom, front, left, etc. - Describe perspective projection and parallel projection - Define Walk - Describe Constrained Orbit - Describe Material and light control - Describe Planner mapping - Describe Texture map - Describe Opacity control - Define Render context - Describe Render sampling Practical Activity - Perform Render a 3D Model | Total: 02 Hrs Theory: 06 Hrs Practical: 08 Hrs | - Computer System - Computer table - CAD software |  | Class Room or Demonstration Room Workshop |

# Digital Electronics

## 0714E&A72: @Design Operational Amplifier.

Objective: This module covers the skills and knowledge required to Construct a Non-inverting amplifier using operational amplifier, construct an Inverting amplifier using and Construct a differentiator circuit using operational amplifier. Construct Operation Amplifier.

Duration:  24 Hours					Theory: 06 Hours						Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Construct a Non-inverting amplifier using operational amplifier | - Identify the Operational amplifier and its terminals (Inverting, Non-inverting Inputs &Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of non-Inverting Op-Amp. - Select the components for Non-Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Non-Inverting Op-Amp circuit. - Measure the output frequency response & gain - Draw the characteristic curves of Op-Amp. - Generate the Output report | - Define Op-Amp. - Define all types of Amplifier. Practical Activity - Construct a Non-inverting amplifier using operational amplifier | Total: 08Hrs Theory: 02Hrs Practical:06Hrs | - Resistors - Op-amp - Capacitors - Function generator - Power supply |  | Class Room Training Workshop. Lab/ Field Visit. |
| - Construct an Inverting amplifier using operational amplifier | - Identify the Operational amplifier and its terminals (Inverting, Non-inverting Inputs &Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of Inverting Op-Amp. - Select the components for Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Inverting Op-Amp circuit. - Measure the output frequency response & gain - Draw the characteristic curves of Op-Amp. - Generate the Output report | - Define Op-Amp. - Define all types of Amplifier. - Explain Datasheets. Practical Activity - Construct an Inverting amplifier by using operational amplifier. | Total: 08Hrs Theory: 02Hrs Practical:06Hrs | - Resistors - Op-amp - Capacitors - Function generator - Power supply |  | Class Room Training Workshop. Lab/ Field Visit. |
| - Construct a differentiator circuit using operational amplifier | - Draw the Schematic diagram of differentiator circuit using Op-Amp. - Select the components for differentiator circuit. - Implement differentiator circuit. - Perform the operations of differentiator circuit. - Measure the output, frequency response & gain - Draw the characteristic curves of differentiator circuit. - Generate the Output report | - Define differentiators. - Explain frequency, gain etc. Practical Activity - Construct a differentiator circuit using operational amplifier. | Total: 08Hrs Theory: 02Hrs Practical:06Hrs | - Resistors - Op-amp - Capacitors - Function generator Power supply |  | Class Room Training Workshop. Lab/ Field Visit. |

## 0714E&A73: @Verify truth table using digital gates.

Objective: This module covers the skills and knowledge required to Verify the truth table of AND gate, Verify the truth table of OR gate, Verify the truth table of NOT gate, Verify the truth table of NAND gate, Verify the truth table of NOR gate, Verify the truth table of XOR gate and Verify the truth table of XNOR gate Verify the truth table of AND gate, OR gate, NOT gate, NAND gate, NOR gate, XOR gate and XNOR gate.

Duration:  44 Hours					Theory: 11 Hours						Practical: 33 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Verify the truth table of AND gate | - Identify the symbol of logic gate, IC & logic function. - Place (AND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain all types of gates. - Define truth table. - Define ICs. Practical Activity - Perform Verification of truth table of AND gate and verify by using LED. | Total: 04Hrs Theory: 01Hrs Practical:03Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm |  | Class Room Training Workshop. Lab/ Field Visit. |
| - Verify the truth table of OR gate | - Identify the symbol of logic gate, IC & logic function. - Place (OR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain all types of gates. - Define truth table. - Define LEDs. Practical Activity - Perform Verification of truth table of OR gate and verify by using LED.. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm |  | Class Room and workshop |
| - Verify the truth table of NOT gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOT gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. Practical Activity - Perform to make NOT gate and verify by using LED. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm |  | Class Room and workshop |
| - Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. - Explain NAND Gate. Practical Activity - Construct and Prove NAND gate with the help of IC and LED. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm |  | Class Room and workshop |
| - Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. - Explain NAND Gate. Practical Activity - Construct and Prove NAND gate with the help of IC and LED. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm gate |  | Class Room and workshop |
| - Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. - Explain NAND Gate. Practical Activity - Construct and Prove NAND gate with the help of IC and LED and verify the truth table. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm gate |  | Class Room and workshop |
| - Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. - Explain NAND Gate. Practical Activity - Construct and Prove NAND gate with the help of IC and LED. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm gate |  | Class Room and workshop |
| - Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. - Explain NAND Gate. Practical Activity - Construct and Prove NAND gate with the help of IC and LED. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm |  | Class Room and workshop |
| - Verify the truth table of NOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. - Explain NOR Gate. Practical Activity - Construct and Prove NOR gate with the help of IC and LED. | Total: 04Hrs Theory: 01Hrs Practical:03Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm |  | Class Room and workshop |
| - Verify the truth table of XOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. - Explain XOR Gate. Practical Activity - Construct and Prove XOR gate with the help of IC and LED. | Total: 04Hrs Theory: 01Hrs Practical:03Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm |  | Class Room and workshop |
| - Verify the truth table of XNOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. | - Explain logic gate ICs. - Explain NOR gate. Practical Activity - Construct and Prove XNOR gate with the help of IC and LED. | Total: 04Hrs Theory: 01Hrs Practical:03Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board - Wire 0.5 mm |  | Class Room and workshop |

## 0714E&A74: @Construct and verify combinational logic circuits.

Objective: This competency standard covers the skills and knowledge required to Apply Karnaugh mapping & Boolean algebra to simplify logic expressions, Construct & verify the truth table of Half adder, Construct & verify the truth table of Full adder, Construct & verify the truth table of Half Subtraction,   Operate seven segment display with seven segment decoder, Construct & verify the truth table of Full Subtraction, Verify Encoder, Verify Encoder and Verify multiplexer and DE- multiplexer Construct & verify the truth table of Half adder, full adder and Half Subtraction.

Duration:  36 Hours					Theory: 09 Hours						Practical: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Apply Karnaugh mapping & Boolean algebra to simplify logic expressions | - Identify the SOP & POS - Apply Boolean algebra & Karnaugh mapping to simplify SOP & POS. - Construct logic circuits with simplified SOP & POS. | - Explain Karnaugh mapping & Boolean algebra to simplify logic expressions - Explain seven segment display with seven segment decoder - Explain truth table and logic circuit.. Practical Activity Perform to Apply Karnaugh mapping & Boolean algebra to simplify logic expressions. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Tool set - Bread board Wire 0.5 mm |  | Class Room and workshop |
| - Construct & verify the truth table of Half adder | - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a half-adder circuit using one XOR gate and two NAND gates. | - Operate seven segment display with seven segment decoder - Define Adder circuits. Practical Activity Construct an Inverting amplifier Construct a logic gates with the help of IC and LED. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - gate - OR gate - NAND gate - XOR gate - NOR gate NOT gate |  | Class Room and workshop |
| - Construct & verify the truth table of Full adder | - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a full-adder circuit using two ICS, &7486 and &7400. | - Operate seven segment display with seven segment decoder Practical Activity Construct an adder circuits. | Theory- 01 Hrs Practical- 03Hrs Total- 04 Hrs | - gate - OR gate - NAND gate - XOR gate - NOR gate NOT gate |  | Class Room and workshop |
| - Construct & verify the truth table of Half Subtraction | - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input | - Explain Karnaugh mapping & Boolean algebra to simplify logic expressions - Operate seven segment display with seven segment decoder - Define Adder circuits. Practical Activity Construct and verify the truth table of half subtraction. | Theory- 01 Hrs Practical- 03Hrs Total- 04 Hrs | - gate - OR gate - NAND gate - XOR gate - NOR gate NOT gate |  | Class Room and workshop |
| - Construct & verify the truth table of Full Subtraction | - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. | - Explain Karnaugh mapping & Boolean algebra to simplify logic expressions Practical Activity Construct and verify the truth table of full subtraction. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - gate - OR gate - NAND gate - XOR gate - NOR gate NOT gate |  | Class Room and workshop |
| - Verify Decoder | - Place (Decoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. | - Explain Karnaugh mapping & Boolean algebra to simplify logic expressions - Demonstrate seven segment display with seven segment decoder Practical Activity Construct a decoder with the help of IC and LED. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - gate - OR gate - NAND gate - XOR gate - NOR gate NOT gate |  | Class Room and workshop |
| - Operate seven segment displays with seven segment decoder. | - Insert (7 segment decoder IC) and 7 segment display on bread board. - Identify the input, output, Vcc and ground pin. - Connect segment display with seven segment decoder input output pins. - Record & verify the output result against each given input. | - Explain Karnaugh mapping & Boolean algebra to simplify logic expressions - Operate seven segment display with seven segment decoder Practical Activity Construct a decoder with the help of IC and LED. | Theory Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - gate - OR gate - NAND gate - XOR gate - NOR gate NOT gate |  | Class Room and workshop |
| - Verify Encoder | - Place (Encoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. | - Explain Karnaugh mapping & Boolean algebra to simplify logic expressions - Operate seven segment display with seven segment decoder Practical Activity Construct and verify encoder circuits. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - gate - OR gate - NAND gate - XOR gate - NOR gate NOT gate |  | Class Room and workshop |
| - Verify multiplexer and DE- multiplexer | - Implement following function with multiplexer F(ABC)=∑ (0,2,3,4,5,6): - Implement 4-to-1 mux and one 2-to-1 mux. - Implement 1-to-4 dmux using 1-to-2 dmux. | - Explain DMUX. - Explain MUX. Practical Activity Construct DMUX circuit and verify. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - gate - OR gate - NAND gate - XOR gate - NOR gate NOT gate |  | Class Room and workshop |

## 0714E&A75: Construct and verify functions of flip flop.

Objective: This module covers the skills and knowledge required to Demonstrate  work ethics, Aware of factors affecting personal health, Resolve problems or disagreements with others, Participate in professional development, Work with others, Work independently, Speak and listen effectively, Interpret documentation, Communicate with signals and Communicate with electronic equipment

Duration:  16 Hours					Theory: 04 Hours						Practical: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Construct and verify the truth table of RS latch | - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. | - Explain the truth table of RS latch using NAND gate - Explain the truth table of clocked RS latch using NAND gate - Explain the truth table of RS latch using NAND gate, clocked RS latch using NAND gate, D flips flop and JK flip flop. Practical Activity. Construct the truth table of clocked RS latch using NAND gate | Theory- 01 Hrs Practical- 03Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Digital clock |  | Class Room and workshop |
| - Construct and verify the truth table of clocked RS latch using NAND gat | - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. | - Explain the truth table of RS latch using NAND gate - Explain the truth table of clocked RS latch using NAND gate - Explain the truth table of RS latch using NAND gate, clocked RS latch using NAND gate, D flip flop and JK flip flop. Practical Activity. Construct the truth table of RS latch using NAND gate, clocked RS latch using NAND gate, D flip flop and JK flip flop. | Theory- 01 Hrs Practical- 03Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Digital clock |  | Class Room and workshop |
| - Verify function of D flip flop. | - Insert (D flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. | - Explain the function of D flip flop. - Explain the functions of oscilloscope. Practical Activity. Construct the D flip flop and Verify the truth tables. | Theory- 01 Hrs Practical- 03Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate Digital clock |  | Class Room and workshop |
| - Verify function of JK/T flip flop | - Insert 74112 (JK flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. | - Explain the function of JK flip flop. - Explain all types of flip flop. Practical Activity. Construct the JK flip flop and Verify the truth tables. | Theory- 01 Hrs Practical- 3Hrs Total- 04 Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Digital clock |  | Class Room and workshop |

## 0714E&A76: @Use 555 Timer IC as Multivibrator

Objective: This module covers the skills and knowledge required to Construct 555 IC as Actable Multivibrator, Construct 555 IC as Mono-stable Multivibrator, and Construct 555 IC as Bi-stable Multivibrator and verify its set and reset conduction. Construct 555 IC as Actable, nonstable & beatable Multivibrator and observe their outputs.

Duration:  15 Hours					Theory: 06 Hours						Practical: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - LU1. Construct Astable Multivibrator using 555 IC | - Draw circuit diagram for Actable Multivibrator - Place 555 IC on bread board/trainer - Make connection as per diagram. - Apply voltage to circuit. - Recode the output signal wave shape from oscilloscope. | - Explain all types of multivibrators. - Explain the operations of 555 IC. Practical Activity Construct 555 IC as Astable Multivibrator | Theory- 02 Hrs Practical- 03 Hrs Total- 05Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Digital clock - 555 IC |  | Class Room and workshop |
| - LU2. Construct Mono-stable Multivibrator using 555 IC | - Draw circuit diagram for Mono-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. | - Explain all types of multivibrators. - Explain the operations of 555 IC. - Explain the waveform of oscilloscope. Practical Activity Construct 555 IC as Mono-stable Multivibrator | Theory- 02 Hrs Practical- 03 Hrs Total- 05Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Digital clock - 555 IC |  | Class Room and workshop |
| - LU3. Construct Bi-stable Multivibrator using 555 IC and verify its set and reset conduction | - Draw circuit diagram for Bi-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. | - Explain all types of multivibrators. - Explain the operations of 555 IC. - Explain the waveform of oscilloscope. Practical Activity Construct Bi-stable Multivibrator using 555 IC and verify its set and reset conduction | Theory- 02 Hrs Practical- 03 Hrs Total- 05Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate - Digital clock - 555 IC |  | Class Room and workshop |

## 0714E&A77: @Construct shift registers and counters using Flip Flops

Objective: This module standard covers the skills and knowledge required to construct a 4-bit shift register by Using Flip Flops, construct a 4-bit binary counter Using Flip Flops, Construct 4-bit synchronous counter with D flip-Flops and Repair & Troubleshoot combinational logic circuits.

Duration:  20 Hours					Theory: 08 Hours						Practical: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Construct a 4 bit shift register by Using Flip Flops | - Draw circuit diagram 4-bit register. - Make connection of D-Flip Flop as per diagram to construct 4-bit shift register. - Apply data at the input of register and give clock pulse - Recode the output according to the input. | - Explain all types of Flip Flops. - Explain function generator - Explain oscilloscope. - Explain shift register. Practical Activity Construct a 4 bit shift register by Using Flip Flops | - 02 Hrs Practical- 03 Hrs Total- 05Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate |  | Class Room and workshop |
| - Construct a4-bit binary counter Using Flip Flops | - Draw circuit diagram counter. - Make connection of JK-Flip Flop as per diagram to construct 4-bit binary counter. - Connect LEDs to the outputs pins. - Apply the clock pulse and record the output. | - Explain all types of Flip Flops. - Explain binary counters. - Explain shift register. Practical Activity Construct a4-bit binary counter Using Flip Flops | - 02 Hrs Practical- 03 Hrs Total- 05Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate |  | Class Room and workshop |
| - Construct 4-bitsynchronous Counter with D flip-Flops | - Draw circuit diagram synchronous counter. - Make connection of JK-Flip Flop as per diagram to construct 4-bit synchronous counter. - Connect LEDs to the output pins. - Apply the clock pulse and record the output. | - Explain all types of Flip Flops. - Explain binary counters. - Explain shift register. - Explain synchronous and asynchronous counter. Practical Activity Construct 4-bitsynchronous Counter with D flip-Flops | - 02 Hrs Practical- 03 Hrs Total- 05Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate |  | Class Room and workshop |
| - Repair & Troubleshoot - combinational logic circuits | - Identify principles and operations of types of logic gates - Locate drawing and diagrams. - Check the grounding system in electronic equipment. - Locate external and internal digital IC faults | - Explain the grounding system. - Explain the IC faults. Practical Activity Check the grounding system in electronic equipment and make combination of logic gates. | - 02 Hrs Practical- 03 Hrs Total- 05Hrs | - AND gate - OR gate - NAND gate - XOR gate - NOR gate - NOT gate |  | Class Room and workshop |

# AC & DC machines (Motor and Generator)

## 0714E&A78: @Verify Basic Laws of Electrical Machines

Objective: This module covers the skills and knowledge required to verify Faraday’s law by moving permanent magnet inside the coil, verify Faraday’s law by moving coil near the magnet field, verify Faraday’s law using relative motion of coil and magnet, verify Faraday’s Law using simple loop generator, verify EMF through induction, verify Torque induce in a current carrying loop and verify Mutual induction.

Duration:  24 Hours					Theory: 6Hours						Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Verify Faraday’s law by moving permanent magnet inside the coil. | - Construct a coil. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow. - Record the effect of movement of magnet on reading of Galvanometer. - Hold the magnet inside the coil and do not move. - Record the effect on reading of Galvanometer again. | - Define first law of Faraday - Define second law of Faraday - Define to find the direction of induced emf - Define the role of magnetic strength in Faraday’s Law - PRACTICAL ACTIVITY: - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow. Record the effect of movement of magnet on reading of Galvanometer. | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Coil - Galvanometer. - Voltmeter - Transformer. - Magnet. - Connecting leads. - Transformer - DC source - Rheostat | - Class room/ Demonstration room/ workshop |
| - Verify Faraday’s law by moving coil near the magnet field. | - Construct a coil. - Connect Galvanometer with coil. - Fix permanent magnet and move the coil fast and slow on it. - Record the effect of movement of coil on reading of Galvanometer. - Hold the coil near the magnetic field and do not move. - Record the effect on reading of Galvanometer. | - Define the role of conductor’s length or turns in Faraday’s Law - Define the emf induced in 2nd coil, when voltage is applied to 1st coil. - Define what happens when a magnet is moved in and out of a wire coil. - Explain what happens when a magnet is pushed into a coil. - PRACTICAL ACTIVITY: - Connect Galvanometer with coil. Hold the coil near the magnetic field and do not move. Record the effect on reading of Galvanometer | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Coil - Galvanometer. - Voltmeter - Transformer - Magnet. - Connecting leads. - Transformer - DC source - Rheostat - DC source | - Class room/ Demonstration room/ workshop |
| - Verify Faraday’s law using relative motion of coil and magnet. | - Construct a coil. - Connect Galvanometer with coil. - Make relative motion of coil and magnet. - Record the effect of movement on reading of Galvanometer. | - Define, what is core - Define, what is induction - Define Faraday’s first law of Electro-Magnetic Induction - Define the meaning of coupled coils. - Define what happens to the current in the coil when a magnet is accelerated in the coil. - PRACTICAL ACTIVITY: - Identify how changing magnetic fields can cause current to flow in wires. | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Coil - Galvanometer. - Voltmeter - Transformer - Magnet. - Connecting leads. - Transformer - DC source - Rheostat | - Class room/ Demonstration room/ workshop |
| - Verify Faraday’s Law using simple loop generator. | - Identify single loop generator and its parts. - Select field winding. - Connect Galvanometer with single loop coil. - Rotate the single loop coil of generator in the field - Measures the voltage induces in loop generator. | - Define how Faraday’s law applies to a generator. - Explain how you calculate the emf of a generator. - Explain how you increase the EMF of a generator. - PRACTICAL ACTIVITY: - Select field winding and Connect Galvanometer with single loop coil. | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Coil - Galvanometer. - Voltmeter - Transformer - Magnet. - Connecting leads. - Transformer - DC source - Rheostat | - Class room/ Demonstration room/ workshop |
| - Verify Torque induce in a current carrying loop | - Construct a coil. - Apply DC voltage to this coil. - Place a current carrying loop in this coil. - Check the direction of force to verify the torque produce. | - Define the Lenz’s Law - Define the torque - Define force produce on current carrying conductor. - Describe what Torque is on a current loop. - Define how you find the magnetic field in a circular loop. - Explain what a current carrying circular loop is. - Explain the difference between coil and loop. - PRACTICAL ACTIVITY: - Check the direction of force to verify the torque produce by the Dc voltage to this coil. | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Coil - Galvanometer. - Voltmeter - Transformer - Magnet. - Connecting leads. - Transformer - DC source - Rheostat | - Class room/ Demonstration room/ workshop |
| - Verify Mutual induction | - Construct 2 coils on two different legs of single core. - Apply AC voltage to the any one coil. - Check the volts induce in another coil by volt | - Define what mutual induction is. - Define, Which machine work on the principle of mutual induction - Describe what is self-inductance and mutual inductance. - Define what the formula for mutual inductance is. - PRACTICAL ACTIVITY: - Check the volts induce in other coil by volt on two different legs of single core. | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Coil - Galvanometer. - Voltmeter - Transformer - Magnet. - Connecting leads. - Transformer - DC source - Rheostat | - Class room/ Demonstration room/ workshop |

## 0714E&A79: @Analyze Dc Generator

Objective: This module covers the skills and knowledge required to calculate different terms used in armature winding of DC machines, develop Single Layer Lap Winding, develop Double Layer Wave Winding, measure the resistance of field winding, armature winding and terminal polarity, identify DC shunt generator, identify DC series generator and identify DC compound generator.

Duration:  24 Hours					Theory: 6Hours						Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Calculate different terms used in armature winding of DC machines | - Count the number of slots (S) of armature. - Calculate pole pitch (Y). - Calculate Coil span/pitch. - Calculate Back pitch (Yb) - Calculate Front pitch (Yf) - Calculate Resultant Pitch (YR) - Calculate commentator pitch (Yc) | - Define the difference between armature and field winding. - Explain the term armature slots. - Define the pole pitch (Y). - Define the coil span/pitch - Define the Back pitch (Yb) - Define the Front pitch (Yf) - Define the Resultant Pitch (YR) - Define the commutation pitch (Yc) - PRACTICAL ACTIVITY: - Calculate Front pitch (Yf) used in armature winding of DC machines | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Galvanometer - DC machine - Connecting wires - Sleeve - Armature - Winding machine - Insolating paper - Carbon brushes - Soldering wire - Stator frame | - Class room/ Demonstration room/ workshop |
| - Develop Single Layer Lap Winding | - Count the number of slots in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Subtract the value of back pitch from the added value. - Connect added value from front side to subtracted value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. | - Define the lap winding - Define, How many parallel paths in lap winding - Define the meaning by single layer winding - Define the wave winding - Define how many parallel paths in wave winding. - Define the difference between lap winding and wave winding. - PRACTICAL ACTIVITY: - Calculate average pitch, back pitch and commentator pitch Single Layer Lap Winding | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Galvanometer - DC machine - Connecting wires - Sleeve - Armature - Winding machine - Insolating paper - Carbon brushes - Soldering wire - Stator frame | - Class room/ Demonstration room/ workshop |
| - Develop Double Layer Wave Winding | - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Add the value of front pitch in previous added value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. | - Define the meaning by double layer winding - Demonstrate the reading on the scale of galvanometer. - Define the difference between field winding and armature winding - Explain the technique to identify the terminal polarity - Explain why short pitch winding is used. - Explain what full pitch is winding. - PRACTICAL ACTIVITY: - Develop Double Layer Wave Winding Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Galvanometer - DC machine - Connecting wires - Sleeve - Armature - Winding machine - Insolating paper - Carbon brushes - Soldering wire - Stator frame | - Class room/ Demonstration room/ workshop |
| - Measure the resistance of field winding, armature winding and terminal polarity | - Open the terminal box. - Connect the ohm meter with the terminal of both winding. - Read down ohm meter reading. - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. - Rotate the generator slowly by hand. - Touch the probe of galvanometer with each winding. - Identify armature with the deflection of galvanometer pointer. - Mark the polarity of terminals with the deflection of galvanometer. | - Explain the technique to identify the terminal polarity. - Explain the different between field winding and armature winding - Explain, which winding having low resistance and why. - Explain why field winding resistance high is. - Explain why the series field winding is wound with few turns of thick wire. - Define the types of armature winding. - Explain, which winding having low resistance and why. - PRACTICAL ACTIVITY: - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Galvanometer - DC machine - Connecting wires - Sleeve - Armature - Winding machine - Insolating paper - Carbon brushes - Soldering wire - Stator frame | - Class room/ Demonstration room/ workshop |
| - Identify DC shunt generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC shunt generator from measured value of resistance. - Disconnect both windings. - Identify the shunt field winding of shunt generator. - Identify armature of shunt generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. | - Explain the number of winding in shunt generator - Explain, how to disconnect both winding of shunt generator - Define the characteristics of a DC generator. - Explain how a DC shunts generator work. - Define the different types of DC generators. - PRACTICAL ACTIVITY: - Measure the terminal resistance of DC shunt generator with ohm meter | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Galvanometer - DC machine - Connecting wires - Sleeve - Armature - Winding machine - Insolating paper - Carbon brushes - Soldering wire - Stator frame | - Class room/ Demonstration room/ workshop |
| - Identify DC series generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC series generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series generator. - Identify armature of series generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. | - Define what Series DC generator is. - Explain what the different types of DC generators are. - Describe what the applications of DC series generator are. - Explain how does a DC generator work? - Define what the function of brushes in DC generator is. - Explain what the main parts of a DC generator are. - Define hat is the function of armature in DC generator. - Explain the difference between a DC motor and a DC generator. - PRACTICAL ACTIVITY: - Measure the terminal resistance of DC Series generator with ohm meter | - Theory-01Hrs - Practical-03Hrs - Total- 04Hrs | - Galvanometer - DC machine - Connecting wires - Sleeve - Armature - Winding machine - Insolating paper - Carbon brushes - Soldering wire - Stator frame | - Class room/ Demonstration room/ workshop |

## 0714E&A80:@ Perform Tests on Dc Generators

Objective: This competency standard covers the skills and knowledge required to Plot the load characteristics of D.C shunt motor, plot the load characteristics of D.C series motor, plot the load characteristics of D.C compound motor, control the speed of DC series motor, control the speed of DC shunt motor, start  DC shunt motor with 3 point starter, identify the effect of back EMF in DC motor, calculate BHP of motor by Brake test, calculate efficiency of the motor by Swinburne Test and perform Regenerative / Hopkinson/ Back to back Test.

Duration: 40 Hours						Theory: 10 Hours					Practical: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plot the load characteristics of D.C shunt motor | - Couple electro dynamo meter to the DC motor with belt - Connect shunt motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC shunt motor.. | - Define the techniques to control the speed of DC shunt motor - Explain why we never start DC series motor without load. - Describe what the characteristics of DC shunt motor are. - Explain what a DC shunt motor is used for. - Explain why a DC shunt motor is called a constant speed motor. - Demonstrate the RPM measurement of the motor - PRACTICAL ACTIVITY: - Measure and calculate the current, speed and torque of shunt motor. | Theory – 01 Hrs Practical – 03 Hrs Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Plot the load characteristics of D.C series motor. | - Select DC series motor and achieve connection as per diagram. - Turn on DC power supply and increase the load to full value. - Record the value of speed and current on each load. - Draw the graph between load current and speed | - Define why a dc series motor should not be started without load. - Explain why series winding having low turns and shunt winding having high number of turns. - Explain how a dc series motor works. - PRACTICAL ACTIVITY: - Draw the graph between load current and speed of DC series motor | Theory – 01 Hrs Practical – 03 Hrs Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Plot the load characteristics of D.C compound motor | - Couple electro dynamo meter to the DC motor with belt - Connect compound motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC compound motor. | - Explain what the characteristics of a cumulatively compounded DC motor are. - .Define what is DC compound motor. - Describe what the applications of DC compound motor are. - Explain where DC compound motors used are. - Describe the different between shunt, series and compound motor load characteristics. - Demonstrate the RPM measurement of the motor. - PRACTICAL ACTIVITY: - Set the dynamo meter control knob in such position to produce a minimum starting load on the DC Compound motor. | Theory – 01 Hrs Practical – 03 Hrs - Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Control the speed of DC series motor. | - Make the connection as per circuit diagram. - Insert external resistances in series to armature and field coil. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different supply voltages. - Make graph between speed and applied voltage. | - Define why we need to control the speed of a DC motor. - Describe can any DC motor be variable speed. - Explain which speed control method of dc series motor is used for electric train. - PRACTICAL ACTIVITY: - Make a connection of speed of DC series motor as per circuit diagram. | Theory – 01 Hrs Practical – 03 Hrs - Total – 4 Hrs Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Control the speed of DC shunt motor. | - Make the connection as per circuit diagram. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different field current. - Make graph between speed and field current - Fixed the field current and very armature current - Record the speed at different armature current. - Make graph between speed and armature current | - Explain how can we control the speed of DC shunt motor. - Describe why it is necessary to control the speed of a DC motor. - Explain what methods are used to change the speed of DC shunt motor. - Define what are the factors affecting the speed of a DC shunt motor. - PRACTICAL ACTIVITY: - Make graph between speed and armature current of DC shunt motor. | Theory – 01 Hrs Practical – 03 Hrs - Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Start DC shunt motor with 3 point starter. | - Make the connection as per circuit diagram. - Switch on the supply and move starter handle from off position to position 1. - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. | - Describe what 3-point starter is. - Define why we use 3 point starter in DC shunt motor. - Explain what the drawbacks of 3 point starter are. - Explain why rheostat of DC motor kept in minimum position at starting is. - PRACTICAL ACTIVITY: - Adjusts starter handle from off position to position 1 on Dc shunt motor. | Theory – 01 Hrs Practical – 03 Hrs - Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Identify the effect of back EMF in DC motor. | - Select DC series motor and make connection as per diagram. - Turn on DC power supply and increase the load 0 to full value. - Record the value of speed and current on each load. - Calculate back EMF by using the recoded values and appropriate formula. - Make graph between speed and back EMF and interpret the result. | - Explain what the effect of back EMF in DC motor is. - Define how you calculate back EMF of a dc motor. - Describe what the formula of back EMF is. - Explain what EMF equation of DC motor is. - Define how you reduce the back EMF of a dc motor. - PRACTICAL ACTIVITY: - Calculate back EMF by using the recoded values of current and speed on each load and appropriate formula on Dc motor. | Theory – 01 Hrs Practical – 03 Hrs - Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Calculate Brake Horse Power (BHP) of motor by Brake test. | - Connect the spring balance with one end of the rope. - Connect other end of rope with weight (W1). - Measure the radius of the pulley. - Hang the weight on the pulley of the motor and fix the spring balance end with earth. - Run the motor. - Measured the RPM of the motor. - Adjust the weight to pass full load current from motor. - Calculate spring balance weight and hanged weight (W1) - Calculate shaft torque. - Calculate BHP of motor by shaft torque. | - Define how you calculate brake horsepower. - Describe what BHP formula is. - Explain what BHP is and how it is calculated. - Define how you increase horsepower. - Explain perform the calculation of BHP from shaft torque. - PRACTICAL ACTIVITY: - Measured the RPM of the motor by adjusting load on it. | Theory – 01 Hrs Practical – 03 Hrs - Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Calculate efficiency of the motor by Swinburne Test | - Perform DC test to find the value armature resistance and field resistance - Run the motor at rated voltage without load. - Adjust the value of shunt regulator to attain rated speed. - Measure Field current and No-load current with ampere meter. - Calculate total copper losses and iron losses. - Calculate input and output power. - Calculate efficiency from calculated power. | - Define what is the purpose of performing retardation test after Swinburne's test. - Describe, Why Swinburne's test is economical. - Define, what the disadvantages of Swinburne’s test are. - Explain, Can you conduct Swinburne's test on series motor. - PRACTICAL ACTIVITY: - Measure Field current and No-load current with ampere meter by Swinburne test | Theory – 01 Hrs Practical – 03 Hrs - Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |
| - Perform Regenerative / Hopkinson/ Back to back Test | - Identify two shunt machines. - Couple the shafts of both machines. - Connect the machines electrically in such a way that one machine work as a motor and other as a generator. - Connect external power supply to the machines to overcome the machine losses. - Run the both machine at no load. - Reduce the field excitation of one machine which work as a motor. - Increase the field excitation of one machine which work as a generator. - Adjust the field winding so that voltage of generator becomes equal to the applied voltage which result the machines runs in parallel - Calculate the efficiency by using reading of voltmeter and ammeter. | - Define what Hopkinson test is. - Describe, can Hopkinson's test be performed on any two dc machines. - Explain what the advantages of Hopkinsons test are. - PRACTICAL ACTIVITY: - Connect the machines electrically in such a way that one machine work as a motor and other as a generator. | Theory – 01 Hrs Practical – 03 Hrs - Total – 4 Hrs | - Tachometer - Voltmeter - Ammeter - Connecting wire - Wattmeter - Spring balance - Ohm meter - DC compound motor - Electro dynamo meter | Class room/ Demonstration room/ workshop |

## 0714E&A81: @Operate and test stepper and servo motors

Objective: This module covers the skills and knowledge required to operate and speed control of AC series motor, operate miniature (reluctance and hysteresis) single phase AC motors, construct and operate stepper motor and construct and operate and develop control circuit with the help of servo motor.

Duration: 16 Hours						Theory: 04 Hours					Practical: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Operate and speed control of AC series motor. | - Connect the armature and field winding in series. - Apply rated voltage to the motor. - Measure the effect on motor speed by change in applied voltage. | - Define how AC motor speed controls work. - Explain the working of AC series motor - Explain the effect of applied voltage on speed of the motor. - PRACTICAL ACTIVITY: - Connect the armature and field winding in series of AC series motor. | - Theory – 1 Hrs - Practical – 03 Hrs - Total – 4 Hrs | - AC series motor - Tachometer - Connecting wires - Reluctance motor - Hysteresis motor - Tachometer - Stepper motor - Servo motor - Control circuit | - Class room/ Demonstration room/ workshop |
| - Operate miniature (reluctance and hysteresis) single phase AC motoqrs. | - Connect the stator of motor with single phase supply. - Apply mechanical load by hand - Measure the effect of load change on motor speed. - Draw torque speed curves. | - Explain the working of reluctance motor. - Explain the working of hysteresis motor - PRACTICAL ACTIVITY: - Draw torque speed curves of Single phase Ac motor. | - Theory – 1 Hrs - Practical – 03 Hrs - Total – 4 Hrs | - AC series motor - Tachometer - Connecting wires - Reluctance motor - Hysteresis motor - Tachometer - Stepper motor - Servo motor - Control circuit | - Class room/ Demonstration room/ workshop |
| - Construct and operate stepper motor. | - Connect the stator of the motor with control circuit. - Adjust the control voltage of control unit. - Measure the RPM. - Readjust the control voltage of control unit. - Measure the RPM again. - Make comparison between Control voltage and RPM. | - Explain the working of stepper motor. - Define what the working principle of stepper motor is. - Describe, can stepper motor turn both ways. - PRACTICAL ACTIVITY: - Make comparison between Control voltage and RPM of stepper motor. | - Theory – 1 Hrs - Practical – 03 Hrs - Total – 4 Hrs | - AC series motor - Tachometer - Connecting wires - Reluctance motor - Hysteresis motor - Tachometer - Stepper motor - Servo motor - Control circuit | - Class room/ Demonstration room/ workshop |
| - Construct, Operate and develop control circuit of servo motor. | - Pair the servo motor with some encoder to provide position and speed feedback. - Compare the measured position to external input to the controller. - Check the effect of feedback on motor position. | - Explain the working of servo motor. - Define how you control a servo motor. - Explain, Can servo motors rotate 360. - PRACTICAL ACTIVITY: - Pair the servo motor with some encoder to provide position and speed feedback. | - Theory – 1 Hrs - Practical – 03 Hrs - Total – 4 Hrs | - AC series motor - Tachometer - Connecting wires - Reluctance motor - Hysteresis motor - Tachometer - Stepper motor - Servo motor - Control circuit | - Class room/ Demonstration room/ workshop |

# Robotic Programming

## 0714E&A60: @Object Oriented Programming-I

Objective: This module covers the skills and knowledge on C, Classes, Object, Inheritance, Polymorphism and Encapsulation.

Duration:  24 Hours					Theory: 06 Hours						Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Developing object-oriented programs (OOP) | - Open OOP IDE for coding - Code a simple program and create simple class - Create object of that class - Create variable of object - Compile a code - Debug the code (in case of error) - Run a code | - Define object-oriented programming (OOP). - Define the characteristics of OOP - Differentiate between object and class - Usage of IDE - Define debugging of a code - Describe the class - Describe the object - Explain variable of object - PRACTICAL ACTIVITY: Initialize a Class name “CAR” have data members of name, speed, and model then create object of class and assign “FORD”, 45km/h, Model A. | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - PC System - IDE Software (C++ Language) | Class room/ Demonstration room/ workshop/ Computer lab |
| - Develop OOP based program by using inheritance | - Open OOP IDE for coding - Create a class - Create inherited class. - Create object for inherited class - Utilize data type of inherited class - Utilize data type of parent class - Compile and run the code | - Define Inheritance - Define types of inheritance - Define inherited and parent class - Explain the use of inheritance - PRACTICAL ACTIVITY: Create a parent class as “Shape” have width and height and child class as “Rectangle get its height and weight assign value 5,7 and print the area of rectangle. | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - PC System - IDE Software (C++ Language) | Class room/ Demonstration room/ workshop/ Computer lab |
| - Develop a program using operator overloading | - Open OOP IDE for coding - Create class - Create object of that class - Create variable of object - Utilize overloading - Compile a code - Debug the code (in case of error) - Run a code | - Define the operator overloading - Define a Class - Define Object - Define Constructors and Destructors - Define operator overloading using different operators. - PRACTICAL ACTIVITY: - Initialize the class “Box” - And give the function of get volume - And return volume (LxBxH) - Set function of Len, breadth and Overload + operator to add two Box objects and assign values to each box, and add two box and define the 3rd box to sum the volume of box1 and 2. | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - PC System - IDE Software (C++ Language) | Class room/ Demonstration room/ workshop/ Computer lab |

## 0714E&A61: @Object Oriented Programming-II

Objective: This module covers the skills and knowledge on C, Classes, Object, Polymorphism, Encapsulation and Interfaces.

Duration:  24 Hours					Theory: 06 Hours						Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop a Class in OOPs using polymorphism technique | - Open OOP Compiler - Create class - Create Objects in classes - Create function in class for polymorphism - Call polymorphism functions with specific object - Compile and run the code | - Explain differentiate between object and class - Usage of IDE - Define debugging of a code - Define Polymorphism - Define Parent class and child class - PRACTICAL ACTIVITY: - Make a Class Add have e two function name “SUM” with the one adding two argument while other adding 3 argument then call the 2 object with pass two and three values for each. | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - PC System - IDE Software (C++ Language) | Class room/ Demonstration room/ workshop/ Computer lab |
| - Develop program using Encapsulation technique | - Open OOP Compiler - Create class - Create Objects in classes - Apply encapsulation on defining class - Call protected, public and private data with object - Compile and run the code | - Define and understand Encapsulation - Define protected, public and private data. - PRACTICAL ACTIVITY: - Make a Class name box have private members with num, and char and public members with get number and get constant function and Setter function to assign the number. Call the object with number 100 and char “A” and print the output | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - PC System - IDE Software (C++ Language) | Class room/ Demonstration room/ workshop/ Computer lab |
| - Develop Interface | - Create the interface unit - Declare the reference variable of the object class - Make the methods for child object using encapsulation | - Define and understand interfaces - Define reference variable of the object class - PRACTICAL ACTIVITY: - Create a class name “ANIMAL” for the virtual void function “SOUND” and function sleeping. Make a class dog calling public animal class. And assign sound as woof for dog. Call object of sound and sleep to print. | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - PC System - IDE Software (C++ Language) | Class room/ Demonstration room/ workshop/ Computer lab |

# Robotic System

## 0714E&A84: @Identify various parts of Robotics.

Objective: This module covers the skills and knowledge related to fundamental of robots, Identification of sensors and controller used in robot design and the software development environment to program controller and interfacing of robot parts.

Duration: 24 Hours						Theory: 06 Hours				Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Various types of Sensors | - Identify temperature sensors. - Identify sound sensors. - Identify proximity sensors. - Identify pressure sensors. - Identify light sensors. - Identify position sensors. - Identify voltage sensors. - Identify current sensors. - Identify the vision sensors. - Identify power rating of each sensor. - Identify the signal input/output of each sensor using datasheet. | - Define sensors and transducers - Explain working principle of various sensors. - Describe how to read datasheets of sensors. - PRACTICAL ACTIVITY - Identify type of sensor i.e. Temperature sensor, pressure sensor, proximity sensor, light sensors, etc. | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - Temperature sensors. - Sound sensors. - Proximity sensors. - Pressure sensors. - Light sensors. - Position sensors. - Voltage sensors. - Current sensors. - the vision sensors. - Power rating of each sensor. - Datasheets of sensors | Classroom or Demonstration room Workshop |
| - Identify Various Controllers/Microprocessors | - Identify the open loop controllers. - Identify the closed loop controllers. - Identify the algorithms for both controllers. - Identify the components of both controllers. - Draw circuit diagram for controllers. - Draw the block diagram and flow charts of both controllers. - Identify the power supply for controllers. | - Define open loop control system. - Describe closed loop control system - Explain how to select specific controller. - Describe power supply requirement for controller and other parts. - PRACTICAL ACTIVITY - Name down the list of controllers along their pin outs. | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - Microcontroller - Arduino Uno - Arduino Mega - Raspberry pi | Classroom or Demonstration room Workshop |
| - Identify Designing and Development Software | - Identify the programming languages. - Identify the program development architecture for the robot. - Identify the software tools for the different types of controllers. | - Describe software IDE for microcontrollers - Define software IDE for Arduino controller. - Explain software for Raspberry pi. - PRACTICAL ACTIVITY - Name down the software requirement use for various types of controllers. | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - Computer system - Software of various microcontroller | Classroom or Demonstration room Workshop |

## 0714E&A85: @Assemble various Robotic parts.

Objective: This module covers the skills and knowledge related to fundamental of robots, Identification of sensors and controller used in robot design and the software development environment to program controller and interfacing of robot parts.

Duration: 24 Hours						Theory: 6 Hours				Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Assemble the different types of Manipulators or Rovers. | - Identify parts and main body and of the manipulator. - Identify the links and joints. - Assemble links, joints and overall assembly of the manipulator. - Identify the kinematics (movement) of a manipulator. | - Describe parts of robots - Define Links, joints, assembly and fixers. - Explain kinematics of manipulators. - PRACTICAL ACTIVITY - Identify type of links, joints and end effectors | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - Links - Joints - End effectors - Nuts - Bolts - Drills - Grippers - Conveyor belt - Sensor and actuators | Classroom or Demonstration room Workshop |
| - Set up Various Types of End Effectors with the Robot. | - Identify the types of End Effectors. - Assemble the End Effectors. - Assembly of the End Effectors with the manipulator. - Drives for the End Effector. - Identify the kinematics (movement) of the end effectors. | - Define end effectors. - Describe types of end effectors and their applications - Explain how to run end effectors. - Describe power supply requirement for end effector. - PRACTICAL ACTIVITY - Name down the list of end effectors along their applications. | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - Links - Joints - End effectors - Nuts - Bolts - Drills - Grippers - Conveyor belt - Sensor and actuators | Classroom or Demonstration room Workshop |
| - Assemble various Actuators/Drives with the Robot. | - Identify Hydraulic Actuators. - Identify Electrical Actuators. - Identify Pneumatic Actuators. - Identify Mechanical Actuators. - Identify Locomotive Actuators. - Identify Manipulation Actuators. - Identify the power supply for each actuator. - Assemble actuators with manipulators and end effectors. | - Describe Hydraulic actuators - Explain Electrical actuators - Describe Pneumatic actuators - Define Mechanical actuators - PRACTICAL ACTIVITY - List down actuators name and define their principle of working. | - Theory-02Hrs - Practical-6Hrs - Total- 08Hrs | - Links - Joints - End effectors - Nuts - Bolts - Drills - Grippers - Conveyor belt - Sensor and actuators - Hydraulic Actuators. - Electrical Actuators. - Pneumatic Actuators. - Mechanical Actuators. - Locomotive Actuators. - Manipulation Actuators. | Classroom or Demonstration room Workshop |

## 0714E&A86: @Robotics Control Boards

Objective: This module covers the skills and knowledge required to install, run, interface and application of various types of embedded control board.

Duration: 32 Hours						Theory: 08 Hours					Practical: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform installation of operating system. | - Insert memory card in pc. - Download and mount Raspbian operating system on SD card. - Insert SD card in raspberry pi and turn it on. - Connect HDMI to LCD. - Run installation on Raspberry pi. - After completing installation restart raspberry pi | - Describe small size Embedded computer system. - Describe Raspbian operating system. - Define installation of operating system. - PRACTICAL ACTIVITY : - Download and install Raspbian Operating system and ready a raspberry pi board. | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - Raspberry pi - Jumpers - SD card - LED - Actuators - Power Adaptor - HDMI cable | Classroom or Demonstration room Workshop |
| - Run Python on Raspberry pi | - Download and install Python on raspberry pi. - Open python shell compiler on Raspberry pi. - Design code on Python shell. - Compile code and see output. | - Describe Python programming language - Explain python shell compiler - Define syntax of commands use in Python - PRACTICAL ACTIVITY : - Download and install Python in Raspbian operating system and design program for calculator | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - Raspberry pi - Jumpers - SD card - LED - Actuators - Power Adaptor - HDMI cable | Classroom or Demonstration room Workshop |
| - Control GPIO. | - Open python shell program on Raspberry pi operating system. - Design program on python to control GPIO. - Run compiler and operate actuator. | - Describe GPIO - Define python code to control GPIO - PRACTICAL ACTIVITY : - Design a program to control GPIO of Raspberry pi. | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - Raspberry pi - Jumpers - SD card - LED - Actuators - Power Adaptor - HDMI cable | Classroom or Demonstration room Workshop |
| - Interface sensors and actuators | - Open python shell program on Raspberry pi operating system. - Connect jumpers with GPIO pins and sensor/actuators. - Design program on python to control GPIO. - Run compiler and operate actuator. | - Describe sensor and actuators which can be interface with raspberry pi - PRACTICAL ACTIVITY : - Connect temperature sensor with GPIO and take readings using python code. | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - Raspberry pi - Jumpers - SD card - LED - Actuators - Power Adaptor - HDMI cable - Humidity sensor | Classroom or Demonstration room Workshop |

## 0714E&A87: @Develop Robot program.

Objective: This module covers the skills and knowledge required to Identify fundamental of Programming and interfacing of sensor and actuators as they are essential in robot designing.

Programming involved interfacing SONAR, temperature sensors and generation of PWM signal that is used to control the speed of DC motor.

Duration: 24 Hours						Theory: 06 Hours					Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop program to interface temperature sensor LM35 with Arduino | - Identify the pin configuration of Arduino and LM 35 temperature sensor - Make connection of power, ground and analog pin of LM 35 with Arduino - Connect Arduino with computer (PC) and open Arduino IDE and select port and Devices - Write and upload the code of LM35 into Arduino - Observe the value of temperature in serial monitor Screen in PC - Measure the temperature of ice and calibrate reading by tuning in programming. | - Define Arduino connect to temperature sensor. - Explain lm35 temperature sensor. - Describe analog sensor connect to Arduino. - Define what sensors are available for Arduino. - PRACTICAL ACTIVITY: Connect the temperature sensors with Arduino and program to observe its reading in serial monitor. | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - Robotic fastening tool kit - CAD software - Measuring tool kit - Complete design of the Robot | Class room/ Demonstration room/ workshop/ Computer lab |
| - Develop program to operate DC motor with PWM based speed control | - Identify the components and connection of circuits. - Connections the potentiometer to analog pins and connects the MOSFET to digital Pin of Arduino. - Connect the DC motor with separate Power Supply unit with MOSFET as switch. - Write code to take value of voltage and make switch case statements to generate PWM signal of multiple Duty cycle. - Upload the code and run DC motor. | - Define PWM control motor speed. - Explain control the speed of a DC motor. - Describe Arduino control DC motor speed. - Define connection of DC motor with separate power supply. - PRACTICAL ACTIVITY: - Connect the potentiometer to analog pins and connects the MOSFET to digital Pin of Arduino | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - Robotic fastening tool kit - CAD software - Complete design of the Robot - Measuring tool kit | Class room/ Demonstration room/ workshop/ Computer lab |
| - Develop program to operate SONAR and measure distance with Arduino | - Identify the Pin configuration of Sonar and Arduino. - Make connection according to circuit diagram. - Connection Arduino with PC and open Arduino IDE. - Write and upload the code in Arduino. - Run the code and measure value at serial monitor. - Verify measured value of distance with Scale. | - Define measure distance with a sonar sensor with an Arduino. - Define different Pin configuration of sonar and Arduino. - Explain ultrasonic sensor work with Arduino. - Describe the principle of SONAR sensor. - Explain the control data structure in different software. - PRACTICAL ACTIVITY: - Connect the SONAR sensors with Arduino and program to observe its reading in serial monitor | - Theory-02Hrs - Practical-6Hrs Total- 08Hrs | - Robotic fastening tool kit - CAD software - Complete design of the Robot - Measuring tool kit | Class room/ Demonstration room/ workshop/ Computer lab |

## 0714E&A88: @ Develop robotic arm structure.

Objective: This module covers the skills and knowledge related to fundamental of robots arm and assembling of robot the various End effector used to perform different kinds of mechanical jobs, and the development of robot structure.

Duration: 16 Hours						Theory: 04 Hours					Practical: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design the Robotic Arm | - Specify the types of robotic arms. - Select the links and joints type for robotic arm. - Plan the axes of motion (rotation and straight-line movement) of robotic arm. - Specify the degrees of freedom for end effector. - Specify the reach envelope for robotic arm. - Identify the speed and payload for robotic arm. - Design the model of links, joints and overall assembly of the manipulator. | - Define the usage of different types of robotic arm. - Explain the operating principles of a robotic arm. - Define the development of computer aided design (CAD) model. - Define the kinematic structure ad workspace of robotic arm - PRACTICAL ACTIVITY: - Design the CAD model of links, joints and overall assembly of the manipulator | Theory – 02 Hrs Practical – 06 Hrs Total – 08 Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit. - Complete design of the sensor assembly | Class room/ Demonstration room/ workshop/ Computer lab |
| - Develop the structure of Robotic Arm | - Assemble manipulators. - Assemble End Effectors with the manipulators. - Assemble actuators with manipulators and end effectors. - Assemble Controller with actuators and power supply statements to generate PWM signal of multiple Duty cycle. | - Define the movement and design of a manipulator. - Explain the type of manipulators for the Robotic Arm. - Describe to identify the drive for the End Effector. - Define development of computer aided design (CAD) model. - Describe the usage of robotic fastening tool kit. - Explain the movement of an end effector. - Explain the movement of manipulator. - PRACTICAL ACTIVITY: - Assemble actuators with manipulators and end effectors | Theory – 02 Hrs Practical – 06 Hrs Total – 08 Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit - Complete design of the sensor assembly | Class room/ Demonstration room/ workshop/ Computer lab |

## 0714E&A89: @Configure and test robotic arm

Objective: This module covers the skills and knowledge related to configuration of Robotic Arm and selection and configuration controller for specific industrial process, it also includes testing of various parts of robot

Duration: 24 Hours						Theory: 06 Hours					Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Select parts of Robotic Arm | - Select Controller. - Select Manipulator. - Select Effector. - Select Actuators. - Select Software. - Select power supply. - Select connecting wires. - Identify the components (Resistor, Capacitor and Potentiometer). - Identify electric wiring of actuators, controllers and power supply. | - Define types of controller. - Explain types of End effector. - Describe power supply and required power according to application. - PRACTICAL ACTIVITY: Select electric wiring of actuators, controllers and power supply. | Theory – 02 Hrs Practical – 06 Hrs Total – 08 Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit. - Power supply wires - Robot operating software | Class room/ Demonstration room/ workshop/ Computer lab |
| - Program the controller | - Assemble the system according to circuit diagram - Open IDE for controller. - Develop the algorithm flowchart to implement on controller - Upload the prepared code. - Upload the code and run DC motor. | - Define the problem-solving techniques and error identification. - Explain the algorithm flowchart - Explain the circuit diagram - PRACTICAL ACTIVITY: - Develop the algorithm flowchart to implement on controller | Theory – 02 Hrs Practical – 06 Hrs Total – 08 Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit. - Power supply & wires - Robot operating software | Class room/ Demonstration room/ workshop/ Computer lab |
| - Test the Robotic Arm | - Identify movement of robotic arm. - Perform the arm movement detection test - Check the robotic arm System feasibility and manipulability for end effectors. - Check of Robotic arm operating conditions. - Check all the connections of the components. - Recognize the program algorithm for the robotic arm | - Demonstrate the arm movement detection test - Demonstrate how to check the robotic arm System feasibility and manipulability for end effectors. - Demonstrate how to check of Robotic arm operating conditions. - PRACTICAL ACTIVITY: - Check the robotic arm System feasibility and manipulability for end effectors. | Theory – 02 Hrs Practical – 06 Hrs Total – 08 Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit. - Power supply wires - Robot operating software | Class room/ Demonstration room/ workshop/ Computer lab |

## 0714E&A90: @Develop Object Recognition System

Objective: This module covers the skills and knowledge related to introduce the student to the machine vision system’s design and development for object recognition. Explore the extraction of object from video feed using artificial intelligence algorithm to command the actuator according to its application

Duration: 25 Hours						Theory: 10 Hours					Practical: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop the object recognition system | - Specify the types of object recognition system. - Identify camera for real time video feed. - Identify the actuators for object recognition system. - Select the components types. - Plan the maneuverability of end effectors | - Define the usage of different types of object recognition system. - Define the operating principles of an object recognition system. - Define the types of Object Recognition System - Define the type of modules of camera for different purposes? - Describe the different types actuators for object recognition system. - Explain the components use in object recognition system. - Explain the maneuverability of end effectors - Explain the design of an object recognition system - Describe the workspace of object recognition system - Define the operating principles of the controllers. - Define the connections and configuration of controller - Define the programming principle languages. - Define the various programming techniques associated with industrial Remotely Operated Vehicle - Explain the control data structure in different software. - Define the control data acquisition principles. - Explain the control data transmission medium. - Describe the packages, libraries and data handling in software. - Define the algorithm for the specified movement - Define the performance measurement criteria. - Explain to check the proper data feed. PRACTICAL ACTIVITY: Select the appropriate controller and Install its required software in computer | Theory – 02 Hrs Practical – 03 Hrs Total – 05 Hrs | - Controller - Computer - Software - Connector - Fastening tool kit - Motors - battery | Class room/ Demonstration room/ workshop/ Computer lab |
| - Implement Vision Algorithm | - Get the Video sequence - Identify the background subtraction from video by programming algorithm - Identify the Noise filtration from video by programming algorithm. - Calculate the field of view of vision system. - Identify the frame grabber for vision system. - Detect the object - Techniques used for 3D vision - Object location and orientation measurement results store it in memory - Data transmission to controller - Identify actuator to perform task according to object detection. | - Define connect camera to the controller - Describe to get the video from the camera and send it to monitor. - Explain the artificial intelligence algorithm for object recognition. - Describe the different object recognition approaches in Deep learning. - Explain the classifier use for object recognition. - Define how to filter the noise from video - Describe the principles for classification of object recognition - Describe how to calculate the field of view of vision system. - Describe the different type of frame grabber for vision system - Explain the different Techniques used for 3D vision - Determine how to perform Object location - Determine how to perform orientation measurement - Explain how to transmit the Data to controller PRACTICAL ACTIVITY: Program the vision algorithm on specified controller by getting video streams from the camera. | Theory – 02 Hrs Practical – 03 Hrs Total – 05 Hrs | - Controller - Computer - Software - Connector - Fastening tool kit - Motors - battery | Class room/ Demonstration room/ workshop/ Computer lab |
| - Configure the object recognition system | - Select the Controller. - Identify the components (Resistor, Capacitor and Potentiometer) - Draw the circuit diagram - Electric wiring of actuators and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply for controllers | - Describe the types Controller. - Explain the use of fundamental electronics components (resistors, capacitors, inductors, diodes, and transistors) in robotics system. - Explain the basic techniques use to wire the actuators - Describe how to Design the algorithm flowchart - Determine the types of Power supply - Describe how to calculate the power distribution and requirement of Voltage and current for the system. - Describe the essential factors for battery selection. - Describe the power supply for controllers. PRACTICAL ACTIVITY: Interface the camera and controller and properly wired together according to pin configuration. | Theory – 02 Hrs Practical – 03 Hrs Total – 05 Hrs | - Controller - Computer - Software - Camera - Electronics components (Resistor, Capacitor and Potentiometer) - Battery - Actuator (Robotics Arm) | Class room/ Demonstration room/ workshop/ Computer lab |
| - Program the object recognition system | - Program the controller to lift the object from one point and move it to another point. - Program the controller to read the values from potentiometer. - Program the controller to map the values to actuator to move manipulator and end effector. - Program the movements and positions of actuators. - Program the angle of end effector to determine grabbing of object. | - Define the basic concepts of Programing - Explain how to program on specific controller - Define how to program object recognition algorithm. - Describe how to program the controller to lift the object from one point and move it to another point. - Explain how controller reads the reading from sensors. - Describe how to program the controller will read the values from potentiometer. - Describe how to program the controller to map the values to actuator to move manipulator and end effector. - Determine how to program the movement of actuators and positions. - Determine how to determine the angle of end effector to determine grabbing of object PRACTICAL ACTIVITY: Program the controller to pick the object from one position to another. | Theory – 02 Hrs Practical – 06 Hrs Total – 08 Hrs | - Controller - Computer - Software - Camera - Actuator (Robotic Arm) - Electronics components (Resistor, Capacitor and Potentiometer) - Battery | Class room/ Demonstration room/ workshop/ Computer lab |
| - Test and evaluate the object recognition system. | - Identity and check the proper data feed. - Recognize the data points for image recognition system. - Verify the pattern for image recognition. - Check the performance accuracy of trained module. - Verify the reliability, maintenance and safety measures for object recognition system. | - Describe the types of data coming and check the proper data feed. - Define the data points for image recognition system? - Demonstrate how to verify the pattern for image recognition. - Determine how to check performance accuracy of trained module. - Demonstrate the reliability, maintenance and safety measures for object recognition system. PRACTICAL ACTIVITY: Verify the reliability, maintenance and safety measures for object recognition system. | Theory – 02 Hrs Practical – 06 Hrs Total – 08 Hrs | - Controller - Computer - Software - Camera - Actuator (Robotic Arm) - Electronics components (Resistor, Capacitor and Potentiometer) - Battery - Paper - Pencil - Calculator - Multimeter | Class room/ Demonstration room/ workshop/ Computer lab |

## 0714E&A91: @Perform Assembly and Operation of Remotely Operated Robot

Objective: This module covers the skills and knowledge related to introduce the student to the one most use category of robot that remotely operated vehicle’s design and development for specific purposes. This module covers operating principles of remotely operated vehicles and describes types of remotely operated vehicles found in industry. Explore the main consideration that’s is mobile base, actuator, sensors, transmission medium for data, and assembling it with links and joints for remotely operated vehicle. Also introduce to the applications of remotely operated vehicle in different sectors

Duration: 24 Hours						Theory: 6 Hours					Practical: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design the Remotely Operated Robot | - Specify the types of remotely operated mobile robot. - Identify mobile base of the ROR. - Identify the actuators for ROR. - Select the components type for ROR. - Plan the maneuverability (mobility and severability) of ROR. - Determine the sensor for positioning. - Identify the sensors to avoid obstacles. - Design the model and overall assembly of the ROV. | - What are the types of remotely operated mobile robot? - What are the uses of remotely operated mobile robot? - Describe the different types of mobile base of the ROR. - Describe the different types of actuators for ROR. - Describe the different types of the components type for ROR. - Describe the different types of Plan the maneuverability (mobility and severability) of ROR. - Describe the different types of Determine the sensor for positioning. - Describe the different types of Identify the sensors to avoid obstacles. - Describe the different types of Design the model and overall assembly of the ROV - Understand the operating principles of a ROV. - Understand the development of computer aided design (CAD) model. - Understand the kinematic structure ad workspace of ROV. - Understand the movement and design of a ROV. - Understand the type of manipulators for the ROV. PRACTICAL ACTIVITY: Select the appropriate sensors and Controller for Remotely Operated Vehicle | Theory – 01 Hrs Practical – 03 Hrs Total – 4 Hrs | - Computer system - Complete design of the sensor assembly - Basic ROV components - Software - Sensors - Actuators | Class room/ Demonstration room/ workshop/ Computer lab |
| - Develop the structure of Remotely Operated Vehicle | - Identify the design of the ROV. - Assembly of the parts of ROV. - Assembly of the base with actuator. - Mount and assemble actuators with base and sensors. - Identify the fastener and accessories of ROV. | - Understand the usage of robotic fastening tool kit. - Understand the movement of an end effector. - Understand the operation of each sensor. - Understand the use of each sensor for industrial operations of ROV. - Understand the design of the ROV. - Determine how to Assemble of the parts of ROV. - Determine how to Assemble of the base with actuator. - Determine how to mount and assemble actuators with base and sensors. - Determine how to fastener and accessories of ROV. - PRACTICAL ACTIVITY: - Select the appropriate sensors and actuator. | Theory – 01 Hrs Practical – 03Hrs - Total – 4 Hrs | - Computer system - Complete design of the sensor assembly - Basic ROV components - Software - Sensors - Actuators | Class room/ Demonstration room/ workshop/ Computer lab |
| - Installation of Sensors | - Installation of positioning sensor - Installation of robot vision (camera). - Installation of distance sensors. - Setting the transmitter/receiver - Setting the monitor. - Shape and design of the sensors. - Types and application and sensors. | - Determine how to fastener and accessories of ROV. - Determine different type of positioning sensor and its installation techniques - Determine different type of robot vision (camera) and its Installation techniques. - Determine different type of distance sensors and its Installation techniques. - Determine how to set the transmitter/receiver - Determine the Setting for the monitor. - Shape and design of the sensors. - Determine the Types and application and sensors. - PRACTICAL ACTIVITY: - Mount/Assemble actuators with base and sensors. | Theory – 01Hrs Practical – 03 Hrs - Total – 4Hrs | - Computer system - Complete design of the sensor assembly - Basic ROV components - Software - Sensors - Actuators - Measuring tool kit - Robotic fastening tool kit. - Mounting accessories | Class room/ Demonstration room/ workshop/ Computer lab |
| - Configure the ROR | - Identify and Select the Controller. - Identify the components (Resistor, Capacitor, and Motor Controller). - Data acquisition from sensor and display on monitor. - Draw the circuit diagram. - Electric wiring of sensors, camera, receiver, actuators and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply for controllers. | - Types of sensors and its pin configuration. - Electronics components (Resistor, Capacitor, and Motor Controller). - How to perform Data acquisition from sensor and display on monitor. - Understand the electronics wiring Electric wiring of sensors, camera, receiver, actuators and controller. - Understand the algorithm flowchart to implement on controller. - Understand the power distribution and the power supply for controllers and sensors - PRACTICAL ACTIVITY: - Draw the Electronics Circuit diagram of ROV | Theory – 01Hrs Practical – 03 Hrs - Total – 4Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit. - Trainer - Manipulator links and joints - Computer system - Mounting accessories - Complete design of the sensor assembly - Basic ROV components - Complete design of the ROV - Software - Sensors - Trainer - Wiring and accessories | Class room/ Demonstration room/ workshop/ Computer lab |
| - Program the ROR | - Program the controller to change direction of robot according to the receive signal. - Program the controller to get the live camera display on monitor. - Get the values from positioning sensor to map the location and send it to monitor. - Program the distance measuring sensor for collision avoidance and distance measurement from object. - Program to display the all sensors values on monitor. | - Understand the software for specific controller - Understand the programming - Language for specific controller - Understand the Algorithm for ROR - Understand the data the controller to change direction of robot according to the receive signal. - Understand how to program the controller to get the live camera display on monitor. - Understand how to Get the values from positioning sensor to map the location and send it to monitor. - Understand how the distance measuring sensor for collision avoidance and distance measurement from object. - Understand how to Program the sensors from controller to display the all sensors values on monitor. - PRACTICAL ACTIVITY: - Program the controller to display the sensors output on the monitor. | Theory – 01Hrs Practical – 03 Hrs - Total – 4 Hrs | - Computer system - Software - Sensors | Class room/ Demonstration room/ workshop/ Computer lab |
| - Test and Evaluate the ROR | - Identification of movement of the ROR. - Preform the robot movement detection test - Check the ROR System feasibility and manipulability. - Check of ROR operating conditions - Check the condition of each component and overall assembly of the ROR. - Check all the connections of the components. - Recognize the program algorithm for the ROV | - Identification of movement of the ROR. - Understand how to perform the robot movement detection test - Understand steps required to check Industrial ROR System feasibility and manipulability. - Industrial ROR operating conditions - Understand the condition of each component and overall assembly of the ROR. - Understand all the connections of the components. PRACTICAL ACTIVITY: Perform the steps required to check ROR System feasibility and manipulability. | Theory – 01Hrs Practical – 03 Hrs Total – 4 Hrs | - Manipulator links and joints - Measuring tool kit - Robotic fastening tool kit. - Trainer - Manipulator links and joints - Computer system - Mounting accessories - Complete design of the sensor assembly - Basic ROV components - Complete design of the ROV - Software - Sensors - Trainer - Wiring and accessories | Class room/ Demonstration room/ workshop/ Computer lab |

## Module: Develop workplace policy and procedures for sustainability.

Objective: After the completion of this competency standard, the Trainee will be able to develop and implement a workplace
sustainability policy and to modify the policy to suit changed circumstances. It applies to individuals with managerial responsibilities who undertake work developing approaches to create, monitor and improve strategies and policies within workplaces and engage with a range of relevant stakeholders and specialists.

Duration:  76 Hours			Theory:  10 Hours			Practice:  66 Hours          	        Credit Hours:  0.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop workplace sustainability policy | Trainee will be able to: - Define scope of sustainability policy - Gather information from a range of sources to plan and develop policy - Identify and consult stakeholders as a key component of the policy development process - Include appropriate strategies in policy at all stages of work for minimizing resource use, reducing toxic material and hazardous chemical use and employing life cycle management approaches - Make recommendations for policy options based on likely effectiveness, timeframes and cost - Develop policy that reflects the organization s commitment to sustainability as an integral part of business planning and as a business opportunity - Agree to appropriate methods of implementation, outcomes and performance indicators | - Outline the environmental or sustainability legislation, regulations and codes of practice applicable to the organization identify internal and external sources of information and - Explanation of plan and development of the organization sustainability policy | Theory- 2 Hrs Practical-15 Hrs Total- 17 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Communicate workplace sustainability policy | Trainee will be able to: - Promote workplace sustainability policy, including its expected outcome, to key stakeholders. - Inform those involved in implementing the policy about expected outcomes, activities to be undertaken and assigned responsibilities. | - Outline organizational systems and procedures that relate to sustainability | Theory- 2 Hrs Practical-15 Hrs Total- 17 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Implement workplace sustainability policy | Trainee will be able to: - Develop and communicate procedures to help implement workplace sustainability policy - Implement strategies for continuous improvement in resource efficiency - Establish and assign responsibility for recording systems to track continuous improvements in sustainability approaches | - Outline typical barriers to implementing policies and procedures in an organization and possible strategies to address them. | Theory- 2 Hrs Practical-15 Hrs Total- 17 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU4. Review workplace sustainability policy implementation | Trainee will be able to: - Review workplace sustainability policy implementation. - Investigate successes or otherwise of policy. - Monitor records to identify trends that may require remedial action and use to promote continuous improvement of performance. - Modify policy and or procedures as required to ensure improvements are made | - Explain workplace policy development processes and practices | Theory- 4 Hrs Practical-21 Hrs Total- 25 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Manage meetings with customers

Objective: After the completion of this competency standard, the Trainee will be able to manage a range of meetings including
overseeing the meeting preparation processes, chairing meetings, organizing the minutes and reporting meeting outcomes. It applies to individuals employed in a range of work environments who are required to organize and manage meetings within their workplace, including conducting or managing administrative tasks in providing agendas and meeting material. They may work as senior administrative staff or may be individuals with responsibility for conducting and chairing meetings in the workplace

Duration:  76 Hours			Theory:  10 Hours			Practice:  66 Hours          	        Credit Hours:  0.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Plan and Prepare for meetings | Trainee will be able to: - Develop agenda in line with stated meeting purpose - Ensure style and structure of meeting are appropriate to its purpose - Identify meeting participants and notify them in accordance with organizational procedures - Confirm meeting arrangements in accordance with requirements of meeting - Dispatch meeting papers to participants within designated timelines | - Outline responsibilities of the chairperson and explain group dynamics in relation to managing meetings | Theory- 03 Hrs Practical- 22Hrs Total- 25 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Conduct meetings | Trainee will be able to: - Chair meetings in accordance with organizational requirements, agreed conventions for type of meeting and legal and ethical requirements - Conduct meetings to ensure they are focused, time efficient and achieve the required outcomes - Ensure meeting facilitation enables participation, discussion, problem solving and resolution of issues - Brief minute-taker on method for recording meeting notes in accordance with organizational requirements and conventions for type of meeting | - Describe options for meetings including face-to-face, tele-conferencing, web-conferencing and using webcams | Theory- 03 Hrs Practical- 22Hrs Total- 25 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Follow up meeting’s agenda for implementation | Trainee will be able to: - Check transcribed meeting notes to ensure they reflect a true and accurate record of the meeting and are formatted in accordance with organizational procedures and meeting conventions - Distribute and store minutes and other follow-up documentation within designated timelines, and according to organizational requirements - Report outcomes of meetings as required, within designated timelines | - Devise meeting terminology, structures, arrangements as per organizational procedures | Theory- 4Hrs Practical-22 Hrs Total- 26 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Manage and select your team (workforce) according job requirement

Objective: After the completion of this competency standard, the Trainee will be able to manage all aspects of recruitment
selection and induction processes in accordance with organizational policies and procedures. It applies to individuals or human resource personnel who take responsibility for managing aspects of selecting new staff and orientating those staff in their new positions. It is not assumed that the individual will be directly involved in the selection processes themselves, although this may well be the case.

Duration:  76 Hours			Theory:  10 Hours			Practice:  66 Hours          	        Credit Hours:  7.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop team selection policies and procedures | Trainee will be able to: - Analyze strategic and operational plans and policies to identify relevant policies and objectives - Develop recruitment, selection and induction policies and procedures and supporting documents - Review options for technology to improve efficiency and effectiveness of recruitment and selection process - Obtain support for policies and procedures from senior managers - Trial forms and documents supporting policies and procedures and make necessary adjustments - Communicate policies and procedures to relevant staff and provide training if required | - Describe recruitment and selection methods, including assessment centers - Describe the purpose of employee contracts and industrial relations - Summarize relevant legislation, regulations, standards and codes of practice that may affect recruitment, selection and induction | Theory- 03 Hrs Practical- 22 Hrs Total- 25 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Select team according to job | Trainee will be able to: - Determine future human resource needs in collaboration with relevant managers and sections - Ensure current position descriptors and person specifications for vacancies are used by managers and others involved in recruitment, selection and induction processes - Provide access to training and other forms of support to all persons involved in recruitment and selection process - Ensure advertising of vacant positions complies with organizational policy and legal requirements - Utilize specialists where necessary - Ensure selection procedures are in accordance with organizational policy and legal requirements - Ensure processes for advising applicants of selection outcome are followed - Ensure job offers and contracts of employment are executed promptly, and new appointments are provided with advice about salary, terms and conditions | - Explain the concept of outsourcing - Explain why terms and conditions of employment are an important aspect of recruitment | Theory- 03Hrs Practical-22 Hrs Total- 25 Hr | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Manage staff induction | Trainee will be able to: - Provide access to training and ongoing support for all persons engaged in staff induction - Check induction processes are followed across the organization - Oversee management of probationary employees and provide them with feedback until their employment is confirmed or terminated - Obtain feedback from participants and relevant managers on extent induction process is meeting its objectives - Make refinements to induction policies and procedures | Explain the relevance of psychometric and skills testing programs to recruitment. | Theory- 4 Hrs Practical-22 Hrs Total- 26 Hr | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Manage workforce planning

Objective: After the completion of this competency standard, the Trainee will be able to manage planning in relation to an
organization’s workforce including researching requirements, developing objectives and strategies, implementing initiatives and monitoring and evaluating trends. It applies to individuals who are human resource managers or staff members with a role in a policy or planning unit that focuses on workforce planning.

Duration:  76 Hours			Theory:  10 Hours			Practice:  66 Hours          	        Credit Hours:  7.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Research workforce requirements | Trainee will be able to: - Review current data on staff turnover and demographics - Assess factors that may affect workforce supply - Establish the organization’s requirements for a skilled and diverse workforce | Outline industrial relations relevant to the specific industry | Theory- 03 Hrs Practical- 15Hrs Total- 18 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Develop workforce objectives and strategies | Trainee will be able to: - Review organizational strategy and establish aligned objectives for modification or retention of the workforce - Consider strategies to address unacceptable staff turnover, if required - Define objectives to retain required skilled labor - Define objectives for workforce diversity and cross-cultural management - Define strategies to source skilled labor - Communicate objectives and rationale to relevant stakeholders - Obtain agreement and endorsement for objectives and establish targets - Develop contingency plans to cope with extreme situations | Explain current information about external labor supply relevant to the specific industry or skill requirements of the organization | Theory- 03 Hrs Practical- 15Hrs Total- 18 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Implement initiatives to support workforce planning objectives | Trainee will be able to: - Implement action to support agreed objectives for recruitment, training, redeployment and redundancy - Develop and implement strategies to assist workforce to deal with organizational change - Develop and implement strategies to assist in meeting the organization’s workforce diversity goals - Implement succession planning system to ensure desirable workers are developed and retained - Implement programs to ensure workplace is an employer of choice | - Strategies to assist workforce to deal with organizational change & work environment | Theory- 03 Hrs Practical- 15Hrs Total- 18 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU4. Monitor and evaluate workforce trends | Trainee will be able to: - Review workforce plan against patterns in exiting employee and workforce changes - Monitor labor supply trends for areas of over- or under-supply in the external environment - Monitor effects of labor trends on demand for labor - Survey organizational climate to gauge worker satisfaction - Refine objectives and strategies in response to internal and external changes and make recommendations in response to global trends and incidents - Regularly review government policy on labor demand and supply - Evaluate effectiveness of change processes against agreed objectives | Describe labor force analysis and forecasting techniques | Theory- 01Hrs Practical- 21Hrs Total- 22 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Undertake project work

Objective: After the completion of this competency standard, the Trainee will be able to undertake a straightforward project or
a section of a larger project. It covers developing a project plan, administering and monitoring the project, finalizing the project and reviewing the project to identify lessons learned for application to future projects. This unit applies to individuals who play a significant role in ensuring a project meets timelines, quality standards, budgetary limits and other requirements set for the project.

Duration:  76 Hours			Theory:  10 Hours			Practice:  66 Hours          	        Credit Hours:  7.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Define project | Trainee will be able to: - Access project scope and other relevant documentation - Define project stakeholders - Seek clarification from delegating authority of issues related to project and project parameters - Identify limits of own responsibility and reporting requirements - Clarify relationship of project to other projects and to the organization’s objectives - Determine and access available resources to undertake project | Give examples of project management tools and how they contribute to a project Explain the organization’s procedures and processes that are relevant to managing a project including: lines of authority and approvals quality assurance human resources budgets and finance recordkeeping reporting | Theory- 03 Hrs Practical- 15Hrs Total- 18 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Develop project plan | Trainee will be able to: - Develop project plan in line with the project parameters - Identify and access appropriate project management tools - Formulate risk management plan for project, including Work Health and Safety (WHS) - Develop and approve project budget - Consult team members and take their views into account in planning the project - Finalize project plan and gain necessary approvals to commence project according to documented plan | - Outline types of documents and other sources of information commonly used in defining the parameters of a project | Theory- 03 Hrs Practical- 15Hrs Total- 18 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Administer and monitor project | Trainee will be able to: - Take action to ensure project team members are clear about their responsibilities and the project requirements - Provide support for project team members, especially with regard to specific needs, to ensure that the quality of the expected outcomes of the project and document time lines are met - Establish and maintain required recordkeeping systems throughout the project - Implement and monitor plans for managing project finances, resources and quality - Complete and forward project reports as required to stakeholders - Undertake risk management as required to ensure project outcomes are met - Achieve project deliverables | - Outline the organization’s mission, goals, objectives and operations and how the project relates to them | Theory- 03 Hrs Practical- 15Hrs Total- 18 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU4. Finalize project | Trainee will be able to: - Complete financial recordkeeping associated with project and check for accuracy - Ensure transition of staff involved in project to new roles or reassignment to previous roles - Complete project documentation and obtain necessary sign-offs for concluding project | - Explain processes for identifying and managing risk in a project | Theory- 01Hrs Practical- 21Hrs Total- 22 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Prepare and implement negotiation

Objective: After the completion of this competency standard, the Trainee will be able to prepare for and participate in a process of negotiation, Coordinate support services, Restore order, Provide leadership direction and guidance to the work group.

Duration:  76 Hours			Theory:  10 Hours			Practice:  66 Hours          	        Credit Hours:  7.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Prepare for the negotiation | Trainee will be able to: - Identify objectives and preferred outcome of the negotiation and determine minimum acceptable outcome - Understand in relation to what can be offered and what is needed from the other party - Gather information regarding the other party – objectives, needs, preferences, resources, what they want to achieve – in order to determine best negotiating points - List and rank the issues to consider concessions that may be made. - Find examples and refine negotiation argument - Check information to ensure it is correct and up-to-date. - Develop a negotiation plan that includes information about the other party and its interests and a set of responses and strategies to the anticipated tactics. - Prepare an agenda in advance, which includes discussion topics, participants, location and schedule | - Explain organization’s policies, guidelines and procedures related to control and surveillance, safety and preventing and responding to incidents and breaches of orders covered in the range of variables. | Theory- 02 Hrs Practical- 12Hrs Total- 14 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Participate in negotiations | Trainee will be able to: - Analyze all aspects of the incident for degree of hazard, priorities, optional outcomes and appropriate strategies - Analyze and determine strategies and priorities on the incident sought from a range of sources - Assess long term objectives against resources and priorities - Apply a range of communication techniques to make and maintain contact with the key people - Provide clear and factual information to enable an honest and realistic assessment of the interests of the key people and their positions - Resolve the conflict and express their likely consequences clearly and do an analysis of the benefits - Reassess points of disagreements for common positive positions | - Outline the principles of effective communication | - 02 Hrs Practical- 12Hrs Total- 14 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Coordinate support services | Trainee will be able to: - Assess the need for support services in terms of the determined strategies and priorities - Negotiate the resources of support services according to established procedures and availability - Provide information on strategies to support services and maintain the communication - Delegate roles and responsibilities according to expertise and resources | - Explain organization’s management and accountability systems | - 02 Hrs Practical- 12Hrs Total- 14 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU4. Restore order | Trainee will be able to: - Assess the incidents for degree of risk and take appropriate action to reduce and remove the impact of the incident and restore order - Take action designed to minimize risk and the preserve the safety and security of all involved - Take action to prevent the escalation of the incident appropriate to the circumstances and agreed procedures. - Carry out the use of force for the restoration of control and the maintenance of security in the least restrictive manner. - Complete reports accurately and clearly provided to the appropriate authority promptly - Review, evaluate and analyze the incident and the organizational response to it and report it promptly and accurately. | - Describe teamwork principles and strategies | Th- 02 Hrs Practical- 12Hrs Total- 14 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU5. Provide leadership direction and guidance to the work group | Trainee will be able to: - Link between the function of the group and the goals of the organization - Participate in decision making routinely to develop, implement and review work of the group and to allocate responsibilities where appropriate - Give opportunities and encouragement to others to develop new and innovative work practices and strategies - Identify conflict and resolve with minimum disruption to work group function - Provide staff with the support and supervision necessary to perform work safely and without risk to health - Allocate tasks within the competence of staff and support with appropriate authority, autonomy and training - Supervise appropriately the changing priorities and situations and takes into account the different needs of individuals and the requirements of the task | - Outline the guidelines for use of equipment and technology - Explain code of conduct | Th- 02 Hrs Practical- 18Hrs Total- 20 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

## Module: Maintain professionalism in the workplace

Objective: After the completion of this competency standard, the Trainee will be able to maintain a professional image in the
workplace, including behaving ethically, demonstrating motivation, respecting timeframes and maintaining personal appearance.

Duration:  76 Hours			Theory:  10 Hours			Practice:  66 Hours          	        Credit Hours:  7.6

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Respect work timeframes | Trainee will be able to: - Demonstrate punctuality in meeting, set working hours and times. - Utilize working hours only for working and follow company regulations. - Complete work tasks within deadlines according to order of priority - Supervisors are informed of any potential delays in work times or projects | - Explain application of good manners and right conduct - Outline the company code of conduct/values | Theory- 02 Hrs Practical- 15Hrs Total- 17 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU2. Maintain personal appearance and hygiene | Trainee will be able to: - Clean hair, body and nails regularly. - Wear suitable cloths for the workplace, and respect local and cultural contexts - Meet specific company dress code requirements | - Describe common products used for oral and personal hygiene | Theory- 02 Hrs Practical- 15Hrs Total- 17 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |
| LU3. Maintain adequate distance with colleagues and clients | Trainee will be able to: - Respect personal space of colleagues and clients with reference to local customs and cultural contexts. - Keep sufficient distance from others - Avoid cross transmission of infections (especially through respiration) | - State workplace hygiene standards - Explain basic practices for oral and personal hygiene | Theory- 02 Hrs Practical- 15Hrs Total- 17 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Mulmediaprojector | Class Room |
| LU4. Work in an ethical manner | Trainee will be able to: - Follow company values/ethics codes of ethics and/or conduct, policies and guidelines. - Use company resources in accordance with company ethical standards. - Conduct personal behavior and relationships in accord with ethical standards and company policies. - Undertake work practices in compliance with company ethical standards, organizational policy and guidelines. - Instruct co-workers on ethical, lawful and reasonable directives. - Share company values /practices with co-workers using appropriate behavior and language. - Report work incidents /situations and/or resolved in accordance with company protocol /guidelines. | - Outline the Company regulations, performance and ethical standards - Explain work responsibilities/job functions - Describe communication skills | Theory- 04 Hrs Practical- 21Hrs Total- 25 Hrs | Internet MS Office Smartphone Laptop / Computer Smartphone Printer White board Projector screen Multimedia projector | Class Room |

# List of Tools and Equipments:

(FOR A CLASS OF 25 STUDENTS)

| Name of Trade |  | Robotics Technology |  |
| --- | --- | --- | --- |
| Sr. # |  | Description |  | Quantity |
| 1. |  | Steel-toed footwear, |  | 25 Nos. |
| 2. |  | hard hat, |  | 25 Nos. |
| 3. |  | safety gloves, |  | 25 Sets |
| 4. |  | appropriate safety glasses, |  | 25 Nos. |
| 5. |  | high visibility vest, |  | 25 Nos. |
| 6. |  | hearing protection, |  | 25 Nos. |
| 7. |  | respirators, masks, |  | 25 Nos. |
| 8. |  | De-electric boots and gloves for protection from electrical shock. |  | 25 Nos. |
| 9. |  | First Aid Box (Include all necessary bandages and medicines) |  | 02 Sets |
| 10. |  | fire extinguishers, |  | 04 Nos. |
| 11. |  | fire blankets, |  | 04 Nos. |
| 12. |  | Sand Bucket with stand |  | 02 Sets |
| 13 |  | basic tools, such as grease gun, air pump etc |  | 10 Sets |
| 14 |  | Allen Key |  | 10 Sets |
| 15 |  | Work Bench (able to hold 06 bench wises) |  | 05 Nos. |
| 16 |  | Bench Wise |  | 30 Nos. |
| 17. |  | hammer, |  | 05 each size |
| 18 |  | screwdrivers, |  | 05 each size |
| 19. |  | pliers, |  | 05 each size |
| 20. |  | self-locking pliers, |  | 05 each size |
| 21. |  | adjustable wrench, |  | 05 each size |
| 22. |  | assorted other wrenches, measuring tape(100m) |  | 05 each size |
| 23. |  | Basic supplies, such as grease, oil, window cleaner, rags, ice scraper, whisk broom. |  | 05 each |
| 24. |  | Color-code cards, utility documentation. Logbooks Service Manuals, OHS Regulation, |  | 10 sets |
| 25 |  | Fix Safe guards |  | 04 Nos |
| 26 |  | Moveable Safe guards |  | 04 N0s. |
| 27 |  | Solder |  | 04 N0s. |
| 28 |  | Wire stripper |  | 04 N0s. |
| 29 |  | Nose plier |  | 04 N0s. |
| 30 |  | Solder wire |  | 04 N0s. |
| 31 |  | Insulation remover |  | 04 N0s. |
| 32 |  | Flat File |  | 25 Nos. |
| 33 |  | Half round file |  | 10 Nos. |
| 34 |  | Round File |  | 10 Nos. |
| 35 |  | Mild Steel Sheet (4 x 8 ft) |  | 02 Nos |
| 36 |  | Mild Steel round work piece (Dia 30) |  | 10 ft |
| 37 |  | Mild Steel flat bar (W:50xL:100xT:20mm) |  | 25 Nos. |
| 38 |  | GI pipe 1”, 2” |  | 20 Feet each |
| 39 |  | Lathe machine (5 ft) |  | 02 Nos |
| 40 |  | Revolving center |  | 02 Nos |
| 41 |  | Dead center |  | 02 Nos |
| 42 |  | Draw bar |  | 02 Nos |
| 43 |  | Face plate |  | 02 Nos |
| 44 |  | Knurling Tool |  | 02 Nos |
| 45 |  | HSS Tool bit |  | 10 Nos |
| 46 |  | Pedestal Drill machine |  | 02 Nos |
| 47 |  | Hand Grinder |  | 02 Nos |
| 48 |  | Drill bits |  | 02 Set |
| 49 |  | Drill shank |  | 02 Set |
| 50 |  | Drill chuck |  | 02 Nos |
| 51 |  | Counter sink tool |  | 02 Nos |
| 52 |  | Counter bore tool |  | 02 Nos |
| 53 |  | Machine reamer set |  | 02 Set |
| 54 |  | Hand Reamer set |  | 02 Set |
| 55 |  | Universal Milling machine |  | 01 Nos. |
| 56 |  | Machine wise |  | 01 Nos. |
| 57 |  | Tail stock with dead center |  | 01 Set |
| 58 |  | Indexing head |  | 01 Nos. |
| 59 |  | Indexing plate set |  | 01 Set |
| 60 |  | Gear cutting tool set |  | 01 Set |
| 61 |  | T-slot cutting tool |  | 02 Nos. |
| 62 |  | Woodruff cutter |  | 02 Nos |
| 63 |  | End mill cutter |  | 02 Nos |
| 64 |  | Portable welding machine 3 Phase 300 Amps. |  | 02 Nos, |
| 65 |  | Welding table |  | 03 Nos, |
| 66 |  | Toggle |  | 02 Nos, |
| 67 |  | Leather gloves |  | 01 Nos, |
| 68 |  | Welding apron |  | 05 Nos, |
| 69 |  | Oxygen Cylinder with protector cap |  | 01 Nos, |
| 70 |  | Acetylene Cylinder with protector cap |  | 01 Nos. |
| 71 |  | Welding Torch |  | 01 Nos. |
| 72 |  | Welding Tip (Nozzle) |  | 01 Set |
| 73 |  | Pressure Regulators |  | 01 Nos. |
| 74 |  | Hose & Hose fittings |  | 30 ft. |
| 75 |  | Filler Rod |  | 01 Box |
| 76 |  | Flux Material |  | 500 gm. |
| 77 |  | Surface Table |  | 01 Nos. |
| 78 |  | - Surface plate. |  | 02 Nos. |
| 79 |  | - Steps and collars. |  | 02 Set |
| 80 |  | Folding rule |  | 02 Nos. |
| 81 |  | Combination set. |  | 02 Nos. |
| 82 |  | Scriber. |  | 02 Nos. |
| 83 |  | Tri square. |  | 02 Nos. |
| 84 |  | Divider. |  | 02 Nos. |
| 85 |  | Spirit level |  | 02 Nos. |
| 86 |  | Radius gauge |  | 02 Nos. |
| 87 |  | Filler gauge |  | 02 Nos. |
| 88 |  | Plug gauge |  | 02 Set |
| 89 |  | Ring gauge |  | 02 Set |
| 90 |  | Outside micrometer |  | 02 Nos. |
| 91 |  | Inside micrometer |  | 02 Nos. |
| 92 |  | Depth micrometer. |  | 02 Nos. |
| 93 |  | Bevel protractor. |  | 02 Nos. |
| 94 |  | Vernier Bevel protractor. |  | 02 Nos. |
| 95 |  | Dial protractor. |  | 02 Nos. |
| 96 |  | Sine bar. |  | 02 Nos. |
| 97 |  | Sine block |  | 02 Nos. |
| 98 |  | Sine plate. |  | 02 Nos. |
| 99 |  | Sine table. |  | 02 Nos. |
| 100 |  | Sine center. |  | 02 Nos. |
| 101 |  | Gear tooth caliper |  | 02 Nos. |
| 102 |  | Gear Inspection machine. |  | 01 Nos. |
| 103 |  | Gear testing machine. |  | 01 Nos. |
| 104 |  | Parkinson Gear Tester. |  | 01 Nos. |
| 105 |  | Multi meters |  | 02 Nos. |
| 106 |  | Resistors of different values |  | 04 Nos. (Each) |
| 107 |  | Small pieces of copper wire |  | 01 roll |
| 108 |  | Breadboard. |  | 04 Nos. |
| 109 |  | Connecting wires |  | 02 Rolls |
| 110 |  | Rheostat. |  | 04 Nos. |
| 111 |  | Incandescent lamp of different watts. |  | 04 Nos. |
| 112 |  | Voltmeter |  | 04 Nos. |
| 113 |  | Ampere meter |  | 04 Nos. |
| 114 |  | Multimeter |  | 04 Nos. |
| 115 |  | Combination plier |  | 04 Nos. |
| 116 |  | Phase tester |  | 04 Nos. |
| 117 |  | Energy meter |  | 04 Nos. |
| 118 |  | Thin coated copper wire. |  | 02 Rolls |
| 119 |  | Dry cell battery |  | 04 Nos. |
| 120 |  | Horse shoe magnet. |  | 04 Nos. |
| 121 |  | Transformer. |  | 02 Nos. |
| 122 |  | Compass needle. |  | 02 Nos. |
| 123 |  | Bar Magnet. |  | 04 Nos. |
| 124 |  | Galvanometer. |  | 02 Nos. |
| 125 |  | Capacitors, 4.7 μF |  | 04 Nos. |
| 126 |  | Capacitors, 10, 100 & 8.2 μF. |  | 04 Nos. (each) |
| 127 |  | Voltmeter. |  | 04 Nos. |
| 128 |  | D.C. power supply unit. |  | 02 Nos. |
| 129 |  | Bridging plugs. |  | 04 Nos. |
| 130 |  | Connecting leads. |  | 04 |
| 131 |  | Multimeter. (With option of Capacitance measurement) |  | 04 Nos. |
| 132 |  | D.C. power supply unit |  | 02 Nos. |
| 133 |  | Variable D.C powers supply 0-30 Voltas |  | 02 |
| 134 |  | Capacitor (Sanyo Operated Volts 6.3 V, SE 30) |  | 02 Nos. (each) |
| 135 |  | D.C Analog or Preferably Digital voltmeter Range 0-50V. |  | 02 Nos. (each) |
| 136 |  | L.E.D (3 to 9 V) |  | 02 Nos. (each) |
| 137 |  | Diode |  | 04 Nos. |
| 138 |  | Resistor |  | 04 Nos. |
| 139 |  | Bread board |  | 04 Nos. |
| 140 |  | Digital Multimeter |  | 04 Nos. |
| 141 |  | Oscilloscope Diode |  | 04 Nos. |
| 142 |  | Wire cutter |  | 04 Nos. |
| 143 |  | Wire stripper |  | 04 Nos. |
| 144 |  | Zener diode |  | 04 Nos. |
| 145 |  | Light Emitting diode |  | 04 Nos. |
| 146 |  | Soldering iron |  | 04 Nos. |
| 147 |  | FET Transistor |  | 04 Nos. |
| 148 |  | MOSFET Transistor |  | 04 Nos. |
| 149 |  | UJT |  | 04 Nos. |
| 150 |  | SCR |  | 04 Nos. |
| 151 |  | Breadboard or trainer |  | 01 Nos. |
| 152 |  | DIAC |  | 04 Nos. |
| 153 |  | TRIAC |  | 04 Nos. |
| 154 |  | Functions Generator |  | 01 Nos. |
| 155 |  | Op-Amp |  | 04 Nos. |
| 156 |  | White chart paper |  | 100 Nos. |
| 157 |  | Graphics and drawing sheets |  | 200 Nos. |
| 158 |  | Pencil |  | 30 Nos. |
| 159 |  | Eraser |  | 30 Nos. |
| 160 |  | Sharpener |  | 30 Nos. |
| 161 |  | Tee |  | 25 Nos. |
| 162 |  | Drawing Board |  | 25 Nos. |
| 163 |  | Set Square |  | 25 Nos. |
| 164 |  | Geometry set |  | 25 Nos. |
| 165 |  | French Curve |  | 25 Nos. |
| 166 |  | Printer |  | 01 Nos. |
| 167 |  | Multimedia projector |  | 01 Nos. |
| 168 |  | Computer |  | 25 Nos. |
| 169 |  | CAD software |  | 01 |
| 170 |  | IDE Software (C Language) |  | 01 |
| 171 |  | Power supply |  | 02 |
| 172 |  | AND gate |  | 02 Nos. |
| 173 |  | OR gate |  | 02 Nos. |
| 174 |  | NAND gate |  | 02 Nos. |
| 175 |  | XOR gate |  | 02 Nos. |
| 176 |  | NOR gate |  | 02 Nos. |
| 177 |  | NOT gate |  | 02 Nos. |
| 178 |  | Tool set |  | 02 Nos. |
| 179 |  | Bread board |  | 02 Nos. |
| 180 |  | Wire 0.5 mm |  | 02 Rolls |
| 181 |  | Digital clock |  | 04 Nos. |
| 182 |  | 555 IC |  | 02 |
| 183 |  | - Connecting leads. |  | 02 Rolls |
| 184 |  | Coil |  | 02 Rolls |
| 185 |  | - Armature |  | 02 Nos. |
| 186 |  | - Winding machine |  | 02 Nos. |
| 187 |  | - Insolating paper |  | 02 Nos. |
| 188 |  | - Carbon brushes |  | 04 Nos. |
| 189 |  | - Soldering wire |  | 02 Rolls |
| 190 |  | Stator frame |  | 02 Nos. |
| 191 |  | - Spring balance |  | 02 Nos. |
| 192 |  | - Ohm meter |  | 02 Nos. |
| 193 |  | - DC compound motor |  | 02 Nos. |
| 194 |  | Electro dynamo meter |  | 02 Nos. |
| 195 |  | - AC series motor |  | 02 Nos. |
| 196 |  | - Reluctance motor |  | 02 Nos. |
| 197 |  | - Hysteresis motor |  | 02 Nos. |
| 198 |  | - Stepper motor |  | 02 Nos. |
| 199 |  | - Servo motor |  | 02 Nos. |
| 200 |  | Control circuit |  | 02 Nos. |
| 201 |  | Temperature sensors. |  | 02 Nos. |
| 202 |  | IDE Software (C++ Language) |  | 01 |
| 203 |  | Sound sensors. |  | 02 Nos. |
| 204 |  | Proximity sensors. |  | 02 Nos. |
| 205 |  | Pressure sensors. |  | 02 Nos. |
| 206 |  | Light sensors. |  | 02 Nos. |
| 207 |  | Position sensors. |  | 02 Nos. |
| 208 |  | Voltage sensors. |  | 02 Nos. |
| 209 |  | Current sensors. |  | 02 Nos. |
| 210 |  | Vision sensors. |  | 02 Nos. |
| 211 |  | Microcontroller |  | 02 Nos. |
| 212 |  | - Arduino Uno |  | 02 Nos. |
| 213 |  | - Arduino Mega |  | 02 Nos. |
| 214 |  | Raspberry pi |  | 01 |
| 215 |  | Software of various microcontroller |  | 01 |
| 216 |  | - Links |  | 02 |
| 217 |  | - Joints |  | 02 |
| 218 |  | - End effectors |  | 02 |
| 219 |  | - Nuts |  | 02 Dozen (each size) |
| 220 |  | - Bolts |  | 02 Dozen (each size) |
| 221 |  | Screws |  | 02 Dozen (each size) |
| 222 |  | - Grippers |  | 02 Nos. |
| 223 |  | Conveyor belt |  | 02 Nos. |
| 224 |  | Sensor |  | 02 Nos. |
| 225 |  | Actuator |  | 02 Nos. |
| 226 |  | - Jumpers |  | 02 Nos. |
| 227 |  | - SD card |  | 02 Nos. |
| 228 |  | - LED |  | 02 Nos. |
| 229 |  | - Power Adaptor |  | 02 Nos. |
| 230 |  | HDMI cable |  | 02 Rolls |
| 231 |  | Humidity sensor |  | 02 Nos. |
| 232 |  | Robotic fastening tool kit |  | 02 Nos. |
| 233 |  | Measuring tool kit |  | 02 Nos. |
| 234 |  | Manipulator links and joints |  | 02 Set |
| 235 |  | Complete design of the Robot |  | 02 Nos. |
| 236 |  | Complete design of the sensor assembly |  | 02 Nos. |
| 237 |  | Robot operating software |  | 01 |
| 238 |  | Camera |  | 02 Nos. |
| 239 |  | Actuator (Robotics Arm) |  | 02 Nos. |
| 240 |  | Potentiometer |  | 02 Nos. |
| 241 |  | Calculator |  | 02 Nos. |
| 242 |  | - Basic ROV components |  | 02 Nos. |
| 243 |  | - Complete design of the ROV |  | 02 Nos. |
| 244 |  | - Remotely Operated Robot (ROR) Trainer |  | 01 Nos. |
| 245 |  | calibration standards measuring devices |  | 01 Nos. |
| 246 |  | - calibration angles measuring devices |  | 01 Nos. |
| 247 |  | - Robots geometry/Length angle alignment measuring device |  | 01 Nos. |
| 248 |  | Robotic Arm |  | 02 Nos. |
| 249 |  | - OHS Standard Manual |  | 01 |
| 250 |  | - friction Measuring Device |  | 01 Nos. |
| 251 |  | Model error detection devices |  | 01 Nos. |
| 252 |  | Strain gauge |  | 02 Nos. |
| 253 |  | Capacitive pressure sensor |  | 02 Nos. |
| 254 |  | Electromagnetic pressure sensor |  | 02 Nos. |
| 255 |  | Optical pressure sensor |  | 02 Nos. |
| 256 |  | Force balancing |  | 02 Nos. |
| 257 |  | Pressure sensor |  | 02 Nos. |
| 258 |  | Lm35 ic solid state temperature sensor |  | 02 Nos. |
| 259 |  | Negative temperature coefficient (ntc) thermistor |  | 02 Nos. |
| 260 |  | Resistance temperature detector (rtd) |  | 02 Nos. |
| 261 |  | Thermocouple |  | 02 Nos. |
| 262 |  | Semiconductor-based sensors |  | 02 Nos. |
| 263 |  | Retro-reflective sensors |  | 02 Nos. |
| 264 |  | Diffuse reflection sensors |  | 02 Nos. |
| 265 |  | BLDC motor |  | 02 Nos. |
| 266 |  | Solenoid valve |  | 02 Nos. |
| 267 |  | Robot Installation Manual |  | 02 Nos. |
| 268 |  | - Error detection tool kit |  | 02 Nos. |
| 269 |  | - Inclinometer |  | 02 Nos. |
| 270 |  | - Mechanical strength measuring tools |  | 01 Nos. |
| 271 |  | - Coordinate calibration tools |  | 01 Nos. |
| 272 |  | Tool Box |  | 02 Nos. |
| 273 |  | - ISO9283 |  | 01 |
| 274 |  | - Vibration sensors |  | 02 Nos. |
| 275 |  | - Jumpers |  | 02 Nos. |
| 276 |  | Turbo C, C++, Code block |  | 01 |
| 277 |  | Line following trainer kit |  | 01 |
| 278 |  | Jumper wires |  | 02 Rolls |
| 279 |  | Chassis for line following robot |  | 01 |
| 280 |  | Bluetooth transceiver |  | 02 Nos. |
| 281 |  | Smart phone |  | 02 Nos. |
| 282 |  | Relay control board |  | 02 Nos. |
| 283 |  | WiFi module |  | 01 Nos. |
| 284 |  | - Door Lock Mechanism |  | 01 Nos. |
| 285 |  | - Controller (Arduino) |  | 01 Nos. |
| 286 |  | - Android Application (Dabble) |  | 01 Nos. |
| 287 |  | - Security System Mechanism |  | 01 Nos. |
| 288 |  | Water level indication Mechanism |  | 01 Nos. |
| 289 |  | - 28 pin DIP IC Socket |  | 01 Nos. |
| 290 |  | - 16MHz Crystal Oscillator |  | 01 Nos. |
| 291 |  | - USB to Serial adapter |  | 01 Nos. |
| 292 |  | - ICs |  | 01 Nos. |
| 293 |  | - Mini Servo Motor |  | 01 Nos. |
| 294 |  | - DVD/CD Drives |  | 01 Nos. |
| 295 |  | - PCB Circuit Board |  | 01 Nos. |
| 296 |  | - 2pins Screw Terminal |  | 01 Nos. |
| 297 |  | 2x 4 pins Screw Terminal Connector |  | 01 Nos. |
| 298 |  | Mobile robot base |  | 01 Nos. |
| 299 |  | Power diode |  | 01 Nos. |
| 300 |  | Resistor of 1 K ohms |  | 01 Nos. |
| 301 |  | Centre tapped transformer |  | 01 Nos. |
| 302 |  | Inductor 2mH |  | 01 Nos. |
| 303 |  | Resistor of 10 K & 100 K ohms |  | 01 Nos. |
| 304 |  | Power MOSFET IRF 840 |  | 01 Nos. |
| 305 |  | Power Diode 1N4007 |  | 01 Nos. |
| 306 |  | DC Source 12V |  | 01 Nos. |
| 307 |  | Resistor 10K |  | 01 Nos. |
| 308 |  | - IC 7404 |  | 01 Nos. |
| 309 |  | Controller IC |  | 02 Nos. |
| 310 |  | Hydraulic bench |  | 02 Nos. |
| 311 |  | Control Valve |  | 02 Nos. |
| 312 |  | Centrifugal pump |  | 02 Nos. |
| 313 |  | Sump tank |  | 02 Nos. |
| 314 |  | Vertical pipe |  | 02 Nos. |
| 315 |  | Channel |  | 02 Nos. |
| 316 |  | Drain Valve |  | 02 Nos. |

# 6.	Members of the Curriculum Development Committee

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Mr. Ismail Ahmed | DUET | Dawood University of Engg. & Tech. |
| 2 | Mr. Talha Tariq | DUET | Dawood University of Engg. & Tech. |
| 3 | Mr. Liaqat Ali Jamro | Director Academics | STEVTA |
| 4 | Mr. Zeeshan Ali | Head of Industrial Engg. Deptt. | Dawood University of Engg. & Tech. |
| 5 | Mr. Taufiq Ahmed | Automation Engineer | Dewan Cement |
| 6 | Mrs. Nimra Ghazal | DUET | Dawood University of Engg. & Tech. |
| 7 | Mr. Mustafa Mohiuddin | DUET | Dawood University of Engg. & Tech. |
| 8 | Mr. Aurangzeb Ansari | UIT | Usman Institute of Technology |
| 9 | Dr. Noman Somroo | MUET | Mehran University of Engg. & Tech. |
| 10 | Mr. Usama Khan | Assistant Manager, Maintenance | PCI, Automotive Pvt. Ltd. |
| 11 | Mr. Yasir Arafat | Assistant Director | NAVTTC |
| 12 | Mr. Shaikh Asim Qamar | Certified DACUM Expert | iCAT Transmission |