| Amir Sarwar Asst. Director TTP Ghulam Ahmed Joya Manager PVTC Khurram Ejaz Dy. Director PCRWR Dr. Adil Hussain Farm Manager Research Farm Koont. Ammar Siddique Water management Officer Irrigation department M Arham Mahfooz Data Engineer Vision Developer M. Farooq Subject Expert MNSUA Multan Dr. Mubeen Sarwar Assistant Professor University of Layyah Umair Ali Project engineer AI Mudasr LTD Abdullah Manzoor Site In-charge, DHA Coordinators Mr. Muhammad Ishaq Deputy Manager, NAVTAC, Islamabad DACUM Facilitator Muhammad Saeed Ahmed | Assessment Evidence Guide For Water Conservation Supervisor Level-4 Summative Assessment Activity-3 |  |
| --- | --- | --- |
|  | Encl: 1. Assessments 2. Limited Response Questions 15th-19th December, 2025 Lahore |  |
|  | National Vocational & Technical Training Commission |  |

Section A: Instruction Sheet for the Candidate

| Qualification | Water Conservation Supervisor Level-4 |
| --- | --- |
| Competency Standards | - Perform Water Demand Assessment and Conservation Auditing - Carry out Water Resource Assessment and Monitoring - Perform In-situ soil moisture conservation techniques - Implement rainwater harvesting - Manage wastewater recycling and reuse systems - Perform On Farm Water Management - Maintain Water Supply System - Apply GIS and Remote Sensing in Water Resource Management - Prepare water usage Report - Manage community-based water conservation projects |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |

Section A1: Guidance for Candidate

| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 hrs. time frame (for practical demonstration & assessment): Scenario: You are assigned as a Water Conservation Supervisor on a mixed-crop farm where water availability is limited and uneven water application has been reported across different fields. The farm uses surface and/or groundwater sources connected to an on-farm irrigation or distribution network. GIS maps and/or remote sensing outputs showing field boundaries, irrigation zones, and water infrastructure are available. Some areas show signs of over-irrigation, while others indicate water stress and losses. You are required to perform following activity Activity: Execute GIS-supported on-farm water management operations by using spatial and field data to operate irrigation or distribution systems, apply in-situ conservation measures, correct location-specific water losses, and validate improved water use through comparative field measurements. |
| --- | --- |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to demonstrate the following criteria: |
| Minimum Evidence Required | - Load or access GIS or remote sensing data showing farm layout and water infrastructure. - Identify irrigation or water management zones using GIS-based information. - Measure available water at the farm supply source serving mapped zones. - Operate irrigation or distribution systems according to zone-wise requirements. - Apply in-situ soil moisture conservation techniques at identified field locations. - Locate water losses, leakage, or uneven application using GIS and field observation. |

Section A2: Self-Assessment Checklist

| Assessment Task | Scenario: You are assigned as a Water Conservation Supervisor on a mixed-crop farm where water availability is limited and uneven water application has been reported across different fields. The farm uses surface and/or groundwater sources connected to an on-farm irrigation or distribution network. GIS maps and/or remote sensing outputs showing field boundaries, irrigation zones, and water infrastructure are available. Some areas show signs of over-irrigation, while others indicate water stress and losses. You are required to perform following activity Activity: Execute GIS-supported on-farm water management operations by using spatial and field data to operate irrigation or distribution systems, apply in-situ conservation measures, correct location-specific water losses, and validate improved water use through comparative field measurements. |
| --- | --- |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Load or access GIS or remote sensing data showing farm layout and water infrastructure. |  |  |
| - Identify irrigation or water management zones using GIS-based information. |  |  |
| - Verify GIS-identified zones through on-farm field inspection. |  |  |
| - Measure available water at the farm supply source serving mapped zones. |  |  |
| - Measure water flow or volume entering on-farm distribution systems. |  |  |
| - Measure water application or delivery at selected GIS-identified field locations. |  |  |
| - Record measured water use against corresponding farm zones or fields. |  |  |
| - Operate irrigation or distribution systems according to zone-wise requirements. |  |  |
| - Adjust valves, outlets, or schedules based on spatial variation in water demand. |  |  |
| - Apply in-situ soil moisture conservation techniques at identified field locations. |  |  |
| - Locate water losses, leakage, or uneven application using GIS and field observation. |  |  |
| - Correct leaks, blockages, or uneven distribution at identified locations. |  |  |
| - Re-operate irrigation or distribution systems after corrective actions. |  |  |
| - Measure water application again to obtain after-intervention values. |  |  |
| - Compare before-and-after measurements to confirm improved on-farm water use. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Section B : Assessors Judgment Guide

| Qualification | Water Conservation Supervisor Level-4 |
| --- | --- |
| Competency Standards | - Perform Water Demand Assessment and Conservation Auditing - Carry out Water Resource Assessment and Monitoring - Perform In-situ soil moisture conservation techniques - Implement rainwater harvesting - Manage wastewater recycling and reuse systems - Perform On Farm Water Management - Maintain Water Supply System - Apply GIS and Remote Sensing in Water Resource Management - Prepare water usage Report - Manage community-based water conservation projects |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ___________________ Signature: _____________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Section B1: Observation Checklist

| Assessment Task |  | Scenario: You are assigned as a Water Conservation Supervisor on a mixed-crop farm where water availability is limited and uneven water application has been reported across different fields. The farm uses surface and/or groundwater sources connected to an on-farm irrigation or distribution network. GIS maps and/or remote sensing outputs showing field boundaries, irrigation zones, and water infrastructure are available. Some areas show signs of over-irrigation, while others indicate water stress and losses. You are required to perform following activity Activity: Execute GIS-supported on-farm water management operations by using spatial and field data to operate irrigation or distribution systems, apply in-situ conservation measures, correct location-specific water losses, and validate improved water use through comparative field measurements. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Loaded or access GIS or remote sensing data showing farm layout and water infrastructure. |  |  |  |  |  |
|  | Identified irrigation or water management zones using GIS-based information. |  |  |  |  |  |
|  | Verified GIS-identified zones through on-farm field inspection. |  |  |  |  |  |
|  | Measured available water at the farm supply source serving mapped zones. |  |  |  |  |  |
|  | Measured water flow or volume entering on-farm distribution systems. |  |  |  |  |  |
|  | Measured water application or delivery at selected GIS-identified field locations. |  |  |  |  |  |
|  | Recorded measured water use against corresponding farm zones or fields. |  |  |  |  |  |
|  | Operated irrigation or distribution systems according to zone-wise requirements. |  |  |  |  |  |
|  | Adjusted valves, outlets, or schedules based on spatial variation in water demand. |  |  |  |  |  |
|  | Applied in-situ soil moisture conservation techniques at identified field locations. |  |  |  |  |  |
|  | Located water losses, leakage, or uneven application using GIS and field observation. |  |  |  |  |  |
|  | Corrected leaks, blockages, or uneven distribution at identified locations. |  |  |  |  |  |
|  | Re-operated irrigation or distribution systems after corrective actions. |  |  |  |  |  |
|  | Measured water application again to obtain after-intervention values. |  |  |  |  |  |
|  | Compared before-and-after measurements to confirm improved on-farm water use. |  |  |  |  |  |
| Competent |  |  | Not Competent |  |  |  |

Section B2: Knowledge Assessment

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
| 1 | How to establish operational control over field-level water resources and activities? |  |  |
| 2 | What is the criteria to assess water availability, movement, and use through field-based measurements? |  |  |
| 3 | Why to regulate irrigation or distribution practices based on measured water demand is necessary? |  |  |
| 4 | Where do operational inefficiencies occur at field level, and how do you identify them? |  |  |
| 5 | What are the methods to apply suitable soil-moisture conservation practices at the field site? |  |  |
| 6 | How do you integrate rainwater harvesting, storage, or recharge systems into field operations? |  |  |
| 7 | How to correct leaks, blockages, or distribution losses efficiently during operation? |  |  |
| 8 | How do you test water quality for its relevance and suitability to field-level use? |  |  |
| 9 | What the water-quality results indicate, and how to interpret them for operational decisions? |  |  |
| 10 | What are the methods to balance conservation objectives with crop needs, system capacity, or site constraints? |  |  |
|  | What is the purpose of filtration in rainwater harvesting? |  |  |
| 1 11 | why the maintain of safe and environmentally responsible practices are important while operating in the field? |  |  |
| 1 12 | How do you adjust operational control based on feedback from system performance or field observations? |  |  |
| 1 13 | What field measurements, logs, or actions must be recorded, and how do you document them accurately? |  |  |
| 1 14 | What methods are used to confirm improvement in field-level performance using comparative indicators? |  |  |
| 1 15 | What evidence or outcomes are required to justify field-level decisions in a measurable and observable manner? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________

Material Required:

| Consumable | Non-Consumable |
| --- | --- |
| - Field log sheets / water measurement recording sheets - Marker pens / pencils - Flag markers or pegs for identifying GIS zones or problem areas - Tape or tags for temporary marking of leaks or uneven zones - Repair materials for minor leaks (rubber washers, sealing tape, clamps) - Soil mulch material (if applicable for in-situ moisture conservation) - Cleaning materials (rags, small broom) - Lime powder or chalk for marking - PPEs | - Mock-ups or partially installed water conservation structures - Site plan and system layout drawings - Laptop / tablet with basic GIS software or GIS maps loaded - Printed GIS maps or farm layout maps (field boundaries, irrigation zones) - GPS device or smartphone with GPS functionality - Flow meter (portable) or water measuring device - Measuring cylinder / container for volumetric water measurement - Pressure gauge (if pressurized irrigation system is used) - Soil moisture meter or soil auger - Irrigation system control tools (valve keys, gate handles, etc.) - Basic hand tools set (spanner, wrench, pliers, screwdriver) - Field notebook / clipboard - Calculator |