Course / Subject Name NQ CIVIL-324 HYDRAULICS & IRRIGATION ENGINEERING

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) | Formative Assessment (Part D) |
| --- | --- | --- | --- |
| LU (01-04) - The specific weight of water is taken as - 1000 kg /cm3 - 9810 N/m3 - one ml/cm3 - 1000 kg /m3 - The ratio of the specific weight of a liquid to the specific weight ofwater is called - Specific weight - Specific gravity - Specific volume - Uinit weight - Specific gravity is expressed in - gram/cm3 - kg/m3 - Nill - Unity - Atmospheric pressure is measured by - Barometer - Piezometer - Manometer - All of these - Value of standard atmospheric pressure is taken as column of height ofwater equal to - 14.7m - 10.3m - 101.3m - 0.76m - Pressure of one Pascal ‘Pa’ is equal to - 100 Lb/in2 - one kg/m2 - one N/m2 - 100 N/m2 - Piezometer tube is used for to find: - Velocity - Pressure - Area - Specific Gravity - Pressure measured by manometer is called as - Gauge pressure - Atmospheric pressure - Absolute pressure - All of these - Heavy liquid used in manometer is - Water - Mercury - Bromide - none of these - The statement ‘All liquid insert pressure in all directions equally’ is called. - Newton’s law - Pascal law - Archimedes’ law - Engineering law - Position of centre of pressure on a drowned surface lies always…………..the centroid of the surface - Below - At - Above - Anywhere - A circular plate is immersed vertically in water, total pressure on it will be calculated by the formula - ω A.x̄ - π Rx - π r2x - π Dx - Submerged weight of a body as compared to that in air is - Same - More - Less - No effect - A gravity dam resists water pressure due to its - Height - Top width - weight - Core - Resultant pressure "R" at dam is calculated by - P/W - P.W - √𝑃2 + 𝑊2 - P+W - Sluice gates are usually made of: - One Metal Wall & I-Beam - Two Metal Walls & I-Beam - Three Metal Walls & I-Beam - One Metal Wall & T-Beam - If the resultant pressure of a dam passes within the base then It shows stability against: - Tension failure - Compression failure - Sliding Failure - All of these - Centre of gravity of volume of liquid displaced is called - Metacentric - Centre of buoyancy - Centroid - Centre of gravity - A body is said to be in stable equilibrium When its Centre of gravity lies at the position as compared with its metacenter - Below - Coincides - Above - Any Where - The distance between Centre of gravity of a floating body and its metacenter is called as: - Full depth - Full height - Metacentric height - Gravity depth - In case of stable equilibrium metacenter lies at, the position, as compared with Centre of gravity - Below - Coincides - Above - Any Where - Depth of immersion of a floating body is called as - Toe - Draught - Heel - Buoyancy - The resultant upward pressure experienced by a floating body is called - Viscosity - Adhesion - Cohesion - Buoyancy - Metacentric height in meters of a merchant ship is kept as - 1.5 to 4.5m - 1.00 to 1.5m - 0.30 to 1.00m - 0.00 to0.30m - A floating body has zero metacenter height, it will be in such type of equilibrium - Stable - Unstable - Neutral - Any one ANSWER KEY 01; (b). 02; (b). 03; (a). 04; (c). 05; (b). 06; (c). 07; (b). 08; (a). 09; (d). 10; (a). 11; (a). 12; (a). 13; (c). 14; (c). 15; (c). 16; (b). 17; (b). 18; (b). 19; (a). 20; (c). 21; (c). 22; (b). 23; (d). 24; (c). 25; (c). | LU (05-08) - The quantity of a liquid flowing per second through a pipe is termed as - Velocity - Discharge - Acceleration - All of these - Unit of discharge in S.I system. - Cumec - Netwon/second - Meter/second - ft/sec - Formula of discharge - Q = V/A - Q = A/V - Q = AV - All of these - In a continuity of flow if area of flow decrease then velocity will - Increase - Decrease - Constant - No effect - A flow in which the velocity do not change from point to point is termed as - Steady flow - Uniform flow - Laminar flow - Turbulent flow - If at any section in the passage of flow the rate of discharge remains constant, the flow is termed as - Steady flow - Uniform flow - Laminar flow - Turbulent flow - For a perfect incompressible liquids flowing in continuous stream, Total energy of particles remains same while particles moves from one position to the others. This statement is called - Continuity Equation - Bernoulli Theorem - Pascal law - Archimedes Principle - According to Bernoulli Equation - Z+p+v=C - Z +P/w+v/g =C - Z +P/w+v2/g =C - Z +P/w+v2/2g=C - Bernoulli Equation is applied for - Venturimeter - Orifice meter - Pitot Tube - all of these - A venturimeter is used to measure - velocity of flowing liquid - Discharge of flowing liquid - Pressure of flowing liquid - all of these - A Pitot tube is used to measure - velocity of flowing liquid - Discharge of flowing liquid - Pressure of flowing liquid - all of these - In order to avoid the tendency of the separation at the throat in a venturimeter, the ratio of diameter at throat to that of the pipe - 1/16 to 1/8 - 1/8 to 1/4 - 1/4 to 1/3 - 1/3 to 1/2 - A long pipe is considered whose length is more than …………………………….times of its diameter. - 10 - 100 - 1000 - 500 - Hydraulic mean depth of a circular pipe is equal to - d/2 - d/4 - d/6 - d/8 - Loss of head due to sudden enlargement. - (v1-v2)2/2g - 0.5v2/2g - 0.375v2/2g - V2/2g - Loss of head at entrance in the pipe - (v1-v2)2/2g - 0.5v2/2g - 0.375v2/2g - V2/2g - Loss of head due to sudden contraction - (v1-v2)2/2g - 0.5v2/2g - 0.375v2/2g - V2/2g - Loss of head at exit of pipe is give by - (v1-v2)2/2g - 0.5v2/2g - 0.375v2/2g - V2/2g - Major loss of head is due to - Bend - Obstruction - Friction - All of These - Hydraulic gradient "i" is calculated by - hf/l - hfxl - l//hf - f/lh - The co-efficient of discharge is the ratio between actual discharge and ……………discharge - Theoretical - Minimum - Maximum - All - Average value of co-efficient of discharge of flowing liquid through orifice is taken as - 0.97 - 0.62 - 0.67 - 0.57 - An orifice is said to be small if available head of water is more than atleast……………….. times its height. - 3 - 5 - 8 - 10 - The continuous stream of a liquid that comes out from an orifice is termed as - Venacontracta - Mouth piece - Jet - Nappe - Ratio between the areas of venacontracta and that of the orifice is termed as co-efficient of - Contraction - Velovity - Dischrge - Energy ANSWER KEY 01; (b). 02; (a). 03; (c). 04; (a). 05; (b). 06; (a). 07; (b). 08; (d). 09; (d). 10; (b). 11; (a). 12; (d). 13; (c). 14; (a). 15; (a). 16; (b). 17; (c). 18; (d). 19; (c). 20; (a). 21; (a). 22; (b). 23; (b). 24; (c). 25; (a). | LU (09-12) - An opening in the side of a tank with the upstream liquid level below the top edge of the opening. - Orifice - Mouthpiece - Notch - Weir - Bottom edge of a notch is called - Head - Sill - Nappe - Bed - The sheet of liquid flowing over a notch - sheet of liquid - Flow of liquid - Nappe - None of these - Best shape of notch - Triangular - Rectangular - Trapezoidal - Circular - Purpose of providing a notch is to measure. - Velocity - Discharge - Sound - Bed - Discharge measured by a notch is of this type - Actual - Theoretical - Total - Anyone - Formula used to determine discharge through a right angled vee-notch - √2𝑔ℎ - 1.147h5/2 - 1.1417h3/2 - 9.81 h3/2 - A weir is a structure built across a river or canal to - Control water flow - Store water - Divert water - None of the above - The purpose of a weir is to - Increase the velocity of water - Decrease the velocity of water - Store water - None of the above - The type of flow that occurs over a weir is - Laminar flow - Turbulent flow - Transitional flow - Steady flow - The height of water above the weir crest is known as - Weir crest - Weir height - Backwater - Freeboard - The most common shape of a weir is - Triangular - Rectangular - Trapezoidal - Circular - Which of the following is NOT a type of open channel - Canal - Culvert - River - Pipe - What is the hydraulic radius of an open channel? - The ratio of the cross-sectional area to the wetted perimeter - The ratio of the wetted perimeter to the cross-sectional area - The depth of flow - The width of the channel - What is the Manning equation used for? - To calculate the discharge of an open channel - To calculate the velocity of flow in an open channel - To calculate the depth of flow in an open channel - To calculate the slope of an open channel - Which of the following factors affect the Manning roughness coefficient? - Channel Slope - Channel Roughness - Channel Geometry - All of the above - Which of the following is NOT a type of flow in open channels? - Uniform flow - Non-uniform flow - Critical flow - Steady flow - What is the Froude number used for? - To determine if flow is subcritical, critical, or supercritical - To determine the velocity of flow in an open channel - To determine the depth of flow in an open channel - To determine the slope of an open channel - What is one potential challenge associated with irrigation? - Reduced soil fertility - increased water use efficiency - Reduced crop yields - Increased water pollution - What is the total cultivable area in Pakistan? - 25 million hectares - 22 million hectares - 20 million hectares - 18 million hectares - Which river is the primary source of irrigation water in Pakistan? - Jhelum River - Sutlej River - Indus River - Ravi River - What is the largest dam in Pakistan? - Mangla Dam - Tarbela Dam - Warsak Dam - Hub Dam - Which canal is the largest and longest canal in Pakistan? - Sutlej Canal - Chashma Canal - Thal Canal - Nara Canal - What is the main crop grown in the irrigated areas of Punjab? - Wheat - Rice - Sugarcane - Cotton - Single purpose reservoir is used for - More than one purpos - One purpose - One or more than one - Two purposes only - is a type of dam based of material - Earth filled dam - Arch dams - Rigid dam - Overflow dam - Earthen dam is also known as: - Gravity dam - Rigid dam - Embankment dam - Detention dam - Pitching prevents ______ in earthen dams. - Sudden draw down - Collect the rain water - Seepage - Slipping - Hydraulic failure occurs nearly ____ %: - 40 - 50 - 30 - 20 - Slogging is same as ______ when reservoir is full: - Overtopping - Slope failure - Piping - For pressure - Rising of dam is important for hydro power generation: - Flood control - Increased capacity - Hydro power generation - All of these - _____ is a type of seepage failure: - Piping - Sloughing - Both above - Overtopping - For construction of head works: - Path should be narrow - Side should be accessible - Bank should be well defined - All of these - Main function of ________ is to provide proper anchorage to floor: - Cut of - Downstream apron - Inverted filter - Crest shutters - Struts are provided to resist: - Force - Thrust - Friction - Flow - Toe walls are provided at:: - Upstream side - Downstream side - Both above - Parallel to flow - It consists of sand & gravels: - Sheet pile - Block protection - Undersluices - Inverted filter - Adjustable gates are provided for: - Dam - Barrage - Weir - All of these - When water flows over a weir: - Its pressure increases - It creates thin sheet of water - Energy reduces - All of these - The slope of glacis is: - 4:1 - 4:3 - 2:3 - 3:1 ANSWER KEY 01; (c). 02; (b). 03; (c). 04; (a). 05; (b). 06; (a). 07; (b). 08; (a). 09; (c). 10; (b). 11; (c). 12; (b). 13; (d). 14; (a). 15; (b). 16; (d). 17; (d). 18; (a). 19; (d). 20; (b). 21; (c). 22; (b). 23; (d). 24; (b). 25; (b). 26; (a). 27; (c). 28; (a). 09; (a). 30; (c). 31; (d). 32; (c). 33; (d). 34; (a). 35; (b). 36; (c). 37; (d). 38; (b). 39; (b). 40; (a). | LU (13-16) - What is the primary factor that determines the water requirement of crops? - Soil type - Temperature - Crop type - Irrigation method - What is the term used to describe the volume of water required to be delivered to crops to meet their irrigation needs? - Irrigation demand - Crop yield - Command statement - Evapotranspiration rate - Which of the following irrigation methods is most efficient in terms of water use? - Flood irrigation - Furrow irrigation - Sprinkler irrigation - Drip irrigation - Which of the following crops is most tolerant of water stress? - Rice - Corn - Tomato - Cotton - What is the term used to describe the amount of water that can be stored in the soil and is available to plants? - Field capacity - Permanent wilting point - Available water capacity - Soil moisture tension - What is the recommended method for determining crop water requirements? - Soil moisture sensors - Weather data - Crop growth stage - All of these - Which of the following irrigation methods is the most efficient in terms of water conservation? - Flood irrigation - Dip irrigation - Sprinkler irrigation - Furrow irrigation - Which of the following irrigation methods is most suitable for hilly terrain? - Flood irrigation - Dip irrigation - Sprinkler irrigation - Furrow irrigation - Which of the following irrigation methods is most suitable for crops that require frequent watering? - Flood irrigation - Dip irrigation - Sprinkler irrigation - Furrow irrigation - Which of the following is a disadvantage of sprinkler irrigation? - It leads to waterlogging and soil erosion - It requires a lot of labor to install - It can lead to salt buildup in the soil - It is not suitable for crops with shallow roots - Canals are: - Artificial waterway - Natural waterway - Both above - None of these - It is a type of canal: - Lined - Free board - Service - Berm - For cutting side slope is taken as: - 1.5:1 - 1:1 - 1:1 to 1.5:1 - 1.5:1 to 2:1 - Banks are provided: - To keep water in a path - To construct service road - Both above - None of these - UCC stands for: - Undefined Canal Course - Underlined Canal Course - Upper Chenab Canal - Undefined Chenab Canal - Main canal contains the discharge: - 5-10 cumecs - 0.25-3 cumecs - 10 and more than 10 cumecs - 7 cumecs - Vc stands for: - Controlled velocity - Critical velocity - Constant velocity - None of these - It is a condition of silting: - Vact<V - Vact>V - Vact=V - None of these - Minimum bed of canal: - 1:1000 - 1:5000 - 1:8000 - 1:10000 - Which fall is suitable for all kind of discharges? - Ogee - Sarda - Inglis - Straight - Aqueduct is a - Bridge - Canal - Road - Path - Crest wall is provided: - Parallel to drain - Across the drain - Incline to drain - All of these - If flexibility of an outlet is more than unit, it is called: - Sub proportional outlet - Super proportional outlet - Hyper proportional outlet - None of these - Silt drawing capacity of an outlet must be: - 5% - 1% - 7% - 10% - is a type of an out let: - Non-modular - Flexible - Rigid - All of these ANSWER KEY 01; (c). 02; (c). 03; (d). 04; (a). 05; (c). 06; (d). 07; (b). 08; (c). 09; (c). 10; (c). 11; (a). 12; (a). 13; (c). 14; (c). 15; (c). 16; (c). 17; (b). 18; (a). 19; (b). 20; (c). 21; (a). 22; (b). 23; (c). 24; (d). 25; (d). |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) | Formative Assessment (Part D) |
| --- | --- | --- | --- |
| - Define the term Hydraulics - Enlist the properties of liquids - Define the term Specific Weight - Define the term Specific Gravity - Define the term Viscosity - Define intensity of Pressure - Define Head of Pressure - Define Atmospheric Pressure - Define Gauge pressure - Define Absolute Pressure - Which heavy liquid is used in Manometer? - Which liquid is used in Piezometer? - What is the specific gravity of Mercury? - Define Hydrostatics? - Write down the formula to determine Center of Pressure acting on an immersed surface - Write down the formula to determine Center of Pressure acting on an immersed surface - Write down formula to determine Moment of Inertia through centroid of a semi-circle of diameter "d" - State condititons of Equilibrium of floating bodies. - Define draught. - Define Metacentre - Define Metacentric height - Define Centre of buoyancy. - Define Stable Equilibrium - Define Unstable Equilibrium - State Archimedes‘ Principle | - Define the term Hydrokinematicas - Define Path line - Define Streamline - Define Potential Line - Define Flow Net - Define Hydraulic Grade Line ] - Define Total Energy Line - State limitations of Bernoulli’s Theorem - Write down the practical application of Bernoulli’s Theorem - Define Total Head - Write down the formula of Head - Draw sketch of Venturimeter - Draw sketch of Pitot Tube - Enlist components of Venturimeter - Write down the forlmula of velocity determination using Pitot Tube. - Define Short Pipe - Define Long Pipe - Enlist the types of minor head loss of flowing liquid - State Darcy's formula of major head loss - Define hydraulic gradient - Classigy orrifice according to shape - Classify orrifice according to discharge - Define Venacontracta - Enlist hyrdraulic Coefficients - State the formula used to find discharge through rectangular orifice | - Enlist types of Notch. - Define Crest of Notch. - Define Velocity of Approach. - Write down the advantages of V-notch - Write down the formula to find discharge through trapezoidal notch - Write the formula to find discharge through Stepped notch. - Enlist the different types of weirs based on their shapes - What are the factors that affect the discharge over a weir - Define Weir Coefficient - Write down the formula of discharge for Sharp Crested Weir - What is the use of Cippoletti Weir? - Write down the formula for the V-Notch Weir - Define Open Channel - What is closed conduit? - What is the Manning's equation used for in open channel flow? - How does the slope of an open channel affect the flow velocity? - What is critical depth in open channel flow? - What is the best shape for a channel to minimize frictional losses? - What is hydraulic jump? - State the conditions of most economical trapezoidal channel. - What is irrigation? - What is the total length of the irrigation canals in Pakistan? - What is the percentage of irrigated land in Pakistan? - Define Kharif Crop - Define Rabi Crop - Define multipurpose reservoir. - Define storage reservoir. - What are main elements for site selection for reservoir? - Enlist types of dams based on material. - Enlist types of dams based on design. - Enlist main components of earthen dam. - Enlist types of seepage failure. - Enlist types of weir. - Define rock fill weir. - Define weir breast. - Define crust shutters. - Define toe wall. - What are main types of weir w.r.t shape? - What are different perameters of flow of water over weir? - Define sheet pile. | - Define the term "command statement" in relation to crop water requirements. - What are the factors that influence the calculation of the crop water requirement? - Define Duty - Define Delta - Define Base Period - Define Kor Depth - Define drip irrigation method - Define sprinkler irrigation method - Write down the components of sprinkler irrigation method. - Write down the components of sprinkler irrigation method. - Enlist components of canal section. - Define freeboard. - Define counter berm - Enlist main types of canal - Define branch canal - Enlist types of distributary canal - Write the formula of discharge according to Mannings - Enlist the name of regimes. - Define Trapezoidal notch falls. - Enlist necessity of canal fall - Define siphon aqueduct. - Define super passage. - Define flexibility. - Define sensitivity. - Define efficiency. |

C . Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) | Formative Assessment (Part D) |
| --- | --- | --- | --- |
| Assessment Task 1 Hydrostatics pressure and center of pressure of vertically immersed surface - Gather required equipment & Select immersed surface shapes like Rectangle, Triangle, Square or Circular plates - Measure surface dimensions - Fill the water tank to desired depth & suspend the immersed surface at varying depths.. - Measure pressure using manometers at multiple points. - Record pressure values and corresponding depths. - Measure the distance from the free surface to the centroid (CG) of the immersed surface. - Calculate the moment of inertia (I) about CG. - Calculate CP coordinates - Verify CP location experimentally by applying a force at calculated CP and observing rotational equilibrium. - Record obsevations & calculations Assessment Task 2 Metacentric height of a floating body in lab - Fill the tank with water and insert the floating body and ensure that it is in stable equilibrium. - Set the horizontal sliding weight to position ‘x’ from the center towards left side of the vertical bar. - Give the model a small angular displacement in clockwise direction. - Measure the angle θ in the stable equilibrium. - Calculate GM experimentally. Draw a relationship between angle of heel θ(x-axis) and GM (y-axis), then obtain GM when equals zero - Plot the metacentric height for different angle of heel and calculate the metacentric height for zero angle of heel. - Calculate GM theoretically and compare - Record obsevations & calculations | Assessment Task 1 Determination of coefficient of discharge using Venturi-meter - Prepare required apparatus for observations as per operating manual - Measure flow rates using volumetric tank. - Determine readings/values by varying discharge using control valve - Observe readings through piezometric tubes at inlet and throat - Calculate head through piezometer readings - Calculate actual and theoretical discharge - Calculate coefficient of discharge using formula. - Develop table comprising observations & calculations Assessment Task 2 Determination of pipe flow losses in pipes - Set up the apparatus on the hydraulics bench - Fill pipe system and manometer with water by opening valves i.e, check calve, gate valve and flow control valve - Set the flow rate at approximately 17 liters/minute - Record height readings from all manometers - Draw hydraulic gradient line. - Calculate losses using required formulae - Develop table comprising observations & calculations | Assessment Task 1 Finding the velocity-by-velocity rod and by current meter - Select a measurement location - Deploy the apparatus in water submerging cups or propeller at full - Measure depth of apparatus below water surface - Measue velocity using calibration curve - Develop table comprising observations & calculations Assessment Task 2 Draw the plan of a multipurpose irrigation project - Draw plan of a multipurpose irrigation project - Draw plan showing river training and protection works - Draw layout of head works - Draw weir’s section and label it parts - Draw plan and section of a silt ejector - Label all necessary details | Assessment Task 1 Design and draw a plan showing the general layout of head works of a perennial canal - Use of a contour map with respect to canal route - Draw alignment of canal, distributary and water course on given scale - Prepare design of canal by Kennedy Critical velocity theory - Prepare design of canal by Lacey's Regime velocity - Find design parameters. - Draw typical X-sections of channels Assessment Task 2 Draw the plane and L-section of a masonry flume & Demonstration and drawings of an Aqueduct crossing - Identify required hydraulic structure and prepare drawing sheet as per requirement - Draw plane and L-section of a masonry flume - Prepare drawings of an Aqueduct crossing - Prepare drawings of a pipe outlet - Prepare drawings of adjustable proportional module (A.P.M) Outlet |