Course / Subject Name NQ CIVIL-324 HYDRAULICS & IRRIGATION ENGINEERING

ASSESSMENT TASKS (SUMMATIVE ASSESSMENT)

ASSESSMENT TASK 01

DETERMINATION OF PIPE FLOW LOSSES IN PIPES                                                                   03:00 Hours

APPARATUS: Stop watch, Hydraulic bench, Piezometer tube & Apparatus for losses in pipes

    - Wear required PPEs

    - Choose pipes with varying diameters and materials (e.g., copper, PVC).

    - Prepare and calibrate the required equipments.

    - Set up the apparatus on the hydraulics bench

    - Fill pipe system and manometer with water by opening valves i.e, check calve, gate valve and flow control valve

    - Set the flow rate at approximately 17 liters/minute

    - Take height readings from all manometers

    - Draw hydraulic gradient line.

    - Calculate losses using required formulae

    - Develop table comprising observations & calculations

    - Submit the results to lab instructor

ASSESSMENT TASK 02											01:00 Hour

A MOST ECONOMICAL TRAPEZOIDAL CHANNEL HAVING SIDE SLOPE AT 60" WITH THE HORIZONTAL HAS A BED SLOPE OF 1 IN 1600. FIND THE BED WIDTH AND THE DEPTH OF THE CHANNEL FOR A DISCHARGE OF 194 CUMECS. TAKE M FOR MANNINGS FORMULA AS 60.

- Idenentify given data and required data as per above noted statement.

- Compute required data using appropriate formulae

- Present the solution in tabular form

ASSESSMENT TASK 03												01:00 Hour

FIND THE DUTY OF WATER, IF IRRIGATION CANAL RUNS FOR 140 DAYS IN A SEASON WHEN WHEAT IS GROWN. DELTA IS FOUND TO BE 12 INCHES

- Idenentify given data and required data as per above noted statement.

- Compute required data using appropriate formulae

- Present the solution in tabular form

OR

ASSESSMENT TASK 01													03:00 Hours

METACENTRIC HEIGHT OF A FLOATING BODY IN LAB

APPARATUS: 	Hydraulic bench, Metacentric height apparatus & Electronic digital balance

  - Wear required PPEs

  - Choose a model with known dimensions and mass.

  - Record initial depth & Gradually tilt using inclination device.

  - Fill the tank with water and insert the floating body and ensure that it is in stable equilibrium.

  - Set the horizontal sliding weight to position ‘x’ from the center towards left side of the vertical bar.

  - Give the model a small angular displacement in clockwise direction.

  - Measure the angle θ in the stable equilibrium.

  - Calculate GM experimentally. Draw a relationship between angle of heel θ(x-axis) and GM (y-axis), then obtain GM when equals zero

  - Plot the metacentric height for different angle of heel and calculate the metacentric height for zero angle of heel.

  - Calculate GM theoretically and compare

  - Record obsevations & calculations

  - Submit the results to lab instructo

ASSESSMENT TASK 02										01:00 Hour

PRODUCE IRRIGATION MAP OF PAKISTAN

- Select appropriate drawing instruments

- Identify drawing instruments and prepare drawing sheet as per give standards.

- Draw map of Pakistan and show rivers on it.

- Draw barrages, main canals, head works and link canals

ASSESSMENT TASK 03										01:00 Hour

DESIGN AN IRRIGATION CHANNEL IN ALLUVIAL SOIL FROM FOLLOWING DATA USING LACEY’S THEORY: DISCHARGE = 15.0 CUMEC, LACEY’S SILT FACTOR = 1.0; SIDE SLOPE = ½ : 1

- Identify given data and required data as per above noted statement.

- Compute required data using appropriate formulae

- Present the solution in tabular form

OR

ASSESSMENT TASK 01														03:00 Hours

DETERMINATION OF COEFFICIENT OF DISCHARGE USING VENTURIMETER / WEIR / NOTCH / ORRIFICE METER

APPARATUS: Hydraulic bench, Venturimeter, Weirs, Notches, Orifice meter apparatus

FOR VENTURIMETER

- Wear required PPEs

- Prepare required apparatus for observations as per operating manual

- Measure flow rates using volumetric tank.

- Determine readings/values by varying discharge using control valve

- Observe readings through piezometric tubes  at inlet and throat

- Calculate head through piezometer readings

- Calculate actual and theoretical discharge

- Calculate coefficient of discharge using formula.

- Develop table comprising observations & calculations

- Submit the results to lab instructor

FOR WEIR/NOTCH

- Wear required PPEs

- Familiarize with weir / notch

- Prepare equipment

- Calibrate equipment for accurate measurement

- Calculate area of notch/weirs using required formulae

- Establish steady flow rate

- Measure flow rate (Actual Discharge)

- Record water level

- Calculate theoretical discharge using formulae

- Caluclate coefficient of discharge using formula

- Plot Cd vs Discharge

- Develop observations & calculations in tabular form

- Submit results to the concerned

FOR ORRIFICE METER

- Wear required PPEs

- Familiarize with weir / notch

- Prepare equipment

- Calibrate equipment for accurate measurement

- Set constant head in tank

- Measure horizontal and vertical coordinates

- Calculate actual discharge using required apparatus

- Use formulae to calculate coefficient of velocity and discharge

- Develop observations & calculations in tabular form

- Submit results to the concerned

ASSESSMENT TASK 02										01:00 Hour

A DISTRIBUTARY CANAL HAS A CULTURABLE COMMAND AREA OF 8000 HECTARES. THE INTENSITY OF IRRIGATION FOR RABI CROP IS 70% AND BASE PERIOD IS 120 DAYS. THE DEPTH OF APPLICATION OF WATER IS 80CM. CALCULATE THE DISCHARGE TO BE SUPPLIED IN THE CHANNEL.

- Identify given data and required data as per above noted statement.

- Compute required data using appropriate formulae

- Present the solution in tabular form

ASSESSMENT TASK 03										01:00 Hour

DESIGN AN IRRIGATION CHANNEL WITH FOLLOWING DATA

- FULL SUPPLY DISCHARGE = 06 CUMECS

- RUGOSITY COEFFICIENT (N)=0.225

- C.V.R m = 1.0

- BED SLOPE = 1 IN 5000

ASSUME OTHER RESONABLE DATA

- Identify given data and required data as per above noted statement.

- Compute required data using appropriate formulae

- Present the solution in tabular form