Course / Subject Name NQ-Shop-132 Engineering Workshop Practice

- Assessment Tasks (Summative Assessment)

| Summative Assessment |
| --- |
| Activity 1: - Find three hazards in the wood workshop, list three PPE items and their uses, and show or explain first aid for a wound, burn, or CPR. (2 Hours) Performance criteria: - Select tools safely according to their functions and OHS requirements. - Check lubricants, fluids, and maintain tools as per manufacturer recommendations. - Clean the work area, dispose of materials properly, and store tools securely. - Select personal protective equipment (PPE) - Identify hazards, OHS issues and risks in work area - Identify the type of timber, and perform a compressive strength test on the given timber sample. Select tools for sharpening, and demonstrate sharpening of a plane iron, chisel, scraper, router bits, and carving tools. (3 Hours) Performance criteria: - Report compressive strength - Perform plane iron sharpening - Perform chisel sharpening - Perform scraper sharpening - Perform router bit sharpening - Perform craving tool sharpening - Find out compressive strength - Perform sharpening |
| Activity 2: - Create a drawing of a product given below as per your choice (Fig: 1 / Fig: 2) and estimate the quantity of timber needed based on the drawing. (3 Hours) or Fig: 1 Fig:2 Performance criteria: - Use appropriate drawing tools for drafting. - Identify components of product. - Draw a plan, cross-section, and elevation of the product. - Estimate the quantity of timber required based on the drawing. - Create timber joint on the given dimensions, insert fastenings into the holes. Select the appropriate tools, prepare the timber surface for polish, and perform the spirit off process to ensure a smooth finish. (2 Hours) Performance criteria: - Select appropriate tools and equipment for the task. - Clean and store tools, equipment, and rubbers properly after use. - Create timber joints, drill holes, and insert fastenings. - Prepare and lightly sand the timber surface for a smooth finish. - Apply polish, oil, and pumice powder as needed for the job. |  |

Activity 3: - Draw the electrical wiring for any two rooms of given floor plan. Measure the voltage and current in a circuit, calculate the resistance using Ohm’s law, and determine the power based on the voltage and current values. (3 Hours) Performance criteria: - Arrange tools, materials, and work permits as per the job needs. - Follow OHS rules and use the right tools to measure voltage and current. - Draw and use circuit diagrams to calculate resistance and power. - Mark and record measurements clearly. - Prepare an electrical wiring plan. - Use the right tools to measure voltage and current. - Conduct insulation testing, load calculation for earthing, removing insulation, making joints, aligning cables, testing wiring, and grounding in institute lab. (2 Hours) Performance criteria: - Use appropriate tools, materials, and obtain the necessary work permit to perform wiring tasks. - Demonstrate insulation removal, joint-making, and align cables following safety standards. - Arrange tools, materials, and obtain the required work permit for the wiring task. - Perform insulation testing on cables. - Perform color coding, tagging, or numbering for wire identification as per standards. - Calculate the load required for earthing and ensure proper grounding of electrical appliances. - Test wiring for continuity, circuit functionality, and verify results with a live test.