Course / Subject Name: NQ-103 Civil Engineering Drawing-I

- Assessment Tasks (Summative Assessment)

Summative Assessment

Activity No 1: Task 1: Prepare drawings Plan of a given figure (Annex-A). Performance Critera - Select appropriate drawing tools (e.g., scales, pencils, compasses, protractors) and Place the drawing sheet (Standered size) on the drawing board. - Draw the layout on drawing sheet accoriding to the specification/Standered. - Select the scale (1’ = 1/4” to 1’ = 1/8”) for drawing the given plan. - Draw plan of given drawing accroding to the selected scale. - Mark the openings on the drawn plan by using symbols of doors, windows and ventilators on the drawing sheet using lead pencil. Task 2: Prepare drawings (Elevation and section) of a given figure (Annex-A). Performance Criteria - Draw Front elevation base on the detail from the plan. - Draw AA-Section and represent the interior cut details of the structure to Show walls, floors, roof, and other elements visible in the section, Clearly. - Draw single stroke lettering according to standards. - Draw all necessary dimensions, including wall thickness, room sizes, and clearances. - Label all views (Plan, Elevation, Section) and add appropriate notes or details where required. - Ensure all drawings are clean, legible, and free of errors. OR Activity No 2: Task 1: Draw ellipse by parallelogram method (Take major axis 10 cm and minor axis 6 cm) Performance Critira - Select appropriate tools and Place the drawing sheet (Standered size) on the drawing board. - Draw the layout on drawing sheet accoriding to the specification/Standered. - Select the scale (1 = 10 to 1 = 20) for drawing the given plan. - Draw ellips according to dimension and selected scale. Task 2: Draw orthographic projection of a given object by 3rd Angle Projection System (Annex-2a). Performance Critira - Draw orthographic projections according to dimension and selected scale. Task 3: Draw the given x-section of the step foundations on a drawing sheet (Annex-2b) Performance Critira - Select the scale (1’ = 1/2” to 1’ = 1/4”) for drawing the given plan. - Sketch the given foundation cross section on the drawing sheet as per the selected scale with the help of lead pencil. - Draw dimensions on the drawing. - Draw single stroke lettering according to standards. - Ensure all drawings are clean, legible, and free of errors.

Annex-A

Annex-2a							Annex- 2b