Course / Subject Name: - NQ-Civil-374 RCC and Concrete Technology

- MCQs

| Formative Assessment (Part-A) 15 |  |  |  |  |  | Formative Assessment (Part-B) |  |  |  |  |  | Formative Assessment (Part-C) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LU1-LU4 |  |  |  |  |  | LU5-LU8 |  |  |  |  |  | LU9-LU12 |  |  |  |  |  |
| - Ingredient of Concrete is - Fine aggregate - Corse aggregate - Water - All of these - Main purpose of a slump test. - To measure workability - To measure strength - To measure durability - To measure density - The standard size of the slump cone is - 300 mm - 500 mm - 600 mm - 700 mm - Water-cement ratio in concrete directly affects - Workability - Strength - Durability - All of the above - Separation of ingredients of fresh concrete is called - Shrinkage - Bleeding - Segregation - Creep - Which is not destructive test method for hardened concrete? - Pull out Test - Rebound hammer test - Core cutter test - Split Cylinder test - What is the main purpose of a compressive strength test? - Evaluate durability - Assess concrete curing - Measure load-bearing capacity - Determine air content - What is the main purpose of the rebound hammer test? - Determine compressive strength - Evaluate curing quality - Assess surface hardness - Measure workability - The rebound hammer provides results in terms of: - Load - Hardness number - Strength index - Velocity - What is the first step in steel fixing preparation - Cutting steel bars - Checking for defects - Marking steel bars - Placing reinforcement - Best shape of aggregates for good workability is - Conical - Elongated - Flaky - Rounded - Tools used for bending steel bars - Wrench - Hammer - Vee-Bee apparatus - Rebar bender - Steel used in RCC work. - Mild steel - Wrought iron - Cost iron - Stainless steel - What is the main objective of destructive testing? - Ensure durability - Measure load-bearing capacity - Test chemical properties - Identify defects in fresh concrete - Standard height of a slump cone - 150 mm - 200 mm - 300 mm - 450 mm |  |  |  |  |  | - Equipment is commonly used to compact freshly poured concrete. - Vibrator - Hammer - Trowel - Chisel - Curing important for freshly placed concrete - Increases workability - Prevents bleeding - Ensures strength development - Reduces weight - Which method is used for placing concrete in underwater conditions - Hand pouring - Tremie method - Vibration method - Pumping method - Why is formwork essential in concreting operations? - Prevents segregation - Maintains the shape of the structure - Ensures uniform curing - Provides reinforcement - What is the main challenge in cold weather concreting? A. Bleeding B. Segregation C. Delayed setting D. High evaporation rate - What method is effective for concreting in underwater conditions? A. Slip-form method B. Tremie method C. Vacuum dewatering D. Centrifugal method - What is added to concrete to accelerate setting in cold weather? A. Retarder B. Silica fume C. Air-entraining agent D. Calcium chloride - What is the value of the strength reduction factor (ϕ) for flexure in the ACI code? A. 0.6 B. 0.7 C. 0.9 D. 1.0 - Which material property influences the design shear strength of concrete? A. Compressive strength B. Tensile strength C. Modulus of elasticity D. Bond strength - What is the main mechanism to resist shear in beams? A. Concrete tensile strength B. Shear reinforcement C. Beam width D. Longitudinal reinforcement - Which concrete compressive strength value is typically used in ACI design equations? A. Ultimate strength B. 28-day strength C. Early strength D. Shear strength - What is the primary purpose of the reinforcement in a rectangular beam under bending? A. To resist compression B. To resist tension C. To prevent deflection D. To increase the beam's width - Which of the following is the main purpose of the neutral axis in a beam? A. To divide the beam into compressive and tensile zones B. To calculate the deflection C. To increase the bending strength D. To calculate shear stress - What does the term "balanced section" mean in beam design? A. When the beam is reinforced with the correct amount of steel B. When the beam's width is equal to its depth C. When the concrete has reached its maximum strength D. When the beam's steel reinforcement is not needed - What does the term "effective depth" refer to in a beam design? A. The total depth of the beam B. The depth of the beam minus the cover C. The depth of the concrete used D. The depth of the tensile reinforcement |  |  |  |  |  | - What is the primary factor in the design of a slab using the ACI code? A. Slab thickness B. Live load C. Moment of inertia D. Bending strength - In the ACI code, what is the recommended maximum span-to-depth ratio for slabs? A. 10 B. 20 C. 30 D. 40 - What is the primary purpose of providing shear reinforcement in slabs as per the ACI code? A. To resist bending moments B. To resist shear forces C. To prevent thermal expansion D. To prevent shrinkage cracks - What is the primary function of the bottom reinforcement in a slab? A. To resist tension B. To resist compression C. To prevent shrinkage D. To resist shear forces - Which of the following is considered when designing an axially loaded column for stability? A. Lateral load B. Buckling length C. Column height D. All of the above - Which of the following is a key factor when designing short columns under axial load? A. Lateral load capacity B. Buckling effect C. Concrete compressive strength D. Both A and B 7. What is the typical design axial load capacity for reinforced concrete columns? A. 0.2fc′Ag B. 0.5fc′Ag C. 0.85fc′Ag D. 1.2fc′Ag 8. How is the slenderness ratio of a column defined? A. Column height divided by the column diameter B. Column height divided by the column width C. Column height divided by the effective length D. Column width divided by column depth 9. What is the purpose of providing dowels in column footings? A. To prevent column misalignment B. To resist bending moments C. To connect the footing with the column D. To increase the size of the footing 10. What is the most common failure mode for column footings? A. Shear failure B. Bending failure C. Torsional failure D. Settlement failure 11. Which of the following is NOT typically used in footing design? A. Axial load B. Column material C. Soil bearing capacity D. Reinforcement spacing 12. Which of the following is a factor affecting footing size? A. Soil bearing capacity B. Column height C. Column material D. Footing color 13. In the design of concrete stairs, what does the term "rise" refer to? A. The horizontal length of a step B. The vertical distance between two consecutive steps C. The total height of the stairway D. The thickness of the tread 14. In the design of concrete stairs, what is the purpose of the handrail? A. To provide support for reinforcement B. To enhance the aesthetic appearance C. To provide safety and stability for users D. To transfer load to the wall 15. What is the typical range of "tread" in stair design? A. 200-300 mm B. 100-150 mm C. 300-500 mm D. 250-400 mm |  |  |  |  |  |
| Answer Key |  |  |  |  |  | Answer Key |  |  |  |  |  | Answer Key |  |  |  |  |  |
| 1 | D | 6 | B | 11 | A | 1 | A | 6 | B | 11 | B | 1 | D | 6 | D | 11 | B |
| 2 | A | 7 | A | 12 | D | 2 | C | 7 | D | 12 | B | 2 | B | 7 | C | 12 | A |
| 3 | A | 8 | C | 13 | A | 3 | B | 8 | C | 13 | A | 3 | B | 8 | C | 13 | B |
| 4 | D | 9 | B | 14 | B | 4 | B | 9 | A | 14 | A | 4 | A | 9 | C | 14 | C |
| 5 | C | 10 | C | 15 | C | 5 | C | 10 | B | 15 | B | 5 | D | 10 | D | 15 | D |

- Limited Response Questions

| Formative Assessment (Part-A)10 | Formative Assessment (Part-A) | Formative Assessment (Part-A) |
| --- | --- | --- |
| Q1. Define the purpose of the slump test for fresh concrete. Q2. Describe the procedure for conducting a compaction factor test. Q3. List the different types of slumps and their significance in determining workability. Q4. Describe the process of preparing concrete samples for compressive strength testing. Q5. Perform a compressive strength test and record the results. Q6. Identify the purpose of using a rebound hammer in non-destructive testing. Q7. Explain the steps involved in conducting an ultrasonic pulse velocity (UPV) test. Q8. List the tools required for steel fixing preparation and their uses. Q9. Describe the process of marking and cutting steel bars as per specifications. Q10. Explain how to inspect steel bars for defects before fixing. | Q1. Define the concreting operations. Q2. Discuss the procurement and storing of concrete materials. Q3. Describe method of batching i.e., by volume and by weight Q4. State structure and working of batching plant. Q5. Discuss mixing methods of concrete and types of concrete mixers. Q6. Define form work for concrete. Q7. Define transportation of concrete. Q8. State method of placing concrete. Q9. Enlist various types of concrete surface finishing. Q10. Describe objective of concrete curing. Q11. Enlist the methods for under water concreting. | - Differentiate between one way and two-way slabs. - Calculate the minimum thickness of a one-way slab with a 4-meter span and simple supports as per ACI Code. - Determine the effective depth of a slab for a 5-meter span with 25 mm cover and 12 mm diameter bars. - Explain the role of tie reinforcement in preventing buckling in axially loaded columns. - State ACI two-way slab bending moment coefficients and its use. - Calculate the design ultimate load for a slab with a dead load of 2 kN/m² and a live load of 4 kN/m². - Find out load carrying capacity of columns. - Explain the purpose of tie reinforcement in axially loaded columns. - Describe the arrangement of main and distribution bars in the reinforcement of staircase waist slabs. - Calculate the bending moment for a staircase waist slab with a span of 4 m and a load of 5 kN/m². |

- Assessment Task

| Formative Assessment (Part-A) | Formative Assessment (Part-B) | Formative Assessment (Part-C) |
| --- | --- | --- |
| - Perform Slump cone test for fresh Concrete (1:2:4) Performance criteria - Identify and select apparatus for test - Select ingredients of concrete according to requirement. - Mix material according to ratio. - Perform slump test according to standard procedure - Interpret slump results for consistency of fresh concrete - Describe and conduct Compression Test on hardened concrete. Performance criteria - Identify and select apparatus for test - Prepare concrete according to concrete mix ratio. - Prepare cube, cylinder and beam specimens for destructive tests - Cure concrete samples for specified duration - Perform compressive strength on concrete sample to determine compressive strength of concrete. - Conduct Rebound Hammer test on hardened concrete Performance criteria - Identify and select apparatus for test - Calibrate apparatus, if required - Mark suitable location for testing - Perform rebound hammer test on sample to find its compressive strength. - Identify the grade and size of given steel bar. Performance criteria - Select tools required for steel fixing. - Identity steel bars as per design. - Perform straightening of steel bars for steel fixing. - Perform cutting and bending of steel bars as per given design. - Perform binding and fixing of steel bars as per given design. | - Design a simply supported reinforced concrete rectangular beam with a Uniformly Distributed Load (UDL) for flexure using the ACI code. The beam has a length of 25 ft, a width of 14 in, and a depth of 28 in. The data is given below fc’ = 5000 psi fy = 60,000 psi. The beam is subjected to a UDL = 3 kips/ft. Determine the required area of reinforcement, select the reinforcement size and spacing, and check for minimum and maximum reinforcement requirements. Performance criteria - Calculate factored load using given/estimated loads. - Draw shear force diagram and find critical shear. - Check for shear strength of concrete and calculate shear reinforcement. Q2. Design a cantilever reinforced concrete rectangular beam with following data - Clear Span 5.5 m - Width = 300mm - Depth =600mm - compressive strength (fc’) = 28 MPa - yield strength of reinforcement (fy)= 420 MPa. - UDL of 37.5 kN/m Calculate Area of reinforcement=AS=? Performance criteria - Calculate factored load using given/estimated loads. - Draw shear force diagram and find critical shear. - Check for shear strength of concrete and calculate shear reinforcement. | - Design a simply supported one-way reinforced concrete slab for a room with a clear span of 12 ft. The slab is to support a live load of 100 psf and a dead load of 30 Psf excluding its self-weight. Data is given below fc′=4,000 psi fy =60,000 psi Performance criteria - Calculate thickness of slab according to dimensions and end conditions - Calculate factored load using given/estimated loads. - Calculate maximum bending moment for one way slab. - Calculate maximum bending moment for two-way slab using ACI moment coefficients. - Find area of steel required for maximum bending moment. - Select rebar size and calculate spacing of reinforcing bars. - Draw reinforcement details for slab. Q2. Design a RCC column 4m height having Dia 500mm, with axial load 1200 KN, Use concrete strength fc′= 30 MPa and yield strength fy = 440 MPa Performance criteria - Select type of column. - Calculate axial load on column. - Compute size of column required to support axial load. - Select size and calculate no. of longitudinal bars. - Select size and type of ties and calculate spacing. - Draw reinforcement details of column. Q3. Design steps of horizontal spanning stair when steps have an effective length 8ft, R=6”, T=10”, Live load 160 lbs/sft concrete strength fc′=4,000 psi and yield strength fy =60,000 psi Performance criteria - Identify type of stairs w.r.t design requirements. - Calculate thickness of waist slab. - Calculate factored load on stairs. - Calculate maximum bending moments. - Calculate area of steel according to ACI code. - Calculate spacing of bars. - Draw reinforcement details of stairs. |

SUBMITTED BY: -

AJAZ SADIQ, AMJAD RAFIQUE