Course / Subject Name: - NQ-Civil-374 RCC and Concrete Technology

Summative Assessment

Q.1 Design a simply supported reinforced concrete rectangular beam with a Uniformly Distributed Load (UDL) and Point load at Centre for flexure using the ACI code. The beam has a length of 25 ft, a width of 14 in, and a depth of 28 in. The concrete compressive strength (fc’) is 5000 psi, and the yield strength of reinforcement(fy) is 60,000 psi. The beam is subjected to a UDL of 3 kips/ft. The Point load at Centre is 85 Kips. Determine the required area of reinforcement, select the reinforcement size and spacing, and check for minimum and maximum reinforcement requirements. Performance criteria - Calculate factored load using given/estimated loads. - Draw shear force diagram and find critical shear. - Calculate Reactions - Check for shear strength of concrete and calculate shear reinforcement. - Calculate bending moments - Draw bending moment diagram Q2. Prepare destructive tests (Compression Strength Test) for cylinder at UTM. Performance criteria - Use PPEs - Identify and select apparatus for test - Select concrete cylinder according to concrete mix ratio. - Fix cylinder at UTM for destructive tests ((Compression Strength Test) - Apply load on Cylinder - Determine Compressive Load - Calculate compressive strength of concrete using formula.

SUBMITTED BY: -

AJAZ SADIQ, AMJAD RAFIQUE