Course / Subject Name _______NQ-Civil-273-Construction techniques_________________

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 6) | LU (7 to 12) | LU (13 to 18) |
| - Which of the following is a key environmental benefit of sustainable construction? - Increased construction costs - Increased waste production - Reduced energy consumption - Increased material use - Which material is considered eco-friendly in sustainable construction? - Plastic - Bamboo - Concrete - Steel - What is a common energy-efficient design strategy? - High-energy heating systems - Passive solar design - Open windows all day - Thick concrete walls - What is an example of a water-saving system in construction? - High-flow toilets - Excessive irrigation systems - Rainwater harvesting - Non-insulated pipes - Which of the following is a goal of green construction? - Use of harmful chemicals - Minimize energy consumption - Increased carbon emissions - Use of plastic materials - What is the first step in preparing a building layout? - Calculate material quantities - Obtain work instructions and specifications - Install scaffolding - Pour concrete - What is used to ensure the building corners are accurate? - GPS - String lines - Survey pegs - Laser beams - Which method is used to set a right angle in construction? - Triangulation principles - Laser level - 90-degree angle tool - Measuring tape - What is essential to check before setting out a building line? - The color of the materials - Measurements of the existing building or boundary - Site drainage - Site orientation - Which equipment is used for marking the site boundary? - Leveling machine - String lines - Digger - Pile driver - What should be checked before installing a termite barrier? - Weather conditions - Previous work on-site - Site color - Number of workers - Which personal protective equipment (PPE) is essential during termite barrier installation? - Hard hat and gloves - Eye protection only - Steel-toed boots only - None of the above - What is the purpose of a termite barrier in construction? - Prevent water leakage - Protect against fire - Prevent termite access - Improve aesthetic value - Which of the following is a common material for physical termite barriers? - Wood - Metal sheets - Concrete - Plastic membranes - Which material is commonly used for waterproofing? - Rubber - Clay - Bitumen - Cement - What is the first step in waterproofing works for construction? - Apply the membrane - Clear the work area and prepare the surface - Install insulation - Add drainage pipes - What is essential for ensuring waterproofing installation success? - Proper curing of the waterproofing membrane - Fast installation - Use of large quantities of material - Avoidance of testing - Which method is used to test the waterproofing system? - Pressure testing - Flood testing - Color change - Temperature monitoring - Which of the following is considered false work in construction? - Scaffolding - Concrete walls - Foundation - Brick masonry - Which type of shoring system is commonly used in construction? - Timber shoring - Steel cables - Aluminum rods - Concrete blocks - What is the main purpose of underpinning in construction? - To provide temporary support - To stabilize or strengthen foundations - To increase the height of walls - To decorate the building - What is a key safety consideration during scaffolding installation? - Ensure all components are securely connected - Avoid using personal protective equipment - Skip load calculations - Use untreated wood only - What should be assessed when preparing for shoring activities? - Site topography and weather conditions - Material availability only - Aesthetic appeal of materials - Number of workers - What is the primary use of formwork in construction? - To provide support for walls and columns - To build temporary scaffolding - To create molds for concrete pouring - To insulate the building - What is an essential requirement for formwork installation? - Ensuring the surface is level and clean - Immediate pouring of concrete - Installing insulation - Ignoring alignment checks - What should be done after removing formwork components? - Discard all components - Clean and store components safely for reuse - Leave components on site - Burn the used components - What is critical when assembling formwork for suspended slabs? - Ensuring proper clearance and safety - Ignoring structural measurements - Using only wood for formwork - Pouring concrete immediately - Which material is commonly used for formwork in construction? - Aluminum - Steel - Timber - Plastic - When dismantling formwork, what is the proper sequence? - Remove the formwork supports first - Start with the central components - Dismantle from top to bottom - Remove the concrete before the formwork - What should be done if formwork components are damaged during installation? - Continue installation - Replace or repair the damaged components - Ignore the damage if it is minor - Remove all formwork Answer Key 1.c; 16.b. 2.b; 17.a. 3.b; 18.b. 4.c; 19.a. 5.b; 20.a. 6.b; 21.b. 7.c; 22.a. 8.a; 23.a. 9.b; 24.c. 10.b; 25.a. 11.b; 26.b. 12.a; 27.a. 13.c; 28.c. 14.d; 29.c. 15.c; 30.b. | 1. What is the first step in constructing formwork for stairs and ramps? a) Place, fix, and brace landing bearers b) Calculate material quantity requirements c) Check footings for accuracy d) Erect and brace stairs formwork 2. What is the purpose of calculating the rise, going, and pitch of stairs? a) To determine the cost of materials b) To provide the location of landings, stringers, treads, and posts c) To calculate the height of the building d) To ensure the proper alignment of the structure 3. Which material is commonly used in the construction of masonry steps? a) Steel b) Concrete c) Bricks or Blocks d) Wood 4. What is the purpose of bracing risers at mid-span during stair formwork installation? a) To prevent deflection under load of wet concrete b) To increase the width of the stairs c) To enhance the aesthetic appeal of the stairs d) To speed up the installation process 5. What should be checked before starting the formwork construction for stairs? a) Load capacity of the materials b) Design and geometry of the stairs c) Weather conditions d) All of the above 6. What is the main factor in determining the correct design for earthquake-resistant buildings? a) Type of foundation used b) Type of reinforcement used c) Building configuration, material properties, and ductility d) Building height 7. What type of foundation is commonly used for large buildings with deep foundations? a) Pile foundations b) Slab foundations c) Raft foundations d) Combined foundations 8. Which of the following is a key component of structural analysis for high-rise buildings? a) Calculating elevator capacity b) Analyzing load performance on building structures c) Estimating material costs d) Selecting building decoration 9. What is one of the main safety concerns during the construction of formwork? a) Ensuring proper curing of concrete b) Ensuring structural stability and integrity c) Minimizing the use of expensive materials d) Managing the waste materials 10. What is the role of damp coursing in high-rise construction? a) To improve energy efficiency b) To prevent moisture from rising through the foundation c) To provide structural support to the building d) To reduce the amount of reinforcement needed 11. What is the primary goal when selecting elevator types for a building? a) Minimizing installation costs b) Reducing maintenance time c) Maximizing speed d) Meeting the building's height and usage needs 12. Which of the following factors impacts elevator and escalator load capacity? a) Building design and expected passenger volume b) Speed of travel c) Material selection d) None of the above 13. What is a common feature of all seismic-resistant buildings? a) Use of flexible building materials b) High-rise structure c) Energy-efficient lighting d) Structural reinforcements 14. What does the term "ductility" refer to in seismic-resistant construction? a) The building's resistance to fire b) The ability of materials to withstand stress without breaking c) The ability of materials to absorb water d) The strength of the foundation 15. What should be included in a pile foundation drawing? a) The depth of the foundation b) The number of floors in the building c) The material of the building frame d) The color of the exterior 16. Which of the following is an essential component of a masonry staircase? a) Steel reinforcement b) Mortar for joints c) Precast concrete treads d) Wooden steps 17. What is a significant aspect to consider when preparing a drawing for an earthquake-resistant building? a) The location of elevators b) The choice of seismic zones and reinforcement techniques c) The number of windows d) The type of roofing materials 18. What is a key advantage of a framed structure building? a) Cost efficiency b) Ease of installation c) Flexibility in design d) Reduced maintenance 19. What is the purpose of using a smart control system in elevators? a) To reduce the cost of installation b) To improve traffic handling efficiency c) To increase the speed of travel d) To enhance the aesthetic appeal of the elevator 20. What is the importance of calculating the material quantity for stairs and ramps? a) To ensure the structure's durability b) To optimize material usage and cost c) To meet aesthetic requirements d) To speed up the construction process 21. Which material is commonly used for formwork in stair construction? a) Timber b) Steel c) Aluminum d) Both a and b 22. What should be done if formwork components are found to be damaged after stripping? a) Reuse them without modification b) Discard them safely c) Repair and reuse them d) Send them for recycling 23. What is the primary concern when planning the construction of a large, high-rise building? a) Aesthetic design b) Structural stability c) Elevator speed d) Cost reduction 24. What is one of the first steps in constructing a large building foundation? a) Determining the building height b) Calculating the load distribution c) Preparing the foundation plan view d) Selecting the type of elevator system 25. What does a seismic zone map show? a) The types of construction materials used in the area b) The expected earthquake frequency and magnitude in the area c) The height of the buildings d) The number of floors in the buildings 26. Which of the following is considered a key factor for the performance of elevators? a) Noise level b) Travel speed c) Color of the elevator doors d) Elevation of the building 27. What is the main function of damp coursing in construction? a) To prevent water ingress through the foundation b) To enhance building aesthetics c) To support the load of the structure d) To increase the building height 28. What type of reinforcement is typically used in seismic-resistant buildings? a) Horizontal reinforcement b) Vertical reinforcement c) Both horizontal and vertical reinforcement d) No reinforcement 29. What should be considered when selecting materials for an earthquake-resistant building? a) Availability and cost b) Flexibility and energy dissipation properties c) Material color d) All of the above 30. What is the purpose of calculating the load capacity of an elevator? a) To determine the number of elevators needed b) To ensure the elevator can handle peak traffic c) To reduce energy consumption d) To select the correct design for the building Answer Key 1.b; 16.b. 2.b; 17.b. 3.c; 18.c. 4.a; 19.b. 5.d; 20.b. 6.c; 21.d. 7.a; 22.b. 8.b; 23.b. 9.b; 24.c. 10.b; 25.b. 11.d; 26.b. 12.a; 27.a. 13.d; 28.c. 14.b; 29.b. 15.a; 30.b. | 1. What is the primary goal of sound insulation in buildings? A) To improve aesthetics B) To reduce noise transmission C) To enhance thermal insulation D) To strengthen the structural integrity 2. Which material is commonly used for soundproofing in buildings? A) Concrete B) Glass C) Acoustic panels D) Wood 3. What is the key consideration when selecting insulation materials for ceilings? A) Aesthetic appearance B) Thermal and soundproofing performance C) Cost of materials only D) Durability of materials 4. What is the first step in the installation of sound insulation in buildings? A) Measuring the insulation material B) Preparing the work area C) Selecting tools and equipment D) Risk assessment 5. Which factor is important for ceiling insulation installation? A) Weather conditions only B) Minimizing noise and energy loss C) Aesthetic value D) Choosing the cheapest material 6. What is the purpose of using dust-suppression procedures during ceiling insulation? A) To increase the speed of work B) To minimize health risks in the work area C) To reduce the cost of materials D) To improve the installation aesthetics 7. In an auditorium, what is the primary role of sound-absorbing panels? A) To reduce the cost of construction B) To increase the volume of sound C) To control the acoustics and reduce unwanted noise D) To enhance the visual appeal of the space 8. What factor is NOT typically assessed when planning the acoustics of an auditorium? A) Room geometry B) Seating layout C) Impact of local weather conditions D) Surface materials 9. What type of fault is commonly found in construction during the inspection process? A) Electrical wiring faults B) Foundation cracks C) Plumbing leaks D) Poor ventilation 10. Which tool is used for inspecting cracks in construction? A) Steel plates B) Crack gauges C) Tape measures D) Wrenches 11. What is the first step in assessing construction faults in a building? A) Performing repairs B) Identifying signs of faults through visual inspection C) Installing safety measures D) Cleaning the work area 12. Which type of construction machinery is often used for large-scale excavation projects? A) Hydraulic excavators B) Laser-guided systems C) Automated robotic arms D) Tower cranes 13. What is the purpose of Building Information Modeling (BIM) in construction projects? A) To track the project's budget B) To provide 3D visualizations and improve coordination C) To predict weather conditions D) To increase worker productivity 14. What is an essential aspect of using construction machinery on a project site? A) Minimizing operator training B) Ensuring the machinery is properly maintained and serviced C) Using the cheapest available equipment D) Reducing the time spent on repairs 15. What is the primary concern when selecting machinery for a construction project? A) Machinery color B) Availability of operators C) Appearance of the machinery D) Project requirements and machinery needs 16. In the context of construction faults, what should be done after identifying a fault? A) Ignore the fault if it seems minor B) Implement repairs according to building specifications C) Report the fault to local authorities D) Postpone repairs until the project is finished 17. What is the main goal of assessing and repairing construction faults? A) To ensure that the building looks aesthetically pleasing B) To improve the safety, functionality, and structural integrity of the building C) To reduce labor costs D) To meet local building codes only 18. What is the purpose of using mounting brackets in an auditorium’s acoustic system? A) To secure sound-absorbing materials in place B) To decorate the walls C) To improve the visual design of the auditorium D) To adjust the temperature in the room 19. Which of the following is NOT an acoustic challenge in an auditorium? A) Room geometry B) Surface materials C) Audience behavior D) Volume of the space 20. What type of soundproofing material is typically used in an auditorium? A) Carpet tiles B) Sound-absorbing panels C) Paint D) Window film 21. When selecting machinery for construction, why is it important to consider training requirements for operators? A) To ensure efficiency and safety during the operation B) To minimize operational costs C) To increase the lifespan of machinery D) To meet aesthetic standards 22. What factor is NOT considered when evaluating ceiling insulation materials? A) Thermal performance B) Soundproofing requirements C) Aesthetic appearance D) Material durability 23. What is one of the main roles of personal protective equipment (PPE) in insulation installation? A) To improve work performance B) To prevent injuries and ensure safety C) To reduce costs D) To meet aesthetic standards 24. What is the purpose of air barriers in ceiling insulation? A) To enhance the visual appeal of ceilings B) To prevent the infiltration of air and improve energy efficiency C) To increase the soundproofing of a room D) To reduce installation time 25. What should be done after completing the installation of insulation in a building? A) Clean the work area and dispose of materials safely B) Re-check the work area for missing insulation C) Wait for inspection before cleaning D) Leave the site immediately 26. What is a critical aspect of ensuring the success of the construction machinery's operation? A) Selection of the cheapest machinery B) Correct setup, operation, and maintenance of machinery C) Ignoring safety protocols to reduce delays D) Only focusing on the appearance of the machinery 27. What should be assessed when identifying faults in construction? A) Worker experience B) The material cost C) The impact on structural integrity, safety, and functionality D) The aesthetic impact on the building 28. What type of faults might affect the roof of a building? A) Cracks in the foundation B) Leaks and uneven surfaces C) Electrical wiring faults D) Plumbing blockages 29. In selecting insulation materials, what is the primary goal? A) To meet budget constraints B) To ensure safety and environmental compliance C) To achieve desired thermal and soundproofing performance D) To minimize installation time 30. What is the purpose of a soundproofing audit in a building project? A) To reduce costs B) To identify and address acoustic challenges C) To ensure legal compliance D) To enhance energy efficiency Answer Key 1.b; 16.b. 2.c; 17.b. 3.b; 18.a. 4.d; 19.c. 5.b; 20.b. 6.b; 21.a. 7.c; 22.c. 8.c; 23.b. 9.b; 24.b. 10.b; 25.a. 11.b; 26.b. 12.a; 27.c. 13.b; 28.b. 14.b; 29.c. 15.d; 30.b. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| - What are the main environmental impacts of building construction? - Give an example of an eco-friendly building material. - What is the purpose of passive solar design in energy-efficient buildings? - What is one water-saving fixture commonly used in sustainable construction? - What should be checked before starting the building layout process? - How do you set a right angle during the layout process? - Why is a termite barrier important in construction? - What is a common material used for waterproofing? - What is scaffolding used for in construction? - What is formwork used for in concrete construction? | - What is the significance of calculating the rise, going, and pitch of stairs during formwork construction? - What type of foundation is typically used for deep foundation systems? - How does ductility contribute to earthquake-resistant building design? - What factors should be considered when selecting materials for earthquake-resistant construction? - How are risers braced at mid-span in stair formwork to prevent deflection? - What is the purpose of checking the accuracy of footings during the construction of stairs and ramps? - How does damp coursing help in preventing damage in buildings? - What is the key function of an elevator's performance metrics in a building? - How does the planning of structural elements differ in large, high-rise buildings compared to smaller structures? - What is the role of termite barriers in construction? - What factors should be assessed when preparing a foundation drawing for a building? - Why is it essential to evaluate structural function and requirements for temporary structural elements? - How does structural analysis influence the planning process for complex building construction? - What are the key components involved in constructing masonry steps and stairs? - How are elevators and escalators selected based on a building's design and intended use? | 1. What is the purpose of risk assessment in sound insulation installation? 2. What is the main factor to consider when selecting ceiling insulation materials? 3. What type of sound is typically targeted in soundproofing? 4. What safety equipment is essential when working with insulation materials? 5. What is the main purpose of sound-absorbing panels in an auditorium? 6. Which type of fault is commonly identified during construction inspections? 7. What is the purpose of Building Information Modeling (BIM) in construction projects? 8. What is one of the primary goals when selecting machinery for construction? 9. What is the first step in assessing construction faults in a building? 10. What material is typically used for ceiling insulation? 11. What is the main role of air barriers in ceiling insulation? 12. What is the main concern when selecting acoustic materials for an auditorium? 13. After installing insulation, what should be done to ensure safety and compliance? 14. What is one key consideration when choosing construction machinery? |

- Assessment Tasks

| Formaive Assessment (Part A) | Formaive Assessment (Part B) | Formaive Assessment (Part C) |
| --- | --- | --- |
| Task No1. Layout a single storey residential buidling on ground from the given drawing. Performance Criteria: - Obtain relevant drawings for lay outing. - Calculate material quantity requirements in accordance with Drawings. - Install pegs at corers of the site where layout is to be done. - Stretch string lines between the pegs to mark site boundaries. - Install pegs at corners of building by measuring from site boundaries or other permanent structure. - Stetch string lines between the pegs to mark boundary lines of the building. - Layout other elements including rooms, hall, washrooms and stair case in the building boundaries. - Check for angles between the boundary lines using trigonometric principals. OR Task No1. Prepare Project Plan for construction of single story residential RCC framed building ensuring sustainability. (Grey structure only) Performance Criteria: - Obtain work instructions, including plans, specifications, quality requirements and operational details, and apply relevant information for planning and preparation. - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Mark the site boundary lines and building lines on ground and locate the positions of walls, columns and foundations. - Complete excavation for foundations to required level and dimensions in accordance with drawings. - Select suitable material for termite protection in accordance sustainability requirements. - Install termite barriers on foundation soil. - Select waterproofing systems, installation techniques and products for water exclusion - Check for water table, suspected water inclusion points and install water stops and hobs in required location in compliance with standards - Perform priming coat on prepared surface of structure - Apply and cure waterproofing membrane to primed surface of structure to correct thickness - Install formwork for foundation construction where required. - Construct foundations and other underground elements of the building. - Apply water proofing membrane at surfaces in contact with soil or suspected for water inclusion. - Install formwork for columns. - Pour concrete of columns. - Install formwork for beams, slabs and stairs. - Pour concrete of beams, slabs and stairs. - Construct walls from required material OR Task No1. Apply Water proofing treatment of a building component according to the structural requirement following SOPs. Conduct flood testing of installation. - Obtain, confirm and apply work instructions. - Calculate material quantity in accordance with plans, specifications and quality requirements. - Select waterproofing systems and products for water exclusion - Check range of waterproofing materials and waterproofing installation technique - Determine termination detailing. - Check basement, water tank and wet area fitment or fixtures for specific measurements and set out in accordance with drawings and specifications. - Check substrate for soundness of fit, curing compounds, moisture content and other contaminants, and reported or remedied as required. - Check flooring installation levels and falls to waste outlets for conformity to specification. - Prepare surface of structure to be waterproofed to manufacturers' specification, including fixings - Install water stops and hobs in required location in compliance with standards - Perform priming coat on prepared surface of structure - Apply and cure waterproofing membrane to primed surface of structure to correct thickness - Apply appropriate bond breakers and fillets as required - Conduct flood testing of installation if required. Activity 2: Set out a single flight stair (4 steps with brickwork base and landing) with 6“riser, 10“ tread , step width 4‘ by marking steps on side wall and erect formwork for pre marked single flight. (1.5 hour) . Performance Criteria: - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify design of stairs and method of joining formwork from job drawings and specifications - Determine rise, going and pitch of stairs from plans - Set out location of footings to layout of designed stairs from plans, specifications - Place, fix and brace landing bearers and joists. - Check footings for accuracy to requirements of plans and specifications. - Erect and brace stairs formwork soffit - Brace risers at mid-span to prevent deflection under load of wet concrete. - Secure formwork tie bolts to plans and specifications to maintain stair width where specified - Remove formwork and bracing/strutting support sequentially and safely. - Clear work area by disposing of materials (reused or recycled) | Task No1. Plan a high rise and complex earthquake resistant building Performance Criteria: - Obtain plans and specifications - Identify building configuration like lightness standards, symmetry, projections and size of building - Identify building material as per specifications - Identify horizontal reinforcement at lintel, plinth, gable and roof level - Identify vertical reinforcement as per specification - Identify main structural principles that apply to erection of large, high rise and complex structures - Analyze performance characteristics of structural elements, including materials identified in building plan, to planning of construction work. - Identify requirements for retaining walls in conjunction with related industry professionals and apply to planning process. - Plan damp coursing and provision of termite barriers and other relevant techniques in accordance with codes, standards and industry practice Task No2. Determine parameters for stairs, elevators and escalators Performance Criteria: - Get the dimensions of stair well form drawings. - Select a proper stair orientation suitable for the location. - Calculate rise and fall stairs and width of flight as per codes and workplace requirements. - Identify specific needs of building, such as number of floors, expected traffic patterns, peak usage times, and population density. - Identify space requirements, cost, maintenance needs, and energy efficiency of elevator - Determine load capacity for elevators and escalators based on expected passenger volumes and traffic patterns. - Select elevator a suitable elevator or escalator type based on building’s height, design, and intended use. - Calculate size and number of elevators or escalators required to handle peak traffic efficiently, using industry-standard methods like handling capacity and interval calculations. - Ensure performance metrics, including acceleration, deceleration, and stopping accuracy, for smooth and safe operation. - Identify impact of speed on energy consumption, safety, and passenger experience Evaluate integration of smart technologies, such as destination control systems and energy-saving modes. - Evaluate integration of smart technologies, such as destination control systems and energy-saving modes. OR Activity 1: Calculate material quantity requirements in provided in activity 1. Construct single flight stair with base as per specifications and requirements (4 steps with brickwork base and landing. Performance Criteria: . - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Make ready materials for work application - Prepare mortar mix to required conformity and specifications. - Construct base brickwork to specifications and requirements of standards and codes. - Construct square steps, level, and plumb and lay to specified bond. - Develop design and profile of steps and aligned to bond and plumb to specifications. - Carry out joints according to work specifications. - Set out brickwork/block work laid to line, with gauge and complete to work specifications. - Clean brick/block faces free of mortar. | Task No.1. Asses and suggest repair for faults of building. Performance Criteria: - Identify signs of faults, such as cracks, settlement, leaks, and uneven surfaces, through visual inspection and diagnostic tools. - Perform detailed inspection of building, covering all major components, including foundations, walls, roofs, floors, and structural elements. - Note the faults on a piece of paper. Task No2. Select machinery for construction of a residential building. Performance Criteria: - Identify modern construction technologies, including Building Information Modeling (BIM), 3D printing, drones, automated construction equipment, and prefabrication. - Analyze specific needs of a construction project, including scale, complexity, and unique challenges. - Evaluate types of advanced construction machinery, such as automated robotic arms, tower cranes, hydraulic excavators, tunnel boring machines (TBMs), and laser-guided systems. - Develop detailed plan for integrating advanced technology and machinery into construction process. - Enlist steps required for setup, operation, and maintenance of machinery. - Task No1. Install sound insulation safely as per requirements.Evaluate effectiveness of different acoustic materials, such as sound-absorbing panels, diffusers, and bass traps, in controlling sound within the auditorium. Performance Criteria: - Assess risk assessment to identify existing risks and hazards in work area, including electrical risks and hazards. - Confirm insulating material and insulation requirements in accordance with work specifications. - Adopt personal protective equipment (PPE) and clothing according to organizational policies and procedures. - Determine associated material and organize ready for use according to quality requirements and work plans and specifications. - Measure insulation material accurately - Install insulation using approved processes and handling techniques - Install insulation safely without damage or distortion of surrounding environment, electrical and other services and in a manner that maximizes safety of self and others. - Conduct final inspections to ensure installed ceiling insulation conforms to job and manufacturer specifications. - Clean work area and disposed of materials, reused or recycled according to organizational, safety and environmental requirements. - Identify potential acoustic challenges related to auditorium’s shape, size, and construction materials. - Evaluate effectiveness of different acoustic materials, such as sound-absorbing panels, diffusers, and bass traps, in controlling sound within auditorium |