Course / Subject Name NQ-Civil-273-Construction techniques

- Assessment Tasks (Summative Assessment)

Activity 1: Prepare a document for project plan of layout for a single-story 3 marla residential building ensuring sustainability as per standards. (1 hour) Performance Criteria: - Set sustainability goals. - Create a project timeline. - Assign roles to team members. - Identify risks and create solutions. - Ensure compliance with regulations. Activity 2: Perform layout for construction of a single-story residential building in college ground ensuring sustainability as per standards. (3 hour) Performance Criteria: 1. Set up the construction site by clearing debris and marking boundaries. 2. Select tools and material required for layout. 3. Use Auto Level to establish accurate reference points and gridlines. 4. Stake out the corners of the building and key reference points for alignment. 5. Measure the dimensions of the building footprint on the ground. 6. Mark the locations of walls, doors, windows, and structural elements. 7. Mark the main Entrance and secondary exits. 8. Measure the dimensions to verify the layout is drawn to the correct sizes. 9. Mark areas for landscaping, driveways, and walkways. 10. Identify and enlist any non-compliance issues, to meet code standards OR Activity 1: Set out a single flight stair (4 steps with brickwork base and landing) with 6“riser, 10“ tread , step width 4‘ by marking steps on side wall and erect formwork for pre marked single flight. (1.5 hour) . Performance Criteria: - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify design of stairs and method of joining formwork from job drawings and specifications - Determine rise, going and pitch of stairs from plans - Set out location of footings to layout of designed stairs from plans, specifications - Place, fix and brace landing bearers and joists. - Check footings for accuracy to requirements of plans and specifications. - Erect and brace stairs formwork soffit - Brace risers at mid-span to prevent deflection under load of wet concrete. - Secure formwork tie bolts to plans and specifications to maintain stair width where specified - Remove formwork and bracing/strutting support sequentially and safely. - Clear work area by disposing of materials (reused or recycled) Activity 2: Calculate material quantity requirements in provided in activity 1. Construct single flight stair with base as per specifications and requirements (4 steps with brickwork base and landing).(2 hour) Performance Criteria: . - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Make ready materials for work application - Prepare mortar mix to required conformity and specifications. - Construct base brickwork to specifications and requirements of standards and codes. - Construct square steps, level, and plumb and lay to specified bond. - Develop design and profile of steps and aligned to bond and plumb to specifications. - Carry out joints according to work specifications. - Set out brickwork/block work laid to line, with gauge and complete to work specifications. - Clean brick/block faces free of mortar. Activity 3: Identify earthquake resistant building material and techniques for a small frame structure building. (30 minutes) Performance Criteria: - Identify building configuration like lightness standards, symmetry, projections and size of building - Enlist earthquake resistant building material - Enlist earthquake resistant building techniques