Course / Subject Name -NQ-Civil- 153 Civil Engineering Materials & Building Construction

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Lu 1-6 - What is the acceptable range of water absorption for a good quality brick? - 10-20% - 5-10% - 2-4% - 0-2%. - First class bricks shall be of _______ and _______. - Rectangular shape, uniform in size - square shape, long in size - circular and round in shape - none of these - Which of the following tests measures the ability of concrete blocks to resist wear due to traffic? - Abrasion resistance test - Slump test - Water absorption test - Tensile strength test - Stone classified as ------- where main constituent is silica ? - Siliceous stone - Argillaceous stone - Calcareous stone - All - Stone classified as ------- where main constituent is lime ? - Siliceous stone - Argillaceous stone - Calcareous stone - None - Stone classified as ------- where main constituent is clay ? - Siliceous stone - Argillaceous stone - Calcareous stone - All - Granite derived from-----------rock. - Sedimentry - Ignious - Metamorpic - All - Lime stone & sand stone derived from-----------rock. - Sedimentry - Ignious - Metamorpic - All - Marbel stone derived from-----------rock. - Sedimentry - Ignious - Metamorpic - All - ------ stone is strong and durable - Lime - Garnite - Both - None - ------ Suitable method of quarrying hard stone at large scale is. - Dressing - Burning - Blasting - Wedging - For the construction of retaining walls, a______ Stone must be used. - Soft - Hard - Heavy - None of these - Stones are dressed to give them a defined and------- shape. - Smooth - irregular - regular - all of these - Aggreagtes are -----------material. - Inert - Chemically non-reactive - Chemically reactive - Both a & b - Sea sand containing salts should not be used in - RCC - PCC - Both - None - ________aggregates contain different size practical in suitable Proportion - Poorly garded - Uni graded - Well garded - All - Size of course aggragte - 4.75mm - below 4.75mm - 6.75mm - above 4.75mm - OPC stand for - Ordinary Pakistani Cement - Ordinary Portland cement - Ordinary Pacca Cement - none of these - The maximum percentage of ingredient in cement is that of - Magnesium - Alumina - Silica - Lime - Initial setting time cement is aroud - 30-45 minutes - 90-120 minutes - 03 Hours - All - Cement plaster should be cured at least for -------days - 03 - 10 - 28 - None - Excesive water/ cement ratio -------the strength & durability of concrte - Increase - Decrese - No effect - None - Compaction factor for good workability of concrete is - 0.3 - 4.85 - 0.15 - 0.95 - The slump taste of concrete is used to measure its. - Workability - Tensile and compressive strength - Impact value - Homogeneity - Maximum height from wich the fresh concete can be droped - 1.5 m - 3 m - 3.5 - all - The reason of seggregation in concrte - Improper mixing - Concrete dropped for higher height - Excessive vibration - all - The reason of bleeding in concrte - Improper mixing - Excessive W/C ration - Gaps in shuttering/ farmwork - All - Following are non distructive test applicabel on hrdended concrete - Schem hammer/ Rebound hammer test - Ultrasound scans-UV test - Tensile test - Both (a) & (b) - Following distructive test applicabel on hrdended concrete - Comprseeion - Bond test - Tensile test - All 1-b, 2-a, 3-a, 4-a, 5-c, 6-b, 7-b,8-a, 9-c, 10-b, 11-c,12-c,13-c 14-d, 15-a, 16-c, 17-d, 18-b,19-d,20-a, 21-b, 22-c, 23-d, 24-a,25-a 26- d, 27- d, 28-d, 29-d | Lu 7-13 - Seasoning is always apply on ____________. - Timber - Steel - Aluminum - Concrete - Seasoning is the process of ___________ form timber. - Burning - Burning steel - Removing water - Adding water - Which of these methods is used for artificially seasoning timber?. - Air seasoning - Kiln seasoning - Natural seasoning - Water seasoning - What is the main disadvantage of air seasoning timber? - It is very fast - It causes the timber to become brittle - It is time-consuming and dependent on weather condition - It is highly energy-consuming - The foundation that is used when ground is un-stabel. - Mat footing - Combined footing - Strap footing - None - Grillage foundation is callified as - Deep foundation - Shallow foundation - well foundation - all of the above - Foundation whose depth is equal to its width, is called - Shallow - Box - Deep - Raft - A raft foundation is known as - Mat foundation - Cantilever foundation - Pile foundation - All of above - Pile foundation is generally used when the soil is ________ - Compressable - Water logged - Made up type - all of the above - The part of building above plinth level is called ____________. - Foundation - Sub-structure - Super structure - Ground level - The part of building below plinth level is called ____________. - Foundation - Sub-structure - Super structure - Ground level - Foundation of a building can be termed as __________. - Plinth - Lower part - Sub soil - Sub structure - The modified form of English bond is called _______ bond - Header - Flemish - Raking - Duch - In which bond, brick is laid with its length in the direction of a wall __________ - Header - Footer - Stretcher - Flemish - A horizontal layer of bricks laid in mortar is known as: - Stretcher - Header - Closer - Course - The construction of a Temporary structure required to support as unsafe structure is called _______ - Shoring - Underpinning - Scaffoding - Jacking - The damp proof course is measured in __________ - Cu.ft - Sq.ft - Cu.m - Psi - Which is the flexable material used for D.P.C. - Bitumen sheet - Ply wood - Cement mortar - Cement concrete - Which type of lintel is commonly use where span is large __________. - Brick - Steel - Wooden - R.C.C. - Clear horizontal distance between two supports is called ________ - Opening - Spandral - crown - Span - The intermediate support of an arch is known as _________ - Pier - Crown - Both - None - The wedge shaped stone placed at the crown of an arch is called ______ - Skewback - Extrados - Intrados - Key stone/ cap stone - The perpendicular distance between intrados and extrados of an arch is called ______ - Depth of an arch - Width of and arch - Thickness of arch - Pitch of arch. - The wedge shaped blocks used to construct an arch are called . - crown - voussoirs - rise - none of these. - The lower portion of an arch between the skew back and crown is - Depth - Rise - Haunch - Intrados 1-a, 2-c, 3-b, 4-c, 5-a, 6-b, 7-a, 8-a, 9-b, 10-c , 11-b, 12-d, 13-b, 14-c, 15-d, 16-a, 17-b, 18-a, 19-d, 20-d, 21-a, 22-d, 23-a.24-b,25-c | Lu 14-19 - Following ------ tiles are used in wet area like kitchen & bath. - Ceramic - Porcilien - Marbel - UPVc - Following ------ tiles are used in common flooring. - Ceramic - Porcilien - Granite - UPVc - Following ------ floor type used in library building in cold area. - Marbel - Rubber - Granite - UPVc - Skirting is ---------tile/ marbel piece to protect the walls from the damp. - Vertical - Horizontal - Both - None - Purpose of window is ------- . - Enter in to the room - Ventilation - Sun light - Baoth b & c - Commonly used sill level height for door is ------- . - 6 inches - 9 inches - 12 inches - None - Commonly used sill level height for ventilator is ------- . - 06 ft - 01ft - 06 inches - None - An inclined piece of wood fixed in a frame work of door is------ - Louver - Sash bar - Both - None - The vertical members of the frame of door are called, - Shutter - Posts - Styles - Sills - The projections of the head of a door or window frame embedded in the masonry are called - post - Frame - horns - style - Plum bob is used for ---------- - Horizontal level - Vertical level - Inclined level - All - Top level of door and windows is called - sill level - Lintel level - both - None - In order to obtain uniform thickness and true surface of plaster------- 7.5cm wide and of required thickness of the plaster are generally fixed at required intervals. - wooden screeds - Trowel - both - None - First layer of plastering is known as. - Back ground - Rendering/Rough cours - Carzing - Pointing - ______ gives an attractive appearance to the structure and is generally used for superior brick work. - pointing - Painting - graffito - all of these - Following are prerequiset of plastering and room - Electric conduting is done - Plumbing lines are placed, if any - Curing is done - all of these - A sloping roof having slope in -----is called shed roof. - one direction - Two direction - Foure directiom - all of these - A sloping roof having slope in four directions is called -------roof. - Shed - Hipped - Gable - all of these - Shelled roof are used frequently these days for covering - Big structure - Small structure - Both - None - First coat of paint/ distember known as - Under coat - Upper coat - Render coat - Priming coat - Priming and filling of surface is done --------------- . - After two coat - After sigle coat - Before start the paint work - After priming coat - A window fixed at vertical triangular portion of wall under sloppy roof is called,After two coat - Gable window - Bay window - Dormer window - Sky light - Purlins are members which are laid horizontally to support _______ when span of the truss is large. - queen closer - common rafters - king closer - all of these - Thickness of single layer of plaster is kept mm thick. - 5 - 12 - 20 - 24 - Oil paints consist essentially of _______, _______ and _______ - base, vehicle, pigment - base, thiner, drier - base, filler, resins - none of these 1-a, 2-b, 3-b, 4-a, 5-d, 6-d, 7-a,8-a, 9-b, 10-c, 11-b,12-b,13-a 14-b, 15-c, 16-d, 17-a, 18-b,19-a,20-d, 21-c, 22-c, 23-d,24-b,25-d |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Lu 1-6 - What is the main function of brick? - Enumerate the ingredients used in manufacturing of brick and concrete block. - Write the classification of bricks. - What are over burnt bricks? - Write the formula for water absorption of bricks. - State the function of frog of brick? - What is under burnt brick? - Enlist the tests on standard brick. - Define the term brick, tile, tough tile and concrete block. - Enumerate the types of tiles. - Give four specifications for first class bricks - Write down the available sizes of brick, tile, tough tile and concrete block available in the market. - Enlist the required characteristics of dimension tolerance test for a first class brick at construction site. - Enlist the test required for a tough tile. - Enlist the types of rocks from where stone derived. - Enlist the method of dressing of stone. - Enlist the types stones - Enlist the types of rocks based on the geological classification. - Enlist the types of stones based on the scientific classification. - Why we need to polish the stone. - Enlist the main classification of aggregates. - Enlist the type of test on coarse and fine aggregates - Which type of sand is best for plaster work and concrete work? - Enlist the four quality of sand - Define the binder material and cement. - Enlist the type of cement - What is the purpose of adding gypsum in to cement? - Differentiate between initial setting and final setting of cement. - Where rapid hardening cement is suitable? - State the uses of white Portland cement. - Enlist the test need to perform on cement to check the quality. - Enlist International slandered used for cement and mortar - Enlist the type of mortar. - State the use of mortar. - Enlist the common used cement mortar ratio for masonry work and plastering. - What is the effect of adding lime to cement mortar? - State the functions of sand in mortar? - Enlist the operation of concrete. - Enlist the classification of concrete based on the purposes. - State the need of air entrained concrete - Define the term “batching”. - Enumerate different methods of compaction of concrete. - Difference b/w nominal mix and concrete mix design. - Define the curing of concrete - Enlist the properties of fresh concrete. | Lu 7-13 - Enlist the methods of seasoning - Write down the arrangement process for the timber for air seasoning - State the method of kiln seasoning for timber? - State the best method for seasoning and why? - Define the basic purpose of providing a building foundation? - Enlist the type of foundations - Define the shallow foundation. - Enlist the types of shallow foundation. - What is the difference between shallow and deep foundations - Define the raft foundation. - What do you understand by the term “pile foundation”? - Enlist the types of brick masonry bonds - Enlist the type of stone and block masonry - State the dry rubble masonry - Why the bricks should be laid with their frog upward - Define queen closer. - What is Queen Brick - State the purpose of DPC - Write the methods of preventing Dampness. - Why the D.P.C is not provided with in door or veranda opening? - Define the vertical D.P.C. - State the purpose of plinth beam and grade beam - State the objective of lintel - Enlist the types of lintel based on the material used - Differentiate b/w lintel & arch - Enlist the types of arches | Lu 14-19 - State the function of floor - Enlist the basic component of floor - Enlist the type of floor w.r.t building level. - State the three main factors that affect the choice of flooring material - Enlist the common thickness of marble and tile thickness for flooring - Define the terrazzo topping - State the function of door and windows - Define the purpose of ventilator - Enlist the type of door w.r.t material used. - Enlist the types of window w.r.t material. - State the function of door frame - State the standard sizes of doors, windows and ventilators for a house - Define the bay window. - Define the surface finish - Differentiate b/w pointing & plastering - Enlist the tools of pointing - Enlist the types of pointing - Enlist the requirement of plastering - State the Stucco plastering - State the purpose of roof - Enlist the classification of roof - Difference b/w pitch/ sloppy and flat roof - Define Graffito & rock wall finishes - Difference b/w white wash and paint - State the process of prepare solution for white washing / distempering - Enlist the types of paints used for wood and metal surfaces - Enlist the step to require the paint of old surface |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU 1-6 Activities - Perform Dimensional test, water absorption, Efflorescence compressive strength test on given sample of brick. - Select sample - Perform dimension tolerance test for bricks. - Perform water absorption test for bricks. - Perform efflorescence test for bricks. - Perform compressive strength test for bricks - Prepare and submit report. OR - Perform the sieve analysis (gradation test) of course aggregates and prepare gradation curve. - Identify aggregates for testing. - Prepare sample for testing. - Identify tools and equipment for tests. - Classify aggregates by sieve analysis. - Prepare and submit report. OR - Perform fineness test,initial setting time test on the cement and comoressive stregth test on cement mortor. - Identify cement for testing. - Prepare sample for testing of cement. - Identify tools and equipment for cement tests. - Set tools for fineness test. - Determine fineness of cement. - Set apparatus for standard consistency test of cement. - Determine standard consistency of cement paste. - Determine initial setting time of cement. - Perform compressive strength test - Prepare and submit report for quality of cement and mortor strength OR - Perform batching by weight to prepare concrete using local materials, check workability, placing ,compaction and curing at required site including filling of cube/ cylinder for testing. - Prepare ingredients for concreting. - Identify correct method of mixing - Prepare non-porous platform for hand mixing. - Select method of batching. - Batch and mix ingredients of concrete according to specified ratio - Select method of transportation of concrete according to nature of work. - Place / lay concrete in formwork. - Identify and select method of compacting concrete. - Finish concrete. - Cure concrete for specified duration or requirement. | Lu 7-13 Activities - Visit nearby industry/ yard to witness the seasoning process - Identify method of seasoning - Arrange timber stacks for air seasoning - Select kiln for timber seasoning - Arrange scantlings in kiln - Circulate hot air as per specification - Place logs as per specifications in water for water seasoning. - Place end electrodes at specified place of log for electric seasoning - Prepare chemical solution for chemical seasoning as per specifications OR - Build L- shaped bricks masonry wall 9” thick including plinth beam DPC at plinth level with stepped foundation of an appropriate size in English bond as per drawing. - Calculate material quantity in accordance with specifications. - Identify, select and check bricks/blocks for conformity with specifications. - Identify location and structural details of brickwork/ blockwork from plans. - Set out base brickwork construction, below and above floor to location, dimensions and specifications. - Lay DPC at plinth beam - Position and correctly fix wall ties to framework as per specifications. - Keep cavities clear of mortar droppings and bridging. - Build walls to gauge straight and true in plumb, line and level within standards tolerance. - Form control joints in accordance with locations on job drawings and specifications - Brush down brickwork/block work prior to drying to remove unwanted mortar and face is cleaned. - Remove excess mortar from brick/blockwork surfaces and clean cavities free of mortar and debris. - Clean work area, and waste disposed of, reused or recycled in accordance with work specifications - Use PPEs as per requirement OR - Build a wall over plinth beam and construct RCC lintel on a window opening as per the given drawing & specification. - Identify, select and check bricks/blocks for conformity with specifications. - Develop work platform in accordance with workplace requirements. - Identify place of lintel - Mark end bearings of lintel on both sides - Lay bed mortar - Place reinforcement / reinforcement bar with stirrups as per specification - Place form work - Prepare and place concrete Compact and cure before further masonry OR - Construct a segmental brick arch on window opening including erection of centering - Identify and mark place for arch - Prepare skewback for arch - Identify and select type of centering i. e. brick, wooden or metal - Fix wedges between props and rib for wooden centering - Prepare brick centering with dry bricks as per shape of arch - Perform masonry for brick arch Prepare a sand box with plug for striking | Lu 14-19 Activities - Perform terrazzo topping ½” thick in alternate panels after estimating the quantities of ingredients. - Identify area for flooring - Clear surface form debris. - Compact surface and level it. - Prepare subgrade, base / bedding course for flooring - Identify material for concrete floor - Identify tools required - Select and use PPE as per job requirements. - Prepare concrete floor as per specifications - Perform curing OR - Laying ceramic tiles (not greater than 12”x 12”) floor on the prepared bed as per given pattern & size which must require the cutting of tiles. - Identify marble / tile for flooring - Prepare base surface - Lay marble / tile and fill groves - Cure for finishing. OR - Fix a aluminium window in the given wall opening - Identify door / window for installation - Identify location for door / window fixing - Position door / window vertically in wall with poles - Check verticality with plumb bob OR - Perform struck pointing patchwork on old wall after scraping & cleaning the patches - Identify tools for plastering / pointing. - Prepare mortar for plastering / pointing - Prepare surface for plastering / pointing. - Apply rendering / floating coat. - Rack and prepare surface for pointing. - Identify tools for pointing - Select method of pointing - Perform pointing. - Cure surface. OR - Prepare steel surface and apply paint of specific color - Identify tools and surface for painting - Prepare metal surface for painting - Perform priming, stooping and filling of surface - Prepare paint as per specification - Apply paint on prepared surface - Remove stains for old surface before painting - Apply paint on old surface - Apply succeeding and final coats as per specification |