Course / Subject Name:	 NQ-Civil-263 Engineering Mechanics

- MCQs

| Formative Assessment (Part A) (LU1- LU4) | Formative Assessment (Part B) (LU5 – LU8) | Formative Assessment (Part C) (LU9 – LU11) |
| --- | --- | --- |
| - Center of plan geometrical figure is called. - Center - Centroid - Center of gravity - Central axis - Center of Solid geometrical figure is called. - Center - Centroid - Center of gravity - Central axis - Centroid of symmetrical figure lies at - Outside - Center - Left - Right - Centroid of a triangle having base 10cm and altitude 9cm form bottom will lie, - 3cm - 5cm - 6cm - 4.5cm - Centroid of semi-circle lies at a distance, from its base - 4R/2π - 2R/4π - 4R/3π - 1R/2π - Centroid of triangle having height “h” lies at a distance from its base - h/2 - h/3 - 2/3h - 3h2 - Centroid of equilateral triangle having side “s” from base - S/3sin 60o - S/3 - 2S/3 - S/3 sin 30o - Centroid of quarter circle lies at a distance of, - D/2 - 4R/3π - 3R/4π - 5R/3π - Measuring unit of moment of inertia in MKS system is - In4 - cm4 - m4 - ft4 - Moment of inertia of a hollow circle with external diameter “D” and internal diameter as “d” - γ(D4+d4 )/64 - π(D4 -d4 )/64 - π(D4 -d4 )/32 - γ(D4 -d4 )/64 - Moment of inertia of a square having side of 1cm - 1cm4 - 4cm4 - 1/10 cm4 - 1/12 cm4 - M.O.I of a rectangle having b=12cm and d= 1cm - 1cm4 - 4cm4 - 1/12 cm4 - 12 cm4 - M.O.I of a triangle having base “b” and height “h” about axis passing through its centroid - bh3 /12 - bh3 /36 - hb3 /12 - hb3 /12 - M.O.I of equilateral triangle of side “L” and height “h” about its base - Lh3 /12 - hL3 /36 - Lh3 /48 - Lh3 /36 - M.O.I of isosceles triangle having base 2cm and height 6cm about axis passing to CG and normal to base - 1cm4 - 4cm4 - 12 cm4 - 36 cm4 - Polar moment of inertia is equal to - bh3 /12 - πD4 /64 - Ixx+Iyy - Ixx/Iyy - Polar moment of inertia of a circle of diameter “D” is - πD4 /64 - πD3 /64 - πD4 /32 - πD3 /32 - Polar moment of inertia of a square of side “s” is - S3 /12 - S4 /12 - S3 /6 - S4 /6 - Positive bending moment shows - Hogging - Jogging - Sagging - Tearing - Shape of shear force diagram in case of cantilever beam having point load at its free end - Triangle - Rectangle - Circular - Parabolic - Maximum bending moment appearing at the fixed end of a cantilever beam - Zero - Positive - Negative - Sagging - Shape of BMD of simply supported beam having UDL at full span - Cubical - Parabolic - Rectangular - Triangular - Point of contra-flexure acts in the such beams - Cantilever - Simply-supported - Overhanging - Propped - Bending moment at the ends of a simply supported beam is - Maximum - Minimum - Zero - Negative - Shape of BMD of cantilever having a couple at free end - Rectangle - Triangle - Parabolic - Ellipse - Value of shear force at the free end of 3m long cantilever having UDL @100 N/m - Zero - 100 N - 200 N - 300 N - Value of BM at the free end of 3m long cantilever having UDL @100 N/m - Zero - 450 N-m - 300 N-m - 150 N-m - Value of vertical reaction at fixed end of a cantilever 3m long having UDL @100 N/m - Zero - 450 N-m - 300 N - 150 N - Resistance of material against deformation is called - Hardness - Toughness - Resilience - Modulus - Resistance of material against impact is called - Hardness - Toughness - Resilience - Modulus - Strain energy is also called - Potential energy - Kinetic energy - Resilience - Atomic energy - Load applied for hard material in case of Brinell hardness test is - 1000 kg - 2000 kg - 3000 kg - 4000 kg - Hardness test suitable for 0.15 mm thick metal plate - Brinell hardness test - Rockwell hardness test - Vicker’s hardness test - Flow test - Formula to determine Brinell hardness number is - B. H. N = 2P/πDD - D2 - d2 - B. H. N = P/πDD - D2 - d2 - B. H. N = 2P/πDD+ D2 - d2 - B. H. N = P/2πDD - D2 - d2 - Formula is used to determine Rockwell hardness number is - HR=E/e - HR= E+e - HR=E-e - None of these Total MCQs Part A (35) Answer Key: Part A 1. B; 2. C; 3. B; 4. A. 5. C; 6. C; 7. A; 8. B. 9. C; 10. B; 11. D; 12. A. 13. B; 14. C; 15. A; 16. C. 17. C; 18. D; 19. C; 20. B. 21. C; 22. B; 23. C; 24. C. 25. A; 26. A; 27. A; 28. C. 29. A; 30. B; 31. C; 32. C. 33. B; 34. A; 35. C. | - The property of material by virtue of which it disappears the deformation after the removal of load, - Plasticity - Elasticity - Rigidity - Toughness - The capacity to resist plastic deformation of a material is, - Elasticity - Plasticity - Brittleness - uctility - Force or load divided by cross sectional area - Stress - Strain - Pressure - Newton - Ratio of stress and strain in elastic limit - Young’s modulus - Bulk modulus - Section modulus - Shear modulus - Ratio of shear stress to shear strain in elastic limit, - Young’s modulus - Bulk modulus - Modulus of rigidity - Section modulus - Ratio of change in length to original length - Stress - Strain - Lateral strain - Volumetric strain - Ratio of change in diameter to original diameter - Stress - Strain - Lateral strain - Volumetric strain - Ratio of direct stress to volumetric strain - Young’s modulus - Bulk modulus - Shear modulus - Section modulus - Ratio of lateral strain to linear strain - Young’s modulus - Bulk modulus - Shear modulus - Poisson’s ratio - Ratio of shear stress to shear strain - Young’s modulus - Bulk modulus - Shear modulus - Poisson’s ratio - What is the primary function of a Universal Testing Machine (UTM)? - To measure the temperature of materials - To test the mechanical properties of materials - To calculate the cost of materials - To conduct chemical analysis of materials - Which of the following tests can be performed using a Universal Testing Machine - Tensile test - Compression test - Bending test - All of the above - In a UTM, the specimen is held securely during testing by: - A hydraulic system - Grips or jaws - A magnetic holder - Adhesive clamps - What does the load cell in a UTM measure? - Displacement of the specimen - The load or force applied to the specimen - The temperature of the specimen - The speed of the crosshead movement - In a Universal Testing Machine, what does the extensometer measure - The cross-sectional area of the specimen - The applied force on the specimen - The elongation or strain of the specimen - The ultimate tensile strength of the material - Bending stress at neutral axis of the beam is, - Maximum tensile - Maximum compressive - Zero - Negative - Strength of the beam is directly proportional to, - Section modulus - Bulk modulus - Shear modulus - All of these - Bending equation is - M/f=I/y=E/R - I/g=f/r=CB/e - M/I=f/y=E/R - None of these - Bending stress below the neutral axis of a cantilever beam is - Tensile - Compressive - Maximum - Zero - Section modulus of a circle is - 𝜋𝐷2 /24 - 𝜋𝐷3 /24 - 𝜋𝐷3 /32 - 𝜋𝐷4 /64 - Section modulus is equal to - I/y - M/I - Y/I - f/Y - Bending formula is - F=MZ - M=fZ - Z=Mf - M= f/Z - Average shear stress “q” is given by - Q/A - Q x A - A/Q - None of these - Maximum shear stress in case of rectangular section with respect to average shear stress - Equal - Half - Double - 1.5 - Shear stress at the extreme fibers of the beam section - Maximum - Minimum - Zero - 3/2 of average - Moment area method is used to determine; - Slope & deflection - S.F and B.M - Tensile stress - Compressive stress - Deflection at the free end of a cantilever beam having point load “W” at the free end is’ - 𝑊𝑙3 /3𝐸𝐼 - 𝑊𝑙2 /2𝐸𝐼 - 𝑊𝑙4 /3𝐸𝐼 - 𝑊𝑙3 /2𝐸𝐼 - Deflection at the free end of a cantilever beam having total UDL load “W” at the full span is - 𝑊𝑙3 /3𝐸𝐼 - 𝑊𝑙3 /8𝐸𝐼 - 𝑊𝑙3 /48𝐸𝐼 - 𝑊𝑙3 /16𝐸𝐼 - Deflection at mid-span of a simply supported beam having point load “W” at mid-span - 𝑊𝑙3 /3𝐸𝐼 - 𝑊𝑙3 /8𝐸𝐼 - 𝑊𝑙3 /48𝐸𝐼 - 𝑊𝑙3 /16𝐸𝐼 - Value of maximum deflection that acts at a point of cantilever - Fixed end - Free end - Middle - Both ends - Value of deflection at the end of a simply supported beam - Maximum - Minimum - Zero - Average - Resistance against deflection of beam - Hardness - Stiffness - Toughness - Brittleness - Deflection is directly proportional to - Load - Young’s modulus - Section modulus - Moment of inertia - Deflection in beams is inversely proportional to, - Load - Span - MOI - Both a and b - Maximum deflection at mid-span of a simply supported beam loaded with a total UDL “W” at full span - 𝑊𝑙3 /3𝐸𝐼 - 𝑊𝑙3 /48𝐸𝐼 - 5𝑊𝑙3 /384𝐸𝐼 - 𝑊𝑙 3 /16𝐸I Total MCQs Part B (35) Answer Key: Part B 1. B; 2. C; 3. A; 4. A. 5. C; 6. B; 7. C; 8. B. 9. D; 10. C; 11. B; 12. D. 13. B; 14. B; 15. C; 16. C. 17. D; 18. C; 19. B; 20. C. 21. A; 22. B; 23. A; 24. D. 25. C; 26. A; 27. A; 28. B. 29. C; 30. B; 31. C; 32. B. 33. A; 34. C; 35. C. | - A column having slenderness ratio less than 80 is treated as - Short column - Long column - Medium column - Weak column - A long column fails due to - Crushing - Buckling` - Slipping - binding - Equivalent length of column with both ends hinged - 0.707L - 0.5L - 2L - L - Equivalent length of column with both ends fixed - 0.707L - 0.5L - 2L - L - Equivalent length of column with one end fixed and another end hinged - 0.707L - 0.5L - 2L - L - Equivalent length of column with one end fixed and another end free - 0.707L - 0.5L - 2L - L - A strut is a - Compression member - Tension member - Flexural member - Torsion member - Radius of gyration is given by - I/y - J/r - √(𝑰/𝑨) - √(𝑖/𝑚) - The ratio of length of column to its least dimension - Radius of gyration - Slenderness ratio - Stress ratio - Least count - Rankine’s formula is used to calculate crippling load for such column - Short - Long - Both a & b - None - A triangular shape structure used as replacement of beam for long span - T-beam - Pre-stressed beam - Truss - Triangular beam - Tensile member of a truss is called - Tie - Node - Strut - Rafter - Ratio between rise and half span of a truss - Slope - Pitch - Lateral ratio - Buckling ratio - A line joining vertex of truss is called - Sky line - Ridge line - Peak line - Top line - Vertical distance between vertex and supports of a truss is called - Span - Rise - Pitch - Base - A perfect frame is judged with the following relation with number of members “n” and number of joints “j”. - n < 2j-4 - n < 2j-4 - n= 3j-3 - n= 2j-3 - A compressive member of truss is called - Tie - Strut - Node - Eave - Outer slopy members of truss is called - Rafter - Purlin - Strut - Tie - The plate used to join various member of truss - Base plate - Gusset plate - Steel plate - Double plate - The point where principal rafter meets the support is called - Node - Panel point - Heel - Ridges - Angle of repose taken for water, - 15o - 30o - 45o - Zero - The shape of pressure diagram facing a dam - Triangular - Trapezoidal - Rectangular - None of these - Specific mass of water in MKs system - 1000 N/m3 - 1000 kg/m3 - 9.81 kN/m3 - 981 N/m3 - Formula to calculate the horizontal pressure of water acting on a vertical side of dam - 𝑃 = 𝑤ℎ - 𝑃 = 𝑤ℎ/2 - 𝑃 = 𝑤/ℎ - 𝑃 = 2𝑤ℎ - Horizontal pressure acts at the vertical face of the dam at a height of retained material - h/2 - h/3 - 2h/3 - 0.707h - Resultant pressure should pass through the base of retaining wall safe against overturning - Middel third - 1/2 of the base - 3/2 of the base - None of these - Eccentricity of resultant pressure is limited up to of retaining wall safe from tension at its base - e > b/4 - e < b/3 - e < b/6 - e < 2b/3 - Condition for stability of retaining wall against sliding - 𝜇𝑤/𝑝 = 0.5 - 𝜇𝑤/𝑝 = 1 - 𝜇𝑤/𝑝 = 1.5 - 𝜇𝑤/𝑝 > 0.5 - Distance of point f application of resultant force acting on a retaining wall from the line of action of the weight of wall - 𝑦 = 𝑃/3𝑤 - 𝑦 = 𝑤ℎ/2𝑝 - y= P/W x h/3 - 𝑦 = 3𝑝ℎ/𝑤 - Maximum stress developed at the base of retaining wall - f=wb1+6eb - f=wb1-6eb - f=wb1+6be - f=2wb1+6eb Total MCQs Part C (30) Answer Key: Part C 1. A; 2. B; 3. D; 4. B. 5. A; 6. C; 7. A; 8. C. 9. B; 10. C; 11. C; 12. A. 13. B; 14. B; 15. B; 16. D. 17. B; 18. A; 19. B; 20. C. 21. D; 22. A; 23. B; 24. B. 25. B; 26. A; 27. C; 28. C. 29. C; 30. A. |

- Limited Response Questions

| Formative Assessment (Part A) (4) | Formative Assessment (Part B) (4) | Formative Assessment (Part C) (3) |
| --- | --- | --- |
| - Differentiate between centroid and center of gravity - Name the methods of determining the centroid - Define axis of reference and axis of symmetry - Describe the centroidal axis - Give the formula for calculating the CG of square and isosceles triangle - Give the formula for calculating the CG of semi-circle and quarter circle. - Give the formula for calculating the CG of composite figures. - Define Moment of inertia - State theorem of parallel and perpendicular axis of M.O.I - Define radius of gyration. - Write formula for M.O.I through the centroid of square and circle - State polar moment of inertia - Write formula for M.O.I through z-axis - Write formula for M.O.I for isosceles triangle parallel to the base and passing through centroid. - Write formula for M.O.I for isosceles triangle perpendicular to the base and passing through centroid. - Write formula for M.O.I for semi-circle and quarter circle about an axis parallel to the base and passing through centroid. - State shear force and bending moment - Name and sketch different types of supports - Name and sketch different types of beams - State the loads acting on beams - Write the equation of stability used while calculating reactions of beams - Describe the elastic curve of beams - Define the point of contra-flexure - Write formula for maximum shear force and bending moment of a cantilever beam having span “L” and load “W” at free end - Write formula for maximum shear force and bending moment of a simply supported beam having span “L” and load “W” at mid-point - Write formula for maximum shear force and bending moment of a simply supported beam having span “L” and loaded with UDL “w/unit length” over the entire span - Define the hardness of metals - Define toughness of metals - Name different test to check the hardness of metals - Write formula to determine Brinell hardness number - State the precautions to be adopted in Brinell hardness test - State the advantages of Rockwell hardness test - Write the formula to determine Rockwell hardness number - Give the limitation of Brinell hardness test - How tensile strength is determined with the help of BHN | - Define stress and strain - Name different types of stress and strain - Differentiate between linear and lateral strain - Define modulus of elasticity - State Hook’s law - Define shear modulus - State bulk modulus - Define Poisson’s ratio - Differentiate between elasticity and plasticity of a material - State factor of safety. - What is the primary functions of Universal Testing Machine? - What type of materials can be tested by UTM - What is the function of extensometer in UTM - Define yield stress - Define ultimate stress - Define bending stress - Differentiate between pure bending and simple bending - State assumptions of simple bending - State section modulus - State bending equation of beam - Write formula for section modulus of square and circular hollow section - Write the formula for the intensity of shear stress in the beam - Draw the shape of shear stress distribution in I-section - Define neutral axis of a loaded beam. - Give the relation between average and maximum shear stress in rectangular beam. - Define the deflection of beam - Name various methods of deflection calculation in beams - Define stiffness of the beam section - Give significance of deflection of beams - Where maximum deflection will act at cantilever beam loaded UDL at full span - Where maximum deflection will act at cantilever beam loaded with a point loaded at free end - Write the formula to calculate maximum deflection in a cantilever having a point load at mid span - Write the formula to calculate maximum deflection in a cantilever having UDL at half span from fixed end - Write the formula to calculate maximum deflection in a cantilever having span “l” and point load “W” at free end and a UDL at full span - Write the formula to calculate maximum deflection in a simply supported beam having span “l” and point load “W” at mid-span and a UDL at whole span. | - Differentiate between column and strut. - Differentiate between long column and short column - Differentiate between axial loaded and eccentric loaded column - Define slenderness ratio - Give effective length of column with various end conditions - State Euler’s formula for calculating ultimate load for column - State Rankine’s formula for calculating ultimate load for column - Give failure pattern of short columns - Give failure pattern of long column - What is safe applied load on the columns - Define the truss - Name four types of trusses - Name four parts of a truss - Name various method of truss analysis - Differentiate between statically determinate and indeterminate structurers - State the equation of equilibrium used in truss analysis - Define free body diagram of a truss - Define a perfect frame - When truss is analyzed by joint method - When truss is analyzed by section method - Define the term retaining wall - Name four types of retaining wall according to materials - Name the types of retaining wall according to design - Define angle of repose - Draw a sketch of retaining wall with surcharge backfill - Write the formula to calculate pressure on retaining wall - Narrate the conditions of stability of retaining wall - State the stability of retaining wall against overturning and sliding - Differentiate between active earth pressure and passive earth pressure. - Write the formula for maximum and minimum stresses at the base of retaining wall |

- Assessment Tasks

| Formative Assessment (Part A) (4) | Formative Assessment (Part B) (4) | Formative Assessment (Part C) (3) |
| --- | --- | --- |
| Assessment Task 1: Locate the centroid of the given section and find the Moment of inertia of the section on its centriodal axis. Performance Criteria for Assessment task 1: - Ensure ergonomic workstation setup to minimize strain during calculations and computer work. - Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) - Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). - Identify regular / irregular figure for locating centroid. - Draw reference axes (X-axis and Y-axis) - Divide the figure into required segments/sub section - Calculate centroidal of each segment/ sub section using relevant formulas. - Calculate centroid of composite section using relevant formulas. - Locate graphically the centroid of the figure. - Find MOI of each segment/ sub section about centroidal axis. - Apply formula of parallel axes theorem to shift MOI of composite figure on desired axis. Assessment Task 2: Draw Shear Force and Bending Moment diagrams for statically determinate beam Performance criteria Assessment Task 2: - Ensure ergonomic workstation setup to minimize strain during calculations and computer work. - Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) - Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). - Identify types of beams, types of supports and loads acting on them. - Draw load diagram according to given data - Compute reactions from conditions of equilibrium. - Compute shear force at the critical sections of beam. - Compute bending moment at the critical sections of beam - Draw Shear Force diagram for the computed values - Draw the Bending Moment Diagram from the computed values - Determine point of contra-flexure and point of zero shear force if required. - Draw the elastic curve of the beam. Assessment Task 3: Determine hardness of material by Rockwell and Brinell’s Hardness Testers. Performance criteria Assessment Task 3: - Handle weights and loading devices carefully to prevent injury or equipment damage - Use personal protective equipment (PPE) as needed (e.g., safety glasses, gloves). - Maintain proper posture while setting up the experiment to avoid strain or injury. - Keep the workspace clean and free of clutter to avoid tripping hazards - Identify equipment for testing hardness of material - Select specimen and fix correct indenter - Place specimen on platform and set position. - Perform test and note - Reading of pointer on corresponding scale of dial gauge. - Repeat test and tabulate readings. - Lower platform and remove indenter. - Tabulate result in desired format | Assessment Task 1: Determine modulus of elasticity of given metallic wire using young modulus apparatus. Performance criteria for Assessment Task 1: - Handle weights and loading devices carefully to prevent injury or equipment damage - Use personal protective equipment (PPE) as needed (e.g., safety glasses, gloves). - Maintain proper posture while setting up the experiment to avoid strain or injury. - Keep the workspace clean and free of clutter to avoid tripping hazards - Identify the desired tools and equipment - Select the desired tools and equipment - Prepare sample to conduct Young's modulus test of metallic wire. - Measure dimensions of un-stretched wire. - Perform test and note observations. - Compile results of stress and strain in tabular form - Calculate Young's modulus of Elasticity - Calculate strength of material - Compare the values with standards Assessment Task 2: For a given beam cross section determine the maximum bending and maximum shear stresses in beam Performance criteria Assessment Task 2: - Ensure ergonomic workstation setup to minimize strain during calculations and computer work. - Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) - Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). - Note down all the given and required parameters - Identify neutral axis of beam by using suitable formula - Identify radius of curvature of neutral axis. - Calculate maximum bending moment / moment of resistance of section. - Compute distance of extreme fiber from N.A. - Identify Young' modulus of elasticity - Compute cross sectional area of the given beam - Apply bending equation to find bending stress at a distance y from N.A. - Calculate maximum vertical shear force Q - Calculate area (A’) to the point of maximum shear stress - Calculate the centroidal distance from A’ to N. A - Calculate the maximum shear stress using the shear stress formula - Draw the stress distribution diagram Assessment Task 3: Calculate deflection in statically determinate beams by using deflection of beam apparatus Performance criteria Assessment Task 3: - Handle weights and loading devices carefully to prevent injury or equipment damage - Use personal protective equipment (PPE) as needed (e.g., safety glasses, gloves). - Maintain proper posture while setting up the experiment to avoid strain or injury. - Keep the workspace clean and free of clutter to avoid tripping hazards - Identify deflection of beam apparatus. - Select the required tools and equipment’s - Set apparatus and loading condition. - Apply incremental load and calculate bending moment for each load. - Apply bending equation and calculate bending stress for each increment load. - Record deflection on each load - Calculate Young's modulus of elasticity for each load. - Draw graph between each weight and deflection - Prepare table for results - Compute maximum slope and deflection | Assessment Task 1: Calculate buckling load / crushing load and safe load for column by Euler's formula / Rankin’s formula Performance criteria Assessment Task 1: - Ensure ergonomic workstation setup to minimize strain during calculations and computer work. - Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) - Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). - Note down all the given and required parameters - Identify the type of column. - Identify end conditions of column. - Determine equivalent / effective length of column. - Calculate moment of inertia. - Calculate critical load by Euler's formula - Calculate critical load by Rankin-Gordon formula - Identify factor of safety. - Calculate safe load - Compare the results of both formulas Assessment Task 2: Calculate forces in members of statically determinate truss by using method of joint Performance criteria Assessment Task 2: - Ensure ergonomic workstation setup to minimize strain during calculations and computer work. - Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) - Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). - Determine static determinacy of truss - Identify all joints, members, and external forces - label all joints, members, and external forces - Calculate support reactions using equilibrium equations - Proceed joint-by-joint to calculate forces in all truss members (tension or compression). - Tabulate result showing compressive / tensile forces in members - Draw force diagram Assessment Task 3: Determine stability of the given retaining wall by applying the conditions of stability. Performance criteria Assessment Task 3: - Ensure ergonomic workstation setup to minimize strain during calculations and computer work. - Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) - Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). - Note down all the given and required parameters - Draw section of wall. - Calculate weight of wall and its point of action. - Calculate pressure of material retained by wall - Draw pressure diagram. - Locate point of action of resultant force. - Calculate stresses developed at base by Rankine's formula. - Check stability of retaining wall against sliding, overturning, crushing and stresses at the base. - Draw stress distribution diagram at base |