Formative Assessment (Part A)

(4)

- Differentiate between centroid and center of gravity

Ans: Center of gravity: the point through which the whole weight of the body acts, irrespective of the position of body is called center of gravity. While, the center of plan geometrical figures i.e. 2D figures having no mass or weight is called centroid.

- Name the methods of determining the centroid

Ans: Following methods are used to determine the centroid

- By geometrical consideration

- By method of moments

- By method of integration

- By graphical method

- Define axis of reference and axis of symmetry

Ans: the perpendicular lines to each other, selected as reference, while calculating the centroid of a figure is called axis of reference. A line dividing the figure into two same size and shaped parts is called axis of symmetry.

- Define the centroidal axis

Ans: A line of reference axis passing axis through the centroid of a figure is called centroidal axis.

- Write the formula for calculating the CG of square and isosceles triangle

Ans: Following formula is used to calculate the CG of isosceles triangle

X-ordinate of centroid= b/2

Y-ordinate of centroid= h/3

- Write the formula for calculating the CG of semi-circle and quarter circle.

Ans: Following formula is used to calculate the CG of semi-circle

X-ordinate of centroid= R

Y-ordinate of centroid= 4R/3π

Following formula is used to calculate the CG of quarter-circle

X-ordinate of centroid= 4R/3π

Y-ordinate of centroid= 4R/3π

- Write the formula for calculating the CG of composite figures.

Ans: Following formulas are used to calculate the CG of composite figures.

y=a1y1+a2y2+…anyna1+a2+…an

x=a1x1+a2x2+…anxna1+a2+…an

- Define Moment of inertia

Ans: Moment of inertia is the second moment of area about an axis. It measures an object's resistance to rotation around that axis.

- State theorem of parallel and perpendicular axis of M.O.I

Ans: The Parallel Axis Theorem states that the moment of inertia “I” of a body about any axis parallel to an axis passing through it is equal to the moment of inertia of the body about the centroid plus the product of the area of the body and the square of the distance between the two axes. Mathematically, it is given by:

Ixx=Ixo+AK2

Iyy=Iyo+Ar2

The M.O.I about an axis perpendicular to the plan is equal to the sum of M.O.I of the two perpendicular axis through the same point in the plan of object. Mathematically, it is given by

Izz=Ixx+Iyy

- Define radius of gyration.

Ans: Radius of gyration is a property of a body that describes its distribution of mass about an axis of rotation.. Mathematically, it the square root ratio of M.O.I to the cross-sectional area.

Ixx=IxxA

Iyy=IyyA

- Write formula for M.O.I through the centroid of square and circle

Ans: Formula for M.O.I for square of side “b”

𝑏4

𝐼𝑥𝑜 = 12

𝐼𝑦𝑜 = 12

Formula for M.O.I for circle of diameter “D”

Ixo=Iyo=πD464

- State polar moment of inertia

Ans: Polar Moment of Inertia is a measure of an object's capacity to oppose or resist torsion when some amount of torque is applied to it on a specified axis. Mathematically it is equal to the sum of MOI about x-axis and y-axis

- Write formula for M.O.I through z-axis

Ans: Formula for M.O.I about z-axis is given by;

Izz=Ixx+Iyy

- Write formula for M.O.I for isosceles triangle parallel to the base and passing through centroid.

Ans: Formula for M.O.I for isosceles triangle parallel to the base and passing through centroid;

Ixo=bh336

- Write formula for M.O.I for isosceles triangle perpendicular to the base and passing through centroid.

Ans: Formula for M.O.I for isosceles triangle perpendicular to the base and passing through centroid;

Ixo=bh348

- Write formula for M.O.I for semi-circle and quarter circle about an axis parallel to the base and passing through centroid.

Ans: formula for semi-circle of radius “R” about an axis parallel to the base and passing through centroid;

Ixo=0.11 R4

Formula for M.O.I for quarter circle about an axis parallel to the base and passing through centroid.

Ixo=0.055 R4

- State shear force and bending moment

Ans: In mechanics the algebraic sum of vertical forces acting on a particular section of beam is called shear force.

In mechanics, the algebraic sum of moments about the centroidal axis of any selected section of all the loads acting up to that section is called bending moment.

- Name and sketch different types of supports

Ans:

- Name and sketch different types of beams

Ans:

- State the loads acting on beams

Ans: Following are the types of loads acting on beams

- Point Load

- Uniformly Distributed Load

- Uniformly Varying Load

- Irregular Varying load

- Write the equation of stability used while calculating reactions of beams

Ans: following equations are used to determine the support reactions of beams

Σ𝐹𝐻= 0 𝐴𝑙𝑔𝑒𝑏𝑟𝑎𝑖𝑐 𝑠𝑢𝑚 𝑜𝑓 ℎ𝑜𝑟𝑖𝑧𝑜𝑛𝑡𝑎𝑙 𝑓𝑜𝑟𝑐𝑒𝑠

Σ𝐹𝑉= 0 𝐴𝑙𝑔𝑒𝑏𝑟𝑎𝑖𝑐 𝑠𝑢𝑚 𝑜𝑓 𝑣𝑒𝑟𝑡𝑖𝑐𝑎𝑙 𝑓𝑜𝑟𝑐𝑒𝑠

Σ𝑀= 0 𝐴𝑙𝑔𝑒𝑏𝑟𝑎𝑖𝑐 𝑠𝑢𝑚 𝑜𝑓 𝐴𝑙𝑙 𝑚𝑜𝑚𝑒𝑛𝑡𝑠

- Describe the elastic curve of beams

Ans: An elastic curve is the shape of a beam's longitudinal axis when it bends under a load within its elastic limit.

- Define the point of contra-flexure.

Ans: The point of contra flexure is a point along a beam under bending where the bending moment changes sign, meaning it shifts from positive to negative or vice versa. At this point, the bending moment is zero.

- Write formula for maximum shear force and bending moment of a cantilever beam having span “L” and load “W” at free end

Ans: Following formula is used

shear force=W

bending moment=W×L

- Write formula for maximum shear force and bending moment of a simply supported beam having span “L” and load “W” at mid-point

Ans: Following formula is used

shear force=W2

bending moment=WL4

- Write formula for maximum shear force and bending moment of a simply supported beam having span “L” and loaded with UDL “w/unit length” over the entire span

Ans: Following formula is used

shear force=wL2

bending moment=w L28

- Define the hardness of metals

Ans: The hardness of metals refers to their ability to resist, indentations, abrasion or scratches. It is a measure of the material's softness or hardness.

- Define toughness of metals

Ans: Toughness is a material's ability to resist suddenly applied impact load and absorb energy.

- Name different test to check the hardness of metals

Ans: Following test are used to check the hardness of metals

1. Brinell hardness test

2. Rockwell hardness test

3. Vicker’s Hardness test

- Write formula to determine Brinell hardness number

Ans: Following formula is used to determine Brinell hardness number;

B.H.N=2PπDD-D2-d2

Where;

B.H. N= Brinell hardness number

P=Applied load in kilogram

D= diameter of steel ball

d= diameter of impression in mm

- State the precautions to be adopted in Brinell hardness test

Ans: Following precautions are adopted in Brinell hardness test

1. Standard size sample should be used

2. Load should be selected as per type of material

3. Load should be applied gradually

4. Microscope should be used to measure indentation

5. Clean the machine before and after use

- State the advantages of Rockwell hardness test

Ans: following are the advantages of Rockwell hardness test;

1. It minimizes errors over application of the minor load

2. No microscope is required for taking reading it is taken directly

3. It is used for primed surfaces and small parts of machine

4. Test pieces can be used after testing

5. No experience is required to perform this test.

- Write the formula to determine Rockwell hardness number

Ans: Following formula is used to determine Rockwell hardness number

𝐻𝑅=𝐸−𝑒

Where;

HR= Rockwell hardness number

E= constant depends on the type of indenter

e= permanent increase in depth penetration under minor load

- Give the limitation of Brinell hardness test

Ans: Following are the limitation of Brinell hardness test;

1. It is not suitable for very hard materials

2. Thin pieces of metals can be tested

3. Metals whose surface is hardened through case hardening process cannot be tested

- How tensile strength is determined with the help of BHN

Ans: The calculated BHN is multiplied by a particular constant for determining the tensile strength of materials in kg/mm2. Constant depends upon the type of material.

𝑇𝑒𝑛𝑠𝑖𝑙𝑒 𝑠𝑡𝑟𝑒𝑛𝑔𝑡ℎ= 𝐵𝐻𝑁×𝐾

Formative Assessment (Part B)

(4)

- Define stress and strain

Ans: stress: the applied force per unit resisting area is called stress. It is denoted by “f”. Unit of stress is. N/m2 or Kg/cm2 or lb/ft2

Stress = Force/ Area

𝑓=𝑃/𝐴

Strain: it is defined as the ratio of change in dimension to the original dimension. It is denoted by “e”. Strain has no units.

𝑠𝑡𝑟𝑎𝑖𝑛 (𝑒)=𝐶ℎ𝑎𝑛𝑔𝑒 𝑖𝑛 𝑑𝑖𝑚𝑒𝑛𝑠𝑖𝑜𝑛𝑂𝑟𝑖𝑔𝑖𝑛𝑎𝑙 𝐷𝑖𝑚𝑒𝑛𝑠𝑖𝑜𝑛

- Name different types of stress and strain

Ans: Stress has the following types

1. Tensile stress: caused by tensile forces

2. Compressive stress: caused by compressive forces

3. Shear stress: caused by shear forces

Strain has the following types

1. Tensile strain: it is the ratio of elongation to original length

2. Compressive strain: it is the ratio of contraction to original length

3. Shear strain: it the ratio of angular deformation to original length

4. Volumetric strain: it the ratio of change in volume to the original volume

- Differentiate between linear and lateral strain

Ans: Linear strain: It is the strain in a material in the direction in which the force is applied. It is always tensile in nature and the change is positive i.e. increase in dimension.

Lateral strain: It is the strain in the other direction i.e. breadth and depth when force is applied in the linear direction. It is always compressive in nature and the change is negative i.e. decrease in dimension.

- Define modulus of elasticity

Ans: It is defined as the ratio of the direct stress to the direct strain. It is also called Young’s modulus. It is denoted by “E”. Its unit is same as that of stress i.e. N/m2 or Kg/cm2 or Lb/ft2

Modulus of elasticity= direct stress/direct strain

E=fe

- State Hook’s law

Ans: Hook’s law states “Hooke's law in mechanics states that for small displacements or deformations of a material, stress is directly proportional to strain with in elastic limit of the material”

stress ∝strain

stress=Constant×strain

stress=E×strain

f=E×e

E = modulus of elasticity

- Define shear modulus

Ans: It is defined as the ratio of shear stress to shear strain. It is also called modulus of rigidity. It is denoted by “G”.

Shear modulus= shear stress/ shear strain

𝐺=𝑓𝑠/𝑒𝑠

- State bulk modulus

Ans: It is defined as the ratio of the direct stress to the volumetric strain. It occurs when stresses are acting on the planes of the body which are vertical to each other. It is denoted by “K”.

Bulk modulus= Direct stress/ Volumetric strain

K=fev

- Define Poisson’s ratio

Ans: The ratio of the secondary strain to primary strain is called Poisson’s ratio. OR the ratio of lateral strain to linear strain is called Poisson’s ratio. It is denoted by 𝜇.

Poisson’s ratio= Lateral strain/ linear strain

μ=lm=ebel=edel

- Differentiate between elasticity and plasticity of a material

Ans: Elasticity and plasticity are properties of materials that describe their response to external forces.

Elasticity causes reversible deformations and allows the material to regain its original shape after the removal of external load.

Plasticity causes irreversible deformations and allows the material to retain its deformed shape even after the load is removed.

- State factor of safety.

Ans: The factor of safety is defined as the ratio between the ultimate stress of the component material and the working stress it experiences. It is always greater than 1.

Factor of safety= Ultimate stress/working stress

- What is the primary functions of Universal Testing Machine?

Ans: The primary function of a Universal Testing Machine (UTM) is to measure the mechanical properties of materials, such as tensile strength, compressive strength, and flexural strength.

- What type of materials can be tested by UTM

Ans: Universal Testing Machines (UTMs) can test various materials, including metals, plastics, composites, ceramics, and textiles, as well as biomaterials and other advanced materials.

- What is the function of extensometer in UTM

Ans: An extensometer in a UTM measures the precise deformation (strain) of a test specimen, providing accurate data on material elongation, contraction, or deformation.

- Define yield stress

Ans: Yield stress is the stress at which a material begins to deform plastically, marking the transition from elastic to plastic behavior. It's the point where permanent deformation occurs.

- Define ultimate stress

Ans: Ultimate stress (or ultimate tensile strength) is the maximum stress a material can withstand without failing, representing the highest stress value on a stress-strain curve.

- Define bending stress

Ans: Under the application of load the bending occurs in the beam. The stress caused in the beam due to bending is called bending stress.

- Differentiate between pure bending and simple bending

Ans: Pure bending is that type of bending which is only due to applied moment without the application of axial, shear or torsional forces. Normal forces will be created and shear forces will be zero in this case. Simple bending is that type of bending which is due to the application of self-load and external load. Both normal and shear forces will be created in this case

- State assumptions of simple bending

Ans: Following are four assumptions of simple bending

1. Material is isotropic and homogeneous

2. The material obeys Hook’s law

3. The beam has an axis of symmetry in the plane of bending

4. Cross section of the beam remains plane during bending

- State section modulus

Ans: section modulus is the geometric property of the cross section of a beam which is equal to ratio of moment of inertia and depth of neutral axis. It is denoted by “Z”

Section modulus= Moment of inertia/ depth of neutral axis

Z=Iy

- State bending equation of beam

Ans: Following is the bending equation

MI=fy=ER

Where;

M= bending moment

I= moment of inertia about centroidal axis

f= bending stress at any distance “y” form neutral axis

y= distance from neutral axis

E= modulus of elasticity or young’s modulus

R= radius of curvature of the deflected beam

- Write formula for section modulus of square and circular hollow section

Ans: Formula for section modulus of square section is;

D36

Formula for section modulus of circular hollow section is

𝜋(D3-d3)/32

- Write the formula for the intensity of shear stress in the beam

Ans: Formula for the intensity of shear stress in the beam is;

q=QIb×A'y̅

Where;

q= intensity of shear stress at a particular point

Q= shear force at the section

I= moment of inertia about centroidal axis

b= breadth of the beam

A’= cross section area above the line under consideration

𝑦̅= centroidal distance of A’ from the neutral axis

- Draw the shape of shear stress distribution in I-section

Ans:

- Define neutral axis of a loaded beam.

Ans: The neutral axis is an axis in the cross section of a beam in which there is no longitudinal or bending stress or any strain. For symmetric, homogeneous and isotropic section the neutral axis is the geometric centroid of the section

- Give the relation between average and maximum shear stress in rectangular beam.

Ans: Following is the relation between average and maximum shear stress in rectangular beam.

qmax=32qavg   for rectangular section

qmax=43qavg   for circular section

qavg=QA

- Define the deflection of beam

Ans: The deflection at any point on the axis of beam is defined as “the distance between the position of the beam axis before and after bending”. It is denoted by “Y”.

- Name various methods of deflection calculation in beams

Ans:

- Double integration method

- Moment area method

- Unit load method

- Energy method

- Virtual work method

- Define stiffness of the beam section

Ans: Stiffness is defined as “the resistance of the beam section against deflection”. If the modulus of elasticity of the material of beam is high its stiffness will be more. Also beam will high section modulus will have more stiffness.

- Give significance of deflection of beams

Ans: Following is the significance of deflection of beams

1. It gives the resistance of beam against deflection

2. We can have an idea of the maximum safe load to be applied on beams

3. Size, shape and material of beam can be selected easily

4. Prevents visible sagging and ensures user comfort in floors or bridges.

- Where maximum deflection will act at cantilever beam loaded UDL at full span

Ans: For a cantilever beam subjected to a uniformly distributed load (UDL) across its entire span, the maximum deflection occurs at the free end of the beam.

- Where maximum deflection will act at cantilever beam loaded with a point loaded at free end

Ans: For a cantilever beam with a point load (P) applied at the free end, the maximum deflection occurs at the free end itself.

- Write the formula to calculate maximum deflection in a cantilever having a point load at mid span

Ans: Formula to calculate maximum deflection in a cantilever having a point load at mid span is

Y=Wl133EI+Wl122EI(l-l1)

- Write the formula to calculate maximum deflection in a cantilever having UDL at half span from fixed end

Ans: Formula to calculate maximum deflection in a cantilever having UDL at half span from fixed end is;
Y=Wl148EI+Wl36EI(l-l1)

- Write the formula to calculate maximum deflection in a cantilever having span “l” and point load “W” at free end and a UDL at full span

Ans: Formula to calculate maximum deflection in a cantilever having span “l” and point load “W” at free end and a UDL at full span
Y=Wl133EI+Wl48EI(l-l1)

- Write the formula to calculate maximum deflection in a simply supported beam having span “l” and point load “W” at mid-span and a UDL at whole span.

Ans: Formula to calculate maximum deflection in a simply supported beam having span “l” and point load “W” at mid-span and a UDL at whole span.
Y=Wl1348EI+5Wl4384EI(l-l1)

Formative Assessment (Part C)

(3)

- Differentiate between column and strut.

Ans: Column: A vertical structural member designed to support compressive loads and transfer them to the foundation is called column.

Strut: A structural member that resists compressive forces and can be positioned in any orientation (vertical, inclined, or horizontal) is called strut.

- Differentiate between long column and short column

Ans: Long column: if the length of the column is more than 15 times the least dimension of its cross section it is called long column.

Short column: if the length of the column is less than 15 times the least dimension of its cross section it is called short column.

Long columns have a high slenderness ratio i.e. more than 80, whereas short columns have a low slenderness ratio i.e. Less than 80.

- Differentiate between axial loaded and eccentric loaded column

Ans: Axially Loaded Column: A column where the load acts along its central axis, resulting in uniform stress distribution across the cross-section.

Eccentrically Loaded Column: A column where the load does not act along the central axis, causing bending in addition to compression.

- Define slenderness ratio

Ans: Slenderness Ratio: The slenderness ratio of a column is the ratio of its effective length to its least radius of gyration. It is a dimensionless parameter that indicates the tendency of a column to buckle under compressive loads.

- Give effective length of column with various end conditions

Ans:

- State Euler’s formula for calculating ultimate load for column

Ans: Euler’s formula is used to calculate the ultimate buckling of long columns. To get safe load on column the ultimate load is divided by a FOS.

Mathematically;

P=π2EILe2

Where;

P=Ultimate buckling load

E= modulus of elasticity of material

I= moment of inertia of column section

Le= effective length of column

- State Rankine’s formula for calculating ultimate load for column

Ans: Rankine’s formula is used to calculate ultimate load for both short and long columns. For short column the load is crushing load and for long column the load is buckling load.

Mathematically;

P=fc A1+aLeK2

Where;

P= critical load

fc= critical stress

A= cross sectional area of column

Le= effective length of column

K= least radius of gyration

a= Rankine’s constant depends on the material of column

for mild steel a= 1/7500

for cast iron a= 1/1600

for wrought iron a=1/9000

- Give failure pattern of short columns

Ans: Short column directly fails by crushing under the load application.

- Give failure pattern of long column

Ans: Long column buckles on the application of load and fails by bending.

- What is safe applied load on the columns

Ans: When the buckling/ crushing load of a column is divided by a factor of safety it results in safe applied load on column.

- Define the truss

Ans: A truss is a structural framework made of straight members arranged in triangular units, designed to support loads efficiently by distributing forces through tension and compression.

- Name four types of trusses

Ans: four types of trusses are

1. Pratt Truss

2. Warren Truss

3. Howe Truss

4. King Post Truss

- Name four parts of a truss

Ans: Ans: four parts of a truss are;

1. Top Chord

2. Bottom Chord

3. Web Members

4. Nodes (Joints)

- Name various method of truss analysis

Ans: Following are the methods of truss analysis

1. Method of Joints

2. Method of Sections

3. Graphical Method

4. Finite Element Method (FEM)

- Differentiate between statically determinate and indeterminate structurers

Ans: Statically Determinate Structures: Structures whose internal forces and reactions can be determined using only the equations of static equilibrium.

Statically Indeterminate Structures: Structures that require additional methods, such as compatibility equations, to determine internal forces and reactions because the equations of static equilibrium alone are insufficient.

- State the equation of equilibrium used in truss analysis

Ans: The equations of equilibrium used in truss analysis are

- Sum of horizontal forces

Fx=0

- Sum of vertical forces

Fy=0

- Sum of moments about any point

M=0

- Define free body diagram of a truss

Ans: free body diagram is a simplified representation of the entire truss or an isolated portion of it, showing all the external forces, reactions, and internal forces acting on it.

- Define a perfect frame

Ans: perfect frame is a truss structure in which the number of members (𝑚) and joints (𝑗) satisfy the equation: 𝑚=2𝑗−3

This ensures the frame is both statically determinate and stable

- When truss is analyzed by joint method

Ans: The Method of Joints is used to analyze a truss when the forces in all member of a truss are required.

- When truss is analyzed by section method

Ans: The Method of Sections is used to analyze a truss when we require forces in specific members of truss and not in the entire members.

- Define the term retaining wall

Ans: A retaining wall is a strong wall built to hold back soil or water or any other material to stop them from sliding or falling, especially on sloped land.

- Name types of retaining wall according to materials

Ans: Types of retaining wall according to materials

1. Earthen retaining wall

2. Masonry retaining wall

3. Rock fill retaining wall

4. P.C.C retaining wall

5 R.C.C retaining wall

- Name the types of retaining wall according to design

Ans: Types of retaining wall according to design

1. Gravity retaining wall

2. Cantilever retaining wall

3. Counter fort retaining wall

- Define angle of repose

Ans: The angle of repose is the steepest natural angle at which a pile of loose material, like sand or gravel, can rest without sliding or collapsing. It shows how stable the material is.

- Draw a sketch of retaining wall with surcharge backfill

Ans:

- Write the formula to calculate pressure on retaining wall

Ans: Formula to calculate pressure on retaining wall

1. Horizontal pressure

(a) Level backfills

P=γh221-sinφ1+sinφ

(b) Surcharge back fill

Ph=γh22cosα-cos2α-cos2φcosα+cos2α-cos2φ

- (c) Super imposed load surcharge

- P=γh221-sinφ1+sinφ

- 2. Vertical pressure

W=P=γh(a+b)2

- Narrate the conditions of stability of retaining wall

Ans: The stability conditions for a retaining wall include:

1. Overturning: The wall must resist rotation caused by lateral earth pressure to prevent tipping over.

2. Sliding: The wall must resist horizontal movement due to the force of lateral earth pressure.

3. Tensile Stresses: The design should ensure no tensile stresses develop in the base, which can lead to cracking.

4. Crushing: The compressive stress at the toe of the wall must not exceed the allowable bearing capacity of the foundation soil.

- State the stability of retaining wall against overturning and sliding

Ans: Stability of Retaining Wall against Overturning:

Factor of safety for overturning≤Balancing MomentOverturning Moment

1.5≤3w(b-X)P.h

Stability of Retaining Wall Against Sliding:

Factor of safety for sliding≤Frictional ForceHorizontal Force

1.5≤μWP

- Differentiate between active earth pressure and passive earth pressure.

Ans: Active Earth Pressure: Active earth pressure is the pressure exerted by the soil on a wall when the wall moves away or tilts outward.

Pa=γH1-sinφ1+sinφ

Passive Earth Pressure: Passive earth pressure is the pressure exerted by the soil on a wall when the wall moves inward or is pushed into the soil

Pa=γH1+sinφ1-sinφ

- Write the formula for maximum and minimum stresses at the base of retaining wall

Ans: Formula for maximum and minimum stresses at the base of retaining wall

fmax=Wb1+6eb

fmin=Wb1-6eb