Course / Subject Name: NQ-Civil-263 Engineering Mechanics

Assessment Tasks (Summative Assessment 02)

| Assessment Task 01 |  | For a given beam loaded as shown with the given channel-cross-section, determine the centroid and moment of inertia of the section and draw the Shear Force Diagram (SFD) and Bending Moment Diagram (BMD). Also, find the maximum bending and shearing stress |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Ensure ergonomic workstation setup to minimize strain during calculations and computer work. |  |  |  |  |  |
|  | Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) |  |  |  |  |  |
|  | Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). |  |  |  |  |  |
|  | Ensure adequate lighting and ventilation in the workspace to reduce fatigue and eye strain |  |  |  |  |  |
|  | Identify the geometric properties of the I-cross-section (dimensions of flange, web, etc.). |  |  |  |  |  |
|  | Identify regular / irregular figure for locating centroid. |  |  |  |  |  |
|  | Draw reference axes (X-axis and Y-axis) |  |  |  |  |  |
|  | Divide the figure into required segments/sub section |  |  |  |  |  |
|  | Calculate centroidal of each segment/ sub section using relevant formulas. |  |  |  |  |  |
|  | Calculate centroid of composite section using relevant formulas. |  |  |  |  |  |
|  | Locate graphically the centroid of the figure. |  |  |  |  |  |
|  | Find MOI of each segment/ sub section about centroidal axis. |  |  |  |  |  |
|  | Apply formula of parallel axes theorem to shift MOI of composite figure on desired axis. |  |  |  |  |  |
|  | Identify types of beams, types of supports and loads acting on them. |  |  |  |  |  |
|  | Draw load diagram according to given data |  |  |  |  |  |
|  | Compute reactions from conditions of equilibrium. |  |  |  |  |  |
|  | Compute shear force at the critical sections of beam. |  |  |  |  |  |
|  | Compute bending moment at the critical sections of beam |  |  |  |  |  |
|  | Draw Shear Force diagram for the computed values |  |  |  |  |  |
|  | Draw the Bending Moment Diagram from the computed values |  |  |  |  |  |
|  | Determine point of contra-flexure and point of zero shear force if required. |  |  |  |  |  |
|  | Draw the elastic curve of the beam |  |  |  |  |  |
|  | Identify neutral axis of beam cross section |  |  |  |  |  |
|  | Compute moment of inertia about N. A |  |  |  |  |  |
|  | Compute the maximum bending moment |  |  |  |  |  |
|  | Compute the maximum distance from N.A |  |  |  |  |  |
|  | Calculate the maximum bending stress using the flexural formula |  |  |  |  |  |
|  | Calculate maximum vertical shear force Q |  |  |  |  |  |
|  | Calculate area (A’) to the point of maximum shear stress |  |  |  |  |  |
|  | Calculate the centroidal distance from A’ to N. A |  |  |  |  |  |
|  | Calculate the maximum shear stress using the shear stress formula |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

| Assessment Task 02 |  | Calculate the modulus of elasticity using Young’s modulus apparatus. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Handle weights and loading devices carefully to prevent injury or equipment damage |  |  |  |  |  |
|  | Use personal protective equipment (PPE) as needed (e.g., safety glasses, gloves). |  |  |  |  |  |
|  | Maintain proper posture while setting up the experiment to avoid strain or injury. |  |  |  |  |  |
|  | Keep the workspace clean and free of clutter to avoid tripping hazards |  |  |  |  |  |
|  | Ensure adequate lighting and ventilation in the lab for accurate readings and a safe environment |  |  |  |  |  |
|  | Set up the Young’s modulus apparatus correctly according to specifications |  |  |  |  |  |
|  | Use the desired tools and equipment |  |  |  |  |  |
|  | Measure the initial length and diameter of the test specimen |  |  |  |  |  |
|  | Apply incremental loads to the specimen and measure the corresponding elongations using the apparatus dial gauge. |  |  |  |  |  |
|  | Record all measurements in a table for load vs. elongation. |  |  |  |  |  |
|  | Plot a stress-strain graph using the data, and identify the elastic limit |  |  |  |  |  |
|  | Calculate the modulus of elasticity E using the formula |  |  |  |  |  |
|  | Validate the result by comparing it with standard values for the material |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

| Assessment Task 03 |  | Calculate the deflection in a simply supported beam using the deflection of beam apparatus. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Handle weights and loading devices carefully to prevent injury or equipment damage |  |  |  |  |  |
|  | Use personal protective equipment (PPE) as needed (e.g., safety glasses, gloves). |  |  |  |  |  |
|  | Maintain proper posture while setting up the experiment to avoid strain or injury. |  |  |  |  |  |
|  | Keep the workspace clean and free of clutter to avoid tripping hazards |  |  |  |  |  |
|  | Ensure adequate lighting and ventilation in the lab for accurate readings and a safe environment |  |  |  |  |  |
|  | Set up the simply supported beam in the deflection of beam apparatus. |  |  |  |  |  |
|  | Measure and note the beam’s length, cross-sectional dimensions, and material properties (E and I). |  |  |  |  |  |
|  | Apply loads at specified points on the beam as per the experiment setup |  |  |  |  |  |
|  | Measure the deflection at various points along the beam using the dial gauge. |  |  |  |  |  |
|  | Record load and deflection readings in a table for analysis |  |  |  |  |  |
|  | Use the deflection formula for a simply supported beam to calculate theoretical deflection |  |  |  |  |  |
|  | Compare experimental deflection with theoretical calculations and analyze discrepancies |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |