Course / Subject Name: NQ-Civil-263 Engineering Mechanics

Assessment Tasks (Summative Assessment 03)

| Assessment Task 01 |  | A mild steel specimen of 50mm diameter and 2m length is subjected to a load of 50KN which extends by 10 mm. Calculate the tensile stress, strain and modulus of elasticity. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Ensure ergonomic workstation setup to minimize strain during calculations and computer work. |  |  |  |  |  |
|  | Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) |  |  |  |  |  |
|  | Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). |  |  |  |  |  |
|  | Ensure adequate lighting and ventilation in the workspace to reduce fatigue and eye strain |  |  |  |  |  |
|  | identify all given parameters from the problem statement |  |  |  |  |  |
|  | calculate the cross-sectional area of the specimen |  |  |  |  |  |
|  | Compute tensile stress using the appropriate formula. |  |  |  |  |  |
|  | Determine strain based on the provided data. |  |  |  |  |  |
|  | calculate the modulus of elasticity using the stress and strain values. |  |  |  |  |  |
|  | Write down the correct unit of the required parameters |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

| Assessment Task 02 |  | A cantilever beam of rectangular section 20cm x 10 cm having length of 5m is subjected to UDL over its entire length. Cross section of the beam is also shown. Calculate the geometric centroid, MOI and Polar MOI of the given T-section. Also Calculate the maximum deflection, maximum bending and maximum shearing stress. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Ensure ergonomic workstation setup to minimize strain during calculations and computer work. |  |  |  |  |  |
|  | Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) |  |  |  |  |  |
|  | Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). |  |  |  |  |  |
|  | Ensure adequate lighting and ventilation in the workspace to reduce fatigue and eye strain |  |  |  |  |  |
|  | Identify the geometric properties of the T-cross-section (dimensions of flange, web, etc.). |  |  |  |  |  |
|  | Calculate centroidal of each segment/ sub section. |  |  |  |  |  |
|  | Calculate the centroid of the cross-section using the required formulas |  |  |  |  |  |
|  | Find MOI of each segment/ sub section about centroidal axis. |  |  |  |  |  |
|  | Calculate the moment of inertia about centroid using the parallel axis theorem |  |  |  |  |  |
|  | Calculate the Polar Moment of inertia |  |  |  |  |  |
|  | Identify and categorize the types of loads acting on the beam |  |  |  |  |  |
|  | Specify the supports and their constraints |  |  |  |  |  |
|  | Draw a detailed load diagram based on the given data |  |  |  |  |  |
|  | Identify neutral axis of beam cross section |  |  |  |  |  |
|  | Compute moment of inertia about N. A |  |  |  |  |  |
|  | Compute the maximum bending moment |  |  |  |  |  |
|  | Compute the maximum distance from N.A |  |  |  |  |  |
|  | Calculate the maximum bending stress using the flexural formula |  |  |  |  |  |
|  | Calculate maximum vertical shear force Q |  |  |  |  |  |
|  | Calculate area (A’) to the point of maximum shear stress |  |  |  |  |  |
|  | Calculate the centroidal distance from A’ to N. A |  |  |  |  |  |
|  | Calculate the maximum shear stress using the shear stress formula |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

| Assessment Task 03 |  | Calculate the forces in the members BC, FC and FE of the given truss using method of section |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Ensure ergonomic workstation setup to minimize strain during calculations and computer work. |  |  |  |  |  |
|  | Handle all drawing and measuring equipment safely to prevent injuries (e.g., sharp-edged tools etc.) |  |  |  |  |  |
|  | Maintain proper posture and take regular breaks to avoid repetitive strain injuries (RSIs). |  |  |  |  |  |
|  | Ensure adequate lighting and ventilation in the workspace to reduce fatigue and eye strain |  |  |  |  |  |
|  | Identify static determinacy of truss |  |  |  |  |  |
|  | Identify and label all joints, members, and external forces |  |  |  |  |  |
|  | Calculate support reactions using equilibrium equations |  |  |  |  |  |
|  | Perform the section cut at appropriate location on the truss member |  |  |  |  |  |
|  | Calculate the unknown forces in the required members |  |  |  |  |  |
|  | Tabulate result showing compressive / tensile forces in members |  |  |  |  |  |
|  | Draw the force diagram |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |