Course / Subject Name: NQ-Civil-PHT- Public Health Technology

Assessment Tasks (Summative Assessment)

| Assessment Task 01 |  | Perform water quality test for determination of given parameters i.e. Color, Oduor, pH, TDS, Turbidity and Hardness in the Lab. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Time Required |
|  | Carry out OHS requirements |  |  |  |  | 5 Hours |
|  | Identify national / international guide lines of quality standards of water |  |  |  |  |  |
|  | Identify and prepare equipment for physical tests of water |  |  |  |  |  |
|  | Perform colour & odour test on sample of water |  |  |  |  |  |
|  | Perform pH test on sample of water |  |  |  |  |  |
|  | Perform turbidity test on sample of water |  |  |  |  |  |
|  | Perform TDS test on sample of water |  |  |  |  |  |
|  | Perform hardness tests on sample of water |  |  |  |  |  |
|  | Tabulate results against each test |  |  |  |  |  |
|  | Analyze the results with guidelines |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

| Sr# | Tools/Equipment |
| --- | --- |
| 1 | TDS Meter |
| 2 | pH Meter |
| 3 | Turbidity Meter |
| Materials |  |
| 1 | Water Hardness Test Kit |

| Assessment Task 02 |  | Design an R.C.C circular sanitary sewer for a town with the following data: Population of town = 75000 persons Rate of water supply = 125 lpcd Peak factor = 3 Velocity of sewage flow = 1 m/sec Assume 80% of water supply as a sewage flow Note: Assume suitable data if part any missing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Time Required |
| 1. | Identify type of area and Location |  |  |  |  | 2.5 Hours |
| 2. | Determine population and its growth rate |  |  |  |  |  |
| 3. | Identify rate of water supply |  |  |  |  |  |
| 4. | Calculate average rate of water supply per day and per second |  |  |  |  |  |
| 5. | Calculate Maximum flow |  |  |  |  |  |
| 6. | Calculate the area |  |  |  |  |  |
| 7. | Calculate the discharge |  |  |  |  |  |
| 8. | Calculate total sewage |  |  |  |  |  |
| 9. | Determine size of sewer |  |  |  |  |  |
| 10. | Prepare the drawing of sewer |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

| Assessment Task 03 |  | A catchment area of 200 hectares consists of various surfaces. The areas and their individuals’ impervious factors are given below. Calculate dry and wet weather flow in cusecs when rainfall is recorded 40 mm / hour Sr#; Surface; Area; Impervious Factor. 1; Roof; 15%; 0.90. 2; Road; 20%; 0.60. 3; Open Space; 65%; 0.12. |
| --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Time Required |
| 1. | Identify the type of area |  |  |  |  | 2.5 Hours |
| 2. | Determine the area of the roof, road and open space |  |  |  |  |  |
| 3. | Impervious factor |  |  |  |  |  |
| 4. | Determine the dry weather flow |  |  |  |  |  |
| 5. | Determine the wet weather flow |  |  |  |  |  |
| 6. | Determine the population |  |  |  |  |  |
| 7. | Calculate the rate of water supply |  |  |  |  |  |
| 8. | Calculate the total quantity of water supplied in 24 hours |  |  |  |  |  |
| 9. | Calculate the discharge |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

| Assessment Task 04 |  | Installation of plumbing fixture (Wash Hand Basin) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Time Required |
|  | Carry out OHS requirements |  |  |  |  | 5Hours |
|  | Identify and interpret drawing of fixture |  |  |  |  |  |
|  | Identify tools and equipment required for installation of fixtures |  |  |  |  |  |
|  | Perform measurements and mark out Installation point |  |  |  |  |  |
|  | Fix Pedestal and Install wash hand basin |  |  |  |  |  |
|  | Fix Pillar cock, Rubber connection, waste pipes with coupling |  |  |  |  |  |
|  | Check the levels |  |  |  |  |  |
|  | Connect outlet pipes with water supply |  |  |  |  |  |
|  | Perform final quality inspection |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

| Sr# | Tools/Equipment |
| --- | --- |
| 1 | Electric Drill |
| 2 | Measuring Tape |
| 3 | Hammer |
| 4 | Chisel |
| 5 | Electric Grinder with Disc |
| 6 | Screw Driver |
| 7 | Pipe Wrench |
| 8 | Wrench China |
| 9 | Hand Level |
| 10 | Electric Heater |
| 11 | Hack Saw |
| Materials |  |
| 1 | Wash hand basin with accessories |
| 2 | Rawal bolt set |
| 3 | Led Pencil |
| 4 | Cutting Disc |
| 5 | Hacksaw Blade |