Course / Subject Name: Civil 313 Advanced Quantity Surveying

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU- (1-2) | LU- (3-5) | LU- (6-8) |
| - Maximum limit of percentage of steel in R.C.C structure by volume is: a) 4% b) 8% c) 10% d) 15% - Extra length of steel bar due to hook is taken as: a) 4D b) 6D c) 7D d) 9D - Density of mild steel in kg/m3 is taken as: a) 5850 b) 6500 c) 7850 d) 8000 - Which of the following are the items of work in RCC framed structure buildings? a) Excavation b) Brickwork c) Reinforcement detailing d) All of these - How is the quantity of excavation calculated? a) Length × Width b) Area × Depth c) Perimeter × Depth d) Volume × Density - The quantity of concrete for an RCC column is calculated by which formula? a) Cross-sectional area × Height b) Volume ÷ Density c) Perimeter × Height d) None of the above - How is the quantity of bricks in a wall generally calculated? a) Volume of Wall ÷ Volume of one brick b) Volume of Wall × Volume of one brick c) Length × Thickness ÷ Mortar quantity d) Area × Thickness - In a bar bending schedule, which of the following is not included? a) Bar number b) Bar length c) Unit weight d) Cost of bar - What does MRS stand for in cost estimation? a) Market Reference System b) Monthly Rate Schedule c) Material Rate Schedule d) Market Rate Schedule - Flooring material like tiles is generally measured in: a) Cubic meters b) square meters c) Running meters d) Number of tiles - Surface rendering (plaster) is calculated based on: a) Perimeter b) Cross-sectional area c) Surface area d) None of the above - The purpose of DPC is: a) To prevent heat transfer b) To prevent dampness c) To increase strength d) To act as a foundation - A sectional view is used to show: a) The roof details b) Vertical details through a cut c) All external elevations d) Horizontal layout - The total volume of a 4 m × 3 m × 0.15 m slab is: a) 0.12 m3 b) 1.8 m3 c) 3.6 m3 d) 6 m3 - The number of bars required in a column footing is calculated by: a) Perimeter ÷ Bar spacing b) Length × Bar spacing c) (Total length ÷ Spacing) + 1 d) None of the above - Which is deducted while calculating plaster quantities? a) Window and door openings b) Floor tiles c) Beam areas d) None of the above - What does BOQ stand for in estimation? a) Balance of Quantities b) Bill of Quantities c) Base of Quotation d) Basis of Quality - What is the standard unit of measurement for structural steel members? a) Cubic meters b) square meters c) Metric tons d) All of the above - What is the formula for calculating the weight of a steel member? a) Volume × Density b) Length × Area × Density c) Weight = Cross-sectional area × Density × Length d) Volume ÷ Density - The term ‘Gusset Plate’ in a truss refers to: a) A member used for drainage b) A plate used to connect members c) A structural column d) A bolt in the truss - Structural steel painting quantity is calculated in: a) Square meters b) Cubic meters c) Kilograms d) Metric tons - Which of the following is commonly used to calculate the cost of steel in an estimate? a) Density of steel b) MRS/PWD schedule of rates c) Truss slope angle d) Painting area - What does the term "abstract of cost" refer to? a) Detailed breakdown of material and labor b) Summary of total cost of the project c) Only the labor cost of the project d) Painting cost summary - What is the primary purpose of a gutter in a steel shed? a) Drain water from the roof b) Strengthen the truss structure c) Support the roofing material d) Provide lateral stability - What is the role of purlins in a truss system? a) Support roofing sheets b) Provide drainage c) Connect truss to columns d) Act as vertical supports Answer Keys: 1; B; 11; C; 21; A. 2; D; 12; B; 22; B. 3; C; 13; B; 23; B. 4; D; 14; B; 24; A. 5; B; 15; C; 25; A. 6; A; 16; A. 7; A; 17; B. 8; D; 18; C. 9; D; 19; A. 10; B; 20; B. | - A bridge having span less than 6m is called: a) Culvert b) Bridge c) Causeway d) Syphon - End supports of bridge which are constructed parallel to the flow of water is called: a) Pier b) Abutment c) Wing Wall d) Toe Wall - Middle supports of bridge constructed parallel to the flow of water are called: a) Pier (b) Abutment (c) Wing Wall (d) Toe Wall - A bridge which is not at right angle to the direction of flow of water in plan is called. (a) Low Level (b) High Level (c) Skew (d) Deck - Abutment and piers are considered as ... (a) Sub-structure (b) Super structure (c) Base (d) Compulsory - Pre-stressed concrete beams ae suitable for bridges having. (a) Heavy loads (b) Longer span (c) Shorter span (d) Both A & B - End of the pier is designed triangular shape at upstream side, it is called (a) Cut water (b) Ease water (c) Round Nose (d) Up Nose - What is primary function of a culvert (a) To provide a passage for pedestrians (b) To allow for the free flow of water under roads or railways (c) To act as a bridge for vehicles (d) To support soil erosion prevention - Which type of material is commonly used for constructing a culvert? (a) Steel (b) Wood (c) Concrete (d) Plastic - Which of the following are the types of culvert. (a) Box culvert (b) Pipe culverts (c) Arch culvert (d) All of these - What type of culvert is best suited for areas with a high volume of traffic? (a) Slab culvert (b) Pipe culvert (c) Box culvert (d) Arch culvert - T-Beam and slab type bridges are suitable up to ....... meter span. (a) 20 (b) 25 (c) 30 (d) 35 - Walls constructed below the floor of a bridge and across the flow of water are termed as (a) Pier (b) Abutment (c) Wing Wall (d) Toe Wall - The clear distance between two supports is called…… (a) Span (b) Shorter span (c) Long span (d) None of these - A bridge that carries a roadway or rail traffic over a distance by suspending the deck from two long cables that run between two towers: (a) Cable stayed bridge (b) Suspension bridge (c) Concrete Bridge (d) Steel bridge - A water storage tank constructed below ground level is called ....... water tank. (a) Underground reservoir (b) High level reservoir (c) Surface reservoir (d) Over head reservoir - A water tank constructed at a reasonable height from ground level is called....... (a) Underground reservoir (b) High level reservoir (c) Surface reservoir (d) Over head reservoir - Elevated water tank is also termed as. (a) Underground reservoir (b) High level reservoir (c) Surface reservoir (d) Over head reservoir - A space left at the top of water in a storage tank is called as. (a) Open space (b) Free space (c) Free board (d) Depth board - A pipe provided at a maximum water level in a water storage tank to flow on exceeding the capacity of tank is called. (a) Over flow pipe (b) vent pipe (c) Blind pipe (d) Washout pipe Answer Keys: 1; A; 11; C. 2; B; 12; A. 3; A; 13; D. 4; C; 14; A. 5; A; 15; B. 6; D; 16; A. 7; A; 17; D. 8; B; 18; D. 9; C; 19; C. 10; D; 20; A. | - Software containing spread sheet. a) M.S word b) M.S Excel c) Power point d) Flash - Number of worksheets exist in M.S Excel file by default. a) 1 b) 2 c) 3 d) 4 - Cell address of column "F" and Row No. 15 is presented as; a) 15F b) F15 c) E15F d) F15E - In M.S Excel short cut key to move the cursor in cell A1 a) Shift + A b) Shift + home c) Ctrl + A d) Ctrl + home - In M.S Excel, cell E10 has a value of 45.9482. Putting the cursor in cell F10 giving a formula = Round (E10.2) the answer will be. a) 45 b) 45.948 c) 45.95 d) 45.94 - In MS Excel option in border tab. a) None b) Outline c) Inside d) All of these - F5 shortcut key displayed a) Go to dialogue box b) Search dialogue box c) Replace dialogue box d) All of these - In AutoCAD 2D Modelling, which axis is not accessible for drafting? a) X b) Y c) Z d) WCS - A Polyline can be broken into individual lines and arcs using which of the following command? a) BREAK b) TRIM c) EXPLODE d) OVERKILL - Origin of the AutoCAD drawing space is, a) 0,0 b) 1,0 c) 0,1 d) 1,1 - Scrolling of mouse can perform which following action? a) Zoom in / Zoom out b) Pan & Scan c) Extents / all d) Scale - Software Civil 3D is suitable for such project: (a) Road (b) Land Development (c) Pipe Network (d) All of these - Task carried out by Civil 3D: (a) Drawing work (b) Quantity Takeoff (c) Management (d) All of these - Type of data used in Civil 3D: (a) Static (b) Dynamic (c) Relative (d) None of the above - In Civil 3D, toolbox tab is used to: (a) Create data (b) Import Data (c) Reports (d) All of these - In Civil 3D data used in surface creation: (a) TIN (b) DEM (c) GIS (d) All of these - PC-I Performa is used for: (a) Planning (b) Survey (c) Progress (d) Evaluation - PC-II Performa is used for: (a) Planning (b) Survey and feasibility (c) Progress (d) Evaluation - PC-V Performa is used for: (a) Planning (b) Survey (c) Progress (d) Evaluation - The PC-II is prepared.......................... PC-I. (a) Before (b) After (c) at the same time (d) None of the above Answer Keys: 1; B; 11; A. 2; C; 12; D. 3; B; 13; D. 4; D; 14; A. 5; C; 15; C. 6; D; 16; D. 7; A; 17; A. 8; C; 18; B. 9; C; 19. 10; A; 20. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU- (1-2) | LU- (3-5) | LU- (6-8) |
| - Define Framed Structure building. - State the components of framed structure. - Enlist item of work of frame structure building. - What is meant by partition wall? - State items of work for surface rendering along with unit of measurement and unit of rate? - State items of work for doors & windows along with unit of measurement and unit of rate? - What is meant by premium? - What is meant by stucco plaster? - What is Truss? - State parts of Truss? - Define fabrication? - State the formula to calculate the unit area weight of iron plates? - Diffrentiate between ridge and valley? - Define purlin ? - Diffrentiate between tie and strut in truss. | - Define culvert? - State the different parts of RCC Slab culvert? - Enlist three type of culverts? - Define toe wall? - State the different parts of segmental arch culvert? - State the difference between bridge and culvert? - Enlist parts of substructure of Bridge? - Enlist parts of superstructure of RCC Bridge? - Why railing is provided in bridge? - What is meant by grating in bridge? - State the parts of underground masonry water tank? - State the parts of RCC overhead water tank? - Diffrentiate between Inlet and washout pipe. - Diffrentiate between Outlet and over flow pipe. - Define sump well? | - Define the worksheet functions in Excel? - Give the formula to add A1 to A10 in Excel. - What is conditional formating in Excel. - What is the formula in EXCEL to calculate the area of circle? - State the Interface of AutoCAD Civil 3D? - State the use of Templates, Settings, and Styles? - State how to create, edit, view, and analyze surfaces? - State Steps in designing a project in AutoCAD Civil 3D? - Give the use of civil 3D software. - Define static point in civil 3D - Enlist the method of editing alignment in civil 3D. - PC stand for ? - What is the meaning of PC-I. - Define the PC-II. - What is DNIT? |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU- (1-2) | LU- (3-5) | LU- (6-8) |
| Activity 1 Estimate the quantities of a one storey R.C.C frame structured building having two rooms, also prepare bar bending schedule and abstract of cost. Performance criteria: - Identify and interpret plan and sectional views in drawing. - Calculate quantities of excavation works. - Calculate quantities of lean and RCC column foundation concrete. - Calculate quantities of RCC column - Calculate quantities for Brick work and DPC. - Calculate quantities for beam and slab. - Calculate quantities for Surface Rendering. - Calculate quantities for Doors & Windows. - Calculate number of bars for each structure. - Calculate length of bars for each structure. - Calculate unit length weight for each dia. of bar. - Prepare table of bar bending schedule. - Prepare Bill of Quantities. - Identify MRS/PWD schedule of rates - Prepare abstract of cost. - Calculate premium/ contractor's profit on amounts of relevant rates. - Calculate contingencies and building services. - Calculate total cost of frame structure building Activity 2 Calculate the quantities of each item of a steel truss having span 30 ft and also calculate abstract of cost. Performance criteria: - Calculate quantities of steel structural members. - Calculate quantities of Structural plates and joints. - Calculate quantities for painting. - Prepare Bill of Quantities for steel structure. - Identify MRS/PWD schedule of rates - Prepare abstract of cost. - Calculate premium/ contractor's profit on amounts of relevant rates. - Calculate contingencies and building services. - Calculate total cost of steel structure. | Activity 1 Calculate the quantities of a RCC slab Culvert of span 15 ft. Performance criteria: - Identify and interpret plan and sectional views in drawing - Calculate quantities of all items of work for substructure for RCC Slab culvert. - Calculate quantities of all items of work for superstructure RCC Slab culvert. - Calculate quantities of all items of work for road structure RCC Slab culvert. - Prepare bar bending schedule of each structural member of RCC. - Prepare Bill of Quantities for culvert. Activity 2 Estimate the quantities two span R.C.C deck slab, length of each span is 25 ft. Performance criteria: - Identify and interpret plan and sectional views in drawing. - Calculate quantities of items of for work below bed level of bridge. - Calculate quantities of items of work for above bed level of abutments, wing walls & pier of bridge. - Calculate quantities of super structure of bridge. - Calculate quantities for TST Surfacing. - Prepare bar bending schedule of each structural member Prepare Bill of Quantities for bridges. Activity 3 Calculate the quantities of a Circular underground R.C.C water tank. Performance criteria: - Identify and interpret plan and sectional views in drawing. - Calculate quantities of excavation & foundation concrete for water tank. - Calculate quantities of RCC work for water tank. - Calculate quantities of flooring concrete & plaster for water tank. - Calculate quantities of iron steps, manhole cover inlet and outlet pipes for water tank. - Prepare bar bending schedule of each structural member - Prepare Bill of Quantities for water tank. | Activity 1 Prepare detail estimate of a R.C.C frame structured building having two rooms in MS Excel. Performance criteria: - Prepare abstract of quantities table in Excel. - Input data of measurements in formulated table. - Apply formulas for quantities calculations. - Prepare schedule of bars for columns, beams, slabs and lintels for framed structure class room. - Add quantity of each item of work and type relevant rates from MRS/ CSR in abstract of cost table. - Calculate amount by formula, sum up amounts and add premium, contractor profit. - Add contingencies and Work charges Activity 2 Use of Autodesk Civil 3D software for Design of a road project for 1 km length as per data collected from the civil engineering project. Performance criteria: - Open AutoCAD Civil 3D - Create new *.dwg file in desired units-feet or meters for road project - Save excel file in *.csv file format - Create formats for data import. - Import *.csv file in CAD file. - Create, edit and print parcels for project. - Create points and point groups and work with survey figures for project. - Create surfaces, edit surface, view and analyse surface for project. - Create and edit horizontal and vertical alignments for project. - Create data shortcuts for project. - Create sites, profile, cross-section, assemblies, corridors, and intersection for project. - Create Grading solutions for project. - Create gravity fed pipe and pressure pipe networks for project. - Perform quantity take of project. - Perform volume calculations for project. - Create plan and profile sheets by use of plan production tools for project. |