Course / Subject Name:  Civil 283- Quantity Surveying

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU- (1-4) | LU- (5-8) | LU- (9-12) |
| - An assessment of the cost based on certain rates of materials and labour is called: (a) Quantity surveying (b) Surveying (c) Mapping (d) a & b - A rough calculation on quantities of various works & their expenditure. (a) Drawing (b) Estimate (c) Specification (d) All of these - Data required for preparation of estimate: (a) Drawing (b) Rates of items (c) Specification (d) All of these - In SI system unit of measurement for length is. (a) Meter (m) (b) Inch (in) (c) Foot (ft) (d) Centimeter (cm) - One ft is equal to: (a) 2.54 cm (b) 30.48 cm (c) 12 cm (d) 0.83 cm - Volume of a rectangular solid is: (a) V = l × w × h (b) V = πr² x h (c) V = 2πr2 x h (d) None of these - In the case of shape wall, the Length of each step of perpendicular wall remains constant. (a). H (b) U (C) O (d) E - Which is the unit of payment of DPC. (a) Cubic feet (b) Square feet (c) 100 cubic feet (d) 100 square meter - Amount for the contingencies is taken of building cost. (a) 3-5% (b) 3-49% (c) 1-2% (d) 2-3% - Such estimate is prepared to get administrative approval. (a) Rough Cost (b) Detailed (c) Revised (d) All of these - Such estimate is prepared to get technical sanction. (a) Rough Cost (b) Detailed (c) Revised (d) All of these - Length, breadth, heights are written combined in this method. (a) P.W.D. (b) English (c) Centre line (d) Separate wall - Long and short wall method of estimating is also called . (a) Centre line (b) Revised (c) Supplement (d) Out to out and in to in - For circular or polygonal shaped building with symmetrical section of wall this method of estimating is most suitable. (a) Centre line (b) Crossing (c) Supplementary (d) Out to out and in to in - Area occupied by this is included in the plinth area estimate. (a) Walls (b) sunshade (c) Courtyard (d) All of these - The measuring unit of brick on edging is (a) cft (b) sft (c) m (d) cu.m - What is the main purpose of an Annual Repair Estimate in construction projects? (A) To estimate the cost of initial construction (B) To estimate the cost of major renovations (C) To estimate the cost of routine and periodic maintenance activities (D) To estimate the cost of utilities and service charges - Which of the following is typically considered a Special Repair? (a) Repainting of walls after 5 years (b) Fixing a leaky faucet (c) Replacing the roof after storm damage (d) Servicing air conditioners annually - Annual repair estimate is type of....... - Rough cost estimate - Detailed estimate - Annual estimate - All of these - Special repair estimates are made in the same way as........ - Rough cost estimate - Detailed estimate - Special Estimate - None of these Answer Key: 1; A; 8; B; 15; A. 2; B; 9; A; 16; B. 3; D; 10; A; 17; C. 4; A; 11; B; 18; C. 5; B; 12; A; 19; B. 6; A; 13; D; 20; B. 7; D; 14; A. | - Horizontal distance between centroids of the sections of cutting and filling is termed as. (a) Interval (b) I.M.P (c) Lift (d) Lead - More accurate method of calculating earth work is (a) Mean depth (b) Mean area (c) Prismoidal (d) Graphical - The point on which value of depth in cutting or height of filling is zero (a) Interval (b) Lead (c) Lift (d) I.M.P - X Section of a road at which area of cutting and filling both exists is (a) Cutting (b) Partially in cutting & Filling (c) Filling (d) None of these - For earth work unit of measurement is. (a) m2 (b) Km ( C) m3 (d) m - Which is the formula of mean depth? (a) d1+d22 (b) d1-d22 (c) d1xd22 (d) d1+d2d - A prepared soil surface which receive traffic load from structure: (a) Sub grade (b) Sub base (c) Base (d) Toping - The compulsory part of road structure, to protect the subgrade: (a) Sub grade (b) Sub base (c) Base (d) Toping - Transverse slope of road appearing in X-section: (a) Cross slope (b) Country slope (c) Camber (d) Gradient - Longitudinal slope of a road is called: (a) Cross slope (b) Country slope (c) Camber (d) Gradient - Construction and expansion joints are provided in case of such road: (a) Earthen (b) WBM (c) Flexible (d) Rigid/ Concrete - Screening (stone dust) required for macadam base: (a) 10 % (b) 15 % (c) 20 % (d) 25% - A table showing quantity of materials required for a particular work: (a) Material statement (b) Estimate (c) Analysis of rates (d) All of these - Cost of any item is determined per unit quantity of work, by: (a) Estimate (b) Analysis of rates (c) Quotation (d) Tender - The volume of one bag of cement in M.K.S system is: (a) 0.25 Cum (b) 0.025 Cum (c) 0.35 Cum (d) 0.035 Cum - Volume of one bag of cement is: (a) 1.25 Cft (b) 1.50 Cft (c) 1.75 Cft (d) 2.00 Cft - Weight of one bag of cement is: (a) 40 Kg (b) 50 Kg (c) 55 Kg (d) 60 Kg - Dry volume of concrete is always more than its wet volume, by: (a) 1.45 % (b) 1.54 % (c) 1.64 % (d) 1.50 % - Bricks will be required for one Cum masonry: (a) 500 No. (b) 550 No. (c) 600 No. (d) 620 No. - Number of bricks taken in 100 CFt masonry work: (a) 750 (b) 1050 (c) 1350 (d) 1550 Answer Key: 1; A; 8; C; 15; D. 2; C; 9; C; 16; A. 3; D; 10; D; 17; B. 4; B; 11; D; 18; B. 5; C; 12; C; 19; A. 6; A; 13; A; 20; C. 7; A; 14; B. | - Which of the following types of water supply systems is commonly used in residential buildings for direct water distribution from the municipal system? (a) Borewell water supply (b) Overhead tank water supply (c) Hand pump water supply (d) Rainwater harvesting system - Which of the following is NOT typically considered a sanitary fixture in a modern bathroom? (a) Washbasin (b) Bathtub (c) Showerhead (d) Ceiling fan - Which type of pipe is primarily used for drainage and sewer systems due to its durability and corrosion resistance? (a) Copper pipes (b) PVC pipes (c) Brass pipes (c) Galvanized steel pipes - Which of the following is the items of water supply system. (a)Pipes (b) valves (c)Storage tank (d) All of these - What is the standard unit of measurement for laying of water supply pipes in a trench? (a) Cubic meter (m³) (b) Linear meter (lm) (c) Square meter (m²) (d) Kilogram (kg) - Estimation of water supply scheme is based on. (a) Poppulation (b) No. Houses (c) Both a & b (d) None of these - The tank which collect, store and treat waste water from household is called. (a) Water tank (b) Septic Tank (c) Overhead tank (d) All of these - The manhole constructed to join the sewer lines of different levels: (a)Deep manhole (b) Shallow manhole (c)Large manhole (d) Lamp manhole - Hydraulic statement shows: (a) Levels (b) Velocity (c) Discharge (d) All of These - Longitudinal slope of a sewer line is called: (a) Cross slope (b) Country slope (c) Gradient (d) Camber - Construction work of sewer line should be started from: (a) Start point (b) End point (c) Middle (d) Any where - Lowest level inside the sewer pipe is called: (a) Invert Level (b) Bed Level (c) Minimum Level (d) Crest Level - Flushing cisterns are used for flushing out: (a) Water closet (b) Urinal (c) Kitchen sink (d) a & b - Working chamber & access shaft are the parts of: (a) Septic tank (b) Soakage pit (c) Deep manhole (d) Gully trap - Traps are used to maintain the: (a) Floor level (c) Water seal (c) Drainage (b) Flushing - The method of calculating the present marketable cost of a building: (a) Quantity surveying (b) Surveying (c) Valuation (d) a & b - Valuation of a building depends on. (a) Location (b) Size (c) Structure (d) All of these - The value of dismantled materials: (a) Scrap value (b) Book value (c) Salvage value (d) Market value - The gradual exhaustion of the usefulness of a property: (a) Depreciation (b) valuation (c) Taxation (d) None of these - The rate of depreciation is depends on: (a) Initial condition (b) Quality of maintenance (c) Mode of use (d) All of these Answer Key: 1; B; 8; A; 15; B. 2; D; 9; D; 16; C. 3; B; 10; C; 17; D. 4; D; 11; B; 18; A. 5; B; 12; A; 19; A. 6; A; 13; D; 20; D. 7; B; 14; C. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-4) | LU (5-8) | LU- (9-12) |
| - What is quantity surveying? - Enumerate the types of estimates. - Enlist the data required for detailed estimation. - What is rough cost estimate? - Enlist the systems of measurement. - Differentiate between FPS and SI system. - Calculate cost of construction work. if plinth area is 550 sqm, and the rate is Rs. 4850/sqm. - What is schedule of rates. - What is contingencies. - What is difference between composite and labour rate? - What is bill of quantities (B.O.Q) - Enlist six items of special repair estimate. - What is annual repair estimate and special repair estimate. - Enlist five items of work of a building, estimate. - When specail repair estimate is carried out. | - What is earth work - What is lead and lift in earth work - Enlist the methods of calculating earth work in plain area. - Write the formula for calculation of earth work quantity by mean area method. - What is economical irrigation channel. - Enlist components of road structure. - What is camber in road - What is gradient in road - Deferentiate between flexible and rigid pavements - What is service road - What is material statement - What is rate analysis. - What schedule of labor? - How much is the contractor’s profit and overhead charges in rate analysis? - What is non-schedule item? | - Differentiate between water supply and sewerage system. - Differentiate the sewage and sewerage. - Enlist item of work for water supply system. - How rough cost estimation of water supply is carried out. - Enlist the work items for a manhole. - Enlist the types of trap - Differentiate septic tank and soakage pit. - Enlist the sanitary items for bath room. - Enlist the G.I specials for water supply line. - Enlist types of sewerage system. - Enumerate the purposes of valuation - What is sinking fund? - Differentiate between scrap value and salvage value. - Diffrentiate between market value and book value. - Enumerate the methods of calculating depreciation. |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-4) | LU (5-8) | LU- (9-12) |
| Activity Prepare rough cost and detailed estimate of a single room of 12 ft by 14 ft and ceiling height is 12 ft. Cross section of the wall is given. Performance Criteria: - Identify drawing details of proposed room - Calculate covered area of building - Prepare rough cost estimate - Select type of walls - Prepare BOQ,s - Select MRS/PWD rates of qualities as per specifications - Take measuremnts from given plan and section - Sum up lenghts of centre lines by centre line method - Submit report to the concerned person | Activity 01 Prepare detailed estimate of 1 kilometer urban road as per given specification and cross-section including earth work calculations. Performance Criteria: - Select method of calculation of earth work i.e mean area, mid sectional area and prismoidal formula method. - Identify data of NSL, FL and calculate depth of filling. - Calculate x-sectional area at each RD as per selected method. - Calculate volume of earth work filling / cutting. - Calculate volume of earth work. - Calculate volume of sub-base and base course of road. - Prepare BOQ of road. - Prepare abstract of cost. Activity 02 Prepare rate analysis of concrete for 100 cubic meters Performance Criteria: - Calculate quantities of items of work - Calculate cost of material. - Calculate carriage of material. - Calculate labor cost. - Calculate total cost. - Calculate contingencies. - Calculates contractors' profit. - Calculate total cost and unit cost. | Activity 01 Estimate a water supply and sewerage scheme of 10 houses of 1-kanal area each. Performance Criteria: - Identify water supply sanitation item of works for building - Calculate quantity of fitting and fixtures as per given drawing - Calculate length of pipping as per given drawing - Taking out quantity for manhole and septic tank - Prepare BOQ - Identify MRS/PWD rates for items of work. - Prepare abstract of cost. - Calculate quantity of excavation for sewerage work. - Taking out length of each dia pipe. Activity 02 Calculation of standard rent of government buildings Performance Criteria: - Calculate capitalized value - Calculate annual / monthly rent |