Course / Subject Name: NQ-Civil-204 Civil Engineering Survey II

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU1 | LU2-LU4 | LU5 TO LU8 |
| - Prepare gale’s traverse table - Latitude is determined by the formula: - L x Sinθ - L x Cosθ - L / Sinθ - L / Cosθ - The formula to calculate the departure of a survey line is: - l sin θ - l cos θ - l tan θ - l sin θ / cos sin θ - Value of departure towards West/Left-side is considered as: - Positive - Negative - True - Any one - Sum of external angles of any closed traverse is: - (2n−4)90 - (2n+4)90 - (4−2n)90 - (4+2n)90 - In Gales table, all points of traverse are brought in the quadrant: - 4th - third - First - 2nd - In closed traverse the algebraic sum of departure is: - 90 - 0 - 180 - 360 - Latitude and Departure are both negative in the quadrant: - SE - NE - SW - NW - Balancing of a traverse is done according to the: - Transit rule - Prismoidal rule - Trapezoidal rule - All of these - The closing error in a closed traverse is adjusted by: - Lehmann's Rule - Bwoditch Rule - Slide Rule - Angle Rule - Sum of internal angles of hexagonal traverse is: - 540 - 720 - 900 - 1200 - Algebraic sum of internal angles of five sided closed traverse is: - 540 - 360 - 180 - 90 - In closed traverse the algebraic sum of latitude is: - 90 - 0 - 180 - 360 - Sum of internal angles of any closed traverse is: - (2n−4)90 - (2n+4)90 - (4−2n)90 - (4+2n)90 - Positive departure is also called: - Northing - Southing - Westing - Esting - Positive latitude is also called: - Northing - Southing - Westing - Esting Answer key. 1; D. 2; A. 3; B. 4; B. 5; C. 6; B. 7; C. 8; A. 9; B. 10; B. 11; A. 12; B. 13; A. 14; D. 15; A. | - Perform operations of total stations - Set out horizontal and vertical curves - Set out civil engineering structure - Coordinates of a point are measured with: - Theodolite - Total station - Distance meter - Level - Total station can measure: - Angles - Distance - Both of these - None of these - The Height of a building is measured with total station in a program. - MLM - REM - EDM - RED - In a co-ordinate (N,E,Z) leveling component is - Z - E - N - All of these - Function of optical plummet. - Centering - Over hanging - Levelling - None of these - Radius of one degree curve is: - 1720 m - 1500 m - 1000 m - 100 m - The straight line joining tangent points of a curve is called: - Tangent length - Curve length - Long chord - Straight length - The curve which is introduced at the point of change of gradient is called: - Simple curve - Transition curve - Vertical curve - Compound curve - On reducing the radius of a curve the value of super-elevation will: - Increase - Decrease - Not change - Both a and b - The sum of intersection angle and deflection angle. - 180 - 270 - 120 - 360 - Curve of varying radius is called. - Simple curve - Compound curve - Transition curve - Vertical curve - First tangent point is also called: - Curve point - Initial point - Tangent point - Start points - Permissible value of centrifugal ratio for roads is: - 1/8 - 1/2 - 1/4 - 1/6 - Formula 5730/D gives: - Radius of curve - Central angle - Tangent length - Deflection angle - Super elevation is also called: - Radius of curve - Cantt - Degree of curve - None of these - Vertical curve is designed on the bases of - Radius of curve - Minimum sight distance - Change of gradient - Speed of car - In a circular curve the second tangent point is also known as the - Forward point - Point of tangency (tangent point) - End point - Closing point - The centre of a curve is called: - Intersection point - Apex - Vertex - Mid point - The term V2/gR is known as. - Centreifugal force - Centrifugal ratio - Inertia - Torque - Super elevation for road is: - bV2 / gR - GV2 / gR - gV2 / bR - bV / gR - Marking on ground for excavation according to plan of the structure is termed. - Layout - Alignment - Marking - Setting out - Small temporary wall or fixing board in front of walls for guidance during setting out for any structure is called. - Sign board - Profile - Dwarf wall - Toe-wall - During checking the setting out angle of each corner should be of ....... degree. - 30 - 60 - 90 - 120 - During checking the setting out the both diagonals of rectangle should be - Small - Long - Equal - Parallel - With notations carrying their usual meanings, the super elevation (h) for railways is given by the relation. - bV2 / gR - GV2 / gR - gV2 / bR - bV / gR Answer key:. 1; B. 2; C. 3; B. 4; A. 5; A. 6; A. 7; C. 8; C. 9; A. 10; A. 11; C. 12; A. 13; C. 14; A. 15; B. 16; C. 17; B. 18; B. 19; B. 20; A. 21; A. 22; B. 23; C. 24; C. 25; B. | - Perform operations of GPS - Perform hydrographic survey - Interpret data of remote sensing - Analyze topography by ArcGIS - GPS means. - Geographical positioning system - Geo positioning system - Global positioning system - Great positioning system - Global positioning system (G.P.S) work with the help of - Magnet - Total Station - Satellites - Internet - True bearing is observed with - Prismatic Compass - Theodolite - Total Station - G.P.S - G.P.S is used to observe - Magnetic bearing - True bearing - Absolute location - All of these - True co-ordinates can be observed with. - Magnetic Compass - G.P.S - Auto level - Theodolite - A device used for navigation. - Auto Set level - Theodolite - G.P.S - Total Station - G.P.S can recieve signals through. - Metals - Concrete - Glass - All of these - Hydrographic survey is carried out for mapping of; - Boundaries - Plain areas - Mountainous areas - Water bodies - The process of measurement of depth below the water surface iscalled: - Levelling - Sounding - Ranging - Gauging - Propeller is the part of an instrument used in hydrographic survey - Fathometer - Pilot tube - Sounding machine - Current meter - In hydrographic survey current meter is used to measure: - Discharge of water - Current of water - Area of water - Velocity of water - Pole used to determine sounding is called: - Sounding rod - Ranging rod - Offset rod - Depth pole - Length of sounding rod varies from: - 5 to 8 m - 7 to 10 m - 9 to 12 m - 11 14 m - Lead line is used for measuring: - Levelling - Sounding - Bearing - Horizontal distance - In hydrographic survey, float is used to determine: - Velocity of water - Depth of water - Quality of water - Volume of water - Unit of discharge is: - m/sec - Cumsec - m2/sec - m3/min - Remote sensing is a __________ recording of information. - Contact - Non - contact - Both A and B - None - ______remote-sensing instruments transmit their own electromagnetic waves and then develop images of the earth’s surface as the electromagnetic pulses (known as backscatter) are reflected backfrom the target surface. - Active - Passive - Both - None of these - ____ remote-sensing instruments develop images of the ground surface as they detect the natural energy that is either reflected or emitted from the observed target area. - Active - Passive - Both - None of these - GIS captures and analyses __ data. - Spatial - Geographic - Both a and b - None of the above - Which of the following is a GIS operation? - Geodata - Geoprocessing - Global processing - None of the above - What is the function of airborne imagery? - Takes photographs from aircraft - Observes aircraft - Monitors data - All the above - Which of the following are the types of aerial photography? - Oblique - Vertical - Horizontal - Both a and b - If an oblique photograph is taken at a low angle relative then it is called as ____ oblique. - High - Low - Perpendicular - None of the above - Which of the following are the applications of vertical photography? - Photogrammetry - Image interpretation - Both a and b - None of the above Answer key. 1; C. 2; C. 3; D. 4; D. 5; B. 6; C. 7; C. 8; D. 9; B. 10; D. 11; D. 12; A. 13; A. 14; B. 15; A. 16; B. 17; B. 18; A. 19; B. 20; C. 21; B. 22; A. 23; D. 24; B. 25; C. |

B. Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| - What is meant by coordinates? - What is latitude? - What is departure? - Give the formulae to find the latitude and departure. - Enlist methods of plotting a traverse survey. - What is balancing of traverse? - Define Gale’s traverse table. - What are the sign conventions of latitude and departure? - State Bowditch rule for balancing the traverse. - Define closing error of traverse. | - What is total station? - Name the types of total station. - Name the parts of total station. - What is prism? - Name the types of prism. - What is meant by stack out measurement of distances. - What is meant by offset mode of measurement? - Name the methods to measure horizontal angles. - What is horizontal curve. - Name the types of horizontal curves. - What is simple circular curve? - What is combined curve? - What is reverse curve? - What is transition curve? - Name the components of curve. - What is intersection point? - What is deflection angle? - What is intersection angle? - What is the sum of intersection and deflection angle? - What is tangent points? - What is tangent length? - What is long chord? - What is degree of curve? - What is radius of curve? - What is length of curve? - What is apex distance? - What is the relation between degree of curve and radius of curve? - What is the formula to calculate tangent length? - What is the formula to calculate length of long chord? - What is the formula to calculate length of curve? - What is the formula to calculate rise? - What is the formula to calculate apex? - How chainage of first tangent point is calculated? - How chainage of second tangent point is calculated? - Name the methods to lay out curve. - Name the methods to layout curve by using only linear measurement. - Name the methods to layout curve by both linear and angular measurements. - Give formula to calculate deflection angle of chord. - How setting out angle is computed? - What is vertical curve? - Name the types of vertical curve. - Name the components of vertical curve. - What is summit / crest vertical curve? - What is sag vertical curve? - What is vertex? - What is gradient? - What is the difference between positive and negative gradient? - What is rate of change of gradient? - Give formula to calculate length of vertical curve. - Name the methods to set out vertical curve. - What is setting out of structure? - What is profile board? | - Enlist the modes of GPS - Enlist six parts of GPS - What is initialization of GPS? - What are the main functions of menu mode of GPS? - What are the main modes of compass mode of GPS? - What are the main modes of map mode of GPS? - What are the main modes of trip mode of GPS? - What are the main modes of location mode of GPS? - What is hydrographic survey? - What is sounding? - What is sounding rod? - What is sounding chain? - What is fathometer? - What is lead line? - Name different types of float. - What is float? - What is current meter? - Name different parts of current meter. - Name different methods of sounding. - What is discharge? - Differentiate between cumecs and cusecs. - What is remote sensing? - Enlist techniques used in remote sensing? - What is photographic scale? - Enlist types of data in GIS? |

C. Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Assesment task: - Prepare Gale’s traverse table taking data of already done field survey, compute the coordinates and adjust closing error and plotting of traverse. Performance Criteria: - Identify and compile data of traversing by theodolite - Compute coordinates of traverse - Adjust closing error of traverse - Prepare Gale’s traverse table - Plot traverse using data of traversing | Assesment task: - Setout a simple circular horizontal curve in the field by total station by computing coordinates of suitable data of curve according to field requirments. Performance Criteria: - Identify componentsof horizontal curves - Compute data for layout of simple circular curve by total station - Set out horizontal curve in field by total station | Assesment task: - Analyze the topography to identify suitable areas for development and potential drainage issues by data obtained through GPS. Performance Criteria: - Import data from Total Station/ GPS in ArcGIS software. - Input attribute data in ArcGIS software. - Analyze topography to identify suitable areas for development and potential drainage issues. |