Course / Subject Name- NQ-Civil-124 Civil Engineering Surveying-I

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU 1 | LU 2 - LU7 | LU 8 - LU 12 |
| - The curvature of earth is considered in: - Plane surveying - Geodetic surveying - Hydrographic surveying - Ariel surveying - Surveying which is conducted in order to determine the boundaries offields, estates etc. is called: - Cadastral surveying - Topographical surveying - Engineering surveying - City Surveying - Surveying which is done to prepare detailed drawings of projects involvingroads, railways etc. is called: - Cadastral surveying - Topographical surveying - Engineering surveying - City Surveying - Ranging carried out when terminal points of a survey line are visible iscalled: - Direct ranging - Indirect ranging - Reciprocal ranging - Line ranging - Invar tape is made of an alloy of: - copper and steel - brass and steel - brass and nickel - steel and nickel - The correction for sag is: - always additive - always zero - always subtractive - both additive and subtractive - A survey conducted along a narrow strip of territory for the location design and construction of canal/roads is known as: - Geographical Survey - Topographic survey - Route survey - Hydrographic survey - Surveys which carried out to provide a national grid of control for thepreparation of accurate maps of large area are known as: - Plane surveys - Triangulation surveys - Geographical surveys - Topographical surveys - Precisely the shape of earth is: - Spheroid - Ellipsoid - Oblate spheroid - Oblate ellipsoid - A survey which consists of horizontal and vertical location of certainpoints by linear and angular measurements, and is made to determine thenatural features of country is known as: - Geodetic survey - Cadastral survey - Topographical survey - City survey - Cross-staff is used for: - taking perpendicular offsets - taking oblique offsets - measuring horizontal angles - measuring the bearing of lines - Oblique offsets are used to: - locate broken boundaries - check the accuracy work - locate boundary of property - locate the control points - Offsets may be taken by setting the right angle in the ratio of: - 1: 2: 3 - 2: 3: 4 - 3: 4: 5 - 4:5:6 - Slope correction is denoted: - always additive - always zero - always subtractive - both additive and subtractive - Formula for correction of applied pressure in base line measurement: - (Po-P)AEx L - (Po+P)AE x L - (P-Po)AE x L - (P-Po)AE x L Answer Key: B. A. C. A. D. B. B. C. B. C. A. B. C. C. C. | - An arbitrary surface with reference to which the elevation of points is measured and compared is called: - Datum Surface - Level Surface - Horizontal Surface - Vertical Surface - First staff reading taken after very setting up of instrument is called: - Inter sight - Fore sight - Back sight - Change sight - Height of a point in the vertical plane measured above or below the datumis called: - Change point - Station point - Reduced level - Reduced distance - A fixed Point of reference of known elevation is called: - Change point - Station point - Datum - Bench mark - The apparent movement of the image relative to the cross-hair is called: - Magnification - Focusing - Parallax - Tilting - Bench mark established at the end of a work day or when there is a breakin the work is called: - G.T.S B.M - Permanent B.M - Temporary B.M - Arbitrary B.M - An imaginary line passing through the intersection of crosshairs at thediaphragm and optical centre of object glass is called: - Line of collimation - Axis of bubble tube - Axis of telescope - Line of bubble tube - The sensitiveness of a bubble tube can be increased by: - Decreasing dia. of observer - Increasing viscosity of liquid - Increasing the roughness of the walls of the tube - Increasing the radius of curvature of the tube - The rise and fall method for obtaining the reduced level of points provides a check on: - Fore sight - Back sight - Intermediate sight - all sights - Cross sectioning and longitudinal sectioning are usually involved in: - Check levelling - Differential levelling - Simple levelling - Profile levelling - In reciprocal levelling, the error is not eliminated completely: - Due to earth’s curvature - Due to refection - Due to non-adjustment of line of collimation - Due to non-adjustment of the bubble tube - Staff reading is made clear by rotating: - Foot screw - Eye piece - Micrometer screw - Focusing screw - The specific part of tilting level is: - Foot screw - Micrometer screw - Clamping screw - Focusing screw - The method of levelling in which the height of mountains is found by observing the temperature at which water boil is known: - Barometric levelling - Reciprocal levelling - Longitudinal levelling - Hypsometry - The height of instrument is equal to: - Reduced level of bench mark plus back sight - Reduced level of bench mark plus fore sight - Back sight plus fore sight - Back sight plus fore sight minus intermediate sight - The collimation method for obtaining the reduced level of points does not provide a check on: - Fore sight - Back sight - Intermediate sight - Change point - To find the true difference of level between two points, the level should be kept: - At either of the two points - At any point on which the line joining these two points - Exactly midway between these two points - All of these - The method of surveying used for determining the relative heights ofpoints on the surface of the earth is called: - Longitudinal Levelling - Levelling - Contouring - Precise Levelling - A point in levelling where both back sight and fore sight are taken iscalled: - Reference point - Station point - Instrument point - Change point - In this level telescope can neither removed from its socket and nor it canbe revolved: - Tilting level - Auto set level - Cook’s level - Cushing’s level - An imaginary line passing through point of equal elevation is called: - Datum line - vertical line - Horizontal line - Contour line - The boundary line of still water in a lake indicates: - Horizontal surface - Contour line - Water line - Datum line - The process of tracing of contour lines on the surface of earth is called: - Levelling - Contouring - Surveying - Tachometry - The horizontal distance between two consecutive contours is called: - Horizontal equivalent - Peg interval - Reduced level - Contour interval - The vertical distance between two any consecutive contours is called: - Horizontal equivalent - Peg interval - Reduced level - Contour interval - Close contour lines on a contour map represent: - Uniform slope - Flat ground - Steep ground - Cliff - Contour lines cross ridge lines at: - Acute angle - Right angle - Negative angle - Obtuse angle - A map showing contour lines is known as: - Index map - Topographic map - Contour map - Relief map - 29.Direct method of contouring is: - Economical - Durable - Accurate - Fast - When contours of different elevation cross each other, it indicates a/an: - Vertical cliff - Over hanging cliff - Saddle - Steep slope Answer Key: A. C. C. D. B. C. A. C. A. D. A. C. D. C. C. D. D. B. D. D. D. B. B. A. D. C. B. C. C. B. | - The principle of compass surveying is: - Triangulation - Trilateration - Traversing - Parallelism - The true meridian passes through: - Magnetic poles - Geographical poles - Arbitrary poles - Graphical poles - A set of straight lines joining the various survey stations is called: - Triangle - Rectangle - Traverse - Quadrilateral - The function of theodolite is to measure: - Horizontal Angle - Vertical Angle - Deflection Angle - All of these - The size of theodolite is expressed by: - Dia of main scale - Length of telescope - Weight of theodolite - Magnifying power - The other name of horizontal axis of theodolite is: - Z-axis - Trunnion axis - Zero axis - None of these - The axis of rotation of telescope in the horizontal plane is called: - Horizontal axis - Vertical axis - Trunnion axis - None of these - Transiting of a theodolite is also called: - Reversing - Plunging - Swinging - Inverting - The theodolite which shows readings in digits is called: - Vernier - Micrometer - Digital - All of these - Number of horizontal cross hairs in stadia diaphragm is: - 1 - 2 - 3 - 4 - Rotating telescope of theodolite through 180 in vertical plane is called: - Rotating - Transiting - Centring - Swinging - The theodolite whose telescope can rotate through complete revolution in vertical plane is called: - Non-transit - Transit - Tilting - All of these - The face left position of theodolite is also called: - Telescope inverted - Telescope normal - Telescope reversed - None of these - 14.When telescope bubble of theodolite is upward it is called: - Telescope normal - Face left - Face right - Both a and b - When the bubble of telescope of the theodolite is downward, it is called. - Telescope normal - Face left - Telescope inverted - Diaphragm - Reticles of a telescope are cleared with: - Foot screws - Object glass - Eye piece - Focusing screw - Function of optical plummet of theodolite: - Centering - Sighting - Set in case - None of these - At the time of measurement of angle, bubble axis of theodolite should be at such condition with respect to its vertical axis: - Coincident - Parallel - Normal - Any one - The equation used to represent pythagoras theorem is: - Hyp= Base + Perp. - Hyp=Base – Perp. - Hyp2=Base2 + Perp2. - Hyp2=Base2 – Perp2. - Formula for finding perpendicular in triangle is: - tanø = Perp. x Base - Base = tanø x Perp. - Perp. =tanø x Base - None of these Answer Key: c. b. c. d. a. b. b. a. c. c. b. b. b. d. c. c. a. c. c. c. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| 1. Write two purposes of surveying. 2. What is plane surveying? 3. What is geodetic surveying? 4. What are the units used for linear measurements in FPS and MKS system? 5. What is topographical map? 6. What is cadastral map? 7. Enlist classification of survey based on instruments used. 8. Enlist types of chain used in surveying. 9.Enlist types of measuring tapes used in surveying. 10. Enlist the type of corrections made to remove error in measurement of base line. 11. Give formula for correction of absolute length during base line measurement 12.Give formula for temperature correction during base line measurement. 13. Give formula for sag correction during base line measurement. 14. Give formula for pull correction during base line measurement. 15.Give formula for mean sea level correction during base line measurement | 1. What is Levelling? 2. What is Level Line? 3. What is Horizontal line? 4. What is Plumb line? 5. What is Datum Surface? 6. What is Reduced Level? 7. Differentiate between back sight and fore sight readings. 8. What is Bench Mark? 9.Enlist types of Bench Mark. 10. What is Contouring? 11. What is Contour map? 12. Enlist types of contour. 13. Differentiate between ridge and valley contours. 14. Enlist types of methods of contouring. 15. Write two principles of drawing contour lines. | 1. What is Compass? 2. What is Compass Survey? 3. What is principle of compass survey? 4. What are the types of compass? 5. What is Meridian? 6. What are the types of meridian? 7. What is true meridian? 8. What is Bearing? 9.Enlist types of bearing. 10. What is Azimuth? 11. Differentiate between fore bearing and back bearing. 12. What is observed bearing? 13. What is calculated bearing? 14. What is Whole circle Bearing? 15. What are angles and its types? 16. What is Theodolite? 17. Enlist types of theodolite. 18. What are the parts of theodolite?. 19. Differentiate between Left face and right face of theodolite. 20. Enlist types of methods of contouring. 21. Write two principles of drawing contour lines. 22. What is trigonometric leveling? 23. What is vertical angle? 24. What do you know about line of collimation? 25. What is traverse? 26. Enlist the types of traverse. 27. Enlist different methods of traversing. 28. Enlist the methods of checking a traverse. |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Assesment task: - Measure baseline using tape and apply taping corrections Performance Creteria: - Select start and end points of line. - Identify and select appropriate tool to be used for measurement. - Range line for measurement according to SOP - Take measurements by measuring tape - Apply taping corrections to measurements, if required | Assesment task: - Perform profile leveling and cross section leveling on a pre-marked center line of a route 200 ft long at equal interval of 25 ft and 10 ft offset on each side Performance Creteria: - Adjust Level at suitable point - Locate temporary benchmark. - Observe first reading on Bench Mark to initiate work - Take readings at every specified interval of existing track (Longitudinally) - Observe staff reading on specific interval starting from center line to right & left sides and record in field book - Continue staff readings on every Reduced Distance of specified interval with same pattern (center, Left & Right) of given line - Calculate Reduced Levels - Compile data obtained from cross sectioning and long sectioning | Assesment task 1: - Perform traversing by digital theodolite of a closed traverse consisting of 5 sides Performance Creteria: - Mark station points to cover given area - Perform temporary adjustment oftheodolite on startingpoint of traverse - Measurebearing of first line - Measurehorizontal internal / external angle between all lines by shifting theodolite on each station - Measure distances between station points - Compute bearings of each line using internal / external angles. Assesment task 2: - Measure the height of building by digital theodolite. Performance Creteria: - Mark station points for accessible base point - Perform temporary adjustment of theodolite on selected station point - Set theodolite for vertical angle measurement - Take reading on benchmark - Measure and record vertical angle of point for elevation - Measure horizontal distances by stadia method/tape from station according to requirement - Calculate elevation of point by trigonometry. |