Course / Subject Name: NQ-Civil-124 Civil Engineering Survey I

Assessment Tasks (Summative Assessment)

| Assessment Task 01 |  |  | Perform profile leveling and cross section leveling on a pre-marked center line of a route of specified length, interval and offset on each side as per the data assigned by assessor Note: Assessor should provide the suitable dimensions according to the given time frame. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Time Required |
|  | Carry out OHS requirments |  |  |  |  | 03 hours |
|  | Adjust Level at suitable point |  |  |  |  |  |
|  | Locate temporary benchmark. |  |  |  |  |  |
|  | Observe first reading on benchmark to initiate work |  |  |  |  |  |
|  | Take readings at every specified interval of existing track (Longitudinally) |  |  |  |  |  |
|  | Observe staff reading on specific interval starting from center line to right & left sides and record in field book |  |  |  |  |  |
|  | Continue staff readings on every Reduced Distance of specified interval with same pattern (center, Left & Right) of given line |  |  |  |  |  |
|  | Calculate Reduced Levels |  |  |  |  |  |
|  | Compile data obtained from cross sectioning and long sectioning |  |  |  |  |  |
| Materials Required |  |  |  |  |  |  |
| Sr. |  | Equipment Tool |  |  |  |  |
| 1 |  | Tripod stand |  |  |  |  |
| 2 |  | Auto level |  |  |  |  |
| 3 |  | Measuring tape |  |  |  |  |
| 4 |  | Levelling staff |  |  |  |  |

| Assessment Task 02 |  |  | Perform countouring of area assigned by assessor by squaring method. Note: Assessor should provide the approperiate area according to the given time frame. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Time Required |
|  | Carry out OHS requirments |  |  |  |  | 03 hours |
|  | Identify and select instruments for contouring |  |  |  |  |  |
|  | Perform temporary adjustment of levelling instrument |  |  |  |  |  |
|  | Locate points at regular intervals |  |  |  |  |  |
|  | Take readings of staff at specified intervals |  |  |  |  |  |
|  | Compute reduced levels for readings taken |  |  |  |  |  |
|  | Draw squares for indirect method |  |  |  |  |  |
|  | Draw contour lines on drawing sheet by computing reduced level of contours by interpolation |  |  |  |  |  |
|  | Recheck calculations |  |  |  |  |  |
|  | Label contours on drawing sheet |  |  |  |  |  |
| Materials Required |  |  |  |  |  |  |
| Sr. |  | Equipment Tool |  |  |  |  |
| 1 |  | Tripod Stand |  |  |  |  |
| 2 |  | Auto Level |  |  |  |  |
| 3 |  | Levelling staff |  |  |  |  |
| 4 |  | Measuring tape |  |  |  |  |
| 5 |  | Peg |  |  |  |  |
| 6 |  | Mallet |  |  |  |  |
| 7 |  | Drawing table |  |  |  |  |
| 8 |  | T-scale |  |  |  |  |
| 9 |  | Set Square |  |  |  |  |

| Assessment Task 03 |  |  | Perform traversing of area assigned by assessor using digital theodolite considering requirements (number of sides and offset details) given by assessor. Note: Assessor should provide the approperiate number of sides and offsets according to the given time frame. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Time Required |
|  | Carry out OHS requirements |  |  |  |  | 03 hours |
|  | Mark station points to cover given area |  |  |  |  |  |
|  | Perform temporary adjustment of theodolite on starting point of traverse |  |  |  |  |  |
|  | Measure bearing of first line |  |  |  |  |  |
|  | Measure horizontal internal / external angle between all lines by shifting theodolite on each station |  |  |  |  |  |
|  | Measure distances between station points |  |  |  |  |  |
|  | Measure offsets of distinct objects nearby |  |  |  |  |  |
|  | Compute bearings of each line using internal / external angles. |  |  |  |  |  |
|  | Compile observed data in tabular form |  |  |  |  |  |
| Materials Required |  |  |  |  |  |  |
| Sr. |  | Equipment Tool |  |  |  |  |
| 1 |  | Pegs |  |  |  |  |
| 2 |  | Mallet |  |  |  |  |
| 3 |  | Ranging rods |  |  |  |  |
| 4 |  | Tripod stand |  |  |  |  |
| 5 |  | Digital Theodolite |  |  |  |  |
| 6 |  | Measuring tape |  |  |  |  |
| 7 |  | Compass |  |  |  |  |

| Assessment Task 04 |  |  | Measure a line through different obstacles using triangulation approach. Note: Assessor should provide the approperiate length of sides according to the given time frame. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Time Required |
|  | Carry out OHS requirements |  |  |  |  | 03 hours |
|  | Carry out reconnaissance of area |  |  |  |  |  |
|  | Select tools and equipments |  |  |  |  |  |
|  | Mark stations according to requirement |  |  |  |  |  |
|  | Measure base line according to standard procedure by applying necessary corrections |  |  |  |  |  |
|  | Measure required angles by theodolite |  |  |  |  |  |
|  | Calculate length of line using triangulation |  |  |  |  |  |
| Materials Required |  |  |  |  |  |  |
| Sr. |  | Equipment Tool |  |  |  |  |
| 1 |  | Pegs |  |  |  |  |
| 2 |  | Mallet |  |  |  |  |
| 3 |  | Ranging rods |  |  |  |  |
| 4 |  | Tripod Stand |  |  |  |  |
| 5 |  | Digital theodolite |  |  |  |  |
| 6 |  | Measuring tape |  |  |  |  |

Note: There are 4 options for summative assessment.

1st option: Task 1 & Task 3

2nd option: Task 2 & Task 3

3rd option: Task 1 & Task 3

4th option: Task 2 & Task 4