Course / Subject Name ____NQ-Civil-333_ Transportation Engineering__

- Assessment Tasks (Summative Assessment)

Task No1:

Produce sketches of Highway geometrics and cross section of Flexible/Rigid Pavement

Performance Criteria:

- Identify drawing tools for drawing

- Fix drawing sheet on drafting table

- Sketch cross section of a highway outlining geometric elements

- Label individual geometric elements on sketch

- Label carriageway, formation width, and right of way on the sketch

- Sketch subgrade layer as foundation of flexible pavement

- Draw sub-base layer with specified thickness

- Indicate material of subbase layer

- Draw base course with specified thickness

- Indicate material of base layer

- Draw binder layer with specified thickness

- Indicate material of binder layer

- Sketch the surface or wearing course

- Indicate material of surface layer

- Show requierd camber for surface drainage in rigid pavement

- Sketch subgrade layer as foundation of rigid pavement

- Draw base course with specified thickness

- Indicate material of base layer

- Sketch surface concrete slab with specified thickness

- Show requierd camber for surface drainage in rigid pavement

- Detail with requried annotations

- Label all the sketches

Task No2:

Perform quality tests on stone metal and betumen used in road construction including Los Angles Abration test, and Flash and fire point test

Performance Criteria:

- Wear Personal Protective Equipment (PPE), including gloves, safety goggles, and dust masks.

- Use well-maintained and calibrated equipment to avoid malfunctions.

- Conduct tests in a designated and ventilated area to minimize exposure to dust and noise.

- Identify equipment for Los Angeles Test

- Prepare sample for test

- Place the charges and sample in drum

- Operate the machine

- Determine mass loss after specified number of revolutions

- Calculate percentage of wear

- Heat betumen sample to make it softer such that it can be poured in the cup for falsh and fire point test

- Pour sample in the cup

- Make the top surface of cup and sample levelled

- Place the cup with specimen in the apparatus

- Apply heat to sample in the cup

- Stir the bitumen to maintain uniform temperature in sample

- Record the temperature for flash point

- Record the temperature for fire point

- Assessment Tasks (Summative Assessment)

Task No1:

Produce sketches of permanant way and layout plan of an airport

Performance Criteria:

- Identify drawing tools for drawing

- Fix drawing sheet on drafting table

- Sketch subgrade as foundation of permanant way

- Sketch ballast layer over the subgrade.

- Draw sleepers

- Indicate the length and thickness of sleepers.

- Add rails on top of the sleepers

- Indicate profile of rails

- Indicate width of track between the rails

- Add fasteners used to secure the rails to the sleepers.

- Label all components clearly with dimensions

- Indicate material of each component

- Mark boundaries of airport on drawing sheet to sketch layout plan of airport

- Sketch runway in any standard orientation

- Sketch apron area

- Sketch terminal area

- Add taxiways connecting runways to apron and terminals.

- Add hangars area

- Add parking area for vehicals

- Mark air trafffic control towers

- Label all elements clearly

Task No2:

Perform quality tests on betumen used in road construction including ductility test and Softening point test

Performance Criteria:

- Wear Personal Protective Equipment (PPE), including gloves, safety goggles, and dust masks.

- Use well-maintained and calibrated equipment to avoid malfunctions.

- Conduct tests in a designated and ventilated area to minimize exposure to dust and noise.

- Identify equipment for ductility test of betumen

- Assemble bracket moulds to recieve specimen

- Heat the sample to make it softer such that it can be poured in moulds

- Pour the sample into bracket moulds

- Make the top surface of bracket moulds and sample levelled

- Place the sample in ductilitimeter such that sample is completely drowned in water

- Stretch the two ends of moulds

- Measure the distance covered during test

- Identify equipment for softening point test

- Heat the sample to make it softer such that it can be poured into the rings

- Level the top surface of specimen with top surface of rings

- Assemble the apparatus to perform test

- Place specimen rings with balls on top in water bath

- Apply heat to water bath to increase the temperature until the balls fall the standard distance

- Record temperature at which balls fall the standard distance