Course / Subject Name: NQ-ET 125 Principles of Electrical Engineering

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-9) | LU (10-14) | LU (15-20) |
| - If 2.2kΩ Resistor is connected across 12V Supply, the current will be - 4.5 mA - 5.4 mA - 6.5 mA - 5 mA - According to Ohms law - V= R/V - V= IR - V= V/R - V= R/I - According to law of resistance the resistance of conductor is directly proportional to - Area - Length - Specific resistance - Both B and C - The number of path for the flow of current in a series circuit is - Many - Two - Four - One - What is the total resistance of a parallel circuit with two resistors of 10Ω and 4Ω? - 0.7Ω - 2.8Ω - 3Ω - 0.6Ω - In a parallel circuit, if the voltage across one resistor is 6V, what is the voltage across the other resistors? - 3V - 6V - 9V - 12V - In a series circuit with three resistors, if the current flowing through one resistor is 1mA, what is the current flowing through the other two resistors? - 1mA and 2mA - 2mA and 3mA - 1mA and 1mA - 2mA and 2mA - What is the power consumed by a 2kΩ resistor in a series circuit with a total voltage of 5V? - 1.5W - 0.0125W - 0.5W - 1.3W - In a series circuit with three resistors, if one resistor is removed, what happens to the total resistance? - Increases - Decreases - Remains the same - Becomes zero - One HP is equal to - 756 watt - 746 watt - 748 watt - 796 watt - In a parallel circuit, what happens to the total current if one resistor is short-circuited? - Increases - Decreases - Remains the same - Becomes zero - Thermal efficiency is the ratio of - Heat actually utilized / total heat produced electrically - Total heat produced electrically / heat actually utilized - Heat loss/ heat produce - all of them - Electrical energy bill is calculated with: - KWh × rate/unit - KW × rate/unit - KVh × rate/unit - Watt sec × rate/unit - According to Kirchhoff’s current law algebraic sum of all current meeting at a point is - Same - Zero - Different - 1 - If the band colors of resistor is Red-Red-Red-Gold, then the value of resistor is - 2.5 kΩ + 5% - 2.5 kΩ + 10% - 2 kΩ + 5% - 22.5 kΩ + 5% - Unit of Current is - Ampare - Volt - Ohm - Watt - Multi meter is used to measure - Voltage - Current - Resistance - all of the above - Watt meter is used to measure - Voltage - Power - Resistance - all of the above - In series circuit by adding resistor, total resistance will be - Increase - Decrease - constant - None of the above - In series circuit the voltage drop across each resistor is - same - Different - Decrease - Increase - The reciprocal of Resistance is - Impedance - Conductance - Reluctance - current - In Parallel circuit the voltage across each resistor is - Same - Different - Decreas - Increase - Tolerance for Silver band resistor is - + 5% - + 10% - + 15% - + 20% - According to Laws of resistance R is equal to - ƍl/A - Vl/A - ƍl/ - ƍl × A - According to Kirchhoff’s voltage law Voltage Rise is equal to - Voltage drop - Current drop - Voltage loss - None of the above - 0.75 HP is equal to - 550 Watt - 540 Watt - 559.5 Watt - 545 watt - 1 kilocalorie is equal to - 4100 joules - 4500 joules - 3187 joules - 4187 joules - What is the formula for electrical power? - P = V × I - P = V ÷ I - P = I × R - P = R ÷ V - If a device has a power rating of 500W and it operates for 5 hours, how much energy does it consume? - 550Wh - 500Wh - 2500Wh - 2000Wh - If a device consumes 200W of power, and the voltage supply is 230V, what is the current drawn by the device in Amperes? - 0.86A - 1A - 2A - 5A Answer Keys: 1; b; 2; b; 3; d; 4; d; 5; b. 6; b; 7; c; 8; b; 9; b; 10; b. 11; b; 12; a; 13; a; 14; b; 15; a. 16; a; 17; d; 18; b; 19; a; 20; b. 21; b; 22; a; 23; b; 24; a; 25; a. 26; c; 27; d; 28; a; 29; c; 30; a. | - Magnet lines of force always enter from - Horizontally - Vertically - North to South - South to North - Flux per unit area in magnet is called - Flux - Flux density - MMF - Ampere turns - Weber is the unit of - MMF - EMF - Flux - Flux density - The force which derives the flux in a magnet circuit is called - MMF - Flux density - EMF - Ampere turns - According to Fleming left hand rule first figure indicate - Magnetic field - Current - Motion of conductor - EMF - When a conductor cut magnetic flux then____ induced - E.M.F - Current - Flux - Resistance - Inductance is denoted by - M - W - Ohm - L - EMF is induced only when - Flux Linked with coil - Flux Change Remain constant - Both B & C - Which law state that “effect always opposes the cause” - Faradays law - Fleming law - Lenz law - Coulomb’s law - Cork screw rule determine the direction of - current - Power - Magnetic field - Both a & c - When magnetic field is stationary and conductor is in motion then induced EMF is called - Statically induced EMF - Dynamically induced EMF - Mutual induced EMF - Self-induced EMF - To find the direction of motor rotation, we use - Ohms law - Mutual induction law - Faraday’s law - Fleming left hand rule - Capacitor stores electric charge in - Plates - Dielectric - Battery - Both a & b - Unit of capacitance is - Farad - Micro farad - Henry - Both a & b - The charge of 10 micro farad capacitor when voltage is applied to 10V - 1mC - 0.1mC - 0.01C - 0.1mC - The formula to find the capacitance is - C=QV - C=Q/V - C=V/Q - C=Q/W - Four capacitors of 2 µ F are connected in parallel the equivalent capacitance is - 8 µF - 0.5 µF - 2 µF - 6 µF - By increasing the Number of capacitors in series, the total capacitance will - Decreases - Increases - Remain same - None of these - Opposites charge ____ each other - Attract - Repel - Not effected - None of these - The material between two capacitor plates is called - Semiconductor material - Magnetic material - Dielectric - None - What is the purpose of a capacitor in a circuit? - To increase power - To filter out DC signals - To store energy - To regulate voltage - A coil of wire has an inductance of 10 mH. If the current through the coil changes at a rate of 5 A/s, what is the induced voltage? - 50 V - 100 V - 500 V - 1000 V - An electronic component which can store energy in the form of electrostatic field is called - Capacitors - Inductor - Resistor - None of these - A coil of wire has an inductance of 5 H. If a current of 2 A flows through the coil, what is the energy stored in the magnetic field? - 10 J - 20 J - 50 J - 100 J - Which of the following laws states that the induced voltage in a coil is proportional to the rate of change of magnetic flux? - Faraday's law of induction - Lenz's law - Ohm's law - Kirchhoff's law - What is the effect of increasing the plate area of a capacitor on its capacitance? - Decreases capacitance - Increases capacitance - No effect on capacitance - Decreases capacitance and increases inductance - Which of the following types of capacitors has the highest capacitance per unit volume? - Ceramic capacitor - Film capacitor - Electrolytic capacitor - Air capacitor - A coil of wire has an inductance of 10 mH. If the current through the coil changes from 0 A to 5 A in 0.1 s, what is the induced voltage? - 0.5 V - 10.0 V - 5.0 V - 0 V - What is the principle behind the operation of an electric generator? - Electromagnetic induction - Electromagnetic radiation - Electromagnetic conduction - Electromagnetic resistance - Which of the following statements about Lenz's law is true? - the induced current flows in the same direction as the changing magnetic flux - The induced current flows in the opposite direction to the changing magnetic flux - The induced current flows in a direction perpendicular to the changing magnetic flux - The induced current flows in a direction parallel to the changing magnetic flux Answer Keys: 1; c; 2; b; 3; c; 4; a; 5; a. 6; a; 7; d; 8; b; 9; c; 10; d. 11; b; 12; d; 13; a; 14; d; 15; a. 16; b; 17; a; 18; a; 19; a; 20; c. 21; c; 22; a; 23; a; 24; a; 25; a. 26; b; 27; c; 28; a; 29; a; 30; b. | - What is the unit of power factor? - Watts - Volts - Amperes - None of the above - What is the power factor of a purely resistive circuit? - 0 - 0.5 - 1 - 2 - What is the purpose of using an oscilloscope in an AC circuit? - To measure voltage - To measure current - To measure power factor - To display the waveform - What is the RMS value of a sinusoidal waveform with a peak value of 10V? - 5V - 7.07V - 10V - 14.14V - What is the effect of low power factor on a power system? - Increased efficiency - Decreased efficiency - No effect - Increased power consumption - How can the power factor of a circuit be improved? - By adding a capacitor - By adding an inductor - By adding a resistor - By adding a transformer - What is the phase sequence of a three-phase system? - RYB - RBY - BYR - YRB - What is the function of a wattmeter in a three-phase circuit? - To measure voltage - To measure current - To measure power - To measure power factor - What is the difference between a single-phase and three-phase power system? - Number of phases - Number of wires - Type of load - Type of generator - How is the power factor of a circuit calculated? - Using the formula PF cos(θ)= V/I - Using the formula PF cos(θ)= P/VI - Using the formula PF cos(θ)= V/IR - Using the formula PF cos(θ)= sin(θ) - What is the purpose of using a capacitor in a power factor correction circuit? - To increase the power factor - To decrease the power factor - To increase the voltage - To decrease the voltage - What is the effect of adding an inductor to a circuit on the power factor? - Increases the power factor - Decreases the power factor - No effect on the power factor - Increases the voltage - What is the unit of measurement for inductive reactance? - Ohms - Henrys - Farads - Siemens - What is the Average value of a sinusoidal waveform with a peak value of 20V? - 12.72V - 14.14V - 20V - 28.28V - How can the power factor of a circuit be improved using a capacitor? - By adding a capacitor in series with the load - By adding a capacitor in parallel with the load - By adding a capacitor in series with the power source - By adding a capacitor in parallel with the power source - What is the phase angle between the voltage and current in a circuit with a power factor of 0.8? - 36.8° - 45.7° - 60.3° - 90° - What is the main difference between AC and DC supply? - AC supply has a fixed voltage, while DC supply has a variable voltage - AC supply has a variable voltage, while DC supply has a fixed voltage - AC supply has a sinusoidal waveform, while DC supply has a rectangular waveform - AC supply has a rectangular waveform, while DC supply has a sinusoidal waveform - What is the RMS value of a sinusoidal waveform with a peak value of 5V? - 10V - 14.14V - 3.53V - 28.28V - Frequency in Pakistan is - 60 c/s - 65c/s - 50 c/s - 55 c/s - The speed of two pole generator is 3000RPM the frequency is - 30 c/s - 60 c/s - 50 c/s - 90 c/s - What is the formula for calculating three-phase power? - P = √3 x V x I x cos(θ) - P = 3 x V x I x cos(θ) - P = √3 x V x I x sin(θ) - P = 3 x V x I x sin(θ) - in 3 phase system the angular displacement among three phases - 90° - 120° - 0° - 360° - To measure active power____ meter is used - Ampare meter - KVAR meter - KWh meter - KW meter - Formula of reactive power is - Q = V x I x sin(θ) - Q = VIR - Q = V x I x cos(θ) - Q = V x I - Apparent power is denoted by - Q - S - R - A - The voltage between phase wire and neutral wire is - line voltage - Phase voltage - Phase to phase voltage - Both a and b - The four wire three phase star connected system has a line current is 60A, the phase current is - 30A - 34.64A - 40.5A - 55A - we can check different waveforms in - Oscilloscope - Watt meter - Megger - None of these - Time period is the reciprocal of - Frequency - Speed - Poles - impedance - Impedance is denoted by - I - XL - Z - Xc Answer Keys: 1; d; 2; a; 3; d; 4; b; 5; d. 6; a; 7; a; 8; c; 9; a; 10; b. 11; a; 12; b; 13; a; 14; a; 15; b. 16; a; 17; b; 18; c; 19; c; 20; c. 21; a; 22; b; 23; b; 24; a; 25; b. 26; b; 27; b; 28; a; 29; a; 30; c. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-9) | LU (10-14) | LU (15-20) |
| - Define Electrical Energy? - Describe active circuit? - Find Current of 100W, 220V light bulb when it is lit? - Define Kirchhoff’s voltage law? - R1=13 ohm, R2=20 ohm are connected in parallel, find total resistance - Define Ohm’s Law - What are the laws of resistance - What is the difference between series and parallel circuit - Enlist different Types of resistors. - What is the value of resistance for color band red-black-black-silver - Two resistors R1=5ohm and R2=7 ohm are connected in parallel, calculate total resistance - two resistors R1and R2 are connected in parallel across 12volt supply if current I=4.7A entering a node, what will be the current in R2 when current in R1 is 2.5A? - Define thermal efficiency. - What is the trade unit of electrical energy. - Find Current of 200W, 240V light bulb when it is lit. - Define Kirchhoff’s current law. - R1=10 ohm, R2=20 and R3=30 ohm are connected in series across 15V DC supply, find total current flowing in the circuit. - what is the relation between Current, Voltage and resistance? - write down the formula of energy billing for a month (30days)? - Define specific resistance of material. | - Write down any 5 properties of magnetic flux. - Define Flux density. - What is the different between magnetic flux and elctric flux? - What is the unit of inductance? - What are the different magnetic materials? - What is the direction of magnetic lines of force around a straight wire carrying current? - What happens to the magnetic lines of force when the current in the wire is increased? - What is the purpose of using a compass to verify the properties of magnetic lines of force? - what is the Magnetic Effects of Electric Current? - What rule is used to identify the direction of the magnetic field produced around a straight wire carrying current? - What happens to the magnetic field when the current in the wire is increased? - Define Faradays law. - Define Lenz Law. - What is the relationship between the induced EMF and the number of turns of the coil? - Define Fleming Left hand Rule. - How to calculate total capacitance of two capacitors connected in series? - What is the charge on each capacitor in a series circuit? - Define breakdown voltage of capacitor. - What is dielectric strength. - Two capacitors C1 and C2 are connected in series across 12V supply, calculate charge on each capacitor. | - What is the main difference between AC and DC supply? - What is the function of an oscilloscope? - What is the RMS value of a sinusoidal waveform? - Calculate the average value of a sinusoidal waveform with a peak value of 10V. - What is the power factor of a circuit with a resistance of 10 ohms and an impedance of 20 ohms? - What are the causes of low power factor? - How can power factor be improved? - Write the formula for calculating power factor. - A single-phase motor has a power consumption of 5 kW. Calculate its power factor if the current drawn is 10 A. - What is the phase sequence of a three-phase system? - What is the purpose of using an oscilloscope to measure a sinusoidal waveform? - How does the power factor of a circuit affect its efficiency? - Calculate the total power consumption of a three-phase circuit with a power factor of 0.8 and a line current of 20 A. - What is the difference between a single-phase and three-phase power system? - How can a wattmeter be connected to measure the power consumption of a three-phase circuit? - Calculate the number of units per division on an oscilloscope with a voltage range of 10V and a screen width of 10 cm. - What are the advantages of improving the power factor of a circuit? - How can the power factor of a circuit be improved using a capacitor? - A three-phase motor has a power consumption of 15 kW. Calculate its power factor if the line current is 25 A. - What is the effect of low power factor on the efficiency of a power system? |

C. Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-9) | LU (10-14) | LU (15-20) |
| Construct a circuit in which three resistors of different values are connected in series across 9 volt DC power supply, measure the voltage drop across each resistor, also calculate the current and power consumed by following occupational health and safety. Performance criteria: - Apply Personal Protective Equipment’s (PPEs) - Select three different types of resistors. - Identify the value of resistors by color coding. . Connect resistors in series. - Install the resistors in series circuit - Connect power supply to the circuit - Measure the total voltage by volt meter. - Calculate the voltage drop across each resistor. - Calculate the current in the circuit using Ohm‘s Law: I = V/R - Calculate the total power consumed by using the formula P = V x I - Record and analyze the data - Disassmemble the circuit and place all the components in proper place. | Construct, and test a series RC circuit to investigate the magnetic field around the coil and measure breakdown voltage of capacitor by following heath and safety rules. Performance criteria: - Select PPEs for insuring health and safety - Select the required tools/instruments - Select the required components - Construct the circuit to investigate and record the breakdown voltage of capacitor - install the coil and capacitor in series circuit. - Connect power supply to the circuit - Measure the magnatic field around the coil - Insert iron core in a coil and observe the effect - increase the voltage Gradually - Note down the breakdown voltage of capacitor - analyze and record the data in a table - Disassmemble the circuit and place all the components in proper place. | a) Connect an Oscilloscope to a single phase supply and measure the Maximum voltage ,calculate Vr.m.s, V average, Peak to Peak Voltage Performance criteria: - Select PPEs for ensuring health and safety - Select the required tools/Equipments and instruments - Select the required components - Draw a connection diagram - Construct the circuit to measure Max voltage - Connect Oscilloscope with AC supply - Measure the Vmax value - Calculate Vr.m.s - Calculate V average - Calculate V Peak to Peak b) Connect an inductive load with three phase supply to calculate line voltage aslo check the phase sequence by phase sequence meter. Performance criteria: - Select PPEs for insuring health and safety - Select the required tools/Equipments and instruments - Select the required components - Draw a connection diagram - Construct the three phase circuit with inductive load to measure line voltage and phase sequence. - connect the inductive load installed with three phase supply - Calculate Line Voltage By Voltmeter - Connect Phase sequence Meter and record the phase sequence. - analyze and record the data in a table - Disassmemble the circuit and place all the components in proper place. |