Course / Subject Name: NQ-ET 252 - Electrical Installation, Planning & Estimating

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 3) | LU (4 to 5) | LU (6) |
| - Which PPE is essential when working on earthing installation? a) Hard hat b) Insulated gloves c) Safety boots d) All of the above - What is the primary purpose of safety protocols in earthing installation? a) To increase the speed of installation b) To ensure the safety of workers and electrical systems c) To reduce the cost of installation d) To avoid using protective equipment - Which tool is essential for measuring earthing resistance? a) Multimeter b) Clamp Meter c) Earth resistance tester d) Insulation tester - What is the most critical factor in selecting an earthing location? a) Availability of tools b) Soil resistivity c) Proximity to trees d) Depth of excavation - Why is site assessment essential for earthing installation? a) To minimize costs b) To ensure efficient conductivity and safety c) To meet construction timelines d) To reduce the need for additional tools - Which material is commonly used for earthing rods? a) Aluminum b) Copper c) Plastic d) Iron - What type of cable is typically used for earthing systems? a) PVC cables b) Insulated aluminum cables c) Bare copper cables d) Armored cables - How deep should the pit for the earthing electrode be? a) As shallow as possible b) Based on soil resistivity and regulatory standards c) Fixed at 2 meters d) 1 meter below the surface - What is the purpose of the conductive mixture in the pit? a) To prevent corrosion b) To enhance electrical conductivity c) To stabilize the electrode d) To ensure mechanical strength - What method is commonly used to connect the earthing conductor to the electrode? a) Welding b) Clamp connection c) Bolted connection d) All of the above - What should be done if the earthing resistance is higher than recommended? a) Ignore it b) Add more electrodes or conductive materials c) Replace the electrode with a smaller one d) Decrease the depth of the pit - What is the primary purpose of integrating the earthing system with the main distribution board? a) To complete the electrical circuit b) To prevent overloading c) To ensure fault currents are safely directed to the ground d) To improve the appearance of the installation - What is a common material used in the conductive mixture for backfilling an earthing pit? a) Cement b) Bentonite c) Gravel d) Sand - Why is the conductive mixture critical in the earthing process? a) It provides mechanical support to the electrode. b) It increases soil resistivity for better performance. c) It ensures consistent electrical contact with the soil. d) It minimizes the cost of the installation. - What is the ideal resistance value for a domestic earthing system? a) Less than 10 ohms b) 50-100 ohms c) 1-5 ohms d) Greater than 100 ohms - What tool is typically used for resistance testing in an earthing system? a) Megger b) Earth resistance tester c) Clamp meter d) Voltage tester - When should the resistance testing of an earthing system be performed? a) Before the installation begins b) After completing the installation and periodically thereafter c) Only during the initial site assessment d) After integrating the system into the main board - Which of the following is a type of earthing system? a) Plate earthing b) Pipe earthing c) Rod earthing d) All of the above - How does earthing protect electrical equipment? a) By increasing current flow b) By providing a path for fault currents to flow to the ground c) By reducing voltage fluctuations d) By enhancing the insulation of wires - How often should an earthing system be inspected for optimal performance? a) Monthly b) Annually c) Every 5 years d) Only during installation - What is a common indicator of a malfunctioning earthing system? a) Increased energy efficiency b) Frequent tripping of circuit breakers c) Higher voltage in appliances d) Reduced power supply - The resistance of earthing system should not be; a) More than 5 ohm b) More then 2 ohm c) Less Than 1 ohm d) More Than 3 ohm - The length of earth rod is; a) 4 feet b) 8 feet c) 10 feet d) 11 Feet - In wiring short circuit, megger needle indicates; a) zero b) Infinity c) some reading d)Low Resistance - The maximum size of earthing lead; a) 8 SWG b) 6 SWG c) 2 SWG d)1SWG Answer Keys: Formative Assessment (Part A). 1.d; 14.c. 2.b; 15.c. 3.c; 16.b. 4.b; 17.b. 5.b; 18.d. 6.b; 19.b. 7.c; 20.b. 8.b; 21.b. 9.b; 22.c. 10.d; 23.b. 11.b; 24.a. 12.c; 25.a. 13.b. | - Which of the following PPE is essential for electrical installation work to prevent electrical shock? a) Work gloves b) Insulated rubber gloves c) Leather gloves d) Protective apron - Before starting a domestic wiring installation what is the most important safety consideration? a) Ensure the area is clean and organized b) Make sure all tools are working c) Ensure the installation complies with local regulations and safety standards d) Have the customer sign a waiver - What should be done to prevent electrical hazards during the installation of wiring? a) Keep cables exposed to allow for better cooling b) Avoid using insulated tools c) Ensure all electrical connections are covered and no live wires are exposed d) Allow the wiring to remain untied for easier adjustments - In load calculation what is the formula used to calculate the total load? a) V/I b) PxV c) IxR d) P=VI Answer: d) P=VI - What is the first step in interpreting a wiring plan for installation? a) Cutting the wires b) Identifying the load requirements and locations of switches and sockets c) Selecting the right tools and materials d) Installing the PVC casing - Which factor is most important when selecting PVC casing and capping for wiring? a) The color of the casing b) The size and type of cables to be used c) The price of the material d) The brand of PVC casing - What tool is essential for securely fixing PVC casing and capping to walls? a) Electric drill with masonry bits b) Pliers c) Hammer d) Screwdriver with appropriate screws - How should PVC casing be cut to ensure precision? a) Use a saw with large teeth b) Measure and mark carefully, then cut using a hacksaw or PVC cutter c) Cut randomly to speed up the process d) Use a utility knife for precision cuts - When pulling wires through PVC casing what should be avoided to ensure the wire remains undamaged? a) Pulling wires quickly b) Pulling wires over sharp edges or corners c) Stretching the wire to make it longer d) Using gloves during installation - What is the first step when connecting wires to switches or sockets? a) Strip the insulation from the wires carefully b) Check if the circuit is live c) Use a hammer to connect the wires d) Wrap wires around the switch - What is the primary purpose of performing a visual inspection after installation? a) To ensure the installation is neat and the wires are labeled b) To check if the circuit is live c) To ensure the wires are fully insulated d) To determine if the switches are functioning - During visual inspection which of the following should be checked? a) All connections are tight and secure b) No wires are exposed c) All components are properly secured and aligned d) All of the above - What is the primary purpose of performing tests on a newly installed wiring system? a) To measure the load calculation b) To ensure safety and functionality of the system c) To check the wire colors d) To verify the installation time - Which test should be performed to check the continuity of the wiring? a) Insulation resistance test b) Earth resistance test c) Continuity test using a multimeter d) Voltage test - Which of the following tests is commonly used to check the insulation resistance of the wiring? a) Earth leakage test b) Insulation resistance test using a Megger c) Continuity test d) Power factor test - How can you ensure a neat and professional wiring installation? a) Use excessive quantities of wire b) Securely fasten all cables with cable ties or clips c) Use random wiring paths d) Leave slack in the wires for adjustments - What is the final step after completing the installation and inspection of the wiring system? a) Turn off all circuit breakers b) Test the wiring to ensure it functions correctly c) Immediately connect all electrical appliances d) Seal all wiring with duct tape - Cable used in batten wiring; a) VRI b) PVC c) Flexible d)Communication Cable - Switch board is fitted at a height ground; a) 3 feet b) 5 feet c) 6 feet d) 8 feet Answer Keys: Formative Assessment (Part B). 1.b; 11.a. 2.c; 12.d. 3.c; 13.b. 4.d; 14.c. 5.b; 15.b. 6.b; 16.b. 7.d; 17.b. 8.b; 18.a. 9.b; 19.b. 10.a. | - What is the first step in estimating materials for PVC surface conduit wiring? a) Calculate the cost of materials b) Review the residential wiring plan to understand the layout and requirements c) Measure the length of each wall d) List all the fixtures in the house - What should you focus on while reviewing the wiring plan for a residential project? a) Number of electrical outlets b) The layout and circuit requirements c) Types of wiring materials available d) Size of the rooms - When calculating the total length of PVC conduit, what factor must be considered? a) The number of bends and junction boxes b) The number of circuit breakers c) The color of the conduit d) The type of switches used - How is the total length of PVC conduit determined for a wiring layout? a) By calculating the room's perimeter b) By summing the length of each section of conduit needed for the layout c) By dividing the total area of the house by the average length of conduit d) By estimating the total number of fittings - What is the purpose of estimating the number of junction boxes in the wiring plan? a) To determine the total cost of the project b) To provide space for circuit connections and protect the wiring c) To calculate the total number of switches needed d) To optimize the size of the room - Which of the following factors determines the gauge of wire needed for a circuit? a) The color of the wiring b) The length of the conduit c) The load capacity and power requirements d) The type of room - What should be calculated to determine the number of switchboards and sockets needed? a) The size of the appliances used in the home b) The number of circuits and locations where switches and outlets are required c) The number of electrical engineers on the project d) The length of the walls - How should the number of light fixtures and other electrical fixtures be estimated? a) By the number of electrical outlets b) By reviewing the lighting and fixture requirements in the plan c) By calculating the total wattage of all appliances d) By dividing the total number of rooms by the number of fixtures - What additional accessories must be considered when estimating materials for PVC conduit wiring? a) Paint for the conduit b) Clamps, screws, and mounting hardware c) Cleaning supplies d) Wall paint and decorations - Why are clamps and screws important in estimating materials for PVC conduit wiring? a) They help secure the conduit in place b) They are used to decorate the conduit c) They are used only for aesthetic purposes d) They are only needed for outdoor wiring - Why should allowances for waste and overlaps be included in the material estimate? a) To account for errors during installation and future adjustments b) To reduce the total cost of the materials c) To save space on the plan d) To make the installation process faster - How much extra material should typically be allowed for waste and overlap? a) 5-10% b) 15-20% c) 20-30% d) 50% - What should be included in a detailed material list for the PVC surface conduit wiring? a) Only the cost of each item b) A list of quantities, specifications, and cost breakdown c) Only the dimensions of the conduit d) The number of employees needed for the installation - Why is it important to prepare a detailed document listing materials for the wiring plan? a) To keep track of the installation process b) To communicate the material requirements clearly to the procurement team c) To avoid ordering excessive materials d) To calculate the project cost accurately - What is the purpose of cross checking the material estimate with the wiring plan? a) To ensure that the right colors of wiring are selected b) To verify the accuracy and completeness of the estimate c) To ensure the estimate is within budget d) To optimize the layout of the electrical system - When cross checking the material estimate, which of the following should be ensured? a) The wire color is correct b) The total length of conduit matches the layout c) The cost of materials is minimized d) All fixtures are aesthetically pleasing - How should future modifications be accounted for in the material estimate? a) By reducing the number of items required b) By allowing extra materials for potential changes to the system c) By ignoring future modifications entirely d) By providing the exact number of items needed - What is the recommended percentage of material allowance for waste, overlap, and future modifications in most cases? a) 1-3% b) 10-15% c) 5-10% d) 20-25% - Which of the following is essential for estimating labor costs during PVC conduit installation? a) The number of workers on site b) The total length of the PVC conduit c) The quality of the conduit d) The type of conduit fittings - What should be done to ensure the material estimate fits within the project’s budget? a) Use cheaper materials for everything b) Adjust the material quantities and gauge for the lowest cost c) Cross check the material specifications, quantities, and market prices to stay within budget d) Use more materials than necessary for reliability - Why is it important to calculate the total cost of materials before starting installation? a) To prevent running out of materials mid installation b) To ensure the installation is completed quickly c) To meet project deadlines d) To compare costs between different contractors - What should be considered to ensure timely material delivery? a) Only order the materials the day before installation b) Order materials well in advance based on the project timeline c) Delay delivery until all fixtures are installed d) Order only half of the required materials - How can material shortages be avoided during the installation process? a) By overestimating the required materials b) By using cheaper, low quality materials c) By accurately estimating the material requirements and ordering extra items for flexibility d) By reducing the scope of the project - What should be documented when receiving materials for the PVC conduit installation ? a) Only the number of items received b) The delivery date, quantity, specifications, and any discrepancies from the order c) The contractor’s contact details d) The color and brand of the materials - Why is it important to keep a record of material deliveries? a) To avoid material wastage b) For warranty and quality tracking c) To ensure the correct quantity and type of materials are received d) To optimize labor usage - Before finalizing the material list what should be verified? a) The total number of workers required for installation b) The material specifications match the wiring plan c) The house is completely empty of furniture d) The contractor’s overhead costs - After completing the material estimate what is the final step? a) Reduce the total material quantity to save costs b) Finalize and cross-check the estimate against the wiring plan c) Delay the installation until materials are cheaper d) Discard unnecessary materials Answer Key: Formative Assessment (Part C). 1.b; 15.b. 2.b; 16.b. 3.a; 17.b. 4.b; 18.c. 5.b; 19.b. 6.c; 20.c. 7.b; 21.a. 8.b; 22.b. 9.b; 23.c. 10.a; 24.b. 11.a; 25.c. 12.a; 26.b. 13.b; 27.b. 14.b. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 3) | LU (4 to 5) | LU (6) |
| - Define Earthing System. - Enlist Different Method of Earthing. - Enlist the Components of Earthing. - What type of earthing is most commonly used for domestic applications? - What does PPE stand for in the context of installation work? - Define Earth Continuity Conductor. - What tools and equipment are essential for earthing installation? - Define Earth Electrode. - What factors should be considered when selecting earthing rods, cables, and connectors? - What is the purpose of excavating a pit during the earthing installation process? - Why is a conductive mixture used for backfilling the earthing pit? - Which Instrument is used to verify compliance with earthing standards? - What are the steps to integrate the earthing system with the main distribution board (MDB)? - What common issues might arise during earthing installation and how can they be addressed? - Why is proper earthing essential in domestic and commercial wiring systems? - Define Service Line. - Why earthing is necessary? | - Define Electrical Wiring. - Enlist Deferent types of wiring? - What personal protective equipment (PPE) is required for domestic wiring installation using PVC casing and capping? - Why is it important to adhere to safety standards and local regulations during wiring installation? - Define Electrical Circuit. - Enlist the tools used for Electrical Wiring. - How do you ensure accurate measurement and cutting of PVC casing? - What is the proper method to securely fix PVC casing and capping to walls or surfaces? - How do you pull wires through the casing without causing damage or kinks? - What precautions should be taken when connecting wires to switches, sockets, and fixtures? - How do you conduct a visual inspection to ensure a neat and professional finish? - What are the key tests to perform on the wiring after installation to ensure functionality and safety? - What common issues may arise during domestic wiring installation and how can they be resolved? - Enlist Methods of Testing of Electrical Installation. - Differentiate Between Concealed and Surface Conduit Wiring. - Write down the five advantages of cleat wiring. - What is bus bar? | - Define Estimation of Electrical Installation. - What is the importance of Estimating? - Enlist Different type of Estimating Tools. - Enlist Necessary steps for Preparing Task Estimating? - Define Labor Cost. - What is Contingency in Electrical Estimation? - Enlist Different Factor Effecting Material Cost - What is the process for calculating the number of switchboards, sockets, and other fixtures? - Why is it important to estimate additional accessories like clamps and screws? - How do you account for waste, overlaps, and potential future modifications in the estimate? - What information should be included in a detailed material list with quantity specifications and costs? - How do you ensure accuracy and completeness when cross checking the material estimate with the wiring plan? - What are some potential errors that could occur during the estimation process and how can they be avoided? - Why is it important to consider circuit load while estimating materials? - What role does cost estimation play in planning for residential wiring projects? |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 3) | LU (4 to 5) | LU (6) |
| Install Earthing System for Domestic/Commercial Wiring and Connect with MDB. Performance Criteria: - Select the PPE for installation work - Follow safety protocols and ensure regulatory compliance. - Select appropriate tools and equipment required for Earthing. - Conduct site assessment to determine suitable earthing location. - Select Suitable earthing rods, cables, and connectors. - Excavate pit to required depth and install earthing electrode securely. - Backfill pit with conductive mixture for enhanced conductivity. - Connect earthing conductor to electrode using approved methods. - Conduct resistance testing to ensure compliance with standards. - Integrate the earthing system with the main distribution board. | Carry out domestic wiring installation using PVC Casing Caping for Desired Plan. Performance Criteria: - Select the PPE for installation work - Ensure the installation adheres to safety standards and local regulations. - Perform Load Calculation for the Desired Plan. - Interpret the wiring plan accurately and prepare for installation. - Select appropriate PVC casing and capping, wires, and accessories/Tools. - Measure and cut PVC casing to required lengths, ensuring precision. - Fix the PVC casing and capping to walls or surfaces. - Pull wires through the casing as per the wiring plan avoiding damage. - Connect wires to switches, sockets, and fixtures as specified, ensuring secure and compliant connections. - Perform visual inspection to confirm a neat and professional finish - Perform all Necessary Test of Wiring. | Estimate quantity of materials required as per desired residential plan for PVC surface conduit wiring. Performance Criteria: - Review the residential wiring plan to understand the layout and requirements. - Calculate the total length of PVC conduit needed for the wiring layout. - Calculate and select the required number and type of bends, junction boxes, and fittings. - Estimate the quantity and gauge of wires based on circuit load and plan specifications. - Calculate the number of switchboards, sockets, and other fixtures. - Include allowances for waste, overlaps, and potential future modifications. - Prepare a detailed Document like material list with quantities specifications and Cost. - Cross check the estimate with the wiring plan to ensure accuracy and completeness. |